DU (MSCP) Handlers allowing TSX-Plus to use Large Discs

DUCM Version V4c, 22-Aug-89

Author:  Chester Wilson, Charleville 4470, Australia.

As with all DECUS software, these programmes are distributed on an "as  is,

where  is"  basis.   The  author is interested in any bugs or problems, but

makes no committment of any sort as to the maintenance of this product.

This manual contains three parts:

(1)     Users' Guide

(2)     DUCM Manual (V4)

(3)     WA (D0, D1 etc) Manual (V3)

The Users' Guide should be sufficient reading to use these handlers, or for

non-technical readers.  For fuller details, see parts (2) & (3).

DUCM - DU Multi-Unit Internally-Queueing Handler et alii             Page 2

Table of Contents

USERS' GUIDE

1.1     Introduction

1.2     Disc Usage

1.3     Changing Handler Settings

1.3.1   Examples

1.4     TSX-Plus System Generation

1.5     Creating More Handlers

1.6     Modifying TSX

DUCM MANUAL ITSELF

2.1     Introduction

2.2     Update History

2.3     SET Commands

2.4     SPFUNs

2.4.1   Error Code I/O

2.4.2   Partition Table I/O

2.4.3   Partition Size

2.4.4   Unit Size

2.4.5   Status Table

2.4.6   Physical I/O

2.4.7   MSCP Bypass

2.5     TSMUNG - Modifying TSX

2.6     Assembly Options

2.7     Running DUCM as a Programme

2.7.1 Examining & Modifying the Handler in Memory

2.7.2 Examining & Modifying the Handler on Disc

2.8     Error Messages

WA - A MUNGING HANDLER FOR LARGE DISCS

3.1     Introduction

3.1     How it works

3.2     SET Commands

3.3     Running WA as a Programme

3.4     Restriction on /U & /P

3.5     Setting Up

3.6     Safety Precautions & Recommended Use

3.7     Example of Command File

INDEX

CHAPTER 1

Users' Guide

1.1  Introduction.

These handlers allow large discs to be fully used under TSX-Plus.  With the

normal DEC or S & H DU handlers a maximum of 256 Megabytes can be used.

Four handlers are supplied:  DUCM.TSX, D1.TSX  D2.TSX  and  D3.TSX.   These

enable discs with a total capacity of 1024 Megabytes to be supported.  More

disc capacity can be handled - see "Creating More Handlers".

These handlers consist of a primary handler,  DUCM.TSX,  that  manages  the

disc  controller, and the first 256 Megabytes of storage, and the secondary

handlers D1,D2 and D3.TSX that allow the user access to the storage  beyond

this.   The  use  of  multiple  handlers is the means of getting around the

limitation of 8 units (32 Megabytes) per  handler  imposed  on  any  device

handler by RT-11/TSX-Plus.

NOTE THAT DUCM.TSX IS NOT A COPY OF THE NORMAL TSX-Plus DU HANDLER SUPPLIED                      ___

BY  S & H,  DU.TSX.   ONLY  DUCM.TSX  CAN  BE  USED WITH THESE "LARGE DISC"

HANDLERS.  It IS however designed to be used as a REPLACEMENT for DU.TSX.

These handlers are delivered pre-configured for two  512  Megabyte  drives.

You may reconfigure them as shown below.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 1-2

Users' Guide

1.2  Disc Usage.

RT-11 and TSX-Plus communicate with large MSCP discs by  dividing  them  up

into  a  number of "disc partitions".  This is because the operating system

refers to a disc by means of 16 bit blocks numbers - therefore 64K 512 byte

blocks (32 Megabytes) is the maximum that can be referred to as a "physical

unit".  You use discs larger than 32 Megabytes by breaking them up  into  a

number  of  32  Megabyte  "Partitions",  each  of which is referred to as a

physical unit - i.e.  DU0:, DU1:, DU7:  etc.  However, as their are only  8

units  allowed per controller, with the present handlers you can only use a

disc of 256 Megabytes (8 times 32 Megabytes), i.e.:-

Drive               Partition      Addressing Range (Megabytes)

(Octal)

DU0:                0                0- 31

DU1:                1               32- 63

DU2:                2               64- 95

DU3:                3               96-127

DU4:                4              128-159

DU5:                5              160-191

DU6:                6              192-223

DU7:                7              224-255

As you can see, each handler allows 256 megabytes of disc space to be used.

This group of 256 megabytes may be on one physical drive or spread across a

number of drives.

For  discs  larger  than  256  Megabytes,  we  add  another  handler  (D1).

Therefore D1 refers to:-

Drive               Partition      Addressing Range (Megabytes)

(Octal)

D10:                10             256-287

D11:                11             288-319

D12:                12             320-351

D13:                13             352-383

D14:                14             384-415

D15:                15             416-447

D16:                16             448-479

D17:                17             480-511

In the examples below  we  have  four  disc  handlers  to  cater  for  1024

Megabytes on two physical drives.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 1-3

Users' Guide

1.3  Changing Handler Settings.

We recommend that you change these setting by  running  the  handler  as  a

programme, and that you ONLY do this under RT-11.  From version 3, the SETs

have been removed from the DUCM handler if build for TSX-plus, for  safety.

You may use SETs on the secondary handlers D1, D2, and D3.TSX.

.R D1.TSX   (also D2.TSX, D3.TSX etc)

The following options are available:-

Option              Description

/H                  Type help text

/L                  List the current values in internal tables

(in the following switches, "unit" is one of the  handlers  logical  units.

It must be a single digit in the range 0 - 7.)

unit/U:n            Specify the PHYSICAL unit number to which this

logical unit refers.

unit/P:nn           Specify the PARTITION to whibh this logical

unit refers. Append a "." to make the partition

number decimal rather than octal.

/R                  If "/R" is included in any command line

unit is set to be READ ONLY.

More options exist, full details  may  be  found  in  chapter  3,  the  "WA

Handler" documentation.

1.3.1  Examples.

.R D1.TSX              Run the driver under RT-11

*4/U:3/P:1             Associate D14: with physical unit three

partition 1

*0/P:2/R               Associate D10: with physical unit zero

partition 2 READ ONLY

*1/P:10.               Associate D11: with physical unit zero

partion 10. (12 octal)

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 1-4

Users' Guide

Here is an example of how to set up enough logical discs to be able to  use

the  disc  capacity  of  a controller with two Fujitsu Eagles - 2351's (400

Megabytes formatted each)

The primary handler is set up so that the units D00:  to  D07:   correspond

to partitions 0 to 7.  Do not alter these settings.

Set up the second handler.

.R D1.TSX              Run the second handler

*0/U:0/P:8.            unit zero part 8

*1/U:0/P:9.            unit one part 9

*2/U:0/P:10.           unit two part ten

*3/U:0/P:11.           unit three part eleven

*4/U:0/P:12.           unit four part eleven (this last partition

is not the full 32 megabytes - 64K blocks)

*^C

This covers the first Eagle - D00 through D07 and D10 through D14 now refer

to the 13 (decimal) 32 Megabyte partitions on the disc.

Now for the second Eagle.  Set up the third handler:-

.R D2.TSX              Run the third handler

*0/U:1/P:0             Start with unit zero part zero....

*1/U:1/P:1             now unit 1 part 1

*2/U:1/P:2             two ibid

*3/U:1/P:3             three and so forth

*4/U:1/P:4             four

*5/U:1/P:5             five

*6/U:1/P:6             six

*7/U:1/P:7             seven

*^C

.R D3.TSX              Run the fourth handler

*0/U:1/P:8.            unit zero part eight

*1/U:1/P:9.            unit one part nine

*2/U:1/P:10.           unit two part ten

*3/U:1/P:11.           unit three part eleven

*4/U:1/P:12.           unit four part twelve (this last partition

is not the full 32 megabytes/64K blocks)

*^C

And this is it.  We  now  have  26  (decimal)  logical  units.   God  Bless

TSX-Plus and all who sail upon her in large discs!

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 1-5

Users' Guide

1.4  TSX-Plus System Generation

Determine the number of 32 megabyte units you need for your  configuration.

As  you  can  see from the example above, you need one handler per 8 units.

Create enough handlers by copying D1.TSX.  Remember DUCM.TSX is the handler

that  talks  to the controller.  You only have one of these per controller,

but you can have as many "Large Disc Handlers" as you want.

Edit TSGEN.MAC to include as many large disc handlers  you  require.   They

are specified as:-

DEVDEF        <DU>,MAPH,DMA,HANBUF

DEVDEF        <D1>,MAPH,DMA,HANBUF

DEVDEF        <D2>,MAPH,DMA,HANBUF

DEVDEF        <D3>,MAPH,DMA,HANBUF

You must copy DUCM.TSX across to your system so that it becomes DU.TSX:

.copy ducm.tsx sy:du.tsx

1.5  Creating More Handlers.

The four handlers discussed above were copied as follows:

Handler        Created From        Comments

DU.TSX         DUCM.TSX            The actual MSCP handler

D1.TSX         WA.TSX              Large disc Handler - first copy

D2.TSX         WA.TSX              Large disc Handler - second copy

D3.TSX         WA.TSX              Large disc Handler - third copy

You may create more.  There is a command file LGSET.COM.   It  creates  the

handlers  as above, and then sets them up for two 512 Megabyte drives.  You

may wish to edit this file for your own use.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 1-6

Users' Guide

1.6  Modifying TSX

It is possible to run the DUCM DU handler "mapped".  This saves low  memory

space,  allowing  your system to handle more jobs.  To do this you must use

the TSMUNG programme on your TSX.SAV or it will not allow the DU handler to

be mapped.  This programme is provided on this distribution.

.Run TSMUNG SY:TSX.SAV

Successful TSX Modification

.

If you do this, you must NOT try to run using the DEC or S & H DU  handlers

(the system will fail to start TSX-plus).

CHAPTER 2

DUCM Manual Itself

2.1  Introduction

DUCM is designed to  speed  multi-user  DU  use,  notably  under  TSX-plus.

TSGEN.MAC  should  be  set up to allow DU to be mapped into high memory, as

this conserves almost  1k  of  low  memory.   [Problem:   with  V6.01  this

requires  munging of TSX.SAV due to a new restriction - see TSMUNG later in

manual.]

This document describes DUCM V4, 1-Jan-89.  The  use  of  versions  earlier

than  V1g  on TSX+ V6 or later can lead to system crashes on heavily loaded

systems, and versions earlier than V4 on TSX+ V6.31 can lead to lost  queue

elements if the spooler is active.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-2

DUCM Manual Itself

2.2  Update History

Changes from V1 to V2 of DUCM:

(1) SPFUNs 377 and 376 (read & write respectively,  returning  status)  are

now supported;

(2) SPFUN 372 (DEC's partition table I/O) has been included -  see  details

especially re RT V5.4 & later in manual.

(3) SPFUNs 361 & 362 have been removed, being replaced by 372.

(4) SETs are no longer allowed under TSX.

V2 to V2a [23-Oc-86]

Bug  fix  which  caused  odd  corruptions  or  crashes  on  rare  occasions

especially under heavy swapping.

V2b [8-Dec-86]

Allow "hidden" block (block 65535) between partitions to be accessed.   Fix

bug  where  couldn't  run DUCM as a programme under RT11 (don't give RT11 a

.gval with a byte address!).  Add /D option to allow setting up  a  handler

without using monitor SETs.

V2c [20-Feb-87]

Addition of time-out support;  correction of bugs in  serious  error  code.

[This  was  added  to  track down a controller bug, but appears to be worth

general inclusion.  The ability to cause a pseudo disc timeout  by  running

DUCM  and using the /Z switch, which is capable of crashing systems, may be

thought by some  to  be  inadvisable.   If  so,  assemble  without  timeout

support!]

V2d [19-Apr-87]

Repair a bug in timeout code where it was checking the  wrong  location  to

see  whether  there was a request being processed.  Make the pseudo-EMT for

timing out work only from job 1 (usually the CTY) on TSX-plus.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-3

DUCM Manual Itself

V2e [27-Aug-87]

Bug fix in testing P.STS for errors:  caused funnies getting device size on

RQDX?   board.   Can  anyone  tell  me why this board returns 20000 in this

word?

V3 [6-Feb-88]

Addition of MSCP Bypass.  Completely remove SETs if built for  TSX.   NOTE:

I  DO NOT TRUST THE CODE USED BY DEC FOR GETTING THE BUFFER ADDRESS FOR THE

MSCP BYPASS BUFFER SUPPLIED BY THE USER IN THE WCNT  ARGUMENT.   THIS  CODE

ASSUMES  THAT THE USER INSTRUCTION ADDRESS REGISTERS ARE SET UP FOR THE JOB

IN QUESTION, ($RELOC - NOTE THAT TSX IS MORE SOPHISTICATED)  AND  THIS  MAY

NOT BE TRUE IN A SYSTEM RUNNING MANY JOBS ALL ACCESSING THE DISC!

The STATU$ word has been added to the DUCM  handler,  as  per  the  DEC  DU

handler,  at  <dustrt+16>.  See DEC documentation for the use of this.  The

error table report now includes this word.

There is an odd restriction in this  version  of  DUCM.   The  SRQD11-A  of

Webster  Computer  Corporation  has a bug inasmuch as if any of a series of

blocks is read, that block number being in  a  range  a  little  above  the

actual  available  length  of the disc, an error is caused which causes all

reads to return in error until a block from the first track is  read.   The

code  to  get around this follows RWMSG:  in the handler, and is commented.

Unless you get rather a huge number of  bad  blocks,  there  should  be  no

problem with the slight delay this code introduces.

V3a [17-Jun-88]

A time-out error during initialization failed to realize that  the  element

to return was not yet in the active queue, and consequently hung the system

(thanks Chris Kleinschmidt!)

V3b [5-Dec-88]

Make initial call to RETIME at fork level, so that it doesn't upset getting

queue  elements  away  in the handler initialization.  This caused problems

with TSX V6.31.

Make /D change default for DU6 and DU7 for ALL jobs, not just job 0 (thanks

Ralph Zwier!)

V4 [1-Jan-89]

Rewrite of much of earlier part of handler to get round nasty TSX bug where

multiple elements could be passed to a handler.  NOT a pleasant task!

V4a [23-Apr-89]

Allow changing of all unit/partitions if job 1, so can set up  multiple  DU

handlers  from  TSX  without messing around with RT start-up files (special

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-4

DUCM Manual Itself

for SEQEB!).

Make Unit Size SPFUN (SP.USZ) return the size of the unit relative  to  the

partition  specified  in  the  request  - ie the number of blocks available

using physical I/O specifying this unit.

V4b [23-Jul-89]

Correct bug in disastrous error code, where after 10 disastrous  controller

errors the handler would write crash information past the end of one of its

internal tables.

Decrease time-out to 5 seconds (from 10 seconds).

Add code to more accurately mimic TSX-plus mapped handler interrupts in the

internal  pseudo-interrupt  code  (stacks  PAR5,  340, and a special return

address).

V4c [22-Aug-89]

Correct bug in /D code, where /D/V reset the IP register  address  but  not

the SA register address.

Make time-out 10 seconds again,  as  occasionally  get  this  time-out  for

unknown reasons, and 10 seconds reduces the number of these.

Add /W switch to allow setting of all job units 6 /or 7 under TSX from  job                                                  _

1.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-5

DUCM Manual Itself

2.3  SET Commands

This is a single-port handler only, but can cope  with  multiple  units  on

that port.  The DEC commands

.SET DUx UNIT=n

.SET DUx PART=m

.SET DU VECTOR=vvv        and

.SET DU CSR=ccc

are valid if used under RT-11.  SETs are NOT permitted  under  TSX-plus  on

the  DUCM  handler  itself,  as TSX-plus conveniently rewrites that handler

over itself.  This is generally helpful, but with DUCM it  causes  loss  of

queue element information and kills the system.

The SET DU PORT command merely returns an error message.

An additional pair of commands allow the restriction  of  the  MSCP  Bypass

facility  where it is felt that it could be used in ways detrimental to the

system:

.SET DU NOBYPASS          and conversely

.SET DU BYPASS

Unit and partition values may be altered under  programme  control.   These

values  will  not  be  retained  across  a  reboot  or other re-load of the

handler.  Under RT-11 any  logical  unit  may  have  its  logical  unit  or

partition  altered,  but  under TSX-plus this is restricted to DU6 and DU7.

Separate parameters are kept under TSX-plus for units 6 & 7 for  each  job,

the  units DU0 through DU5 being common to all jobs.  [They can only be set

up by a SET command issued under RT-11].

If the handler is run as a job, it allows the user to examine  the  current

partition table, and to alter the partition table as above.  It also allows

a handler to have its units and partitions set up for a  different  system,

without having to use monitor SETs.

As from Version 3, it is not possible to perform any SETs for DUCM.TSX even

if it renamed to something else (eg XX.SYS) under RT-11.

The setting of DUCM parameters has been much simplified from version 2b on.

This  is  done  by  running the DUCM handler as a programme, and using a /D

switch - see section 2.7.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-6

DUCM Manual Itself

2.4  SPFUNs

2.4.1  Error Code I/O (SP.RIO 377, SP.WIO 376)

Read & write returning error codes (SPFUNs 377  &  376  respectively)  have

been  implemented.   These  use  the  buffer  address and word count as per

normal I/O, but return a status word in the first word of the buffer (which

therefore  must be one word longer than the specified word count).  This is

used for bad block replacement purposes  in  RT-11  from  version  5.4  on.

Without it, DUP from the 5.4 release kit will not work.

[Note that a negative word count causes  either  of  these  two  SPFUNs  to

function as the other.  This appears to be a feature...  !]

The error returns are:

100000 (& carry clear):  no error - normal return

100002 (& carry clear):  error which recovered upon retry

100200 (& carry set):  BBR bit has been  set  for  this  block  (no

retries attempted)

177777 (& carry set):  I/O error occurred on this request  and  did

not recover on retry

2.4.2  Partition Table I/O (SP.DAT 372)

SP.DAT  (372)  which  reads  or  writes  the  partition  table   has   been

implemented, with variations.

(1) Under TSX-plus, only units 6 & 7 may have  their  units  or  partitions

changed  by use of this request, unless this request comes from job

1 (allowing special setting  up  for  multiple  computer  systems).

[Normally  the  read/write  request  form  of this SPFUN has a word

count argument of 1 or -1 respectively.   If  a  value  (octal)  of

100000  is  used  from  job 1 under TSX, the values for units 6 & 7

will be reset for ALL jobs, rather than just job 1.]

(2) As this is a single-port handler, any value entered in  the  PORT  byte

will be taken as a high-order value for PARTITION.  You will need a

BIG disc to be able to use this!

(3) RT-11 changed the format of the  partition  table.   From  version  5.4

onward there are two additional words at the start of the partition

table:  <.rad50 /DU/> and  <.word 8.>.   The  DUCM  handler  checks

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-7

DUCM Manual Itself

which  version  of RT it is being run under (or which version of RT

that TSX appears to be)  and  returns  (or  uses)  the  appropriate

partition table format.

2.4.3  Partition Size (SP.PSZ 373)

The partition size SPFUN is the same as that for the DEC DU handler.

2.4.4  Unit Size (SP.USZ 300)

This SPFUN returns unit size (rather than partition size) in two words in a

manner  analogous  to SP.PSZ (373).  The first word receives the high-order

value of the size.  From V4a on, the length returned is from the  start  of

the  partition  to the end of the unit.  Thus if the logical unit specified

is partition zero, the full size of the corresponding physical unit will be

returned.   If  the  logical  unit  corresponds  to  partition  1, the size

returned will be the physical unit size less 65536, or one partition.   The

size returned is the number of blocks which may validly be accessed on that

logical unit using the physical I/O SPFUN (SP.PIO).

2.4.5  Status Table (SP.STS 363)

This has been included as a primitive form of error logging.  This  returns

a table in the form:

1 word (du.ver) Version Number (currently 206 octal)

1 word          No. of init's performed successfully

1 word          No. of retries performed in init'ing

1 word          No. of bloody disasters (SA negative)

1 word          No. of AVAIL errors

1 word          No. of IO retries

1 word          No. of IO errors

1 word          No. of double-linked requests

1 word          STATU$ word: status of last disc operation

1 word          No. of Time-outs

2-word pairs (maximum of 30.) IP/SA pairs for each time-out.

This was included mainly for debugging, but has not been removed.  It has

been extended to include the time-out support in V2c and the status word

in V3, and the number of double-linked requests in V4.

2.4.6  Fake Time-Out (SP.FAK 200)

For testing the time-out system a maintenance SPFUN, SP.FAK (200) has  been

included.   This  causes  a fake disc time-out.  When it is used, the block

number must be set to 100000 or the SPFUN is ignored.   It  is  ignored  if

DUCM is not assembled with time-out support.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-8

DUCM Manual Itself

Under TSX-plus, for safety, this request must be made from job  1  (usually

the CTY primary line).

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 2-9

DUCM Manual Itself

2.4.7  Physical I/O (SP.PIO 301)

[For an alternative under TSX-plus,  see  associated  WA  handler  and  its

documentation - chapter 3 of this manual.]

Due to the restriction in RT-11 that  normal  I/O  may  only  access  65565

blocks   on   any  individual  unit,  and  DU  devices  can  extend  rather

dramatically beyond this limit, a means has been provided to  allow  simple

access to a larger block number:

SP.PIO is for physical rather than logical I/O.  It allows a  24-bit  block

number,  but  only allows reading or writing whole blocks.  This is done by

using the Word Count word of the queue element as follows:

High-order byte:  number of blocks to transfer (negative if  write,

positive if read, zero seek [useless]);

Low-order byte:  high order eight bits  of  block  address.   [This

byte does NOT change sign in a write!]

Note that physical I/O will use the UNIT parameter of whatever  logical  DU

was  selected,  and  adds the PARTITION parameter of that specified logical

unit to the high-order word of the block number.  [This  was  changed  with

DUCM  V3;   previous versions returned an error if the partition number was

not zero.] The number of blocks available on a particular unit  for  access

by  physical  I/O  may  be  obtained by use of the unit size SPFUN (SP.USZ)

[from V4a on].

Thus:

.SET DU3 UNIT=2,PART=0

will allow physical I/O on DU3, starting at block 0, and

.SET DU3 UNIT=0,PART=2

will allow physical I/O on DU3, starting at block 131072.

As an example of use of physical I/O, on the next page is  a  copy  of  the

testing programme used with this handler:

[Note:  .SPFUN area,channel,function,buffer,wordcount,block]

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-10

DUCM Manual Itself

.Title	PIOTST - for testing physical I/O capability of DU:

.Nlist	bex,toc,sym

.Enabl	lc,gbl

; Must be assembled with CMAC, and linked with CVLLIB

.mcall	.csigen,.wait,.spfun

sp.pio=301			;Physical I/O SPFUN

area:	.blkw	7		;General EMT area

limit:	.limit			;So we keep track of memory

hlimit:	0			;ibid (unused as yet)

buffer:	.blkw	256.		;I/O buffer

defext:	.rad50	/            /	;Default extensions for CSIGEN

Start::

.csigen	limit+2,#defext	;Find which device we want to read.

mov	r0,hlimit	;New memory limit after handlers loaded.

.wait	#3		;Did he specify a device?

bcc	s1		;Yes - use it.

type	</PIOTST V1 5-Oct-85/<0>>;No - just tell him which version.

br	start

s1:

type	</Block Number: /<200>>

call	rn2t		;Read double-precis number into R2/R3

;(octal, decimal. or 0hex)

tstb	tlinb		;Check for null line in TT input buffer

beq	start		;If so, try for a different device.

mov	#<1 * 400>,r5	;1 block to read - <.byte 0,1>

bisb	r2,r5		;Add in high-order block number

;(R3 has low-order block number)

.spfun	#area,#3,#sp.pio,#buffer,r5,r3

bcc	s2

type	</SPFUN Error reading that block/<0>>

br	s1

s2:

type	</First word: /<200>>

mov	buffer,r0	;Show him the first word

call	w8t		;(in octal, to the terminal)

call	tcrlf		;Append a <cr><lf>

type	</Alter to: /<200>> ;(Probably folly to allow him to alter?)

call	rnt		;Read single-precision number

tstb	tlinb		;If null line, don't alter anything.

bne	s3

type	</Not altered./<0>>

br	s1

s3:

mov	r0,buffer	;If he means it, alter it

mov	#<-1 * 400>,r5	;Write 1 block (negative = write)

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-11

DUCM Manual Itself

bisb	r2,r5		;High-order block number again.

;(R3 still has low order)

.spfun	#area,#3,#sp.pio,#buffer,r5,r3

bcc	s1

type	</SPFUN Error writing that block/<0>>

br	s1

.end	start

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-12

DUCM Manual Itself

2.4.8  MSCP Bypass Spfun

Before DUCM V3, it was considered that to allow MSCP Bypass operation on  a

multi-user system such as TSX-plus was potentially dangerous.  However, DEC

are using this in the JREAD/JWRITE and IGTDUS subroutines, and to  fail  to

allow it may limit the use of programmes designed for the use of this.

Both the old (371) and the new  (360)  values  for  the  BYPASS  SPFUN  are

allowed.

There is one restriction:  the user's MSCP buffers must be 60 (octal) bytes

long.   This  is the same size as DEC use, both in their handler and in the

abovementioned subroutines, so there should be no problems.

For higher security, the bypass option may be disabled.  Under  RT-11  this

may be done by the command

.SET DU NOBYPASS

(to re-enable, use SET DU BYPASS), or through the  use  of  the  /D  switch

under TSX-plus, running DUCM.TSX as a programme [section 2.7].

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-13

DUCM Manual Itself

2.5  TSMUNG - for modifying TSX plus

With the advent of TSX-plus version 6, no matter what is specified  in  the

TSGEN file (eg DEVDEF <DU>,MAPH,HANBUF,DMA) TSX refuses to allow the DU (or

the DY) handler to be mapped into high memory.  This  is  fine  to  protect

people trying to do this with DEC or S & H handlers, but is a nuisance when

we have a legitimate handler capable of mapped performance.

A programme TSMUNG is provided.  This modifies the default  values  set  up

for  the  DU  (and optionally the DY) device.  It is run, using the name of

the TSX.SAV file to be modified appropriately.

To allow DU to be mapped, append /U to the filename.  To  allow  DY  to  be

mapped, append /Y to the filename.  [No switches implies /U]

.Run TSMUNG

*sy:tsx.sav

Successful TSX modification

.

or

.Run TSMUNG sy:tsx.sav

Successful TSX modification

.

If it should be desired to run the DY handler mapped in addition to the  DU

handler  (WARNING  -  again, make sure your handler is capable of being run

mapped and is not the standard DEC or S & H handler), append a /Y and a  /U

to the filename.  For example:

.Run TSMUNG du:tsx.sav/y/u

Successful TSX modification

.

(Merely specifying DU:TSX.SAV/Y will allow the DY handler to be mapped  but

NOT the DU handler.)

If TSMUNG cannot find the relevant string of device names in TSX,  it  will

tell  you so.  If this is the case, you are probably trying to operate upon

a version of TSX different from that for which TSMUNG was written (versions

6.01 through 6.2).

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-14

DUCM Manual Itself

2.6  Assembly Options

NUMBER OF JOBS:

As TSX-plus now allows more than 32 jobs (and has a whole word  set  up  in

the  queue  element for the job number!), the number of jobs allowed for in

the DUCM partition table is now an assembly parameter.

The parameter is "ts$job", with a default value of 50 (decimal).

It is defined such that a prefix file with a different value will  override

the definition within DUCM.

UNIBUS SYSTEMS:

Normally DUCM is set up to allow use of the WA handler to  access  multiple

units  and  partitions  of  large discs.  This can only be used under Q-bus

systems (as under unibus systems the word used for  passing  partition  and

unit  information  across  to  DUCM is used for the unibus mapping register

value).

If you are using a unibus system, you will need to  assemble  DUCM  with  a

prefix file containing

$XLINK=0

for safety.  This suppresses the WA code linkage.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-15

DUCM Manual Itself

2.7  Running DUCM as a Programme

The handler may be run as a programme (R DU.TSX, or R DU.SYS).  The  format

of  this  has  been  changed  from  earlier  versions  to allow multiple DU

handlers on large systems.  When run, it expects a normal CSI-style command

string,  with  a device name (of a DU-type device - using the DUCM handler)

and switches as necessary.

From V2b on, it also allows a user to set up the partition table,  CSR  and

vector  within  a  handler  file  itself.   This  allows a user to set up a

handler for another system without having to use monitor SET commands, with

all their complications.

Switches available are shown by use of the /H switch:

.Run DUCM.SYS

*/H

DUCM Internally-Queueing Multi-Unit DU Handler V4c 22-Aug-89

/E      Type the error table

/L      Type the current units and partitions

/S      Type the size of the device by partitions

/C:n    Change the partition/unit for logical unit n (6 or 7 only if

TSX) used with one or both of the following two switches:

/P:n    Partition number

/U:n    Physical unit number

/H      Type this message

/D      Modify partitions & units in disc handler, not memory

(may only be used with /L, /V and /X -

if used with /V allows alteration of CSR & vector)

/X      Exit after performing relevant task.

/W      On TSX, allow /C to change units 6  7 for ALL users.                                           _

/Z      Cause a fake disc time-out.

An unknown switch types the following message:

?DU-E-Illegal switch /x.  For help type /H

2.7.1  Examining & Modifying the Handler in Memory

To see the size of a particular DU device:

.R DU.SYS

*du:/s

Total unit physical size (starting at specified partition) = 622676

9 partitions each of 65535 blocks

1 partition of 32852 blocks

*^C

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-16

DUCM Manual Itself

To type the current units and partitions:

.R DU.SYS

*DU:/L

Details of Handler in Memory:

Logical         Physical        Partition

Unit             Unit

0               0                0

1               0                1

2               0                2

3               0                3

4               0                4

5               0                5

6               0                6

7               0                7

*^C

To examine the error table:

.R DU.SYS

*du:/e

Version Number: 206

Number of successful INIT's: 1

Number of retries on INIT's: 0

Number of INIT timeouts:     0

Number of bloody disasters:  0

Number of AVAILABLE errors:  2

Number of I/O retries:       2

Number of I/O errors:        0

double-linked requests:    0

Value of STATU$ word:        0

Number of Time-out errors:   0

*^C

It is possible to change the physical unit or partition value  for  logical

unit  6  or unit 7 under TSX, or those of any unit under RT-11 (at your own

risk if you change those of the system device!).  This is done using the /C

switch,  in conjunction with /P (for partition number) and /U (for physical

unit number).  [From V4a on, a user on job 1 under TSX may change any unit.

Altering  units  0 through 5 affect everyone using the system, and altering

units 6 and 7 usually only affect his job.  From V4c on, if /W is added  to

the  command  line  however, changing unit 6 or 7 is also global across the

system.  BE WARNED:  this is designed for start-up  only;   if  used  while

users  are  actually  using  unit  6 or 7 it will change their value on the

fly!).  For example, to change unit 6 to partition 3 on unit 2:

.R DU.TSX

*DU:/C:6/P:3/U:2

If the partition requested does not  exist,  the  user  will  be  informed.

Under TSX-plus, only the DU unit (6 or 7) for the particular job (except at

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-17

DUCM Manual Itself

per the last paragraph) will be altered.  [Under TSX-plus,  the  associated

WA  handler  is  recommended  for safer and more consistent access in large

systems.]

2.7.2  Examining & Modifying the Handler on Disc (DU)

From Version 2b of DUCM on, a provision has  been  made  for  altering  the

physical unit & partition table in a disc copy of a handler, without having

to use monitor SETs.  This provides considerable convenience in setting  up

handlers  for systems other than the one being worked upon.  The vector and

the CSR may also be altered in this way.

To examine the contents of the physical  unit  &  partition  table  in  the

handler being run as a programme, use the /D switch with the /L switch.

To alter the contents of the physical unit & partition table in the handler

being run, use the /D switch alone.

To alter the CSR and vector, as well as the unit &  partition  tables,  use

both /D and /V.

Examples:

.RUN DUCM.SYS

*/d/l

DUCM Internally-Queueing Multi-Unit DU Handler V4c 22-Aug-89

Details of Handler on Disc:

Built for TSX-plus

Includes device time-out support

Vector: 154    CSR: 172150      Bypass allowed.

Logical         Physical        Partition

Unit             Unit

0               0                0

1               0                1

2               0                2

3               0                3

4               0                4

5               0                5

6               0                6

7               0                7

To alter this table, use the /D switch alone.  Instructions will be typed:

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-18

DUCM Manual Itself

.RUN DUCM.SYS

*/d              [Change partitions & bypass]

To alter the partition/unit table in the handler being run as a programme,

enter three decimal numbers separated by spaces: respectively the logical

unit number (0 through 7), then the corresponding physical unit number

and partition number.  When finished, type a blank line.

> 0 0 0

> 1 1 0

> 2 2 0

> 3 3 0

> 4 4 0

> 5 5 0

> 6 6 0

> 7 7 0

>

[For a TSX handler, from version 3b on, a change to unit 6 or 7 sets the

default for ALL jobs, as DU6 and DU7 have a separate entry for each job

on the system.]

For the Bypass option, answer ON or OFF to allow or disallow the use

of this facility on your system.  (A blank line leaves it as it is.)

Bypass is currently ON

ON or OFF? OFF

*/d/l            [List details only]

DUCM Internally-Queueing Multi-Unit DU Handler V4c 22-Aug-89

Details of Handler on Disc:

Built for TSX-plus

Includes device time-out support

Vector: 154    CSR: 172154      Bypass disallowed.

Logical         Physical        Partition

Unit             Unit

0               0                0

1               1                0

2               2                0

3               3                0

4               4                0

5               5                0

6               6                0

7               7                0

*/x

.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-19

DUCM Manual Itself

Run ducm.tsx

*/d/v            [Change vector, CSR, partitions & bypass]

The present Vector & CSR addresses will be typed.  Enter the new values

or merely a carriage-return if you do not wish to change that value.

Vector [154]: 160

Control & Status Register [172150]: 172154

To alter the partition/unit table in the handler being run as a programme,

enter three decimal numbers separated by spaces: respectively the logical

unit number (0 through 7), then the corresponding physical unit number

and partition number.  When finished, type a blank line.

>                [Did not want to change partitions]

For the Bypass option, answer ON or OFF to allow or disallow the use

of this facility on your system.  (A blank line leaves it as it is.)

Bypass is currently ON

ON or OFF? OFF

*/x

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 2-20

DUCM Manual Itself

2.8  Error Messages

?DU-E-Required Handler must be loaded

?DU-E-DU class device not specified

?DU-E-Handler version differs from that of programme being run

?DU-E-/C requires a value (6 or 7 only if TSX)

?DU-E-/C may only change units 6 or 7 under TSX-plus

?DU-E-/C can only change units 6 or 7

?DU-E-/P must have a value specified

?DU-E-/U must have a value specified

?DU-E-Only one of any of /C, /P, or /U on any 1 line

?DU-E-/P requires /C to be of any use

?DU-E-/U requires /C to be of any use

?DU-E-/C requires /P &/or /U to effect a change

?DU-E-Non-existent unit/partition specified: unit u, partition p

?DU-E-Cannot read DU status table

?DU-E-Cannot read DU partition table

?DU-E-Cannot save status of input file

?DU-E-Cannot restore status of input file

?DU-E-Cannot Read Status Table

?DU-E-Cannot ascertain status of DU:

?DU-E-Device DU: not available

?DU-E-Cannot save status of DU:

?DU-E-Cannot restore status of DU:

?DU-E-Cannot Read Status Table

?DU-E-Partition table read failed

?DU-E-Failed to write partition table

?DU-E-Cannot determine size of DU

?DU-E-/D must not have a device specified,

as it works on the actual handler being

run as a programme.

?DU-E-/E, /C, /P, /U, & /S may not be used with /D

?DU-E-Insufficient memory for Handler Block I/O

?DU-E-/V may only be used with /D

?DU-E-I/O Error on Handler Block

?DU-E-Three and only three decimal numbers expected

?DU-E-Logical Unit Numbers must be from 0 through 7

?DU-E-CSR must be even and greater than 160000

?DU-E-Vector must be from 0-374, and divisible by 4

CHAPTER 3

WA - a Munging Handler for LARGE Discs

This section descibes WA version 3, of 29-Dec-88

The WA handler provides a fairly rigid way of accessing partitions on a  DU

disc  under  the  TSX-plus  operating  system.  Each copy of the WA handler

installed allows access to a further eight partitions.

Each of the WA units can be set up (either with SET commands, or by running

the  handler as a programme) to access any desired unit or partition within

the disc.  Additionally, it is possible to use WA to access individual disc

image files (a la LD).

Used sensibly, the WA handler can be a boon to a large disc  system.   Used

without  due caution, it suffers from the problem of putting a lot of power

into careless hands.

WA must be used with the internally-queueing DUCM DU:  handler by the  same

author.   Both  the DU and the WA handler (and any copies of the WA handler

used to access multiples of 8 units) may be mapped handlers under TSX-plus,

though  V2  of  WA  operates with unmapped handlers also.  Use with the DEC

handler will lead to disc errors or system crashes.  Version 2a allows  the

WA  handler to operate unmapped, possibly allowing extra speed for use with

streaming magnetic tape at the expense of low memory.

It can only be used  on  Q-bus  systems,  NOT  unibus  systems.   With  the

handlers  mapped,  the  processor  must  be  capable  of executing the MARK

instruction.  It cannot be used under RT-11.

The WA handler is distributed  as  three  handlers:   D1.TSX,  D2.TSX,  and

D3.TSX (see the first chapter of this manual).

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-2

WA - a Munging Handler for LARGE Discs

Update History

V2 - initial release.

V2a [5-Dec-86]

Allow unmapped WA for speedies mag tape handling

V2b [8-Feb-87]

Make I/O past end of file return EOF a la LDs, rather than errors a la disc

handlers.

V 3 [29-Dec-88]

Allow WA to cope with multiple requests  at  handler  initiation  time,  to

manage nasty bug in TSX V6.31.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-3

WA - a Munging Handler for LARGE Discs

3.1  How it works

An I/O request passed to the WA handler is  modified  as  appropriate  then

passed on to the DU handler for actual execution (similar to the LD handler

under RT-11).  Various details of the I/O request  are  modified,  allowing

access  to physical units and partitions apart from those accessible by SET

commands on the DU handler.

[The TSX I/O queue element is longer than the RT-11 equivalent.  There is a

word  in  it  for  the  address of a unibus mapping register table, q$umrx.

This word is used for  passing  across  the  physical  unit  and  partition

number,  one  byte  apiece.  This limits the unit and partition number to a

maximum of 255 (decimal), but with present  disc  hardware  this  does  not

appear to be a severe restriction!  This is why this handler cannot be used

on a unibus system, because  this  word  is  then  used  for  its  intended

purposes.]

Four pieces of information and a flag are stored within the WA handler  for

each unit.  These are:

PHYSICAL UNIT  NUMBER  -  the  physical  disc  unit  the  desired

partition is residing upon.  This may well be different

from the logical DU units (DU0 through DU7),  as  these

may be set to different units and partitions.

PARTITION NUMBER - as RT-11/TSX can only access individual device

units with a capacity of up to 65535 blocks (one 16-bit

word), devices having more than this are broken up into

"partitions",  each  65535  blocks  long.   (There is a

spare block between partitions,  actually  making  each

partition  commence  on  a 16-bit boundary on the disc.

This  is  necessary  for  the  SIZE  spfun  to  operate

properly.)  The lowest partition on a disc is partition

zero,  starting  at  physical  block   0.    The   next

partition,  partition  1, commences on block 65536, and

partition two on block 131072, et cetera.

OFFSET - This defines a starting offset within the partition.  It

is  for  use  with  the "pseudo-LD" facility of WA, and

will not normally be used  by  people  who  are  merely

using WA to access large discs.  It allows a disc-image

file to be used as an apparent physical WA  unit.   For

example,  suppose  you have the file TSX.DSK sitting on

your system disc commencing at block 12345.   It  would

be  possible  to access that file, if the offset within

the relevant partition were set to 12345.

SIZE - Again, used with the "pseudo-LD" facility.  If a  file  is

to  be  used as an WA unit, its size should be put into

this field.

WRITE FLAG - Each unit may be set to be READ ONLY if desired.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-4

WA - a Munging Handler for LARGE Discs

3.2  SET Commands

The SET commands available for WA are:

SET WA HELP - types a list of available SET WA commands.

SET WA RELOAD - dummy SET command to force  TSX  to  reload  the  first

memory  block of the handler, after modifying values by

running WA.TSX as a programme.

SET WA DUVEC=oct - sets the vector assumed for the DU  device  (default

154)

SET WAn UNIT=u - sets the physical unit associated  with  logical  unit

WAn to the value u (decimal).

SET WAn PART=p - sets the partition associated with logical unit WAn to

the value p (decimal).

SET WAn OFFSET=o - sets the offset value associated with  logical  unit

WAn to the value o (decimal).

SET WAn SIZE=s - sets the size value associated with logical  unit  WAn

to  the  value  s (decimal).  If this is not specified,

device size spfun requests will be passed across to the

DU  handler.   If  size  is  specified, it will be used

directly in response to these requests.

SET WA (NO)WRITE - (disallow)/allow writing on this unit.

SET WA TSX - allow SET WA commands from TSX-plus, and allow  WA  to  be

run as a user programme under TSX-plus.

SET WA NO TSX - disallow the use of  WA  SET  commands  unless  running

under  RT-11 (ibid for running WA as a user programme).

Once this option has  been  set,  it  is  necessary  to

return  to  RT-11 to use SET WA TSX to unset it.  It is

recommended that all values necessary for WA be set  up

in  an  RT-11  command  file,  with  this  as  the last

command.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-5

WA - a Munging Handler for LARGE Discs

3.3  Running WA as a Programme

Due to two factors:  (1) the danger of allowing access to the fields of  WA

while  running TSX, and (2) the inconvenience of having to rename WA.TSX to

WA.SYS before being able to perform SETs  under  RT-11,  another  means  of

setting up the WA tables has been provided.

.R WA.TSX

Run the WA handler as a user programme.  It uses normal CSI-style commands,

as detailed here:

/H

Type a help text.

/L

List the current values in the internal table.

In the following switches, "unit" is one of the logical WA units.  It  must

be specified as a single digit (0, 1, ..., 7).

unit/U:n.

Sets the physical unit value associated with the relevant unit to the value

n.  It is necessary to append a "." to make the number decimal (octal being

assumed otherwise).

unit/P:n.

Sets the partition value associated with the relevant unit to the value  n.

(See comment under /U re decimal/octal values.)

unit/O:n.

Sets the offset value associated with the relevant unit to the value n.

unit/S:n.

Sets the size value associated with the relevant unit to the value n.

unit/F,filename

Accesses the "filename" file (the second file in the CSI string), and  uses

its  starting  block  number  as the offset value, and its size as the size

value to be associated with the desired unit.  It  is  still  necessary  to

specify  relevant  physical  units and partitions, as otherwise the handler

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-6

WA - a Munging Handler for LARGE Discs

will assume physical unit zero, partition zero - despite the fact that  the

filename  may be something like "DU2:MONITR.DSK".  The default extension is

".DSK".

/R

If /R is included in any command line, that unit  is  set  READ  ONLY.   An

attempt to write to that unit will return an error.

/T, /N

/T allows running WA as a  programme  under  TSX-plus.   /N  disallows  it.

(Analogous to the SET WA (NO) TSX command.) If a user tries to run WA under

TSX after use of the /N switch, it will merely list  the  contents  of  its

internal tables.

A unit specified with no switches merely has its table entries  cleared  to

zero.   All  the  specifications  for  a  particular logical unit should be

performed on the one line, as switches unspecified are assumed to have  the

value zero (/F implies /O & /S).

Examples:

Associate WA4 with physical unit 3, partition 1:

*4/U:3/P:1

Associate WA0 with physical unit zero, partition 2, read only:

*0/P:2/R

Associate WA7 with SY:TSX.DSK

*7/f,sy:tsx.dsk

This assumes that SY:  is DU:, unit zero, partition zero.  If instead  DU1,

through  SET  commands,  were  associated  with unit 4, partition 1, and we

wished to set WA6 to being UCL.DSK on that particular disc, we  would  need

to enter the command

*6/f,du1:ucl/u:4/p:1

At about this stage, you start to see why access to this facility should be

restricted to the non-malevolent!

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-7

WA - a Munging Handler for LARGE Discs

3.4  Restriction on /U & /P

If the unit and partition are both specified to be zero (/U:0/P:0),  rather

that  using  unit  zero, partition zero, the disc area accessed will be the

current unit and partition for DU0:.  (This is unlikely to be a problem.)

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-8

WA - a Munging Handler for LARGE Discs

3.5  Setting Up

(1) Determine the number of WA units required for your system.  If you need

more  than  eight,  you  may  need several copies of the WA handler on your

system under different names (eg WA, WB, WC, or D0, D1, D2, ....  etc).

(2) Edit TSGEN.MAC to include whichever WA units  you  will  require.   The

DEVDEF parameters for each WA device will be

DEVDEF <WA>,MAPH,DMA,HANBUF

(3) Build your TSX system in the normal way.

(4) Hack your TSX.SAV so that the value associated with DU:   is  501,  not

the  default  201.  [See TSMUNG, in chapter 2.] Note that this is no longer

essential with WA  V2,  but  is  preferable  if  you  wish  to  generate  a

reasonably large TSX system.

(5) Build your DU handler:  .macro tsxpre+ducm/obj, link/nobit ducm.

(6) Build your WA handler:  .macro wa, link/nobit wa.

(7) Get back to RT-11

(8) Copy ducm.sav to sy:du.tsx

(9) Copy wa.sav to sy:wa.tsx, (also sy:wb.tsx et cetera, pro re nata).

(10) Set up a command file to run the WA handlers and  enter  the  required

values (sample below)

(11) Run these command files to set the tables in the handlers.

(12) Boot your TSX-plus system.

DUCM - DU Multi-Unit Internally-Queueing Handler et alii           Page 3-9

WA - a Munging Handler for LARGE Discs

3.6  Safety Precautions & Recommended Use

- Make sure you are running on a Q-bus system.

- Make sure you are NOT trying to use the DEC or S & H DU handler for  your

TSX DU.TSX.

- Make sure that unless you have some major reason for allowing it,  access

to the WA handlers is prohibited through the SET WA NO TSX command.

- Set up your system so that if you  are  using  pseudo-LD  access,  either

there are no LD units mounted using the same files, or that there can be no

cache controversy (eg make that WA handler NOMOUNT in TSGEN,  or  run  with

cache=0, or be bloody careful!).

- If you alter WA, by running it as a programme under TSX, it is  necessary

then  to type SET WA RELOAD to tell the operating system that modifications

have been made so that it can re-load the handler with the new information.

(RELOAD  is  a dummy SET which actually does nothing, forcing TSX to reload

the first block of the handler.)

- To run WA as a programme under TSX you need to have privileges  to  write

to .TSX files on the system disc.

- If you have a particular need for extremely high speed access, make  your

WA handlers UNMAPPED.  This allows for less code in swapping the request to

the DU handler at the expense of low memory space (about 400 words).  [This

may be more useful with 11/23 systems.]

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 3-10

WA - a Munging Handler for LARGE Discs

3.7  Example of Command File

The SET Wx RELOAD commands are only necessary if these command files are to

be run under TSX-plus rather than RT-11.

Set up WA to access the second group of eight partitions on the disc,  with

the last of these write protected:

------------------------------

r wa.tsx

0/p:8.

1/p:9.

2/p:10.

3/p:11.

4/p:12.

5/p:13.

6/p:14.

7/p:15./r

/n

^C

set wa reload

------------------------------

Set up both an WA and WB (a copy of WA) to cover a few things:

------------------------------

r wa.tsx

0/p:8.

1/p:9.

2/p:10.

3/p:11.

4/p:12.

5/p:13.

6/p:14.

7/p:15.

/n

^C

set wa reload

r wb.tsx

0/p:16.

1/f,sy:tsx

2/f,sy:ucl

3/f,sy:monitr/r

4/f,sy:work1

5/f,du1:mordat/p:1/r

6/f,du5:again/u:1/p:2

7/p:23.

^C

set wb reload

------------------------------

DUCM - DU Multi-Unit Internally-Queueing Handler et alii          Page 3-11

WA - a Munging Handler for LARGE Discs

Index

Absolute I/O SPFUN . . . . . . . . . . . . . . . . . . . . . 2-9

Bad Block Replacement Error  . . . . . . . . . . . . . . . . 2-6

Bug - Hardware -> restriction  . . . . . . . . . . . . . . . 2-3

Bypass - MSCP  . . . . . . . . . . . . . . . . . . . . . . . 2-12

Changing Partitions & Units (DU) . . . . . . . . . . . . . . 2-15

Changing the DU Handler on Disc  . . . . . . . . . . . . . . 2-17

CSR Alteration (by running DUCM as programme)  . . . . . . . 2-17

Current DU Partition Listing . . . . . . . . . . . . . . . . 2-16

Disc Handler - Modifying & Examining (DU)  . . . . . . . . . 2-17

Disc Usage . . . . . . . . . . . . . . . . . . . . . . . . . 1-2

DUCM Error Messages  . . . . . . . . . . . . . . . . . . . . 2-20

DUCM Manual  . . . . . . . . . . . . . . . . . . . . . . . . 2-1

DUCM SET Commands  . . . . . . . . . . . . . . . . . . . . . 2-5

DUCM Update History  . . . . . . . . . . . . . . . . . . . . 2-2

Error Code SPFUNs  . . . . . . . . . . . . . . . . . . . . . 2-6

Error Messages - DUCM  . . . . . . . . . . . . . . . . . . . 2-20

Error Table - Examining (DU) . . . . . . . . . . . . . . . . 2-16

Examining Error Table (DU) . . . . . . . . . . . . . . . . . 2-16

Fake Time-Out SPFUN  . . . . . . . . . . . . . . . . . . . . 2-7

Getting Size of DU Unit  . . . . . . . . . . . . . . . . . . 2-15

Handler for LARGE Discs - WA . . . . . . . . . . . . . . . . 3-1

Hardware bug restriction . . . . . . . . . . . . . . . . . . 2-3

IGTDUS . . . . . . . . . . . . . . . . . . . . . . . . . . . 2-12

Jobs under TSX . . . . . . . . . . . . . . . . . . . . . . . 2-14

JREAD  . . . . . . . . . . . . . . . . . . . . . . . . . . . 2-12

JWRITE . . . . . . . . . . . . . . . . . . . . . . . . . . . 2-12

Large Discs - WA Handler . . . . . . . . . . . . . . . . . . 3-1

Limitation on RT disc storage  . . . . . . . . . . . . . . . 1-2

List Current DU Partitions . . . . . . . . . . . . . . . . . 2-16

Mapping DU Handler . . . . . . . . . . . . . . . . . . . . . 2-13

Mapping DY Handler . . . . . . . . . . . . . . . . . . . . . 2-13

Modifying TSX  . . . . . . . . . . . . . . . . . . . . . . . 1-6, 2-13

MSCP Bypass  . . . . . . . . . . . . . . . . . . . . . . . . 2-12

Number of Jobs under TSX . . . . . . . . . . . . . . . . . . 2-14

Partition Size SPFUN . . . . . . . . . . . . . . . . . . . . 2-7

Partition Table - Changing DU Handler on Disc  . . . . . . . 2-17

Partition Table SPFUN  . . . . . . . . . . . . . . . . . . . 2-6

Physical I/O SPFUN . . . . . . . . . . . . . . . . . . . . . 2-9

Precautions for WA . . . . . . . . . . . . . . . . . . . . . 3-9

Programme use of DUCM  . . . . . . . . . . . . . . . . . . . 2-15

Programme use of WA  . . . . . . . . . . . . . . . . . . . . 3-5

DUCM - DU Multi-Unit Internally-Queueing Handler et alii                                                       Page 3-12

WA - a Munging Handler for LARGE Discs

Q$UMRX Offset  . . . . . . . . . . . . . . . . . . . . . . . 3-3

Queue Elements . . . . . . . . . . . . . . . . . . . . . . . 3-3

Restriction - hardware bug . . . . . . . . . . . . . . . . . 2-3

RT Partition Table . . . . . . . . . . . . . . . . . . . . . 2-6

Running DUCM as Programme  . . . . . . . . . . . . . . . . . 2-15

SET Commands - DUCM  . . . . . . . . . . . . . . . . . . . . 2-5

SET Commands - WA  . . . . . . . . . . . . . . . . . . . . . 3-4

SETs for DUCM (under RT-11 only) . . . . . . . . . . . . . . 2-5

Setting up Handlers  . . . . . . . . . . . . . . . . . . . . 1-3

Setting up WA  . . . . . . . . . . . . . . . . . . . . . . . 3-8

Size of DU Unit  . . . . . . . . . . . . . . . . . . . . . . 2-15

Size of Partition SPFUN  . . . . . . . . . . . . . . . . . . 2-7

Size of Unit SPFUN . . . . . . . . . . . . . . . . . . . . . 2-7

SP.DAT SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-6

SP.FAK SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-7

SP.PIO SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-9

SP.PSZ SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-7

SP.RIO SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-6

SP.STS SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-7

SP.USZ SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-7

SP.WIO SPFUN . . . . . . . . . . . . . . . . . . . . . . . . 2-6

Status Table SPFUN . . . . . . . . . . . . . . . . . . . . . 2-7

Time-Out Fake SPFUN  . . . . . . . . . . . . . . . . . . . . 2-7

TS$JOB . . . . . . . . . . . . . . . . . . . . . . . . . . . 2-14

TSMUNG - For Modifying TSX . . . . . . . . . . . . . . . . . 2-13

TSX Modification . . . . . . . . . . . . . . . . . . . . . . 1-6

TSX Modifying  . . . . . . . . . . . . . . . . . . . . . . . 2-13

TSX System Generation  . . . . . . . . . . . . . . . . . . . 1-5

TSX TSGEN for WA . . . . . . . . . . . . . . . . . . . . . . 3-8

Unibus Mapping Register Pointer  . . . . . . . . . . . . . . 3-3

Unibus System Restriction  . . . . . . . . . . . . . . . . . 2-14

Unit Size SPFUN  . . . . . . . . . . . . . . . . . . . . . . 2-7

User's Guide . . . . . . . . . . . . . . . . . . . . . . . . 1-1

V5.4 Partition Table . . . . . . . . . . . . . . . . . . . . 2-6

Vector Alteration (by running DUCM as programme) . . . . . . 2-17

Version Number . . . . . . . . . . . . . . . . . . . . . . . 2-7

Version Number (DU)  . . . . . . . . . . . . . . . . . . . . 2-16

WA - Munging Handler for LARGE Discs . . . . . . . . . . . . 3-1

WA - Precautions . . . . . . . . . . . . . . . . . . . . . . 3-9

WA - Programme Use . . . . . . . . . . . . . . . . . . . . . 3-5

WA - Setting up  . . . . . . . . . . . . . . . . . . . . . . 3-8

WA Handler - Unibus Restriction  . . . . . . . . . . . . . . 3-1

WA SET Commands  . . . . . . . . . . . . . . . . . . . . . . 3-4

###########################################################################################################################################################################################################################################################################################################################################