An RT-11 Application

N. A. Bourgeois, Jr.

Sandia National Laboratories

Systems Engineering Division 1738

P. O. Box 5800

Albuquerque, NM 87185

Abstract

An application of the RT-11  single-user  operating

system  is  described.   The  application  involves  the

development of real time  signal  processing  algorithms

for  a  dedicated  microcomputer  system.   The hardware

included in the seven systems  of  this  application  is

discussed.   Reasons  for the selection of and continued

use of RT-11 are given.

80j15a

Page   1

16 Oct 80

This  document  is  issued   by   Sandia

National  Laboratories, operated for the

United States Department  of  Energy  by

Sandia Corporation.

Notice

This  document  was  sponsored  by   the

United  States  Government.  Neither the

United States Government nor the  United

States  Department of Energy, nor any of

their  employees,  nor  any   of   their

contractors,  subcontractors,  or  their

employees makes any warranty, express or

implied,  or assumes any legal liability

or  responsibility  for  the   accuracy,

completeness   or   usefulness   of  any

information,   apparatus,   product   or

process  disclosed,  or  represents that

its use  would  not  infringe  privately

owned rights.

This work was sponsored and funded by:

Headquarters, ESD / OCB / Stop 36

Hanscom Air Force Base, MA 01731

Page   2

16 Oct 80

Introduction

Our groups task at the Sandia National Laboratories

is  the  development of signal processing algorithms for

intrusion   detection   applications.    This   includes

execution both in real time and with recorded data.  The

ultimate goal is a dedicated microprocessor  system  for

each  of  these applications.  We are using seven PDP-11

family machines as tools to achieve this end.

We selected RT-11 [1] for its fast response and its

ability  to  operate  on  the  LSI-11  as well as on the

larger PDP-11 machines.  We plan to continue using RT-11

and  would  select  it  again  if  we were starting over

today.

Hardware

During  the  course  of  this  project  our   tools

(PDP-11'S)  have grown from one PDP-11/10 and one LSI-11

to one PDP-11/34, one PDP-11/10 and five  LSI-11's.   We

are currently upgrading the PDP-11/34 to a PDP-11/60 and

the PDP-11/34 will replace the PDP-11/10.  I  shall  now

describe these systems as if our present upgrade program

were completed.

The two larger machines and three of  the  LSI-11's

are   permanently   located   in   our  laboratories  in

Albuquerque.  One LSI-11 is  configured  as  a  portable

system   and   one  LSI-11  is  installed  in  a  mobile

laboratory.  The UNIBUS machines have  four  RK05's,  an

RX02  and 128K words of memory.  The Q-BUS machines each

have an RX02 and 32K words  of  memory.   The  PDP-11/34

also  has  an  additional  RX02, a line printer, and two

nine-track mag tape units.

Utilization

There is consideral overlap in the  utilization  of

our  computer  systems.  Software development may be and

is done to some  degree  on  all  machines.   Also,  all

systems  are  capable  of performing data analysis.  The

LSI-11's are used  for  data  recording  and  real  time

playback.

Page   3

16 Oct 80

PDP-11/60.  This system is used primarily for  data

analysis.   It  is  also  used  for  original  algorithm

development using recorded data.   There  are  two  work

stations  tied to this system.  Each station consists of

a VT52 and a Tektronix 4015 with a 4631 hard copy unit.

PDP-11/34.    This   is   our   primary    software

development    system.     Operating   system   software

maintenance is performed here.  System software for  all

PDP-11 family machines is supported from this PDP-11/34.

There are four VT100 work  stations  connected  to  this

system.

LSI-11 (Computer room system).  The initial testing

of  real time implementations of both data recording and

signal processing algorithms is done on  this  computer.

I  try  to maintain on this machine at least one each of

any device that is on any of the other LSI-11's.

LSI-11 (Dual system).  Testing  of  various  target

microprocessor systems is accomplished in the laboratory

with this dual processor system.  One  CPU  is  used  to

control  and  to deliver one or two channels of recorded

data in real time to  a  target  microprocessor  system.

The second CPU monitors the progress of the system under

test.

LSI-11 (Portable system).  This system is  packaged

to  be  airline  checkable.   It  is  used for both data

acquisition and real time  algorithm  execution  in  the

field.

LSI-11 (Mobile system).   A  motor  home  converted

into  a  mobile  laboratory  houses  this  system.   Its

primary function is to control and monitor  up  to  four

target   microprocessor  systems  in  the  field.   Data

acquisition and real time algorithm  execution  is  also

accomplished with this machine.

Software

RT-11 is the only DEC disk operating system capable

of  running on all of our PDP-11 family computers.  Most

of our data rates are handled with  ease  under  RT-11's

interrupt   protocol.    On  occasion  we  must  run  an

interrupt service routine outside of the  system,  which

incidently, RT-11 does permit.

A near RT-11 environment is provided on our  larger

machines  with a time sharing executive [2] from another

vendor.  Thus, we use essentially the syntax of only one

operating  system on all of our computers while allowing

Page   4

16 Oct 80

multiple user access to our larger machines.

Our real time software is all written in  MACRO-11.

Other application software is done in FORTRAN and PASCAL

with some even in BASIC.  Our technique is to develop an

algorithm   using  FORTRAN,  then  to  implement  it  in

MACRO-11 using integer arithmetic.  Once this real  time

code  is  proven  both in the lab and in the field it is

translated into the target microprocessor's language.

Page   5

16 Oct 80

References

1.  "RT-11, Version 4.0 Single-User  Operating  system",

AE-3393M-TC, Digital Equipment Corporation, Maynard,

MA, April 1980.

2.  "General Release Notes for  TSX-Plus  Version  1.4",

S & H Computer Systems, Inc., Nashville, TN.

####################################################################################################################################################################################################################################################################################################################################################################################################################################################