X-NEWS: eisner comp.os.vms: 15065

Relay-Version: VMS News - V6.1 30/1/93 VAX/VMS V5.5-2; site eisner.decus.org

Path: eisner.decus.org!noc.near.net!howland.reston.ans.net!wupost!waikato.ac.nz!comp.vuw.ac.nz!zl2tnm!toyunix!don

Newsgroups: comp.os.vms

Subject: Re: SLIP

Message-ID: <14143197@zl2tnm.gen.nz>

From: don@zl2tnm.gen.nz (Don Stokes)

Date: 17 Jul 93 04:44:41 GMT

Sender: news@zl2tnm.gen.nz (GNEWS Version 2.0 news poster.)

References: <1993Jul15.023938.1@cc.curtin.edu.au>

Distribution: world

Organization: The Wolery

Lines: 120

zrepachol@cc.curtin.edu.au (Paul Repacholi) writes:

> Why should I run a different protocol over say a DUP like dumb sync

> interface with a 9600 bps modem versus a dumb DL like async devise

> with a 9600 bps modem?

Simple: bit-oriented vs byte-oriented.

In async comms, you have characters coming at you with an easy-to-spot

start bit to say when the character starts.  In sync comms, you don't;

all you know is where the bits start and end (the modems do this; clocking

is (usually) done in the DCE and fed to the DTE, whereas in async comms

the clocking is done inside the UART in the DTE).

This means that sync comms requires periodic synchronisation so that

the two ends agree about which bits are coming when.

There are two ways to do this: synch characters and out-of-band framing.

Synch characters are easy -- they're just characters, eight-bit patterns

that the receiver looks for when it's not sure about where in the bit

stream the characters start; typically, a packet will start with one or

more SYN characters.  Having recognised a SYN, the rest of the packet

can be read off the bit stream as bytes.

Out-of-band framing is a little more interesting.  Each packet both starts

and (usually) ends with a frame delimiter, usually 01111110, and usually the

hardware is looking for this bit pattern to delimit the packet and pass it

to the software.  Because that pattern may appear in the packet data, a

technique called bit-stuffing is used, and this really requires hardware

assistance to do efficiently.  If the sender finds six '1' bits to send,

it sends five '1's and then follows them with a '0'.  It then sends the

sixth '1' and continues.  The receiver then on receipt of five '1's checks

the following bit; if it's a '1', this is a frame delimiter, if it's a '0'

it discards the '0' and takes the next bit as the sixth; thus six '1's

never appear in the bit-stream except when they represent a frame delimiter

or an error.

Most modern synchronous links these days use HDLC or SDLC, which utilise

the latter method.  I'm not sure what DDCMP does (I only ever just plugged

it in and watched it go 8-).  Braindamaged protocols such as 2780 tended to

use SYN characters.  (I think I have permanent psychological scars from

staring too long at 2780/3780 traces.  Gack.  7-layer model?  Eh wassat?)

The problem here is that sync links usually have hardware helping them

along to do bit-stuffing, framing and often FCS computation, whereas async

hardware tends to be basically stupid.  If you try to model the async

protocols on the sync protocols, you find that you have to do something

special to create an eight-bit clean protocol, since you have no method of

doing out-of-band framing (see below).  You also have to do a lot of the

stuff you were doing in hardware in software, which means that the solutions

chosen have to be lightweight or they burden the CPU.

Enter SLIP.  SLIP, Serial Line IP is the minimalist's minimalist protocol.

All SLIP provides is framing; it has no FCS, relying on the higher level

protocols to decide whether a packet is bad.  (In my own SLIP code, I do

a few things to try to discard packets that might be bad, but the lack of

a real frame check means that this checking is pretty minimal.)

This is what SLIP does:

SLIP defines four special characters: END (0300 octal), ESC (0333), ESC_END

(0334) and ESC_ESC (0335).  Receipt of an END character terminates the

packet.  Note that there is no packet start character.

If an END character appears in the data stream, it is replaced with two

bytes: ESC ESC_END.  To complete the picture, and ESC character in the data

stream is escaped with ESC ESC_ESC.

That's it.  No FCS, no protocol type, nothing but simple-minded framing.

Piss-easy to implement.  Bugger-all overhead.  Really easy to do in VMS:

just sling a bunch of largish QIOs at the terminal driver with terminator

masks specifying just the END character and what you get back are properly

framed packets.

There is one optimisation to this, which is optional, and that is to send

an END character at the beginning of each packet; this means that your

SLIP driver gets to deal with zero-length packets, but if there's any noise

between packets, the noise and the next packet are treated as separate

things and the real packet doesn't get lost.

SLIP is described by RFC 1055.  It includes a C code implementation, and

discussion of what SLIP doesn't do.  It's six pages long.  8-)

There are a few gotchas with SLIP: since there is no frame checking in

the protocol, your higher level protocols had better check their packets

themselves, and in UDP checking the checksum field is *optional*.  NFS

for example can be used without UDP checksums, but you should be able

to turn UDP checksumming on in your NFS implementation.  TCP does check

what it's doing.

PPP is a bit more complex, and includes most of the features found in the

synchronous protocols.  Packets are frame-checked and bad packets discarded

before being passed up the protocol stack, and there is a packet type field

to allow more than one protocol to be forwarded over a PPP link.

There are PPP versions for both sync and async lines; the async stuff

includes dealling with things like XON/XOFF flow control and other

nastyness intended for dealling with character oriented terminals rather

than packet oriented network links.  However, PPP has more overhead on

async serial links than SLIP, both in terms of bytes sent and processing

required.  If you're just shipping IP around the place, SLIP is easiest,

most common and lowest overhead, but has a few caveats.

One other factor in all this is TCP header compression.  A single keystroke

in a telnet session gets sent with 20 bytes of IP header and another 20

bytes of TCP header as well as any data-link layer overhead.  Van Jacobson

TCP header compression can collapse that 40 byte overhead into four bytes

(plus data plus DLL overhead) using the principle that very little in that

40 bytes actually changes from packet to packet and you only need to send

what changes plus the checksum.  This makes telnet sessions over 9k6 or

worse links beareable.  Since SLIP lacks a protocol type field, TCP

compression on SLIP links has to be implemented in the SLIP driver, rather

than implementing it separately and passing the compressed packets to SLIP

as a separate protocol.  RFC 1144 describes how all this works.  (There's

a section entitled "Compatibility with past mistakes" that describes the

hackery required to do this -- I've implemented SLIP with TCP compression,

and yes, it is a bit yucky.... 8-)

--

Don Stokes, CSC Network Manager, Victoria University of Wellington, New Zealand

Ph+64 4 495-5052  Fax+64 4 471-5386  Work:don@vuw.ac.nz  Home:don@zl2tnm.gen.nz

############################################################################################################################################################################################################################################