Fixes put into the Better Board (Big Board II) ROMs

As of 07-0-85:

1   Moved scroll code out of VSYNC interrupt to speed up the execution.

2   Made the bell turn-off an interrupt service (4Hz) function.

The combination of the above two allows operation at better

than 9600 baud without any character loss!

3   Disable the VSYNC interrupt during CLRLINE.  This prevents

the block move from sometimes copying the cursor into the line.

4   Move the setting of MOVECS in SETCSR to before the CTC is

enabled.  This should prevent some strange cursor placement.

5   Moved the 6845 setup tables to CRTTABLE.EQU.  This will allow easier

updates of other peoples code, while leaving their edited tables

to remain.

6   Added equates and hooks to support either CRC, or Checksum, or no

power-up testing.  When no testing selected, optional ROM

at 1000H is tested for two jumps at the beginning.

7   Fixed handling of NULLS inside of ESCAPE sequences.

########################################################