NAME

install -- integrate new device drivers into Cpm, Revision 1.3.

SYNOPSIS

install -string newbios address [ prlfile2 address2 ... ]

DESCRIPTION

Install integrates new device drivers into an existing Cpm 2 system.

"Newbios" is a skeletal Cpm bios module in prl format (see the prl

document) containing the new drivers. Install first loads newbios at

the given address (usually the top of Cpm), then patches it into the

existing bios by exchanging the addresses of the 17 jump vectors that

begin each bios. If additional prlfiles are given, they are loaded at

the requested addresses with no patching. Finally, install prints

(top of newbios) + (newbios buffer size, if any) + 1 as the "new load

address". Additional drivers can now be installed at this address.

The flagged argument (-string) is copied to 80H and left there for

examination by the newely installed "Newbios", or a NULL if no

flagged argument is present. The bios modules provided here (see below)

use this as an initial drive specification. i.e. -c specifies that the

newely installed bios should respond to drives C on.

The Cpm system must be "standard": its bios must begin with the jump

table defined in the "Cpm 2.0 Alteration Guide", and it must store

the address of this table at location 1 in memory.

A "skeletal" bios module begins with the same jump table, but may

leave some of the required functions unimplimemted. The corresponding

jump vectors are left as jump selfs.

EXAMPLES

To install the m26 hard disk as drives C, D, and E in a 48K Cpm

(assuming that you have memory above 48K), execute

install -C M26.prl C000

Here, M26.prl is the "skeletal bios" (provided with install)

containing the m26 drivers, and C000 (hex for 48K) is the top of Cpm.

If you only have 48K of memory in your system, you will first have to

move your Cpm downward to make room. At this writing, the m26 drivers

require approximately 3.5K bytes, (see FILES below), so you should

construct a 44K Cpm, and type

install -C M26.prl B000

(B000 is 44K in hex.) If you don't know how to move your Cpm downward

see your system manual. The required program is usually called

"movcpm".

To install the Disk Jockey as drives L and M in a 44K Cpm (with at

least 48K of memory), type

install  dj.prl  B000  djram.prl  E400

Here, dj.prl contains the Disk Jockey 2D drivers, B000 is the top of

the 44K Cpm (the dj drivers require 3.25K), djram.prl is updated

controller firmware to be loaded into the controller ram, and E400 is

the address of the controller ram. (If you have a non-standard

controller, use the controller address + 400 hex.)

To install both the m26 and the Disk Jockey, you could first install

one and then install the other on top of it (at the printed "new load

address"). This would require 6.75K of memory above Cpm. As a better

alternative, there is a combined driver package that requires only

5K of memory. To use it, configure a 43K Cpm (assuming 48K of

memory, or use available memory - 5K), and type

install  M26+dj.prl  AC00  djram.prl  E400

FILES

The provided files are:

install.com Install program.

install.doc This document.

prl.doc     Prl file format.

M26.prl     M26 drivers, require 3.5K, default to drives N,O, and P.

dj.prl      Dj drivers, require 3.25K, default to drives L and M.

djram.prl   Dj ramware. Load at controller address + 400 hex.

M26+dj.prl  Combined m26 and dj drivers, require 5K, default to drives

L and M for the Disk Jockey, and N,O and P for the M26.

M26+dj.asm  Assembler source for M26.prl, dj.prl, and djram.prl.

Requires the Digital Research assembler "mac", or

"asm" plus hand editing of the macros.

djram.asm   Assembler source.

BUGS

Addresses must be specified in 4-digit hex (with no leading 0 and no

trailing H). Install should also accept octal and X-notation.

When the DJ drivers are first accessed, they locate the controller by

searching memory for a certain pattern in the controller rom. The

search is done carefully, but nevertheless might clobber somebody's

memory-mapped io.

install.doc  8/26/80  bdg

#####################