Start up AMPRO from floppy disk drive using ZCMD26 working disk.

Floppy disk you boot from will be drive A.

FOR 58K CPM:

DDT CPM58.COM

-IZCMD26.HEX

-R3d80

-G0

Run SYSGEN which is on floppy drive A.  SYSGEN will ask for source

drive,  press RETURN to take RAM image.   SYSGEN will then ask for

destination drive.   Press "A" to write new ZCMD26 to floppy drive

A.  Write new ZCMD26 system to drive A and then reboot system.

FOR 57K CPM:

DDT CPM57.COM

-IZCMD26.HEX

-R4180

-G0

Run SYSGEN which is on floppy drive A.  SYSGEN will ask for source

drive,  press RETURN to take RAM image.   SYSGEN will then ask for

destination drive.  Press "A" to write  new ZCMD26 to floppy drive

A.  Write new ZCMD26 system to drive A and then reboot system.

Once  you  have written new ZMD26 to floppy drive A  and  REBOOTED

system with new ZCMD26 now active,  run the program HINIT on drive

A.

HINIT will ask if you want to clear hard disk assignments.  Enter

a "Y" (YES).

HINIT  will  display a menu with two choices,  select  the  first

choice which will allow you to DEFINE your hard disk drive.

For  the  SCSI drive controller address enter  a  "0".

For  the  Controller type enter a "1" (Generic burst mode SCSI  -

Adaptec 4000).

For the drive logical unit number enter a "0".

You  will  then return to the original HINIT menu having  the  two

options.  Select  the  second menu choice and define the six  hard

disk drive assignments.

For hard disk drive "F" assign a drive size of "7450".

(The "7450" is the number of "K bytes" for each drive partition.)

For hard disk drive "G" assign a drive size of "7450".

For hard disk drive "H" assign a drive size of "7450".

For hard disk drive "I" assign a drive size of "7450".

For hard disk drive "J" assign a drive size of "7450".

For hard disk drive "K" assign a drive size of "7450".

Exit the HINIT program gracefully.  (ESC, I think).

Run SYSGEN on floppy drive A again and when SYSGEN asks for source

drive   enter  the  floppy  drive  "A".    When  SYSGEN  asks  for

destination drive enter the hard disk drive "F" (this is the first

hard disk drive partition).  You will write your new ZCMD26 to the

first  hard  disk  partition.   This ZCMD26 system  is  not  fully

configured yet.

Exit SYSGEN gracefully.  (By pressing <RETURN>).

Run the program "SWAP" on floppy drive A.

What  you  will need to do is "juggle or swap  drive  assignments

until all the hard disks partitions are reassigned from drive A to

drive F,  the floppy drives are G,  H,  I,  and J, and the virtual

drive will be disk K.   Remember that ORIGINALLY the floppy drives

are  A and B with drives C and D optional floppy drives which  are

NOT installed.   The assignments for the hard disk are  ORIGINALLY

drives  F thru K.   You will need to swap the drives and hard disk

configurations until you have this configuration:

Original configuration            Final Configuration

A  (present floppy)                A  (originally Hrd Dsk drive F)

B  (present floppy)                B  (originally Hrd Dsk drive G)

C  (floppy not installed)          C  (originally Hrd Dsk drive H)

D  (floppy not installed)          D  (originally Hrd Dsk drive I)

E  (virtual - foreign format)      E  (originally Hrd Dsk drive J)

F  First hard disk partition       F  (originally Hrd Dsk drive K)

G  Second hard disk partition      G  (originally Floppy Dsk drive A)

H  Third hard disk partition       H  (originally Floppy Dsk drive B)

I  Fourth hard disk partition      I  (originally uninstalled floppy drive C)

J  Fifth hard disk partition       J  (originally uninstalled floppy drive D)

K  Sixth hard disk partition       K  (originally virtual foreign drive E)

To get an idea what is happening,  use the LIST option to list the

drive assignments in the SWAP program.

SWAP will ask you for what drives you wish to swap.

Here is one swap scheme (not very efficient).

Swap drive "G" with drive "L".   L now has H.D. drive################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################