MTEST - tests user RAM memory

Syntax:

mtest hex1 hex1 [/[c][r][h][p][s=xx][l=xx]]

options

hex1        - Must be supplied. Specifies the starting segment

address where the memory test should begin. For

example to begin memory just beyond the first 64K

use 1000.

hex2        - Must be supplied. Specifies the last segment address

for search.

c           - Specifies that the search should be continued

indefinitely.  The default is to do the search only

once.

r           - Multiple bit patterns can be tested in each memory

location.  With this option only the first occurence

of an error is reported.  The default is to specify

each occurence.

h           - Specifies that the program should be terminated on the

first occurence of an error.  The default is to continue

the search from hex1 through hex2 regardless of error.

p           - Display the segment address of the current test.  With

standard output redirection (see below) this option is

is not advised.

s=xx        - bit patters from 0 to 255 can be tested in each memory

l=xx          address; however, testing all possible combinations takes

a while. To shorten the test (but also rendering it less

thorough) set the starting code value (s=) and the last

code value (l=).

Examples:

mtest b800 b8a0                         - test the graphics board

RAM locations specified.

mtest b800 b8a0 /c                      - same as above but do it

continuously until operator

hits control- break.

mtest b800 b8a0 /ch                     - same as above but if error

occurs exit program.

mtest b800 b8a0 /s=170l=170ch           - same as above but use only

one bit patter for test.

mtest b800 b8a0 /hc                     - continuous test but if

error occurs halt.

Redirection:

output can be directed to screen (default) or

printer by appending >prn: to the command line

disk by appending >d:fn.ft to the command line

--------------------------------------------------------------------------

R.H. Wollenberg  472-4597

###############################