T A N D E M

----------------------------------------------------------------

THE TANDEM REMOTE SYSTEM      870829

------------------------

The TANDEM remote  system does what other  modem programs cannot

do.  It allows you  to run virtually all PC-DOS  software from a

remote location;  Home-Office, School, Classroom,  Factory, etc.

Even run those pesky  programs that bypass  the operating system

and write directly  to the screen memory.   You can also operate

PC-DOS  from  a dumb  terminal.   TANDEM will  do what the  CTTY

command  and  many   modem program  SHELL   or GATEWAY  commands

promise, but fail to deliver.

TANDEM is broken  down into  several modules  to provide greater

flexibility.    There  are  four functional levels.   The lowest

level is a fully  interrupt  driven replacement  for the  serial

port  drivers  (COM1,  COM2)  in  the  ROM  BIOS.    This is the

ICOM12.COM module  on your diskette.  The  next level interfaces

the selected serial  port  with the keyboard  and screen memory.

When  TANDEM  is used  with  a dumb terminal  (H/Z-19 or VT-52),

these   modules   provide   key   code   and character   graphic

translation.  The  modules  are identified  on your diskette  as

follows:  RHSTKBtp.COM  and  RHSTSCtp.COM.    Where  t=  C or  H

indicating  a remote  computer  or H/Z-19 terminal,  and p=  the

serial port number,  1 or  2.  The third level  is required only

when  the  TANDEM system  is  used in conjunction  with a modem.

The  HMODEM.EXE  file  is the  host modem program  that provides

password  protection  and  modem  control  for  the host system.

RMODEM.EXE  is  a remote  modem  program  for  use on the remote

computer  system.    The fourth  level  is the  PC-DOS operating

system and all of its applications software.

The TANDEM remote  system  is warranted to  be free from copying

defects when shipped.  There are no other  warranties, expressed

or  implied,  including,   but  not  limited   to,  any  implied

warranties  of  merchantability  or  fitness  for any particular

application.  In  no event  will Louis E. Wheeler  be liable for

consequential  damages  even  if advised of  the possibility  of

such damages.  This  software  has been thoroughly  tested,  but

there may  be bugs  for which  Louis E. Wheeler  cannot be  held

responsible.  No  liability or responsibility  is assumed to the

customer  or  any   other   person  or  organization   for   any

application or use  of this  software or documentation  sold  or

distributed by Louis E. Wheeler or any dealer.

GETTING STARTED

---------------

Before  attempting  to  run  the TANDEM system,  it is suggested

that you do the following:

1.  Format a blank  diskette and make a  COPY of your original

diskette.  Then store  your  original in a  safe place (just  in

case).

2.  Using the copy  you  have just made,  COPY the COMMAND.COM

file from your boot diskette onto the TANDEM working diskette.

3.  Run the MAKEBAT  program.   MAKEBAT will  create two batch

files that will make  it  easier to run the  TANDEM system.  You

will be asked to indicate whether you are  running TANDEM with a

remote computer or  H/Z-19 terminal.  You  will also be asked to

select  a  serial (COM)  port  and the baud rate  at which  your

modem will be operated.

4.  After  running  MAKEBAT,  make  a  second   COPY  of  this

diskette.  It will  be useful  to have two  copies, one for  the

host computer, and one for the remote computer.

5.  Using  any  ASCII  text  editor,  create   a  file  called

PASSWRDS.DAT that  contains a list of (up  to 10) passwords that

are to allow access  to your host computer.   Each password must

be on a separate  line  and  can be entered  using any printable

character and may  contain  up to 80-characters.   If you  wish,

the password file  can be  created using the  COPY command as in

the following example:

A:>COPY CON: PASSWRDS.DAT <RETURN>

JOHN SMITH<RETURN>

BOB JONES<RETURN>

BJ345-77XJ<RETURN>

^Z<RETURN>

Any  of  these  three   passwords,   (JOHN   SMITH,  BOB  JONES,

BJ345-77XJ,  will  permit  access  to the host  computer from  a

remote location.

OPERATING INSTRUCTIONS

----------------------

For the host computer:

1.  Turn on your  equipment  (including your  modem) and  boot

the computer in the normal way.

2.  Place one of  the diskette  you have  created (see GETTING

STARTED) in drive A: and "log" onto this drive.

3.  At the A:> prompt, enter the command  HOST and press ENTER

(or RETURN) to execute the HOST batch file.

4.  The  programs  (or commands)  in  the batch  file will  be

executed.  The HMODEM  program  will initialize  your modem  and

ready  it  for auto-answer  at  the  selected  baud  rate.  Your

screen will clear  and the  host computer will  then be ready to

accept calls.

5.  If  you wish,  you can  continue to use  the host computer

from its  local keyboard  by  first typing OK  and a RETURN  (or

ENTER), then type CONNECT followed by a RETURN  (or ENTER).  The

rest is as though  you were operating from  a remote computer or

terminal.  This technique  can also be used  to determine if the

host system is functioning  correctly.  Note:  the words  OK and

CONNECT must be entered using capital letters.

For the remote computer:

1.  Turn  on your  equipment  (including modem)  and boot  the

computer in the normal way.

2.  Insert the  TANDEM  diskette in any disk  drive and  enter

REMOTE to execute the REMOTE batch file.

3.  The  programs  (or commands)  in  the batch  file will  be

executed.  The RMODEM  program  will then ask  you to enter  the

modem dial command.   An  example, for most  modems, would be as

follows:  ATDT 488-1234  <RETURN> assuming  your phone number is

488-1234.  Refer to your modem manual for details.

4.  When the host  system  answers the phone,  the host system

will clear your screen  and ask you to enter  a password.  Enter

one  of the  passwords  from  the PASSWRDS.DAT  file you created

earlier, followed by a RETURN (or ENTER).

5.  If  you have  entered  the  correct password,  the  TANDEM

prompt, TANDEM:>, should appear on your screen.   There are four

commands that are  accepted  from the TANDEM  prompt: CLS,  DIR,

DOS, and BYE.

CLS - clears the screen and re-displays the TANDEM prompt.

DIR - display the currently logged drive directory.

DOS - accesses  the PC-DOS  operating system.   You may  enter

any command or run  any program  just as you  normally would  if

you were sitting  at the  host computer console  keyboard.  With

two exceptions, all  keys  are functional.   The Control-NUMLOCK

(to   freeze    the    screen    when   scrolling)    and    the

Control-Alternate-DELETE  (system  reset) keys  do not function.

{If you are using  an H/Z-19  terminal, hitting  the ESCAPE  key

and then a Control-(A  thru  Z) key will produce  the equivalent

of  the  Alternate-(A  thru  Z)  keys,  this  provides  for  the

alternate  key  codes  not  available on the  terminal.}  Before

signing  off,  enter  EXIT  and a  RETURN at the  PC-DOS prompt.

This will take you back to the TANDEM prompt.

BYE - informs the  host  system to hang up  the phone (modem).

and get ready to  receive  the next call.   You should turn  off

the remote modem  and hang up the phone.   See your modem manual

for instructions on how to cause your modem to hang up.

For direct connection via RS-232C cables:

1.  Connect  any  two IBM-PC  compatible   computers  together

using a "null modem"  cable.  Designate one  of the computers as

the "host" computer and the other as the "remote" computer.

On the "host" computer:

2.  Insert a TANDEM  diskette  in drive A:,  and log onto  the

drive.

3.  Run the ICOM12 program.

4.  Set  the baud  rate  on the  desired COM  port  using  the

PC-DOS  "MODE"  program.    Refer  to  your  PC-DOS  manual  for

instructions on how to use the MODE program.

5.  Select  and  run one  of the keyboard  interface programs,

RHSTKBC1 for COM port 1, or RHSTKBC2 for COM port 2.

6.  Similarly,  select  and  run  one of the  screen interface

programs, RHSTSCC1 for COM port 1, or RHSTSCC2 for COM port 2.

7.  Run the DIRECT program to enable the screen interface.

On the "remote" computer:

8.  Insert a TANDEM  diskette  in any drive  and log onto  the

drive.

9.  Execute the  REMOTE  batch file.  When  the RMODEM program

asks you to dial the phone, just enter CLS  to clear the screen.

You should now see the same information displayed  on the "host"

and "remote"  computer  screens.   Both the  "host" and "remote"

keyboards should  also be  active and the host  computer  can be

controlled from either.

PROBLEM PROGRAMS

----------------

Virtually any PC-DOS  command  will work and  any program can be

controlled from a  remote computer or terminal.   However, there

are  a few  exceptions.   The  programs  that  will not function

properly are those  that fall into one or  more of the following

categories:

1.  Those programs  that manipulate the  COM ports.  Such as a

modem program.

2.  Those  programs   that   assume  direct   control  of  the

keyboard.  The QuickBASIC  screen  editor is  one example.  Most

word processing programs tested do not exhibit this problem.

3.  Those  programs   that  improperly  use   the  timer  tick

interrupt  (INT  1C).   Some  "Pop-Up" clock  programs fit  this

category.  However,  they  will function correctly  if they  are

loaded prior to starting the TANDEM system.

4.  Those   programs   that   use  dot addressable   graphics.

Programs that use character graphics are okay.

5.  Programs  that  use  animation  (usually  games)  may  not

perform satisfactorily.

FILES ON THE DISTRIBUTION DISKETTE

----------------------------------

The following  files  should  be on your distribution  diskette,

should  any  of the  files  be  missing,  write  and  request  a

replacement diskette.

1.  ICOM12.COM          Replacement for INT 14.

2.  RHSTKBC1.COM        Keyboard, computer, COM1.

3.  RHSTKBH1.COM        Keyboard, H/Z-19, COM1.

4.  RHSTKBC2.COM        Keyboard, computer, COM2.

5.  RHSTKBH2.COM        Keyboard, H/Z-19, COM2.

6.  RHSTSCC1.COM        Screen, computer, COM1.

7.  RHSTSCH1.COM        Screen, H/Z-19, COM1.

8.  RHSTSCC2.COM        Screen, computer, COM2.

9.  RHSTSCH2.COM        Screen, H/Z-19, COM2.

10. HMODEM.EXE          Host modem program.

11. RMODEM.EXE          Remote modem program.

12. MAKEBAT.EXE         Program to create .BAT cmds.

13. DIRECT.EXE          Program for direct connect.

14. PASSWRDS.DAT        Example password file.

15. HOST.BAT            Sample host batch file.

16. REMOTE.BAT          Sample remote batch file.

17. TANDEM.DOC          These instructions.

18. READ-ME             A note to the user.

DISTRIBUTION and UPDATES

------------------------

This first version  of TANDEM  is being distributed  at the very

low, low  cost of  only $6.oo  -- which we hope  will cover  our

expenses.

If  there  is sufficient  interest  in  the  TANDEM  system,  an

expanded  version  will  be  produced having  advanced features,

such  as file  transfer   between   host  and  remote,  improved

password checking,  etc.  The updated version  will be priced at

$2O.oo, still a very low cost.

For those  of you  who would  like to see how  TANDEM works,  or

would like to customize  TANDEM for your specific  requirements,

the complete source  code  will be made available  for a nominal

charge of $4O.oo.

To order the latest version, or a copy of  the source code, send

your  check  or money  order  along with a note  indicating  the

items you are requesting, to:  Louis E. Wheeler

P.O. Box 888

Oceano, CA  93445

California  residents,  please  add  6%  for  sales  tax.   Site

licenses are available.

We would also like to hear if you encounter  any problems (bugs)

or have any suggestions for improving our product.

We sincerely hope that TANDEM serves you well.