PLINK65.DOC

PLINK is a CP/M transient command which allows the user to

communicate with a remote computer that is not running a special

program.  It allows the user to selectively copy received text

into memory and to later write the text onto a CP/M disk file.

It also allows text to be transmitted from a disk file to the

remote computer a line at a time, suppressing the line feeds and

waiting for a trigger character such as a line feed from the

remote computer before sending the next line of text.  Optionally

the trigger character may be a BELL code, an X-ON, or text may

be sent without pause including line feeds after the carriage

return.  Note that none of these options work for sending to a

DEC which uses an X-OFF, X-ON protocol when it gets more input than

it can handle.

This program currently supports the following modems or computers

via conditional assembly:

1. PMMI modem

2. Any serial I/O board (TUART included)

3. TRS-80 model 1

4. TRS-80 model 2

5. Heath H8 with 8251 UART at port 330q

6. D.C. Hayes 80-103A or Micromodem 100

7. MITS 2SI/O board, ports 10h &11h = console, 12h & 13h = modem

8. Intel SBC or National BLC multi-bus boards using 8251 USART

Originally written by L.E. Hughes (EDCAM) in July, 1977.  Many

modifications have been made since this time, as shown in the

following summary.

Fixes/updates (in reverse order to minimize reading time):

June 26, 1981. Added message when exiting if last buffer was

not saved.  Ted Shapin.

June 14, 1981, by Keith Petersen, W8SDZ.  Changed port

equate to 'equ' instead of 'set'.  ASM doesn't like 'set'

when later conditionals are based on a label defined that

way.

June 7, 1981, by Tom Jorgenson (CP-MIG).  Changed CP/M

origin from being via SETs to referenced to BASE, added

TRUE/FALSE rather than numeric values (for readability),

changed ^Q function to ^W (write) because some systems

(notably Micronet) use ^S/^Q to suspend/resume output,

changed page 0 references in TRS routines to use

BASE equate properly, changed PORT equates to default

to TRUE, reinserted Heath equates, and cleaned up code

in several places.

June 7, 1981, by Keith Petersen, W8SDZ.  Fixed problem with

equates which prevented assembly by 'ASM' when TUART option

was selected.

June 6, 1981, by Keith Petersen, W8SDZ.  Added version number,

cleaned up file.

May 12, 1981, by T. Shapin.  Added code for 8251 USART on Intel

SBC or National BLC multibus board with modified CP/M origin.

Added prompt to signon. Added toggle to ^Y to save or ignore

incoming text. Added ^C abort on file name response.

(for earlier update info, see PLINK65.ASM).

PLINK currently supports two way transfer of text files between

the  CP/M disk and the remote computer.  The following control

codes may be initiated from the console keyboard:

Control-E      Exit PLINK to CP/M "warm-boot".

Control-T      Transmit ASCII file to remote system, asks for

drive (A, B, etc.) and filename.typ.

Control-C      Aborts transmition of file to remote system.

Control-Y      Switches between saving and ignoring incoming

ASCII data in RAM buffer, for later transfer

to disk.

Control-Q      Writes RAM buffer to disk, and asks for drive

and filename.typ.

Del (delete)   Backspace when in command mode (e.g. ^T or ^Q).

Control-U      Aborts current line when in command mode.

(Note: all other control codes are passed to modem output, and

may be interpreted by the remote system as various control

functions.)

When sending to another computer, PLINK waits for a trigger

response after sending each line before it will send the next.

The following "trigger" equate is set to "lf" (linefeed)

by default.  An optional trigger char may be passed via fcb1

ie:  PLINK B		will set trigger to "bell"

The following options are allowed:

1. B = bell  07h

2. X = xon   11h

3. U = upload no trigger check at all and send line feeds

any other ascii character may be passed through fcb1.

##########