DISKED

DISKETTE EDITOR

(Release 1)

Diskette Editor is a program that I wrote because I needed it.  It

basically is a sector editing program but could be used as just a

"snooping" device.  Unlike many others, it can read the "special"

sectors that various programs use in their copy protection schemes.

Diskette Editor is more of a tool, than a utility, for example, using

it you can undelete files like many utilities, except it is not

automatic; but this gives you more power.

STARTING UP THE PROGRAM

Diskette Editor is started by typing DISKED at the DOS prompt.  No

command line arguments are used.  The screen that comes up after

loading, is Diskette Editor's only screen.  The screen has a great deal

of information on it.  (Note: All the numbers displayed and inputted in

Diskette Editor, and the numbers in this manual, are in hexadecimal

unless otherwise specified).

GENERAL INFORMATION ON THE SECTOR EDITING SCREEN

Diskette Editor's main screen has a lot of information on it, but it is

organized in an efficient way.  The screen is comprised of two colors.

The colors vary according to the type of monitor you have, but one of

the colors is bright and the other is dim.  The brighter color is the

highlight color.  The highlight shows what key to press for what

function.  For instance, the command "Verify" has the "V" in the

highlight color which signifies that it is executed by pressing the "V"

key.  Also, "Mask" has the "k" in highlight which shows "k" is pressed.

From now on, the highlighted key will be written in between "[" and

"]", so in the above examples, the verify command would be [V]erify,

and the mask command would be Mas[k].

To execute the commands the depression of the shift key is not

necessary, but allowed.  For most of the cases this will seem natural

to you, but in others it is different from what you might expect.  An

illustration of one of these cases would be pressing the period key for

the [>] command.  This was done as a convenience.

OVERVIEW OF THE SECTOR EDITING SCREEN

The most noticeable part of the screen are the two windows.  When the

program first starts they are both blank except for a copyright notice

in the left window.  When a sector is read in from a diskette, its data

is displayed in these two windows.  The one on the left side of the

screen (labeled "[H]exadecimal") displays the data in hexadecimal

bytes.  The window on the right (labeled "[A]SC II") is the same data

in its corresponding ASC II (text) form.  Exotic characters (not

alphabetic, numerical, or punctuation marks) are displayed as periods.

(To change this, see the Mas[k] command in COMMANDS OVERVIEW).

The bottom-most line contains information on the current drive, side,

track number, sector number, and sector size (in decimal).  The two

lines above the bottom-most line, contain a list of available commands

in the highlighted form.  For a description of the use of these

commands, see the COMMANDS OVERVIEW section.  The line above those is

the status line.  Most of the messages to you from Diskette Editor are

displayed here.

EDITING SECTOR DATA

Before you can edit a sector, you must first read the data in from a

diskette.  The [R]ead command does this.  It reads one sector from the

current drive, side, and track.  The windows are then filled with the

data read from the diskette.  When you first run Diskette Editor, the

sector data buffer is filled with zeros, the current drive is A, the

current side is 0, the current track is 00, and the current sector

number is 01.  This is the diskette's boot record which is read by the

BIOS during a reboot.

The default mode when Diskette Editor is started is hexadecimal.  (This

is changed with the [H]exadecimal and [A]SC II commands).  A pointer

highlights the current byte in the data, either the hexadecimal or the

ASC II.  The four arrow keys on the numeric keypad move it in the

indicated directions.  If you move the cursor into one of the four

sides, the window will scroll, or the pointer will wrap, if there is

more data in that direction.  The [Home] key moves the pointer to the

first byte of the first page, and the [End] key moves the pointer to

the last byte of the last page.  The [PgUp] key displays the previous

page of data, if one exists, and the [PgDn] key displays the next page

DISKED  page 2

of data, again if one exists.

The two windows fit 256 bytes of data (one page).  A normal diskette

sector is 512 bytes so there are two pages.  Using the keys mentioned

in the above paragraph, it is possible to view all the data in a

sector.  Instead of using the [PgDn] key, you could press the cursor

down key in the numeric keypad 16 times but the [PgDn] key is

perceptibly faster.

To change a byte use the [M]odify command.  (The space bar also

activates the modify command).  This lets you change the byte being

highlighted by the pointer.  If you are in the hexadecimal mode, type

in a new hexadecimal value; if in the ASC II mode, type the character

to replace the highlighted.  After a byte is modified the pointer is

moved automatically to the next byte.  To modify the next one you must

either press the space bar or [M]odify.

All the modifications that you make are to the sector data stored in

the computer's memory, not the actual sector on the diskette.  To save

the changes that you made, use the [W]rite sector command.  Be very

careful when writing to the diskette!

That is all that is required to know for basic diskette editing.  But

there is much more to Diskette Editor--read on!

SCANNING A DISKETTE

When a diskette is formatted, 40 (decimal) tracks are laid out on it.

They are numbered from 00 to 27 (hexadecimal).  Within each track there

are sectors.  In the earlier versions of DOS (1.x) there were only 08

sectors in each track.  With the later versions there are 09.

Diskettes also have two sides (unless you have one of the older

single-sided diskette drives) which are numbered 00 and 01.  Some

software comes on single-side diskettes which means that only one side

of the diskette is being used.

The above paragraph describes an ordinary diskette.  But there are many

"copy protection" schemes that entail using abnormal formats.  A number

of things can easily be done to copy protect a program.  Giving sectors

weird numbers is popular.  The sector numbers can range from 00 to FF.

Adding an extra track is also commonly done.  Sector sizes can also be

changed from 512 to 128, 256, or 1024 bytes (decimal).  But the most

popular of all these tricks is a combination of them.  DOS (all

versions) expects there to be 40 (decimal) tracks of 08, or 09, sectors

of 512 (decimal) bytes.  If this is not so, it reports a read error and

no data is transferred.  When DISKCOPY copys a disk, it produces a

normally formatted diskette.  When the "copy protected" program first

runs, it checks to see if the diskette it was loaded off of has the

extraordinary format, if not it was a DISKCOPYed copy so it aborts.

The Sca[n] command is used to find these abnormal formats.  When its

window opens, it asks you the starting track.  Enter the desired track

number to start on or 00 to start with the first.  It then asks you for

the ending track.  Enter it or 27 for the last.  Next comes the lowest

sector number to search for, then the highest.  Diskette Editor's scan

command will the proceed to look, starting from the starting track, for

all the sectors of any size in the range entered for the sector

numbers, to the ending track, reporting anything it finds.  To read in

an unusually sized sector, see the BYTES PER SECTOR option under

DISKETTE PARAMETER TABLE.

DISKETTE PARAMETER TABLE

Between the two windows there is the diskette parameter table and it is

labeled as so.  This table controls how the diskette drives will act.

Most of the values in the table should not be changed unless you know

what you are doing.  But there are a couple that will be very helpful.

Here is the table:

Diskette Parameter Table

Byte #  Default Value       Description

0    DF              1st nibble stop rate for drive head

2nd nibble disk head unload time

1    02              1st nibble disk head load time

2nd nibble DMA mode select

2    25              Wait time before motor off

*3    02              Bytes per sector

*4    09              Highest sector number on track

DISKED  page 3

5    2A              Gap length between sectors

6    FF              Data length value

7    50              Formatting gap length

8    F6              Formatting fill byte

9    00              Head settle time

10    02              Motor start delay

Only table values with asterisks should be modified unless you are

familiar with diskette drive operations (i.e. you understand the

descriptions) otherwise you can damage your diskette drives.  Two

things you should note are some of the default values vary with DOS,

and in Diskette Editor the descriptions are abbreviated.

To modify a value in the table, press the highlighted number next to

the one you want.  The last option does not have a "10" next to it,

instead it has an "X".  To modify it press the [X] key.  The original

table is restored when you exit from Diskette Editor.

BYTES PER SECTOR (3) -- this is the one you will want to play around

with the most.  Changing this will enable you to read in some "copy

protected" sectors.  This controls the number of bytes per sector.  If

you enter a 00 there is 128 bytes per sector, a 01 there are 256 bytes

per sector, a 02 (normal) there are 512 bytes per sector, or a 03 1024

bytes per sector.  Any other number is invalid, but Diskette Editor

will not warn you of this.  To find out how to locate unusual sized

sectors, see the Sca[n] command.  To read an abnormally sized sector

reported by the Sca[n] command, the current sector size must match the

size revealed.  To find out how to create your own unusual sized

sectors, see the [F]ormat command.

HIGHEST SECTOR NUMBER ON TRACK (4) -- this is used by Diskette Editor's

[+] and [-] (see below) commands.  It controls when the current sector

number will wrap back to a 01.  Under DOS 1.x the default value is 08

(8 sectors per track), and under DOS 2.x and 3.x it is a 09 (9 sectors

per track).

COMMANDS OVERVIEW

[ESC]--cancels a command or input at any time.

[H]exadecimal--switches the current mode to hexadecimal.  The

highlighted cursor will move to the left window if it was in the

right. [M]odifications will be in hexadecimal.

[A]SC II--switches the current mode to ASC II (text mode).  The

highlighted cursor will move to the right window if it was in the

left. [M]odifications will be in ASC II.

[M]odify--lets you change the byte being highlighted by the pointer.

If the mode is ASC II, type the appropriate character otherwise type

in the desired hexadecimal value.

[R]ead--reads in the current sector.  The information line

(bottom-most) tells where the current sector is located.

[W]rite--writes the sector data stored in the computer's memory to the

diskette.  Be careful to write the sector to the same place you read

it from unless you are specifically copying the data.  A yes/no

warning is given before the data is written.

[V]erify--verifies that the current sector is readable.  The sector

data is not transferred to the sector data buffer in the computer's

memory.

[D]rive--toggles between drive A and drive B. Note: Diskette Editor

does not support a hard disk, so the drives are limited to A and B.

[T]rack--allows you to type in a track number for the current track.

If Auto-Read is on, the current sector is read from the new track.

Entering a track number greater than 27 results in a warning with an

"Are you sure?"

[S]ector--allows you to enter a new sector number for the current

sector number.  Valid sectors numbers are from 00 to FF, although

normal sector numbers range from 01 to 09.  If Auto-Read is on, the

new sector is read.

DISKED  page 4

[+]--the plus key increments the current sector number by one and reads

it from the diskette if auto-read is on (see A[u]to Read).  If the

current sector number is the highest sector number on the track,

then the current sector number will be 01.  (To change the highest

value see HIGHEST SECTOR NUMBER ON TRACK under the DISKETTE

PARAMETER TABLE section).

[-]--the minus key decrements the current sector number by one and

reads it from the diskette if auto-read is on.  If the current

sector number is 01, the current sector number becomes the highest

sector number on the track.

[<]--the less than sign key decrements the current track number by one

and reads in the current sector off of the new track if auto-read is

on.  If the current track was 01, the current track becomes 27.

[>]--the greater than sign key increments the current track number by

one and reads in the current sector off of the new track if

auto-read is on.  If the current track was 27, the current track

becomes 01.

S[i]de--flips the side of the diskette being edited.  If Auto- Read is

on, the sector from the opposite side is read.

A[u]to Read--toggles Auto-Read on/off.  If Auto-Read is ON, all

commands that change the position of the current sector ([+], [-],

[T]rack, etc.) will automatically read in the resulting sector.  The

default value is ON.

Clr Key Buffer[:]--(Clear keyboard type-ahead buffer)--toggles the

flushing of the keyboard type ahead-buffer on/off.  If OFF,

characters may be typed before a prompt for them is given.  If ON,

those characters are flushed.

[ALT-R]--[hold ALTERNATE key then press "R"]--Resets the disk system.

This command should ordinarily not be needed.

[F9]--changes the highlight text color.

[F10]--changes the normal text color.

Mas[k]--controls the ASC II display.  Normally non-text characters are

displayed as periods.  Diskette Editor determines if a character is

a valid text character by checking to see if it is above a low value

and if it is below a high value.  Mas[k] lets you change these high

and low values.  (Low value of 00 and a high value of FF disables

the character masking).  After you change the lower and upper

bounds, you then can modify the character used as the mask

character.  (Normally a period).

Fil[l]--fills the sector data buffer with a specified byte.  This does

not change the sector on the diskette unless you use the [W]rite

command.

S[e]arch--searches a user specified range on the diskette for a pattern

of hexadecimal or ASC II (text) data.  Enter the data in either

hexadecimal or ASC II (as you indicate).  Then enter the starting

and ending track and sector numbers.  Diskette Editor will then ask

if you want to search both sides, meaning side 0 and side 1 of the

diskette.

To Diskette Editor a match is when the data is exactly the same.  So

"Are you sure?" does not match "ARE YOU SURE?"  There is one case when

Diskette Editor treats a partial match as a whole match.  This is when

Diskette Editor is scanning the edge of the sector data buffer.  If you

have it search for "password" and at the end of its buffer there is

"passw" a match would be found.  Quite often, a sector ends with the

same byte as the data you are searching for begins.  Diskette Editor

treats this also as a match, although it might not be.  Be wary when a

match occurs at offset 1FF (for a sector size of 512 bytes).

When a match is found, you have the option of searching for the next

occurrence of the search data, or view the data previously found.  To

view the matched data just found, press the ESC key, otherwise press

any other key to continue the search.

DISKED  page 5

WARNING WARNING --POTENTIALLY DANGEROUS COMMAND-- WARNING WARNING

\/

[F]ormat--formats the current track to your specifications.  (For a

complete description of a diskette see SCANNING A DISKETTE above).

The drive and track number that will be formatted are displayed.

You are then asked for the number of sectors on the track.  This can

range from 01 to FF, but only low numbers will actually work; if the

number is too big the sectors will overlap on the track.

Diskette Editor will then ask information about each sector.  The

sectors are stored sequentially on the track, but the they can be

called by any number desired.  Diskette Editor displays the physical

sector number and asks what logical sector number you want it

called.  The logical number can range from 00 to FF.  In DOS 1.x,

there are 8 sectors per track and they number from 01 to 08.  In DOS

2.x and 3.x there are 9 sectors per track and they number from 01 to

09.  If you give your sectors any other numbers DOS will be unable

to read them.  This is used in some "copy-protection" schemes.

Diskette Editor will ask you the size of the sector.  The sector

can be 128, 256, 512, or 1024 bytes in length.  Normally sectors are

512 bytes.  When prompted, enter a 0 for 128 bytes, a 1 for 256

bytes, a 2 for 512 bytes, or a 3 for 1024 bytes.  This is another

"copy-protection" method used by some companies.  Most often a

combination of weird sector numbers and sizes are used.  (See the

Sca[n] command to find out if a track contains any of these weird

combinations).

After all the information about the track is entered, a final yes/no

warning message is displayed.  If you answer yes, the track is

formatted.  This DESTROYS ALL THE DATA that was on that track.  Be

very, very careful with this command.  I recommend that you never

use this command unless you have a good reason to.

Sca[n]--see SCAN A DISKETTE above for a complete description of the

Sca[n] command and disk formats.

[C]onvert--converts, for your convenience, a number from either

hexadecimal or decimal to its equivalent hexadecimal or decimal form

on the display.

[Q]uit--exits Diskette Editor.

Diskette Editor warns you before it does anything potentially

dangerous, but be very careful when you use it; you can accidentally

lose an entire diskette's data.

Diskette Editor belongs to the U.S.S. crowd.  (User Supported

Software).  You are given a trial period of two weeks in which you can

use it at no cost.  If you use it after the trial period you must

register your copy for $30.00.  If you register or not you are

encouraged to distribute copies of Diskette Editor as long as there is

not a disk fee of more than $10.00.  My name and address for

registration, comments, or bug reports is:

Paul Troiano

531 Village Road West

Princeton Jct, NJ 08550

Diskette Editor has no warranty implied or expressed.  In no event will

the author be held liable for damages caused by the use of this

program.

------------------------------------------------------------

This disk copy provided as a service of

The Public (Software) Library

the software library of

The Houston Area League of PC Users

For a copy of the latest monthly software library newsletter

and a list of the 600+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705

Houston, TX 77235-5705

(713) 721-6104

Newsletter subscriptions are available for $12 a year.