Command QDR                                                 Version 2.5

-----------                                                 Nov 2, 1986

Purpose

-------

Re-format a floppy  disk quickly, optionally  with a new  volume

label, deleting all files and sub-directories.

Format

------

QDR [d:][label]  [/F][/1][/3][/4][/8] [/N] [/V]

Remarks

-------

The d: parameter supplies the drive letter, A to K. If no  drive

letter  is  supplied,  A  is  used.   A  fixed  disk  can not be

re-formatted.

You may supply a volume label (up to 11 characters)  immediately

following the drive letter.  For example:

QDR b:scratch

The /N option suppresses  the "press ENTER to  continue" prompt.

Otherwise, press  the enter  key to  reformat the  disk, or  the

ESCape key to cancel.

Use the /F option to force a full format of all tracks.  The /1,

/3, /4, and /8 options apply to a full format.

Bad clusters remain marked as bad unless a full format is

used, then clusters are marked as unusable if they can not be

formatted.  Use the /V option to receive a display of the

cluster numbers that are marked as not usable.

The DOS error level is set to 0 for successful completion, or to

1 if any errors occur.

Options

-------

/F - forced format; uses BIOS to format and verify each track

/1 - forces single sided format, SSSD/8 or SSSD/9

/3 - forces DSQD format on 720K capacity drives

/4 - forces DSQD format on 1.2 mb drives

/8 - forces 8-sector format; for SSSD/8 or DSDD/8

/N - no prompt; skips the "Press ENTER" message and reply

/V - verbose messages; displays a message for each cluster

that is marked as unusable

Formatting

----------

To perform a full format of a disk sectors, use the /F  options.

It is recommended that you use DOS FORMAT since QDR is not  much

faster when doing a full format.  Both mark bad clusters.

You may suppy /1 to force  SSSD format, /4 for DSQD (1.2mb),  or

/3 for DSQD (720kb), or /8 for eight-sectors (SSSD or DSDD).

Unless  /F  is  used,  the  diskette  must  have been previously

formatted.  The allowable diskette  formats are: DSQD (1.2mb  or

720kb), SSSD/8, SSSD/9, DSDD/8, and DSDD/9.  When information is

available for other drive types, they will be added to the  data

in the internal device table.  Your system hardware and software

must support the device type.

Restrictions

------------

(c) Copyright Vernon D. Buerg 1986. All rights reserved.

May be copied  and distributed in  its original form  only.  For

private, personal use.  Not for sale or hire.  Use this  program

at your  own risk.   No warranty  is given  and no  liability is

assumed.