Following instructions were furnished for use with

DISSAM.COM

Dissassembler commands etc.

INstruction   do we want instructins to proceed each interogation?

Object file input (is this coming from a disk?)

OFFSET?  offset will generally be 0; is amount required

for program to wrap from true location to 0100 (or wherever

you loaded it;  100 is usual in cp/m stuff)

reader is always NO

format-hex-ascii  will not come up; only for readers

offset  same as above; entered twice

start and end of where you have it  (0100 and endadd from load

or SID;  inspect end of the thing to see, and put that in.

source file output  usually yes

known data areas    if you say yes, give start, end, hex for

first;  will prompt again; continue until you say no.

after you get an output, you inspect; if you find junk or ascii

stuff, you declare those to be data areas.

The first task is to define data areas.  The ASCII listing is

useful in doing this.  You can get this with the ascii study in

SID, through dumps, etc.

A series of meaningless move register to register instructions

are probably ASCII since most printable ascii is 40H to 7fH

Using the cross-reference listing, locate areas that are

referenced by three or more instructions.  They will usually

be+

i/o routines or vectors

small utility routines

frequently used data

2.  Once you have defined the data areas and isolated the

i/o and utility routines, edit the generated source by

inserting comments, and assemble.  Almost without exception

the UNDEFINEDs will denote one of two things:

Dynamic instructin modifications

reference to an undiscovered data area

After the second or third disassembly it's all up to you...

##############################