B    Instructions for CURVE Program

This program derives the linear, exponential, logarithmic, and power

equations to fit a set of input data points x(I),y(I).  All the equations

are converted to the linear form Y=A+BX, and the linear form is fit to

the data by the method of least mean squares.

Type     Original Equation   Linearized Equation    Conversion Equations

Linear          y=a+bx             y=a+bx               Y=y,X=x;a=A,b=B

Exponential      y=ae^bx         ln y=ln a + bx        Y=ln y,X=x;a=e^A,b=B

Power          y=ax^b         ln y=ln a + b ln x   Y=ln y,X=ln x;a=e^A,b=B

Logarithmic     y=a+b ln x         y=a+b ln x           Y=y,X=ln x;a=A,b=B

A=($Y$X^2-$X$XY)/(n$X^2-($X)^2)      B=(n$XY-$X$Y)/(n$X^2-($X)^2)

where the "$" denotes "summation" over the n values.

The program may be considered as operating on two levels.  Level 1

is the data entry part of the program, and Level 2 provides regression

analysis and curve plotting.  Level 1 commands are shown in Table 1 and

are active at all times, including during Level 2 operation.  Level 2

commands are shown in Table 2 and are active only after the data has been

entered and the DONE command has been given.  A Level 1 command or an un-

recognizable entry made in Level 2 will return operation to Level 1.

Commands may be entered at any time by typing in the command followed by

the ENTER key.  Ten of the commands may also by entered by the special

function keys F1 through F10.  Commands may be upper case or lower case,

but upper and lower case letters should not be mixed in the same command.

At the beginning of the program the user is prompted to enter the

maximum number of data points.  The number entered should be large enough

to allow for the maximum number of data points that might need to be

entered, because changing it later will erase any data already entered.

Pressing the ENTER key with no number entry will default the maximum num-

ber of points to 25.  The number may be changed at any time by entering

the DIM command.

The user is then prompted for an optional title.  It may be any

combination of lower case letters, upper case letters, numbers, and

keyboard characters.  The title will be used on the printout.

A data point is entered in the form X,Y followed by the ENTER key,

where X and Y are the actual numbers, separated by a comma.  A data point

may be deleted by the DELETE or DEL command.  If the DELETE command alone

is entered, the user will be prompted for the data point to be deleted.

The data point may be specified by entering the actual data values in the

format X,Y or by entering the subscript number (no comma).  The DEL

or DELETE command may also be followed immediately by the data point

or the subscript before the ENTER key.  After the deletion, the array

subscripts will be renumbered.  The array of data showing the subscripts

may be relisted at any time by entering the LIST command or the F1 key.

Example:  To delete X(7)=100,Y(7)=8.9, enter DEL7 or DEL100,8.9.

B

After all the data points are entered, type DONE or hit the F2 key.

The linear, exponential, power, and logarithmic equations will be displayed,

showing the calculated equation coefficients and the correlation

coefficients.  Then enter the LIST command or hit the F1 key and a table

will be displayed showing all the input data and the values of Y calculated

from each of the four derived equations for each X value of input data.

To calculate values of Y for a new X, simply type in the value of X (no

comma) followed by the ENTER key.  Y will be calculated for each of the

four equations and added to the table listing.  This may be repeated as

many times as desired.  To print the equations, correlation coefficients,

and listing of input data and calculated values to the printer, enter the

PRINT command or hit the F10 key.

Several screen plotting options are available.  The PLOT command

(key F5) will provide plotting of three of the curves in three colors

in the medium resolution mode.  The user chooses which three of the four

equations are to be plotted according to the menu prompt.  The user may

also specify the maximum and minimum values of X and Y to be plotted by

entering Y in response to the prompt.  For any other response to the

prompt, such as just hitting the ENTER key, the program will choose the

maximum and minimum values so as to include all of the input data points.

In addition to the three curves, the plot will show the input data points

as small squares on the screen.  There are no grid lines on the display

other than the X and Y axes.  The numbers in the upper left, lower left,

and lower right corners of the screen are the values of X and Y at the

edges of the screen.  The LIN, EXP, POW, and LOG commands (keys F6, F7,

F8, and F9 respectively) are similar except that the plot is a single

equation in the high resolution mode.  After the equation(s) are plotted

the program will accept further Level 1 and Level 2 commands while the

plots are still on the screen.  The user can erase the plot screen and

return the display of the commands assigned to the special function keys

by hitting the ENTER key once.  Hitting the ENTER key again will return

the program to Level 1.  If the user accidently gets back to Level 1 by

hitting the wrong key, the DONE command will return the program to Level 2.

Note that additional data points can be added without destroying the

existing input data by entering X,Y as before.  To start over with a

completely new set of data, enter RESET (key F3) followed by the ENTER

key.

All calculations are carried out in single precision unless the DBL

command is entered.  BASIC does not provide double precision exponential,

power, and logarithmic functions, so only the linear equation is shown

with double precision coefficients.  However, the double precision mode

still improves the accuracy of the other equations by preventing further

accumulation of errors in the arithmatic calculations.  Calculations

carried out in single precision can easily reduce the accuracy of the

calculated coefficients to only 3 or 4 digits.  The DBL command may be

entered anytime, but it destroys all previously entered data.  The SNG

command returns to single precision calculations.

After a command is completed or a data entry is made, the program

displays the "?" prompt and is ready to accept a new command or data entry.

The program runs continuously once started; it may be stopped by CTRL+BREAK

or by turning the machine off.

TABLE 1:  Level 1 Commands

LIST             Lists all input data points in the format

X(I)=number,Y(I)=number.

DELETE or DEL    Delete a previously entered data point.  The user is

prompted to enter the data point to be deleted.  The

data point may be specified by entering the the X and Y

values separated by a comma.  The data point may also be

specified by entering a single number (no comma) rep-

resenting the array subscript shown in the data list.

DELETE x,y       Delete a previously entered data point.  The x and y after

or DEL x,y       the DELETE command are the actual numbers to be deleted.

Both values must be given and must be separated by a

comma.  A number without a comma would be interpreted as

a subscript.  The space after the command is optional.

DELETE i         Delete a previously entered data point.  The i after the

or DEL i         DELETE command is the array subscript number shown in the

input data list.  To display the current input data list,

enter the LIST command (key F1).  The space after the

command is optional.

DONE             Calculate the equation coefficients and correlation

coefficients for each of the four types of equations

and display the resulting equations and correlation

coefficients.  Illegal data points are ignored in the

calculations.  (For example, 0 and negative values of

X are illegal in the logarithmic equation, so the

logarithmic curve fit is calculated from the legal input

data points only.)  Level 2 commands are accessable after

the DONE command.

RESET            Erase all previously entered data and start over.

DBL              Erase all previously entered data and start over using

double precision arithmetic.  Since all BASIC functions

are single precision, the results for the exponential,

power, and logarithmic equations are limited to an

accuracy of six digits.  However, the calculations are

are double precision, which helps prevent the accumulation

of roundoff errors.

SNG              Erase all previously entered data and start over using

double precision arithmetic.  Note that accumulated

roundoff errors can reduce the effective accuracy of the

end results to only 3 or 4 digits or even less.

DIM              Erase all previously entered data, reset the maximum

number of data points, and start over.

TABLE 2:  Level 2 Commands

LIST             List a table of X and Y input data values and calculated

values of Y for each X using the four derived equations.

For trend line analysis, key in a new value of X (no

comma) followed by the ENTER key, and it will be added

to the table listing along with the calculated values of

Y for each of the four equations.

PLOT             Plot 3 of the 4 derived equations on the screen in 3

colors in the medium resolution mode.  The user selects

the 3 equations to be plotted in response to a menu

prompt.  The user may also specify the maximum and

minimum end points of the plot by entering Y in response

to the prompt; for any other entry, such as just hitting

the ENTER key, the program will choose the end points so

as to include all of the input data points.  The input

data points are indicated by small squares on the plot

screen.

LIN              Plot the derived linear, exponential, power, or

EXP              logarithmic equation on the screen in the medium

POW              resolution mode.  The user may specify the maximum and

LOG              minimum end points of the plot as described above.

PRINT            Print the title, the four derived equations, the

correlation coefficients, the input data values, the

values of Y caluclated from each of the four equations

for each input X data value, and the values of Y

calculated from the equations for any new values of X

that were entered.