TIMESET VERSION 4.20 MANUAL

November 17, 1988

VERSION 4.20 of TIMESET.EXE

A program that sets your computer to Naval Observatory Time

by Peter L. Petrakis, Ph.D., M.P.H.

Life Sciences Editorial Services

Annapolis, Maryland 21401

Scientific and Technical Editorial Services, Computer Programming

Serving Government and the Private Sector Since 1975

TIMESET Version 4.20, like its predecessor versions, is donated to the public

domain.  This version is very similar to Versions 4.00 and 4.01 except for many

valuable enhancements and eradication of some minor bugs:

1.   Timesetting accuracy is greatly improved by assembly language

modules for extremely fast data string parsing and error checking.

2.   Faster, more dramatic graphics made possible by MicroHelp, Inc.'s

new QB/PRO5 module library (sheduled for release in January 1989).

3.   Except for low-volume ticks to mark seconds on continuous

time displays, this version makes absolutely no beeps or other

sound effects unless you specifically ask for them with a

command line switch.

4.   This version has an interrupt-driven time and date display

that is concurrent with dialing, data gathering from the Naval

Observatory, and other program activities.  Thus a running display

of the current time is always visible.

1

5.   Countdown to zero seconds now occurs offline with computers that

don't really need a countdown (i.e., non-AT&T, non-Olivetti), with

no significant loss of accuracy.  This feature has been incorporated

to cut down on excessive use (and abuse) of the Naval Observatory's

service and to reduce long distance phone bills.

6.   Help screens are back inside the program rather than in a separate

file.  This was made feasible by some new MicroHelp, Inc. assembly

language modules (in QB/PRO5, scheduled for release in January 1989)

that allow screens to be stored in a compressed form and expanded

for display when needed.  The screen data compression feature allows

the screens to be kept inside the program at a tolerable cost

in program size.  Putting the help screens back inside the program

also eliminates a problem reported by some users that TIMESET

was not able to find its help screen files.

7.   A bug in the TIMESET configuration program CONFIGTS.EXE that

caused erroneous configuration for universal time has been

fixed.

8.   Toggling between daylight and standard time is no longer followed

by program reloading from the disk.  Instead, the simultaneously

altered program copy in RAM is re-run.  This eliminates the problem

of getting the reconfigured disk program into memory when a large

disk cache is present.  Also, the mysterious extra second gained

during the toggling procedure has been eliminated.

Contents of this Download Package

The download package TIMSET42.ARC (TIMSET.ARC on Compuserve) should contain

the following files:

TIMSET42.DOC      What you are reading now.

TIMESET.EXE       The main program, TIMESET Ver. 4.20

CONFIGTS.EXE      Version 1.1 of program for configuring

TIMESET Ver. 4.20

CLKDRV.SYS        A clock driver by Tom Strickland for

automatically passing time and date

between the DOS clock/calendar and

nearly all real-time clocks except

the AT CMOS.  It goes in your

CONFIG.SYS file if your RT clock is

not an AT CMOS clock or clone.

REALTIME.COM      Strickland's program for reading time

and date from a non-CMOS real-time clock

while in DOS.

2

DOSTIME.COM       Strickland's program for reading the DOS

clock/calendar without having to use the

TIME and DATE commands.  Displays time

and date together without prompting you for

changes.

RTCLOCK.COM       Strickland's program for transferring time

and date between DOS and a non-CMOS

real-time clock, in either direction.

If you upload TIMESET, please be sure all of these files are included.

I encourage you to upload the package promiscuously.  However, I would

appreciate it very much if you would also ask sysops to minimize the perpet-

uation of obsolescence by getting rid of any previous versions of TIMESET.  I

especially hope that this version, with offline countdown, will replace all

earlier versions that are still out there.  Too many people have been tying up

the Naval Observatory's valuable time service by doing online countdowns when

their computers don't require it.  Version 4.20 was written to eliminate that

problem, but it won't do much good unless it's disseminated along with requests

to sysops to scrap previous versions.

You are forbidden to distribute any copy of TIMESET and its companion files

that you have modified.  You are also forbidden to charge money for these

programs.  They are supposed to be free.

Introduction

Over a year has gone by since I uploaded the first version of TIMESET, a

program to dial the Naval Observatory's digital time service in Washington,

D.C. and get highly accurate time and date information to set the calling

computer's clock.  I was so naive about the problems of making complex programs

compatible with other computers that I didn't even bother to give that first

version a number.  Let's call it Version Zero.

To my surprise, Version Zero brought a flood of telephone calls and CIS/BBS

messages from all over the country.  The messages were of two kinds:

enthusiastic reports and problem reports.  Either way, this was evidence that

there was quite a lot of interest and that the program was worth improving.

For the first few months, the improvements consisted mainly of trying to

eliminate major bugs and solve problems of compatibility with the incredible

variety of systems out there, especially modems that vary from superb to cheap

and junky.  By November 1987 I was up to Version 3.4.  With that version the

problem reports dropped precipitously.  This could mean either that I had

solved the major problems or that people had simply grown bored with the whole

idea.  I like to think the first reason is the main one.

Anyway, with the serious problem reports down to a manageable level, I began

looking for ways to enhance the program.  Around December 1987, I got a call

from Tom Strickland, of Alexandria, VA.  Tom, a superb programmer who is expert

in several languages, volunteered to write a real-time clock driver and an RT

3

clock reader/writer that could eliminate the clumsy situation where the

numerous manufacturers of non-CMOS-type RT clocks use essentially the same chip

but write different software for it.  Tom also wrote a module linkable to

TIMESET so the program can talk to the RT clock the same way his driver and

other clock setting and reading programs do.

Tom's software, in addition to the module already in previous versions for

setting an AT CMOS clock, means that TIMESET can now set -- without clumsy

configuration routines -- virtually any real-time clock in addition to the DOS

clock/calendar.  Before Tom wrote these programs, it was a nightmare trying to

configure TIMESET to use as many of those RT clocks as possible, because I had

to figure out a way for the program to use each manufacturer's software through

a shell.  With some RT clock software it was impossible, because the software

would not accept input from the command line and insisted on prompting for it,

one item at a time.  By the time the prompts could be answered, any advantage

of getting time from the Naval Observatory would be lost.  Time flies when

you're diddling with prompts on a computer screen.  All that was solved

starting with Version 4.00, because Tom's supporting software makes all those

different brands act like a single brand that TIMESET can address directly.

This means, however, that you'll have to put aside the software that came with

your RT clock and use Strickland's instead (I'll explain this more later).

(Yes, yes, I know -- sooner or later, somebody is going to tell me they have

an RT clock that neither Tom's programs nor TIMESET can talk to.)

Before I proceed, I want to mention something about the phone number you will

see on a couple of the screens of TIMESET.  That phone number has been in all

previous versions of the program.  It comes right after the words, "Life

Sciences Editorial Services."   It's there for advertising purposes, and it

ought to be clear that it is my phone number, not the Naval Observatory's.  It

ought to be, but I have had to learn to live with calls from people trying to

get Naval Observatory time by dialing my number.

So, let me say it up near the top so it won't be missed:

IF YOU WANT TO CALL THE NAVAL OBSERVATORY'S DIGITAL TIME SERVICE,

THE NUMBER IS: 202-653-0351.

IF YOU WANT TO TALK TO ME, THE PHONE NUMBER IS: 301-261-1370.

There are no time signals at this number.

More About Features of Version 4.20

In addition to introducing standardization in dealing with RT clocks other

than the AT CMOS, TIMESET Version 4.2 is much faster and much more responsive

than earlier versions, and vastly easier to configure.

Among the features:

4

o    In addition to greatly enhanced graphics, Version 4.20 provides

a continuous display of the current system time and date on every

screen, even while modem intialization, dialing, and data collection

are occurring.  This is accomplished by a concurrent processing

feature that reads and displays the system clock/calendar at

every tick of the computer timer (18.2 times/sec) even while other

program activities are in progress.

o    As with Versions 4.00 and 4.01, configuration is handled by a

separate program that lets you use function keys to select

time zone, daylight/standard time, serial port, immediate

setting or countdown to zero seconds, telephone access codes,

etc.  Configuration also includes setups for color (CGA) and

monochrome.  This program allows configuration to be done

in seconds.  Switching between daylight and standard time

can still be done within TIMESET itself, as before, but version

4.20 eliminates certain toggling problems caused by the presence

of a disk cache or the WordPerfect Library (explained below).

o    A bug in CONFIGTS.EXE that caused an error during configuration

for universal time has been fixed.  (The revised version of

CONFIGTS.EXE is 1.1; the previous version had no number.)

o    No separate configuration data file is needed to run TIMESET

version 4.20.  Instead of writing your configuration data into

a separate file, the configuration program writes the data

directly to TIMESET itself.  However, you can override the

configurations written to TIMESET at any time by loading the

program with a configuration file that you can write with an

ASCII editor.  Just name the file on the command line, and

TIMESET will use information in that file to replace any or

all of its own built-in configurations.  This feature is

included because some corporate and government institutions

have highly complex procedures for making a telephone call.

Since it's not feasible to accommodate all dialing possibilities

in a program like TIMESET, the unfortunates who have to work

with such systems can at least attempt to write their own dialing

"script" for TIMESET.

o    The sending of initialization code and dialing strings to

the modem can be accompanied by sound.  This is mainly to

reassure those who have to "fly blind" with internal modems

that the program really is trying to send something to the

modem.  Unlike previous versions, however, Version 4.20 makes

hardly a sound unless you ask for sound effects with a

switch on the command line.

o    Versions 4+ really can use communication ports 1 to 4.

Previous versions made that claim, but this was largely due

to ambiguity in Microsoft BASIC manuals about which ports

5

can be used and my own inability to check the action with

ports 3 and 4 (since I only have 1 and 2).  Since error

trapping and recovery have been in the program from the

very beginning, I never once saw an error message when I

tried to open ports 3 and 4!  Versions 4.+ use ports 3 and

4 by making port vector changes that fool BASIC into

thinking it's using port 1 when it's really using port 3,

or thinking that it's using port 2 when it's really using

port 4.

o    Two methods are now used to hang up the modem.  First, the

program tries dropping DTR, a method that works surely and

instantly with most modems.  However, TIMESET then checks

for a carrier on the line, and if one is still present it

sends the escape code and "ATH" up to 10 times, with pauses

and checking for carrier each time.  If a carrier is still

present after 10 tries with +++(pause)ATH(pause)(check), a

warning is printed on your screen (it now seems very

unlikely that the warning would be needed).

o    Breaking out of the program is now done easily at any time

by pressing a specially assigned key (no more control-

break).  Whether the program ends normally or because of

action on your part, the last thing it does is reset the

communication port vectors to their normal values, so your

regular communication program won't get confused.

o    Response to a command to cancel dialing in progress is now

fast and sure, and canceling can be done even in the middle

of dialing--something earlier versions could not do.

o    Thanks to significant improvements that came with

Microsoft's Basic Compiler Version 6.0 (BC6), the program

is much more compact.  It also incorporates a host of

assembly language modules from MicroHelp, Inc.  That's not

to say it's tiny (it's in compiled BASIC, after all).  Version

4.20 takes about 124k on disk, but that's down from about

140k in version 3.42.  It takes  about 142k of RAM, but that's

down from about 210k in v. 3.42 and it's less RAM than many

communications programs use.  (More about program size later

in this document.)

o    Help screens have been moved back into the program in Version

4.20.  As noted above, they are built into the program in a

compressed form and are expanded for display when needed.

o    Version 4.20 recognizes the following modem result codes:

CONNECT, NO CARRIER, NO DIALTONE, and BUSY.  (NO DIALTONE is

the only result that was not recognized by previous versions.)

6

Configuration

Configuring TIMESET Version 4.2 is very simple and very fast.  The elaborate

rigmarole of interminable prompting for information about your system is gone,

and with it a lot of the decision-tree coding that contributed so much to

program size in previous versions.

First, be sure the configuration program CONFIGTS.EXE and TIMESET.EXE are on

the same drive and directory before you start configuration and that you have

switched to that drive and directory.  Also be sure that it's version 4.+ of

TIMESET and not some earlier version.

When you run CONFIGTS.EXE, you will see a screen divided up into several

rectangular areas, with each area dealing with a different aspect of

configuration.  Your configuration choices are made by pressing function keys

F1 to F10 for an item on the list within each rectangle.  As soon as you make

your choice from the menu in one box the next box becomes highlighted.

A separate box is shown for the following configuration items:

Time zone:     Eight times zones in the western hemisphere are

named (F1-F8), but any time zone in the world can

be configured by using the hours offset from

Greenwich under standard time (F10).  You can also

set your computer to universal time if you want (F9).

If you opt for the hours offset method (F10), you

will be prompted for the offset.  A help window

pops up to show you how to write it.

Season:        Daylight (F1) or standard (F2) time for the time

zone chosen on the previous box, unless you chose

universal time.  Daylight and standard do not

apply to universal time, because it does not

change with the seasons.  If you chose universal

time on the previous box, this box will be

skipped and the word "NULL" will be put in the

configuration for time season.

Port:          Press F1-F4 to choose among communication ports

1-4.  (Caution: if you have a mouse or other device

that normally uses communication port 1 or 2, there

could be conflicts when TIMESET changes the vectors

to those for 3 and 4.)

Set clock:     There are two ways to set the time with TIMESET:

(a) as soon as the first clean data string is

received (F1) and (b) after a countdown to zero

seconds (F2).  AT&T and Olivetti computers require a

countdown to zero seconds.

7

Either F1 or F2 will be accepted during configuration,

but TIMESET checks the computer's ROM to see if it's

an Olivetti or AT&T.  If it is, the program will choose

the countdown method no matter what you chose during

configuration.  It can't be any other way; if

TIMESET immediately set the time on one of these

computers, their bad habit of dropping seconds to

zero whenever you change the DOS clock/calendar

could cause the time to be off by as much as 59

seconds.  The countdown method has changed with

Version 4.20 of TIMESET.  It now takes place offline

unless your computer is an AT&T or Olivetti (see

Offline Countdown section, below).

RT clock:      TIMESET can set three different types of computer

clocks: (F1) the volatile DOS clock/calendar that

every MSDOS computer comes equipped with; (F2)

the AT CMOS real-time clock/calendar; and (F3) a

type of RT clock/calendar that comes under many

different brand names but is virtually always

based on the same clock chip or family of clock

chips.  Option F4, labeled Driver, can be used if

you have one of those clocks and Strickland's RT

clock driver is installed in your system (I'll

have more to say about that later).

The non-CMOS type of RT clock has been a real

headache for me in developing TIMESET, because

every company that sells RT clocks based on that

common clock chip writes its own software for it,

and each one writes it differently.  For example,

some will accept time and date on the command

line, and some will pass time from the DOS clock/

calendar merely by being run with an appropriate

command line parameter, but others insist on

prompting for both time and date, then asking

questions about whether you really, truly, honestly

want to set that time or date.  A program like

TIMESET, which has to plug the time into the

computer as fast as possible, can't waste time

answering silly questions raised by another program.

However, as I mentioned in the Introduction, Tom

Strickland has written some software for this

type of RT clock that allows TIMESET to "talk" to

them all in a single common "language."  In

effect, Tom has created an Esperanto for the

world of non-CMOS RT clocks.  A module he wrote

is incorporated into TIMESET for directly setting

that type of RT clock, whatever its brand name.

8

So, if that's the kind of clock you have, select

F3 ("Other RT").

You will have to put your regular RT software

aside if you use this method, because it will

very likely return the wrong year if the clock

has been set by Strickland's module in TIMESET.

However, a substitute is at hand: Strickland's

RTCLOCK.COM program.  Just use that program to

set and read your RT clock from now on, instead

of the software that came with the clock.

(Tom's programs are discussed in detail in a

later section of this document.)

There is one exception to selecting F3 if you

have one of these "other" RT clocks.  If Tom's RT

clock driver, CLKDRV.SYS is in your CONFIG.SYS

file, it will automatically pass the time and

date from DOS to the RT clock whenever the DOS

clock is changed.  TIMESET always sets the DOS

clock/calendar, so if you have installed the

driver, you must select F4 (or F1-- they are really

the same here) and let Tom's driver make the transfer

to the RT clock, which it does any time DOS time and

date are changed.  In other words, you must configure

TIMESET to adjust only the DOS clock/calendar and let

the Strickland RT clock driver handle the rest.

You should also select option F4 if your RT clock

comes with any memory-resident software that

automatically passes time changes back and forth

between the DOS and RT clocks.  An example is the

AST 6-Pac board with AST's ASTCLOCK.COM program.

With ASTCLOCK.COM made memory-resident (by running

ASTCLOCK /R), you won't need Strickland's programs.

Just choose (F4) and ASTCLOCK will take care of

the transfer from DOS to the RT clock automatically.

The same would apply to any memory-resident clock

driver you may already own.

If you have an AT CMOS-type clock, select F2

(that activates an assembly language module I

adapted from a magazine article I was steered to

by a user who works for Hewlett-Packard).

If you have nothing but a DOS clock/calendar,

select F1.

9

No matter what you choose at this point, the DOS

clock/calendar will always be included in the

timesetting process.

Call type:     Local (F1) or long distance (F2).  If you live in

the immediate area of Washington, D.C., F1 will

open a new window with two new F1/F2 choices.  F1

in the new window is for simple local dialing,

i.e., no prefixes of any kind.  F2 in the new

window is for local dialing that requires a

prefix and/or a corporate account number, and if

you choose that option you will be prompted for

that information.

If you select long distance a new window will

open up with three choices: (F1) simple long-

distance dialing, (F2) complex long distance

dialing, and (F3) trunk line dialing.

Simple long-distance dialing (F1) involves

creating a dialing string that consists only of

an area code and the seven-digit basic phone

number, that is, nnn-nnn-nnnn.  Please don't get

confused about the meaning of the word "simple"

in this context.  To many people these days,

"simple long-distance dialing" means dialing a

standard 10-digit long-distance number preceded

by a 1 (11 digits in all).  They are surprised,

as I was, to learn that there are many significant

areas of the country (at least parts of California,

for example) that still use the old 10-digit system

(without 1 as a prefix).  "Simple long-distance dialing"

refers to that kind of dialing, where there is only

an area code and a 7-digit number.  If you live in

an area where a prefix to that 10 digit number is

needed, even if it is only a 1, select F2,

"Complex long-distance dialing."

Pressing F2 brings you a new window with four new

choices: F1, for people who need a prefix ahead

of a long-distance number (e.g., 102-881 for AT&T

lines, or simply a 1); F2 is the same as F1 except that

it brings a prompt for a local corporate account

number to go in after the number is dialed; F3 is

for people who need to dial an outside carrier,

supply an account number, then the long distance

number; F4 is the same as F3 except that it

brings an additional prompt for a local corporate

account number.

I am not aware that there is trunk line dialing

to the Naval Observatory, but I included that

10

option in case there is or will be.  If you make

that choice, you will be prompted to type in all

the numbers you need and the necessary number of

pauses between them.

In all of these long-distance options, it's up to

you to supply any pauses that might be needed to

dial successfully.  A pause must be indicated by

the tilde character (~), not with the comma.

TIMESET will translate it to a comma, the default

pause command for most modems, before sending it.

You will never be asked to supply the Naval

Observatory's area code or basic phone number,

because the configuration program supplies those

numbers for you and combines them in the proper

way with any information you supply.

Dial method:   Tone (F1), pulse (F2), or auto (F3).  Choose

which method you want and what your modem allows.

(The auto feature is for some of the newer modems

that can use the ATD command instead of ATDT or

ATDP.)

Baud rate:     The required setting for nearly all modems is 1200

baud (F1).  However, there are some very fancy MNP-

type modems that start out at 9600 or 19200 baud,

then drop down to the speed of the modem at the other

end of the line.  Don't press F2 if you don't

have one of these specialized high-speed modems.

If you have either a 1200- or 2400-baud standard

modem, you must select F1 (1200 baud).

Modem codes:   This is a prompt that appears as soon as you have

chosen the baud rate.  Type any additional

initialization codes you want to send to the

modem.  You must ALWAYS put in at least an Xn

value at this point.  Which Xn value depends on

whether or not your modem can return the word

BUSY when there is a busy signal; this is usually

X4.  If your modem cannot return the word BUSY,

choose X1.  This will at least return NO CARRIER,

which is a result that TIMESET also watches for.

With a modem that can't say BUSY, you have to

depend on NO CARRIER to let TIMESET know that a

connection can't be made.  You may have to

experiment with settings of register S7 to

determine the best value to bring those words to

TIMESET in timely fashion if the line is busy.

Start with X1S7=10, then vary S7 up or down a

second or two at time until NO CARRIER registers

after about 4 to 6 busy tones.  (Don't worry

11

about not getting a busy signal; they occur very

often on the Naval Observatory's line these

days.)  Another setting that might be useful here,

if your modem supports it, is &C1, which tells the

modem to wait for a carrier before sending a result

code (this is handled by a physical switch on some

modems).  Finally, your modem can be set here to

follow DTR (if it isn't already set to do that) by

typing in &D2 if your modem can handle the & commands

or by setting the appropriate physical switch if it

cannot.  This setting allows the modem to be put on

hook (hung up) in the fastest and most efficient way,

and it is required for proper operation of TIMESET

Version 4.20.

(Note: If you do set your modem for the X1 result codes,

TIMESET version 4.20 will print the word BUSY on your

screen if a connection is not made in the time specified

in S7.  The actual result code from the modem is NO

CARRIER, but TIMESET version 4.20 assumes that an

absence of carrier during the dial-up process occurs

because the line is busy.  Later, however, during the

data string capture process, a loss of carrier will

produce the NO CARRIER message on your screen.)

Interaction:   There are two ways to run TIMESET--

interactively, with pauses for input from you

(F1) or automatically (F2), with the program

skipping the opening screen and all menus, going

directly to dialing, then jumping right back out

to DOS when the time and date have been received.

You would choose F2 if, for example, you wanted

to have TIMESET in an AUTOEXEC.BAT file as part

of your boot-up procedure.  In the automatic

mode, TIMESET will redial up to 10 times if the

line is busy; in nonautomatic mode, you have to

press keys to dial and redial.

Monitor:       Version 4.+ of TIMESET lets you configure for

either a CGA color monitor (F1) or a monochrome

monitor (F2).  Either way, TIMESET will put on a

great show.

Believe me, describing the configuration routine takes a lot more time than

actually doing it.

Creating a Configuration File to Override the Built-In Configuration

Most users will not need to bother with this option, and I'm not even sure

it's necessary for anyone.  However, it is included for those who are

experimentally inclined.  It might be useful for some people who have

12

extraordinarily complicated phone systems, but there is no guarantee that

anything they try here will work.  An external configuration file, created by

you with an ASCII editor, also might be useful for temporary setups.  However,

the configuration program CONFIGTS.EXE makes configuration changes very easy to

do.  Furthermore, a number of command line parameters (discussed later) can

be used to temporarily override TIMESET's internal configuration.

Here is what the external configuration file should contain.  Items that

might be useful for people trying to make bizarre telephone systems work with

TIMESET are marked with asterisks (*).  Follow this order exactly, use only the

key words that are supplied in capital letters, avoid typos, be sure to leave

an empty line if you don't want to change the existing setting for a particular

item, and don't put more than one item on a line:

Time zone      NEWFOUNDLAND, ATLANTIC, EASTERN, CENTRAL,

MOUNTAIN, PACIFIC, ALASKA, HAWAII, UNIVERSAL, or

hours offset from Greenwich, England (negative

number if west of Greenwich and east of the

International Date Line).

Season         STANDARD, DAYLIGHT, OR NULL.  Use NULL if you are

setting your clock to universal time.  Universal

time has no seasons (it's, uh, universal).

Comm port      Only COM1, COM2, COM3, and COM4 are allowed.

Don't put a colon after the port name.

Method         IMMEDIATE or COUNTDOWN.

Clock type     DOS, AT, or RT.  Use DOS if Strickland's RT clock

driver is present, otherwise use RT (more on this

later).

* Carrier      This is a line you can work with if you have an

unusually complicated phone system.  A long

distance carrier's phone number normally goes

here, but it could also be any telephone number

you have to dial before you can dial the Naval

Observatory.  If you don't use this line you must

still type in NULL (unless it's already present

in the internal configuration).

* Phone number This is another line to work with if you are

trying to adapt TIMESET to an unusually

complicated phone dialing system, although all

the other lines must also be present, whether

filled in or left blank.  You must include the

Naval Observatory's phone number here: 653-0351

if you are calling locally; 202-653-0351 if you

are calling long distance.  Do NOT write the area

13

code like this: (202); use only the numbers,

without parentheses.

How you fill in this line is up to you, because I

don't know how your system works.  Here are some

things to remember:

(1) You will have to put everything on one line,

for this and the preceding item, which limits you

to 80 characters for each of these lines.

(2) Pauses are made with the tilde character (~).

You can adjust the pause duration with modem

register S8 (see "Initialize" below).

The manual for the Hayes 2400 Smartmodem has some

commands that might be useful here.  For example,

the @ command means "wait for the time specified

in register S7 for one or more rings followed by

5 seconds of silence before processing the next

symbol in the dial string" (p. 4-21).

How you work all this out is up to you, and it

depends on the peculiarities of your system.  I

can only provide a few tools; I can't advise you

how to use them.  We've come too far from "Number

please" for me to be of much help.  Finally,

there is no guarantee that anything you try on

this line or the one preceding it will do any

good if your telephone system is the exotic kind.

Dial method    TONE, PULSE, OR AUTO.  Use AUTO only if your

modem is the kind that can respond to ADT.

Baud rate      1200 is the number to use in the vast majority of

cases.  However, there are some newer, very

expensive modems that are locked in at 9600 or

19200 baud at the outset, then step down if

necessary to match the speed of the remote modem.

Initialize     This is the string to initialize the modem.  It

must contain at least an Xn value.  Use X4 if

your modem can return the word BUSY, or whatever

Xn value produces it with your modem.  If your

modem can't return BUSY, you will have to use X1,

which is the minimum that most modems have.  X1

will return the words NO CARRIER after an

interval determined by the value in the S7

register.  The trick here is to set S7 to a value

that will produce NO CARRIER within a reasonable

time if the line is busy, so the program can quit

waiting for a connection.  Start with S7=10 and

14

adjust the value up or down till you can get NO

CARRIER on the TIMESET screen after 4 or 5 busy

tones.  A setting that usually works is X1S7=10.

Other registers can be modified here, too.  For

example, register S8 is normally S8=2 (the

factory default value), making each pause command

worth 2 seconds, but you can change that here

if you want to.  In fact, you can change any S

register here.  Consult your modem manual.

Interaction    If you want to run TIMESET interactively, that

is, with menus and prompts, type in PAUSE.  If

you want it to run automatically, without menus

and with immediate dialing (up to 10 times if the

line is busy), type NOPAUSE (one word).

Monitor        COLOR or MONO.

Here is what a completed command file might look like if you wanted to change

the time zone, communication port, clock type, method of dialing, baud rate,

user interaction, and type of display:

PACIFIC             (change to Pacific time zone)

[blank]             (use TIMESET's configuration for season)

COM1                (change to comm port 1)

[blank]             (use TIMESET's configuration for set method)

AT                  (change to AT's CMOS clock)

[blank]             (use TIMESET's configuration for LD carrier)

[Your dialing concoction goes on this line. Include the Observatory number.]

TONE                (change to tone dialing)

1200                (change to 1200 baud)

[blank]             (use TIMESET's configuration for modem initializing)

PAUSE               (change to interactive mode)

MONO                (change to monochrome monitor)

The point is that you only need to enter data for the items you want to

change.  The lines for items you don't want to change can be left blank.

Always count your lines.  There must be 12 of them--absolutely, always, without

exception--even if some are left blank to force TIMESET to use its own

configuration for those items.  If there are not 12 lines, whether empty or

filled, TIMESET will get hopelessly confused, with results that are

unpredictable.

To use your own configuration file, you have to tell TIMESET where it is by

putting its drive, directory, and name on the command line, like this:

TIMESET /F<Drive>:\<Directory>\<FileName>

If that command line is present, TIMESET will substitute configuration values

put into it by CONFIGTS.EXE with values you have in your file.

15

As I said, however, this procedure is mainly for people who have bizarre

phone systems, and it may not do them much good.  Most of us are much better

off using CONFIGTS.EXE, which should cover the great majority of dialing

requirements and is designed for rapid changes.

In a later section you will find some ways to make TIMESET ignore some of

its built-in configurations by putting special instructions on the DOS command

line.

Modem Switches: Setting Response to DTR

With Version 4.20, you must set your modem to follow DTR.  On some modems

this is done with a physical switch setting on the modem itself (consult your

modem manual to find the right switch).  With others it's done by sending the

&D2 command to the modem via software.  If your modem is the second type, you

can put the &D2 command in during configuration of TIMESET with CONFIGTS.EXE.

You could also do it with any communications program, and you could also make

it permanent in the modem's nonvolatile memory by following it with &W

(&D2&W).  If you make it permanent you can forget about it when configuring

TIMESET.

Offline Countdown

All versions of TIMESET have had an option of doing a countdown to zero

seconds while online to the Naval Observatory.  The countdown feature was

incorporated mainly to accommodate the limitations of AT&T and Olivetti

computers, which cannot set any second but zero.  Since I have an AT&T, I

couldn't have developed TIMESET without that feature.  However, it is

evident from chronic busy signals from the Naval Observatory, which usually

end right on the minute, that many, many people who don't need a countdown are

using it routinely (either that or AT&T computers have become the leading

brand, which is wildly improbable).

A countdown can take up to nearly a minute, and there are only 1440 minutes

in a day.  Clearly, it does not take many people using an unnecessary online

countdown to tie up a valuable service for a trivial reason.  This was not a

big problem before the Naval Observatory's time service became widely known,

but now it is a problem, and it will undoubtedly get worse if something isn't

done about it.

It's time to start "phasing out" this problem.  Version 4.20 of TIMESET

(and any future versions) will not allow you to do a countdown online unless

you really need one, that is, unless you are using an AT&T or an Olivetti.  If

you don't have one of these computers, TIMESET will still do a countdown if you

insist, but it will do it offline.  As soon as you connect with the Naval

Observatory, the program will grab the first clean data string and hang up

immediately.  Starting with that one data string as the baseline, the program

will use the computer's own internal timer, which ticks 18.2 times per second,

to count off the remaining seconds to zero.  Note that this countdown is not

based on seconds in the system clock, which can be wrong (otherwise why would

16

you be calling the Naval Observatory?).  Instead it's based on the "heartbeat"

of the computer itself, which occurs at high speed and fairly high accuracy,

irrespective of what the computer clock says.

Since this critical offline counting operation is handled by an assembly

language module, there is no great loss of accuracy in comparison with online

countdowns.  There is some loss of accuracy, however.  How much loss may

depend on your computer, though your time setting should not be off by more

than a second, depending on how long the countdown lasts (i.e., what the

second happens to be when you get online).  If you can't tolerate up to one

second of inaccuracy the solution is really very simple: don't use the count-

down and configure for immediate timesetting instead.  The only people who can

get a highly accurate countdown are those who do it online, and the only people

who can do it online are those who have an AT&T or Olivetti.  AT&T and Olivetti

owners are the only ones who really need a countdown.  I'm sorry to have to do

this to people who do countdowns routinely simply because they enjoy them, but

fun was never the purpose for the countdown feature in TIMESET.  From the

outset that has been a compromise measure regrettably made necessary by AT&T

and Olivetti's faulty clock design.

In addition to increasing the capacity of the Naval Observatory's time

service significantly and giving others a fair chance to use it, the new

feature is much more tolerant of noisy lines (needing only one good data string

to start the countdown and no additional strings to sustain it).  Thus it can

reduce long-distance phone bills.  For all these reasons, as well as for the

host of other improvements in version 4.20 of TIMESET, it is to everyone's

advantage to get rid of any previous versions and use this one.  I strongly

urge the wholesale scrapping of all previous versions of TIMESET.

AT&T and Olivetti owners don't need to anything to get an online countdown,

because version 4.20 checks ROM to detect those brands and automatically

selects an online countdown (even if you configure for immediate timesetting).

However, if you have one of those computers and don't want to do the countdown

online, you can get an offline countdown by putting a pound sign (#) on the

command line when you run TIMESET.

Version 4.20 of TIMESET has an added feature to make offline countdowns as

accurate as possible.  The timer in MSDOS computers is said to tick at a rate

of 18.20648 times per second, but that rate is only nominal, and different

machines tick at slightly different rates.  TIMESET 4.20 lets you determine

the tick rate for your own computer, which is very likely to deviate by some

fraction from the nominal rate.  If you don't have an AT&T or Olivetti and have

configured for countdown (or have indicated that you want one by putting /CD on

the command line), an option for calculating your computer's ticking rate will

appear on the opening menu.  If you press C, TIMESET will measure that rate and

use it during the offline countdown.  Measuring the rate takes a full minute.

If you choose not to determine that rate, TIMESET will use the nominal 18.20648

rate instead (which might or might not be accurate enough).

17

Running TIMESET

Loading TIMESET: Command Line Parameters

If you load TIMESET without any command line parameters, that is, by just

typing TIMESET at the DOS prompt, it will run according to the configuration

you gave it with CONFIGTS.EXE.  However, some or all of the configuration

settings can be superseded by parameters you can put on the DOS command line

when you load TIMESET.

As already noted, you can tell TIMESET to use a configuration file you wrote,

using the /Ffilespec command line parameter.  You can make that file replace

all of the built-in configuration settings if you want to, although it's a lot

easier to reconfigure the program.

If you have configured TIMESET for a color monitor, you can make it display

in monochrome by entering /M on the command line.  (There is no parameter for

the reverse situation, because there is no way to make a monochrome system show

color.)

If you have configured TIMESET for automatic (noninteractive) mode, you can

make it run interactively by entering /I on the command line.  This feature is

included mainly to give users who operate the program automatically an easy way

to toggle between standard and daylight time and perform other functions that

require interaction with the program.  Conversely, if you have configured

TIMESET for interactive operation, you can make it run automatically by

entering /A on the command line.

If you get tired of looking at the opening screen with the dazzling TIMESET

logo, you can skip it and go directly to the opening menu by putting /S on the

command line.  (If you're like me, you soon get impatient with programs that

insist on making you look at an opening screen, however attractive it may be,

for the 10-thousandth time before getting down to business.)  This parameter

has no relevance if TIMESET is configured for noninteractive mode, because the

opening screen and menu are skipped anyway.

Except for a tick every second when time is being displayed, TIMESET Version

4.20 is totally silent by default, in deference to many users who complained

that Version 4.00 was waking up the family when they ran it late at night, or

violating the decorum of the office when they ran it there.  Version 4.01

offered much improvement by having only a beep or two in a few places, but even

that was too much for some users.  (One user said he found the beeps

"EXCRUCIATING!," which seems like exaggeration to me).  However, some people

love sound effects.  If you do, you can get lots of them with Version 4.20 by

putting /E on the command line.

Three new command line switches have been added to TIMESET version 4.20.

They are /CD, /NCD, and #.  /CD ("CountDown") lets you change to the countdown

method if you have configured the program for immediate timesetting with

CONFIGTS.EXE.  /NCD ("No CountDown") lets you change to immediate timesetting

if you configured TIMESET for countdown with CONFIGTS.  /NCD is ignored if your

computer is an AT&T or Olivetti, however, because those computers must have a

18

countdown to zero seconds to set the time accurately.  I could have included

the /CD override in previous versions, but I didn't want to encourage use of

the countdown method by people who don't really need it.  Now that the

countdown is done offline with most computers, that concern is no longer

relevant.  If you have an AT&T or Olivetti and want an offline countdown, you

can get it by putting # on the command line, otherwise the countdown is done

online by default with these computers.

There is no need to have spaces between any of these switches.

TIMESET Preconfigured for Automatic (Noninteractive) Mode

If TIMESET is configured by CONFIGTS.EXE for noninteractive or automatic

mode, all you have to do is run it and it will start dialing right away,

skipping the opening screen and the front menu.  If the line is busy it will

dial up to 10 times before giving up.  When the computer clock is set, it will

exit to DOS, leaving a message on the screen telling you what the computer time

was and what it was changed to, and what the date was and what it was changed

to.  However, you can interrupt automatic operation by pressing the backspace

key to go "back" to the opening menu, as long as you do it soon enough,

preferably before dialing is completed.  As soon as you hit the backspace key,

you switch into interactive mode for the rest of the session, and you can then

check your configuration, toggle between standard and daylight time, view the

help screens, change time or date settings manually, shell to DOS, or quit the

program (the help screens provide details about all these menu functions--see

next section).

TIMESET Configured for Interactive Mode

If TIMESET is configured for interactive mode, it becomes a menu-driven

program and you have to press keys to make things happen.  The opening screen,

the first menu, and the help screens tell you what keys do what.

Here is a brief rundown on what particular keys do:

D -- Dial the Naval Observatory

T -- Toggle between daylight and standard time.  This

actually changes the configuration of TIMESET.EXE

on the disk as well as in the copy that is in memory,

swapping the words DAYLIGHT and STANDARD.  It also

adds or subtracts an hour from the system clock(s),

depending on what's needed.  If TIMESET is configured

for COUNTDOWN (or if you have an AT&T 6300 or

Olivetti), none of this will happen until your clock

reaches zero seconds.

19

S -- Shell to DOS.  The function is here if you need

it for any reason.  Type EXIT in DOS to get back

to TIMESET.

Q -- Quit TIMESET.

H -- Help screens.  Unlike versions 4.00 and 4.01, help

screens in version 4.20 are inside the program

itself instead of a separate external file.  This

feature greatly simplifies the way these screens

are handled.  The price for this is additional disk

file size and memory requirements, but I think it's

a small price for added convenience.  The help

screens are kept in compressed form until they are

needed.

C -- Count ticks per second.  This option appears if your

computer is a non-AT&T/Olivetti and you have

requested a countdown.  It also appears if your

computer is an AT&T/Olivetti and you have requested

an offline countdown by putting # on the command

line.  If you press C, a window opens on the screen

to display a count of 60 seconds.  During that period,

the program counts the number of ticks in the system

timer, then it calculates and displays the number of

ticks per second in your system that it will use in

pacing the offline countdown after you have

disconnected from the Naval Observatory.  After the

tick rate is determined, the program jumps back to

the opening menu so you can press D to dial.

Backspace Key

The backspace key works on any screen of TIMESET, and it always does the same

thing -- it take you back to a previous screen, usually the last one that was

displayed.  If you are at the opening menu and have configured for interactive

operation, pressing the backspace key will take you all the way to the top of

the program so you can gaze in wonder at the cosmos displayed on the program

logo.

Keys for Manual Time Changes

The top and bottom rows of the numeric keypad are used for manual adjustment

of either time or date.  The top row is for incrementing, the bottom row for

decrementing.

20

If a shift key is not held down, the top row keys from left to right will

advance hours, minutes, and seconds by one unit, and the bottom row keys from

left to right will subtract one unit of hours, minutes, or seconds.  You can

see these changes occur immediately on the big time display at the opening

menu.

If a shift key is held down, the same keys advance or decrement month, date,

or year (from left to right).  You can also see those changes taking place

immediately on the date display.

With either manual time or date adjustment, the changes also occur

immediately in your system's clock(s), so be careful not to touch those keys

unless you really want to make the changes.

The main purpose of manual adjustments is to synchronize two or more

computers in the same office.  Only one of them needs to call the Naval

Observatory; the others, also running TIMESET, can be adjusted manually to

synchronize with the time display of the one that has the "official" time.  The time

display changes colors at 30 seconds and at the top of the minute, making those

times useful as end-points.  With a little practice the manual synchronization

can be done very accurately.

You can also use this technique to synchronize with short-wave radio time

signals from the National Bureau of Standards (WWV), or from Canada (CHU), from

your local telephone company, or from the Naval Observatory's audio time

service (900-410-TIME).  Finally, you can use the manual method to reset your

computer to another time zone anywhere in the world without having to

reconfigure TIMESET and call the Naval Observatory again.  This could be useful

for someone in Pasadena who has to run an unattended upload/download when it's,

say, Monday, 9:00 am standard time in Tokyo (Sunday, 4 pm PST in Pasadena), or

someone whose business requires him to know, at a glance and without doing

calculations, what time and day it is in Saudi Arabia.

There is one limitation on using the manual setting keys, however, and again

it involves the old problem of AT&T 6300 and Olivetti computers--inability to

set any second besides zero.  In fact, if you press any of the manual

timesetting keys with one of these computers, whether it's for time or date,

the DOS seconds will immediately jump to zero.  For this reason, I don't

recommend using the manual time and date keys with these computers, unless you

aren't particular about extremely exact time (in which case, you probably don't

need TIMESET).

Toggling Between Standard and Daylight Time

As noted, you can toggle between standard and daylight time at the opening

menu by pressing T.  With Versions 4.00 and 4.01, there were a couple of

situations where this did not work well: (1) When the public domain program

EMMCACHE (and probably other programs) had set up a large disk cache in memory;

and (2) when TIMESET was operating as a memory-resident program under the

WordPerfect Library.

21

The problem in both of these situations was that even though the copy of

TIMESET that was on the disk was reconfigured, the copy that was in the disk

cache or stored someplace in memory (RAM or expanded memory) by the WordPerfect

Library was not.  The result was that the copy of TIMESET likely to be reloaded

after toggling was not the altered disk copy but the copy that was already in

memory--with the previous configuration still intact.

That problem has been corrected in Version 4.20.  The program makes the

changes on the disk file, as before, but it does not attempt to load and run

the altered program.  Instead, Version 4.20 makes a change in the copy of

itself that's in memory, as well as in the disk copy, then simply jumps to its

own beginning.  Of course the change will also be evident the next time the

program is loaded from the disk.

If you want to toggle daylight/standard and are running TIMESET while in a

directory other than the one that actually contains the program's disk file,

you will need a statement in the DOS environment that tells the program where

to find the disk copy of itself so it can make the necessary change in that

file.  The DOS command to use is: SET TSET=<drive:>\<directory>.  Thus, if

TIMESET.EXE were in a directory called NAVYTIME on drive C, the command would

be: SET TSET=C:\NAVYTIME.  You may want to put such a SET command in your

AUTOEXEC.BAT file.

Dialing

Everything proceeds automatically when you press D to dial the Observatory.

Result codes from the modem (CONNECT, BUSY, NO CARRIER, etc.) are displayed in

big letters.  If for some reason you change your mind about dialing, you can

press backspace to abort the call, and the command will be heeded promptly

(unlike earlier versions of TIMESET).

Breaking Out of the Program

If you have to break out of TIMESET for any reason, you can do it at any

time by pressing either control-C or function key F10.  Control-break is

nonfunctional starting with version 4.00 of TIMESET, because the exit it makes

is very ungraceful.  Certain things have to be done before TIMESET leaves the

scene.  Most notably, the communication port vectors have to be changed to

their normal values if the user has configured for COM3 or COM4.  Also, the

modem has to be reset to its default values.  All this requires a more orderly

exit than can be obtained with control-break.  The exiting therefore might take

a couple of seconds with control-C or F10, but it will happen, and it will

happen in the right way.

Corrupt String Rejection: Line Noise

Line noise is the greatest enemy of any program designed to call the Naval

Observatory's digital time service.  We learn to live with noise when we use

22

ordinary communications programs, reading through the garbage to extract real

information, unless it's horrendously noisy.  But every single character in the

incoming Naval Observatory data string has to be present and in its proper

place.  There can be no dropped characters, no disallowed characters, and no

extra characters.  It is useless to try to set a computer's clock with data

extracted from a string that has been corrupted in any of these ways by line

noise, and TIMESET does not try to.  Versions 4+ of TIMESET apply more

stringent tests than any previous versions to be sure an incoming string is

usable before processing it to extract time and date.  If the line is

chronically noisy, TIMESET simply will not work at all, nor should it be

expected to.  In Version 4.20, the string testing is done with an assembly

language module for maximum speed (and thus more accurate timesetting).

It was inconceivable to me when I wrote the earliest versions of TIMESET that

anybody could have a line that was noisy all the time.  I anticipated

occasional bursts of noise, but I soon learned that there are people whose

phone lines are continuously noisy.  Some of these people would call to report

that the program just "sat there" while the RD light on the modem flashed every

second.  But the program was not just "sitting there," it was busily rejecting

garbled data strings and waiting for a clean one that never came.

The earliest versions of TIMESET did not anticipate the chronic noise

problem, so they did seem to "just sit there" until the Naval Observatory

modem hung up on them.  Versions 4+ count the number of corrupt strings, and if

15 seconds worth of garbage arrives, TIMESET hangs up on the Naval Observatory

and flashes a message on your screen.  It also displays the strings, whether

good or bad, so you can see what is happening.  If line noise occurs

chronically or frequently, check for loose phone connections in your house or

office, call the phone company to check the lines, or both.  But don't blame

TIMESET; it's only a dumb beast that lacks the cognitive powers to extract

meaningful information from garbage (difficult even for humans, especially when

listening to George Bush or even trying to read his lips).  It's up to you to

do all you can to ensure that TIMESET has clean data to work with.

Naval Observatory Data

As soon as clean data are obtained from the Naval Observatory, the program

converts the data to time and date for the zone you selected, plugs it into

your computer, and hangs up.  You should get in the habit of inspecting the

information sent by the Naval Observatory to make sure it wasn't garbled by a

noisy connection.  The information will be displayed at the top of the final

screen and should look like this:

47014 219 224449 UTC

That is, a group of 5 numbers, a space, a group of 3 numbers, a space, a

group of 6 numbers, a space, and finally, UTC.  If you see any other pattern

or any other characters besides these numbers, grouped exactly this way and

terminated by the letters "UTC," press the spacebar to call again.  You didn't

get a correct reading.  I have never seen this happen.  The program is designed

to stay on line until a clean data string is obtained (or 15 dirty ones) or

23

until time runs out.  The error checking and corrupt string rejection, though

very good, are probably not perfect, so you should always inspect the top line

of the final screen to be sure it fits the pattern shown above.

Below the top line you will see an interpretation of the data that were used,

followed by a box containing a summary of what just happened, i.e., what your

computer's time was and what it is now, what your computer's date was and what

it is now.

Near the bottom of the final screen you will see a continuous display of

the current time, based on the Naval Observatory setting that was just made.

The time is now displayed to the 10th of a second on any computer.

If you see a warning about your modem on the final screen, it means the

hangup probably failed and that you may still be online to the Naval

Observatory.  This is nothing disastrous, because the Naval Observatory will

cut you off after 60 seconds online anyway.  It shouldn't happen, however--not

with a hanging-up procedure that involves dropping DTR and, if necessary,

sending ATH to the modem up to 10 times, with checking for carrier each time.

You have at least four options when the final screen is displayed.  You can

press control-C or F10 to quit the program, the spacebar to dial again, the

backpace key to go back to the opening menu (for example, to change the

configuration), and Alt-C to display a conventional analog clock (and find out

about Life Sciences Editorial Services).  If you are running TIMESET under the

WordPerfect Library program you have a fifth option: you can use WordPerfect

Library commands to switch directly to another program (see section on Word

Perfect Library compatibility, below).

Tom Strickland's Supporting Programs

Tom Strickland of Alexandria, VA has contributed four excellent programs that

enhance Version 4.x of TIMESET.  They are RTCLOCK.COM, REALTIME.COM,

DOSTIME.COM, and CLKDRV.SYS and are in a separate .ARC file.  Basically, the

purpose of these programs is to bring some uniformity to setting and reading a

host of real-time clocks (non-AT CMOS) that are sold under different brand

names and use different proprietary software for setting and reading.  I call

these, "other RT clocks."  Since all of these clocks are based on the same

clock chip or family of clock chips (made mainly by Motorola, I think), they

literally invite creation of public domain software that can circumvent all

proprietary ways of "talking" to them (including some pretty clumsy ways, in my

opinion).  Tom, an extremely talented young programmer, has responded to the

invitation.  Here are descriptions of his programs and how to use them.  Please

keep in mind that they are only for "other" RT clocks, not for the AT CMOS.

RTCLOCK.COM.  This is a program for moving time and date from DOS to

the RT clock and vice versa.  Use it to set the RT clock as follows:

set the time and date in the DOS clock/calendar first (either with

the TIME and DATE commands or, much more accurately, with TIMESET),

24

then enter, RTCLOCK -S (S for "send).  The time and date will be

moved to the RT clock.  Thereafter, any time you want to set the DOS

clock/calendar from the RT clock, just enter, RTCLOCK -G (G for

"get").

REALTIME.COM.  This is a program for reading the RT clock after it

has been set with RTCLOCK.COM (or with TIMESET).  Just enter REALTIME

at the DOS prompt and the RT's time and date will be displayed

together on your screen.

DOSTIME.COM.  This is the counterpart of REALTIME.COM.  When you run

it, it shows you the time and date in the DOS clock/calendar.  This

program is useful with any computer, not just those that are equipped

with one of the "other" RT clocks.  Thus, if you want to know the

time and date in DOS, just run DOSTIME (a 500-byte program) and you'll

get both at once, without those annoying prompts to enter a new time

or date that DOS always gives you when you use the TIME or DATE

commands.

CLKDRV.SYS.  I saved the best till last.  This is a clock driver that

Tom wrote to make the DOS clock/calendar and the "other" RT clock

behave as if they are one and the same.  You install it in your

system by putting this line in your CONFIG.SYS file:

DEVICE = CLKDRV.SYS

Here is what happens when CLKDRV.SYS is installed:

1.  Any time you ask for the time or date in DOS, either with the DOS

TIME and DATE commands or with Tom's DOSTIME.COM program (or, for

that matter, with any program that gets time and date from DOS,

including TIMESET), the driver instantly moves the RT clock time and

date to DOS, so what is presented to you is the time and date from

the most accurate source in your system--your RT clock.  Whenever you

ask for DOS time and date, the DOS clock/calendar is corrected from

the RT clock, so what you get from DOS are time and date values that

are in the RT clock at that instant.

2.  Any time you change the time and date in DOS, either with the

regular DOS commands or with a program (including TIMESET), the

driver instantly puts the same settings into the RT clock.  This

makes it unnecessary for you to use any software at all to set your

RT clock, because whenever you set the time and date in DOS, you also

set them in the RT clock (or rather the driver does).

Remember that if you have set your RT clock with Tom Strickland's software,

you also have to read it with his software.  The software that came with your

RT clock will not return the right year if the clock has been set with any of

Tom's programs.  Put that software away.  You no longer need it.  You have to

use some kind of software to read and set your RT clock, and it might as well

be Tom's, especially if you want to use TIMESET to set your RT clock.  TIMESET

25

and Tom's RT clock software are designed to work together.  In fact, Tom also

wrote the assembly language module that TIMESET uses to set "other" RT clocks,

and that module uses the same algorithms as his other RT clock programs.

Rest assured that the aim here is not to "lock" you into anything, it's to

make TIMESET (or any other program) compatible with as many RT clocks as

possible in the most efficient way possible.  Tom's programs also bring some

order to a truly chaotic situation created by a bunch of companies, each of

which tries to make its implementation of the very same product (a clock chip)

seem "different" from everybody else's.  The only practical way they differ is

in the software each company writes to talk to their own clock and nobody

else's.  Tom's programs can talk to at least most of them, and he has made an

outstanding contribution by writing them.

A very important point to remember is that if you have CLKDRV.SYS installed

in your system, you must NOT configure TIMESET to set the "other" RT clock or

try to set the RT clock with RTCLOCK.COM.  You don't need to set or read the RT

clock with software, because the driver does this for you any time you change

or ask for the time or date while in DOS or in a program.  So, you can either

have Tom's driver installed for automatic time/date transfer or you can set the

RT clock directly with disk software, but you can't do both.  Use one method or

the other.  Thus, if you are configuring TIMESET and the driver is installed,

be sure to select F4 (Driver) on the configuration menu, not F3 (Other RT).

Only if the driver is NOT installed can you choose "Other RT" as a TIMESET

configuration option.

Personally, I think the driver is the best way to go.  The only person I know

of who has found CLKDRV.SYS unsuitable is one of my beta testers who has a

favorite computer game that makes changes in his DOS clock/calendar, by design.

With the driver installed, any DOS time or date change made by the game is

instantly passed to his RT clock.  The result is that he quickly corrupts his

most dependable time source.  Of course he can always get things right again by

calling the Naval Observatory with TIMESET, or resetting his RT clock

indirectly by changing time in DOS some other way (e.g., DOS TIME and DATE

commands).  However, he prefers to forego the convenience of the driver so he

can play his computer game without disturbing his RT clock.

[ADDENDUM:  I was not aware when I wrote the first "edition" of this

document that the AST Six-Pac board comes with a program called

ASTCLOCK.COM, which can be installed memory-resident by putting /R on

its command line.  If you have that board and that software, or any

other clock and software that act the same way, you clearly don't need

to use any of Strickland's software.  If you make ASTCLOCK memory-

resident, it should do the job Tom's CLKDRV.SYS would do--transfer DOS

time and date to the RT clock automatically whenever there is a change.

So, if you have the AST board and ASTCLOCK is memory-resident, put Tom's

programs aside and configure TIMESET to change only the DOS clock/

calendar.  Memory-resident ASTCLOCK will automatically pass the DOS time

and date to the RT clock.]

26

Compatibility with the WordPerfect Library

If you don't have the WordPerfect Library program, you can skip this section.

TIMESET is designed for full compatibility with WordPerfect Corporation's

Library program.  That is, you can switch from TIMESET to another program on

the Library menu just by holding down the ALT and SHIFT keys and pressing a

keyboard character that corresponds to the desired program's letter on the

Library menu.  This takes you directly to the selected program without going

through that menu.  ALT-SHIFT-key functions operate at the two points of the

TIMESET program where you are presented with options for keyboard input--at the

opening menu and at the completion of the call to the Naval Observatory.

(Obviously it can't work if you are operating TIMESET in automatic mode,

because there is no prompting for input in that mode.)  When you switch to

another program in this manner, TIMESET is left resident in memory (in expanded

memory if you have it on your system), just like any WordPerfect Corporation

program running under the Library.

The program also accepts the CTRL-F1 command, which produces the familiar

WordPerfect-style line, "1 Go to the Shell" at the bottom of the screen.  You

can press the "1" key at this point, followed quickly by the menu letter of the

program you want next, although this is not as simple as pressing ALT-SHIFT and

a letter key.  You can cancel the CTRL-F1 command and erase the bottom line by

pressing F1, the cancel key in WordPerfect programs.  The CTRL-F1 command is

not very practical at this point, I feel, since there is a simpler way to move

to another program (the ALT-SHIFT method).  I only included it because most

WordPerfect Corporation programs have it, for some reason.

The program is able to detect when the WordPerfect Library is present.  Thus,

if you run it under the Library, an additional prompt will be seen on the first

and final menus indicating that you can enter ALT-SHIFT commands to call other

programs and functions under the Library.  If you run it as an ordinary DOS

program in the absence of the Library, this prompt will not appear on your

screen and the ALT-SHIFT commands will not work.

If you have set up the Library program to load TIMESET as a memory-resident

program when you first load the Library itself, TIMESET will load invisibly.

That is, you won't see any sign of it on your screen other than a message from

the Library telling you it's being loaded.  The program is there, memory-

resident and ready to run whenever you press the right keys according to the

Library's own rules.  Similarly, when you decide to quit the Library, TIMESET

will simply be unloaded from memory without any sign of it on your screen and

without the need for any further action on your part.

These features are identical to those in the WordPerfect Corporation's own

programs (WordPerfect word processor, Calculator, Program Editor, Macro Editor,

Calendar, etc.), all of which are designed to work under the WordPerfect

Library.

When you configure the WordPerfect Library to use TIMESET, answer Y(es) when

you are asked if this is a WordPerfect program.

27

[Note: Some systems may crash, requiring a reboot, when the user attempts to

quit the WordPerfect Library/Shell with TIMESET still resident in memory.  If

this happens with your system, you will have to get in the habit of running

TIMESET as a "normal" program, that is, by always exiting from it by pressing

Q (at the opening menu), Control-C, or F10 each time you are through using it.

In other words, you'll have to avoid pressing S on the opening menu or using

ALT-SHIFT-letter commands to shell to other programs in the Library.  I have no

idea what causes this problem.  If I find out, I will fix it.

A Note About Program Size

A recurring complaint about TIMESET is that it is a large program.  What

can I say?  Yes, it is large, but I'm not the least bit apologetic about it.

The source code for TIMESET is about as compact and as structured as it can be.

There is no "spaghetti code" in this program.  It's tightly written, highly

organized, and its critical functions are handled by linked assembly language

modules.  Uncompiled, its size is about 40 kbytes.  Compile and link it and--

presto!--its size triples.  That is an inherent property of compiled BASIC

programs, and BASIC is the language I am most at home with.  No apology for

that either.  BASIC, particularly its QuickBASIC implementation, has become a

rich and very powerful professional language in recent years, and programs

written in it are very easy to maintain and revise.  It's a bit ironic to think

that people who like to sneer at BASIC programs are among those who routinely

download and upload programs by the thousands over RBBS, which is mostly a set

of compiled BASIC programs comprising one of the best BBS systems around and

probably the most popular.

If disk space is a critical problem for you, by all means get one of the

smaller programs that are out there.  They may well do the job you want.

I'm confident, though, that none so far will do "the same job" that a program

like TIMESET does, contrary to the blurb that accompanies one such program.

(You might even find the one out there whose author has plagiaristically

named "TIMESET.")  What these other programs do so far is the barest humdrum

minimum, namely setting the DOS clock/calendar--with clumsy configuration

procedures, no runtime configuration overrides, no running split-second time

displays (big or small), no before/after clock status reports, no accommodation

for the countdown requirements of AT&T computers, no manual time and date

synchronizations, rudimentary noisy string rejection (if any), no graphics,

and no fun.  Now, I could have written TIMESET that way and had a much smaller

program.  In fact, those bare bones comprised the very earliest pre-release

prototype, but my goal all along has been to make this program as complete and

as sophisticated as possible in response to the suggestions and requests of

the program's many users, including a lot of high-tech professionals who use

it in their work.  If it's too big for you, don't grumble--find another

program, or write one yourself (only please don't call it TIMESET).

28

Conclusion

This program is simple and fun to use, and better than ever.  It comes to you

free from Life Sciences Editorial Services of Annapolis, Maryland (i.e., from

Pete Petrakis, who had a lot of fun developing it).  It's intended to let the

computing world know there is an experienced freelance professional scientific

and technical writer/editor/programmer here in Annapolis.  In short, it's a

promotional gimmick that gives you, free of charge, something of value (I hope)

while telling you that Life Sciences Editorial Services is available (for

money, naturally).

I hope this is the last version of TIMESET I'll have to put out (yes, I know

I've said that before).   Developing  this program  has been great fun and a

real challenge, educational for me as well, but I have other things I want to

do and other things I have to do (e.g., earn a living).  I can't say the

program is perfect, but I can say without hesitation that this version is the

best, most bug-free version so far--at least on my system.  (Note the

qualifier.)

I'd be very interested in hearing from users of the new TIMESET.  I have

attached a clip-and-fold response sheet to the end of this document.  Please

fill it out, fold and tape it, and mail it to me.  That way I can get a clearer

idea of what further improvements are needed or desired.  You can also reach

me on Compuserve (76555,1175).

Pete Petrakis, Life Sciences Editorial Services, Annapolis, Maryland 21401

29

ACKNOWLEDGEMENTS

One of the greatest satisfactions I have gotten from developing TIMESET is

the chance to meet, electronically, some outstanding people who came forth to

offer suggestions for improving the program or actually assisted me in

improving it.  In fact, some of them have become good friends of mine, despite

the fact that we have never met face to face.

My special thanks go to Tom Strickland, who wrote the excellent supporting

programs that so tremendously enhance TIMESET.  I haven't met him personally,

but I know him as a brilliantly talented young programmer who is going to go

far.  Special thanks also to Bob Germer, who at critical stages has

participated actively in developing this and earlier versions of TIMESET,

though he lives far away in the wilds of New Jersey.  He and I collaborated by

phone for hours.  I've never met him personally, either, but he's a good

friend.  I also thank Hans Lustig, Anders Sterner, Joan Friedman, Terry McGuire,

Bruce Felstein, Mark Novisoff, and countless others who either did tests for me,

made valuable suggestions, or simply educated me.  I wish I could remember the

names of the hundreds of people who have called or left BBS messages with

suggestions, praise, and problem reports (lots of those) after using previous

versions of TIMESET.  But they know who they are, and I want them all to know

that I'm grateful.

I also thank Charles Robertson, the author of SETTIME.COM, a very compact

program that also gets time from the Naval Observatory.  It was his program

that gave me the idea of dispensing with a separate configuration file for

TIMESET (except as an option) and writing the configuration directly into the

program itself.

Finally, I thank my wife Julia, who has an outstanding talent for assisting a

programmer in spotting flaws early: whenever I wanted to show her some newly

installed feature of TIMESET, it would fail right then and there.  I call that

"hands-off beta testing."

30

APPENDIX A

Standard Time Differences

for Various World Cities

Relative to Greenwich, England

Note: These differences are based on standard time for all cities listed.

Tokyo          +9 hours

Hong Kong      +8 hours

Beijing        +8 hours

Jakarta        +8 hours

Bombay         +5.5 hours

Karachi        +5 hours

Moscow         +3 hours

Nairobi        +3 hours

Riyadh         +3 hours

Cairo          +2 hours

Johannesburg   +2 hours

Rome           +1 hours

Paris          +1 hours

>       GREENWICH       0 hours       <      (0  Longitude)

Dakar           0 hours

Reykjavic       0 hours

Caracas        -4 hours

New York       -5 hours

Chicago        -6 hours

Mexico City    -6 hours

Denver         -7 hours

San Francisco  -8 hours

Anchorage      -9 hours

Honolulu       -10 hours

31

DISCLAIMER

It is important for you to realize that in using TIMESET you assume the

risk of using it.  I do not promise that it will work on your system or that

it will do all the things you might hope or wish it would do.  Nor can I

guarantee that it will not harm your computer or your data files in some way or

cause something else adverse to happen.  I have labored diligently to prevent

any such problems with this program and I have not yet heard of any serious

consequences, but it is vain and dumb to assert that any product is totally

risk free.  It works on my computer and on a lot of other people's computers,

but I must disclaim any responsibility for any direct or consequential damages

from your use of this program.

Neither I nor this program have any connection with the U.S. Naval

Observatory.  I am a self-employed technical writer/editor/programmer, and my

firm is called Life Sciences Editorial Services.  It is an independent business

located in Annapolis, Maryland, and it, too, has absolutely no connection with

the Naval Observatory.

I wrote TIMESET and put it into the public domain to make the public aware of

my existence and the availability of my services.  I included my phone number

in the program and in this documentation so people could call me to discuss (1)

possible work assignments or (2) problems or questions concerning TIMESET.

Therefore, please do not dial 301-261-1370 (my phone number) expecting to get

the Naval Observatory.  If you need to talk to someone at the Naval

Observatory, a good place to start is with their public information office.

However, it's futile and inappropriate to ask the Naval Observatory technical

questions about TIMESET, because they didn't write the program.

To sum up: Life Sciences Editorial Service and the U.S. Naval Observatory

are different, totally separate entities.  Therefore, if you have questions or

comments about TIMESET (or want to talk business with me), call Life Sciences

Editorial Services (i.e, me) at (301) 261-1370.  If you have questions about

the Naval Observatory, please call the Naval Observatory's public information

office.  If you want to call their digital time service and see their display

on a standard communications program, the number is (202) 653-0351.  Just

remember that if you do that for more than a few seconds you are abusing a

system that a lot of people have come to depend on.

Pete Petrakis

76555,1175

32

USER RESPONSE FORM -- TIMESET VERSION 4.20

Your Name:________________________________________________

Address:_____________________________________________

City:________________________________________________

State:_________________________  Zip:________________

Phone number:____________________

YOUR ANSWERS TO THESE QUESTIONS WILL BE GREATLY APPRECIATED:

Where did you obtain your copy of TIMESET, Version 4.02?

Are you satisfied/dissatisfied with it? (check or circle)

Please describe any problems with the program.  Include information about your

system (computer and modem brands and models, telephone dialing system, etc.).

Add a separate sheet if necessary.

What improvement would you most like to see in future versions?

What purpose does TIMESET serve for you?  (check) Business/professional  Other

Please elaborate briefly:

Thank you.  Please mail this sheet to me so I can be aware of any difficulties

or needed improvements.  I'll try to notify you of changes in TIMESET and

announce any related new programs.

Pete Petrakis

Life Sciences Editorial Services

1236 River Bay Road

Annapolis, MD 21401