Here is some information copied verbatim from the IBM technical reference for

the 3278/79 adapter.  This may be of interest if you need to get at this sucker

at a really low level.

-----------------------------------------------------------

IBM PERSONAL COMPUTER 3278/79 ADAPTER TECHNICAL REFERENCE

There are 8KB of RAM reserved for the adapter.  The beginning address is

CE000; the ending address is CFFFF.

BASE ADDRESS  REGISTER

------------  ----------------------------------------------

2D0           Interrupt/Status Register

2D1           Visual/Sound Register

2D2           Cursor Address Register (7-0)

2D3           Cursor Address Register (15-8)

2D4           Control Register

2D5           Scan Code Register

2D6           Terminal ID Register

Interrupt/Status Register - 2D0

8-bit register accessed by the PC.  Bits 0-5 are Interrupt (I) bits, set

by the adapter and reset by the PC.  Bits 6 and 7 are Status (S) bits,

set and reset by the adapter.  Interrupt Level-2 is used by the adapter.

BIT   I/S    FUNCTION

---  -----   --------------------------------------------------

0     I     Keystroke Accepted

1     I     Reset Command Decoded

2     I     Visual/Sound Register Updated

3     I     Reserved

4     I     Base Buffer Modified Complete

5     I     Load I/O Address Command Decoded

6     S     Buffer Being Modified

7     S     Interrupt Generated

Visual/Sound Register - 2D1

8-bit register set and reset by the adapter except for the sound alarm

(bit 6) which is reset by any Write operation from the PC to this

register with any data.  No other bits are affected by a Write operation

from the PC to this address.  Each time this register is loaded by the

adapter logic, bit 2 of the Interrupt/Status Register is set.

BIT  FUNCTION                                            FRAME BIT

---  --------------------------------------------------  ---------

0   0 = 80 characters per line                             3

1   Inhibit Feature Step of Address Counter                5

2   Inhibit Display                                        6

3   Inhibit Cursor                                         7

4   Reverse Cursor                                         8

5   Blink Cursor                                           9

6   Sound Alarm

7   Enable Clicker

Cursor Address Register - 2D2, 2D3

16 bit register internal to and set by the adapter.  This register

requires two addresses and can be read by the PC.  This register

contains the current value of the I/O Address Counter.  In 3278/79

emulation mode, this register indicates the location of the cursor.

Control Register - 2D4

This is an 8-bit Read/Write register internal to the emulation adapter

and accessed by the PC.  It requires one address to read and write.

BIT  FUNCTION

---  --------------------------------------------------

0   Coax Enable

1   Reserved

2   Reserved

3   Keystroke Available

4   Reserved

5   Test

6   Reserved

7   Disable Interrupt

Bit 0 - Set by the PC, causing the adapter to become enabled and on-

line.  After this bit is set, the first POLL causes a POR

response to be sent to the control unit.  The bit is initially

reset during hardware power-on.  It is also reset by the RESET

command or by the PC.  When this bit is reset, the adapter

ignores all transmissions from the control unit.  Thus, it

causes the control unit to timeout and set the "GO TO SLEEP"

latch for that line.

Bit 1 - Reserved.

Bit 2 - Reserved.

Bit 3 - Set by the PC, notifies the adapter that the Scan Code Register

has been loaded.  This data is then handled as a keystroke.

This bit is reset by a POLL ACK or RESET command from the

control unit.

Bit 4 - Reserved.

Bit 5 - Set by the PC, allows the PC to do limited functional

testing of the adapter.  The coaxial link is automatically

disabled.

Bit 4 - Reserved.

Bit 7 - Initially reset by the hardware power-on cycle.  Then it is set

and reset by the PC.  Setting this bit inhibits the sending of

interrupts to the PC, but it does not prevent setting the

interrupt/status bits (0-5).  If an interrupt is set when the

Mask bit is on, subsequent removal of the Mask bit will allow

that interrupt to pass through to the PC.

Scan Code Register - 2D5

8-bit register internal to the adapter, set and reset by the PC.  This

register requires one address.  In 3278/79 emulation mode, this

register is used by the PC to send a keystroke code to the control unit.

The PC will not change the contents of this register if bit 3 of the

Control Register is set.  The scan code placed in this register must be

the ones-complement of the data to be sent to the control unit.

Terminal ID Register 2D6

8 bit register internal to the adapter, set and reset by the PC.  It

requires one address.  This register must be set by the PC before

enabling the coaxial link.  The Terminal ID placed in this register must

be the ones-complement of the data to be sent to the control unit.

Commands sent from the Control Unit and recognized by the Adapter

-----------------------------------------------------------------

Note: The adapter will respond to all good transmissions.

Coaxial

Frame Bits

COMMANDS                                          5 6 7 8 9

------------------------------------------------  ----------

READ    Poll (include priority)                   0 0 0 0 1

Read Data                                 0 0 0 1 1

Read Address Counter High                 0 0 1 0 1

Spare                                     0 0 1 1 1

Read Terminal ID                          0 1 0 0 1

Read Multiple                             0 1 0 1 1

Read Status                               0 1 1 0 1

Spare                                     0 1 1 1 1

Poll Acknowledge                          1 0 0 0 1

Spare                                     1 0 0 1 1

Read Address Counter Low                  1 0 1 0 1

Spare                                     1 0 1 1 1

Spare                                     1 1 0 0 1

Spare                                     1 1 0 1 1

Spare                                     1 1 1 0 1

Spare                                     1 1 1 1 1

WRITE   Reserved                                  0 0 0 0 0

Reset                                     0 0 0 1 0

Load Address Counter High                 0 0 1 0 0

Clear                                     0 0 1 1 0

Spare                                     0 1 0 0 0

Load Control Register                     0 1 0 1 0

Write Data                                0 1 1 0 0

Insert Byte                               0 1 1 1 0

Search Forward                            1 0 0 0 0

Search Backward                           1 0 0 1 0

Load Address Counter Low                  1 0 1 0 0

Load Mask                                 1 0 1 1 0

Spare                                     1 1 0 0 0

Spare                                     1 1 0 1 0

Spare                                     1 1 1 0 0

Escape                                    1 1 1 1 0