AT HARD DISK DIAGNOSTIC

ATDIAG.COM IS A HARD DISK FORMAT PROGRAM FOR THE IBM-PC 'AT' ONLY!  It was

uploaded to this system and has been tested on my AT in every manner that I

could think of.  It was written as an alternative to IBM's Advanced Diagnostic

program costing $275.  As with any program having the ability to write to a

storage device, you should have a good idea what you desire to accomplish

before attempting to use it.  IT HAS THE ABILITY TO DESTROY ALL DATA ON THE

DISK!

The program is menu driven with the ability to capture error data to a printer.

The function keys are used to select options to be performed, F1 - F7 are

'safe' keys.  They will run drive checkout tests on the test cylinder, prepare

the hard disk for relocation and scan all tracks for data errors.  F8 will do a

surface analysis on the disk, destroying all data on the drive (after a warning

prompt which gives you an 'out').  F9 will format the disk giving you the

option to either retain the currently flagged defects or to try to reclaim

them.  If you reclaim them, I suggest you run the surface analysis after the

format to reflag the truly defective tracks.  In my testing a previously

flagged track was not always reflagged leading me to the believe that some

marginal track conditions may not be caught (either by ATDIAG or by the IBM

Adv.  Diags.) hence the last function key exists.  F10 will allow flagging of a

specific sector from user supplied data.

My personal experience with ATDIAG.COM has been favorable.  My AT has 3 tracks

flagged by the manufacturer (IBM supplied CMI drive), they are (cyl-hd) 049-3,

050-0 and 150-3.  Both ATDIAG and IBM Adv.  Diags.  flag cyl 049 and 150,

neither flags 050 leading me to believe that this track is very marginal.  I

reflagged it with ATDIAG.  You will still need to run FDISK from DOS to create

the DOS partition and then DOS FORMAT to build the file allocation table,

directory and transfer the system to the hard disk.  In the final analysis;

1) The program will do what it claims, and 2) It doesn't cost $275.00.

It also has a distinct advantage over the IBM Advanced Diagnostics, in that it

formats from the lower numbered tracks to the higher ones.  This means that

drives having fewer cylinders than those for which IBM has made provision can

be formatted easily.

Documentation by:

Gene Lowry - Sysop

Bigfoot's BBS  (408) 225-1845

San Jose, CA.

8 February 1985

Minor changes by:

DeForrest Home

12 MAY 1985

########################################################################################################################