Changes in X.PC version 2.01

The changes made for X.PC revision 1 of release 2 consist of the

patches to revision 0, and new error codes to inform the applica-

tion of a failure at the physical, link, and packet levels.  You

may now request the channel state on channel 0, which may return

the call cleared (code 8) state.  If this state is returned, then

the following new call clearing codes apply:

10 = Reset Packet (one channel is reset, the "call" is cleared)

------ the following only appear on channel 0 ------

11 = Restart Packet (all channels are reset, calls are cleared)

12 = Restart Ending (remote X.PC is going into character mode)

13 = Remote X.PC Lost (no packets received in the past 32 sec.)

14 = MODEM Status Lost (DSR and/or CR has gone to "off" state)

Restart 11 may be a problem, since it will always be received when

X.PC goes into packet mode.  The application should read this when

it has completed entry to packet mode, then channel 0's call state

should remain disconnected (0) until one of the above exceptions

occurs.  The code 10 should be software compatible with earlier

X.PC applications; and such applications ignore channel 0, so the

new codes should also be software compatible.

The TYMNET X.PC application, APP, will now display the new call

state codes, if Call Status for channel 0 is requested, while in

packet mode.  Only in character mode will the application check

channel 0 automatically, so you could see MODEM Status Lost on the

screen without making a special request.

MODEM Status Lost is only returned when communication output is

attempted, and the DSR or CD status bit is found to be off, after

it had been on before.	If these bits are never on, they are not

required --- but in this case, we cannot detect MODEM failure.

The following files wre changed:

LAYER	FILE	   ENTRY	     DESCRIPTION

p_config.d     N.A.	new X.PC revision (1) for version 2

p_padfnc.d     N.A.	new call clearing codes for application

p_statup.e     N.A.	new status update codes for PAD layer

app p_applcl.c call_state/clear  display new call clearing messages

pad p_statup.c   chstat_upd	new call clearing codes into DCB for APP

p_driver.c   chr_stats	terminate packet mode with intr. on

p_initdr.c     (all)	add new call clearing messages

pkt packtim.c	  rr_time	test/reset pkt_recvd every 32 seconds

packirst.c   rstr_done	call PAD with new Restart codes

p_pkcomm.c   pack_read	restart all channels after RNR ends

packout.c	  purge_rr	corrected for 1 channel in rep_que

frm p_linkot.c   strt_ouframe	call to purge_rr with packet in rep_que

p_comint.asm xmit_byte	if bad MODEM status, call xmit_fail

p_chario.c   xmit_fail	added xmit_fail function

p_linkin.c   pbyte_in	if good CRC1 received, set pkt_recvd TRUE

xys p_timque.c   set_time	if function address = 0, call abort