GT PowerComm - 11.20

Copyright (c) 1985-1987: by P & M Software Co.

Host Mode Documentation

by: P & M Software Co.

9350 Country Creek #30

Houston, TX 77036

Installation of Host Mode

-------------------------

The most important thing about Host Mode is proper installation of the

modem control strings and result codes.  These strings and codes must not

just control the modem, they MUST WORK TOGETHER.  For example, if the modem

init strings specify "verbose" result codes, then the result code table better

have the "verbose" codes in it, not the "terse" codes.  There are 6 specific

areas that must be installed; all of these options and strings are accessible

via the ALT-I command.  Let's discuss each of these items in turn:

1.  The "Modem Init String".   The default value is:

AT V1 Q0 E0 X1 S0=0 S2=43|

Assuming that one wants to use the default string, there is really only

one thing one SHOULD DO to this string: change the Xn command.  The most

powerful Xn command available for the modem is the one to choose.  With

a USRobotics Courier 2400 modem, use either X5 or X6.  With a normal

Hayes-compatible 1200 baud modem, use X1.  For other modems consult the

modem manual.

2.  The "Host Mode: Modem Init String".  This string is found under item

#31 of the config screen.  The default value is:

AT V1 Q0 E0 M0 X1 S0=1 S2=255|

There are several things one could do to this string.  First, one must

modify the X1 command to match the value added to the Modem Init

String above.  Secondly, if one desires the speaker to be heard during

Host Mode, remove the M0 from the string.  Thirdly, the S0=1 may be

changed to S0=0, THIS IS THE ONLY OTHER VALID VALUE!  If S0=0 is

used, the modem will not answer automatically, GT will count the rings

and command the modem to answer after the 2nd ring.  This is why S0=1

and S0=0 are the only permissible values!  In Host Mode, GT will not

let the modem ring more than twice.

3.  The Modem Answer String.  This string is located with the Host Modem

Init String under item #31 of the config screen.  The default value is:

ATA|

GT will issue this string to the modem whenever the ring count reaches

2.  This string should not be changed, unless GT should not be allowed

to answer the modem.  For example, one could erase this string and set

S0=5, so that GT could not answer after 2 rings, but the modem would

then answer on the 5th ring.  I don't advise anyone to do this.

4.  The Result Codes.  Because the modem init strings above contain V1,

"verbose" result codes should be programmed.  They can be found under

item #30 of the config screen.  There are 12 possible results.  If

the modem does not support all of these, simply leave the extra strings

in their default state.  The program comes with the default result codes

set for use with a USRobotics Courier 2400 modem.  One should consult the

manual that comes with the modem and modify this table to match the

manual.  Remember, use the "verbose" or word codes, not the "terse"

codes.  It IS POSSIBLE to use the "terse" codes, but they are not as

compatible with such services as PC Pursuit.  It is especially important

that the result codes for the CONNECT results are correct.  GT uses these

to determine the baud rate of the incoming call.  If they are wrong,

then GT will not be able to talk to the incoming caller, because the

modem and GT will not be at the same rate; they will be out-of-sync.

5.  It is highly recommended that logging be TRUE during Host Mode.

Otherwise, all of the messages to the operator and other records

of the Host Mode activities will be discarded.

6.  How to allow GT to hang-up during Host Mode.  Since the "escape code"

must be disabled during Host Mode, otherwise some caller may crash

your system, the normal hang-up string, "~+++~ATH", will not work!

Therefore, GT cannot hang-up the modem, unless you set the modem to

normal DTR operation.  During Host Mode, GT will drop the DTR signal

whenever he wants the modem to hang-up.  Most modems come from the

factory set so that they ignore the DTR signal - you must reset a

DIP switch or, in the case of the Hayes 2400 modem, you must make

an entry in the Modem init string to do this.  In any case, if the

modem ignores DTR, it will not hang up during Host Mode operations.

Host Mode Text Files

--------------------

The following text files must be created and/or maintained with an editor

that can produce plain ASCII files.

GTWELCOM.BBS    This file is displayed to a caller when he/she connects

with the GT host.  It should identify the system to

callers.

GTPASSWD.BBS    This file contains the list of passwords that people must

know to access the system.  The entries in this file are

composed of one line per password, and each line contains

the following information, beginning in column 1:

Level  Password  [First-Name  Last-Name  Phone-Number]

These items can have a variable number of blanks between

them, and the "Level" must begin in column 1.  EACH

PASSWORD MUST BE UNIQUE.

The [] which have been placed around the name and phone

number, indicate that these items are optional and need

not be included in the file.  The name and phone number

should be entered when you want to allow the named person

an optional call-back, to the number listed.  This is a

very handy option when you want the bill for the phone

call to be absorbed by the Host system.  GT will offer

the call-back, when the person calls and uses this

password.  If they are not at the proper number, the

call-back should be declined.  If the call-back is

accepted, GT will hang-up and call the listed number in

15 seconds.  To receive the call, one should have his

modem set to the auto-answer mode.  Usually this can be

done by sending this string to the modem, AT S0=1|.  The

| represents the Carriage Return character.  GT will try

3 times to complete the call-back.  If GT cannot complete

the call-back, he will return to the waiting-for-call

state.

There are 4 meaningful security level ranges. Each range,

except for the 1st, has 5 levels within so that different

directories in the GTDIR.BBS file may be assigned to

unique levels.

A ..... Highest level.  A caller with this level

will have complete access to the system.  Use

with utmost CAUTION.

B-F ... A caller with one of these levels will not be

allowed to change directories (except via the

GTDIR.BBS file authorization), or use the DOS

door.  This caller will be limited to the default

directory at the time he calls.

G-K ... A caller with one of these levels will not be

allowed to do anything prohibited to B-F, and

in addition, the caller will not be allowed to

upload files to the system.

Any other level:

A caller with any level not shown above will

not be allowed to change directories, upload or

download files on the system.  They may still

T)ype any files that are typeable and leave

messages to the operator.

NOTE: It is possible to append a single character to

the level indicator.  Thus "A1" would be a valid

level.  This extra character is used to identify

a custom bulletin file for this caller.  This

extra character is optional, but if present

MUST BE a character that is legal within a file

name.  Read about BULLET files below.

GTBULLET.BBS    This file is displayed for the caller after he/she

completes login.  It is useful to leave notes here

for expected callers.

BULLETx.BBS     The "x" maybe any character that can be a legal part of

a filename.  This character must match the character

which is found after the level indicator in the password

file, to enable custom bulletin files to be displayed

for callers.  For example: if a caller's level was "BZ",

then the program would search for a file named

BULLETZ.BBS to display whenever the caller logged onto

the program.

NOTE: Custom bulletin files are OPTIONAL.  If you do

not wish to use them, simply use single letters

for level indicators.

GTMENU.BBS      This file is displayed for callers as the main menu.

It may be suppressed by a caller by selecting X)pert

mode.

GTHELP.BBS      This file is displayed for the caller if he/she requests

help with the main menu.

GTBYE.BBS       This file is displayed for the caller when he/she logs

off, i.e. selects the G)oodbye command from the main

menu.

GTUSER.BBS      This file is created whenever a call comes into the

system.  It will contain the security level and name

of the current caller, or if no call is in progress, the

previous caller to the system.  The file contains just

this 1 line of text.

GTDIR.BBS       This file contains a list of directories that may be

accessed by callers.  There are two types of lines

in this file: the actual directory lines and comment

lines.  The comment lines are displayed to the caller

when he askes to see the list of directories.  The

list displayed will contain only those directories the

caller is authorized to access.  The formats:

Comment Lines:  contain a blank in the 1st column,

the remainder of the line is displayed

for the caller.

Directory Lines: Column 1 - Security Level. The minimum

level required to access the

directory.  If this column

contains an "=" then the

actual line is shifted to the

right 1 column and the "="

acts to reserve the directory

for the indicated security

level - which would now be

in column 2. (See example.)

One or more blank columns follows the

level, then the complete DOS name of

the directory, including all drive

and PATH information.  One or more

blank columns follows the directory

name, then a description of the

directory is given.

Example:

This is a comment.

A C:\DOS\TEST The description goes here.

=B C:\LOTUS\DATA This is for "B" level callers only.

C A:\ The description again goes here.

This example shows three directories, one with an "A" security level, one

with a "C" security level.  These are the minimum levels that callers must

have to "see" these directories.  The "=B" directory may only be accessed

by a caller with the "B" security level.

Note: when Host Mode is started, the then default directory will be made

available to all callers, regardless of the contents of GTDIR.BBS.

So it is advised that this directory require the least security and

that it be the 1st entry in the GTDIR.BBS file.

Running Host Mode

-----------------

Once installed, Host Mode is fairly self-explanatory, but here is a quick

once over.

Host Mode is started by pressing "ALT -".  The first thing one should notice

is that GT has automatically entered "Half duplex".  This is so that anything

typed on the Host console will echo so that it can be seen.  GT will set up the

split screen, which is the normal Host Mode screen.  Then GT will send the Host

Mode Modem Init String to the modem and wait for a call.  While GT is waiting

for a call, there are 2 commands available: [Esc], which will terminate Host

Mode and carriage return, which allows one to logon to GT's Host Mode from the

local console.  This is normally done to test out the text files.

Once GT has answered an incoming call, there is one command available to the

operator, [Ctrl-X], which will cause the current caller to be disconnected.

If the caller selects the P)age Operator command, the page is answered by

pressing any key.  The [Ctrl-Z] key will end the chat with the caller.

That's really all there is to operating Host Mode.

Host Mode LOG

-------------

A few sections above, I suggested that logging be TRUE, while GT is running in

Host Mode.  Here is why:  during Host Mode operations, GT will log all of the

calls received, who called and the password used.  Also, the LOG will

contain any messages left by the callers, and a complete description of all

file transfers.  I consider the gathering of this information to be critical

to the operation of a GT Host.

In fact, this information is so valuable, that Mr. James Davis of Houston

has written a companion program called GTLOG, which does an excellent job

of analyzing the GT.LOG file.  I recommend the GTLOG program and, if it is

useful, I suggest that one let James know.

The DOS Door

------------

The DOS Door is like a DOS Shell, except that it is for remote callers,

instead of the local operator.  When the "Remote DOS Door" option of the

main menu is selected, GT will shell to the file GTDOOR.BAT.  This file

will set up for the remote shell by executing the proper CTTY command, then

execute a secondary DOS shell and let the user into the system.  He will

have the console, so be cautious!  The caller will be instructed to issue

the "EXIT" command to return to GT.

While the DOS Door is open and the caller is out in the system, GT will not

be idle.  GT will act as a watchdog.  If the caller should hang up while the

DOS Door is open, GT will force the system to re-boot and will drop the DTR

signal to the modem.  If commands to start GT are placed in the AUTOEXEC.BAT

file and a script is used to start Host Mode, then GT will automatically

recycle.  The script command to start Host Mode is simply HOST.

Note: For this process to work correctly, the COMMAND.COM file must

reside in a directory pointed to by the PATH environment variable.

In order for this to work on a normal drive C: hard disk, "C:\" should

be added to the PATH variable.  For example:

PATH=c:\dos;c:\lotus;c:\

This will ensure that the COMMAND.COM file can be located for execution

by the GTDOOR.BAT runstream.

Using the Remote DOS Door

-------------------------

What can one do, once one is running DOS remote via a CTTY command?  Well,

one can't use any of the Fn keys or the other special keys on the IBM key-

board!  One can't run any program that does direct hardware control of the

screen or keyboard.  All screen and keyboard input must be done through DOS.

Well, then what can one do?  One can run any DOS command; for example COPY,

ERASE, DIR and others, including EDLIN, all work properly.  In fact, any

program that uses the standard DOS handles for input and output to the console

will work.  But one still won't have F1 - F10 or the other special keys.  Well,

that's not quite so...if the Host Mode computer is set up so that the ANSI.SYS

device driver is loaded via the CONFIG.SYS file.  Just put a line like this one

into the CONFIG.SYS file:

DEVICE=ANSI.SYS

Naturally, ANSI.SYS must be located in the root directory of the boot disk.

After that has been done, re-boot the computer.  ANSI.SYS will then be loaded.

One can now re-map the keyboard so that the special keys are mapped to Ctrl keys

instead.  This will enable programs to be run that use these keys; for example

EDLIN uses the arrow keys as well as several function keys.

How does one re-map keys with ANSI.SYS?  The following sequence must be issued

for each key that needs to be re-mapped:

ESC [ ##;##;##;...p

The (...) indicate that the sequence may be repeated as needed, but each

sequence only re-maps one key.  Once a file containing these mapping commands

has been created, they can be sent to ANSI.SYS by TYPEing the file.  The "##"

in the sequence above indicate the ASCII codes for the keys to be mapped.  For

example, let's map F3 to the Ctrl-D key.  The ASCII codes for F3 are 0,61

and the ASCII code for Ctrl-D is 4.  (Note: the special keys have two codes -

the first is always 0.  These codes are really called extended ASCII codes.)

Therefore, the proper ANSI.SYS command to re-map F3 to Ctrl-D would be:

ESC [ 4;0;61p

Note: Blanks are included for readability only, the "p" must be lowercase,

and the symbol ESC stands for the escape character, CHR$(27).

Here is a handy list of extended ASCII codes:

Key       Codes                 Key            Codes

---       -----                 ---            -----

F1        0,59                  Right Arrow    0,77

F2        0,60                  Left Arrow     0,75

F3        0,61                  Up Arrow       0,72

F4        0,62                  Down Arrow     0,80

F5        0,63                  Home           0,71

F6        0,64                  End            0,79

F7        0,65                  PgUp           0,73

F8        0,66                  PgDn           0,81

F9        0,67                  Ins            0,82

F10       0,68                  Del            0,83

Here are some actual ANSI sequences!

Escape Codes         Meaning to ANSI.SYS

------------         -------------------

#[4;0;61p           F3          -> Crtl-D

#[5;0;62p           F4          -> Ctrl-E

#[19;0;77p          Right Arrow -> Ctrl-S

#[1;0;75p           Left Arrow  -> Ctrl-A

With these kinds of mappings, almost any program should be able to run, if it

uses the DOS handles for standard input and output.  This should make the

"Remote DOS Door" quite useful.

#