A MEMORY RESIDENT

FLOPPY DISK PROGRAM

by

Michail J. Karas

This   is   from   the  Lifelines/The   Software   Magazine,

September1982 issue,  about ADD-ON MODULE Construction. An add-on

module  is designed for use under single user  CP/M-80  2.2.  The

next several paragraphs, from the previous articles, January, and

February  1982,  show  the  procedure  to get a  version  up  and

running.

The source file,  RAMDISK.ASM,  being very similar to a BIOS  for

CP/M-80 2.2, requires the extension disk definitions to have disk

parameter  tables,   check  vector  and  allocation  vectors.  In

addition,  a directory buffer must be allocated. An ADD-ON MODULE

generates  the  appropriate tables through the use of  the  macro

library  DISKDEF.LIB provided by Digital Research on the  CP/M-80

2.2 distribution diskette. This macro capability requires a macro

assembler to properly process the DISKDEF macros.

An  additional implementation requirement is the availability  of

the  assembler  output  as  a .REL  file.  The  Digital  Research

Relocating  Macro Assembler RMAC is just the tichet  to  generate

the  appropriate  .REL  file and process  the  DISKDEF.LIB  macro

includes.  The  command structure below shows how to assemble  an

ADD-ON  MODULE program like the RAM disk demo listing on the disk

as RAMDISK.ASM. The following is an example on the assembly.

A>RMAC RAMDISK $+S PZ<cr>

The resulting .REL file has to be converted to a page relocatable

format.  After agonizing over the problem of easily making a "BIT

MAP",  I found that the Digital Research Link program distributed

with  RMAC and PL/I-80 can generate page relocatable files  (.PRL

type) which are compatible with MP/M-80. After reviewing the LINK

output format of a .PRL file,  I designed the PRLMOVE program  to

relocate the LINK output files. The command structure below shows

how to produce the required .HEX file from PRLMOVE.

A>MAC PRLMOVE $+S PZ<cr>

The  command  structure below shows how to produce the .PRL  file

from the RMAC output.

A>LINK RAMDISK[OP]<cr>

where  [OP] directs LINK to generate the desired .PRL  file.  The

program PRLMOVE.HEX and the page relocatable RAMDISK.PRL must  be

combined into a single executable command file as follows:

A>DDT<cr>

DDT Vers. 2.2

-IRAMDISK.PRL<cr>

-R<cr>                   <-Load  of .PRL file to RAM to  address

0100H  with  code image  starting  at

address 0200H.

MEXT PC

nnmm  0000               <-Convert nn to decimal  and  remember

value.

-IPRLMOVE.HEX<cr>

-R<cr>                   <-Read  PRLMOVE program in  over  the

.PRL file at load address of 0100H.

-G0<cr>                  <-Exit DDT to CP/M-80.

A>SAVE dd RAMDISK.COM    <-Save dd pages of memory to get command

file. dd = converted nn from above!

This  results  in  the  .COM command file necessary  to  make  an

executable module out of the ADD-ON MODULE.

The present design of the ADD-ON MODULE with the negative  offset

pointer calculation reentry to CCP requires that the BDOS and CCP

combination  be genuine (REAL Digital Research CP/M-80 Ver 2.2 of

total length 01600H bytes. If your system uses a modified BDOS or

CP/M-80 system look-alike,  then the negative offset to the stack

pointer will require a different value. See NOT$PRES: in listing.

Also,  for non-genuine CP/M-80 systems, if the CCP is a different

size  (not  0800H)  bytes,  the program PRLMOVE will have  to  be

modified  to set down the relocation load address calculation  by

an amount equal to the difference in the alien CCP size.

################################################################