\JC

X120 BOARD ERRATA

\JL

*****  IMPORTANT NOTE  *****    One signal has accidently been

overlooked on the X120B double density board.  This is the 8" HEAD

LOAD signal that returns to the 820 board.  In its present state

the heads on any 8" drive that is connected to the J1 (37 pin)

connector on the 820 board will stay loaded. To correct this,

simply place a small jumper wire between pin 28 of the 2791 (2793

maybe) and pin 28 of J1 on the X120 board.  This will restore

normal head load operation to your 8" drives.  This jumper is only

necessary if you plan to connect your 8" drives using the 37 pin

connector at J1 on the 820 board.  The signal is already correctly

implimented on the X120 J4 connector. It is also unnecessary to

install the TEST jumper (F) for normal operation, as the 279x

chips have an internal pull-up resistor.

8" DOUBLE DENSITY OPERATION        You need to upgrade your clock

rate on your XEROX 820 board to 4 or 5 Mhz to be able to run your

8" drives on double density. This is not necessary to run 5 1/4"

double density.  This is because the data transfer rate for 8"

drives is twice as fast as for 5 1/4" drives, and the processor

can't quite keep up at 2.5 Mhz.  The conversion to 5 Mhz is fairly

simple, since all of the necessary signals are already present on

the board, providing of course that the components on your board

are fast enough to keep up.  Most of the factory assembled boards

have RAM that is fast enough, but many of the Z80 components (PIO,

SIO, CTC, & CPU) are not. You may find that replacing these chips,

though not very expensive, is more work or more complicated than

you want to undertake.  The CPU should be a 'B' rated part, with

the rest being at least an 'A' rating.  The actual modication is

this:

\I15

Cut the CAS trace between U11 pin 5 and U25 pins 11 or 13

Cut the MUXC trace between U11 pin 4 and U71 pin 1

Cut the 2.5 Mhz trace between U10 pin 4 and U9 pin 3

Jumper U11 pin 4 to U25 pins 11 and 13

Jumper U11 pin 3 to U71 pin 1

Jumper U10 pin 5 to U9 pin 3

\I10

This completes the 5 Mhz modification.  The modification to run 4

Mhz is the same, but you will need to replace your 20 Mhz crystal

with a 16 Mhz, and change your floppy disk clock rates by removing

U12, and jumpering U12 pin 8 to U10 pin 5, and U12 pin 9 to U10

pin 4.  The 4 Mhz modification seems to be a bit more touchy than

the 5 Mhz, and I recommend that you only use it if you know that

the parts in your board will only make the 4 Mhz rate.

GOOD LUCK,  BG  10/2/84 (revised 12/5/84)