\JC

NOTES ON VERSION 2.XX ROMS

\JL

Version 2 ROMs (2.00, etc.) will operate slightly differently from

the description in the manual.  The most immediate difference is

their ability to auto-boot.  The rom only signs on with "XPRO

SYSTEM MONITOR VERSION 2.00", then waits for an input from the

keyboard or disk drives.  To auto boot, place a disk in drive 'A'

and close the door, and the disk will boot CP/M.  To use the

mini-monitor, leave the drive empty, or the drive door open, and

hit carriage return. The mini-monitor menu should appear on your

screen, similar to the display shown in the manual.  You may use

the commands in the same manner as listed in the manual, and boot

using the 'B' command.  This command will boot from either 5 1/4"

or 8".  The auto-boot option is useful for those of you using a

serial terminal as a console device, those running bulletin boards

with your system, and for those times that using the keyboard to

initiate a boot is a nuisance.  To use the serial port as a

console device, change the IOBYTE assignment in your XBIOS.AZM to

the assign the CON: to TTY: (see your CP/M manual, bits 0,1 will

be 0; the IOBYTE is presently set to 81H or 01H; change it to 80H

or 00H).

If you plan to try the rom out before you make your double density

installation, you will need to be aware of one item that may cause

it to look like it's not working.  This is the redefinition of bit

6 on the system parallel port.  This bit was used by Xerox to

blank the screen without changing the video memory.  We are using

it as a MOTOR ON signal for the 5 1/4" disk drives.  The system

will come up with the drives running in order to auto-boot. After

an input comes from the keyboard, the MOTOR ON signal is turned

off, which has the result of blanking your screen.  To remedy this

pin 3 of U104 (74LS14) must be disconnected from pin 8 of U105

either by cutting the trace on the board, or removing pin 3 of

U104 from its hole in the board (or socket). Pin 3 of U104 should

then be grounded by jumpering it to pin 7 of U104.  The

mini-monitor menu should work correctly after this mod is done.

The rom is also sensitive to the polarity of bit 7 (the highest

bit) from your keyboard.  If bit 7 is pulled high or floating (not

connected to anything), the rom may go catatonic on you while you

are trying to use the mini-monitor.  This is generally not a

problem after you boot up, since the XBIOS is capable of handling

8 bit ASCII from the keyboard. To make sure that the mini-monitor

works correctly, bit 7 should be grounded at you keyboard, or at

pin 11 of U104 on the Xerox board. Be careful not to short the

outputs of a chip on your keyboard. (This should work correctly in

later revisions of the rom)

GOOD LUCK,  BG  12/24/84 (revised 1/16/85)