SOS INSTALLATION GUIDE

The SOS  distribution  medium  contains  the  following

files:

1)   INSTAL.DOC - this text

2)   SOS.TSK

3)   SOSBIG.TSK

4)   SOS.HLP

5)   SOSUG.DOC

6)   RSX.RTS

SOS runs as  a  non-priviledged  utility  program  from

account  [1,2].  Two versions of the task file are included:

SOS.TSK and SOSBIG.TSK.  SOS.TSK  is  an  overlayed  version

that  runs  in 11K, SOSBIG.TSK is not overlayed and requires

24K. The performance differs  only  slightly  during  normal

editing,  but the overlayed version has much higher overhead

during initialization and termination.  The choice of  which

to  use  is  dependent  on  whether  the limited resource is

memory or disk accesses.

Two   documentation   files   are   included   on   the

distribution  tape.   A short help file, SOS.HLP, contains a

brief  description  of  the  commands,  and  is  printed  in

response  to  the  HELP  command  in  SOS.   A more complete

document, SOSUG.DOC, explains each command  in  more  detail

with all the options and parameters that are available.

The program runs under the RSX run-time system;  a copy

is  included  in  this  kit  for  convenience.  If it is not

already installed on the system, do the following:

RUN $PIP

[0,1]RSX.RTS=DT:RSX.RTS/CO:T

^Z

and add the following to the START.CTL sequence:

RUN $UTILTY

ADD RSX

^Z

The files should be  copied  into  the  system  library

account with the following sequence of commands:

RUN $PIP

[1,2]SOS.TSK=DT:SOS.TSK/CO:T

[1,2]SOSBIG.TSK=DT:SOSBIG.TSK/CO:T

[1,2]SOS.HLP=DT:SOS.HLP/CO:T

Page 2

[1,2]SOSUG.DOC=DT:SOSUG.DOC/CO:T

=[1,2]SOS.TSK<104>/RE

=[1,2]SOSBIG.TSK<104>/RE

=[1,2]SOS.HLP<40>/RE

=[1,2]SOSUG.DOC<40>/RE

^Z

The  CCL  command  may  be  added  into  the  START.CTL

sequence:

RUN $UTILTY

CCL SOS=$SOS.TSK;0

or

CCL SOS=$SOSBIG.TSK;0

^Z