User's Guide for $PLOT10 and $PLOTHP

Programs for  creating  figures  on

the  Tektronix  4010-1 terminal and

HP7200 plotter.

by J. Krupp

Middlebury College

Middlebury Vermont

$PLOT10 and $PLOTHP User's Guide                           Page 1

Version 6A-00                                         19-Aug-77

INTRODUCTION

============

This document describes a pair of programs which  allow  the

user  to create figures on the Tektronix 4010-1 terminal and then

to reproduce them on the HP7200A plotter.  Each program  consists

of  a  set of BASIC-PLUS user-defined functions.  After writing a

program using these functions, following  the  directions  below,

the  user  then  appends  the program $PLOT10 for plotting on the

Tektronix terminal or $PLOTHP for plotting on the HP plotter.  No

modification to the user's program is required in converting from

one set of plotting functions to the other.

The user creates a plot or figure by invoking any  of  eight

user-defined  functions.   These  functions perform all the basic

operations needed for creating plots, namely:

FNT0% --- Initializes  plotting  parameters;   optionally  clears

screen

FNT1% --- Scales user data to plotting dimensions

FNT2% --- Draws curves from user data

FNT3% --- Graphic input from the terminal ($PLOT10 only)

FNT4% --- Draws a labeled axis at a given position and angle

FNT5$ --- Creates a character string from a given number

FNT6% --- Draws a character string of given  height  at  a  given

position and angle on the screen

FNT7% --- Similar to FNT2%, except points measured in inches, not

the user's units.

A detailed description of how to use each of these functions

is presented below.  It is assumed that the user is familiar with

programming in BASIC-PLUS and knows how he  wishes  to  configure

the  plot.   The  use of these functions should allow the user to

concentrate on the logic of his program and not  the  details  of

how  the  Tektronix  terminal  or  HP  plotter work.  Each of the

functions is described below in terms of  its  implementation  in

the  $PLOT10  program.   Minor  differences in these functions as

implemented in $PLOTHP are described separately afterwards.

$PLOT10 and $PLOTHP User's Guide                           Page 2

Version 6A-00                                         19-Aug-77

Basic Characteristics of the Tektronix 4010 Terminal

====================================================

The user needs a general awareness of  the  capabilities  of

the  terminal  before  he  can  adequately plan his figures.  The

terminal operates in two modes -- alpha and graph. In alpha mode,                                  _____     _____

the  terminal operates like an ordinary terminal;  35 lines of 74

characters can be displayed.  Rather than scrolling  like  a  CRT

when  the  display is full, information is over-printed beginning

at the top of the screen with a margin at column 35. This is done

because  the  display  is a storage tube, rather than a refreshed

CRT (see Tektronix technical literature for more on this  point).

The  printing  of  information  can  be  interrupted  by typing a

'CTRL/S' and resumed with a 'CTRL/Q' (this is a function  of  the

RSTS/E  operating  system, not the terminal hardware). The screen

can be cleared by typing the PAGE key.  NOTE:  The PAGE key  must

be  struck  after  the  terminal  is first turned on to clear the

"fully written" condition of the screen.

In graph mode the screen can be thought of as a grid of 1024

points horizontally, by 780 points vertically;  the resolution is

approximately 140 points per inch.  Figures are made by 'drawing'

a series of vectors on the screen.  Vectors may be dark or light.

A light vector is simply a line  connecting  two  points  on  the

screen.   A  dark vector does not appear on the screen.  In other

words, to draw a line from point A to point B, the terminal draws

a  light vector;  to move from point A to point B without drawing

a line, the terminal draws a dark vector.   The  terminal  has  a

crosshair  feature  which,  when  activated, can be positioned by

turning the thumb screws to  the  right  of  the  keyboard.   The

terminal  is  capable  under  computer  control  of  sending  the

location  of  either  the  cursor[1]  or  the  crosshair  to  the

computer.   The  functions  in  $PLOT10  give  the  user complete

command over these capabilities.

IMPORTANT NOTE: The method by which the Tektronix screen holds an_________ ____

image  could cause a permanent residual image on the screen if it

were maintained for  a  long  period  of  time.   To  avoid  this

problem,  the  screen automatically dims after a 90 second period

in which no information is sent  or  received  by  the  terminal;

simply  depressing  the  SHIFT  key will restore the image.  This

feature is operative only in alpha mode, so that when a figure is

completed  it  is necessary to return the terminal to alpha mode.

Alpha mode is easily detected by  the  presence  of  a  pulsating

cursor  on the screen;  this cursor is not visible in graph mode.

Most of the functions below return the  terminal  to  alpha  mode

before  returning  control to the user's program.  In particular,

FNT7%() with the parameter F%<=0% can always be used as the  last

function  call in a plotting sequence to assure that the terminal

is left in alpha mode.  Alpha mode can also  be  entered  at  any

time by striking the RETURN key on the terminal.

----------

[1]  The  cursor  is  the  location  on the screen where the next

character will be printed (in alpha mode) or line will  be  drawn

from (in graph mode).

$PLOT10 and $PLOTHP User's Guide                           Page 3

Version 6A-00                                         19-Aug-77

Writing a Program to Plot

=========================

To create a figure using these plotting functions, the  user

writes  a  program which makes the necessary computations for the

figure and combines these computations with the  proper  plotting

functions  as demonstrated in the sample program of the Appendix.

Once this program is written and  SAVEd,  the  user  appends  the

plotting functions by typing

APPEND $PLOT10

for the Tektronix functions, or

APPEND $PLOTHP

for  the HP version of the functions.  It is recommended that the

user not SAVE the program with the APPENDed functions;  rather he

should  SAVE  only the portion he wrote, and APPEND the functions

each time.  This is far more economical in terms of disk storage.

The only restrictions on the user program are as follows:

1)  The user's program can contain no line numbers in the  range

10000 to 10999, the line numbers used in $PLOT10.

2)  The user should not use any variable or array name beginning

with  the  letter 'O', except for the special variables 'O1'

and 'O2' to be described below.

3)  The user cannot have any file open on  channels  11  or  12.

that  is, no statements of the form 'OPEN F$ AS FILE 12' can

appear  in  the  user's  program.   No  references  such  as

'PRINT#11...' are allowed either.

Each function has from 1 to 5  arguments  which  are  to  be

supplied by the user when the function is called.  Arguments must

match the type specified.  In particular, if an argument  in  the

descriptions  below is indicated as an integer type variable (the

'%' following the name), then an  integer  variable  or  constant

must  be  used  in  that  position  when  invoking  the function.

Likewise, if the variable is of floating point type (e.g., 'X0'),

then all constants must explicitly contain a decimal point (e.g.,

use '3.' rather than '3'). To call the function, a statement such

as

100 P=FNT0%(1.,1.,1%)

can  be  used.   The  value  returned by most of the functions is

meaningless, and the assignment statement is used as a convenient

way  to  invoke  the  function  (however,  FNT3%() and FNT5$() do

return a meaningful value as explained  below).   Note  that  the

functions  perform  all  necessary input/output operations to the

terminal.  A statement like 'PRINT FNT4%(X,Y,A,H,L$)' would cause

unpredictable results and is an incorrect use of these functions.

A detailed description of each of the functions follows;   a

sample  program  using  these functions is listed in the Appendix

and may be run on the Tektronix terminal by typing:

$PLOT10 and $PLOTHP User's Guide                           Page 4

Version 6A-00                                         19-Aug-77

OLD $TEKDEM

Ready     _____

APPEND $PLOT10

Ready     _____

RUN

When using these functions to plot on the terminal, the user

may  detect  the  plot  being  created  in  'bursts'. This occurs

because points are stored internally in the program and then sent

to  the  terminal.   To  make sure all points given to a plotting

function are actually sent to the terminal, this internal  buffer

can  be  dumped  by  using  any  of the functions that return the

terminal to alpha mode.  In particular, if  the  terminal  is  in

alpha  mode,  then  no  points are stored in the plotting program

buffer.

This document pertains to  Version  6A-00  of  the  plotting

programs, dated on or after 23-Mar-76. The current version of the

program in the library can be determined by listing line 10999 of

either program.

$PLOT10 and $PLOTHP User's Guide                           Page 5

Version 6A-00                                         19-Aug-77

FNT0%(X0,Y0,F%) -- INITIALIZATION / CLEARING THE SCREEN

===============

This function with ABS(F%)=1  must  be  the  first  plotting

function   called   by  the  user's  program.   It  is  used  for

initializing various parameters in the  plotting  routines.   The

point  (X0,Y0)  is  the  physical  origin of the plot measured in

inches from the lower left corner of  the  screen.   The  integer

variable F% is a flag which is interpreted as:

ABS(F%) = 1%  Initializes all plotting  function  parameters.   A

call  of  FNT0%()  with ABS(F%)=1% must precede any                                                 ____

other plotting function call.

ABS(F%) = 2%  Used to reset the values of X0 and Y0. Cancels  any

scaling  previously specified by use of the FNT1%()

function below.

F%  = 0%  Clears the screen  and  moves  the  cursor  to  the

"HOME"  position  at  the  upper left corner of the

screen.  Scaling is left  intact  and  (X0,Y0)  are

ignored for this case.

For positive values of  F%,  the  screen  is  cleared;   for

negative values of F%, the contents of the screen are unaffected.

The terminal is always placed in alpha  mode  by  this  function.

The  significance  of  the  values  X0 and Y0 in this function is

twofold.  All references to positions on the screen  measured  in

inches  are  made  relative to the value of X0 and Y0 in the last

call to FNT0%(). For example, if a call to FNT0%() uses values of

X0=2  and  Y0=3,  then  a subsequent call to the labeling routine

such as

P=FNT6%(2.,-1.,0.,H,'LABEL')

would cause the word 'LABEL' to be printed starting at a point  4

inches to the right and 2 inches up from lower left corner of the

screen (see description of FNT6%() below for further  details  on

this  point).  The values of X0 and Y0 are also closely connected

to the values used in  the  scaling  function  FNT1%()  described

next.

$PLOT10 and $PLOTHP User's Guide                           Page 6

Version 6A-00                                         19-Aug-77

FNT1%(P0,Q0,P1,Q1)  --  SCALING

==================

This function is used  to  scale  the  user's  data  to  the

physical size of the screen.  The parameters are as follows:

P0,Q0   The origin in user units, i.e., the value of  the  user's

data at the point (X0,Y0) used in the call to FNT0%.

P1,Q1   The  increment  in  the  user's  data  per  inch  in  the

horizontal and vertical directions, resp.

If this function is  not  called  by  the  user,  the  other

functions assume the user's units are the terminal's units, i.e.,

P0 and Q0 are zero, P1 and Q1 are 140 (units/inch).

$PLOT10 and $PLOTHP User's Guide                           Page 7

Version 6A-00                                         19-Aug-77

FNT2%(X,Y.F%,G%) -- CURVE DRAWING (USER UNITS)

================

This function is used  to  draw  vectors  from  the  current

cursor location to the point (X,Y), measured in the user's units,

according to the values of F%, G%,  and  the  terminal's  current

mode.   F% determines whether the vector will be light or dark as

follows:

ABS(F%)=2 If the terminal is in graph mode, draw a line from  the

current  location to (X,Y). If the terminal is in alpha

mode, no line will be drawn.

ABS(F%)=1 Move  to  the  point  (X,Y)  without  drawing  a  line,

regardless of the current mode of the terminal.

F%=0  Puts the terminal into alpha mode and interprets X  and

Y  as  the  row and column to which the cursor is sent.

No line is drawn.

If F%<=0, then the terminal is placed in  alpha  mode  after

the  move;   if  F%>0,  then the terminal is placed in graph mode

after the move.  See the discussion above for the significance of

graph  vs.   alpha  mode.   In  particular note that the terminal

should always be left in alpha mode when a plot is finished.  For

F%=0%, rows are numbered from 1 to 35 beginning at the top of the

screen, columns from 1 to 74 (left to right).

The value of G% determines which of several special  symbols

will  be plotted at the given data point.  G%=0 causes no special

symbol to be plotted.  Values of G% from 1 to 9  cause  the  nine

symbols listed in Figure 1 to be plotted at the point (X,Y).  The

height of these symbols is approximately 0.1 inches.

After each call to this function, the variables  O1  and  O2

contain  the  current  cursor  location in inches relative to the                                           ______

origin defined by the last call of  FNT0%().  See  discussion  of

this point below under the description of the FNT4%() and FNT6%()

functions.

Curves are usually drawn by using this function in some sort

of loop, for example,

100 I=FNT2%(X(1),Y(1),1%,0%)

110 FOR J=2 TO N

120 I=FNT2%(X(J),Y(J),2%,0%)

130 NEXT J

Note  that  line 100 assures that the first data point is reached

without any line to it from the previous cursor  position.   Line

100  could be omitted and line 110 changed to '110 FOR J=1 TO N',

if the terminal were already in alpha mode.  Also, the plotter is

still  in  graph  mode  after  the  loop  is  exited;  some other

plotting functions such as FNT7% () should be used to return  the

terminal to alpha mode.

$PLOT10 and $PLOTHP User's Guide                           Page 8

Version 6A-00                                         19-Aug-77

FNT3%(F%) -- GRAPHIC INPUT

=========

This function activates the graphic input capability of  the

Tektronix  terminal.   It  may be used to find the current cursor

location (F%<>0%) or to enable  the  terminal  crosshair  feature

(F%=0%).  In either case the function returns a special value and

makes use of the variables O1 and O2.

F% <> 0%  This causes the terminal to  send  the  current  cursor

location.   The  function  converts  this to the user's

units (as determined from parameters set in the FNT0%()

and  FNT1%()  functions)  and  puts the X or horizontal

value into O1,the Y or  vertical  value  into  O2.  The

function  returns  the value of the low-order 5 bits of

the termnal status word (see the Tektronix manual for a

description).  In  alpha  mode the point returned in O1

and O2 corresponds to the  lower  left  corner  of  the

blinking cursor.  For example the statement

100 I=FNT3%(-1%):  I=FNT2%(O1,O2,1%,1%)

draws  a  triangular  shaped  symbol centered about the

current cursor location  and  leaves  the  terminal  in

graph mode.

F% = 0%   This  causes  the  terminal  crosshair  feature  to  be

enabled.   The  user  may  position  it anywhere on the

screen by using the knobs to the right of the keyboard.

By  striking  any key the user causes several things to

happen.  First the terminal sends the location  of  the

crosshair  and  ASCII  value  of  the struck key to the

computer and simultaneously turns  off  the  crosshair.

On  receipt  of this information, the plotting function

converts the crosshair location  to  user's  units  and

stores  them  in  the  variables O1 and O2 as described

above.  The ASCII value of the struck key  is  returned

by the function.

The  ASCII  value  of  the  characters  may be found in

Appendix D of the BASIC-PLUS Language Reference Manual.

Use  of  this feature allows for coded graphic input to

the user's program.  For example, striking  the  letter

'A'  causes the function to return a value of 65; a '1'

gives 49; a 'CTRL/A' gives 1. Only the  special  RSTS/E

control  characters CTRL/C, CTRL/O, CTRL/S, CTRL/Q, and

CTRL/Z cannot be sent in this way.  Note that only  one

character  can  be  sent and that the RETURN key is not

struck (unless, of course, that is the value  you  want

sent to the program).

This function always returns the terminal to alpha mode.  In

using  the  Tektronix  terminal  to  develop figures for eventual

plotting on the HP plotter, keep in mind that the crosshair input

(F%=0) is not implemented for the HP version of the function.

$PLOT10 and $PLOTHP User's Guide                           Page 9

Version 6A-00                                         19-Aug-77

FNT4%(X1,Y1,A,L,L$) -- AXIS ROUTINE

===================

This function draws  an  axis  through  the  point  (X1,Y1),

measured  in  inches from the point (X0,Y0) specified in the last

call of FNT0%(), at an angle of A  degrees  with  the  horizontal

(normal  X-axis  has A=0, normal Y-axis has A=90).  The length of

the axis is ABS(L); the labeling along the axis will  be  on  the

positive or negative side depending upon the sign of L. The label

L$ is written adjacent to the labeling.  Ordinarily L is negative

for  an  X-axis, positive for a Y-axis.  The values printed along

the axis are determined by the last reference to  FNT1%().  If  a

vertical  axis  is  specified,  the  Y  scaling  values are used;

otherwise, the X scaling is used.  Therefore a  special  call  to

FNT1%()  prior  to  drawing an oblique axis is necessary to allow

for a special scaling.  If the string  L$  is  the  null  string,

i.e.,  L$="",  then  an axis with tick marks and no annotation is

drawn.  If L$ is  a  string  consisting  of  just  spaces,  e.g.,

L$=" ",  then the tick marks are annotated, but no label appears.

It  is  frequently  desirable  to  use  L$=""  to  suppress   the

annotation  for  plots on the HP7200, since labeling is extremely

slow on this device.  NOTE:  The terminal is left in  alpha  mode

after a call to this function.

Note that this function measures the  starting  location  of

the  axis  (X1,Y1)  in inches relative to the origin specified in

the last call to FNT0%().  It is frequently necessary to position

the  axis  at a point measured in the user's own units.  This can

be accomplished by a statement of the form

100 I%=FNT2%(X,Y,1%,0%)+FNT4%(O1,O2,A,L,L$)

The axis is drawn at the point corresponding to  (X,Y),  measured

in  the  user's  units.   This  occurs since the FNT2%() function

returns the location of (X,Y), in inches, in the variables O1 and

O2.

FNT5$(N,D%) -- ROUNDING OF NUMBERIC VALUES

===========

This function returns a string representation for the number

N,  rounded  to D% decimals to the left (D%<0) or right (D%>0) of

the decimal point.  This function is used internally by the  axis

routine  for  labeling the axis.  It is provided as a convenience

to the user.  For numbers which would appear in exponential  form

when  PRINTed  in  BASIC-PLUS,  the  results of this function are

unpredictable, and  the  user  is  cautioned  against  using  the

function in this case.

$PLOT10 and $PLOTHP User's Guide                          Page 10

Version 6A-00                                         19-Aug-77

FNT6%(X1,Y1,A,H,L$) -- SYMBOL/LABELING ROUTINE

===================

This function creates a label from the string L$ of height H

inches  and  angle  A  degrees  with  the horizontal at the point

(X1,Y1). (X1,Y1) is measured in inches  from  the  point  (X0,Y0)

used  in  the  last  call to FNT0%(). Any height greater than .05

inches is valid;  values between .1 and .2 seem to  produce  very

legible results.  Setting the string L$ equal to CHR$(K%+15%) for

K%=1,2,...9  will  cause  the  symbol  created  by  FNT2%()  with

G%=l,2,...9,  respectively,  to be drawn.  When one of these nine

special values is  given  in  L$,  the  corresponding  symbol  is

centered  about the given location.  For all other characters the

specified location is assumed to be that of the lower-left corner

of  the  character.  Except for the special centered symbols, the

terminal is placed in alpha mode after the label is drawn.  After

a  call  to  FNT6%(),  the variables O1 and O2 contain the cursor

location in inches relative to the point (X0,Y0) used in the last

call  to  FNT0%().   When  it  is  desired to place labeling at a

position measured in the user's units, the technique of the  last

paragraph  describing  the  FNT4%()  function  may  be  used (see

above).

FNT7%(X1,Y1,F%) -- CURVE DRAWING (INCHES)

===============

This function operates similarly to  FNT2%().  Here  (X1,Y1)

are  measured in inches relative to the point (X0,Y0) used in the

last call to FNT0%(). The use of the parameter F% is  similar  to

its use in FNT2%(), namely

ABS(F%)=2 If the terminal is in graph mode, draw a line from  the

current  location  to  (X1,Y1).  If  the terminal is in

alpha mode, no line will be drawn.

ABS(F%)=1 Move to the  point  (X1,Y1)  without  drawing  a  line,

regardless of the current mode of the terminal

F%=0  Puts the terminal into alpha mode, and moves to (X1,Y1)

without drawing a line (identical to F%=-1).

If F%<=0, then the terminal is placed in  alpha  mode  after

the move;  if F%>0 the the terminal is placed in graph mode after

the move.  This function is often more convenient  than  FNT2%(),

particularly  for various types of annotation which do not depend

in anyway upon the way in which the user's data is  scaled.   Use

of  this  function with F%=0 is ordinarily the last function call

in any program and insures the terminal is in alpha mode  at  the

completion of the plot.

$PLOT10 and $PLOTHP User's Guide                          Page 11

Version 6A-00                                         19-Aug-77

Special Considerations When Using $PLOTHP

=========================================

For the most part the  above  detailed  discussion  of  each

function applies equally to both the $PLOT10 and $PLOTHP versions

of the function.  However, there are some  important  differences

for  a  few  of the functions and these are discussed below.  The

most important consideration  is  that  of  speed.   The  maximum

plotting  speed  of the HP plotter is about 55 points per minute;

for the Tektronix terminal, operating at  120  CPS,  the  minimum

speed  in  graph  mode  is  30 points per second, and for certain                                          ______

annotation, like that along an axis,  2-3  times  this  speed  is

possible.   Hence the overall speed ratio between the two devices

is from 30:1 to 100:1. In particular, since the worst end of this

range  governs  for most annotation, it is recommended that plots

developed on the Tektronix terminal should  be  stripped  of  all

unnecessary  annotation before being reproduced on the HP plotter

(see the discussion above on the axis routine, FNT4%()  for  more

details on this point).

Except as noted below, the user's program should be  totally

unaffected by changes between $PLOT10 and $PLOTHP.

1)  Before running a plotting program for the HP plotter, it  is

necessary  to  assign the plotter.  This can only be done on

our system[1] by typing

RUN $DEVICE

The 'Ready' message will immediately appear,  and  the  user

may  then  proceed  by  calling up his program and APPENDing

$PLOTHP.  Note that typing 'RUN $DEVICE'  destroys  whatever

was  previously  in  the  user's  work  space (see MANIAC/11

User's Guide for more on this point).

When the user is finished plotting but  wishes  to  continue

with  other computing activities, it is necessary to release

the plotter for use by others by typing

DEASSIGN KB9:

If the user is signing-off after completing the  plot,  this

is unnecessary.

2)  A call to FNT0%() with ABS(F%)=1, the  initialization  call,

causes  the  following  question to be printed at the user's

terminal.

X-LEN,Y-LEN (INCHES)?

----------

[1]  Assigning  a  keyboard  is  a  priveleged  operation  on our

system;  hence the ASSIGN system command cannot be used.   Typing

'RUN $DEVICE' is equivalent to typing 'ASSIGN KB9:' on systems in

which assigning a keyboard is not a priveleged function.

$PLOT10 and $PLOTHP User's Guide                          Page 12

Version 6A-00                                         19-Aug-77

The user should type  the  length  of  the  X-  and  Y-axis,

respectively, as determined by the physical adjustments made

on the plotter.  The  plotter  X-axis  is  parallel  to  the

longer  dimension  of  the  plotter.  If the values typed in

response to this question do not correspond  to  the  actual

pen  adjustments,  the  resulting  figure will be distorted.

Furthermore, the minimum values for these  lengths  must  be                                                         ____

greater  than  the maximum size of the corresponding lengths

on the Tektronix terminal;  the maximum  dimensions  of  the

Tektronix  screen are 7.3 inches horizontally, by 5.6 inches

vertically (approximately).  If the values typed are exactly

the  same as the pens are set, the figure will be reproduced

in the same physical dimensions as on the Tektronix  screen.

If  the values entered are greater (smaller) than the actual

pen adjustments, then the corresponding dimension will  plot

shorter (longer) than on the Tektronix screen by a ratio of

(physical length of axis)/(given length of axis)

Resolution of the HP plotter is 10000 units in both X and  Y

directions,  i.e.,  1/10000th  of the physical length of the

axis.

3)  In some programs  for  the  Tektronix  terminal  it  may  be

convenient  for  the  user  to position the cursor using the

plotting functions and then use a PRINT statement to  output

text  to  the screen.  The user is cautioned that any output

created in this manner will appear at the user terminal, not

on  the  plot,  when  using  the HP plotter.  In particular,

using the FNT3%() function to locate the cursor  after  such

an  action  will  yield  very  different results between the

Tektronix and HP versions of the ploting  functions  (see  4

below).

4)  For the HP version of  FNT3%(),  only  the  cursor  location

function  has  any  meaning.  An attempt to refrence FNT3%()

with the parameter F%=0 causes the  message  "GRAPHIC  INPUT

BEING  IGNORED"  to  be printed at the user's terminal.  For

F%<>0, the function works in a similar, but  not  identical,

manner  to  the  Tektronix  version  of  the  function.  The

distinction  arises  from  the  manner  in  which  the   two

different  implementations  of  the  function  determine the

current cursor location.  For the Tektronix version  of  the

function, the terminal actually sends the cursor location to

the computer and the program interprets this in terms of the

user's  units.   For  the  HP  version,  the  program itself

'remembers' to where the pen was last  moved,  and  it  then

converts  this  location  to  the user's units.  Thus on the

Tektronix, the value returned  will  always  be  the  cursor

location,  even  if  it has been moved by the user's program

using other than plotting function calls (e.g., by  PRINTing

a  value  on the terminal). However, for the HP function the

location returned in O1 and O2 will be valid only if the pen

position on the plotter has not been manually reset.

$PLOT10 and $PLOTHP User's Guide                          Page 13

Version 6A-00                                         19-Aug-77

APPENDIX

========

The following is a listing of $TEKDEM.BAS.  Users are  urged

to  examine  the use of plotting functions in this program and to

see the results of running the program on the Tektronix terminal.

It  is  hoped that this example will clarify some of the detailed

discussion in this document.

10!!!TEKDEM     VERSION 6A-00   EDIT 00         6-APR-76

20!!!DESCRIPTION--DEMONSTRATION PROGRAM FOR THE TEKTRONIX 4010-1 TERMINAL

USING THE PLOTTING FUNCTIONS CONTAINED IN $PLOT10.BAS.  PROGRAM

PERFORMS LINEAR LEAST SQUARES FIT OF USER SUPPLIED DATA.

21!     IT THEN PLOTS THE DATA POINTS AND FITTED CURVE.  USING THE GRAPHIC

INPUT FUNCTION OF THE TERMINAL IT THEN ALLOWS THE USER TO

SELECTIVELY REMOVE CERTAIN DATA VALUES AND HAVE A NEW REGRESSION

22!     LINE PLOTTED.

30!!!DIRECTIONS--USER MUST APPEND $PLOT10 BEFORE RUNNING.  SAMPLE DATA IS

CONTAINED IN LINES 1000-1999.  THE USER MAY SUPPLY HIS OWN IF HE

WISHES.  THE FORMAT FOR DATA ENTRY IS:

31!             N - # OF PAIRS OF DATA

X1,Y1,X2,Y2,X3,Y3,...,XN,YN

WHERE Y IS THE DEPENDENT VARIABLES.  PROGRAM WILL HANDLE AS MANY AS

32!     100 PAIRS OF OBSERVATIONS.

40!!!ADDITONAL NOTES--THIS PROGRAM IS PRIMARILY INTENDED FOR DEMONSTRATION

PURPOSES. AS SUCH, THE BASIC USED IS QUITE SIMPLE AND STRAIGHT-

FORWARD.  AN APPLICATION PROGRAM BASED UPON THIS TECHNIQUE COULD

41!     EASILY BE MADE MORE EFFICIENT BY USING SOME OF THE ADVANCED FEATURES

OF BASIC-PLUS.

99!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

100 DIM X(100),Y(100)

110 P=FNT0%(0.,0.,1%)

120 P=FNT6%(0.,4.,0.,.25,'$PLOT10 DEMONSTRATION PROGRAM')

130 P=FNT6%(3.6,2.,X,.12,'  $TEKDEM') FOR X=0 TO 315 STEP 45

140 P=FNT6%(0.,0.,0.,.12,'TYPE <CR> TO CONTINUE')

150 INPUT X

160 P=FNT0%(0.,0.,0%)

!CLEAR THE SCREEN AFTER PRINTING HEADER AND GIVE INSTRUCTIONS

170 PRINT'BASED UPON THE MIN AND MAX FOR X AND Y WHICH WILL BE PRINTED,'

180 PRINT'SELECT VALUES FOR XMIN,XMAX,YMIN,YMAX FOR DATA PRESENTATION.'

190 PRINT'FOR SMAPLE DATA SUPPLIED, THE VALUES -400,200,-2,8 ARE GOOD.'

200 PRINT'THE DATA WILL BE PLOTTED AND A REGRESSION LINE THRU THE DATA'

210 PRINT 'WILL BE DRAWN.  WHEN THE CROSSHAIR COMES ON, MOVE IT TO ONE'

220 PRINT'OR MORE OF THE DATA SYMBOLS AND PRESS ANY KEY EXCEPT "0".'

230 PRINT'AN "X" WILL BE DRAWN THRU THAT POINT. CONTINUE IN THIS WAY'

240 PRINT 'UNTIL AS MANY POINTS AS YOU WISH HAVE BEEN ELIMINATED.'

250 PRINT 'TYPE A "0" (ZERO) TO STOP THE PROCESS, AND A NEW REGRESSION'

260 PRINT 'LINE WILL BE DRAWN.'

270 READ N

280 X1,Y1=0

290 U1,V1=1E38

300 U2,V2=-1E38

$PLOT10 and $PLOTHP User's Guide                                         Page 14

Version 6A-00                                                        19-Aug-77

310 FOR I=1 TO N

320 READ X(I),Y(I)

330 X1=X1+X(I)

340 Y1=Y1+Y(I)

350 IF X(I) < U1 THEN U1=X(I)

ELSE IF X(I)>U2 THEN U2=X(I)

360 IF Y(I) < V1 THEN V1=Y(I)

ELSE IF Y(I)>V2 THEN V2=Y(I)

!THESE TWO LINES ARE USED TO FIND THE RANGE OF THE DATA--

(U1,U2) FOR X AND (V1,V2) FOR Y

370 NEXT I

380 X1=X1/N

390 Y1=Y1/N

!THE PRECEDING LINES READ IN THE DATA PAIRS AND COMPUTE THE MEAN

OF BOTH X (IND. VAR.) AND Y (DEP. VAR.)

400 S1,S2=0

410 FOR I=1 TO N

420 S1=S1+(Y(I)-Y1)*(X(I)-X1)

430 S2=S2+(X(I)-X1)2

440 NEXT I

450 B=S1/S2

460 A=Y1-B*X1

!THESE LINES HAVE COMPUTED THE COEFFICIENTS A  B IN THE LEAST SQUARES

FIT.  THE FITTED CURVE IS Y=A + B*X

470 PRINT 'XMIN='U1;TAB(20);'XMAX=';U2

480 PRINT 'YMIN='V1;TAB(20);'YMAX=';V2

490 INPUT 'XMIN,XMAX,YMIN,YMAX FOR PLOT';A1,A2,B1,B2

!BEGINNING PLOTTING PORTION OF PROGRAM.  USER MUST ENTER LIMITS ON

DATA TO BE USED FOR PLOTTING; PROGRAM DETERMINES OTHER VALUES.

500 P=FNT0%(.5,.5,2%)

!RE-INITIALIZE AND CLEAR SCREEN; ORIGIN OFFSET ENOUGH TO ALLOW FOR

ANNOTATION ALONG AXIS.

510 P=FNT1%(A1,B1,(A2-A1)/6,(B2-B1)/5)

!DATA SCALED SO THAT X DIMENSION WILL BE 6 INCHES, Y DIMENSION

5 INCHES.

520 FOR I=1 TO N

530 P=FNT2%(X(I),Y(I),1%,1%)

540 NEXT I

!EACH OF THE DATA POINTS IS PLOTTED SEPARATELY USING THE TRIANGLE

SYMBOL.

550 P=FNT2%(A1,A+B*A1,1%,0%)+FNT2%(A2,A+B*A2,-2%,0%)

!THE LEAST SQUARES LINE IS DRAWN FROM X=A1 TO X=A2; THE TERMINAL

IS RETURNED TO ALPHA MODE AFTER THE LINE.

560 P=FNT4%(0.,0.,0.,-6.,'IND. VAR.')+FNT4%(0.,0.,90.,5.,'DEP. VAR.')

!THE X AND Y AXES ARE NOW DRAWN; NOTE THAT HERE AND IN THE PREVIOUS

LINE, TWO FUNCTIONS ARE INVOKED AT THE SAME TIME.  THIS CAN BE DONE

TO GROUP RELATED OPERATIONS

570 IF B2-B1>A2-A1 THEN T=(B2-B1)/250 ELSE T=(A2-A1)/250

!TOLERENCE FOR CROSSHAIR INPUT; CALL A HIT IF CROSSHAIR IS

WITHIN SYMBOL.

580 P=FNT3%(0%)

590 IF P=48 THEN 680

600 X=O1: Y=O2

!GET POINT(S) TO DELETE FROM CROSSHAIR LOCATION; TYPING A ZERO (0)

STOP THE PROCESS-- ASCII CODE FOR '0' IS 48.

610 FOR I=1 TO N

620 IF ABS(X(I)-X)<T AND ABS(Y(I)-Y)<T THEN 650

630 NEXT I

$PLOT10 and $PLOTHP User's Guide                                         Page 15

Version 6A-00                                                        19-Aug-77

640 GOTO 580

!CROSSHAIR FINDS VALUES APPROXIMATELY -- TOLERANCE IS T.  IF

NO POINT MEETS CRITERION, ASSUME USER ERROR AND TRY AGAIN.

650 X(I),Y(I)=-1E38

660 P=FNT2%(X,Y,-1%,7%)

!PUT AN 'X' THRU DATA POINT THAT WILL DELETED FROM

NEXT FIT OF DATA.

670 GOTO 580

!FLAG VALUE TO OMIT AND CONTINUE UNTIL USER INPUTS A '0' TO STOP

680 X1,Y1,S1,S2,N1=0

690 FOR I=1 TO N

700 IF X(I)=-1E38 AND Y(I)=-1E38 THEN 740

710 X1=X1+X(I)

720 Y1=Y1+Y(I)

730 N1=N1+1

740 NEXT I

750 X1=X1/N1

760 Y1=Y1/N1

!COMPUTE NEW MEAN FROM REMAINING POINTS

770 FOR I=1 TO N

780 IF X(I)=-1E38 AND Y(I)=-1E38 THEN 810

790 S1=S1+(Y(I)-Y1)*(X(I)-X1)

800 S2=S2+(X(I)-X1)2

810 NEXT I

820 B=S1/S2

830 A=Y1-B*X1

!COMPUTE REVISED COEFFICIENTS.

840 P=FNT2%(A1,A+B*A1,1%,0%)+FNT2%(A2,A+B*A2,-2%,0%)

!DRAW NEW LEAST SQUARES FIT LINE

850 P=FNT7%(0.,-.2,0%)

!MOVE CURSOR BELOW FIGURE AND STOP

860 STOP

1000 DATA 21

1010 DATA -300,-.295, -260, .433, -220, 1.30,-200,1.28

1020 DATA -180, 1.44, -160, 1.298, -120, 1.978, -100, 2.18

1030 DATA -60, 2.70, -40, 3.015,-20, 2.686, -10,2.448,-5,2.55

1040 DATA 20,3.07, 40,4.16,60,4.55,80,4.319,100,5.347,120,4.97

1050 DATA 110, 2.63, 90,3.077

2000END