PDP-8 Simulator Usage

01-Dec-08

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2008, written by Robert M Supnik

Copyright (c) 1993-2008, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	PDP-8 Features	3

2.1	CPU	5

2.2	TSC8-75 ETOS Timeshare Control (TSC)	6

2.3	Programmed I/O Devices	6

2.3.1	PC8E Paper Tape Reader (PTR)	6

2.3.2	PC8E Paper Tape Punch (PTP)	7

2.3.3	KL8E Terminal Input (TTI)	7

2.3.4	KL8E Terminal Output (TTO)	8

2.3.5	LE8E Line Printer (LPT)	8

2.3.6	DK8E Line-Frequency Clock (CLK)	9

2.3.7	KL8JA Additional Terminals (TTIX, TTOX)	9

2.3.8	TD8E/TU56 DECtape (TD)	10

2.3.9	TA8E/TA60 Cassette Tape (CT)	11

2.4	Moving Head Disks	12

2.4.1	RK8E Cartridge Disk (RK)	12

2.4.2	RL8A Cartridge Disk (RL)	12

2.5	RX8E/RX01, RX28/RX02 Floppy Disk (RX)	13

2.6	Fixed Head Disks	14

2.6.1	RF08/RS08 Fixed Head Disk (RF)	14

2.6.2	DF32/DS32 Fixed Head Disk (RF)	15

2.7	TC08/TU56 DECtape (DT)	16

2.8	TM8E Magnetic Tape (MT)	17

3	Symbolic Display and Input	17

This memorandum documents the PDP-8 simulator.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/pdp8/	pdp8_defs.h

pdp8_cpu.c

pdp8_ct.c

pdp8_df.c

pdp8_dt.c

pdp8_lp.c

pdp8_mt.c

pdp8_pt.c

pdp8_rf.c

pdp8_rk.c

pdp8_rl.c

pdp8_rx.c

pdp8_sys.c

pdp8_td.c

pdp8_tsc.c

pdp8_tt.c

pdp8_ttx.c

# PDP-8 Features

The PDP-8 simulator is configured as follows:

device names(s)	simulates

CPU			PDP-8/E CPU with 4KW-32KW of memory

-			KE8E extended arithmetic element (EAE)

-			KM8E memory management and timeshare control

TSC			TSC8-75 ETOS operating system timeshare control

PTR,PTP		PC8E paper tape reader/punch

TTI,TTO		KL8E console terminal

TTIX,TTOX		KL8JA additional terminals

LPT			LE8E line printer

CLK			DK8E line frequency clock (also PDP-8/A compatible)

RK			RK8E/RK05 cartridge disk controller with four drives

RF			RF08/RS08 fixed head disk controller with 1-4 platters

DF			DF32/DS32 fixed head disk controller with 1-4 platters

RL			RL8A/RL01 cartridge disk controller with four drives

RX			RX8E/RX01, RX28/RX02 floppy disk controller with two

drives

DT			TC08/TU56 DECtape controller with eight drives

TD			TD8E/TU56 DECtape controller with two drives

MT			TM8E/TU10 magnetic tape controller with eight drives

CT			TA8E/TU60 cassette tape controller with two drives

Most devices can be disabled or enabled, by the commands:

SET <dev> DISABLED

SET <dev> ENABLED

The simulator allows most device numbers to be changed, by the command:

SET <dev> DEV=<number>

The PDP-8 can support only one of the set {DF32, RF08, RL8A} using the default device numbers, since they all use device numbers 60-61.  The default is the RF08.  To change the disk at device numbers 60-61:

SET RF DISABLED			disable RF08

SET DF ENABLED, or		enable DF32

SET RL ENABLED			enable RL8A

The PDP-8 can only support one of the set {TC08, TD8E} using the default device numbers, since both use device number 77.  The default is the TC08.  To change the DECtape controller to the TD8E:

SET DT DISABLED			disable TC08

SET TD ENABLED			enable TD8E

The PDP-8 can only support one of the set {TM8E, TA8E} using the default device numbers, since both use device number 70.  The default is the TM8E.  To change the device at device number 70:

SET MT DISABLED			disable TM8E

SET CT ENABLED			enable TA8E

Alternately, the device conflict can be eliminated by changing device numbers:

SET RL DEV=50

SET RL ENA

SET TD DEV=74

SET TD ENA

SET CT DEV=73

SET CT ENA

However, devices can only be BOOTed with their default device numbers.

The PDP-8 simulator implements several unique stop conditions:

- If an undefined instruction (unimplemented IOT or OPR) is decoded, and STOP_INST is set

- If a simulated DECtape runs off the end of its reel

The LOAD command supports both RIM format and BIN format tapes.  If the file extension is .RIM, or the r switch is specified with LOAD, the file is assumed to be RIM format; if the file extension is not .RIM, or the -b switch is specified, the file is assumed to be BIN format.

## CPU

The only CPU options are the presence of the EAE and the size of main memory; the memory extension and time-share control is always included, even if memory size is 4K.

SET CPU EAE				enable EAE

SET CPU NOEAE			disable EAE

SET CPU 4K				set memory size = 4K

SET CPU 8K				set memory size = 8K

SET CPU 12K				set memory size = 12K

SET CPU 16K				set memory size = 16K

SET CPU 20K				set memory size = 20K

SET CPU 24K				set memory size = 24K

SET CPU 28K				set memory size = 28K

SET CPU 32K				set memory size = 32K

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 32K.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			15		program counter, including IF as high 3 bits

AC			12		accumulator

MQ			12		multiplier-quotient

L			1		link

SR			12		front panel switches

IF			3		instruction field

DF			3		data field

IB			3		instruction field buffer

SF			7		save field

UF			1		user mode flag

UB			1		user mode buffer

SC			5		EAE shift counter

GTF			1		EAE greater than flag

EMODE			1		EAE mode (0 = A, 1 = B)

ION			1		interrupt enable

ION_DELAY		1		interrupt enable delay for ION

CIF_DELAY		1		interrupt enable delay for CIF

PWR_INT		1		power fail interrupt

UF_INT		1		user mode violation interrupt

INT			15		interrupt pending flags

DONE			15		device done flags

ENABLE		15		device interrupt enable flags

PCQ[0:63]		15		PC prior to last JMP, JMS, or interrupt;

most recent PC change first

STOP_INST		1		stop on undefined instruction

WRU			8		interrupt character

The CPU attempts to detect when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE			enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  At present, the CPU is considered idle if it is executing a KSF/JMP *-1 loop with interrupts disabled (OS/8, DMS-8) or a JMP * loop (TSS/8).

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries.

## TSC8-75 ETOS Timeshare Control (TSC)

ETOS is a timeshared operating system for the PDP-8, providing multiple virtual OS/8 environments for up to 32 users.  It requires a special timeshare control option, the TSC8-75.  The TSC8-75 is normally disabled; to run ETOS, it must be enabled with the command:

SET TSC ENABLED

The TSC8-75 implements these registers:

name			size		comments

IR			12		most recently trapped instruction

PC			12		PC of most recently trapped instruction

CDF			1		1 if trapped instruction is CDF, 0 otherwise

ENB			1		interrupt enable flag

INT			1		interrupt pending flag

Except for operation of ETOS, the TSC8-75 should be left disabled.

## Programmed I/O Devices

### PC8E Paper Tape Reader (PTR)

The paper tape reader (PTR) reads data from a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper tape reader supports the BOOT command.  BOOT PTR copies the RIM loader into memory and starts it running.

The paper tape reader implements these registers:

name			size		comments

BUF			8		last data item processed

DONE			1		device done flag

ENABLE		1		interrupt enable flag

INT			1		interrupt pending flag

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### PC8E Paper Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch implements these registers:

name			size		comments

BUF			8		last data item processed

DONE			1		device done flag

ENABLE		1		interrupt enable flag

INT			1		interrupt pending flag

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### KL8E Terminal Input (TTI)

The terminal interfaces (TTI, TTO) can be set to one of four modes, KSR, 7B, 7B, or 8B:

mode	input characters		output characters

KSR	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit set	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is KSR.

The terminal input (TTI) polls the console keyboard for input.  It implements these registers:

name			size		comments

BUF			8		last data item processed

DONE			1		device done flag

ENABLE		1		interrupt enable flag

INT			1		interrupt pending flag

POS			32		number of characters input

TIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the clock)

### KL8E Terminal Output (TTO)

The terminal output (TTO) writes to the simulator console window.  It implements these registers:

name			size		comments

BUF			8		last data item processed

DONE			1		device done flag

ENABLE		1		interrupt enable flag

INT			1		interrupt pending flag

POS			32		number of characters output

TIME			24		time from I/O initiation to interrupt

### LE8E Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be read or written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

BUF			8		last data item processed

ERR			1		error status flag

DONE			1		device done flag

ENABLE		1		interrupt enable flag

INT			1		interrupt pending flag

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

### DK8E Line-Frequency Clock (CLK)

The real-time clock (CLK) frequency can be adjusted as follows:

SET CLK 60HZ			set frequency to 60Hz

SET CLK 50HZ			set frequency to 50Hz

The default is 60Hz.

The clock implements these registers:

name			size		comments

DONE			1		device done flag

ENABLE		1		interrupt enable flag

INT			1		interrupt pending flag

TIME			24		clock interval

The real-time clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

### KL8JA Additional Terminals (TTIX, TTOX)

The additional terminals consist of two independent devices, TTIX and TTOX.  The entire set is modeled as a terminal multiplexer, with TTIX as the master controller.  The additional terminals perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH TTIX <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.

The additional terminals can be set to one of four modes: UC, 7P, 7B, or 8B.

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is UC.  Finally, each line supports output logging.  The SET TTOXn LOG command enables logging on a line:

SET TTOXn LOG=filename		log output of line n to filename

The SET TTOXn NOLOG command disables logging and closes the open log file, if any.

Once TTIX is attached and the simulator is running, the terminals listen for connections on the specified port.  They assume that the incoming connections are Telnet connections.  The connections remain open until disconnected either by the Telnet client, a SET TTIX DISCONNECT command, or a DETACH TTIX command.

Other special commands:

SHOW TTIX CONNECTIONS 		show current connections

SHOW TTIX STATISTICS 		show statistics for active connections

SET TTOXn DISCONNECT		disconnects the specified line.

The input device  (TTIX) implements these registers:

name			size		comments

BUF[0:3]		8		input buffer, lines 0 to 3

DONE			4		device done flags (line 0 rightmost)

ENABLE		4		interrupt enable flag

INT			4		interrupt pending flag

TIME			24		initial polling interval

TPS			10		polls per second after calibration

The output device (TTOX) implements these registers:

name			size		comments

BUF[0:3]		8		last data item processed, lines 0-3

DONE			4		device done flag (line 0 rightmost)

ENABLE		4		interrupt enable flag

INT			4		interrupt pending flag

TIME[0:3]		24		time from I/O initiation to interrupt,

lines 0-3

The additional terminals do not support save and restore.  All open connections are lost when the simulator shuts down or TTIX is detached.

### TD8E/TU56 DECtape (TD)

The TD8E is a programmed I/O, non-interrupt controller, supporting two DECtape drives (0 and 1).  The TD8E simulator puts a high burden on the host processor, because tape activity is simulated a line (3b) at a time.  Unless the PDP-8 software requires the TD8E, the TC08 should be used to simulate DECtapes.  The TD8E is disabled by default.

TD8E options include the ability to make units write enabled or write locked.

SET TDn LOCKED			set unit n write locked

SET TDn WRITEENABLED		set unit n write enabled

Units can also be set ENABLED or DISABLED.  The TD8E supports the BOOT command, but only for unit 0.

The TD8E supports supports PDP-8 format, PDP-11 format, and 18b format DECtape images.  ATTACH assumes the image is in PDP-8 format; the user can force other choices with switches:

-s					PDP-11 format

-f					18b format

-a					autoselect based on file on file size

The TD8E controller is a data-only simulator; the timing and mark track, and block header and trailer, are not stored.  Thus, read always produces standard values for mark track, header, and trailer words, and write throws mark track, header, and trailer words into the bit bucket.

The TD8E controller implements these registers:

name			size		comments

TDCMD			4		command register

TDDAT			12		data register

TDMTK			6		mark track register

TDSLF			1		single line flag

TDQLF			1		quad line flag

TDTME			1		timing error flag

TDQL			2		quad line counter

LTIME			31		time between lines

DCTIME		31		time to decelerate to a full stop

POS[0:1]		32		position, in lines, units 0 and 1

STATT[0:1]		18		unit state, units 0 and 1

STOP_OFFR		1		stop on off-reel error

The LTIME parameter should not be changed, or OS/8 may fail to run correctly.  The DCTIME parameter should always be at least 100 times greater than LTIME.  Acceleration time is 75% of deceleration time.

### TA8E/TA60 Cassette Tape (CT)

The TA8E is a programmed I/O controller supporting two cassette drives (0 and 1).  The TA8E can be used with the MCPIP program under OS/8, and with the CAPS-8 operating system.  Cassettes are simulated as magnetic tapes with a fixed capacity (93,000 characters).  The tape format is always SimH standard.  The TA8E is disabled by default.

TA8E options include the ability to make units write enabled or write locked.

SET CTn LOCKED			set unit n write locked

SET CTn WRITEENABLED		set unit n write enabled

Units cannot be set ENABLED or DISABLED.  The TA8E supports the BOOT command, but only for CAPS-8, and only for unit 0.

The TA8E controller implements these registers:

name			size		comments

CTSRA			8		status register A

CTSRB			8		status register B

CTDB			8		data buffer

CTDF			1		data flag

RDY			1		ready flag

WLE			1		write lock error

WRITE			1		TA60 write operation flag

INT			1		interrupt request

BPTR			17		buffer pointer

BLNT			17		buffer length

STIME			24		operation start time

CTIME			24		character latency

STOP_IOE		1		stop on I/O errors flag

POS[0:1]		32		position, units 0-1

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	CRC error; if STOP_IOE, stop

## Moving Head Disks

### RK8E Cartridge Disk (RK)

RK8E options include the ability to make units write enabled or write locked:

SET RKn LOCKED			set unit n write locked

SET RKn WRITEENABLED		set unit n write enabled

Units can also be set ENABLED or DISABLED.  The RK8E supports the BOOT command.

The RK8E implements these registers:

name			size	comments

RKSTA			12		status

RKCMD			12		disk command

RKDA			12		disk address

RKMA			12		current memory address

BUSY			1		control busy flag

INT			1		interrupt pending flag

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RL8A Cartridge Disk (RL)

RL8A options include the ability to make units write enabled or write locked:

SET RLn LOCKED			set unit n write locked

SET RLn WRITEENABLED		set unit n write enabled

Units can also be set ENABLED or DISABLED.  The RL8A supports the BOOT command, but only for unit 0.

The RL8A implements these registers:

name			size		comments

RLCSA			12		control/status A

RLCSB			12		control/status B

RLMA			12		memory address

RLWC			12		word count

RLSA			6		sector address

RLER			12		error flags

RLSI			16		silo top word

RLSI1			16		silo second word

RLSI2			16		silo third word

RLSIL			1		silo read left/right flag

INT			1		interrupt request

DONE			1		done flag

ERR			1		composite error flag

STIME			1		seek time, per cylinder

RTIME			1		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

## RX8E/RX01, RX28/RX02 Floppy Disk (RX)

The RX can be configured as an RX8E with two RX01 drives, or an RX28 with two RX02 drives:

SET RX RX8E				set controller to RX8E/RX01

SET RX RX28				set controller to RX28/RX02

The controller is set to the RX8E by default.  The RX28 is not backwards-compatible with the RX8E and will not work with the standard OS/8 V3D floppy disk driver.

RX8E options include the ability to set units write enabled or write locked:

SET RXn LOCKED			set unit n write locked

SET RXn WRITEENABLED		set unit n write enabled

RX28 options include, in addition, the ability to set the unit density to single density, double density, or autosized; autosizing is the default:

SET RXn SINGLE			set unit n single density

SET RXn DOUBLE			set unit n double density

SET RXn AUTOSIZE			set unit n autosize

The RX8E and RX28 support the BOOT command.

The RX8E and RX28 implement these registers:

name			size		comments

RXCS			12		status

RXDB			12		data buffer

RXES			12		error status

RXTA			8		current track

RXSA			8		current sector

STAPTR		4		controller state

BUFPTR		8		buffer pointer

INT			1		interrupt pending flag

DONE			1		device done flag

ENABLE		1		interrupt enable flag

TR			1		transfer ready flag

ERR			1		error flag

CTIME			24		command completion time

STIME			24		seek time, per track

XTIME			24		transfer ready delay

STOP_IOE		1		stop on I/O error

SBUF[0:255]		8		sector buffer array

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RX01 and RX02 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## Fixed Head Disks

With default device addressing, either the RF08 or the DF32 can be present in a configuration, but not both.

### RF08/RS08 Fixed Head Disk (RF)

RF08 options include the ability to set the number of platters to a fixed value between 1 and 4, or to autosize the number of platters:

SET RF 1P				one platter (256K)

SET RF 2P				two platters (512K)

SET RF 3P				three platters (768K)

SET RF 4P				four platters (1024K)

SET RF AUTOSIZE			autosized on ATTACH

The default is one platter.

The RF08 implements these registers:

name			size		comments

STA			12		status

DA			20		current disk address

MA			12		memory address (in memory)

WC			12		word count (in memory)

WLK			32		write lock switches

INT			1		interrupt pending flag

DONE			1		device done flag

TIME			24		rotational delay, per word

BURST			1		burst flag

STOP_IOE		1		stop on I/O error

The RF08 supports the BOOT command.  The default bootstrap is for OS/8.  To bootstrap the 4K Disk Monitor, use the BOOT -D RF command.

The RF08 is a three-cycle data break device.  If BURST = 0, word transfers are scheduled individually; if BURST = 1, the entire transfer occurs in a single data break.

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RF08 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### DF32/DS32 Fixed Head Disk (RF)

DF32 options include the ability to set the number of platters to a fixed value between 1 and 4, or to autosize the number of platters:

SET DF 1P				one platter (32K)

SET DF 2P				two platters (64K)

SET DF 3P				three platters (98K)

SET DF 4P				four platters (128K)

SET DF AUTOSIZE			autosized on ATTACH

The default is one platter.

The DF32 implements these registers:

name			size		comments

STA			12		status, disk and memory address extension

DA			12		low order disk address

MA			12		memory address (in memory)

WC			12		word count (in memory)

WLK			16		write lock switches

INT			1		interrupt pending flag

DONE			1		device done flag

TIME			24		rotational delay, per word

BURST			1		burst flag

STOP_IOE		1		stop on I/O error

The DF32 supports the BOOT command.  The default bootstrap is for OS/8.  To bootstrap the 4K Disk Monitor, use the BOOT -D DF command.

The DF32 is a three-cycle data break device.  If BURST = 0, word transfers are scheduled individually; if BURST = 1, the entire transfer occurs in a single data break.

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

DF32 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## TC08/TU56 DECtape (DT)

DECtapes drives are numbered 1-8; in the simulator, drive 8 is unit 0.  TC08 options include the ability to make units write enabled or write locked.

SET DTn LOCKED			set unit n write locked

SET DTn WRITEENABLED		set unit n write enabled

Units can also be set ENABLED or DISABLED.  The TC08 supports the BOOT command, but only for unit 0.

The TC08 supports supports PDP-8 format, PDP-11 format, and 18b format DECtape images.  ATTACH assumes the image is in PDP-8 format; the user can force other choices with switches:

-s					PDP-11 format

-f					18b format

-a					autoselect based on file on file size

The TC08 controller is a data-only simulator; the timing and mark track, and block header and trailer, are not stored.  Thus, the WRITE TIMING AND MARK TRACK function is not supported; the READ ALL function always returns the hardware standard block header and trailer; and the WRITE ALL function dumps non-data words into the bit bucket.

The DECtape controller implements these registers:

name			size		comments

DTSA			12		status register A

DTSB			12		status register B

INT			1		interrupt pending flag

ENB			1		interrupt enable flag

DTF			1		DECtape flag

ERF			1		error flag

CA			12		current address (memory location 7754)

WC			12		word count (memory location 7755)

LTIME			31		time between lines

DCTIME		31		time to decelerate to a full stop

SUBSTATE		2		read/write command substate

POS[0:7]		32		position, in lines, units 0 to 7

STATT[0:7]		31		unit state, units 0 to 7

STOP_OFFR		1		stop on off-reel error

It is critically important to maintain certain timing relationships among the DECtape parameters, or the DECtape simulator will fail to operate correctly.

- LTIME must be at least 6

- DCTIME needs to be at least 100 times LTIME

Acceleration time is set to 75% of deceleration time.

## TM8E Magnetic Tape (MT)

Magnetic tape options include the ability to make units write enabled or write locked.

SET MTn LOCKED			set unit n write locked

SET MTn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET MTn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MTn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.

The magnetic tape controller implements these registers:

name			size		comments

CMD			12		command

FNC			12		function

CA			12		memory address

WC			12		word count

DB			12		data buffer

STA			12		main status

STA2			6		secondary status

DONE			1		device done flag

INT			1		interrupt pending flag

STOP_IOE		1		stop on I/O error

TIME			24		record delay

UST[0:7]		24		unit status, units 0 to 7

POS[0:7]		32		position, units 0 to 7

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	parity error; if STOP_IOE, stop

# Symbolic Display and Input

The PDP-8 simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as ASCII character

-c			display as two packed sixbit characters

-t			display as two packed TSS/8 sixbit characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		two packed sixbit characters

# or -t		two packed TSS/8 sixbit characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses standard PDP-8 assembler syntax.  There are four instruction classes: memory reference, IOT, field change, and operate.

Memory reference instructions have the format

memref {I} {C/Z} address

where I signifies indirect, C a current page reference, and Z a zero page reference.  The address is an octal number in the range 0 - 07777; if C or Z is specified, the address is a page offset in the range 0 - 177.  Normally, C is not needed; the simulator figures out from the address what mode to use.  However, when referencing memory outside the CPU (eg, disks), there is no valid PC, and C must be used to specify current page addressing.

IOT instructions consist of single mnemonics, eg, KRB, TLS.  IOT instructions may be or'd together

iot iot iot...

The simulator does not check the legality of the proposed combination.  IOT's for which there is no opcode may be specified as IOT n, where n is an octal number in the range 0 - 0777.

Field change instructions (CIF, CDF) have the format

fldchg field

where field is an octal number in the range 0 - 7.  Field change instructions may be or'd together.

Operate instructions have the format

opr opr opr...

The simulator does not check the legality of the proposed combination.  EAE mode A and B mnemonics may be specified regardless of the EAE mode.  The operands for MUY and DVI must be deposited explicitly.