PDP-1 Simulator Usage

01-Dec-2008

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2008, written by Robert M Supnik

Copyright (c) 1993-2008, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	PDP-1 Features	3

2.1	CPU	4

2.2	Programmed I/O Devices	5

2.2.1	Paper Tape Reader (PTR)	5

2.2.2	Paper Tape Punch (PTP)	6

2.2.3	Console Typewriter Input (TTI), Output (TTO)	6

2.2.4	Type 62 Line Printer (LPT)	7

2.2.5	Type 550/555 Microtape (DECtape) (DT)	8

2.2.6	PDP-1D Timesharing Clock (CLK)	8

2.2.7	Type 630 Data Communications Subsystem (DCS, DCSL)	9

2.3	Drums	10

2.3.1	Type 24 Serial Drum (DRM)	10

2.3.2	Type 23 Parallel Drum (DRP)	11

3	Symbolic Display and Input	11

4	Character Sets	12

This memorandum documents the PDP-1 simulator.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_timer.c

sim_tmxr.c

sim/pdp1/	pdp1_defs.h

pdp1_clk.c

pdp1_cpu.c

pdp1_dcs.c

pdp1_drm.c

pdp1_dt.c

pdp1_lp.c

pdp1_stddev.c

pdp1_sys.c

# PDP-1 Features

The PDP-1 is configured as follows:

device name(s)	simulates

CPU			PDP-1 CPU with up to 64KW of memory

optional automatic multiply/divide

optional 16-channel sequence break system

optional PDP-1D extended features

CLK			1Khz time-sharing clock (PDP-1D)

PTR,PTP		integral paper tape reader/punch

TTI,TTO		console typewriter

LPT			Type 62 line printer

DRM			Type 24 serial drum

DRP			Type 23 parallel drum

DT			Type 550 Microtape (DECtape)

DCS,DCSL		Type 630 Data Communications Subsystem

The PDP-1 simulator implements the following unique stop conditions:

- An unimplemented instruction is decoded, and register STOP_INST is set

- More than IND_MAX indirect addresses are detected during memory reference address decoding

- More than XCT_MAX nested executes are detected during instruction execution

- I/O wait, and no I/O operations outstanding (i.e, no I/O completion will ever occur)

- A simulated DECtape runs off the end of its reel

The LOAD command supports RIM format tapes and BLK format tapes.  If the file to be loaded has an extension of .BIN, or switch -B is specified, the file is assumed to be BLK format; otherwise, it defaults to RIM format.  LOAD takes an optional argument that specifies the starting address of the field to be loaded:

LOAD lisp.rim			load RIM format file lisp.rim

LOAD ddt.rim 70000		load RIM format file ddt.rim into

the field starting at 70000

LOAD -B macro.blk			load BLK format file macro.blk

The DUMP command is not implemented.

## CPU

The only CPU options are the presence of hardware multiply/divide and the size of main memory.

SET CPU MDV				enable multiply/divide

SET CPU NOMDV			disable multiply/divide

SET CPU SBS16			enable 16-channel sequence break system

SET CPU NOSBS16			disable 16-channel sequence break system

SET CPU PDP1C			set CPU to standard PDP-1C

SET CPU PDP1DS45			set CPU to PDP-1D, serial# 45 (BBN)

SET CPU PDP1DS48			set CPU to PDP-1D, serial# 48 (Stanford)

SET CPU 4K				set memory size = 4K

SET CPU 8K				set memory size = 8K

SET CPU 12K				set memory size = 12K

SET CPU 16K				set memory size = 16K

SET CPU 20K				set memory size = 20K

SET CPU 24K				set memory size = 24K

SET CPU 28K				set memory size = 28K

SET CPU 32K				set memory size = 32K

SET CPU 48K				set memory size = 48K

SET CPU 64K				set memory size = 64K

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 64K.  Setting the CPU to PDP-1D also enables multiply/divide and the 16-channel sequence break system.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			16		program counter

AC			18		accumulator

IO			18		IO register

OV			1		overflow flag

PF			6		program flags<1:6>

SS			6		sense switches<1:6>

TA			16		address switches

TW			18		test word (front panel switches)

EXTM			1		extend mode

RNGM			1		ring mode (PDP-1D only)

L			1		link (PDP-1D #45 only)

RM			1		restrict mode (PDP-1D)

RMASK			1		restrict memory mask (PDP-1D)

RTB			18		restrict trap buffer (PDP-1D #45 only)

RNAME[0:3]		2		rename map (PDP-1D #45 only)

IOSTA			18		IO status register

SBON			1		sequence break enable

SBRQ			1		sequence break request

SBIP			1		sequence break in progress

SBSREQ		16		pending sequence break requests

SBSENB		16		enabled sequence break levels

SBSACT		16		active sequence break levels

IOH			1		I/O halt in progress

IOS			1		I/O synchronizer (completion)

PCQ[0:63]		16		PC prior to last jump or interrupt;

most recent PC change first

STOP_INST		1		stop on undefined instruction

SBS_INIT		1		initial state of sequence break enable

EXTM_INIT		1		initial state of extend mode

XCT_MAX		8		maximum XCT chain

IND_MAX		8		maximum nested indirect addresses

WRU			8		interrupt character

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries.

If the 16-channel sequence break system is enabled, devices can be assigned to any break level between 0 (the default) and 15, with the following command:

SET <dev> SBSLVL=n		assign device to sequence break level n

Because each PDP-1 configuration was unique, there are no default assignments for the 16-channel sequence break system.

## Programmed I/O Devices

### Paper Tape Reader (PTR)

The paper tape reader (PTR) reads data from or a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper tape reader supports the BOOT command.  BOOT PTR copies the RIM loader into memory and starts it running.  BOOT PTR loads into the field selected by TA<0:3> (the high order four bits of the address

switches).

The paper tape reader implements these registers:

name			size		comments

BUF			8		last data item processed

DONE			1		device done flag

RPLS			1		return restart pulse flag

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### Paper Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch implements these registers:

name			size		comments

BUF			8		last data item processed

DONE			1		device done flag

RPLS			1		return restart pulse flag

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### Console Typewriter Input (TTI), Output (TTO)

The Typewriter is a half-duplex electric typewriter (originally a Friden Flexowriter, later a Sorobon-modified IBM B).  It has only a single buffer and a single carriage state but distinct input and output done and interrupt flags.  The typewriter input (TTI) polls the console keyboard for input.  The typewriter output (TTO) writes to the simulator console window.

The typewriter input implements these registers:

name			size		comments

BUF			6		typewriter buffer (shared)

UC			1		upper case/lower case state (shared)

DONE			1		input ready flag

POS			32		number of characters input

TIME			24		keyboard polling interval

The typewriter output implements these registers:

name			size		comments

BUF			6		typewriter buffer (shared)

UC			1		upper case/lower case state (shared)

RPLS			1		return restart pulse flag

DONE			1		output done flag

POS			32		number of characters output

TIME			24		time from I/O initiation to interrupt

### Type 62 Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer can be disabled and enabled with the SET LPT DISABLED and SET LPT ENABLED commands, respectively.

The line printer implements these registers:

name			size		comments

BUF			8		last data item processed

PNT			1		printing done flag

SPC			1		spacing done flag

RPLS			1		return restart pulse flag

BPTR			6		print buffer pointer

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

LBUF[0:119]		8		line buffer

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape or paper

OS I/O error	x		report error and stop

### Type 550/555 Microtape (DECtape) (DT)

The PDP-1 uses the Type 550 Microtape (later renamed DECtape), a programmed I/O controller.  PDP-1 DECtape format has 4 18b words in its block headers and trailers.

DECtapes drives are numbered 1-8; in the simulator, drive 8 is unit 0.  DECtape options include the ability to make units write enabled or write locked.

SET DTn WRITEENABLED		set unit n write enabled

SET DTn LOCKED			set unit n write locked

Units can also be set ENABLED or DISABLED.

The DECtape controller can be disabled and enabled with the SET DT DISABLED and SET DT ENABLED commands, respectively.

The Type 550 supports PDP-8 format, PDP-11 format, and 18b format DECtape images.  ATTACH assumes the image is in 18b format; the user can other choices with switches:

-t					PDP-8 format

-s					PDP-11 format

-a					autoselect based on file size

The DECtape controller is a data-only simulator; the timing and mark track, and block header and trailer, are not stored.  Thus, the WRITE TIMING AND MARK TRACK function is not supported; the READ ALL function always returns the hardware standard block header and trailer; and the WRITE ALL function dumps non-data words into the bit bucket.

The DECtape controller implements these registers:

name			size		comments

DTSA			12		status register A

DTSB			12		status register B

DTDB			18		data buffer

DTF			1		DECtape flag

BEF			1		block end flag

ERF			1		error flag

LTIME			31		time between lines

DCTIME		31		time to decelerate to a full stop

SUBSTATE		2		read/write command substate

POS[0:7]		32		position, in lines, units 0-7

STATT[0:7]		18		unit state, units 0-7

STOP_OFFR		1		stop on off-reel error

It is critically important to maintain certain timing relationships among the DECtape parameters, or the DECtape simulator will fail to operate correctly.

- LTIME must be at least 6

- DCTIME needs to be at least 100 times LTIME

Acceleration time is set to 75% of deceleration time.

### PDP-1D Timesharing Clock (CLK)

The PDP-1D implements a timesharing clock, which operates at 1Khz.  The clock has a readable counter and generates interrupts at 32 ms and 1 minute intervals.  There is no other visible state.  The clock is disabled by default.

The clock implements these registers:

name			size		comments

CNTR			16		clock counter, range 0-5999910

The clock requires the 16-channel sequence break system and is assigned to two different SBS levels:

SET CLK SBS32MS=n			assign 32 msec interrupt to SBS level n

SET CLK SBS1MIN=n			assign 1 minute interrupt to SBS level n

### Type 630 Data Communications Subsystem (DCS, DCSL)

The Type 630 Data Communications Subsystem provides up to 32 asynchronous interfaces.  The Type 630 consists of two independent devices: DCS for the scanner, and DCSL for the individual lines.  The terminal multiplexer performs input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH DCS <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The number of lines can be changed with SET DCL LINES command:

SET DCS LINES=n		set number of lines to n, where n is 1-32

Each line (each of unit of DCSL) can be set to one of four modes: UC, 7P, 7B, or 8B.

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is UC.  Finally, each line supports output logging.  The SET DCSLn LOG command enables logging on a line:

SET DCSLn LOG=filename		log output of line n to filename

The SET DCSLn NOLOG command disables logging and closes the open log file, if any.

Once DCS is attached and the simulator is running, the multiplexer listens for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connections remain open until disconnected either by the Telnet client, a SET DCS DISCONNECT command, or a DETACH DCS command.

Other special commands:

SHOW DCS CONNECTIONS 		show current connections

SHOW DCS STATISTICS 		show statistics for active connections

SET DCSLn DISCONNECT		disconnects the specified line.

The multiplexer scanner  (DCS) implements these registers:

name			size		comments

BUF[0:31]		8		input buffer, lines 0 to 31

FLG[0:31]		1		line ready flag, lines 0 to 31

SCNF			1		scanner ready flag

SCAN			5		scanner line number

SEND			5		output line number

The individual lines (DCSL) implement these registers:

name			size		comments

TIME[0:31]		24		time from I/O initiation to interrupt,

lines 0 to 31

The multiplexer does not support save and restore.  All open connections are lost when the simulator shuts down or DSC is detached.

## Drums

The PDP-1 supports two drums: the Type 23 parallel drum (DRP) and the Type 24 serial drum (DRM).  Both use device addresses 061-064; accordingly, only one can be enabled at a time.  By default, the

Type 24 serial drum is enabled, and the Type 23 parallel drum is disabled.  The PDP-1D requires the Type 23 parallel drum.

### Type 24 Serial Drum (DRM)

The serial drum (DRM) implements these registers:

name			size		comments

DA			9		drum address (sector number)

MA			16		current memory address

DONE			1		device done flag

ERR			1		error flag

WLK			32		write lock switches

TIME			24		rotational latency, per word

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		drum not ready

Drum data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### Type 23 Parallel Drum (DRP)

The parallel drum (DRP) implements these registers:

name			size		comments

TA			12		track address

RDF			5		read field

RDE			1		read enable flag

WRF			5		write field

WRF			1		write enable flag

MA			16		current memory address

WC			12		word count

BUSY			1		device busy flag

ERR			1		error flag

TIME			24		rotational latency, per word

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		drum not ready

Drum data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

# Symbolic Display and Input

The PDP-1 simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as ASCII character

-c			display as three packed FIODEC characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		three packed FIODEC characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses modified PDP-1 assembler syntax.  There are six instruction classes: memory reference, shift, skip, operate, IOT, and LAW.

Memory reference instructions have the format

memref {I} address

where I signifies indirect reference.  The address is an octal number in the range 0 - 0177777.

Shift instructions have the format

shift shift_count

The shift count is an octal number in the range 0-9.

Skip instructions consist of single mnemonics, eg, SZA, SZS4.  Skip instructions may be or'd together

skip skip skip...

The sense of a skip can be inverted by including the mnemonic I.

Operate instructions consist of single mnemonics, eg, CLA, CLI.  Operate instructions may be or'd together

opr opr opr...

IOT instructions consist of single mnemonics, eg, TYI, TYO.  IOT instructions may include an octal numeric modifier or the modifier I:

iot modifier

The simulator does not check the legality of skip, operate, or IOT combinations.

Finally, the LAW instruction has the format

LAW {I} immediate

where immediate is in the range 0 to 07777.

# Character Sets

The PDP-1's first console was a Frieden Flexowriter; its character encoding was known as FIODEC.  The PDP-1's line printer used a modified Hollerith character set.  The following table provides equivalences between ASCII characters and the PDP-1's I/O devices.  In the console table, UC stands for upper case.

PDP-1			   PDP-1

ASCII			console	 		line printer

000 - 007		none				none

bs			075				none

tab			036				none

012 - 014		none				none

cr			077				none

016 - 037		none				none

space			000	       		000

!			{OR} UC+005			none

"			UC+001			none

#			{IMPLIES} UC+004		none

$			none				none

%			none				none

&			{AND} UC+006		none

'			UC+002			none

(			057				057

)			055				055

*			{TIMES} UC+073		072

+			UC+054			074

,			033				033

-			054				054

.			073				073

/			021				021

0			020				020

1			001				001

2			002				002

3			003				003

4			004				004

5			005				005

6			006				006

7			007				007

8			010				010

9			011				011

:			none				none

;			none				none

<			UC+007			034

=			UC+033			053

>			UC+010			034

?			UC+021			037

@			{MID DOT} 040		{MID DOT} 040

A			UC+061			061

B			UC+062			062

C			UC+063			063

D			UC+064			064

E			UC+065			065

F			UC+066			066

G			UC+067			067

H			UC+070			070

I			UC+071			071

J			UC+041			041

K			UC+042			042

L			UC+043			043

M			UC+044			044

N			UC+045			045

O			UC+046			046

P			UC+047			047

Q			UC+050			050

R			UC+051			051

S			UC+022			022

T			UC+023			023

U			UC+024			024

V			UC+025			025

W			UC+026			026

X			UC+027			027

Y			UC+030			030

Z			UC+031			031

[			UC+057			none

\			{OVERLINE} 056		{OVERLINE} 056

]			UC+055			none

^			{UP ARROW} UC+011		{UP ARROW} 035

_			UC+040			UC+040

`			{RT ARROW} UC+020		036

a			061				none

b			062				none

c			063				none

d			064				none

e			065				none

f			066				none

g			067				none

h			070				none

i			071				none

j			041				none

k			042				none

l			043				none

m			044				none

n			045				none

o			046				none

p			047				none

q			050				none

r			051				none

s			022				none

t			023				none

u			024				none

v			025				none

w			026				none

x			027				none

y			030				none

z			031				none

{			none				none

|			UC+056			076

}			none				none

~			UC+003			013

del			075				none