ASxxxx Assemblers

and

ASLINK Relocating Linker

Version   4.00

November 2003

CHAPTER 1  THE ASSEMBLER                                     1-1

1.1     THE ASXXXX ASSEMBLERS                              1-1

1.1.1     Assembly Pass 1                                  1-2

1.1.2     Assembly Pass 2                                  1-2

1.1.3     Assembly Pass 3                                  1-2

1.2     SOURCE PROGRAM FORMAT                              1-3

1.2.1     Statement Format                                 1-3

1.2.1.1     Label Field                                    1-3

1.2.1.2     Operator Field                                 1-5

1.2.1.3     Operand Field                                  1-5

1.2.1.4     Comment Field                                  1-6

1.3     SYMBOLS AND EXPRESSIONS                            1-6

1.3.1     Character Set                                    1-6

1.3.2     User-Defined Symbols                            1-10

1.3.3     Reusable Symbols                                1-10

1.3.4     Current Location Counter                        1-12

1.3.5     Numbers                                         1-13

1.3.6     Terms                                           1-14

1.3.7     Expressions                                     1-14

1.4     GENERAL ASSEMBLER DIRECTIVES                      1-16

1.4.1     .module Directive                               1-16

1.4.2     .title Directive                                1-16

1.4.3     .sbttl Directive                                1-17

1.4.4     .page Directive                                 1-17

1.4.5     .msg Directive                                  1-17

1.4.6     .error Directive                                1-17

1.4.7     .byte, .db, and .fcb Directives                 1-18

1.4.8     .word, .dw, and .fdb Directives                 1-18

1.4.9     .3byte and .triple Directives                   1-19

1.4.10    .4byte and .quad Directive                      1-19

1.4.11    .blkb, .ds, ,rmb, and .rs Directives            1-20

1.4.12    .blkw, .blk3, and .blk4 Directives              1-20

1.4.13    .ascii, .str, and .fcc Directives               1-20

1.4.14    .ascis and .strs Directives                     1-21

1.4.15    .asciz and .strz Directives                     1-21

1.4.16    .assume Directive                               1-22

1.4.17    .radix Directive                                1-22

1.4.18    .even Directive                                 1-22

1.4.19    .odd Directive                                  1-23

1.4.20    .area Directive                                 1-23

1.4.21    .bank Directive                                 1-25

1.4.22    .org Directive                                  1-26

1.4.23    .globl Directive                                1-27

1.4.24    .local Directive                                1-27

1.4.25    .equ, .gblequ, and .lclequ Directives           1-28

1.4.26    .if, .else, and .endif Directives               1-28

1.4.27    .ifdef, .else, and .endif Directives            1-29

1.4.28    .ifndef, .else, and .endif Directives           1-30

1.4.29    .include Directive                              1-31

1.4.30    .define and .undefine Directives                1-31

1.4.31    .setdp Directive                                1-32

1.4.32    .16bit, .24bit, and .32bit Directives           1-34

1.4.33    .msb Directive                                  1-34

1.4.34    .end Directive                                  1-35

1.5     INVOKING ASXXXX                                   1-36

1.6     ERRORS                                            1-37

Page ii

1.7     LISTING FILE                                      1-38

1.8     SYMBOL TABLE FILE                                 1-40

1.9     OBJECT FILE                                       1-41

CHAPTER 2  THE LINKER                                        2-1

2.1     ASLINK RELOCATING LINKER                           2-1

2.2     INVOKING ASLINK                                    2-2

2.3     LIBRARY PATH(S) AND FILE(S)                        2-3

2.4     ASLINK PROCESSING                                  2-4

2.5     ASXXXX VERSION 4.XX LINKING                        2-7

2.5.1     Object Module Format                             2-7

2.5.2     Header Line                                      2-8

2.5.3     Module Line                                      2-8

2.5.4     Merge Mode Line                                  2-8

2.5.5     Bank Line                                        2-9

2.5.6     Area Line                                        2-9

2.5.7     Symbol Line                                      2-9

2.5.8     T Line                                          2-10

2.5.9     R Line                                          2-10

2.5.10    P Line                                          2-11

2.5.11    24-Bit and 32-Bit Addressing                    2-11

2.5.12    ASlink V4.xx Error Messages                     2-12

2.6     ASXXXX VERSION 3.XX LINKING                       2-14

2.6.1     Object Module Format                            2-14

2.6.2     Header Line                                     2-14

2.6.3     Module Line                                     2-15

2.6.4     Area Line                                       2-15

2.6.5     Symbol Line                                     2-15

2.6.6     T Line                                          2-15

2.6.7     R Line                                          2-16

2.6.8     P Line                                          2-16

2.6.9     24-Bit and 32-Bit Addressing                    2-17

2.6.10    ASlink V3.xx Error Messages                     2-17

2.7     INTEL IHX OUTPUT FORMAT (16-BIT)                  2-20

2.8     INTEL I86 OUTPUT FORMAT (24 OR 32-BIT)            2-21

2.9     MOTORLA S1-S9 OUTPUT FORMAT (16-BIT)              2-22

2.10    MOTORLA S2-S8 OUTPUT FORMAT (24-BIT)              2-23

2.11    MOTORLA S3-S7 OUTPUT FORMAT (32-BIT)              2-24

CHAPTER 3  BUILDING ASXXXX AND ASLINK                        3-1

3.1     BUILDING AN ASSEMBLER                              3-1

3.2     BUILDING ASLINK                                    3-2

APPENDIX A  ASAVR ASSEMBLER                                  A-1

A.1     AVR ASSEMBLER NOTES                                A-1

A.1.1     Processor Specific Directive                     A-1

A.2     AVR REGISTER SET                                   A-2

A.3     AVR INSTRUCTION SET                                A-2

A.3.1     AVR Arithmetic and Logical Instructions          A-4

A.3.2     AVR Bit and Bit-Test Instructions                A-5

A.3.3     AVR Skip on Test Instructions                    A-5

A.3.4     AVR Jump/Call/Return Instructions                A-5

Page iii

A.3.5     AVR Short Branch Instructions                    A-5

A.3.6     AVR Short Branch Instructions with Bit Test      A-6

A.3.7     AVR Data Transfer Instructions                   A-6

APPENDIX B  ASPIC ASSEMBLER                                  B-1

B.1     PIC ASSEMBLER NOTES                                B-1

B.2     PROCESSOR SPECIFIC DIRECTIVES                      B-1

B.2.1     .pic Directive                                   B-2

B.2.2     .picnopic Directive                              B-2

B.2.3     .pic12bit Directive                              B-3

B.2.4     .pic14bit Directive                              B-3

B.2.5     .pic16bit Directive                              B-3

B.2.6     .pic20bit Directive                              B-3

B.2.7     .picfix Directive                                B-4

B.2.8     .maxram Directive                                B-4

B.2.9     .badram Directive                                B-4

B.2.10    .setdmm Directive                                B-5

B.3     12-BIT OPCODE PIC                                  B-5

B.4     14-BIT OPCODE PIC                                  B-6

B.5     16-BIT OPCODE PIC                                  B-7

B.6     20-BIT ADDRESSING PIC                              B-8

B.7     PIC OPCODES                                       B-10

APPENDIX C  ASRAB ASSEMBLER                                  C-1

C.1     ACKNOWLEDGMENT                                     C-1

C.2     PROCESSOR SPECIFIC DIRECTIVES                      C-1

C.2.1     .r2k Directive                                   C-2

C.2.2     .hd64 Directive                                  C-2

C.2.3     .z80 Directive                                   C-2

C.3     RABBIT 2000/3000 ADDRESSING AND INSTRUCTIONS       C-3

C.3.1     Instruction Symbols                              C-3

C.3.2     Rabbit Instructions                              C-5

C.4     Z80/HD64180 ADDRESSING AND INSTRUCTIONS            C-7

C.4.1     Inherent Instructions                            C-8

C.4.2     Implicit Operand Instructions                    C-8

C.4.3     Load Instruction                                 C-9

C.4.4     Call/Return Instructions                         C-9

C.4.5     Jump and Jump to Subroutine Instructions         C-9

C.4.6     Bit Manipulation Instructions                   C-10

C.4.7     Interrupt Mode and Reset Instructions           C-10

C.4.8     Input and Output Instructions                   C-10

C.4.9     Register Pair Instructions                      C-10

C.4.10    HD64180 Specific Instructions                   C-11

APPENDIX D  ASEZ80 ASSEMBLER                                 D-1

D.1     ACKNOWLEDGMENT                                     D-1

D.2     PROCESSOR SPECIFIC DIRECTIVES                      D-1

D.2.1     .adl Directive                                   D-1

D.2.2     .z80 Directive                                   D-2

D.2.3     .msb Directive                                   D-2

D.3     EZ80 ADDRESSING AND INSTRUCTIONS                   D-3

D.3.1     Instruction Symbols                              D-3

Page iv

D.3.2     EZ80 Instructions                                D-5

APPENDIX E  ASZ80 ASSEMBLER                                  E-1

E.1     .hd64 DIRECTIVE                                    E-1

E.2     Z80 REGISTER SET AND CONDITIONS                    E-1

E.3     Z80 INSTRUCTION SET                                E-2

E.3.1     Inherent Instructions                            E-3

E.3.2     Implicit Operand Instructions                    E-3

E.3.3     Load Instruction                                 E-4

E.3.4     Call/Return Instructions                         E-4

E.3.5     Jump and Jump to Subroutine Instructions         E-4

E.3.6     Bit Manipulation Instructions                    E-5

E.3.7     Interrupt Mode and Reset Instructions            E-5

E.3.8     Input and Output Instructions                    E-5

E.3.9     Register Pair Instructions                       E-5

E.3.10    HD64180 Specific Instructions                    E-6

APPENDIX F  ASGB ASSEMBLER                                   F-1

F.1     ACKNOWLEDGEMENT                                    F-1

F.2     INTRODUCTION                                       F-1

F.3     GAMEBOY REGISTER SET AND CONDITIONS                F-1

F.4     GAMEBOY INSTRUCTION SET                            F-2

F.4.1     .tile Directive                                  F-2

F.4.2     Potentially Controversial Mnemonic Selection     F-4

F.4.2.1     Auto-Indexing Loads                            F-4

F.4.2.2     Input and Output Operations                    F-4

F.4.2.3     The 'stop' Instruction                         F-5

F.4.3     Inherent Instructions                            F-5

F.4.4     Implicit Operand Instructions                    F-5

F.4.5     Load Instructions                                F-6

F.4.6     Call/Return Instructions                         F-6

F.4.7     Jump Instructions                                F-6

F.4.8     Bit Manipulation Instructions                    F-6

F.4.9     Input and Output Instructions                    F-7

F.4.10    Register Pair Instructions                       F-7

APPENDIX G  AS8XCXXX ASSEMBLER                               G-1

G.1     ACKNOWLEDGMENTS                                    G-1

G.2     DS8XCXXX ASSEMBLER DIRECTIVES                      G-1

G.2.1     DS80C390 Addressing Mode Directive               G-2

G.2.2     The .msb Directive                               G-3

G.3     DS8XCXXX REGISTER SET                              G-4

G.4     DS8XCXXX INSTRUCTION SET                           G-4

G.4.1     Inherent Instructions                            G-5

G.4.2     Move Instructions                                G-5

G.4.3     Single Operand Instructions                      G-5

G.4.4     Two Operand Instructions                         G-6

G.4.5     Call and Return Instructions                     G-6

G.4.6     Jump Instructions                                G-6

G.5     DS8XCXXX SPECIAL FUNCTION REGISTERS                G-7

G.5.1     SFR Map                                          G-7

G.5.2     Bit Addressable Registers:  Generic              G-8

Page v

G.5.3     Bit Addressable Registers:  Specific             G-9

G.5.4     Optional Symbols:  Control Bits                 G-10

G.6     DS80C310 SPECIAL FUNCTION REGISTERS               G-11

G.6.1     SFR Map                                         G-11

G.6.2     Bit Addressable Registers:  Generic             G-12

G.6.3     Bit Addressable Registers:  Specific            G-13

G.6.4     Optional Symbols:  Control Bits                 G-14

G.7     DS80C320/DS80C323 SPECIAL FUNCTION REGISTERS      G-15

G.7.1     SFR Map                                         G-15

G.7.2     Bit Addressable Registers:  Generic             G-16

G.7.3     Bit Addressable Registers:  Specific            G-17

G.7.4     Optional Symbols:  Control Bits                 G-18

G.8     DS80C390 SPECIAL FUNCTION REGISTERS               G-19

G.8.1     SFR Map                                         G-19

G.8.2     Bit Addressable Registers:  Generic             G-20

G.8.3     Bit Addressable Registers:  Specific            G-21

G.8.4     Optional Symbols:  Control Bits                 G-22

G.9     DS83C520/DS87C520 SPECIAL FUNCTION REGISTERS      G-24

G.9.1     SFR Map                                         G-24

G.9.2     Bit Addressable Registers:  Generic             G-25

G.9.3     Bit Addressable Registers:  Specific            G-26

G.9.4     Optional Symbols:  Control Bits                 G-27

G.10    DS83C530/DS87C530 SPECIAL FUNCTION REGISTERS      G-28

G.10.1    SFR Map                                         G-28

G.10.2    Bit Addressable Registers:  Generic             G-29

G.10.3    Bit Addressable Registers:  Specific            G-30

G.10.4    Optional Symbols:  Control Bits                 G-31

G.11    DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS      G-32

G.11.1    SFR Map                                         G-32

G.11.2    Bit Addressable Registers:  Generic             G-34

G.11.3    Bit Addressable Registers:  Specific            G-35

G.11.4    Optional Symbols:  Control Bits                 G-37

APPENDIX H  AS430 ASSEMBLER                                  H-1

H.1     MPS430 REGISTER SET                                H-1

H.2     MPS430 ADDRESSING MODES                            H-2

H.2.1     MPS430 Instruction Mnemonics                     H-3

APPENDIX I  ASH8 ASSEMBLER                                   I-1

I.1     H8/3XX REGISTER SET                                I-1

I.2     H8/3XX INSTRUCTION SET                             I-1

I.2.1     Inherent Instructions                            I-2

I.2.2     Branch Instructions                              I-2

I.2.3     Single Operand Instructions                      I-3

I.2.4     Double Operand Instructions                      I-4

I.2.5     Mov Instructions                                 I-5

I.2.6     Bit Manipulation Instructions                    I-6

I.2.7     Extended Bit Manipulation Instructions           I-7

I.2.8     Condition Code Instructions                      I-7

I.2.9     Other Instructions                               I-8

I.2.10    Jump and Jump to Subroutine Instructions         I-8

Page vi

APPENDIX J  AS6800 ASSEMBLER                                 J-1

J.1     6800 REGISTER SET                                  J-1

J.2     6800 INSTRUCTION SET                               J-1

J.2.1     Inherent Instructions                            J-2

J.2.2     Branch Instructions                              J-2

J.2.3     Single Operand Instructions                      J-3

J.2.4     Double Operand Instructions                      J-4

J.2.5     Jump and Jump to Subroutine Instructions         J-4

J.2.6     Long Register Instructions                       J-5

APPENDIX K  AS6801 ASSEMBLER                                 K-1

K.1     .hd6303 DIRECTIVE                                  K-1

K.2     6801 REGISTER SET                                  K-1

K.3     6801 INSTRUCTION SET                               K-1

K.3.1     Inherent Instructions                            K-2

K.3.2     Branch Instructions                              K-2

K.3.3     Single Operand Instructions                      K-3

K.3.4     Double Operand Instructions                      K-4

K.3.5     Jump and Jump to Subroutine Instructions         K-5

K.3.6     Long Register Instructions                       K-5

K.3.7     6303 Specific Instructions                       K-5

APPENDIX L  AS6804 ASSEMBLER                                 L-1

L.1     6804 REGISTER SET                                  L-1

L.2     6804 INSTRUCTION SET                               L-1

L.2.1     Inherent Instructions                            L-2

L.2.2     Branch Instructions                              L-2

L.2.3     Single Operand Instructions                      L-2

L.2.4     Jump and Jump to Subroutine Instructions         L-2

L.2.5     Bit Test Instructions                            L-2

L.2.6     Load Immediate data Instruction                  L-3

L.2.7     6804 Derived Instructions                        L-3

APPENDIX M  AS6805 ASSEMBLER                                 M-1

M.1     6805 REGISTER SET                                  M-1

M.2     6805 INSTRUCTION SET                               M-1

M.2.1     Control Instructions                             M-2

M.2.2     Bit Manipulation Instructions                    M-2

M.2.3     Branch Instructions                              M-2

M.2.4     Read-Modify-Write Instructions                   M-3

M.2.5     Register\Memory Instructions                     M-3

M.2.6     Jump and Jump to Subroutine Instructions         M-4

APPENDIX N  AS6808 ASSEMBLER                                 N-1

N.1     68HC08 REGISTER SET                                N-1

N.2     68HC08 INSTRUCTION SET                             N-1

N.2.1     Control Instructions                             N-2

N.2.2     Bit Manipulation Instructions                    N-2

N.2.3     Branch Instructions                              N-3

N.2.4     Complex Branch Instructions                      N-3

N.2.5     Read-Modify-Write Instructions                   N-4

N.2.6     Register\Memory Instructions                     N-5

Page vii

N.2.7     Double Operand Move Instruction                  N-5

N.2.8     16-Bit <H:X> Index Register Instructions         N-5

N.2.9     Jump and Jump to Subroutine Instructions         N-5

APPENDIX O  AS6809 ASSEMBLER                                 O-1

O.1     6809 REGISTER SET                                  O-1

O.2     6809 INSTRUCTION SET                               O-1

O.2.1     Inherent Instructions                            O-3

O.2.2     Short Branch Instructions                        O-3

O.2.3     Long Branch Instructions                         O-3

O.2.4     Single Operand Instructions                      O-4

O.2.5     Double Operand Instructions                      O-5

O.2.6     D-register Instructions                          O-5

O.2.7     Index/Stack Register Instructions                O-5

O.2.8     Jump and Jump to Subroutine Instructions         O-6

O.2.9     Register - Register Instructions                 O-6

O.2.10    Condition Code Register Instructions             O-6

O.2.11    6800 Compatibility Instructions                  O-6

APPENDIX P  AS6811 ASSEMBLER                                 P-1

P.1     68HC11 REGISTER SET                                P-1

P.2     68HC11 INSTRUCTION SET                             P-1

P.2.1     Inherent Instructions                            P-2

P.2.2     Branch Instructions                              P-2

P.2.3     Single Operand Instructions                      P-3

P.2.4     Double Operand Instructions                      P-4

P.2.5     Bit Manupulation Instructions                    P-4

P.2.6     Jump and Jump to Subroutine Instructions         P-5

P.2.7     Long Register Instructions                       P-5

APPENDIX Q  AS6812 ASSEMBLER                                 Q-1

Q.1     68HC12 REGISTER SET                                Q-1

Q.2     68HC12 INSTRUCTION SET                             Q-1

Q.2.1     Inherent Instructions                            Q-3

Q.2.2     Short Branch Instructions                        Q-3

Q.2.3     Long Branch Instructions                         Q-3

Q.2.4     Branch on Decrement, Test, or Increment          Q-4

Q.2.5     Bit Clear and Set Instructions                   Q-4

Q.2.6     Branch on Bit Clear or Set                       Q-4

Q.2.7     Single Operand Instructions                      Q-5

Q.2.8     Double Operand Instructions                      Q-6

Q.2.9     Move Instructions                                Q-6

Q.2.10    D-register Instructions                          Q-6

Q.2.11    Index/Stack Register Instructions                Q-7

Q.2.12    Jump and Jump/Call to Subroutine

Instructions                                     Q-7

Q.2.13    Other Special Instructions                       Q-7

Q.2.14    Register - Register Instructions                 Q-7

Q.2.15    Condition Code Register Instructions             Q-7

Q.2.16    M68HC11 Compatibility Mode Instructions          Q-8

Page viii

APPENDIX R  AS6816 ASSEMBLER                                 R-1

R.1     68HC16 REGISTER SET                                R-1

R.2     68HC16 INSTRUCTION SET                             R-1

R.2.1     Inherent Instructions                            R-2

R.2.2     Push/Pull Multiple Register Instructions         R-3

R.2.3     Short Branch Instructions                        R-3

R.2.4     Long Branch Instructions                         R-3

R.2.5     Bit Manipulation Instructions                    R-3

R.2.6     Single Operand Instructions                      R-4

R.2.7     Double Operand Instructions                      R-5

R.2.8     Index/Stack Register Instructions                R-5

R.2.9     Jump and Jump to Subroutine Instructions         R-6

R.2.10    Condition Code Register Instructions             R-6

R.2.11    Multiply and Accumulate Instructions             R-6

APPENDIX S  AS8051 ASSEMBLER                                 S-1

S.1     ACKNOWLEDGMENT                                     S-1

S.2     8051 REGISTER SET                                  S-1

S.3     8051 INSTRUCTION SET                               S-2

S.3.1     Inherent Instructions                            S-2

S.3.2     Move Instructions                                S-3

S.3.3     Single Operand Instructions                      S-3

S.3.4     Two Operand Instructions                         S-4

S.3.5     Call and Return Instructions                     S-4

S.3.6     Jump Instructions                                S-4

S.3.7     Predefined Symbols:  SFR Map                     S-5

S.3.8     Predefined Symbols:  SFR Bit Addresses           S-6

S.3.9     Predefined Symbols:  Control Bits                S-7

APPENDIX T  AS8085 ASSEMBLER                                 T-1

T.1     8085 REGISTER SET                                  T-1

T.2     8085 INSTRUCTION SET                               T-1

T.2.1     Inherent Instructions                            T-2

T.2.2     Register/Memory/Immediate Instructions           T-2

T.2.3     Call and Return Instructions                     T-2

T.2.4     Jump Instructions                                T-2

T.2.5     Input/Output/Reset Instructions                  T-3

T.2.6     Move Instructions                                T-3

T.2.7     Other Instructions                               T-3

APPENDIX U  AS6500 ASSEMBLER                                 U-1

U.1     ACKNOWLEDGMENT                                     U-1

U.2     6500 REGISTER SET                                  U-2

U.3     6500 INSTRUCTION SET                               U-2

U.3.1     Processor Specific Directives                    U-3

U.3.2     65xx Core Inherent Instructions                  U-3

U.3.3     65xx Core Branch Instructions                    U-3

U.3.4     65xx Core Single Operand Instructions            U-3

U.3.5     65xx Core Double Operand Instructions            U-4

U.3.6     65xx Core Jump and Jump to Subroutine

Instructions                                     U-4

U.3.7     65xx Core Miscellaneous X and Y Register

Page ix

Instructions                                     U-4

U.3.8     65F11 and 65F12 Specific Instructions            U-5

U.3.9     65C00/21 and 65C29 Specific Instructions         U-5

U.3.10    65C02, 65C102, and 65C112 Specific

Instructions                                     U-6

APPENDIX V  AS740 ASSEMBLER                                  V-1

V.1     ACKNOWLEDGMENT                                     V-1

V.2     740 REGISTER SET                                   V-1

V.3     740 INSTRUCTION SET                                V-1

V.3.1     Inherent Instructions                            V-3

V.3.2     Branch Instructions                              V-3

V.3.3     Single Operand Instructions                      V-3

V.3.4     Double Operand Instructions                      V-4

V.3.5     Jump and Jump to Subroutine Instructions         V-4

V.3.6     Miscellaneous X and Y Register Instructions      V-4

V.3.7     Bit Instructions                                 V-4

V.3.8     Other Instructions                               V-4

APPENDIX W  AS1802 ASSEMBLER                                 W-1

W.1     ACKNOWLEDGMENT                                     W-1

W.2     1802 REGISTER SET                                  W-1

W.3     1802 INSTRUCTION SET                               W-1

W.3.1     1802 Inherent Instructions                       W-2

W.3.2     1802 Short Branch Instructions                   W-2

W.3.3     1802 Long Branch Instructions                    W-3

W.3.4     1802 Immediate Instructions                      W-3

W.3.5     1802 Register Instructions                       W-3

W.3.6     1802 Input and Output Instructions               W-3

W.3.7     CDP1802 COSMAC Microprocessor Instruction

Set Summary                                      W-4

APPENDIX X  AS61860 ASSEMBLER                                X-1

X.1     ACKNOWLEDGMENT                                     X-1

X.2     61860 REGISTER SET                                 X-1

X.3     PROCESSOR SPECIFIC DIRECTIVES                      X-2

X.4     61860 INSTRUCTION SET                              X-2

X.4.1     Load Immediate Register                          X-3

X.4.2     Load Accumulator                                 X-3

X.4.3     Store Accumulator                                X-3

X.4.4     Move Data                                        X-3

X.4.5     Exchange Data                                    X-4

X.4.6     Stack Operations                                 X-4

X.4.7     Block Move Data                                  X-4

X.4.8     Block Exchange Data                              X-4

X.4.9     Increment and Decrement                          X-5

X.4.10    Increment/Decrement with Load/Store              X-5

X.4.11    Fill                                             X-5

X.4.12    Addition and Subtraction                         X-6

X.4.13    Shift Operations                                 X-6

X.4.14    Boolean Operations                               X-6

X.4.15    Compare                                          X-7

Page x

X.4.16    CPU Control                                      X-7

X.4.17    Absolute Jumps                                   X-7

X.4.18    Relative Jumps                                   X-8

X.4.19    Calls                                            X-8

X.4.20    Input and output                                 X-8

X.4.21    Unknown Commands                                 X-9

Page 2

P R E F A C E

The  ASxxxx  assemblers  were  written following the style of

several cross assemblers found in the Digital Equipment Corpora-

tion  Users  Society  (DECUS)  distribution of the C programming

language.  The DECUS code was provided with no documentation  as

to  the  input  syntax  or the output format.  Study of the code

revealed that the unknown author of the code  had  attempted  to

formulate  an assembler with attributes similiar to those of the

PDP-11 MACRO assembler (without macro's).  The  incomplete  code

from  the  DECUS C distribution has been largely rewritten, only

the program structure, and C source  file  organization  remains

relatively  unchanged.   However, I wish to thank the author for

his contribution to this set of assemblers.

The  ASLINK  program was written as a companion to the ASxxxx

assemblers, its design and implementation was not  derived  from

any other work.

I  would  greatly  appreciate  receiving  the  details of any

changes, additions, or errors pertaining to these  programs  and

will  attempt  to  incorporate  any  fixes  or  generally useful

changes in a future update to these programs.

Alan R.  Baldwin

Kent State University

Physics Department

Kent, Ohio 44242

U.S.A.

http://shop-pdp.kent.edu/ashtml/asxxxx.htm

baldwin@kent.edu

baldwin@shop-pdp.kent.edu

tel:  (330) 672 2531

fax:  (330) 672 2959

Page 3

E N D   U S E R   L I C E N S E   A G R E E M E N T

This software is FREEWARE which means it is NOT public domain

but  fully  copyrighted  material  that  is  distributed  freely

without  money.   Its  electronic distribution through BBSs, the

Internet, or other such means is encouraged provided no money is

requested in return.

It is forbidden to distribute this software should this file,

or any of the remaining files, change in any way or  be  omitted

from the archive.

If you would like to include this software together with your

own work you MUST include it only as the original  complete  un-

modified archive in which I distribute it and not as independent

files.   If  uncertain,  simply  point  others   or   link   to:

http://shop-pdp.kent.edu/asxhtm/asxxxx.htm

Please note that although I have done my best to ensure there

is no potentially dangerous code  (or  accidental  virus  infec-

tions),  the  nature of programming is such that it forces me to

provide absolutely no warranty, express or  implied,  with  this

version  of the software, and I bear no responsibility for what-

ever damages, direct or consequential, you may suffer  from  its

use.   I definitely do not warrant this software for suitability

for any particular purpose, either.  It is  also  possible  that

the instructions, the extra utilities, or the examples that come

with the software contain errors,  none  of  which  were  inten-

tional.

Page 4

C O N T R I B U T O R S

Thanks  to Marko Makela for his contribution of the AS6500 cross

assembler.

Marko Makela

Sillitie 10 A

01480 Vantaa

Finland

Internet: Marko dot Makela at Helsinki dot Fi

EARN/BitNet: msmakela at finuh

Thanks  to John Hartman for his contribution of the AS8051 cross

assembler and updates to the ASxxxx and ASLINK internals.

John L. Hartman

jhartman at compuserve dot com

noice @ noicedebugger dot com

Thanks  to  G.   Osborn  for  his  contributions  to LKS19.C and

LKIHX.C.

G. Osborn

gary at s-4 dot com

Thanks to Ken Hornstein for his contribution of object libraries

contained in LKLIBR.C.

Ken Hornstein

kenh at cmf dot nrl dot navy dot mil

Page 5

Thanks  to  Bill  McKinnon for his contributions to the AS8XCXXX

cross assembler for the DS8XCXXX series of microprocessors.

Bill McKinnon

w_mckinnon at conknet dot com

Thanks  to Roger Ivie for his contribution of the ASGB cross as-

sembler for the GameBoy.

Roger Ivie

ivie at cc dot usu dot edu

Thanks  to  Uwe  Steller for his contribution of the AS740 cross

assembler.

Uwe Stellar

Uwe dot Steller at t-online dot de

Thanks  to  Shujen Chen for his contribution of the AS1802 cross

assembler.

Shugen Chen

DeVry University

Tinley Park IL

schen at tp dot devry dot edu

Thanks  to  Edgar Puehringer for his contribution of the AS61860

cross assembler.

Edgar Puehringer

edgar_pue at yahoo dot com

Page 6

Thanks to Ulrich Raich and Razaq Ijoduola for their contribution

of the ASRAB cross assembler.

Ulrich Raich and Razaq Ijoduola

PS Division

CERN

CH-1211 Geneva-23

Ulrich dot Raich at cern dot ch

Thanks  to Patrick Head for his contribution of the ASEZ80 cross

assembler.

Patrick Head

patrick at phead dot net

Page 7

ASxxxx Cross Assemblers, Version 4.00, November 2003

Submitted by Alan R.  Baldwin,

Kent State University, Kent, Ohio

Operating System:  MS-DOS, Windows, Linux

or other supporting ANSII C.

Source Langauge:  C

Abstract:

The  ASxxxx  assemblers are a series of microprocessor assem-

blers written in the C programming  language.   This  collection

contains  cross  assemblers  for the AVR, PIC, Rabbit 2000/3000,

ez80,  z80(hd64180),  GameBoy(Z80),  DS8xCxxx,  MPS430,  H8/3xx,

6800(6802/6808), 6801(hd6303), 6804, 6805, 68HC08, 6809, 68HC11,

68HC12, 68HC16, 8051, 8085(8080), 6500,  1802,  740,  and  61860

series  microprocessors.   Each  assembler has a device specific

section which includes:  (1) device description, byte order, and

file  extension  information,  (2)  a table of assembler general

directives, special directives, assembler  mnemonics  and  asso-

ciated operation codes, (3) machine specific code for processing

the device mnemonics, addressing modes, and special directives.

The assemblers have a common device independent section which

handles the details of file input/output, symbol  table  genera-

tion,  program/data  areas,  expression  analysis, and assembler

directive processing.

The  assemblers  provide  the following features:  (1) alpha-

betized, formatted symbol table listings, (2) relocatable object

modules, (3) global symbols for linking object modules, (4) con-

ditional assembly directives, (5) reusable  local  symbols,  and

(6) include-file processing.

The  companion program ASLINK is a relocating linker perform-

ing the following functions:  (1) bind multiple  object  modules

into  a  single  memory  image,  (2) resolve inter-module symbol

references,  (3)  resolve  undefined  symbols   from   specified

librarys of object modules, (4) process absolute, relative, con-

catenated, and overlay attributes in data and program  sections,

(5)  perform  byte and word program-counter relative (pc or pcr)

addressing calculations, (6) define absolute  symbol  values  at

link  time, (7) define absolute area base address values at link

time, (8) produce Intel Hex or Motorola S  record  output  file,

(9)  produce  a  map of the linked memory image, and (10) update

the ASxxxx assembler listing files with the absolute linked  ad-

dresses and data.

The  assemblers  and  linker  have  been  tested using Linux,

DJGPP,  Cygwin,  Symantec  C/C++   V6.1/V7.2,   and   VC6   with

MS-DOS/Windows  3.x/9x/NT/2000/XP.  Complete source code and do-

cumentation for the assemblers and linker is included  with  the

distribution.   Additionally,  test  code for each assembler and

several microprocessor monitors ( ASSIST05 for the 6805,  MONDEB

and  ASSIST09  for  the  6809, and BUFFALO 2.5 for the 6811) are

included as working examples of use of these assemblers.

CHAPTER 1

THE ASSEMBLER

1.1  THE ASXXXX ASSEMBLERS

The  ASxxxx  assemblers are a series of microprocessor assem-

blers written in the C programming language.  Each assembler has

a device specific section which includes:

1.  device  description, byte order, and file extension in-

formation

2.  a  table  of  the assembler general directives, special

device directives, assembler mnemonics  and  associated

operation codes

3.  machine specific code for processing the device mnemon-

ics, addressing modes, and special directives

The device specific information is detailed in the appendices.

The assemblers have a common device independent section which

handles the details of file input/output, symbol  table  genera-

tion,  program/data  areas,  expression  analysis, and assembler

directive processing.

The assemblers provide the following features:

1.  Command string control of assembly functions

2.  Alphabetized, formatted symbol table listing

3.  Relocatable object modules

4.  Global symbols for linking object modules

5.  Conditional assembly directives

THE ASSEMBLER                                           PAGE 1-2

THE ASXXXX ASSEMBLERS

6.  Program sectioning directives

ASxxxx assembles one or more source files into a single relo-

catable ascii object file.  The output of the ASxxxx  assemblers

consists of an ascii relocatable object file(*.rel), an assembly

listing file(*.lst), and a symbol file(*.sym).

1.1.1  Assembly Pass 1

During  pass  1, ASxxxx opens all source files and performs a

rudimenatry assembly of each source statement.  During this pro-

cess  all symbol tables are built, program sections defined, and

number of bytes for each assembled source line is estimated.

At the end of pass 1 all undefined symbols may be made global

(external) using the ASxxxx switch -g, otherwise undefined  sym-

bols will be flagged as errors during succeeding passes.

1.1.2  Assembly Pass 2

During  pass  2  the ASxxxx assembler resolves forward refer-

ences and determines the number  of  bytes  for  each  assembled

line.   The  number  of bytes used by a particular assembler in-

struction may depend upon the addressing mode, whether  the  in-

struction allows multiple forms based upon the relative distance

to the addressed location, or other factors.   Pass  2  resolves

these cases and determines the address of all symbols.

1.1.3  Assembly Pass 3

Pass 3 by the assembler generates the listing file, the relo-

catable output file, and the symbol tables.  Also during pass  3

the errors will be reported.

The  relocatable object file is an ascii file containing sym-

bol references and definitions, program  area  definitions,  and

the  relocatable assembled code, the linker ASLINK will use this

information to generate an absolute load file (Motorola or Intel

formats).

THE ASSEMBLER                                           PAGE 1-3

SOURCE PROGRAM FORMAT

1.2  SOURCE PROGRAM FORMAT

1.2.1  Statement Format

A source program is composed of assembly-language statements.

Each statement must be completed on one line.  A line  may  con-

tain a maximum of 128 characters, longer lines are truncated and

lost.

An  ASxxxx  assembler  statement  may  have  as  many as four

fields.  These fields are identified by their order  within  the

statement  and/or  by separating characters between fields.  The

general format of the ASxxxx statement is:

[label:]  Operator        Operand         [;Comment(s)]

The  label and comment fields are optional.  The operator and

operand fields are interdependent.  The operator field may be an

assembler  directive or an assembly mnemonic.  The operand field

may be optional or required as defined in  the  context  of  the

operator.

ASxxxx  interprets  and  processes source statements one at a

time.  Each statement causes a particular operation to  be  per-

formed.

1.2.1.1  Label Field  -

A  label is a user-defined symbol which is assigned the value

of the current location counter and entered into  the  user  de-

fined  symbol  table.   The  current location counter is used by

ASxxxx to assign memory addresses to the source  program  state-

ments as they are encountered during the assembly process.  Thus

a label is a means  of  symbolically  referring  to  a  specific

statement.

When  a program section is absolute, the value of the current

location counter is absolute;  its value references an  absolute

memory  address.   Similarly, when a program section is relocat-

able, the value of the current location counter is  relocatable.

A  relocation  bias  calculated at link time is added to the ap-

parent value of the current location counter  to  establish  its

effective  absolute  address  at  execution time.  (The user can

also force the linker to relocate sections defined as  absolute.

This may be required under special circumstances.)

If  present,  a  label  must  be  the first field in a source

statement and must be terminated by a colon (:).   For  example,

THE ASSEMBLER                                           PAGE 1-4

SOURCE PROGRAM FORMAT

if  the  value  of  the  current  location  counter  is absolute

01F0(H), the statement:

abcd:     nop

assigns  the  value  01F0(H) to the label abcd.  If the location

counter value were relocatable, the final value of abcd would be

01F0(H)+K, where K represents the relocation bias of the program

section, as calculated by the linker at link time.

More  than  one label may appear within a single label field.

Each label so specified is assigned the same address value.  For

example,  if  the  value  of  the  current  location  counter is

1FF0(H), the multiple labels in the following statement are each

assigned the value 1FF0(H):

abcd:     aq:     $abc:   nop

Multiple labels may also appear on successive lines.  For ex-

ample, the statements

abcd:

aq:

$abc:     nop

likewise  cause  the  same value to be assigned to all three la-

bels.

A  double  colon  (::)  defines the label as a global symbol.

For example, the statement

abcd::    nop

establishes the label abcd as a global symbol.  The distinguish-

ing attribute of a global symbol is that it  can  be  referenced

from  within an object module other than the module in which the

symbol is defined.  References to this label  in  other  modules

are  resolved when the modules are linked as a composite execut-

able image.

The legal characters for defining labels are:

A through Z

a through z

0 through 9

. (Period)

$ (Dollar sign)

_ (underscore)

A  label  may  be  any  length,  however  only  the  first 79

characters are significant and, therefore must be  unique  among

all   labels  in  the  source  program  (not  necessarily  among

THE ASSEMBLER                                           PAGE 1-5

SOURCE PROGRAM FORMAT

separately compiled modules).  An error code(s) (m or p) will be

generated  in the assembly listing if the first 79 characters in

two or more labels are the same.  The m code is  caused  by  the

redeclaration  of  the symbol or its reference by another state-

ment.  The p code is generated because the symbols  location  is

changing on each pass through the source file.

The  label  must  not  start with the characters 0-9, as this

designates a reusable symbol with special  attributes  described

in a later section.

The  label  must  not  start  with  the  sequence $$, as this

represents the temporary radix 16 for constants.

1.2.1.2  Operator Field  -

The  operator field specifies the action to be performed.  It

may consist of an instruction mnemonic (op code) or an assembler

directive.

When  the  operator is an instruction mnemonic, a machine in-

struction is generated and the assembler evaluates the addresses

of  the operands which follow.  When the operator is a directive

ASxxxx performs certain control actions or processing operations

during assembly of the source program.

Leading  and  trailing  spaces  or tabs in the operator field

have no significance;  such characters serve  only  to  separate

the operator field from the preceeding and following fields.

An operator is terminated by a space, tab or end of line.

1.2.1.3  Operand Field  -

When  the  operator is an instruction mnemonic (op code), the

operand  field  contains  program  variables  that  are  to   be

evaluated/manipulated by the operator.

Operands  may  be  expressions  or  symbols, depending on the

operator.  Multiple expressions used in the operand  fields  may

be  separated  by a comma.  An operand should be preceeded by an

operator field;  if it is not, the statement will give an  error

(q  or  o).   All  operands  following instruction mnemonics are

treated as expressions.

The operand field is terminated by a semicolon when the field

is followed  by  a  comment.   For  example,  in  the  following

statement:

label:    lda     abcd,x          ;Comment field

THE ASSEMBLER                                           PAGE 1-6

SOURCE PROGRAM FORMAT

the  tab  between lda and abcd terminates the operator field and

defines the beginning of the operand field;  a  comma  separates

the operands abcd and x;  and a semicolon terminates the operand

field and defines the beginning of the comment field.   When  no

comment  field  follows,  the operand field is terminated by the

end of the source line.

1.2.1.4  Comment Field  -

The comment field begins with a semicolon and extends through

the end of the line.  This field is optional and may contain any

7-bit ascii character except null.

Comments  do not affect assembly processing or program execu-

tion.

1.3  SYMBOLS AND EXPRESSIONS

This  section  describes the generic components of the ASxxxx

assemblers:  the character set, the conventions observed in con-

structing  symbols,  and  the use of numbers, operators, and ex-

pressions.

1.3.1  Character Set

The following characters are legal in ASxxxx source programs:

1.  The  letters  A  through Z.  Both upper- and lower-case

letters are acceptable.  The  assemblers,  by  default,

are  case  sensitive,  i.e.   ABCD and abcd are not the

same symbols.  (The assemblers can be made case  insen-

sitive by using the -z command line option.)

2.  The digits 0 through 9

3.  The  characters . (period), $ (dollar sign), and _ (un-

derscore).

4.  The special characters listed in Tables 1 through 6.

Tables  1  through  6  describe  the various ASxxxx label and

field terminators, assignment operators, operand separators, as-

sembly, unary, binary, and radix operators.

THE ASSEMBLER                                           PAGE 1-7

SYMBOLS AND EXPRESSIONS

Table 1         Label Terminators and Assignment Operators

----------------------------------------------------------------

:   Colon               Label terminator.

::  Double colon        Label  Terminator;   defines the

label as a global label.

=   Equal sign          Direct assignment operator.

==  Global equal        Direct assignment operator;  de-

fines the  symbol  as  a  global

symbol.

=:  Local equal         Direct assignment operator;  de-

fines the symbol as a local sym-

bol.

----------------------------------------------------------------

Table 2         Field Terminators and Operand Separators

----------------------------------------------------------------

Tab                 Item or field terminator.

Space               Item or field terminator.

,   Comma               Operand field separator.

;   Semicolon           Comment field indicator.

----------------------------------------------------------------

THE ASSEMBLER                                           PAGE 1-8

SYMBOLS AND EXPRESSIONS

Table 3         Assembler Operators

----------------------------------------------------------------

#   Number sign         Immediate expression indicator.

.   Period              Current location counter.

(   Left parenthesis    Expression delimiter.

)   Right parenthesis   Expression delimeter.

----------------------------------------------------------------

Table 4         Unary Operators

----------------------------------------------------------------

<   Left bracket        <FEDC   Produces  the lower byte

value of the expression.

(DC)

>   Right bracket       >FEDC   Produces  the upper byte

value of the expression.

(FE)

+   Plus sign           +A      Positive value of A

-   Minus sign          -A      Produces   the  negative

(2's complement) of A.

~   Tilde               ~A      Produces the 1's comple-

ment of A.

'   Single quote        'D      Produces  the  value  of

the character D.

"   Double quote        "AB     Produces the double byte

value for AB.

\   Backslash           '\n     Unix style characters

\b, \f, \n, \r, \t

or '\001   or octal byte values.

----------------------------------------------------------------

THE ASSEMBLER                                           PAGE 1-9

SYMBOLS AND EXPRESSIONS

Table 5         Binary Operators

----------------------------------------------------------------

<<  Double          0800 << 4   Produces the 4 bit

Left bracket                left-shifted   value  of

0800.  (8000)

>>  Double          0800 >> 4   Produces the 4 bit

Right bracket               right-shifted  value  of

0800.  (0080)

+   Plus sign       A + B       Arithmetic      Addition

operator.

-   Minus sign      A - B       Arithmetic   Subtraction

operator.

*   Asterisk        A * B       Arithmetic   Multiplica-

tion operator.

/   Slash           A / B       Arithmetic      Division

operator.

&   Ampersand       A & B       Logical AND operator.

|   Bar             A | B       Logical OR operator.

%   Percent sign    A % B       Modulus operator.

^   Up arrow or     A ^ B       EXCLUSIVE OR operator.

circumflex

----------------------------------------------------------------

Table 6         Temporary Radix Operators

----------------------------------------------------------------

$%,   0b, 0B            Binary radix operator.

$&,   0o, 0O, 0q, 0Q    Octal radix operator.

$#,   0d, 0D            Decimal radix operator.

$$,   0h, 0H, 0x, 0X    Hexidecimal radix operator.

Potential  ambiguities arising from the use of 0b and 0d

as temporary radix  operators  may  be  circumvented  by

THE ASSEMBLER                                          PAGE 1-10

SYMBOLS AND EXPRESSIONS

preceding  all non-prefixed hexidecimal numbers with 00.

Leading 0's are required in any  case  where  the  first

hexidecimal  digit is abcdef as the assembler will treat

the letter sequence as a label.

----------------------------------------------------------------

1.3.2  User-Defined Symbols

User-defined  symbols are those symbols that are equated to a

specific value through a direct assignment statement  or  appear

as  labels.  These symbols are added to the User Symbol Table as

they are encountered during assembly.

The following rules govern the creation of user-defined symbols:

1.  Symbols  can  be  composed  of alphanumeric characters,

dollar signs ($),  periods  (.),  and  underscores  (_)

only.

2.  The  first  character  of a symbol must not be a number

(except in the case of reusable symbols).

3.  The  first 79 characters of a symbol must be unique.  A

symbol  can  be  written  with  more  than   79   legal

characters,  but the 80th and subsequent characters are

ignored.

4.  Spaces and Tabs must not be embedded within a symbol.

1.3.3  Reusable Symbols

Reusable  symbols are specially formatted symbols used as la-

bels within a block of coding that has been delimited as a reus-

able symbol block.  Reusable symbols are of the form n$, where n

is a decimal integer from 0 to 65535,  inclusive.   Examples  of

reusable symbols are:

1$

27$

138$

244$

THE ASSEMBLER                                          PAGE 1-11

SYMBOLS AND EXPRESSIONS

The range of a reusable symbol block consists of those state-

ments between two normally constructed  symbolic  labels.   Note

that a statement of the form:

ALPHA = EXPRESSION

is a direct assignment statement but does not create a label and

thus does not delimit the range of a reusable symbol block.

Note  that  the  range  of a reusable symbol block may extend

across program areas.

Reusable symbols provide a convenient means of generating la-

bels for branch instructions and other  such  references  within

reusable symbol blocks.  Using reusable symbols reduces the pos-

sibility of symbols with multiple definitions appearing within a

user  program.   In  addition,  the use of reusable symbols dif-

ferentiates entry-point labels from other labels, since reusable

labels cannot be referenced from outside their respective symbol

blocks.  Thus, reusable symbols of the same name can  appear  in

other  symbol blocks without conflict.  Reusable symbols require

less symbol table space  than  normal  symbols.   Their  use  is

recommended.

The  use  of  the  same reusable symbol within a symbol block

will generate one or both of the m or p errors.

Example of reusable symbols:

a:      ldx     #atable ;get table address

lda     #0d48   ;table length

1$:     clr     ,x+     ;clear

deca

bne     1$

b:      ldx     #btable ;get table address

lda     #0d48   ;table length

1$:     clr     ,x+     ;clear

deca

bne     1$

THE ASSEMBLER                                          PAGE 1-12

SYMBOLS AND EXPRESSIONS

1.3.4  Current Location Counter

The  period  (.) is the symbol for the current location coun-

ter.  When used in the operand  field  of  an  instruction,  the

period   represents  the  address  of  the  first  byte  of  the

instruction:

AS:     ldx     #.      ;The period (.) refers to

;the address of the ldx

;instruction.

When  used  in  the  operand field of an ASxxxx directive, it

represents the address of the current byte or word:

QK = 0

.word   0xFFFE,.+4,QK   ;The operand .+4 in the .word

;directive represents a value

;stored in the second of the

;three words during assembly.

If  we  assume  the  current  value of the program counter is

0H0200, then during assembly, ASxxxx  reserves  three  words  of

storage  starting  at  location 0H0200.  The first value, a hex-

idecimal constant FFFE, will be stored at location 0H0200.   The

second  value  represented  by  .+4  will  be stored at location

0H0202, its value will be 0H0206 ( = 0H0202  +  4).   The  third

value  defined  by  the  symbol  QK  will  be placed at location

0H0204.

At the beginning of each assembly pass, ASxxxx resets the lo-

cation counter.  Normally, consecutive memory locations are  as-

signed  to  each  byte  of  object code generated.  However, the

value of the location counter can be changed  through  a  direct

assignment statement of the following form:

. = . + expression

The  new  location  counter can only be specified relative to

the current location counter.  Neglecting to specify the current

program  counter  along with the expression on the right side of

the assignment operator will generate the (.) error.   (Absolute

program areas may use the .org directive to specify the absolute

location of the current program counter.)

The following coding illustrates the use of the current location

counter:

.area   CODE1   (ABS)   ;program area CODE1

;is ABSOLUTE

THE ASSEMBLER                                          PAGE 1-13

SYMBOLS AND EXPRESSIONS

.org    0H100           ;set location to

;0H100 absolute

num1:   ldx     #.+0H10         ;The label num1 has

;the value 0H100.

;X is loaded with

;0H100 + 0H10

.org    0H130           ;location counter

;set to 0H130

num2:   ldy     #.              ;The label num2 has

;the value 0H130.

;Y is loaded with

;value 0H130.

.area   CODE2   (REL)   ;program area CODE2

;is RELOCATABLE

. = . + 0H20            ;Set location counter

;to relocatable 0H20 of

;the program section.

num3:   .word   0               ;The label num3 has

;the value

;of relocatable 0H20.

. = . + 0H40            ;will reserve 0H40

;bytes of storage as will

.blkb   0H40            ;or

.blkw   0H20

The  .blkb  and .blkw directives are the preferred methods of

allocating space.

1.3.5  Numbers

ASxxxx  assumes that all numbers in the source program are to

be interpreted in decimal radix unless otherwise specified.  The

.radix  directive  may  be used to specify the default as octal,

decimal, or hexidecimal.  Individual numbers can  be  designated

as  binary, octal, decimal, or hexidecimal through the temporary

radix prefixes shown in table 6.

Negative  numbers  must be preceeded by a minus sign;  ASxxxx

translates such numbers into two's  complement  form.   Positive

numbers may (but need not) be preceeded by a plus sign.

THE ASSEMBLER                                          PAGE 1-14

SYMBOLS AND EXPRESSIONS

Numbers are always considered to be absolute values, therefor

they are never relocatable.

1.3.6  Terms

A  term is a component of an expression and may be one of the

following:

1.  A number.

2.  A symbol:

1.  A  period (.) specified in an expression causes the

current location counter to be used.

2.  A User-defined symbol.

3.  An undefined symbol is assigned a value of zero and

inserted in the User-Defined symbol table as an un-

defined symbol.

3.  A single quote followed by a single ascii character, or

a double quote followed by two ascii characters.

4.  An  expression enclosed in parenthesis.  Any expression

so enclosed is evaluated and reduced to a  single  term

before  the remainder of the expression in which it ap-

pears is evaluated.  Parenthesis, for example,  may  be

used  to  alter the left-to-right evaluation of expres-

sions, (as in A*B+C versus A*(B+C)), or to apply a  un-

ary operator to an entire expression (as in -(A+B)).

5.  A unary operator followed by a symbol or number.

1.3.7  Expressions

Expressions  are  combinations  of  terms  joined together by

binary operators.  Expressions reduce to a value.   The  evalua-

tion  of  an expression includes the determination of its attri-

butes.  A resultant expression value may be one of  three  types

(as  described  later  in this section):  relocatable, absolute,

and external.

THE ASSEMBLER                                          PAGE 1-15

SYMBOLS AND EXPRESSIONS

Expressions are evaluate with an operand hierarchy as follows:

*       /       %       multiplication,

division, and

modulus first.

+       -               addition and

subtraction second.

<<      >>              left shift and

right shift third.

^                       exclusive or fourth.

&                       logical and fifth.

|                       logical or last

except that unary operators take precedence over binary

operators.

A  missing  or  illegal  operator  terminates  the expression

analysis, causing error codes (o) and/or  (q)  to  be  generated

depending upon the context of the expression itself.

At assembly time the value of an external (global) expression

is equal to the value of the absolute part of  that  expression.

For  example,  the expression external+4, where 'external' is an

external symbol, has the value of 4.  This expression,  however,

when  evaluated  at link time takes on the resolved value of the

symbol 'external', plus 4.

Expressions,  when  evaluated  by  ASxxxx,  are  one of three

types:  relocatable, absolute, or external.  The following  dis-

tinctions are important:

1.  An  expression is relocatable if its value is fixed re-

lative to the base address of the program area in which

it appears;  it will have an offset value added at link

time.  Terms that contain labels defined in relocatable

program  areas  will  have  a relocatable value;  simi-

larly, a period (.)  in  a  relocatable  program  area,

representing  the value of the current program location

counter, will also have a relocatable value.

2.  An  expression  is  absolute if its value is fixed.  An

expression whose terms are numbers and ascii characters

will  reduce  to  an absolute value.  A relocatable ex-

pression or term minus a relocatable term,  where  both

elements  being  evaluated  belong  to the same program

area, is an absolute expression.  This is because every

THE ASSEMBLER                                          PAGE 1-16

SYMBOLS AND EXPRESSIONS

term  in  a  program area has the same relocation bias.

When one term is subtracted from the other the  reloca-

tion bias is zero.

3.  An  expression is external (or global) if it contains a

single global reference (plus or minus an absolute  ex-

pression  value) that is not defined within the current

program.  Thus, an external  expression  is  only  par-

tially  defined following assembly and must be resolved

at link time.

1.4  GENERAL ASSEMBLER DIRECTIVES

An  ASxxxx  directive  is placed in the operator field of the

source line.  Only one directive is  allowed  per  source  line.

Each  directive  may  have  a blank operand field or one or more

operands.  Legal operands differ with each directive.

1.4.1  .module Directive

Format:

.module string

The .module directive causes the string to be included in the

assemblers output file as an identifier for this particular  ob-

ject  module.   The  string  may  be  from 1 to 79 characters in

length.  Only one identifier is allowed  per  assembled  module.

The  main use of this directive is to allow the linker to report

a modules' use of undefined symbols.  At link time all undefined

symbols  are  reported  and  the  modules  referencing  them are

listed.

1.4.2  .title Directive

Format:

.title  string

The .title directive provides a character string to be placed

on the second line of each page during listing.

THE ASSEMBLER                                          PAGE 1-17

GENERAL ASSEMBLER DIRECTIVES

1.4.3  .sbttl Directive

Format:

.sbttl  string

The .sbttl directive provides a character string to be placed

on the third line of each page during listing.

1.4.4  .page Directive

Format:

.page

The .page directive causes a page ejection with a new heading

to be printed.  The new page occurs after the next line  of  the

source  program is processed, this allows an immediately follow-

ing .sbttl directive to appear  on  the  new  page.   The  .page

source  line will not appear in the file listing.  Paging may be

disabled by invoking the -p directive.

1.4.5  .msg Directive

Format:

.msg text

where:  text    represents  a  delimited  string.  The string is

printed to the console during the final assembly

pass.

The  .msg  directive  is  useful to report assembly status or

other information during the assembly process.

1.4.6  .error Directive

Format:

.error exp

where:  exp     represents   an  absolute  expression.   If  the

evaluation of the expression results  in  a  non

zero value then an 'e' error is reported and the

text line is listed in the generated error.

THE ASSEMBLER                                          PAGE 1-18

GENERAL ASSEMBLER DIRECTIVES

The  .error  directive  is  useful to report configuration or

value errors during the assembly process.  (The .error directive

is  identical in function to the .assume directive, just perhaps

more descriptive.)

1.4.7  .byte, .db, and .fcb Directives

Format:

.byte   exp             ;Stores the binary value

.db     exp             ;of the expression in the

.fcb    exp             ;next byte.

.byte   exp1,exp2,expn  ;Stores the binary values

.db     exp1,exp2,expn  ;of the list of expressions

.fcb    exp1,exp2,expn  ;in successive bytes.

where:  exp,    represent expressions that will be

exp1,   truncated to 8-bits of data.

.       Each expression will be calculated,

.       the high-order byte will be truncated.

.       Multiple expressions must be

expn    separated by commas.

The  .byte, .db, or .fcb directives are used to generate suc-

cessive bytes of binary data in the object module.

1.4.8  .word, .dw, and .fdb Directives

Format:

.word   exp             ;Stores the binary value

.dw     exp             ;of the expression in

.fdb    exp             ;the next word.

.word   exp1,exp2,expn  ;Stores the binary values

.dw     exp1,exp2,expn  ;of the list of expressions

.fdb    exp1,exp2,expn  ;in successive words.

where:  exp,    represent expressions that will occupy two

exp1,   bytes of data. Each expression will be

.       calculated as a 16-bit word expression.

.       Multiple expressions must be

expn    separated by commas.

The  .word, .dw, or .fdb directives are used to generate suc-

cessive words of binary data in the object module.

THE ASSEMBLER                                          PAGE 1-19

GENERAL ASSEMBLER DIRECTIVES

1.4.9  .3byte and .triple Directives

Format:

.3byte  exp             ;Stores the binary value

.triple exp             ;of the expression in

;the next triple (3 bytes).

.3byte  exp1,exp2,expn  ;Stores the binary values

.triple exp1,exp2,expn  ;of the list of expressions

;in successive triples

;(3 bytes).

where:  exp,    represent expressions that will occupy three

exp1,   bytes of data. Each expression will be

.       calculated as a 24-bit word expression.

.       Multiple expressions must be

expn    separated by commas.

The  .3byte  or .triple directive is used to generate succes-

sive triples of binary data in the object module.  (These direc-

tives   are  only  available  in  assemblers  supporting  24-bit

addressing.)

1.4.10  .4byte and .quad Directive

Format:

.4byte  exp             ;Stores the binary value

.quad   exp             ;of the expression in

;the next quad (4 bytes).

.4byte  exp1,exp2,expn  ;Stores the binary values

.quad   exp1,exp2,expn  ;of the list of expressions

;in successive quads

;(4 bytes).

where:  exp,    represent expressions that will occupy three

exp1,   bytes of data. Each expression will be

.       calculated as a 32-bit word expression.

.       Multiple expressions must be

expn    separated by commas.

The  .4byte or .quad directive is used to generate successive

quads of binary data in the object  module.   (These  directives

are only available in assemblers supporting 32-bit addressing.)

THE ASSEMBLER                                          PAGE 1-20

GENERAL ASSEMBLER DIRECTIVES

1.4.11  .blkb, .ds, ,rmb, and .rs Directives

Format:

.blkb   N       ;reserve N bytes of space

.ds     N       ;reserve N bytes of space

.rmb    N       ;reserve N bytes of space

.rs     N       ;reserve N bytes of space

The  .blkb, .ds, .rmb, and .rs directives reserve byte blocks

in the object module;

1.4.12  .blkw, .blk3, and .blk4 Directives

Format:

.blkw   N       ;reserve N words of space

.blk3   N       ;reserve N triples of space

.blk4   N       ;reserve N quads of space

The .blkw directive reserves word blocks;  the .blk3 reserves

3  byte  blocks(available  in   assemblers   supporting   24-bit

addressing);  the .blk4 reserves 4 byte blocks (available in as-

semblers supporting 32-bit addressing).

1.4.13  .ascii, .str, and .fcc Directives

Format:

.ascii  /string/

.fcc    /string/

.str    /string/

where:  string  is a string of printable ascii characters.

/  /    represent   the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

delimiting  characters  do not match, the .ascii

directive will give the (q) error.

The  .ascii,  .fcc, and .str directives place one binary byte of

data for each character in the string into the object module.

THE ASSEMBLER                                          PAGE 1-21

GENERAL ASSEMBLER DIRECTIVES

1.4.14  .ascis and .strs Directives

Format:

.ascis  /string/

.strs   /string/

where:  string  is a string of printable ascii characters.

/  /    represent   the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

delimiting  characters  do not match, the .ascis

and .strs directives will give the (q) error.

The .ascis and .strs directives place one binary byte of data

for each character in the string into the  object  module.   The

last character in the string will have the high order bit set.

1.4.15  .asciz and .strz Directives

Format:

.asciz  /string/

.strz   /string/

where:  string  is a string of printable ascii characters.

/  /    represent   the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

delimiting  characters  do not match, the .asciz

and .strz directive will give the (q) error.

The .asciz and .strz directives place one binary byte of data

for each character in the string into the object  module.   Fol-

lowing  all  the  character data a zero byte is inserted to ter-

minate the character string.

THE ASSEMBLER                                          PAGE 1-22

GENERAL ASSEMBLER DIRECTIVES

1.4.16  .assume Directive

Format:

.assume exp

where:  exp     represents   an  absolute  expression.   If  the

evaluation of the expression results  in  a  non

zero value then an 'e' error is reported and the

text line is listed in the generated error.

The  .assume  directive  is useful to check assumptions about

assembler values.  (The .assume directive is identical in  func-

tion to the .error directive, just perhaps more descriptive.)

1.4.17  .radix Directive

Format:

.radix  character

where:  character  represents  a single character specifying the

default radix to be used for  succeeding  numbers.   The

character may be any one of the following:

B,b     Binary

O,o     Octal

Q,q

D,d     Decimal

'blank'

H,h     Hexidecimal

X,x

1.4.18  .even Directive

Format:

.even

The .even directive ensures that the current location counter

contains an even boundary value by adding 1 if the current loca-

tion is odd.

THE ASSEMBLER                                          PAGE 1-23

GENERAL ASSEMBLER DIRECTIVES

1.4.19  .odd Directive

Format:

.odd

The  .odd directive ensures that the current location counter

contains an odd boundary value by adding one if the current  lo-

cation is even.

1.4.20  .area Directive

Format:

.area   name    [(options)]

where:  name    represents the symbolic name of the program sec-

tion.   This  name  may  be  the  same  as   any

user-defined  symbol  or  bank as the area names

are independent  of  all  symbols,  labels,  and

banks.

options specify the type of program or data area:

ABS     absolute (automatically invokes OVR)

REL     relocatable

OVR     overlay

CON     concatenate

NOPAG   non-paged area

PAG     paged area

options specify a code or data segment:

CSEG    Code segment

DSEG    Data segment

option  specifies the data area bank:

BANK    Named collection of areas

The .area directive provides a means of defining and separat-

ing multiple programming and data sections.   The  name  is  the

area  label used by the assembler and the linker to collect code

from various separately assembled modules into one section.  The

name may be from 1 to 79 characters in length.

The options are specified within parenthesis and separated by

commas as shown in the following example:

.area  TEST  (REL,CON)  ;This section is relocatable

;and concatenated with other

;sections of this program area.

THE ASSEMBLER                                          PAGE 1-24

GENERAL ASSEMBLER DIRECTIVES

.area  DATA  (REL,OVR)  ;This section is relocatable

;and overlays other sections

;of this program area.

.area  SYS   (ABS,OVR)  ;(CON not allowed with ABS)

;This section is defined as

;absolute. Absolute sections

;are always overlayed with

;other sections of this program

;area.

.area  PAGE  (PAG)      ;This is a paged section. The

;section must be on a 256 byte

;boundary and its length is

;checked by the linker to be

;no larger than 256 bytes.

;This is useful for direct page

;areas.

The  default  area type is REL|CON;  i.e.  a relocatable sec-

tion which is concatenated with other sections of code with  the

same area name.  The ABS option indicates an absolute area.  The

OVR and CON options indicate if program  sections  of  the  same

name  will overlay each other (start at the same location) or be

concatenated with each other (appended to each other).

The  area can be specified as either a code segment, CSEG, or

a data segment, DSEG.  The CSEG and DSEG descriptors are  useful

when  the  microprocessor  code  and  data  unit allocations are

unequal:  e.g.  the executable code  uses  an  allocation  of  2

bytes for each instruction and is addressed at an increment of 1

for every instruction, and the data uses an allocation of 1 byte

for  each element and is addressed at an increment of 1 for each

data byte.  The allocation units are defined by the architecture

of the particular microprocessor.

The  .area  directive also provides a means of specifying the

bank this area is associated with.  All areas associated with  a

particular  bank  are  combined  at  link  time  into a block of

code/data.

The  CSEG,  DSEG,  and  BANK options are specified within the

parenthesis as shown in the following examples:

.area   C_SEG   (CSEG,BANK=C1)

;This is a code section

;and is included in bank C1

.area   D_SEG   (DSEG,BANK=D1)

;This is a data section

;and is included in bank D1.

THE ASSEMBLER                                          PAGE 1-25

GENERAL ASSEMBLER DIRECTIVES

Multiple  invocations  of  the  .area directive with the same

name must specify the same options or leave  the  options  field

blank,  this  defaults  to  the previously specified options for

this program area.

The   ASxxxx   assemblers   automatically  provide  two  program

sections:

'_CODE'         This  is  the  default  code/data  area.

This   program   area   is    of    type

(REL,CON,CSEG).

'_DATA'         This  is the default optional data area.

This   program   area   is    of    type

(REL,CON,DSEG).

The  ASxxxx  assemblers  also automatically generate two symbols

for each program area:

's_<area>'      This is the starting address of the pro-

gram area.

'l_<area>'      This is the length of the program area.

The .area names and options are never case sensitive.

1.4.21  .bank Directive

Format:

.bank   name    [(options)]

where:  name    represents  the  symbolic  name of the bank sec-

tion.   This  name  may  be  the  same  as   any

user-defined  symbol  or  area as the bank names

are independent  of  all  symbols,  labels,  and

areas.   The name may be from 1 to 79 characters

in length.

options specify the parameters of the bank:

BASE    base address of bank

SIZE    maximum size of bank

FSFX    file suffix for this bank

MAP     NOICE mapping

The  .bank  directive allows an arbitrary grouping of program

and/or data areas to be communicated to the  linker.   The  bank

parameters are all optional and are described as follows:

THE ASSEMBLER                                          PAGE 1-26

GENERAL ASSEMBLER DIRECTIVES

1.  BASE, the  starting  address of the bank (default is 0)

may be defined.  This address can be overridden by  us-

ing  the linker -b option for the first area within the

bank.  The bank address is always specified  in  'byte'

addressing.  A first area which is not 'byte' addressed

(e.g.  a processor addressed by a 'word' of 2  or  more

bytes)  has  the  area  address  scaled to begin at the

'byte' address.

2.  SIZE, the  maximum  length  of  the  bank  specified in

bytes.  The size is always specified in terms of bytes.

3.  FSFX, the file suffix to be used by the linker for this

bank.  The suffix may not contain embedded white space.

4.  MAP,  NOICE   mapping   parameter   for  this  bank  of

code/data.

The options are specified within parenthesis and separated by

commas as shown in the following example:

.BANK  C1  (BASE=0x0100,SIZE=0x1000,FSFX=_C1)

;This bank starts at 0x0100,

;has a maximum size of 0x1000,

;and is to be placed into

;a file with a suffix of _C1

The parameters must be absolute (external symbols are not al-

lowed.)

1.4.22  .org Directive

Format:

.org    exp

where:  exp     is  an absolute expression that becomes the cur-

rent location counter.

The  .org directive is valid only in an absolute program section

and will give a (q) error if used in a relocatable program area.

The  .org  directive specifies that the current location counter

is to become the specified absolute value.

THE ASSEMBLER                                          PAGE 1-27

GENERAL ASSEMBLER DIRECTIVES

1.4.23  .globl Directive

Format:

.globl  sym1,sym2,...,symn

where:  sym1,           represent legal symbolic names.

sym2,...        When multiple symbols are specified,

symn            they are separated by commas.

A  .globl directive may also have a label field and/or a com-

ment field.

The  .globl directive is provided to define (and thus provide

linkage to) symbols not  otherwise  defined  as  global  symbols

within  a  module.   In  defining  global  symbols the directive

.globl J is similar to:

J == expression or J::

Because  object  modules  are linked by global symbols, these

symbols are vital to a program.  All internal symbols  appearing

within  a  given program must be defined at the end of pass 1 or

they will be considered undefined.  The assembly directive  (-g)

can  be  be  invoked to make all undefined symbols global at the

end of pass 1.

1.4.24  .local Directive

Format:

.local  sym1,sym2,...,symn

where:  sym1,           represent legal symbolic names.

sym2,...        When multiple symbols are specified,

symn            they are separated by commas.

A  .local directive may also have a label field and/or a com-

ment field.

The  .local  directive is provided to define symbols that are

local to the current assembly process.  Local  symbols  are  not

effected  by  the assembler option -a (make all symbols global).

In defining local symbols the directive .local J is similar to:

J =: expression

The  .local directive and the =:  construct are useful in de-

fining symbols and constants within a header or definition  file

that  contains  many  symbols  specific  to the current assembly

process that should not be exported into the .rel  output  file.

THE ASSEMBLER                                          PAGE 1-28

GENERAL ASSEMBLER DIRECTIVES

A  typical  usage is in the definition of SFRs (Special Function

Registers) for a microprocessor.

The  .local  directive and =:  construct can be overridden by

the .globl directive.

1.4.25  .equ, .gblequ, and .lclequ Directives

Format:

sym1    .equ    expr    ; equivalent to sym1  = expr

sym2    .gblequ expr    ; equivalent to sym2 == expr

sym3    .lclequ expr    ; equivalent to sym3 =: expr

These  alternate  forms  of equivalence are provided for user

convenience.

1.4.26  .if, .else, and .endif Directives

Format:

.if     expr

.                       ;}

.                       ;} range of true condition

.                       ;}

.else

.                       ;}

.                       ;} range of false condition

.                       ;}

.endif

The  conditional  assembly directives allow you to include or

exclude blocks of source code during the assembly process, based

on the evaluation of the condition test.

The  range of true condition will be processed if the expres-

sion 'expr' is not zero (i.e.  true) and the range of false con-

dition  will  be processed if the expression 'expr' is zero (i.e

false).  The range of true condition is optional as is the .else

directive  and  the range of false condition.  The following are

all valid .if/.else/.endif constructions:

.if     A-4             ;evaluate A-4

.byte   1,2             ;insert bytes if A-4 is

.endif                  ;not zero

.if     K+3             ;evaluate K+3

.else

.byte   3,4             ;insert bytes if K+3

.endif                  ;is zero

THE ASSEMBLER                                          PAGE 1-29

GENERAL ASSEMBLER DIRECTIVES

.if     J&3             ;evaluate J masked by 3

.byte   12              ;insert this byte if J&3

.else                   ;is not zero

.byte   13              ;insert this byte if J&3

.endif                  ;is zero

The .if/.else/.endif directives may be nested upto 10 levels.

The  .page  directive  is  processed within a false condition

range to allow extended textual information to  be  incorporated

in  the  source  program  with  out  the need to use the comment

delimiter (;):

.if     0

.page

This text will be bypassed during assembly

but appear in the listing file.

.

.

.

.endif

1.4.27  .ifdef, .else, and .endif Directives

Format:

.ifdef  sym

.                       ;}

.                       ;} range of true condition

.                       ;}

.else

.                       ;}

.                       ;} range of false condition

.                       ;}

.endif

The  conditional  assembly directives allow you to include or

exclude blocks of source code during the assembly process, based

on the evaluation of the condition test.

The  range  of true condition will be processed if the symbol

'sym' is defined (i.e.  true) and the range of  false  condition

will  be  processed  if  the  symbol  'sym'  is not defined (i.e

false).  The range of true condition is optional as is the .else

directive  and  the range of false condition.  The following are

all valid .ifdef/.else/.endif constructions:

THE ASSEMBLER                                          PAGE 1-30

GENERAL ASSEMBLER DIRECTIVES

.ifdef  sym$1           ;lookup symbol sym$1

.byte   1,2             ;insert bytes if sym$1 is

.endif                  ;defined

.ifdef  sym$2           ;lookup symbol sym$2

.else

.byte   3,4             ;insert bytes if sym$1

.endif                  ;is not defined

.ifdef  sym$3           ;lookup symbol sym$3

.byte   12              ;insert this byte if sym$3

.else                   ;is defined

.byte   13              ;insert this byte if sym$3

.endif                  ;is not defined

The .ifdef/.else/.endif directives may be nested upto 10 levels.

1.4.28  .ifndef, .else, and .endif Directives

Format:

.ifndef sym

.                       ;}

.                       ;} range of true condition

.                       ;}

.else

.                       ;}

.                       ;} range of false condition

.                       ;}

.endif

The  conditional  assembly directives allow you to include or

exclude blocks of source code during the assembly process, based

on the evaluation of the condition test.

The  range  of true condition will be processed if the symbol

'sym' is not defined (i.e.  true) and the range of false  condi-

tion  will  be  processed  if  the  symbol 'sym' is defined (i.e

false).  The range of true condition is optional as is the .else

directive  and  the range of false condition.  The following are

all valid .ifndef/.else/.endif constructions:

.ifndef sym$1           ;lookup symbol sym$1

.byte   1,2             ;insert bytes if sym$1 is

.endif                  ;not defined

.ifndef sym$2           ;lookup symbol sym$2

.else

.byte   3,4             ;insert bytes if sym$1

.endif                  ;is defined

THE ASSEMBLER                                          PAGE 1-31

GENERAL ASSEMBLER DIRECTIVES

.ifndef sym$3           ;lookup symbol sym$3

.byte   12              ;insert this byte if sym$3

.else                   ;is not defined

.byte   13              ;insert this byte if sym$3

.endif                  ;is defined

The  .ifndef/.else/.endif  directives may be nested upto 10 lev-

els.

1.4.29  .include Directive

Format:

.include        string

where:  string  represents  a  delimited string that is the file

specification of an ASxxxx source file.

The .include directive is used to insert a source file within

the source file currently being assembled.  When this  directive

is encountered, an implicit .page directive is issued.  When the

end of the specified source file is reached, an  implicit  .page

directive is issued and input continues from the previous source

file.  The maximum nesting level of source files specified by  a

.include directive is five.

The  total  number  of separately specified .include files is

unlimited as each .include file is opened and then closed during

each pass made by the assembler.

The  default  directory  path,  if none is specified, for any

.include file is the directory path of the  current  file.   For

example:   if  the  current  source file, D:\proj\file1.asm, in-

cludes  a  file  specified   as   "include1"   then   the   file

D:\proj\include1.asm is opened.

1.4.30  .define and .undefine Directives

Format:

.define        keyword  string

.undefine      keyword

where:  keyword  is  the  substitutable  string which must start

with a letter and may contain any combination of

digits and letters.

THE ASSEMBLER                                          PAGE 1-32

GENERAL ASSEMBLER DIRECTIVES

where:  string   represents  a  delimited string that is substi-

tuted for the keyword.  The string  may  contain

any   sequence  of  characters  including  white

space.

The  .define  directive specifies a user defined string which

is substituted for the keyword.  The substitution string may it-

self  contain other keywords that are substitutable.  The assem-

bler resumes the parse of the line at the point the keyword  was

found.  Care must be excersized to avoid any circular references

within .define directives, otherwise the assembler may  enter  a

'recursion runaway' resulting in an 's' error.

The  .undefine  directive removes the keyword as a substitut-

able string.  No error is returned if the keyword  was  not  de-

fined.

1.4.31  .setdp Directive

Format:

.setdp [base [,area]]

The set direct page directive has a common format in all the as-

semblers supporting a paged mode.  The .setdp directive is  used

to  inform  the  assembler of the current direct page region and

the offset address within the selected area.  The normal invoca-

tion methods are:

.area   DIRECT  (PAG)

.setdp

or

.setdp  0,DIRECT

for  all  the  68xx microprocessors (the 6804 has only the paged

ram area).  The commands specify that the direct page is in area

DIRECT and its offset address is 0 (the only valid value for all

but the 6809 microprocessor).  Be sure to place the DIRECT  area

at address 0 during linking.  When the base address and area are

not specified, then zero and the current area are the  defaults.

If  a  .setdp directive is not issued the assembler defaults the

direct page to the area "_CODE" at offset 0.

The  assembler  verifies  that  any  local variable used in a

direct variable reference is located in this area.  Local  vari-

able  and  constant value direct access addresses are checked to

be within the address range from 0 to 255.

THE ASSEMBLER                                          PAGE 1-33

GENERAL ASSEMBLER DIRECTIVES

External direct references are assumed by the assembler to be

in the correct area and have valid  offsets.   The  linker  will

check all direct page relocations to verify that they are within

the correct area.

The  6809  microprocessor  allows the selection of the direct

page to be on any 256 byte boundary by loading  the  appropriate

value  into the dp register.  Typically one would like to select

the page boundary at link time, one method follows:

.area   DIRECT  (PAG)   ; define the direct page

.setdp

.

.

.

.area   PROGRAM

.

ldd     #DIRECT         ; load the direct page register

tfr     a,dp            ; for access to the direct page

At  link  time specify the base and global equates to locate the

direct page:

-b DIRECT = 0x1000

-g DIRECT = 0x1000

Both  the  area address and offset value must be specified (area

and variable names are independent).   The  linker  will  verify

that  the  relocated  direct page accesses are within the direct

page.

The  preceeding  sequence  could  be repeated for multiple paged

areas, however an alternate method is to define a non-paged area

and use the .setdp directive to specify the offset value:

.area   DIRECT          ; define non-paged area

.

.

.

.area   PROGRAM

.

.setdp  0,DIRECT        ; direct page area

ldd     #DIRECT         ; load the direct page register

tfr     a,dp            ; for access to the direct page

.

.

.setdp  0x100,DIRECT    ; direct page area

ldd     #DIRECT+0x100   ; load the direct page register

tfr     a,dp            ; for access to the direct page

The  linker  will  verify that subsequent direct page references

are in the specified area and offset address range.  It  is  the

programmers  responsibility  to  load  the  dp register with the

THE ASSEMBLER                                          PAGE 1-34

GENERAL ASSEMBLER DIRECTIVES

correct page segment corresponding to the  .setdp  base  address

specified.

For  those  cases  where a single piece of code must access a

defined data structure within a direct page and there  are  many

pages,  define  a  dumby  direct page linked at address 0.  This

dumby page is used only to define  the  variable  labels.   Then

load  the dp register with the real base address but donot use a

.setdp directive.  This method is equivalent to indexed address-

ing,  where the dp register is the index register and the direct

addressing is the offset.

1.4.32  .16bit, .24bit, and .32bit Directives

Format:

.16bit          ;specify 16-bit addressing

.24bit          ;specify 24-bit addressing

.32bit          ;specify 32-bit addressing

The  .16bit, .24bit, and .32bit directives are special direc-

tives for assembler configuration when default  values  are  not

used.  (Currently only the as8xCxxx and aspic assemblers can use

the .16bit, .24bit, or .32bit directives in special mode.)

1.4.33  .msb Directive

Format:

.msb    n

The  .msb  directive is only available in selected assemblers

which support 24 or 32-bit addressing.

The  assembler operator '>' selects the upper byte (MSB) when

included in an assembler  instruction.   The  default  assembler

mode  is  to  select bits <15:8> as the MSB.  The .msb directive

allows the programmer to specify a particular byte as the  'MSB'

when the address space is larger than 16-bits.

The assembler directive   .msb n  configures the assembler to

select a particular byte as MSB.  Given a 32-bit address of MNmn

(M(3)  is  <31:24>, N(2) is <23:16>, m(1) is <15:8>, and n(0) is

<7:0>) the following examples show how to  select  a  particular

address byte:

.msb 1          ;select byte 1 of address

;<M(3):N(2):m(1):n(0)>

THE ASSEMBLER                                          PAGE 1-35

GENERAL ASSEMBLER DIRECTIVES

LD A,>MNmn      ;byte m <15:8> ==>> A

...

.msb 2          ;select byte 2 of address

;<M(3):N(2):m(1):n(0)>

LD A,>MNmn      ;byte N <23:16> ==>> A

...

.msb 3          ;select byte 3 of address

;<M(3):N(2):m(1):n(0)>

LD A,>MNmn      ;byte M <31:24> ==>> A

...

1.4.34  .end Directive

Format:

.end

.end    exp

where:  exp     represents  any expression, including constants,

symbols, or labels.

The  .end  directive is used to specify a code entry point to

be included in the linker output file.  Review  the  I86  and  S

record formats described in the linker section for details.

The .end directive without an expression is ignored.

THE ASSEMBLER                                          PAGE 1-36

GENERAL ASSEMBLER DIRECTIVES

1.5  INVOKING ASXXXX

The ASxxxx assemblers are command line oriented.  Most sytems

require the option(s) and file(s) arguments to follow the ASxxxx

assembler name:

as6809 [-Options] file1 [file2 file3 ...]

Some  systems  may  request the arguments after the assembler is

started at a system specific prompt:

as6809

argv:  [-Options] file1 [file2 file3 ...]

The options are:

-d      decimal listing

-q      octal   listing

-x      hex     listing (default)

The listing radix affects the

.lst, .rel, and .sym files.

-g      undefined symbols made global

-a      all user symbols made global

-b      display .define substitutions in listing

-bb     and display without .define substitutions

-j      enable NOICE debug symbols

-y      enable SDCC debug symbols

-l      create list   output file1.lst

-o      create object output file1.rel

-s      create symbol output file1.sym

-p      disable listing pagination

-u      disable .list/.nlist processing

-w      wide listing format for symbol table

-z      disable case sensitivity for symbols

relocatable reference flagging:

-f      by  `   in the listing file

-ff     by mode in the listing file

The file name for the .lst, .rel, and .sym files is the first

file name specified in the command line.  All output  files  are

THE ASSEMBLER                                          PAGE 1-37

INVOKING ASXXXX

ascii  text  files which may be edited, copied, etc.  The output

files are the concatenation of all the input files, if files are

to  be  assembled  independently  invoke  the assembler for each

file.

The  .rel  file contains a radix directive so that the linker

will use the proper conversion for this file.  Linked files  may

have different radices.

If  the list (l) option is specified without the symbol table

(s) option, the symbol table is placed at the end of the listing

file.

1.6  ERRORS

The  ASxxxx assemblers provide limited diagnostic error codes

during the assembly process, these errors will be noted  in  the

listing file and printed on the stderr device.

The assembler reports the errors on the stderr device as

?ASxxxx-Error-<*> in line nnn of filename

where  * is the error code, nnn is the line number, and filename

is the source/include file.

The errors are:

(.)   This  error  is caused by an absolute direct assign-

ment of the current location counter

. = expression (incorrect)

rather than the correct

. = . + expression

(a)   Indicates  a machine specific addressing or address-

ing mode error.

(b)   Indicates a direct page boundary error.

(d)   Indicates a direct page addressing error.

(e)   Caused by a .error or .assume directive.

(i)   Caused  by  an  .include file error or an .if/.endif

mismatch.

(m)   Multiple  definitions  of  the  same label, multiple

.module directives, or multiple  conflicting  attri-

butes in an .area or .bank directive.

THE ASSEMBLER                                          PAGE 1-38

ERRORS

(o)   Directive  or  mnemonic error or the use of the .org

directive in a relocatable area.

(p)   Phase error:  label location changing between passes

2 and 3.  Normally caused by having  more  than  one

level of forward referencing.

(q)   Questionable syntax:  missing or improper operators,

terminators, or delimiters.

(r)   Relocation  error:   logic  operation attempted on a

relocatable term, addition of two relocatable terms,

subtraction  of two relocatable terms not within the

same programming area or external symbols.

(s)   String Substitution / recursion error.

(u)   Undefined symbol encountered during assembly.

(z)   Divide by 0 or Modulus by 0 error:  result is 0.

1.7  LISTING FILE

The  (-l) option produces an ascii output listing file.  Each

page of output contains a five line header:

1.  The ASxxxx program name and page number

2.  Assembler Radix and Address Bits

3.  Title from a .title directive (if any)

4.  Subtitle from a .sbttl directive (if any)

5.  Blank line

Each succeeding line contains five fields:

1.  Error field (first two characters of line)

2.  Current location counter

3.  Generated code in byte format

4.  Source text line number

THE ASSEMBLER                                          PAGE 1-39

LISTING FILE

5.  Source text

The error field may contain upto 2 error flags indicating any

errors encountered while assembling this line of source code.

The  current  location  counter  field  displays  the 16-bit,

24-bit, or 32-bit program position.  This field will be  in  the

selected radix.

The generated code follows the program location.  The listing

radix determines the number of bytes that will be  displayed  in

this field.  Hexidecimal listing allows six bytes of data within

the field, decimal and octal allow four bytes within the  field.

If more than one field of data is generated from the assembly of

a single line of source code, then the data field is repeated on

successive lines.

The source text line number is printed in decimal and is fol-

lowed by the source text.  A Source line with a .page  directive

is never listed.

Two  additional options are available for printing the source

line text.  If the -b option is specified then the listed source

line  contains all the .define substitutions.  If the -bb option

is specified then the original source line is printed before the

source line with substitutions.

Two  data  field  options  are  available to flag those bytes

which will be relocated by the linker.   If  the  -f  option  is

specified  then  each  byte to be relocated will be preceeded by

the '`' character.  If the -ff option  is  specified  then  each

byte  to  be relocated will be preceeded by one of the following

characters:

1.  *   paged relocation

2.  u   low  byte of unsigned word or unsigned byte

3.  v   high byte of unsigned word

4.  p   PCR low  byte of word relocation or PCR byte

5.  q   PCR high byte of word relocation

6.  r   low  byte relocation or byte relocation

7.  s   high byte relocation

Assemblers  which  use 24-bit or 32-bit addressing use an ex-

tended flagging mode:

THE ASSEMBLER                                          PAGE 1-40

LISTING FILE

1.  *   paged relocation

2.  u   1st  byte of unsigned value

3.  v   2nd  byte of unsigned value

4.  U   3rd  byte of unsigned value

5.  V   4th  byte of unsigned value

6.  p   PCR 1st  byte of relocation value or PCR byte

7.  q   PCR 2nd  byte of relocation value

8.  P   PCR 3rd  byte of relocation value

9.  Q   PCR 4th  byte of relocation value

10.  r   1st  byte of relocation value or byte relocation

11.  s   2nd  byte of relocation value

12.  R   3rd  byte of relocation value

13.  S   4th  byte of relocation value

1.8  SYMBOL TABLE FILE

The symbol table has two parts:

1.  The alphabetically sorted list of symbols and/or labels

defined or referenced in the source program.

2.  A  list of the program areas defined during assembly of

the source program.

The sorted list of symbols and/or labels contains the follow-

ing information:

1.  Program  area  number (none if absolute value or exter-

nal)

2.  The symbol or label

3.  Directly assigned symbol is denoted with an (=) sign

THE ASSEMBLER                                          PAGE 1-41

SYMBOL TABLE FILE

4.  The  value of a symbol, location of a label relative to

the  program  area  base  address  (=0),  or   a   ****

indicating the symbol or label is undefined.

5.  The  characters:   G - global, R - relocatable, and X -

external.

The list of program areas provides the correspondence between

the program area numbers and the defined program areas, the size

of the program areas, and the area flags (attributes).

1.9  OBJECT FILE

The  object  file is an ascii file containing the information

needed by the linker to bind multiple object modules into a com-

plete  loadable  memory  image.   The object module contains the

following designators:

[XDQ][HL][234]

X       Hexidecimal radix

D       Decimal radix

Q       Octal radix

H       Most significant byte first

L       Least significant byte first

2       16-Bit Addressing

3       24-Bit Addressing

4       32-Bit Addressing

H       Header

M       Module

G       Merge Mode

B       Bank

A       Area

S       Symbol

T       Object code

R       Relocation information

P       Paging information

Refer to the linker for a detailed description of each of the

designators and the format of the information contained  in  the

object file.

CHAPTER 2

THE LINKER

2.1  ASLINK RELOCATING LINKER

ASLINK  is  the  companion  linker for the ASxxxx assemblers.

The linker supports versions 3.xx and 4.xx of the ASxxxx  assem-

blers.   Object  files  from version 3 and 4 may be freely mixed

while linking.  Note that version 3 object files contain only  a

subset of the options available in version 4.

The  program ASLINK is a general relocating linker performing

the following functions:

1.  Bind multiple object modules into a single memory image

2.  Resolve inter-module symbol references

3.  Combine  code  belonging to the same area from multiple

object files into a single contiguous memory region

4.  Search and import object module libraries for undefined

global variables

5.  Perform   byte   and   word  program  counter  relative

(pc or pcr) addressing calculations

6.  Define absolute symbol values at link time

7.  Define absolute area base address values at link time

8.  Produce Intel Hex or Motorola S output files

9.  Produce a map of the linked memory image

10.  Produce  an updated listing file with the relocated ad-

dresses and data

THE LINKER                                              PAGE 2-2

INVOKING ASLINK

2.2  INVOKING ASLINK

Most  sytems require the options to be entered on the command

line:

aslink -[options] -[option arg] files

Some  systems  may  request the arguments after the linker is

started at a system specific prompt:

aslink

argv:  -[options] -[option arg] files

The allowed linker commands are:

1.  -c         ASlink >> prompt mode.

The  ASlink >>  prompt  mode reads linker commands from

stdin.

2.  -f  file   Command file mode.

The  command file mode imports linker commands from the

specified file (extension must be  .lnk),  imported  -c

and  -f  commands  are ignored.  If the directory path,

for a file to be linked, is not specified in  the  com-

mand  file  then  the  path  defaults  to the .lnk file

directory path.

3.  -p/-n   enable/disable echoing commands to stdout.

4.  -i/-s   Intel  Hex  (file.i--) or Motorola S (file.s--)

image output file.

5.  -o/-v   Specifies      that      subsequent      linked

files/libraries will generate object  output  (default)

or  suppress  object  output.   (if option -s or -i was

specified)

6.  -z      Disable Case Sensitivity for Symbols

7.  -m      Generate a map file (file.map).  This file con-

tains a list of the symbols (by area) with absolute ad-

dresses,  sizes  of  linked  areas,  and  other linking

information.

8.  -w      Specifies  that  a  wide listing format be used

for the map file.

9.  -xdq    Specifies  the  number  radix  for the map file

(Hexidecimal, Decimal, or Octal).

THE LINKER                                              PAGE 2-3

INVOKING ASLINK

10.  -u      Generate  an  updated  listing  file (file.rst)

derived from the relocated addresses and data from  the

linker.

11.  file    File(s) to be linked.  Files may be on the same

line as the above options or on a separate line(s)  one

file  per line or multiple files separated by spaces or

tabs.

12.  -b  area=expression

(one definition per line in a linker command file.)

This  specifies  an area base address where the expres-

sion may contain constants and/or defined symbols  from

the linked files.

13.  -g  symbol=expression

(one definition per line in a linker command file.)

This  specifies  the value for the symbol where the ex-

pression may contain constants and/or  defined  symbols

from the linked files.

14.  -k  library directory path

(one definition per line in a linker command file.)

This  specifies one possible path to an object library.

More than one path is allowed.

15.  -l  library file specification

(one definition per line in a linker command file.)

This  specifies a possible library file.  More than one

file is allowed.

16.  -e      or null line, terminates input to the linker.

2.3  LIBRARY PATH(S) AND FILE(S)

The process of resolving undefined symbols after scanning the

input object  files  includes  the  scanning  of  object  module

libraries.   The  linker will search through all combinations of

the library path specifications (input by the -k option) and the

library  file  specifications (input by the -l option) that lead

to an existing library file.  Each library file contains a  list

(one  file  per  line)  of  modules  included in this particular

library.  Each existing object module is scanned for a match  to

the undefined symbol.  The first module containing the symbol is

then linked with the previous modules to resolve the symbol  de-

finition.   The  library  object  modules are rescanned until no

more symbols can be resolved.   The  scanning  algorithm  allows

resolution  of  back references.  No errors are reported for non

existant library files or object modules.

THE LINKER                                              PAGE 2-4

LIBRARY PATH(S) AND FILE(S)

The  library  file  specification may be formed in one of two

ways:

1.  If  the  library  file  contained an absolute path/file

specification  then  this  is   the   object   module's

path/file.

(i.e.  C:\...  or C:/...)

2.  If  the  library  file  contains  a  relative path/file

specification then the concatenation of  the  path  and

this  file  specification  becomes  the object module's

path/file.

(i.e.  \...  or /...)

As  an example, assume there exists a library file termio.lib

in the syslib directory specifying the following object modules:

\6809\io_disk        first object module

d:\special\io_comm   second object module

and the following parameters were specified to the linker:

-k c:\iosystem\    the first path

-k c:\syslib\      the second path

-l termio          the first library file

-l io              the second library file (no such file)

The  linker  will attempt to use the following object modules to

resolve any undefined symbols:

c:\syslib\6809\io_disk.rel     (concatenated path/file)

d:\special\io_comm.rel         (absolute path/file)

all  other path(s)/file(s) don't exist.  (No errors are reported

for non existant path(s)/file(s).)

2.4  ASLINK PROCESSING

The  linker  processes  the  files  in  the  order  they  are

presented.  The first pass through the input files  is  used  to

define  all  program  areas, the section area sizes, and symbols

defined or referenced.  Undefined symbols will initiate a search

of any specified library file(s) and the importing of the module

containing the symbol definition.  After the first pass  the  -b

(area  base  address) definitions, if any, are processed and the

areas linked.

THE LINKER                                              PAGE 2-5

ASLINK PROCESSING

The  area  linking proceeds by first examining the area types

ABS, CON, REL, OVR and PAG.  Absolute areas (ABS) from  separate

object modules are always overlayed and have been assembled at a

specific address, these are not normally relocated (if a -b com-

mand  is  used  on an absolute area the area will be relocated).

Relative areas (normally defined as REL|CON) have a base address

of  0x0000  as read from the object files, the -b command speci-

fies the beginning address of the area.  All subsequent relative

areas  will  be  concatenated  with  proceeding  relative areas.

Where specific ordering is desired, the first linker input  file

should  have  the area definitions in the desired order.  At the

completion of the area linking all area  addresses  and  lengths

have  been determined.  The areas of type PAG are verified to be

on a 256 byte boundary and that the length does not  exceed  256

bytes.  Any errors are noted on stderr and in the map file.

Next  the  global symbol definitions (-g option), if any, are

processed.  The symbol definitions have been delayed until  this

point because the absolute addresses of all internal symbols are

known and can be used in the expression calculations.

Before  continuing  with the linking process the symbol table

is scanned to determine if any symbols have been referenced  but

not defined.  Undefined symbols are listed on the stderr device.

if a .module directive was included in the  assembled  file  the

module  making  the reference to this undefined variable will be

printed.

Constants  defined  as global in more than one module will be

flagged as multiple definitions if their values are not  identi-

cal.

After  the  preceeding  processes are complete the linker may

output a map file (-m option).  This file provides the following

information:

1.  Global symbol values and label absolute addresses

2.  Defined areas and there lengths

3.  Remaining undefined symbols

4.  List of modules linked

5.  List of library modules linked

6.  List of -b and -g definitions

THE LINKER                                              PAGE 2-6

ASLINK PROCESSING

The final step of the linking process is performed during the

second pass of the input files.  As the xxx.rel files  are  read

the code is relocated by substituting the physical addresses for

the referenced symbols and areas and may be output in  Intel  or

Motorola  formats.   The  number of files linked and symbols de-

fined/referenced is limited by the processor space available  to

build the area/symbol lists.  If the -u option is specified then

the listing files  (file.lst)  associated  with  the  relocation

files  (file.rel)  are  scanned  and  used  to create a new file

(file.rst) which has all addresses and data relocated  to  their

final values.

The  -o/-v  options  allow the simple creation of loadable or

overlay modules.  Loadable and overlay modules normally need  to

be  linked  with  a  main module(s) to resolve external symbols.

The -o/-v options can be used to enable object  output  for  the

loadable  or overlay module(s) and suppress the object code from

the linked main module(s).  The -o/-v  options  can  be  applied

repeatedly  to specify a single linked file, groups of files, or

libraries for object code inclusion or suppression.

THE LINKER                                              Page 2-7

ASXXXX VERSION 4.XX LINKING

2.5  ASXXXX VERSION 4.XX LINKING

The  linkers'  input  object file is an ascii file containing

the information needed by the linker  to  bind  multiple  object

modules into a complete loadable memory image.

The object module contains the following designators:

[XDQ][HL][234]

X       Hexidecimal radix

D       Decimal radix

Q       Octal radix

H       Most significant byte first

L       Least significant byte first

2       16-Bit Addressing

3       24-Bit Addressing

4       32-Bit Addressing

H       Header

M       Module

G       Merge Mode

B       Bank

A       Area

S       Symbol

T       Object code

R       Relocation information

P       Paging information

2.5.1  Object Module Format

The   first   line   of   an   object   module  contains  the

[XDQ][HL][234] format specifier  (i.e.   XH2  indicates  a  hex-

idecimal  file  with  most significant byte first and 16-bit ad-

dressing) for the following designators.

THE LINKER                                              PAGE 2-8

ASXXXX VERSION 4.XX LINKING

2.5.2  Header Line

H aa areas gg global symbols

The  header  line  specifies  the number of areas(aa) and the

number of global symbols(gg) defined or referenced in  this  ob-

ject module segment.

2.5.3  Module Line

M name

The  module  line  specifies  the module name from which this

header segment was assembled.  The module line will  not  appear

if the .module directive was not used in the source program.

2.5.4  Merge Mode Line

G nn ii 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

The  mode  structure  contains  the specification (or partial

specification) of one of the assemblers' merge  modes.   Sixteen

bits  may  be  specified  on a single line.  Each assembler must

specify at least one merge mode.  The merging specification  al-

lows  arbitrarily defined active bits and bit positions.  The 32

element arrays are indexed from 0 to 31.  Index 0 corresponds to

bit  0,  ...,  and  31 corresponds to bit 31 of a normal integer

value.

1.   nn is merge mode number

2.   ii is the beginning bit position of the following data

3.   00 ...  merge mode bit elements

The value of the element specifies if the normal in-

teger bit is active (bit <7> is set, 0x80) and  what

destination  bit  (bits  <4:0>,  0  -  31) should be

loaded with this normal integer bit.

THE LINKER                                              PAGE 2-9

ASXXXX VERSION 4.XX LINKING

2.5.5  Bank Line

B string base nn size nn map nn flags nn fsfx string

The  B  line  defines  a  bank name as 'string'.  A bank is a

structure containing a collection of areas.  The bank is treated

as  a  unique linking structure seperate from other banks.  Each

bank can have a unique base  address  (starting  address).   The

size  specification  may  be  used to signal the overflow of the

banks' allocated space.  The Linker combines all areas  included

within  a  bank  as  seperate from other areas.  The code from a

bank may be output to a unique file by specifying the File  Suf-

fix  parameter  (fsfx).   This allows the seperation of multiple

data and code segments into  isolated  output  files.   The  map

parameter  is  for  NOICE processing.  The flags indicate if the

parameters have been set.

2.5.6  Area Line

A label size ss flags ff

The  area  line  defines the area label, the size (ss) of the

area in bytes, and the area flags (ff).  The area flags  specify

the ABS, REL, CON, OVR, and PAG parameters:

OVR/CON  (0x04/0x00 i.e.  bit position 2)

ABS/REL  (0x08/0x00 i.e.  bit position 3)

PAG      (0x10 i.e.  bit position 4)

2.5.7  Symbol Line

S string Defnnnn

or

S string Refnnnn

The  symbol line defines (Def) or references (Ref) the symbol

'string' with the value nnnn.  The defined value is relative  to

the  current area base address.  References to constants and ex-

ternal global symbols will always appear before the  first  area

definition.  References to external symbols will have a value of

zero.

THE LINKER                                             PAGE 2-10

ASXXXX VERSION 4.XX LINKING

2.5.8  T Line

T xx xx nn nn nn nn nn ...

The  T  line contains the assembled code output by the assem-

bler with xx xx being the offset address from the  current  area

base address and nn being the assembled instructions and data in

byte format.  (xx xx and nn nn can be 2, 3, or 4 bytes as speci-

fied by the .REL file header.)

2.5.9  R Line

R 0 0 nn nn n1 n2 xx xx ...

The R line provides the relocation information to the linker.

The nn nn value is the current area index, i.e.  which area  the

current  values  were  assembled.  Relocation information is en-

coded in groups of 4 bytes:

1.  n1 is the relocation mode and object format.

1.  bits <1:0> specify the number of bytes to output

2.  bits <2:3> normal(0x00) /   MSB   (0x0C)

signed(0x04) / unsigned(0x08)

3.  bit 4  normal(0x00)/page   '0' (0x10) reference

4.  bit 5  normal(0x00)/page 'nnn' (0x20) reference

PAGX mode if both bits are set (0x30)

5.  bit 6  normal(0x00)/PC relative(0x40) relocation

6.  bit 7  relocatable area(0x00)/symbol(0x80)

2.  n2 is a byte index and a merge mode index

1.  bits <3:0>  are a byte index into the corresponding

(i.e.  preceeding) T line data (i.e.  a pointer  to

the data to be updated by the relocation).

2.  bits <7:4> are an index into a selected merge mode.

Currently mode 0 simply specifies to  use  standard

byte processing modes and merging is ignored.

3.  xx xx  is the area/symbol index for the area/symbol be-

ing referenced.  the corresponding area/symbol is found

in the header area/symbol lists.

The groups of 4 bytes are repeated for each item requiring relo-

cation in the preceeding T line.

THE LINKER                                             PAGE 2-11

ASXXXX VERSION 4.XX LINKING

2.5.10  P Line

P 0 0 nn nn n1 n2 xx xx

The  P  line provides the paging information to the linker as

specified by a .setdp directive.  The format of  the  relocation

information is identical to that of the R line.  The correspond-

ing T line has the following information:

T xx xx aa aa bb bb

Where  aa aa is the area reference number which specifies the

selected page area and bb bb is the base address  of  the  page.

bb bb will require relocation processing if the 'n1 n2 xx xx' is

specified in the P line.  The linker will verify that  the  base

address is on a 256 byte boundary and that the page length of an

area defined with the PAG type is not larger than 256 bytes.

The  linker  defaults any direct page references to the first

area defined in the input REL file.  All ASxxxx assemblers  will

specify the _CODE area first, making this the default page area.

2.5.11  24-Bit and 32-Bit Addressing

When  24-bit  or  32-bit  addressing is specified in the file

format line [XDQ][HL][234] then the S and T Lines have  modified

formats:

S string Defnnnnnn                      (24-bit)

S string Refnnnnnn                      (24-bit)

T xx xx xx nn nn nn nn nn ...           (24-bit)

S string Defnnnnnnnn                    (32-bit)

S string Refnnnnnnnn                    (32-bit)

T xx xx xx xx nn nn nn nn nn ...        (32-bit)

The  multibyte  formats for byte data replace the 2-byte form

for 16-bit data with 3-byte or 4-byte data for 24-bit or  32-bit

data  respectively.  The 2nd byte format (also named MSB) always

uses the second byte of the 2, 3, or 4-byte data.

THE LINKER                                             PAGE 2-12

ASXXXX VERSION 4.XX LINKING

2.5.12  ASlink V4.xx Error Messages

The linker provides detailed error messages allowing the pro-

grammer to quickly find the errant code.   As  the  linker  com-

pletes  pass 1  over  the  input  file(s)  it  reports  any page

boundary or page length errors as follows:

?ASlink-Warning-Paged Area PAGE0 Boundary Error

and/or

?ASlink-Warning-Paged Area PAGE0 Length Error

where PAGE0 is the paged area.

Also  during  Pass   1  any bank size (length) errors will be

reported as follows:

?ASlink-Warning-Size limit exceeded in bank BANK

where BANK is the bank name.

During  Pass  two the linker reads the T, R, and P lines per-

forming the necessary relocations and  outputting  the  absolute

code.  Various errors may be reported during this process

The P line processing can produce only one possible error:

?ASlink-Warning-Page Definition Boundary Error

file        module      pgarea    pgoffset

PgDef  t6809l      t6809l      PAGE0         0001

The error message specifies the file and module where the .setdp

direct was issued and indicates  the  page  area  and  the  page

offset value determined after relocation.

The R line processing produces various error messages:

?ASlink-Warning-Signed value error

?ASlink-Warning-Unsigned value error

?ASlink-Warning-Byte PCR relocation error

?ASlink-Warning-Word PCR relocation error

?ASlink-Warning-3-Byte PCR relocation error

?ASlink-Warning-4-Byte PCR relocation error

?ASlink-Warning-Page0 relocation error

?ASlink-Warning-PageN relocation error

?ASlink-Warning-PageX relocation error

?ASlink-Warning-Signed Merge Bit Range error

?ASlink-Warning-Unsigned/Overflow Merge Bit Range error

These  error  messages  also specify the file, module, area, and

offset within the area  of  the  code  referencing  (Refby)  and

THE LINKER                                             PAGE 2-13

ASXXXX VERSION 4.XX LINKING

defining (Defin) the symbol:

?ASlink-Warning-Signed value error for symbol  two56

file        module      area        offset

Refby  t           Pagetest    PROGRAM     0006

Defin  t           Pagetest    DIRECT      0100

If the symbol is defined in the same module as the reference the

linker is unable to report the symbol name.  The assembler list-

ing  file(s) should be examined at the offset from the specified

area to locate the offending code.

The errors are:

1.  The  Signed value error indicates an indexing value ex-

ceeded the maximum negative or maximum  positive  value

for the current variable size.

2.  The  Unsigned  value  error indicates an indexing value

was greater than maximum positive value for the current

variable size.

3.  The  byte PCR error is caused by exceeding the pc rela-

tive byte branch range.

4.  The  word PCR error is caused by exceeding the pc rela-

tive word branch range.

5.  The  3-byte PCR error is caused by exceeding the pc re-

lative 3-byte branch range.

6.  The  4-byte PCR error is caused by exceeding the pc re-

lative 4-byte branch range.

7.  The  Page0  error is generated if the direct page vari-

able is not in the page0 range of 0 to 255.

8.  The  PageN  error is generated if the direct page vari-

able is not within the Nth page range of 0 to 255.

9.  The  PageX  error is generated if the direct page vari-

able is not within the extended page range.

10.  The  Signed Merge Bit Range error indicates an indexing

value exceeded the maximum negative or maximum positive

value for the current signed merge variable size.

11.  The  Unsigned/Overflow  Merge Bit Range error indicates

an indexing value was  greater  than  maximum  positive

value for the current unsigned merge variable size.

THE LINKER                                             Page 2-14

ASXXXX VERSION 3.XX LINKING

2.6  ASXXXX VERSION 3.XX LINKING

The  linkers'  input  object file is an ascii file containing

the information needed by the linker  to  bind  multiple  object

modules into a complete loadable memory image.

The object module contains the following designators:

[XDQ][HL][234]

X       Hexidecimal radix

D       Decimal radix

Q       Octal radix

H       Most significant byte first

L       Least significant byte first

2       16-Bit Addressing

3       24-Bit Addressing

4       32-Bit Addressing

H       Header

M       Module

A       Area

S       Symbol

T       Object code

R       Relocation information

P       Paging information

2.6.1  Object Module Format

The   first   line   of   an   object   module  contains  the

[XDQ][HL][234] format specifier  (i.e.   XH2  indicates  a  hex-

idecimal  file  with  most significant byte first and 16-bit ad-

dressing) for the following designators.

2.6.2  Header Line

H aa areas gg global symbols

The  header  line  specifies  the number of areas(aa) and the

number of global symbols(gg) defined or referenced in  this  ob-

ject module segment.

THE LINKER                                             PAGE 2-15

ASXXXX VERSION 3.XX LINKING

2.6.3  Module Line

M name

The  module  line  specifies  the module name from which this

header segment was assembled.  The module line will  not  appear

if the .module directive was not used in the source program.

2.6.4  Area Line

A label size ss flags ff

The  area  line  defines the area label, the size (ss) of the

area in bytes, and the area flags (ff).  The area flags  specify

the ABS, REL, CON, OVR, and PAG parameters:

OVR/CON  (0x04/0x00 i.e.  bit position 2)

ABS/REL  (0x08/0x00 i.e.  bit position 3)

PAG      (0x10 i.e.  bit position 4)

2.6.5  Symbol Line

S string Defnnnn

or

S string Refnnnn

The  symbol line defines (Def) or references (Ref) the symbol

'string' with the value nnnn.  The defined value is relative  to

the  current area base address.  References to constants and ex-

ternal global symbols will always appear before the  first  area

definition.  References to external symbols will have a value of

zero.

2.6.6  T Line

T xx xx nn nn nn nn nn ...

The  T  line contains the assembled code output by the assem-

bler with xx xx being the offset address from the  current  area

base address and nn being the assembled instructions and data in

byte format.

THE LINKER                                             PAGE 2-16

ASXXXX VERSION 3.XX LINKING

2.6.7  R Line

R 0 0 nn nn n1 n2 xx xx ...

The R line provides the relocation information to the linker.

The nn nn value is the current area index, i.e.  which area  the

current  values  were  assembled.  Relocation information is en-

coded in groups of 4 bytes:

1.  n1  is  the  relocation mode and object format, for the

adhoc extension modes refer to asxxxx.h or aslink.h

1.  bit 0  word(0x00)/byte(0x01)

2.  bit 1  relocatable area(0x00)/symbol(0x02)

3.  bit 2  normal(0x00)/PC relative(0x04) relocation

4.  bit 3  1-byte(0x00)/2-byte(0x08) object format

5.  bit 4  signed(0x00)/unsigned(0x10) byte data

6.  bit 5  normal(0x00)/page   '0'(0x20) reference

7.  bit 6  normal(0x00)/page 'nnn'(0x40) reference

8.  bit 7  LSB  byte(0x00)/2nd  byte(0x80)  with 2-byte

mode

2.  n2  is  a byte index into the corresponding (i.e.  pre-

ceeding) T line data (i.e.  a pointer to the data to be

updated  by  the  relocation).   The T line data may be

1-byte or  2-byte  byte  data  format  or  2-byte  word

format.

3.  xx xx  is the area/symbol index for the area/symbol be-

ing referenced.  the corresponding area/symbol is found

in the header area/symbol lists.

The groups of 4 bytes are repeated for each item requiring relo-

cation in the preceeding T line.

2.6.8  P Line

P 0 0 nn nn n1 n2 xx xx

The  P  line provides the paging information to the linker as

specified by a .setdp directive.  The format of  the  relocation

information is identical to that of the R line.  The correspond-

ing T line has the following information:

T xx xx aa aa bb bb

Where  aa aa is the area reference number which specifies the

selected page area and bb bb is the base address  of  the  page.

bb bb will require relocation processing if the 'n1 n2 xx xx' is

specified in the P line.  The linker will verify that  the  base

address is on a 256 byte boundary and that the page length of an

area defined with the PAG type is not larger than 256 bytes.

THE LINKER                                             PAGE 2-17

ASXXXX VERSION 3.XX LINKING

The  linker  defaults any direct page references to the first

area defined in the input REL file.  All ASxxxx assemblers  will

specify the _CODE area first, making this the default page area.

2.6.9  24-Bit and 32-Bit Addressing

When  24-bit  or  32-bit  addressing is specified in the file

format line [XDQ][HL][234] then the S and T Lines have  modified

formats:

S string Defnnnnnn                      (24-bit)

S string Refnnnnnn                      (24-bit)

T xx xx xx nn nn nn nn nn ...           (24-bit)

S string Defnnnnnnnn                    (32-bit)

S string Refnnnnnnnn                    (32-bit)

T xx xx xx xx nn nn nn nn nn ...        (32-bit)

The  multibyte  formats for byte data replace the 2-byte form

for 16-bit data with 3-byte or 4-byte data for 24-bit or  32-bit

data  respectively.  The 2nd byte format (also named MSB) always

uses the second byte of the 2, 3, or 4-byte data.

2.6.10  ASlink V3.xx Error Messages

The linker provides detailed error messages allowing the pro-

grammer to quickly find the errant code.   As  the  linker  com-

pletes  pass 1  over  the  input  file(s)  it  reports  any page

boundary or page length errors as follows:

?ASlink-Warning-Paged Area PAGE0 Boundary Error

and/or

?ASlink-Warning-Paged Area PAGE0 Length Error

where PAGE0 is the paged area.

During  Pass  two the linker reads the T, R, and P lines per-

forming the necessary relocations and  outputting  the  absolute

code.  Various errors may be reported during this process

THE LINKER                                             PAGE 2-18

ASXXXX VERSION 3.XX LINKING

The P line processing can produce only one possible error:

?ASlink-Warning-Page Definition Boundary Error

file        module      pgarea      pgoffset

PgDef  t6809l      t6809l      PAGE0       0001

The error message specifies the file and module where the .setdp

direct was issued and indicates  the  page  area  and  the  page

offset value determined after relocation.

The R line processing produces various errors:

?ASlink-Warning-Byte PCR relocation error for symbol  bra2

?ASlink-Warning-Unsigned Byte error for symbol  two56

?ASlink-Warning-Page0 relocation error for symbol  ltwo56

?ASlink-Warning-Page Mode relocation error for symbol  two56

?ASlink-Warning-Page Mode relocation error

?ASlink-Warning-2K Page relocation error

?ASlink-Warning-512K Page relocation error

These  error  messages  also specify the file, module, area, and

offset within the area of the code referencing (Refby)  and  de-

fining (Defin) the symbol:

?ASlink-Warning-Unsigned Byte error for symbol  two56

file        module      area        offset

Refby  t6800l      t6800l      DIRECT      0015

Defin  tconst      tconst      .  .ABS.    0100

If the symbol is defined in the same module as the reference the

linker is unable to report the symbol name.  The assembler list-

ing  file(s) should be examined at the offset from the specified

area to locate the offending code.

The errors are:

1.  The  byte PCR error is caused by exceeding the pc rela-

tive byte branch range.

2.  The Unsigned byte error indicates an indexing value was

negative or larger than 255.

3.  The  Page0  error is generated if the direct page vari-

able is not in the page0 range of 0 to 255.

4.  The page mode error is generated if the direct variable

is not within the current direct page (6809).

5.  The  2K  Page  relocation  error  is  generated  if the

destination is not within the current  2K  page  (8051,

DS8xCxxx).

THE LINKER                                             PAGE 2-19

ASXXXX VERSION 3.XX LINKING

6.  The  512K  Page  relocation  error  is generated if the

destination  is  not  within  the  current  512K   page

(DS80C390).

THE LINKER                                             Page 2-20

INTEL IHX OUTPUT FORMAT

2.7  INTEL IHX OUTPUT FORMAT (16-BIT)

Record Mark Field    -  This  field  signifies  the  start  of a

record, and consists of an  ascii  colon

(:).

Record Length Field  -  This   field   consists   of  two  ascii

characters which indicate the number  of

data   bytes   in   this   record.   The

characters are the result of  converting

the  number  of  bytes  in binary to two

ascii characters, high digit first.   An

End  of  File  record contains two ascii

zeros in this field.

Load Address Field   -  This  field  consists  of the four ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either four ascii zeros or  the

program entry address.

Record Type Field    -  This  field  identifies the record type,

which is either 0 for data records or  1

for  an End of File record.  It consists

of two ascii characters, with  the  high

digit of the record type first, followed

by the low digit of the record type.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address  field,  the  record type field,

and the data field.  This  sum  is  then

negated  (2's  complement) and converted

to  two  ascii  characters,  high  digit

first.

THE LINKER                                             Page 2-21

INTEL I86 OUTPUT FORMAT

2.8  INTEL I86 OUTPUT FORMAT (24 OR 32-BIT)

Record Mark Field    -  This  field  signifies  the  start  of a

record, and consists of an  ascii  colon

(:).

Record Length Field  -  This   field   consists   of  two  ascii

characters which indicate the number  of

data   bytes   in   this   record.   The

characters are the result of  converting

the  number  of  bytes  in binary to two

ascii characters, high digit first.   An

End  of  File  record contains two ascii

zeros in this field.

Load Address Field   -  This  field  consists  of the four ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either four ascii zeros or  the

program entry address.

Record Type Field    -  This  field  identifies the record type,

which is either 0 for  data  records,  1

for  an  End  of File record, or 4 for a

segment  record.   It  consists  of  two

ascii characters, with the high digit of

the record type first, followed  by  the

low digit of the record type.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address  field,  the  record type field,

and the data field.  This  sum  is  then

negated  (2's  complement) and converted

to  two  ascii  characters,  high  digit

first.

THE LINKER                                             Page 2-22

MOTOROLA S1-S9 OUTPUT FORMAT

2.9  MOTORLA S1-S9 OUTPUT FORMAT (16-BIT)

Record Type Field    -  This  field  signifies  the  start  of a

record and  identifies  the  the  record

type as follows:

Ascii S1 - Data Record

Ascii S9 - End of File Record

Record Length Field  -  This  field  specifies the record length

which includes the  address,  data,  and

checksum   fields.   The  8  bit  record

length value is converted to  two  ascii

characters, high digit first.

Load Address Field   -  This  field  consists  of the four ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either four ascii zeros or  the

program entry address.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address field, and the data field.  This

sum is then  complemented  (1's  comple-

ment)   and   converted   to  two  ascii

characters, high digit first.

THE LINKER                                             Page 2-23

MOTOROLA S2-S8 OUTPUT FORMAT

2.10  MOTORLA S2-S8 OUTPUT FORMAT (24-BIT)

Record Type Field    -  This  field  signifies  the  start  of a

record and  identifies  the  the  record

type as follows:

Ascii S2 - Data Record

Ascii S8 - End of File Record

Record Length Field  -  This  field  specifies the record length

which includes the  address,  data,  and

checksum   fields.   The  8  bit  record

length value is converted to  two  ascii

characters, high digit first.

Load Address Field   -  This  field  consists  of  the six ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of 3rd byte of address.

Low digit of 3rd byte of address.

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either six ascii zeros  or  the

program entry address.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address field, and the data field.  This

sum is then  complemented  (1's  comple-

ment)   and   converted   to  two  ascii

characters, high digit first.

THE LINKER                                             Page 2-24

MOTOROLA S3-S7 OUTPUT FORMAT

2.11  MOTORLA S3-S7 OUTPUT FORMAT (32-BIT)

Record Type Field    -  This  field  signifies  the  start  of a

record and  identifies  the  the  record

type as follows:

Ascii S3 - Data Record

Ascii S7 - End of File Record

Record Length Field  -  This  field  specifies the record length

which includes the  address,  data,  and

checksum   fields.   The  8  bit  record

length value is converted to  two  ascii

characters, high digit first.

Load Address Field   -  This  field  consists of the eight ascii

characters which result from  converting

the  the  binary value of the address in

which to begin loading this record.  The

order is as follows:

High digit of 4th byte of address.

Low digit of 4th byte of address.

High digit of 3rd byte of address.

Low digit of 3rd byte of address.

High digit of high byte of address.

Low digit of high byte of address.

High digit of low byte of address.

Low digit of low byte of address.

In an End of File record this field con-

sists of either eight ascii zeros or the

program entry address.

Data Field           -  This  field consists of the actual data,

converted to two ascii characters,  high

digit first.  There are no data bytes in

the End of File record.

Checksum Field       -  The  checksum  field is the 8 bit binary

sum of the record length field, the load

address field, and the data field.  This

sum is then  complemented  (1's  comple-

ment)   and   converted   to  two  ascii

characters, high digit first.

CHAPTER 3

BUILDING ASXXXX AND ASLINK

The assemblers and linker have been successfully compiled for

Linux, DOS,and various flavors of Windows  using  GCC,  Symantec

C/C++  V6.1/V7.2,  DJGPP,  Cygwin,  and  VC6.  A Linux makefile,

DJGPP makefile, Symantec project files, Cygwin makefile, and VC6

project  files are available to build all the assemblers and the

linker.

3.1  BUILDING AN ASSEMBLER

The  building  of  a typical assembler (6809 for example) re-

quires the following files:

1.  m6809.h

2.  m09ext.c

3.  m09mch.c

4.  m09adr.c

5.  m09pst.c

6.  asxxxx.h

7.  asmain.c

8.  asdbg.c

9.  aslex.c

10.  assym.c

11.  assubr.c

12.  asexpr.c

13.  asdata.c

14.  aslist.c

15.  asout.c

The  first  five  files are the 6809 processor dependent sec-

tions which contain the following:

BUILDING ASXXXX AND ASLINK                              PAGE 3-2

BUILDING AN ASSEMBLER

1.  m6809.h -  header  file containing the machine specific

definitions of constants,  variables,  structures,  and

types

2.  m09ext -  device  description, byte order, and file ex-

tension information

3.  m09pst -  a  table of the assembler general directives,

special device directives, and assembler mnemonics with

associated operation codes

4.  m09mch / m09adr -  machine specific code for processing

the device mnemonics,  addressing  modes,  and  special

directives

The  remaining  files provide the device independent sections

which handle the details  of  file  input/output,  symbol  table

generation,  program/data areas, expression analysis, and assem-

bler directive processing.

3.2  BUILDING ASLINK

The building of the linker requires the following files:

1.  aslink.h

2.  lkmain.c

3.  lklex.c

4.  lkarea.c

5.  lkbank.c

6.  lkhead.c

7.  lksym.c

8.  lkeval.c

9.  lkdata.c

10.  lklist.c

11.  lkrloc.c

12.  lkrloc3.c

13.  lkrloc4.c

14.  lklibr.c

15.  lknoice.c

16.  lksdcdb.c

17.  lkout.c

APPENDIX A

ASAVR ASSEMBLER

A.1  AVR ASSEMBLER NOTES

The  AVR  series  of  processors  uses  a non unified addressing

scheme:  the instruction addressing is 1 per  instruction  word,

each  instruction uses 2 bytes of memory.  The processor data is

addressed as 1 per byte of data.  To properly address  the  pro-

gram/data spaces you, the programmer, must seperate your program

and data into seperate code and data areas.  The  data  area  is

addressed  as 1 per byte and the code area is addressed as 1 per

instruction.

The  assembler/linker  processes the instruction code so that

the linker will output 2 bytes for each instruction  word.   The

instruction  word  address  will  be  the  file  encoded address

divided by 2.

A.1.1  Processor Specific Directive

The  normal PC relative addressing is -2047 to +2048 relative

to the current PC.  For a processor with less than 4K  words  of

program space the AVR relative jump/call can access any location

due to address wrap around.

The  ASAVR cross assembler has one (1) processor specific as-

sembler directive which tells the assembler that the AVR has  4K

words or less of program space.

.avr_4k   0     Normal PC Relative addressing

.avr_4k   1     AVR with <= 4K of Memory

The  remaining  processor specific directives specify the AVR

processor type.

.AT90SXXXX

ASAVR ASSEMBLER                                         PAGE A-2

AVR ASSEMBLER NOTES

.AT90S1200

.AT90S2313

.AT90S2323

.AT90S2343

.AT90S2333

.AT90S4433

.AT90S4414

.AT90S4434

.AT90S8515

.AT90C8534

.AT90S8535

.ATmega103

.ATmega603

.ATmega161

.ATmega163

.ATtiny10

.ATtiny11

.ATtiny12

.ATtiny15

.ATtiny22

.ATtiny28

A  file, avr.sfr, contains definitions for the Spepcial Func-

tion Registers for all the defined processors.  Edit the file to

make  your  selection of processor and then .include the file at

the beginning of your assembler file.

A.2  AVR REGISTER SET

The following is a list of the AVR registers used by ASAVR:

r0-r31  -       8-bit registers

x       -       index register (x = r27:r26)

y       -       index register (y = r29:r28)

z       -       index register (z = r31:r30)

A.3  AVR INSTRUCTION SET

The following tables list all AVR mnemonics recognized by the

ASAVR assembler.  The designation [] refers to  a  required  ad-

dressing mode argument.  The following list specifies the format

for each addressing mode supported by ASAVR:

#data           immediate data

expr            expression

Rd              destination register (0-31)

ASAVR ASSEMBLER                                         PAGE A-3

AVR INSTRUCTION SET

Rd,Rs           destination register (0-31)

source register (0-31)

Rd,#data        destination register (0-31)

immediate data

addr            address

addr,Rs         destination address

source register

Rd,addr         destination register

source address

Rs,b            source register

bit position

Rd,b            destination register

bit position

A               an I/O register (0-31)

A,b             an I/O register (0-31)

bit position

A,Rs            source register to

output register

Rd,A            input register to

destination register

Rd,X            load indirect

Rd,Y

Rd,Z

Rd,-X           load indirect pre-decrement

Rd,-Y

Rd,-Z

Rd,X+           load indirect post-increment

Rd,Y+

Rd,Z+

Rd,Z+Q          load indirect with displacement

X,Rs            store indirect

Y,Rs

Z,Rs

-X,Rs           store indirect pre-decrement

-Y,Rs

-Z,Rs

ASAVR ASSEMBLER                                         PAGE A-4

AVR INSTRUCTION SET

X+,Rs           store indirect post increment

Y+,Rs

Z+,Rs

Z+Q,Rs          store indirect with displacement

label           branch label

The  terms  data, expr, displacement, bit position, A, and label

may be expressions.

Note  that not all instructions are available with every pro-

cessor type.  Not all addressing modes are valid with every  in-

struction,   refer   to   the   AVR  technical  data  for  valid

instructions and modes.

A.3.1  AVR Arithmetic and Logical Instructions

add     Rd,Rs           adc     Rd,Rs

adiw    Rd,#data        sub     Rd,Rs

subi    Rd,#data        sbc     Rd,Rs

sbci    Rd,#data        sbiw    Rd,#data

and     Rd,Rs           andi    Rd,#data

or      Rd,Rs           ori     Rd,#data

cp      Rd,Rs           eor     Rd,Rs

cpi     Rd,#data        cpc     Rd,Rs

cbr     Rd,#data        sbr     Rd,#data

clr     Rd              com     Rd

dec     Rd              inc     Rd

neg     Rd              ser     Rd

tst     Rd

mul     Rd,Rs           fmul    Rd,Rs

muls    Rd,Rs           fmuls   Rd,Rs

mulsu   Rd,Rs           fmulsu  Rd,Rs

ASAVR ASSEMBLER                                         PAGE A-5

AVR INSTRUCTION SET

A.3.2  AVR Bit and Bit-Test Instructions

lsl     Rd              lsr     Rd

rol     Rd              ror     Rd

asr     Rd              swap    Rd

bset    b               bclr    b

sbi     A,b             cbi     A,b

bst     Rs,b            bld     Rd,b

sec                     sez

sen                     sev

ses                     seh

set                     sei

clc                     clz

cln                     clv

cls                     clh

clt                     cli

nop                     sleep

wdr

A.3.3  AVR Skip on Test Instructions

cpse    Rd,Rs

sbrc    Rs,b            sbrs    Rs,b

sbic    A,b             sbis    A,b

A.3.4  AVR Jump/Call/Return Instructions

jmp     addr            rjmp    addr

ijmp                    eijmp

call    addr            rcall   addr

icall                   eicall

ret                     reti

A.3.5  AVR Short Branch Instructions

brcc    label           brcs    label

breq    label           brge    label

brhc    label           brhs    label

brid    label           brie    label

brlo    label           brlt    label

brmi    label           brne    label

brpl    label           brsh    label

brtc    label           brts    label

brvc    label           brvs    label

ASAVR ASSEMBLER                                         PAGE A-6

AVR INSTRUCTION SET

A.3.6  AVR Short Branch Instructions with Bit Test

brbc    b,label         brbs    b,label

A.3.7  AVR Data Transfer Instructions

mov     Rd,Rs           movw    Rd,Rs

ldi     Rd,#data

ld      []              st      []

ldd     []              std     []

lds     Rd,addr         sts     addr,Rs

lpm     []              elpm    []

spm

push    Rs              pop     Rd

in      Rd,A            out     A,Rs

APPENDIX B

ASPIC ASSEMBLER

B.1  PIC ASSEMBLER NOTES

The  PIC  series  of  processors  uses  a non unified addressing

scheme:  the instruction addressing is 1 per  instruction  word,

each  instruction  uses  a  word of memory varying from 12 to 16

bits in length.  The processor data is addressed as 1  per  byte

of  data.   To properly address the program/data spaces you, the

programmer, must seperate your program and  data  into  seperate

code  and  data areas.  The data area is addressed as 1 per byte

and the code area is addressed as 1 per instruction.

The  assembler/linker  processes the instruction code so that

the linker will output 2 bytes for each instruction  word.   The

instruction  word  address  will  be  the  file  encoded address

divided by 2.

B.2  PROCESSOR SPECIFIC DIRECTIVES

The  ASPIC assembler has several processor specific assembler

directives.  These directives specify a processor name, select a

PIC  processor  family  type,  define  the  maximum ram address,

specify ram addresses that should not be  accessed,  and  define

the register file address page.

ASPIC ASSEMBLER                                         PAGE B-2

PROCESSOR SPECIFIC DIRECTIVES

B.2.1  .pic Directive

Format:

.pic    string

The assembler uses the delimited string to define a processor

specific symbol.  e.g:  "p12c508" produces the  symbol  __12c508

having  a  value  of  1.   This  symbol  can  then be used in an

.ifdef/.else/.endif construct.

The  assembler  should  be configured by including directives

similiar to the folowing at the beginning of your assembly file:

.pic            "p12c508"       ; Set PIC Name

.pic12bit                       ; Select PIC Type

The  ASPIC assembler will then be configured for the PIC pro-

cessor type "p12c508".  The .pic directive must precede the  PIC

type directive.  The PIC type directive configures the assembler

base on the processor name and type selection.

An  alternate  method  to configure the ASPIC assembler is as

follows:

.pic            "p12c508"       ; Set PIC Name

.include        "piccpu.def"    ; Selects PIC Type

To  define  the  special function register names, bit values,

and memory constraints for  a  specific  processor  include  the

appropriate definition file:

.include        "p12c508.def"   ; Definitions

B.2.2  .picnopic Directive

Format:

.picnopic

This directive deselects all processor specific mnemonics.

ASPIC ASSEMBLER                                         PAGE B-3

PROCESSOR SPECIFIC DIRECTIVES

B.2.3  .pic12bit Directive

Format:

.pic12bit

This  directive selects the 12-bit instruction word mnemonics

and opcode values to be used during the assembly process.

B.2.4  .pic14bit Directive

Format:

.pic14bit

This  directive selects the 14-bit instruction word mnemonics

and opcode values to be used during the assembly process.

B.2.5  .pic16bit Directive

Format:

.pic16bit

This  directive selects the 16-bit instruction word mnemonics

and opcode values to be used during the assembly process.

B.2.6  .pic20bit Directive

Format:

.pic20bit

This  directive  selects 20-bit addressing and the 16-bit in-

struction word mnemonics and opcode values to be used during the

assembly process.

ASPIC ASSEMBLER                                         PAGE B-4

PROCESSOR SPECIFIC DIRECTIVES

B.2.7  .picfix Directive

Format:

.picfix chip,   mnemonic,       value

This  directive  can  be  used  to "fix" or change the opcode

value of any pic instruction of the currently selected pic type.

e.g.:

.picfix      "p12c671",      "clrw", 0x0103

will  change  the  "clrw"  instruction's opcode to 0x0103 if the

current pic type is "p12c671".

B.2.8  .maxram Directive

Format:

.maxram value

Where value is the highest allowed ram address

B.2.9  .badram Directive

Format:

.badram address

.badram lo:hi

Where  address  is  a single location and lo:hi is a range of

addresses that should not be used.   Multiple  locations  and/or

ranges  may  be  specified  by  seperating  the arguments with a

comma:

.badram         0x23, 0x28:0x2F, ...

The  ASPIC  assembler  will  report an error for any absolute

register file address in the badram range.

ASPIC ASSEMBLER                                         PAGE B-5

PROCESSOR SPECIFIC DIRECTIVES

B.2.10  .setdmm Directive

Format:

.setdmm value

The .setdmm (set Data Memory Map) directive is used to inform

the assembler and linker about which ram bank has been  selected

for  access.   The PIC17Cxxx microprocessor family allows upto 2

(or more) banks of 256 byte ram blocks.  The PIC18Cxxx micropro-

cessor  family allows upto 16 banks of 256 byte ram blocks.  The

data memory map value must be set on a 256 byte boundary.  e.g.:

.setdmm      0x0F00

The  assembler  verifies  that  any  absolute  address to the

register file is within the  256  byte  page.   External  direct

references  are  assumed  by  the assembler to be in the correct

area and have valid offsets.  The linker  will  check  all  page

relocations  to verify that they are within the correct address-

ing range.

B.3  12-BIT OPCODE PIC

The 12-bit opcode family of PIC processors support the following

assembler arguments:

(*)f

(*)f,(#)d

(*)f,(#)b

(#)k

label

where:  f       register file address

d       destination select:

(0, -> w), (1 -> f)

the letters w or f may be used

to select the destination

b       bit address in an 8-bit file register

k       literal constant

label   label name

Items enclosed in () are optional.

The  terms  f, d, b, k, and label may all be expressions.

Note  that  not all addressing modes are valid with every in-

struction,  refer to the processor specific  technical data for

valid modes.

ASPIC ASSEMBLER                                         PAGE B-6

12-BIT OPCODE PIC

PIC12C5XX CPU Type

PIC12C508,      PIC12C509,      PIC12CE518

PIC12C508A,     PIC12C509A,     PIC12CE519

PIC12CR509A

B.4  14-BIT OPCODE PIC

The 14-bit opcode family of PIC processors support the following

assembler arguments:

(*)f

(*)f,(#)d

(*)f,(#)b

(#)k

label

where:  f       register file address

d       destination select:

(0, -> w), (1 -> f)

the letters w or f may be used

to select the destination

b       bit address in an 8-bit file register

k       literal constant

label   label name

Items enclosed in () are optional.

The  terms  f, d, b, k, and label may all be expressions.

Note  that  not all addressing modes are valid with every in-

struction,  refer to the processor specific  technical data for

valid modes.

PIC12C67X CPU Type

PIC12C671,      PIC12C672,      PIC12LC671,

PIC12LC672

PIC12CE673,     PIC12CE674,     PIC12LCE673,

PIC12LCE674

PIC14000 CPU Type

PIC14000

PIC16C15X CPU Type

PIC16C154,      PIC16C156,      PIC16C158

PIC16CR154,     PIC16CR156,     PIC16CR158

PIC16C5X CPU Type

ASPIC ASSEMBLER                                         PAGE B-7

14-BIT OPCODE PIC

PIC16C52

PIC16C54,       PICC16C54A,     PIC16C54B,

PIC16C54C

PIC16CR54,      PIC16CR54A,     PIC16C54B,

PIC16CR54C

PIC16C55,       PIC16C55A,      PIC16C56,

PIC16C56A

PIC16CR56A

PIC16C57,       PIC16CR57A,     PIC16C57B,

PIC16C57C

PIC16C58A,      PIC16CR58A,     PIC16C58B,

PIC16CR58B

PIC16C55X CPU Type

PIC16C554,      PIC16C556,      PIC16C558

PIC16C62X, PIC16C64X and, PIC16C66X CPU Types

PIC16C620,      PIC16C621,      PIC16C622

PIC16C642,      PIC16C662

PIC16C7XX CPU Type

PIC16C71,       PIC16C72,       PIC16CR72

PIC16C73A,      PIC16C74A,      PIC16C76,       PIC16C77

PIC16C710,      PIC16C711,      PIC16C715

PIC16C8X CPU Type

PIC16F83,       PIC16CR83,      PIC16F84,

PIC16CR84

PIC16HV540

PIC16F627,      PIC16F628

PIC16F870,      PIC16F871,      PIC16F872,

PIC16F873

PIC16F874,      PIC16F876,      PIC16F877

PIC16C9XX CPU Type

PIC16C923,      PIC16C924

B.5  16-BIT OPCODE PIC

The 16-bit opcode family of PIC processors support the following

assembler arguments:

(*)f

(*)f,(#)d

(*)f,(#)s

(*)f,(#)b

(*)f,(*)p       /       (*)p,(*)f

(#)t,(*)f

(#)t,(#)i,(*)f

{#}k

label

ASPIC ASSEMBLER                                         PAGE B-8

16-BIT OPCODE PIC

where:  f       register file address

d       destination select:

(0, -> w), (1 -> f)

the letters w or f may be used

to select the destination

s       destination select:

(0, -> f and w), (1, -> f)

the letters w or f may be used

to select the destination

t       table byte select:

(0, -> lower byte)

(1, -> upper byte)

i       table pointer control

(0, -> no change)

(1, -> post increment)

b       bit address of an 8-bit file register

p       peripheral register file address

k       literal constant

label   label name

Items enclosed in () are optional.

The  terms  f, d, s, t, i, b, p, k,  and  label  may  all be

expressions.

Note  that  not all addressing modes are valid with every in-

struction,  refer to the processor specific  technical data for

valid modes.

PIC17CXXX CPU Type

PIC17C42,       PIC17C42A,      PIC17C43,       PIC17C44

PIC17C752,      PIC17C756,      PIC17C756A

PIC17C762,      PIC17C766,      PIC17CR42,

PIC17CR43

B.6  20-BIT ADDRESSING PIC

The  20-bit  addressing  family of PIC processors support the

following assembler arguments:

(*)f(,a)

(*)f,(#)d(,(#)a)

(*)f,(#)s

(*)f,(#)b(,(#)a)

(*)fs,(*)fd

(#)t,(*)f

(#)t,(#)i,(*)f

{#}k

ASPIC ASSEMBLER                                         PAGE B-9

20-BIT ADDRESSING PIC

label(,(#)s)

((#)s)

mm

where:  f       register file address

fs      register file source

fd      register file destination

a       ram access bit

(0, -> ACCESS RAM)

(1, -> RAM BANK)

d       destination select:

(0, -> w), (1 -> f)

the letters w or f may be used

to select the destination

s       fast call/return mode:

(0, -> SLOW), (1, -> FAST)

b       bit address of an 8-bit file register

mm      TBLRD and TBLWT suffixs

('*',  -> no change)

('*+', -> post-increment)

('*-', -> post-decrement)

('+*', -> pre-increment)

k       literal constant

label   label name

Items enclosed in () are optional.

The  terms  f, fs, fd, a, b, d, s, k, and  label  may all be

expressions.

Note  that  not all addressing modes are valid with every in-

struction,  refer to the processor specific  technical data for

valid modes.

PIC18CXXX CPU Type

PIC18C242,      PIC18C252

PIC18C442,      PIC18C452

PIC18C658,      PIC18C858

ASPIC ASSEMBLER                                        PAGE B-10

PIC OPCODES

B.7  PIC OPCODES

The  following table contains all the mnemonics recognized by

the ASPIC assembler.  The processors  supporting  each  mnemonic

are  indicated by the code 'PIC:12:14:16:20' after each instruc-

tion type.  The designation [] refers to a  required  addressing

mode argument.

addwf   []              PIC:12:14:16:20

addwfc  []              PIC:--:--:16:20

andwf   []              PIC:12:14:16:20

comf    []              PIC:12:14:16:20

decf    []              PIC:12:14:16:20

decfsz  []              PIC:12:14:16:20

dcfsnz  []              PIC:--:--:16:20

incf    []              PIC:12:14:16:20

incfsz  []              PIC:12:14:16:20

infsnz  []              PIC:--:--:16:20

iorwf   []              PIC:12:14:16:20

movf    []              PIC:12:14:--:20

negw    []              PIC:--:--:16:--

rlf     []              PIC:12:14:--:--

rlcf    []              PIC:--:--:16:20

rlncf   []              PIC:--:--:16:20

rrf     []              PIC:12:14:--:--

rrcf    []              PIC:--:--:16:20

rrncf   []              PIC:--:--:16:20

subfwb  []              PIC:--:--:--:20

subwf   []              PIC:12:14:16:20

subwfb  []              PIC:--:--:16:20

swapf   []              PIC:12:14:16:20

xorwf   []              PIC:12:14:16:20

movfp   []              PIC:--:--:16:--

movpf   []              PIC:--:--:16:--

movlb   []              PIC:--:--:16:20

movlr   []              PIC:--:--:16:--

movff   []              PIC:--:--:--:20

lfsr    []              PIC:--:--:--:20

clrf    []              PIC:12:14:16:20

cpfseq  []              PIC:--:--:16:20

cpfsgt  []              PIC:--:--:16:20

cpfslt  []              PIC:--:--:16:20

movwf   []              PIC:12:14:16:20

mulwf   []              PIC:--:--:16:20

negf    []              PIC:--:--:--:20

ASPIC ASSEMBLER                                        PAGE B-11

PIC OPCODES

setf    []              PIC:--:--:16:20

tstfsz  []              PIC:--:--:16:20

bcf     []              PIC:12:14:16:20

bsf     []              PIC:12:14:16:20

btfsc   []              PIC:12:14:16:20

btfss   []              PIC:12:14:16:20

btg     []              PIC:--:--:16:20

addlw   []              PIC:--:14:16:20

andlw   []              PIC:12:14:16:20

iorlw   []              PIC:12:14:16:20

movlw   []              PIC:12:14:16:20

mullw   []              PIC:--:--:16:20

retlw   []              PIC:12:14:16:20

sublw   []              PIC:--:14:16:20

xorlw   []              PIC:12:14:16:20

call    []              PIC:12:14:16:20

goto    []              PIC:12:14:16:20

lcall   []              PIC:--:--:16:--

bc      []              PIC:--:--:--:20

bn      []              PIC:--:--:--:20

bnc     []              PIC:--:--:--:20

bnn     []              PIC:--:--:--:20

bnov    []              PIC:--:--:--:20

bnc     []              PIC:--:--:--:20

bov     []              PIC:--:--:--:20

bz      []              PIC:--:--:--:20

bra     []              PIC:--:--:--:20

rcall   []              PIC:--:--:--:20

tablrd  []              PIC:--:--:16:--

tablwt  []              PIC:--:--:16:--

tlrd    []              PIC:--:--:16:--

tlwt    []              PIC:--:--:16:--

tblrd   []              PIC:--:--:--:20

tblwt   []              PIC:--:--:--:20

clrw    []              PIC:12:14:--:--

clrwdt                  PIC:12:14:16:20

daw                     PIC:--:--:16:20

nop                     PIC:12:14:16:20

option                  PIC:12:14:--:--

pop                     PIC:--:--:--:20

push                    PIC:--:--:--:20

retfie  []              PIC:--:14:16:20

return  []              PIC:--:14:16:20

sleep                   PIC:12:14:16:20

ASPIC ASSEMBLER                                        PAGE B-12

PIC OPCODES

tris    []              PIC:12:14:--:--

APPENDIX C

ASRAB ASSEMBLER

C.1  ACKNOWLEDGMENT

Thanks to Ulrich Raich and Razaq Ijoduola for their contribution

of the ASRAB cross assembler.

Ulrich Raich and Razaq Ijoduola

PS Division

CERN

CH-1211 Geneva-23

Ulrich Raich

Ulrich dot Raich at cern dot ch

C.2  PROCESSOR SPECIFIC DIRECTIVES

The  ASRAB  assembler is a port of the ASZ80 assembler.  This

assembler can process Z80, HD64180 (Z180), and Rabbit  2000/3000

(default)  code.   The  following  processor  specific assembler

directives specify which processor to target when processing the

input assembler files.

ASRAB ASSEMBLER                                         PAGE C-2

PROCESSOR SPECIFIC DIRECTIVES

C.2.1  .r2k Directive

Format:

.r2k

The  .r2k  directive  enables processing of the Rabbit 2000/3000

specific mnemonics.  Mnemonics not associated  with  the  Rabbit

2000/3000 processor will be flagged with an 'o' error.  Address-

ing modes not supported by the Rabbit 2000/3000 will be  flagged

with  an 'a' error.  A synonym of .r2k is .r3k.  The default as-

sembler mode is .r2k.

C.2.2  .hd64 Directive

Format:

.hd64

The  .hd64  directive  enables  processing of the HD64180 (Z180)

specific mnemonics not included  in  the  Z80  instruction  set.

Rabbit  2000/3000  mnemonics encountered will be flagged with an

'o' error.  Addressing modes not supported by the HD64180 (Z180)

will be flagged with an 'a' error.  A synonym of .hd64 is .z180.

C.2.3  .z80 Directive

Format:

.z80

The  .z80  directive  enables  processing  of  the  Z80 specific

mnemonics.  HD64180 and Rabbit 2000/3000 specific mnemonics will

be flagged with an 'o' error.  Addressing modes not supported by

the z80 will be flagged with an 'a' error.

ASRAB ASSEMBLER                                         PAGE C-3

PROCESSOR SPECIFIC DIRECTIVES

C.3  RABBIT 2000/3000 ADDRESSING AND INSTRUCTIONS

C.3.1  Instruction Symbols

b       Bit select

(000 = bit 0, 001 = bit 1,

010 = bit 2, 011 = bit 3,

100 = bit 4, 101 = bit 5,

110 = bit 6, 111 = bit 7)

cc      Condition code select

(00 = NZ, 01 = Z, 10 = NC, 11 = C)

d       8-bit (signed) displacement.

Expressed in two\'s complement.

dd      word register select-destination

(00 = BC, 01 = DE, 10 = HL, 11 = SP)

dd'     word register select-alternate

(00 = BC', 01 = DE', 10 = HL')

e       8-bit (signed) displacement added to PC.

f       condition code select

(000 = NZ, 001 = Z, 010 = NC, 011 = C,

100 = LZ/NV, 101 = LO/V, 110 = P, 111 = M)

m       the most significant bits(MSB) of a 16-bit constant

mn      16-bit constant

n       8-bit constant or the least significant bits(LSB)

of a 16-bit constant

r, g    byte register select

(000 = B, 001 = C, 010 = D, 011 = E,

100 = H, 101 = L, 111 = A)

ss      word register select-source

(00 = BC, 01 = DE, 10 = HL, 11 = SP)

v       Restart address select

(010 = 0020h, 011 = 0030h, 100 = 0040h,

101 = 0050h, 111 = 0070h)

x       an 8-bit constant to load into the XPC

xx      word register select

(00 = BC, 01 = DE, 10 = IX, 11 = SP)

yy      word register select

(00 = BC, 01 = DE, 10 = IY, 11 = SP)

zz      word register select

(00 = BC, 01 = DE, 10 = HL, 11 = AF)

ASRAB ASSEMBLER                                         PAGE C-4

RABBIT 2000/3000 ADDRESSING AND INSTRUCTIONS

C  -    carry bit set

M  -    sign bit set

NC -    carry bit clear

NZ -    zero bit clear

P  -    sign bit clear

PE -    parity even

V  -    overflow bit set

PO -    parity odd

NV -    overflow bit clear

Z  -    zero bit set

The  terms  m, mn, n, and x may all be expressions.  The terms b

and v are not allowed to be external references.

ASRAB ASSEMBLER                                         PAGE C-5

RABBIT 2000/3000 ADDRESSING AND INSTRUCTIONS

C.3.2  Rabbit Instructions

The  following list of instructions (with explicit addressing

modes) are available in the  Rabbit  2000/3000  assembler  mode.

Those instructions denoted by an asterisk (*) are additional in-

structions not available in the HD64180 or Z80 assembler mode.

ADC A,n                 DEC IX                  LD A,EIR

ADC A,r                 DEC IY                  LD A,IIR

ADC A,(HL)              DEC r                  *LD A,XPC

ADC A,(IX+d)            DEC ss                  LD A,(BC)

ADC A,(IY+d)            DEC (HL)                LD A,(DE)

ADC HL,ss               DEC (IX+d)              LD A,(mn)

ADD A,n                 DEC (IY+d)             *LD dd,BC

ADD A,r                 DJNZ e                 *LD dd,DE

ADD A,(HL)                                      LD dd,mn

ADD A,(IX+d)            EX AF,AF                LD dd,(mn)

ADD A,(IY+d)            EX DE,HL                LD EIR,A

ADD HL,ss               EX DE,HL               *LD HL,IX

ADD IX,xx               EX (SP),HL             *LD HL,IY

ADD IY,yy               EX (SP),IX             *LD HL,(HL+d)

*ADD SP,d                EX (SP),IY             *LD HL,(IX+d)

*ALTD                    EXX                    *LD HL,(IY+d)

*AND HL,DE                                       LD HL,(mn)

*AND IX,DE               INC IX                 *LD HL,(SP+n)

*AND IY,DE               INC IY                  LD IIR,A

AND n                   INC r                  *LD IX,HL

AND r                   INC ss                  LD IX,mn

AND (HL)                INC (HL)                LD IX,(mn)

AND (IX+d)              INC (IX+d)             *LD IX,(SP+n)

AND (IY+d)              INC (IY+d)             *LD IY,HL

*IOE                     LD IY,mn

BIT b,r                *IOI                     LD IY,(mn)

BIT b,(HL)             *IPRES                  *LD IY,(SP+n)

BIT b,(IX+d)           *IPSET 0                 LD r,g

BIT b,(IY+d)           *IPSET 1                 LD r,n

*BOOL HL                *IPSET 2                 LD r,(HL)

*BOOL IX                *IPSET 3                 LD r,(IX+d)

*BOOL IY                                         LD r,(IY+d)

JP f,mn                 LD SP,HL

CALL mn                 JP mn                   LD SP,IX

CCF                     JP (HL)                 LD SP,IY

CP n                    JP (IX)                *LD XPC,A

CP r                    JP (IY)                 LD (BC),A

CP (HL)                 JR cc,e                 LD (DE),A

CP (IX+d)               JR e                    LD (HL),n

CP (IY+d)                                       LD (HL),r

CPL                    *LCALL x,mn

ASRAB ASSEMBLER                                         PAGE C-6

RABBIT 2000/3000 ADDRESSING AND INSTRUCTIONS

*LD (HL+d),HL           *POP IP                  SBC A,n

*LD (IX+d),HL            POP IX                  SBC A,r

LD (IX+d),n             POP IY                  SBC A,(HL)

LD (IX+d),r             POP zz                  SBC HL,ss

*LD (IY+d),HL           *PUSH IP                 SBC (IX+d)

LD (IY+d),n             PUSH IX                 SBC (IY+d)

LD (IY+d),r             PUSH IY                 SCF

LD (mn),A               PUSH zz                 SET b,r

LD (mn),HL                                      SET b,(HL)

LD (mn),IX              RA                      SET b,(IX+d)

LD (mn),IY              RES b,r                 SET b,(IY+d)

LD (mn),ss              RES b,(HL)              SLA r

*LD (SP+n),HL            RES b,(IX+d)            SLA (HL)

*LD (SP+n),IX            RES b,(IY+d)            SLA (IX+d)

*LD (SP+n),IY            RET                     SLA (IY+d)

LDD                     RET f                   SRA r

LDDR                   *RETI                    SRA (HL)

LDI                    *RL DE                   SRA (IX+d)

LDIR                    RL r                    SRA (IY+d)

*LDP HL,(HL)             RL (HL)                 SRL r

*LDP HL,(IX)             RL (IX+d)               SRL (HL)

*LDP HL,(IY)             RL (IY+d)               SRL (IX+d)

*LDP HL,(mn)             RLA                     SRL (IY+d)

*LDP IX,(mn)             RLC r                   SUB n

*LDP IY,(mn)             RLC (HL)                SUB r

*LDP (HL),HL             RLC (IX+d)              SUB (HL)

*LDP (IX),HL             RLC (IY+d)              SUB (IX+d)

*LDP (IY),HL             RLCA                    SUB (IY+d)

*LDP (mn),HL            *RR DE

*LDP (mn),IX            *RR HL                   XOR n

*LDP (mn),IY            *RR IX                   XOR r

LJP x,mn               *RR IY                   XOR (HL)

LRET                    RR r                    XOR (IX+d)

RR (HL)                 XOR (IY+d)

*MUL                     RR (IX+d)

RR (IY+d)

NEG                     RRC r

NOP                     RRC (HL)

RRC (IX+d)

*OR HL,DE                RRC (IY+d)

*OR IX,DE                RRCA

*OR IY,DE                RST v

OR n

OR r

OR (HL)

OR (IX+d)

OR (IY+d)

ASRAB ASSEMBLER                                         PAGE C-7

Z80/HD64180 ADDRESSING AND INSTRUCTIONS

C.4  Z80/HD64180 ADDRESSING AND INSTRUCTIONS

The following list specifies the format for each Z80/HD64180 ad-

dressing mode supported by ASZ80:

#data           immediate data

byte or word data

n               byte value

rg              a byte register

a,b,c,d,e,h,l

rp              a register pair

bc,de,hl

(hl)            implied addressing or

register indirect addressing

(label)         direct addressing

(ix+offset)     indexed addressing with

offset(ix)     an offset

label           call/jmp/jr label

The  terms  data,  n, label, and offset, may all be expressions.

The terms dir and offset are not allowed to be  external  refer-

ences.

The  following  tables  list all Z80/HD64180 mnemonics recog-

nized by the ASRAB assembler.  The designation []  refers  to  a

required addressing mode argument.  Note that not all addressing

modes are valid with every instruction, refer to the Z80/HD64180

technical data for valid modes.

ASRAB ASSEMBLER                                         PAGE C-8

Z80/HD64180 ADDRESSING AND INSTRUCTIONS

C.4.1  Inherent Instructions

ccf             cpd

cpdr            cpi

cpir            cpl

daa             di

ei              exx

halt            neg

nop             reti

retn            rla

rlca            rld

rra             rrca

rrd             scf

C.4.2  Implicit Operand Instructions

adc     a,[]            adc     []

add     a,[]            add     []

and     a,[]            and     []

cp      a,[]            cp      []

dec     a,[]            dec     []

inc     a,[]            inc     []

or      a,[]            or      []

rl      a,[]            rl      []

rlc     a,[]            rlc     []

rr      a,[]            rr      []

rrc     a,[]            rrc     []

sbc     a,[]            sbc     []

sla     a,[]            sla     []

sra     a,[]            sra     []

srl     a,[]            srl     []

sub     a,[]            sub     []

xor     a,[]            xor     []

ASRAB ASSEMBLER                                         PAGE C-9

Z80/HD64180 ADDRESSING AND INSTRUCTIONS

C.4.3  Load Instruction

ld      rg,[]           ld      [],rg

ld      (bc),a          ld      a,(bc)

ld      (de),a          ld      a,(de)

ld      (label),a       ld      a,(label)

ld      (label),rp      ld      rp,(label)

ld      i,a             ld      r,a

ld      a,i             ld      a,r

ld      sp,hl           ld      sp,ix

ld      sp,iy           ld      rp,#data

ldd                     lddr

ldi                     ldir

C.4.4  Call/Return Instructions

call    C,label         ret     C

call    M,label         ret     M

call    NC,label        ret     NC

call    NZ,label        ret     NZ

call    P,label         ret     P

call    PE,label        ret     PE

call    PO,label        ret     PO

call    Z,label         ret     Z

call    label           ret

C.4.5  Jump and Jump to Subroutine Instructions

jp      C,label         jp      M,label

jp      NC,label        jp      NZ,label

jp      P,label         jp      PE,label

jp      PO,label        jp      Z,label

jp      (hl)            jp      (ix)

jp      (iy)            jp      label

djnz    label

jr      C,label         jr      NC,label

jr      NZ,label        jr      Z,label

jr      label

ASRAB ASSEMBLER                                        PAGE C-10

Z80/HD64180 ADDRESSING AND INSTRUCTIONS

C.4.6  Bit Manipulation Instructions

bit     n,[]

res     n,[]

set     n,[]

C.4.7  Interrupt Mode and Reset Instructions

im      n

im      n

im      n

rst     n

C.4.8  Input and Output Instructions

in      a,(n)           in      rg,(c)

ind                     indr

ini                     inir

out     (n),a           out     (c),rg

outd                    otdr

outi                    otir

C.4.9  Register Pair Instructions

add     hl,rp           add     ix,rp

add     iy,rp

adc     hl,rp           sbc     hl,rp

ex      (sp),hl         ex      (sp),ix

ex      (sp),iy

ex      de,hl

ex      af,af'

push    rp              pop     rp

ASRAB ASSEMBLER                                        PAGE C-11

Z80/HD64180 ADDRESSING AND INSTRUCTIONS

C.4.10  HD64180 Specific Instructions

in0     rg,(n)

out0    (n),rg

otdm                    otdmr

otim                    otimr

mlt     bc              mlt     de

mlt     hl              mlt     sp

slp

tst     a

tstio   #data

APPENDIX D

ASEZ80 ASSEMBLER

D.1  ACKNOWLEDGMENT

Thanks  to Patrick Head for his contribution of the ASEZ80 cross

assembler.

Patrick Head

patrick at phead dot net

D.2  PROCESSOR SPECIFIC DIRECTIVES

The  ASEZ80 assembler is a port of the ASZ80 assembler.  This

assembler can process EZ80 code in Z80 and ADL modes in any com-

bination  within  the  source  file.   The  following  processor

specific assembler directives specify which mode  the  assembler

is to process the assembler source code.

D.2.1  .adl Directive

Format:

.adl    [n]

The  assembler is informed by the .adl directive to assemble the

subsequent code using the full 24-Bit addressing capabilities of

the EZ80 processor.  The optional argument, n, allows the assem-

bly mode to be controlled by a varaible.  A non-zero value for n

enables  ADL  mode  and  a zero value enables the Z80 mode.  The

default mode of the assembler is Z80.

ASEZ80 ASSEMBLER                                        PAGE D-2

PROCESSOR SPECIFIC DIRECTIVES

D.2.2  .z80 Directive

Format:

.z80    [n]

The  assembler is informed by the .z80 directive to assemble the

subsequent code using the banked 16-Bit addressing  capabilities

of the EZ80 processor.  The optional argument, n, allows the as-

sembly mode to be controlled by a varaible.   A  non-zero  value

for  n  enables  Z80 mode and a zero value enables the ADL mode.

The default mode of the assembler is Z80.

D.2.3  .msb Directive

Format:

.msb    n

The  assembler operator '>' selects the upper byte (MSB) when

included in an assembler instruction.  The normal assembler mode

is  to select bits <15:8> as the MSB.  The .msb directive allows

the programmer to specify a particular byte as  the  'MSB'  when

the address space is larger than 16-bits.

For a 24-bit EZ80 address the assembler directive .msb n con-

figures the assembler to select a particular byte as MSB.  Given

a  24-bit  address  of  Mmn (M is <23:16>, m is <15:8>, and n is

<7:0>) the following examples show how to  select  a  particular

address byte:

.msb 1          ;select byte 1 of address

;<M(2):m(1):n(0)>

LD A,>Mmn       ;byte m <15:8> ==>> A

...

.msb 2          ;select byte 2 of address

;<M(2):m(1):n(0)>

LD A,>Mmn       ;byte M <23:16> ==>> A

LD MB,A         ;place in MBASE register

ASEZ80 ASSEMBLER                                        PAGE D-3

PROCESSOR SPECIFIC DIRECTIVES

D.3  EZ80 ADDRESSING AND INSTRUCTIONS

D.3.1  Instruction Symbols

b          Bit select

(000 = bit 0, 001 = bit 1,

010 = bit 2, 011 = bit 3,

100 = bit 4, 101 = bit 5,

110 = bit 6, 111 = bit 7)

cc         condition code C, NC, Z, NZ, P, M, PE, PO

test of single bit in FLAGS register

cc'        condition code C, NC, Z, NZ

test of single bit in FLAGS register

d          an 8-bit two's complement displacement with

value from -128 to 127.

I          Interrupt Page Address Register

ir or ir'  8-bit CPU register IXH(IX:[15:8]),

IXL (IX:[7:0], IYH (IY:[15:8]), IYL (IY:[7:0])

IX/Y       CPU register IX or IY

(IX/Y+d)   A location in memory with address formed by the

sum of the contents of the Index Register, IX

or IY, and the two's complement displacement d.

MB         Z80 Memory Mode Base Address Register

Mmn        A 24-bit immediate data value

(Mmn)      A 24-bit value indicating a location in

memory at this address.

mn         A 16-bit immediate data value

(mn)       A 16-bit value indicating a location in

memory at this address.

n          8-bit immediate data value

R          Refresh Counter Register

r or r'    8-bit CPU register A, B, C, D, E, H, L

rr         16 or 24-bit CPU register BC, DE, HL

rxy        16 or 24-bit CPU register BC, DE, HL, IX, IY

SP         Stack Pointer, Can indicate either the

StackPointer Short register (SPS) or the

StackPointer Long register (SPL).

ASEZ80 ASSEMBLER                                        PAGE D-4

EZ80 ADDRESSING AND INSTRUCTIONS

C  -    carry bit set

NC -    carry bit clear

Z  -    zero bit set

NZ -    zero bit clear

M  -    sign bit set

P  -    sign bit clear

PE -    parity even

PO -    parity odd

The terms b, d, Mmn, mn, and n may all be expressions.

ASEZ80 ASSEMBLER                                        PAGE D-5

EZ80 ADDRESSING AND INSTRUCTIONS

D.3.2  EZ80 Instructions

The  following list of instructions (with explicit addressing

modes) are available for the EZ80.

ADC A,(HL)              DEC (HL)                INI

ADC A,ir                DEC ir                  INI2

ADC A,(IX/Y+d)          DEC IX/Y                INI2R

ADC A,n                 DEC (IX/Y+d)

ADC A,r                 DEC r                   INIM

ADC HL,rr               DEC rr                  INIMR

ADC HL,SP               DEC SP

INIR

ADD A,(HL)              DI                      INIRX

ADD A,ir

ADD A,(IX/Y+d)          DJNZ d                  JP cc,Mmn

ADD A,n                                         JP HL

ADD A,r                 EI                      JP IX/Y

ADD HL,rr                                       JP Mmn

ADD HL,SP               EX AF,AF'

ADD IX/Y,rxy            EX DE,HL                JR cc',d

ADD IX/Y,SP             EX (SP),HL              JR d

EX (SP),IX/Y

AND A,HL                                        LD A,I

AND A,ir                EXX                     LD A,(IX/Y+d)

AND A,(IX/Y+d)                                  LD A,MB

AND A,n                 HALT                    LD A,(Mmn)

AND A,r                                         LD A,R

IM n                    LD A,(rr)

BIT b,(HL)              IM A,(n)                LD (HL),IX/Y

BIT b, (IX/Y+d)         IN r,(BC)               LD (HL),n

BIT b,r                                         LD (HL),r

IN0 r,(n)               LD (HL),rr

CALL cc,Mmn                                     LD IY,(SP+n)

CALL mn                 INC (HL)                LD I,HL

INC ir                  LD I,A

CP A,(HL)               INC IX/Y                LD ir,ir'

CP A,ir                 INC (IX/Y+d)            LD ir,n

CP A,(IX/Y+d)           INC r                   LD ir,r

CP A,r                  INC SP                  LD IX/Y,(HL)

LD IX/Y,(IX/Y+d)

CPD                     IND                     LD IX/Y,Mmn

CPDR                    IND2                    LD IX/Y,(Mmn)

IND2R                   LD (IX/Y+d),IX/Y

CPI                                             LD (IX/Y+d),n

CPIR                    INDM                    LD (IX/Y+d),r

INDMR                   LD (IX/Y+d),rr

CPL                                             LD MB,A

INDR                    LD (Mmn),A

DAA                     INDRX                   LD (Mmn),IX/Y

ASEZ80 ASSEMBLER                                        PAGE D-6

EZ80 ADDRESSING AND INSTRUCTIONS

LD (Mmn),rr             OTDR                    RL r

LD (Mmn),SP             OTDRX

LD R,A                                          RLA

LD r,(HL)               OTI2R

LD r,ir                                         RLC (HL)

LD r,(IX/Y+d)           OTIM                    RLC (IX/Y+d)

LD r,n                  OTIMR                   RLC r

LD r,r'

LD rr,(HL)              OTIR                    RLCA

LD rr,(IX/Y+d)          OTIRX

LD rr,Mmn                                       RLD

LD rr,(Mmn)             OUT (BC),r

LD (rr),A               OUT (C),r               RR (HL)

LD SP,HL                OUT (n),A               RR (IX/Y+d)

LD SP,IX/Y                                      RR r

LD SP,Mmn               OUTD

LD SP,(Mmn)             OUTD2                   RRA

LDD                     OUTI                    RRC (HL)

LDDR                    OUTI2                   RRC (IX/Y+d)

RL (IX+d)               RRC r

LDI                     RL (IY+d

LDIR                                            RRCA

PEA IX+d

LEA IX/Y,IX+d           PEA IY+d                RRD

LEA IX/Y,IY+d

LEA rr,IX+d             POP AF                  RSMIX

LEA rr,IY+d             POP IX/Y

POP rr                  RST n

MLT rr

MLT SP                  PUSH AF                 SBC A,(HL)

PUSH IX/Y               SBC A,ir

NEG                     PUSH rr                 SBC A,(IX/Y+d)

SBC A,n

NOP                     RES b,(IX/Y+d)          SBC A,r

RES b,r                 SBC HL,rr

OR A,(HL)                                       SBC HL,SP

OR A,ir                 RET

OR A,(IX/Y+d)           RET cc                  SCF

OR A,n

OR A,r                  RETI                    SET b,(HL)

SET b,(IX/Y+d)

OTD2R                   RETN                    SET b,r

OTDM                    RL (HL)                 SLA (HL)

OTDMR                   RL (IX/Y+d)             SLA (IX/Y+d)

ASEZ80 ASSEMBLER                                        PAGE D-7

EZ80 ADDRESSING AND INSTRUCTIONS

SLA r                   STMIX                   TSTIO n

SLP                     SUB A,(HL)              XOR A,(HL)

SUB A,ir                XOR A,ir

SRA (HL)                SUB A,(IX/Y+d)          XOR A,(IX/Y+d)

SRA (IX/Y+d)            SUB A,n                 XOR A,n

SRA r                   SUB A,r                 XOR A,r

SRL (HL)                TST A,(HL)

SRL (IX/Y+d)            TST A,n

SRL r                   TST A,r

The  accumulator  'A' argument is optional in all of the fol-

lowing instructions:

ADC A,...               CP A,...                SUB A,...

ADD A,...               OR A,...                TST A,...

AND A,...               SBC A,...               XOR A,...

APPENDIX E

ASZ80 ASSEMBLER

E.1  .hd64 DIRECTIVE

Format:

.hd64

The  .hd64  directive enables processing of the HD64180 specific

mnemonics not included in  the  Z80  instruction  set.   HD64180

mnemonics  encountered  without  the  .hd64  directive  will  be

flagged with an 'o' error.

E.2  Z80 REGISTER SET AND CONDITIONS

The following is a complete list of register designations and

condition mnemonics:

byte registers  -       a,b,c,d,e,h,l,i,r

register pairs  -       af,af',bc,de,hl

word registers  -       pc,sp,ix,iy

C -     carry bit set

M -     sign bit set

NC -    carry bit clear

NZ -    zero bit clear

P -     sign bit clear

PE -    parity even

PO -    parity odd

Z -     zero bit set

ASZ80 ASSEMBLER                                         PAGE E-2

Z80 INSTRUCTION SET

E.3  Z80 INSTRUCTION SET

The  following  list specifies the format for each addressing

mode supported by ASZ80:

#data           immediate data

byte or word data

n               byte value

rg              a byte register

a,b,c,d,e,h,l

rp              a register pair

bc,de,hl

(hl)            implied addressing or

register indirect addressing

(label)         direct addressing

offset(ix)      indexed addressing with

an offset

label           call/jmp/jr label

The terms data, n, label, and offset may all be expressions.

Note  that  not all addressing modes are valid with every in-

struction, refer to the Z80/HD64180  technical  data  for  valid

modes.

The  following  tables  list all Z80/HD64180 mnemonics recog-

nized by the ASZ80 assembler.  The designation []  refers  to  a

required addressing mode argument.

ASZ80 ASSEMBLER                                         PAGE E-3

Z80 INSTRUCTION SET

E.3.1  Inherent Instructions

ccf             cpd

cpdr            cpi

cpir            cpl

daa             di

ei              exx

halt            neg

nop             reti

retn            rla

rlca            rld

rra             rrca

rrd             scf

E.3.2  Implicit Operand Instructions

adc     a,[]            adc     []

add     a,[]            add     []

and     a,[]            and     []

cp      a,[]            cp      []

dec     a,[]            dec     []

inc     a,[]            inc     []

or      a,[]            or      []

rl      a,[]            rl      []

rlc     a,[]            rlc     []

rr      a,[]            rr      []

rrc     a,[]            rrc     []

sbc     a,[]            sbc     []

sla     a,[]            sla     []

sra     a,[]            sra     []

srl     a,[]            srl     []

sub     a,[]            sub     []

xor     a,[]            xor     []

ASZ80 ASSEMBLER                                         PAGE E-4

Z80 INSTRUCTION SET

E.3.3  Load Instruction

ld      rg,[]           ld      [],rg

ld      (bc),a          ld      a,(bc)

ld      (de),a          ld      a,(de)

ld      (label),a       ld      a,(label)

ld      (label),rp      ld      rp,(label)

ld      i,a             ld      r,a

ld      a,i             ld      a,r

ld      sp,hl           ld      sp,ix

ld      sp,iy           ld      rp,#data

ldd                     lddr

ldi                     ldir

E.3.4  Call/Return Instructions

call    C,label         ret     C

call    M,label         ret     M

call    NC,label        ret     NC

call    NZ,label        ret     NZ

call    P,label         ret     P

call    PE,label        ret     PE

call    PO,label        ret     PO

call    Z,label         ret     Z

call    label           ret

E.3.5  Jump and Jump to Subroutine Instructions

jp      C,label         jp      M,label

jp      NC,label        jp      NZ,label

jp      P,label         jp      PE,label

jp      PO,label        jp      Z,label

jp      (hl)            jp      (ix)

jp      (iy)            jp      label

djnz    label

jr      C,label         jr      NC,label

jr      NZ,label        jr      Z,label

jr      label

ASZ80 ASSEMBLER                                         PAGE E-5

Z80 INSTRUCTION SET

E.3.6  Bit Manipulation Instructions

bit     n,[]

res     n,[]

set     n,[]

E.3.7  Interrupt Mode and Reset Instructions

im      n

im      n

im      n

rst     n

E.3.8  Input and Output Instructions

in      a,(n)           in      rg,(c)

ind                     indr

ini                     inir

out     (n),a           out     (c),rg

outd                    otdr

outi                    otir

E.3.9  Register Pair Instructions

add     hl,rp           add     ix,rp

add     iy,rp

adc     hl,rp           sbc     hl,rp

ex      (sp),hl         ex      (sp),ix

ex      (sp),iy

ex      de,hl

ex      af,af'

push    rp              pop     rp

ASZ80 ASSEMBLER                                         PAGE E-6

Z80 INSTRUCTION SET

E.3.10  HD64180 Specific Instructions

in0     rg,(n)

out0    (n),rg

otdm                    otdmr

otim                    otimr

mlt     bc              mlt     de

mlt     hl              mlt     sp

slp

tst     a

tstio   #data

APPENDIX F

ASGB ASSEMBLER

F.1  ACKNOWLEDGEMENT

Thanks  to  Roger Ivie for his contribution of the ASGB cross

assembler.

Roger Ivie

ivie at cc dot usu dot edu

F.2  INTRODUCTION

The  Gameboy uses an 8-bit processor which is closely related

to the 8080.  It is usually described as a modified Z80, but may

be more closely understood as an enhanced 8080;  it has the 8080

register set and many, but not all, enhanced  Z80  instructions.

However,  even  this is not accurate, for the Gameboy also lacks

some basic 8080 instructions (most annoyingly  SHLD  and  LHLD).

ASGB is based on ASZ80 and therefore uses the Z80 mnemonic set.

F.3  GAMEBOY REGISTER SET AND CONDITIONS

The following is a complete list of register designations and

condition mnemonics:

byte registers - a,b,c,d,e,h,l

register pairs - af, bc, de, hl

word registers - pc, sp

C  - carry bit set

NC - carry bit clear

NZ - zero bit clear

Z  - zero bit set

ASGB ASSEMBLER                                          PAGE F-2

GAMEBOY INSTRUCTION SET

F.4  GAMEBOY INSTRUCTION SET

The following tables list all Gameboy mnemnoics recognized by

the ASGB assembler.  The designation [] refers to a required ad-

dressing mode argument.  The following list specifies the format

for each addressing mode supported by ASGB:

#data           immediate data

byte or word data

n               byte value

rg              a byte register

a,b,c,d,e,h,l

rp              a register pair or 16-bit register

bc,de,hl

(hl)            implied addressing or

register indirect addressing

(label)         direct addressing

label           call/jmp/jr label

The terms data, dir, and ext may all be expression.  The term

dir is not allowed to be an external reference.

Note  that  not all addressing modes are valid with every in-

struction.  Although official information is not, as  far  as  I

know, publically available for the Gameboy processor, many unof-

ficial sources are available on the internet.

F.4.1  .tile Directive

Format: .tile /string/

where:  string  is  a  string of ascii characters taken from the

set ' ', '.', '+', '*', '0', '1', '2', and  '3'.

The   string   must   be  a  multiple  of  eight

characters long.

/  /     represent  the  delimiting  characters.   These

delimiters   may   be   any   paired    printing

characters,  as  long  as the characters are not

contained within  the  string  itself.   If  the

delimiting  characters  do  not match, the .tile

directive will give the (q) error.

ASGB ASSEMBLER                                          PAGE F-3

GAMEBOY INSTRUCTION SET

The Gameboy displays information on the screen using a pro-

grammable character set (referred to as  "tiles"  among  Gameboy

developers).   The ASGB cross assembler has a processor-specific

assembler directive  to  aid  in  the  creation  of  the  game's

character set.

Each  character is created from an 8x8 grid of pixels, each

pixel of which is composed of two bits.  The .tile directive ac-

cepts  a single string argument which is processed to create the

byte  values  corresponding  to  the  lines  of  pixels  in  the

character.   The  string  argument  must  be  some multiple of 8

characters long, and be one of these characters:

' ' or '0' - for the pixel value 00

'.' or '1' - for the pixel value 01

'+' or '2' - for the pixel value 10

'*' or '3' - for the pixel value 11

The .tile directive processes each 8-character group of its

string argument to create the two-byte  value  corresponding  to

that  line of pixels.  The example in the popular extant litera-

ture could be done using ASGB like this:

0000 7C 7C                 1         .tile " *****  "

0002 00 C6                 2         .tile "++   ++ "

0004 C6 00                 3         .tile "..   .. "

0006 00 FE                 4         .tile "+++++++ "

0008 C6 C6                 5         .tile "**   ** "

000A 00 C6                 6         .tile "++   ++ "

000C C6 00                 7         .tile "..   .. "

000E 00 00                 8         .tile "        "

Or, using the synonym character set, as:

0010 7C 7C                10         .tile "03333300"

0012 00 C6                11         .tile "22000220"

0014 C6 00                12         .tile "11000110"

0016 00 FE                13         .tile "22222220"

0018 C6 C6                14         .tile "33000330"

001A 00 C6                15         .tile "22000220"

001C C6 00                16         .tile "11000110"

001E 00 00                17         .tile "00000000"

Since .tile is perfectly willing to assemble multiple lines

of a character at once (as long as it is given complete rows  of

pixels), it could even be done as:

.tile " *****  ++   ++ ..   .. +++++++ "

.tile "**   ** ++   ++ ..   ..         "

ASGB ASSEMBLER                                          PAGE F-4

GAMEBOY INSTRUCTION SET

F.4.2  Potentially Controversial Mnemonic Selection

Although the Gameboy processor is based on the Z80, it does

include some features which are not present in the Z80.  The Z80

mnemonic  set  is  not  sufficient  to describe these additional

operations;  mnemonics must be created for the  new  operations.

The  mnemonics ASGB uses are not the same as those used by other

publically-available Gameboy assemblers.

F.4.2.1  Auto-Indexing Loads  -

The  Gameboy provides instructions to load or store the ac-

cumulator indirectly via HL and then subsequently  increment  or

decrement HL.  ASGB uses the mnemonic 'ldd' for the instructions

which decrement HL and 'ldi' for the instructions  which  incre-

ment  HL.   Because the Gameboy lacks the Z80's block moves, the

mnemonics are not otherwise needed by ASGB.

ldd a,(hl)      ldd (hl),a

ldi a,(hl)      ldi (hl),a

F.4.2.2  Input and Output Operations  -

The  Gameboy  replaces the Z80's separate address space for

I/O with a mechanism similar to the zero page addressing of pro-

cessors  such  as  the  6800  or 6502.  All I/O registers in the

Gameboy reside in the address range between 0xff00  and  0xffff.

The  Gameboy adds special instructions to load and store the ac-

cumulator from and into this page of memory.   The  instructions

are  analogous to the Z80's in and out instructions and ASGB re-

tains the 'in' and 'out' mnemonics for them.

in a,(n)        out (n),a

in a,(c)        out (c),a

From  ASGB's  perspective,  the  RAM  available from 0xff80

through 0xffff is composed of unused I/O locations  rather  than

direct-page RAM.

ASGB ASSEMBLER                                          PAGE F-5

GAMEBOY INSTRUCTION SET

F.4.2.3  The 'stop' Instruction  -

The  publically-available  documentation  for  the  Gameboy

lists the 'stop' instruction as the two-byte instruction 10  00,

and the other freely-available Gameboy assemblers assemble it in

that manner.  As far as I can tell, the only rationale for  this

is  that  the  corresponding Z80 instruction ('djnz label') is a

two-byte instruction.  ASGB assembles 'stop' as the one-byte in-

struction 10.

F.4.3  Inherent Instructions

ccf             cpl

daa             di

ei              nop

halt            rla

rlca            rra

rrca            scf

reti            stop

swap

F.4.4  Implicit Operand Instructions

adc a,[]        adc []

add a,[]        add []

and a,[]        and []

cp a,[]         cp []

dec a,[]        dec []

inc a,[]        inc []

or a,[]         or []

rl a,[]         rl []

rlc a,[]        rlc []

rr a,[]         rr []

rrc a,[]        rrc []

sbc a,[]        sbc []

sla a,[]        sla []

sra a,[]        sra []

srl a,[]        srl []

sub a,[]        sub []

xor a,[]        xor []

ASGB ASSEMBLER                                          PAGE F-6

GAMEBOY INSTRUCTION SET

F.4.5  Load Instructions

ld rg,[]        ld [],rg

ld (bc),a       ld a,(bc)

ld (de),a       ld a,(de)

ld (label),a    ld a,(label)

ld (label),sp   ld rp,#data

ld sp,hl        ld hl,sp

ldd a,(hl)      ldd (hl),a

ldi a,(hl)      ldi (hl),a

F.4.6  Call/Return Instructions

call C,label    ret C

call NC,label   ret NC

call Z,label    ret Z

call NZ,label   ret NZ

call label      ret

rst n

F.4.7  Jump Instructions

jp C,label      jp NC,label

jp Z,label      jp NZ,label

jp (hl)         jp label

jr C,label      jr NC,label

jr Z,label      jr NZ,label

jr label

F.4.8  Bit Manipulation Instructions

bit n,[]

res n,[]

set n,[]

ASGB ASSEMBLER                                          PAGE F-7

GAMEBOY INSTRUCTION SET

F.4.9  Input and Output Instructions

in a,(n)        in a,(c)

out (n),a       out (c),a

F.4.10  Register Pair Instructions

add hl,rp       add hl,sp

add sp,#data

push rp         pop rp

APPENDIX G

AS8XCXXX ASSEMBLER

G.1  ACKNOWLEDGMENTS

Thanks  to  Bill  McKinnon  for  his  contributions  to the

AS8XCXXX cross assembler.

Bill McKinnon

w_mckinnon at conknet dot com

This  assembler was derived from the AS8051 cross assembler

contributed by John Hartman.

John L. Hartman

jhartman at compuserve dot com

noice at noicedebugger dot com

G.2  DS8XCXXX ASSEMBLER DIRECTIVES

The  AS8XCXXX  assembler contains directives to specify the

particular SFR (Special Function Registers) to be enabled during

the assembly process.  The following directives are supported:

.DS8XCXXX               ;80C32 core

.DS80C310               ;Dallas Semiconductor

.DS80C320               ;Microprocessors

.DS80C323

.DS80C390

.DS83C520

.DS83C530

.DS83C550

.DS87C520

.DS87C530

.DS87C550

AS8XCXXX ASSEMBLER                                      PAGE G-2

DS8XCXXX ASSEMBLER DIRECTIVES

If  the  microprocessor  selector  directive  is  followed  by a

non-zero argument then the SFR register bit values will also  be

defined.

If  a  microprocessor  selector  directive is not specified

then no SFR or SFR register bit values will  be  defined.   This

mode allows the SFR to be defined by an external assembly source

file.  The file DS8XCXXX.SFR contains the SFR and  SFR  register

bit values for all the microprocessor selector directives.  This

file may be modified to create a new SFR for other  microproces-

sor types.

Also,  if a microprocessor selector directive is not speci-

fied then the following address length assembler directives  are

accepted:

.16bit                  ;16-Bit Addressing

.24bit                  ;24-Bit Addressing

.32bit                  ;32-Bit Addressing

These  directives specify the assembler addressing space and ef-

fect the output format for the .lst, .sym, and .rel files.

The default addressing space for defined microprocessors is

16-Bit except for the DS80C390 microprocessor which is 24-Bit.

G.2.1  DS80C390 Addressing Mode Directive

The  DS80C390 microprocessor supports 16-Bit and 24-Bit ad-

dressing modes.   The  .amode  assembler  directive  provides  a

method  to  select  the addressing mode used by the ajmp, acall,

ljmp, and lcall instructions.  These four  instructions  support

16  and  24 bit addressing modes selected by bits AM0 and AM1 in

the ACON register.  The assembler is 'informed'  about  the  ad-

dressing mode selected by using the .amode directive:

.amode  2       ;mode 2 is 24-bit addressing

If  a  second  argument  is specified and its value is non-zero,

then a three instruction sequence is inserted at the .amode  lo-

cation loading the mode bits into the ACON register:

.amode  2,1     ;mode 2 is 24-bit addressing, load ACON

;mov    ta,#0xAA

;mov    ta,#0x55

;mov    acon,#amode

AS8XCXXX ASSEMBLER                                      PAGE G-3

DS8XCXXX ASSEMBLER DIRECTIVES

G.2.2  The .msb Directive

The .msb directive is available in the ASEZ80 assembler.

The  assembler  operator  '>'  selects the upper byte (MSB)

when included in an assembler instruction.  The  default  assem-

bler  mode is to select bits <15:8> as the MSB.  The .msb direc-

tive allows the programmer to specify a particular byte  as  the

'MSB' when the address space is larger than 16-bits.

The  assembler directive   .msb n  configures the assembler

to select a particular byte as MSB.  Given a 24-bit  address  of

Mmn  (N(2)  is  <23:16>,  m(1) is <15:8>, and n(0) is <7:0>) the

following examples show how to select a particular address byte:

.msb 1          ;select byte 1 of address

;<M(3):N(2):m(1):n(0)>

LD A,>MNmn      ;byte m <15:8> ==>> A

...

.msb 2          ;select byte 2 of address

;<M(3):N(2):m(1):n(0)>

LD A,>MNmn      ;byte N <23:16> ==>> A

...

AS8XCXXX ASSEMBLER                                      PAGE G-4

DS8XCXXX ASSEMBLER DIRECTIVES

G.3  DS8XCXXX REGISTER SET

The  AS8XCXXX  cross assembler supports the Dallas Semiconductor

DS8XCXXX series of 8051-compatible devices.  These  microproces-

sors  retain  instruction set and object code compatability with

the 8051 microprocessor.  The DS8XCXXX family  is  updated  with

several   new  peripherals  while  providing  all  the  standard

features of the 80C32 microprocessor.

The following is a list of the registers used by AS8XCXXX:

a,b             -       8-bit accumulators

r0,r1,r2,r3     -       8-bit registers

r4,r5,r6,r7

dptr            -       data pointer

sp              -       stack pointer

pc              -       program counter

psw             -       status word

c               -       carry (bit in status word)

G.4  DS8XCXXX INSTRUCTION SET

The following tables list all DS8XCXXX mnemonics recognized

by the AS8XCXXX assembler.  The  following  list  specifies  the

format for each addressing mode supported by AS8XCXXX:

#data           immediate data

byte or word data

r,r1,r2         register r0,r1,r2,r3,r4,r5,r6, or r7

@r              indirect on register r0 or r1

@dptr           indirect on data pointer

@a+dptr         indirect on accumulator

plus data pointer

@a+pc           indirect on accumulator

plus program counter

addr            direct memory address

bitaddr         bit address

label           call or jump label

The terms data, addr, bitaddr, and label may all be expressions.

Note that not all addressing modes are valid with every in-

struction.  Refer to  the  DS8XCXXX  technical  data  for  valid

modes.

AS8XCXXX ASSEMBLER                                      PAGE G-5

DS8XCXXX INSTRUCTION SET

G.4.1  Inherent Instructions

nop

G.4.2  Move Instructions

mov     a,#data         mov     a,addr

mov     a,r             mov     a,@r

mov     r,#data         mov     r,addr

mov     r,a

mov     addr,a          mov     addr,#data

mov     addr,r          mov     addr,@r

mov     addr1,addr2     mov     bitaddr,c

mov     @r,#data        mov     @r,addr

mov     @r,a

mov     c,bitaddr

mov     dptr,#data

movc    a,@a+dptr       movc    a,@a+pc

movx    a,@dptr         movx    a,@r

movx    @dptr,a         movx    @r,a

G.4.3  Single Operand Instructions

clr     a               clr     c

clr     bitaddr

cpl     a               cpl     c

cpl     bitaddr

setb    c               setb    bitaddr

da      a

rr      a               rrc     a

rl      a               rlc     a

swap    a

dec     a               dec     r

dec     @r

inc     a               inc     r

inc     dptr            inc     @r

div     ab              mul     ab

pop     addr            push    addr

AS8XCXXX ASSEMBLER                                      PAGE G-6

DS8XCXXX INSTRUCTION SET

G.4.4  Two Operand Instructions

add     a,#data         add     a,addr

add     a,r             add     a,@r

addc    a,#data         addc    a,addr

addc    a,r             addc    a,@r

subb    a,#data         subb    a,addr

subb    a,r             subb    a,@r

orl     a,#data         orl     a,addr

orl     a,r             orl     a,@r

orl     addr,a          orl     addr,#data

orl     c,bitaddr       orl     c,/bitaddr

anl     a,#data         anl     a,addr

anl     a,r             anl     a,@r

anl     addr,a          anl     addr,#data

anl     c,bitaddr       anl     c,/bitaddr

xrl     a,#data         xrl     a,addr

xrl     a,r             xrl     a,@r

xrl     addr,a          xrl     addr,#data

xrl     c,bitaddr       xrl     c,/bitaddr

xch     a,addr          xch     a,r

xch     a,@r            xchd    a,@r

G.4.5  Call and Return Instructions

acall   label           lcall   label

ret                     reti

in      data

out     data

rst     data

G.4.6  Jump Instructions

ajmp    label

cjne    a,#data,label   cjne    a,addr,label

cjne    r,#data,label   cjne    @r,#data,label

djnz    r,label         djnz    addr,label

jbc     bitadr,label

jb      bitadr,label    jnb     bitadr,label

jc      label           jnc     label

jz      label           jnz     label

jmp     @a+dptr

ljmp    label           sjmp    label

AS8XCXXX ASSEMBLER                                      PAGE G-7

DS8XCXXX INSTRUCTION SET

G.5  DS8XCXXX SPECIAL FUNCTION REGISTERS

The  80C32 core Special Function Registers are selected us-

ing the .DS8xCxxx assembler directive.

G.5.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80              SP      DPL     DPH         83

84                              PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1                         8F

90      P1                                  93

94                                          97

98      SCON    SBUF                        9B

9C                                          9F

A0      P2                                  A3

A4                                          A7

A8      IE      SADDR0                      AB

AC                                          AF

B0      P3                                  B3

B4                                          B7

B8      IP      SADEN0                      BB

BC                                          BF

C0                                          C3

C4              STATUS                      C7

C8      T2CON   T2MOD   RCAP2L  RCAP2H      CB

CC      TL2     TH2                         CF

D0      PSW                                 D3

D4                                          D7

D8                                          DB

DC                                          DF

E0      ACC                                 E3

E4                                          E7

E8                                          EB

EC                                          EF

F0      B                                   F3

F4                                          F7

F8                                          FB

FC                                          FF

AS8XCXXX ASSEMBLER                                      PAGE G-8

DS8XCXXX SPECIAL FUNCTION REGISTERS

G.5.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

P1      90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

P2      A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

P3      B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

C0                                          C3

C4                                          C7

T2CON   C8      T2CON.0 T2CON.1 T2CON.2 T2CON.3     CB

CC      T2CON.4 T2CON.5 T2CON.6 T2CON.7     CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

D8                                          DB

DC                                          DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

E8                                          EB

EC                                          EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

F8                                          FB

FC                                          FF

AS8XCXXX ASSEMBLER                                      PAGE G-9

DS8XCXXX SPECIAL FUNCTION REGISTERS

G.5.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

90                                          93

94                                          97

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2             EA          AF

B0                                          B3

B4                                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PT2                         BF

C0                                          C3

C4                                          C7

T2CON   C8      CPRL2   CT2     TR2     EXEN2       CB

CC      TCLK    RCLK    EXF2    TF2         CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

D8                                          DB

DC                                          DF

E0                                          E3

E4                                          E7

E8                                          EB

EC                                          EF

F0                                          F3

F4                                          F7

F8                                          FB

FC                                          FF

Alternates:

SCON    98                                          9B

9C                              FE          9F

T2CON   C8      CP_RL2  C_T2                        CB

CC                                          CF

AS8XCXXX ASSEMBLER                                     PAGE G-10

DS8XCXXX SPECIAL FUNCTION REGISTERS

G.5.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

PCON    0x80    SMOD    SMOD0                       0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

STATUS  0x80            HIP     LIP                 0x10

0x08                                        0x01

T2MOD   0x80                                        0x10

0x08                    T2OE    DCEN        0x01

AS8XCXXX ASSEMBLER                                     PAGE G-11

DS8XCXXX SPECIAL FUNCTION REGISTERS

G.6  DS80C310 SPECIAL FUNCTION REGISTERS

The  DS80C310 Special Function Registers are selected using

the .DS80C310 assembler directive.

G.6.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80              SP      DPL     DPH         83

84      DPL1    DPH1    DPS     PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1     CKCON               8F

90      P1      EXIF                        93

94                                          97

98      SCON    SBUF                        9B

9C                                          9F

A0      P2                                  A3

A4                                          A7

A8      IE      SADDR0                      AB

AC                                          AF

B0      P3                                  B3

B4                                          B7

B8      IP      SADEN0                      BB

BC                                          BF

C0                                          C3

C4              STATUS                      C7

C8      T2CON   T2MOD   RCAP2L  RCAP2H      CB

CC      TL2     TH2                         CF

D0      PSW                                 D3

D4                                          D7

D8      WDCON                               DB

DC                                          DF

E0      ACC                                 E3

E4                                          E7

E8      EIE                                 EB

EC                                          EF

F0      B                                   F3

F4                                          F7

F8      EIP                                 FB

FC                                          FF

AS8XCXXX ASSEMBLER                                     PAGE G-12

DS80C310 SPECIAL FUNCTION REGISTERS

G.6.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

P1      90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

P2      A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

P3      B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

C0                                          C3

C4                                          C7

T2CON   C8      T2CON.0 T2CON.1 T2CON.2 T2CON.3     CB

CC      T2CON.4 T2CON.5 T2CON.6 T2CON.7     CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

WDCON   D8      WDCON.0 WDCON.1 WDCON.2 WDCON.3     DB

DC      WDCON.4 WDCON.5 WDCON.6 WDCON.7     DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

EIE     E8      EIE.0   EIE.1   EIE.2   EIE.3       EB

EC      EIE.4   EIE.5   EIE.6   EIE.7       EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

EIP     F8      EIP.0   EIP.1   EIP.2   EIP.3       FB

FC      EIP.4   EIP.5   EIP.6   EIP.7       FF

AS8XCXXX ASSEMBLER                                     PAGE G-13

DS80C310 SPECIAL FUNCTION REGISTERS

G.6.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

90                                          93

94                                          97

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2             EA          AF

B0                                          B3

B4                                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PT2                         BF

C0                                          C3

C4                                          C7

T2CON   C8      CPRL2   CT2     TR2     EXEN2       CB

CC      TCLK    RCLK    EXF2    TF2         CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

WDCON   D8                                          DB

DC                      POR                 DF

E0                                          E3

E4                                          E7

EIE     E8      EX2     EX3     EX4     EX5         EB

EC                                          EF

F0                                          F3

F4                                          F7

EIP     F8      PX2     PX3     PX4     PX5         FB

FC                                          FF

Alternates:

SCON    98                                          9B

9C                              FE          9F

T2CON   C8      CP_RL2  C_T2                        CB

CC                                          CF

AS8XCXXX ASSEMBLER                                     PAGE G-14

DS80C310 SPECIAL FUNCTION REGISTERS

G.6.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

DPS     0x80                                        0x10

0x08                            SEL         0x01

PCON    0x80    SMOD    SMOD0                       0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

CKCON   0x80                    T2M     T1M         0x10

0x08    T0M     MD2     MD1     MD0         0x01

EXIF    0x80    IE5     IE4     IE3     IE2         0x10

0x08                                        0x01

STATUS  0x80            HIP     LIP                 0x10

0x08                                        0x01

T2MOD   0x80                                        0x10

0x08                    T2OE    DCEN        0x01

Alternates:

PCON    0x80    SMOD_0                              0x10

0x08                                        0x01

AS8XCXXX ASSEMBLER                                     PAGE G-15

DS80C310 SPECIAL FUNCTION REGISTERS

G.7  DS80C320/DS80C323 SPECIAL FUNCTION REGISTERS

The   DS80C320/DS80C323   Special  Function  Registers  are

selected using the .DS80C320 or DS80C323 assembler directives.

G.7.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80              SP      DPL     DPH         83

84      DPL1    DPH1    DPS     PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1     CKCON               8F

90      P1      EXIF                        93

94                                          97

98      SCON0   SBUF0                       9B

9C                                          9F

A0      P2                                  A3

A4                                          A7

A8      IE      SADDR0                      AB

AC                                          AF

B0      P3                                  B3

B4                                          B7

B8      IP      SADEN0                      BB

BC                                          BF

C0      SCON1   SBUF1                       C3

C4              STATUS          TA          C7

C8      T2CON   T2MOD   RCAP2L  RCAP2H      CB

CC      TL2     TH2                         CF

D0      PSW                                 D3

D4                                          D7

D8      WDCON                               DB

DC                                          DF

E0      ACC                                 E3

E4                                          E7

E8      EIE                                 EB

EC                                          EF

F0      B                                   F3

F4                                          F7

F8      EIP                                 FB

FC                                          FF

Alternates:

98      SCON    SBUF                        9B

AS8XCXXX ASSEMBLER                                     PAGE G-16

DS80C320/DS80C323 SPECIAL FUNCTION REGISTERS

G.7.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

P1      90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON0   98      SCON0.0 SCON0.1 SCON0.2 SCON0.3     9B

9C      SCON0.4 SCON0.5 SCON0.6 SCON0.7     9F

P2      A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

P3      B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

SCON1   C0      SCON1.0 SCON1.1 SCON1.2 SCON1.3     C3

C4      SCON1.4 SCON1.5 SCON1.6 SCON1.7     C7

T2CON   C8      T2CON.0 T2CON.1 T2CON.2 T2CON.3     CB

CC      T2CON.4 T2CON.5 T2CON.6 T2CON.7     CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

WDCON   D8      WDCON.0 WDCON.1 WDCON.2 WDCON.3     DB

DC      WDCON.4 WDCON.5 WDCON.6 WDCON.7     DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

EIE     E8      EIE.0   EIE.1   EIE.2   EIE.3       EB

EC      EIE.4   EIE.5   EIE.6   EIE.7       EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

EIP     F8      EIP.0   EIP.1   EIP.2   EIP.3       FB

FC      EIP.4   EIP.5   EIP.6   EIP.7       FF

Alternates:

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

AS8XCXXX ASSEMBLER                                     PAGE G-17

DS80C320/DS80C323 SPECIAL FUNCTION REGISTERS

G.7.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

90                                          93

94                                          97

SCON0   98      RI_0    TI_0    RB8_0   TB8_0       9B

9C      REN_0   SM2_0   SM1_0   SMO_0       9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2             EA          AF

B0                                          B3

B4                                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PT2                         BF

SCON1   C0      RI_1    TI_1    RB8_1   TB8_1       C3

C4      REN_1   SM2_1   SM1_1   SMO_1       C7

T2CON   C8      CPRL2   CT2     TR2     EXEN2       CB

CC      TCLK    RCLK    EXF2    TF2         CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

WDCON   D8      RWT     EWT     WTRF    WDIF        DB

DC      PFI     EPFI    POR     SMOD_1      DF

E0                                          E3

E4                                          E7

EIE     E8      EX2     EX3     EX4     EX5         EB

EC      EWDI                                EF

F0                                          F3

F4                                          F7

EIP     F8      PX2     PX3     PX4     PX5         FB

FC      PWDI                                FF

Alternates:

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

SCON    98                                          9B

9C                              FE          9F

SCON0   98                                          9B

9C                              FE_0        9F

SCON1   C0                                          C3

C4                              FE_1        C7

T2CON   C8      CP_RL2  C_T2                        CB

CC                                          CF

AS8XCXXX ASSEMBLER                                     PAGE G-18

DS80C320/DS80C323 SPECIAL FUNCTION REGISTERS

G.7.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

DPS     0x80                                        0x10

0x08                            SEL         0x01

PCON    0x80    SMOD_0  SMOD0                       0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

CKCON   0x80    WD1     WD0     T2M     T1M         0x10

0x08    T0M     MD2     MD1     MD0         0x01

EXIF    0x80    IE5     IE4     IE3     IE2         0x10

0x08            RGMD    RGSL    BGS         0x01

STATUS  0x80    PIP     HIP     LIP                 0x10

0x08                                        0x01

T2MOD   0x80                                        0x10

0x08                    T2OE    DCEN        0x01

Alternates:

PCON    0x80    SMOD                                0x10

0x08                                        0x01

AS8XCXXX ASSEMBLER                                     PAGE G-19

DS80C320/DS80C323 SPECIAL FUNCTION REGISTERS

G.8  DS80C390 SPECIAL FUNCTION REGISTERS

The  DS80C390 Special Function Registers are selected using

the .DS80C390 assembler directive.

G.8.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80      P4      SP      DPL     DPH         83

84      DPL1    DPH1    DPS     PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1     CKCON               8F

90      P1      EXIF    P4CNT   DPX         93

94              DPX1    C0RMS0  C0RMS1      97

98      SCON0   SBUF0           ESP         9B

9C      AP      ACON    C0TMA0  C0TMA1      9F

A0      P2      P5      P5CNT   C0C         A3

A4      C0S     C0IR    C0TE    C0RE        A7

A8      IE      SADDR0  SADDR1  C0M1C       AB

AC      C0M2C   C0M3C   C0M4C   C0M5C       AF

B0      P3                      C0M6C       B3

B4      C0M7C   C0M8C   C0M9C   C0M10C      B7

B8      IP      SADEN0  SADEN1  C0M11C      BB

BC      C0M12C  C0M13C  C0M14C  C0M15C      BF

C0      SCON1   SBUF1                       C3

C4      PMR     STATUS  MCON    TA          C7

C8      T2CON   T2MOD   RCAP2L  RCAP2H      CB

CC      TL2     TH2     COR                 CF

D0      PSW     MCNT0   MCNT1   MA          D3

D4      MB      MC      C1RMS0  C1RMS1      D7

D8      WDCON                               DB

DC                      C1TMA0  C1TMA1      DF

E0      ACC                     C1C         E3

E4      C1S     C1IR    C1TE    C1RE        E7

E8      EIE             MXAX    C1M1C       EB

EC      C1M2C   C1M3C   C1M4C   C1M5C       EF

F0      B                       C1M6C       F3

F4      C1M7C   C1M8C   C1M9C   C1M10C      F7

F8      EIP                     C1M11C      FB

FC      C1M12C  C1M13C  C1M14C  C1M15C      FF

Alternates:

98      SCON    SBUF                        9B

AS8XCXXX ASSEMBLER                                     PAGE G-20

DS80C390 SPECIAL FUNCTION REGISTERS

G.8.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

P4      80      P4.0    P4.1    P4.2    P4.3        83

84      P4.4    P4.5    P4.6    P4.7        87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

P1      90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON0   98      SCON0.0 SCON0.1 SCON0.2 SCON0.3     9B

9C      SCON0.4 SCON0.5 SCON0.6 SCON0.7     9F

P2      A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

P3      B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

SCON1   C0      SCON1.0 SCON1.1 SCON1.2 SCON1.3     C3

C4      SCON1.4 SCON1.5 SCON1.6 SCON1.7     C7

T2CON   C8      T2CON.0 T2CON.1 T2CON.2 T2CON.3     CB

CC      T2CON.4 T2CON.5 T2CON.6 T2CON.7     CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

WDCON   D8      WDCON.0 WDCON.1 WDCON.2 WDCON.3     DB

DC      WDCON.4 WDCON.5 WDCON.6 WDCON.7     DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

EIE     E8      EIE.0   EIE.1   EIE.2   EIE.3       EB

EC      EIE.4   EIE.5   EIE.6   EIE.7       EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

EIP     F8      EIP.0   EIP.1   EIP.2   EIP.3       FB

FC      EIP.4   EIP.5   EIP.6   EIP.7       FF

Alternates:

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

AS8XCXXX ASSEMBLER                                     PAGE G-21

DS80C390 SPECIAL FUNCTION REGISTERS

G.8.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

P1      90      T2      T2EX    RXD1    TXD1        93

94      INT2    INT3    INT4    INT5        97

SCON0   98      RI_0    TI_0    RB8_0   TB8_0       9B

9C      REN_0   SM2_0   SM1_0   SMO_0       9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2     ES1     EA          AF

P3      B0      RXD0    TXD0    INT0    INT1        B3

B4      T0      T1                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PT2     PS1                 BF

SCON1   C0      RI_1    TI_1    RB8_1   TB8_1       C3

C4      REN_1   SM2_1   SM1_1   SMO_1       C7

T2CON   C8      CPRL2   CT2     TR2     EXEN2       CB

CC      TCLK    RCLK    EXF2    TF2         CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

WDCON   D8      RWT     EWT     WTRF    WDIF        DB

DC      PFI     EPFI    POR     SMOD_1      DF

E0                                          E3

E4                                          E7

EIE     E8      EX2     EX3     EX4     EX5         EB

EC      EWDI    C1IE    C0IE    CANBIE      EF

F0                                          F3

F4                                          F7

EIP     F8      PX2     PX3     PX4     PX5         FB

FC      PWDI    C1IP    C0IP    CANBIP      FF

Alternates:

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

SCON    98                                          9B

9C                              FE          9F

SCON0   98                                          9B

9C                              FE_0        9F

SCON1   C0                                          C3

C4                              FE_1        C7

T2CON   C8      CP_RL2  C_T2                        CB

CC                                          CF

AS8XCXXX ASSEMBLER                                     PAGE G-22

DS80C390 SPECIAL FUNCTION REGISTERS

G.8.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

DPS     0x80    ID1     ID0     TSL                 0x10

0x08                            SEL         0x01

PCON    0x80    SMOD_0  SMOD0   OFDF    OFDE        0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

CKCON   0x80    WD1     WD0     T2M     T1M         0x10

0x08    T0M     MD2     MD1     MD0         0x01

EXIF    0x80    IE5     IE4     IE3     IE2         0x10

0x08    CKRY    RGMD    RGSL    BGS         0x01

P4CNT   0x80            SBCAN                       0x10

0x08                                        0x01

ESP     0x80                                        0x10

0x08                    ESP.1   ESP.0       0x01

ACON    0x80                                        0x10

0x08            SA      AM1     AM0         0x01

P5      0x80    P5.7    P5.6    P5.5    P5.4        0x10

0x08    P5.3    P5.2    P5.1    P5.0        0x01

P5CNT   0x80    CAN1BA  CAN0BA  SP1EC   C1_IO       0x10

0x08    C0_IO   P5CNT.2 P5CNT.1 P5CNT.0     0x01

CxC     0x80    ERIE    STIE    PDE     SIESTA      0x10

0x08    CRST    AUTOB   ERCS    SWINT       0x01

CxS     0x80    BSS   EC96_128  WKS     RXS         0x10

0x08    TXS     ER2     ER1     ER0         0x01

CxIR    0x80    INTIN7  INTIN6  INTIN5  INTIN4      0x10

0x08    INTIN3  INTIN2  INTIN1  INTIN0      0x01

CxCxxC  0x80    MSRDY   ET1     ER1     INTRQ       0x10

0x08    EXTRQ   MTRQ    ROW_TIH DTUP        0x01

PMR     0x80    CD1     CD0     SWB     CTM         0x10

0x08    4X_2X   ALEOFF                      0x01

STATUS  0x80    PIP     HIP     LIP                 0x10

0x08    SPTA1   SPRA1   SPTA0   SPRA0       0x01

MCON    0x80    IDM1    IDM0    CMA                 0x10

0x08    PDCE3   PDCE2   PDCE1   PDCE0       0x01

T2MOD   0x80                            D13T1       0x10

0x08    D13T2           T2OE    DCEN        0x01

COR     0x80    IRDACK  C1BPR7  C1BPR6  C0BPR7      0x10

0x08    C0BPR6  COD1    COD0    CLKOE       0x01

MCNT0   0x80    _LSHIFT CSE     SCB     MAS4        0x10

0x08    MAS3    MAS2    MAS1    MAS0        0x01

MCNT1   0x80    MST     MOF             CLM         0x10

0x08                                        0x01

Alternates:

AS8XCXXX ASSEMBLER                                     PAGE G-23

DS80C390 SPECIAL FUNCTION REGISTERS

PCON    0x80    SMOD                                0x10

0x08                                        0x01

AS8XCXXX ASSEMBLER                                     PAGE G-24

DS80C390 SPECIAL FUNCTION REGISTERS

G.9  DS83C520/DS87C520 SPECIAL FUNCTION REGISTERS

The   DS83C520/DS87C520   Special  Function  Registers  are

selected using the .DS83C520 or DS87C520 assembler directives.

G.9.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80      P0      SP      DPL     DPH         83

84      DPL1    DPH1    DPS     PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1     CKCON               8F

90      PORT1   EXIF                        93

94                                          97

98      SCON0   SBUF0                       9B

9C                                          9F

A0      P2                                  A3

A4                                          A7

A8      IE      SADDR0  SADDR1              AB

AC                                          AF

B0      P3                                  B3

B4                                          B7

B8      IP      SADEN0  SADEN1              BB

BC                                          BF

C0      SCON1   SBUF1   ROMSIZE             C3

C4      PMR     STATUS          TA          C7

C8      T2CON   T2MOD   RCAP2L  RCAP2H      CB

CC      TL2     TH2                         CF

D0      PSW                                 D3

D4                                          D7

D8      WDCON                               DB

DC                                          DF

E0      ACC                                 E3

E4                                          E7

E8      EIE                                 EB

EC                                          EF

F0      B                                   F3

F4                                          F7

F8      EIP                                 FB

FC                                          FF

Alternates:

98      SCON    SBUF                        9B

AS8XCXXX ASSEMBLER                                     PAGE G-25

DS83C520/DS87C520 SPECIAL FUNCTION REGISTERS

G.9.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

P0      80      P0.7    P0.6    P0.5    P0.4        83

84      P0.3    P0.2    P0.1    P0.0        87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

PORT1   90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON0   98      SCON0.0 SCON0.1 SCON0.2 SCON0.3     9B

9C      SCON0.4 SCON0.5 SCON0.6 SCON0.7     9F

P2      A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

P3      B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

SCON1   C0      SCON1.0 SCON1.1 SCON1.2 SCON1.3     C3

C4      SCON1.4 SCON1.5 SCON1.6 SCON1.7     C7

T2CON   C8      T2CON.0 T2CON.1 T2CON.2 T2CON.3     CB

CC      T2CON.4 T2CON.5 T2CON.6 T2CON.7     CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

WDCON   D8      WDCON.0 WDCON.1 WDCON.2 WDCON.3     DB

DC      WDCON.4 WDCON.5 WDCON.6 WDCON.7     DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

EIE     E8      EIE.0   EIE.1   EIE.2   EIE.3       EB

EC      EIE.4   EIE.5   EIE.6   EIE.7       EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

EIP     F8      EIP.0   EIP.1   EIP.2   EIP.3       FB

FC      EIP.4   EIP.5   EIP.6   EIP.7       FF

Alternates:

PORT1   90      PORT1.0 PORT1.1 PORT1.2 PORT1.3     93

94      PORT1.4 PORT1.5 PORT1.6 PORT1.7     97

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

AS8XCXXX ASSEMBLER                                     PAGE G-26

DS83C520/DS87C520 SPECIAL FUNCTION REGISTERS

G.9.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

90                                          93

94                                          97

SCON0   98      RI_0    TI_0    RB8_0   TB8_0       9B

9C      REN_0   SM2_0   SM1_0   SMO_0       9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2     ES1     EA          AF

B0                                          B3

B4                                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PT2     PS1                 BF

SCON1   C0      RI_1    TI_1    RB8_1   TB8_1       C3

C4      REN_1   SM2_1   SM1_1   SMO_1       C7

T2CON   C8      CPRL2   CT2     TR2     EXEN2       CB

CC      TCLK    RCLK    EXF2    TF2         CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

WDCON   D8      RWT     EWT     WTRF    WDIF        DB

DC      PFI     EPFI    POR     SMOD_1      DF

E0                                          E3

E4                                          E7

EIE     E8      EX2     EX3     EX4     EX5         EB

EC      EWDI                                EF

F0                                          F3

F4                                          F7

EIP     F8      PX2     PX3     PX4     PX5         FB

FC      PWDI                                FF

Alternates:

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

SCON    98                                          9B

9C                              FE          9F

SCON0   98                                          9B

9C                              FE_0        9F

SCON1   C0                                          C3

C4                              FE_1        C7

T2CON   C8      CP_RL2  C_T2                        CB

CC                                          CF

AS8XCXXX ASSEMBLER                                     PAGE G-27

DS83C520/DS87C520 SPECIAL FUNCTION REGISTERS

G.9.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

DPS     0x80                                        0x10

0x08                            SEL         0x01

PCON    0x80    SMOD_0  SMOD0                       0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

CKCON   0x80    WD1     WD0     T2M     T1M         0x10

0x08    T0M     MD2     MD1     MD0         0x01

EXIF    0x80    IE5     IE4     IE3     IE          0x10

0x08    XT_RG   RGMD    RGSL    BGS         0x01

SBUF1   0x80    SB7     SB6     SB5     SB4         0x10

0x08    SB3     SB2     SB1     SB0         0x01

ROMSIZE 0x80                                        0x10

0x08            RMS2    RMS1    RMS0        0x01

PMR     0x80    CD1     CD0     SWB                 0x10

0x08    XTOFF   ALEOFF  DME1    DME0        0x01

STATUS  0x80    PIP     HIP     LIP     XTUP        0x10

0x08    SPTA1   SPRA1   SPTA0   SPRA0       0x01

T2MOD   0x80                                        0x10

0x08                    T2OE    DCEN        0x01

Alternates:

PCON    0x80    SMOD                                0x10

0x08                                        0x01

AS8XCXXX ASSEMBLER                                     PAGE G-28

DS83C520/DS87C520 SPECIAL FUNCTION REGISTERS

G.10  DS83C530/DS87C530 SPECIAL FUNCTION REGISTERS

The   DS83C530/DS87C530   Special  Function  Registers  are

selected using the .DS83C530 or DS87C530 assembler directives.

G.10.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80      P0      SP      DPL     DPH         83

84      DPL1    DPH1    DPS     PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1     CKCON               8F

90      P1      EXIF                        93

94                      TRIM                97

98      SCON0   SBUF0                       9B

9C                                          9F

A0      P2                                  A3

A4                                          A7

A8      IE      SADDR0  SADDR1              AB

AC                                          AF

B0      P3                                  B3

B4                                          B7

B8      IP      SADEN0  SADEN1              BB

BC                                          BF

C0      SCON1   SBUF1   ROMSIZE             C3

C4      PMR     STATUS          TA          C7

C8      T2CON   T2MOD   RCAP2L  RCAP2H      CB

CC      TL2     TH2                         CF

D0      PSW                                 D3

D4                                          D7

D8      WDCON                               DB

DC                                          DF

E0      ACC                                 E3

E4                                          E7

E8      EIE                                 EB

EC                                          EF

F0      B               RTASS   RTAS        F3

F4      RTAM    RTAH                        F7

F8      EIP     RTCC    RTCSS   RTCS        FB

FC      RTCM    RTCH    RTCD0   RTCD1       FF

Alternates:

98      SCON    SBUF                        9B

AS8XCXXX ASSEMBLER                                     PAGE G-29

DS83C530/DS87C530 SPECIAL FUNCTION REGISTERS

G.10.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

P0      80      P0.7    P0.6    P0.5    P0.4        83

84      P0.3    P0.2    P0.1    P0.0        87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

P1      90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON0   98      SCON0.0 SCON0.1 SCON0.2 SCON0.3     9B

9C      SCON0.4 SCON0.5 SCON0.6 SCON0.7     9F

P2      A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

P3      B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

SCON1   C0      SCON1.0 SCON1.1 SCON1.2 SCON1.3     C3

C4      SCON1.4 SCON1.5 SCON1.6 SCON1.7     C7

T2CON   C8      T2CON.0 T2CON.1 T2CON.2 T2CON.3     CB

CC      T2CON.4 T2CON.5 T2CON.6 T2CON.7     CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

WDCON   D8      WDCON.0 WDCON.1 WDCON.2 WDCON.3     DB

DC      WDCON.4 WDCON.5 WDCON.6 WDCON.7     DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

EIE     E8      EIE.0   EIE.1   EIE.2   EIE.3       EB

EC      EIE.4   EIE.5   EIE.6   EIE.7       EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

EIP     F8      EIP.0   EIP.1   EIP.2   EIP.3       FB

FC      EIP.4   EIP.5   EIP.6   EIP.7       FF

Alternates:

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

AS8XCXXX ASSEMBLER                                     PAGE G-30

DS83C530/DS87C530 SPECIAL FUNCTION REGISTERS

G.10.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

90                                          93

94                                          97

SCON0   98      RI_0    TI_0    RB8_0   TB8_0       9B

9C      REN_0   SM2_0   SM1_0   SMO_0       9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2     ES1     EA          AF

B0                                          B3

B4                                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PT2     PS1                 BF

SCON1   C0      RI_1    TI_1    RB8_1   TB8_1       C3

C4      REN_1   SM2_1   SM1_1   SMO_1       C7

T2CON   C8      CPRL2   CT2     TR2     EXEN2       CB

CC      TCLK    RCLK    EXF2    TF2         CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

WDCON   D8      RWT     EWT     WTRF    WDIF        DB

DC      PFI     EPFI    POR     SMOD_1      DF

E0                                          E3

E4                                          E7

EIE     E8      EX2     EX3     EX4     EX5         EB

EC      EWDI    ERTCI                       EF

F0                                          F3

F4                                          F7

EIP     F8      PX2     PX3     PX4     PX5         FB

FC      PWDI    PRTCI                       FF

Alternates:

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

SCON    98                                          9B

9C                              FE          9F

SCON0   98                                          9B

9C                              FE_0        9F

SCON1   C0                                          C3

C4                              FE_1        C7

T2CON   C8      CP_RL2  C_T2                        CB

CC                                          CF

AS8XCXXX ASSEMBLER                                     PAGE G-31

DS83C530/DS87C530 SPECIAL FUNCTION REGISTERS

G.10.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

DPS     0x80                                        0x10

0x08                            SEL         0x01

PCON    0x80    SMOD_0  SMOD0                       0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

CKCON   0x80    WD1     WD0     T2M     T1M         0x10

0x08    T0M     MD2     MD1     MD0         0x01

EXIF    0x80    IE5     IE4     IE3     IE          0x10

0x08    XT_RG   RGMD    RGSL    BGS         0x01

TRIM    0x80    E4K     X12_6   TRM2    _TRM2       0x10

0x08    TRM1    _TRM1   TRM0    _TRM0       0x01

SBUF1   0x80    SB7     SB6     SB5     SB4         0x10

0x08    SB3     SB2     SB1     SB0         0x01

ROMSIZE 0x80                                        0x10

0x08            RMS2    RMS1    RMS0        0x01

PMR     0x80    CD1     CD0     SWB                 0x10

0x08    XTOFF   ALEOFF  DME1    DME0        0x01

STATUS  0x80    PIP     HIP     LIP     XTUP        0x10

0x08    SPTA1   SPRA1   SPTA0   SPRA0       0x01

T2MOD   0x80                                        0x10

0x08                    T2OE    DCEN        0x01

RTCC    0x80    SSCE    SCE     MCE     HCE         0x10

0x08    RTCRE   RTCWE   RTCIF   RTCE        0x01

Alternates:

PCON    0x80    SMOD                                0x10

0x08                                        0x01

AS8XCXXX ASSEMBLER                                     PAGE G-32

DS83C530/DS87C530 SPECIAL FUNCTION REGISTERS

G.11  DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

The   DS83C550/DS87C550   Special  Function  Registers  are

selected using the .DS83C550 or DS87C550 assembler directives.

G.11.1  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

80      PORT0   SP      DPL     DPH         83

84      DPL1    DPH1    DPS     PCON        87

88      TCON    TMOD    TL0     TL1         8B

8C      TH0     TH1     CKCON               8F

90      PORT1   RCON                        93

94                                          97

98      SCON0   SBUF0                       9B

9C                              PMR         9F

A0      PORT2   SADDR0  SADDR1              A3

A4                                          A7

A8      IE      CMPL0   CMPL1   CMPL2       AB

AC      CPTL0   CPTL1   CPTL2   CPTL3       AF

B0      PORT3           ADCON1  ADCON2      B3

B4      ADMSB   ADLSD   WINHI   WINLO       B7

B8      IP      SADEN0  SADEN1              BB

BC                      T2CON   T2MOD       BF

C0      PORT4           ROMSIZE             C3

C4      PORT5   STATUS          TA          C7

C8      T2IR    CMPH0   CMPH1   CMPH2       CB

CC      CPTH0   CPTH1   CPTH2   CPTH3       CF

D0      PSW             PW0FG   PW1FG       D3

D4      PW2FG   PW3FG   PWMADR              D7

D8      SCON1   SBUF1                       DB

DC      PWM0    PWM1    PWM2    PWM3        DF

E0      ACC     PW01CS  PW23CS  PW01CON     E3

E4      PW23CON         RLOADL  RLOADH      E7

E8      EIE             T2SEL   CTCON       EB

EC      TL2     TH2     SETR    RSTR        EF

F0      B       PORT6                       F3

F4                                          F7

F8      EIP                                 FB

FC                              WDCON       FF

Alternates:

80      P0                                  83

90      P1                                  93

98      SCON    SBUF                        9B

A0      P2                                  A3

B0      P3                                  B3

C0      P4                                  C3

AS8XCXXX ASSEMBLER                                     PAGE G-33

DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

C4      P5                                  C7

F0              PORT6                       F3

AS8XCXXX ASSEMBLER                                     PAGE G-34

DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

G.11.2  Bit Addressable Registers:  Generic

---------- 4 BITS ----------

----    ----    ----    ----

PORT0   80      P0.7    P0.6    P0.5    P0.4        83

84      P0.3    P0.2    P0.1    P0.0        87

TCON    88      TCON.0  TCON.1  TCON.2  TCON.3      8B

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

PORT1   90      P1.0    P1.1    P1.2    P1.3        93

94      P1.4    P1.5    P1.6    P1.7        97

SCON0   98      SCON0.0 SCON0.1 SCON0.2 SCON0.3     9B

9C      SCON0.4 SCON0.5 SCON0.6 SCON0.7     9F

PORT2   A0      P2.0    P2.1    P2.2    P2.3        A3

A4      P2.4    P2.5    P2.6    P2.7        A7

IE      A8      IE.0    IE.1    IE.2    IE.3        AB

AC      IE.4    IE.5    EI.6    IE.7        AF

PORT3   B0      P3.0    P3.1    P3.2    P3.3        B3

B4      P3.4    P3.5    P3.6    P3.7        B7

IP      B8      IP.0    IP.1    IP.2    IP.3        BB

BC      IP.4    IP.5    IP.6    IP.7        BF

PORT4   C0      P4.0    P4.1    P4.2    P4.3        C3

C4      P4.4    P4.5    P4.6    P4.7        C7

T2IR    C8      T2IR.0  T2IR.1  T2IR.2  T2IR.3      CB

CC      T2IR.4  T2IR.5  T2IR.6  T2IR.7      CF

PSW     D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

SCON1   D8      SCON1.0 SCON1.1 SCON1.2 SCON1.3     DB

DC      SCON1.4 SCON1.5 SCON1.6 SCON1.7     DF

ACC     E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

EIE     E8      EIE.0   EIE.1   EIE.2   EIE.3       EB

EC      EIE.4   EIE.5   EIE.6   EIE.7       EF

B       F0      B.0     B.1     B.2     B.3         F3

F4      B.4     B.5     B.6     B.7         F7

EIP     F8      EIP.0   EIP.1   EIP.2   EIP.3       FB

FC      EIP.4   EIP.5   EIP.6   EIP.7       FF

Alternates:

PORT0   80      PORT0.7 PORT0.6 PORT0.5 PORT0.4     83

84      PORT0.3 PORT0.2 PORT0.1 PORT0.0     87

PORT1   90      PORT1.0 PORT1.1 PORT1.2 PORT1.3     93

94      PORT1.4 PORT1.5 PORT1.6 PORT1.7     97

SCON    98      SCON.0  SCON.1  SCON.2  SCON.3      9B

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

PORT2   A0      PORT2.0 PORT2.1 PORT2.2 PORT2.3     A3

A4      PORT2.4 PORT2.5 PORT2.6 PORT2.7     A7

PORT3   B0      PORT3.0 PORT3.1 PORT3.2 PORT3.3     B3

B4      PORT3.4 PORT3.5 PORT3.6 PORT3.7     B7

PORT4   C0      PORT4.0 PORT4.1 PORT4.2 PORT4.3     C3

C4      PORT4.4 PORT4.5 PORT4.6 PORT4.7     C7

AS8XCXXX ASSEMBLER                                     PAGE G-35

DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

G.11.3  Bit Addressable Registers:  Specific

---------- 4 BITS ----------

----    ----    ----    ----

80                                          83

84                                          87

TCON    88      IT0     IE0     IT1     IE1         8B

8C      TR0     TF0     TR1     TF1         8F

90                                          93

94                                          97

SCON0   98      RI_0    TI_0    RB8_0   TB8_0       9B

9C      REN_0   SM2_0   SM1_0   SMO_0       9F

A0                                          A3

A4                                          A7

IE      A8      EX0     ET0     EX1     ET1         AB

AC      ES0     ET2     ES1     EA          AF

B0                                          B3

B4                                          B7

IP      B8      PX0     PT0     PX1     PT1         BB

BC      PS0     PS1     PAD                 BF

PORT4   C0      CMSR0   CMSR1   CMSR2   CMSR3       C3

C4      CMSR4   CMSR5   CMT0    CMT1        C7

T2IR    C8      CF0     CF1     CF2     CF3         CB

CC      CM0F    CM1F    CM2F                CF

PSW     D0      P       FL      OV      RS0         D3

D4      RS1     F0      AC      CY          D7

SCON1   D8      RI_1    TI_1    RB8_1   TB8_1       DB

DC      REN_1   SM2_1   SM1_1   SMO_1       DF

E0                                          E3

E4                                          E7

EIE     E8      EX2     EX3     EX4     EX5         EB

EC      ECM0    ECM1    ECM2    ET2         EF

F0                                          F3

F4                                          F7

EIP     F8      PX2     PX3     PX4     PX5         FB

FC      PCM0    PCM1    PCM2    PT2         FF

Alternates:

SCON    98      RI      TI      RB8     TB8         9B

9C      REN     SM2     SM1     SMO         9F

SCON    98                                          9B

9C                              FE          9F

SCON0   98                                          9B

9C                              FE_0        9F

T2IR    C8      IE2     IE3     IE4     IE5         CB

CC                                          CF

SCON1   D8                                          DB

DC                              FE_1        DF

EIE     E8      EC0     EC1     EC2     EC3         EB

EC                                          EF

EIP     F8      PC0     PC1     PC2     PC3         FB

AS8XCXXX ASSEMBLER                                     PAGE G-36

DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

FC                                          FF

AS8XCXXX ASSEMBLER                                     PAGE G-37

DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

G.11.4  Optional Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

0x80    0x40    0x20    0x10

0x08    0x04    0x02    0x10

----    ----    ----    ----

DPS     0x80    ID1     ID0     TSL                 0x10

0x08                            SEL         0x01

PCON    0x80    SMOD_0  SMOD0                       0x10

0x08    GF1     GF0     STOP    IDLE        0x01

TMOD    0x80    T1GATE  T1C_T   T1M1    T1M0        0x10

0x08    T0GATE  T0C_T   T0M1    T0M0        0x01

CKCON   0x80    WD1     WD0     T2M     T1M         0x10

0x08    T0M     MD2     MD1     MD0         0x01

RCON    0x80                                        0x10

0x08    CKRDY   RGMD    RGSL    BGS         0x01

PMR     0x80    CD1     CD0     SWB     CTM         0x10

0x08    4X_2X   ALEOFF  DEM1    DEM0        0x01

ADCON1  0x80   STRT_BSY EOC     CONT_SS ADEX        0x10

0x08    WCQ     WCM     ADON    WCIO        0x01

ADCON2  0x80    OUTCF   MUX2    MUX1    MUX0        0x10

0x08    APS3    APS2    APS1    APS0        0x01

T2CON   0x80    TF2     EXF2    RCLK    TCLK        0x10

0x08    EXEN2   TR2     CT2     CPRL2       0x01

T2MOD   0x80                                        0x10

0x08                    T2OE    DCEN        0x01

PORT5   0x80    ADC7    ADC6    ADC5    ADC4        0x10

0x08    ADC3    ADC2    ADC1    ADC0        0x01

ROMSIZE 0x80                                        0x10

0x08            RMS2    RMS1    RMS0        0x01

STATUS  0x80    PIP     HIP     LIP     XTUP        0x10

0x08    SPTA1   SPRA1   SPTA0   SPRA0       0x01

PWMADR  0x80    ADRS                                0x10

0x08                    PWE1    PWE0        0x01

PW01CS  0x80    PW0S2   PW0S1   PW0S0   PW0EN       0x10

0x08    PW1S2   PW1S1   PW1S0   PW1EN       0x01

PW23CS  0x80    PW2S2   PW2S1   PW2S0   PW2EN       0x10

0x08    PW3S2   PW3S1   PW3S0   PW3EN       0x01

PW01CON 0x80    PW0F    PW0DC   PW0OE   PW0T_C      0x10

0x08    PW1F    PW1DC   PW1OE   PW1T_C      0x01

PW23CON 0x80    PW2F    PW2DC   PW2OE   PW2T_C      0x10

0x08    PW3F    PW3DC   PW3OE   PW3T_C      0x01

T2SEL   0x80    TF2S    TF2BS           TF2B        0x10

0x08                    T2P1    T2P0        0x01

CTCON   0x80    _CT3    CT3     _CT2    CT2         0x10

0x08    _CT1    CT1     _CT0    CT0         0x01

SETR    0x80    TGFF1   TGFF0   CMS5    CMS4       0x10

0x08    CMS3    CMS2    CMS1    CMS0        0x01

RSTR    0x80    CMTE1   CMTE0   CMR5    CMR4        0x10

0x08    CMR3    CMR2    CMR1    CMR0        0x01

PORT6   0x80    STADC           PWMC1   PWMC0       0x10

AS8XCXXX ASSEMBLER                                     PAGE G-38

DS83C550/DS87C550 SPECIAL FUNCTION REGISTERS

0x08    PWMO3   PWMO2   PWMO1   PWMO0       0x01

WDCON   0x80    SMOD_1  POR     EPF1    PF1         0x10

0x08    WDIF    WTRF    EWT     RWT         0x01

Alternates:

PCON    0x80    SMOD                                0x10

0x08                                        0x01

T2CON   0x80                                        0x10

0x08                    C_T2    _RL2        0x01

APPENDIX H

AS430 ASSEMBLER

H.1  MPS430 REGISTER SET

The following is a list of the MPS430 registers used by AS430:

Sixteen 16-bit registers provide adddress, data, and

special functions:

pc  /   r0      -       program counter

sp  /   r1      -       stack pointer

sr  /   r2      -       status register

cg1 /   r2      -       constant generator 1

cg2 /   r3      -       constant generator 2

r4      -       working register r4

r5      -       working register r5

...

r14     -       working register r14

r15     -       working register r15

AS430 ASSEMBLER                                         PAGE H-2

MPS430 REGISTER SET

H.2  MPS430 ADDRESSING MODES

The following list specifies the format for each addressing mode

supported by AS430:

Source/Destination Operand Addressing Modes

As/Ad   Addressing Mode Syntax  Description

-----   --------------- ------  -----------

00/0    Register mode   Rn      Register contents are operand.

01/1    Indexed mode    X(Rn)   (Rn + X) points to the operand,

X is stored in the next word.

01/1    Symbolic mode   ADDR    (PC + X) points to the operand,

X is stored in the next word,

Indexed mode X(PC) is used.

01/1    Absolute mode   &ADDR   The word following the

instruction, contains the

absolute address.

10/-    Indirect        @Rn     Rn is used as a pointer to the

register mode           operand.

11/-    Indirect        @Rn+    Rn is used as a pointer to the

autoincrement           operand. Rn is incremented

afterwards.

11/-    Immediate mode  #N      The word following the

instruction contains the

immediate constant N. Indirect

autoincrement mode @PC+ is used.

The  terms  ADDR, X and N may all be expressions.  Note that not

all addressing modes are valid with every instruction, refer  to

the MPS430 technical data for valid modes.

AS430 ASSEMBLER                                         PAGE H-3

MPS430 ADDRESSING MODES

H.2.1  MPS430 Instruction Mnemonics

The following table lists all MPS430 family mnemonics recognized

by the AS430 assembler.  The designations src and dst  refer  to

required source and/or destination addressing mode arguments.

* ADC[.W];ADC.B  dst        dst + C -> dst

ADD[.W];ADD.B  src,dst    src + dst -> dst

ADDC[.W];ADDC.B           src,dst src + dst + C -> dst

AND[.W];AND.B  src,dst    src .and. dst -> dst

BIC[.W];BIC.B src,dst     .not.src .and. dst -> dst

BIS[.W];BIS.B src,dst     src .or. dst -> dst

BIT[.W];BIT.B src,dst     src .and. dst

* BR dst                Branch to .......

* BRANCH dst            Branch to .......

CALL dst              PC+2 -> stack, dst -> PC

* CLR[.W];CLR.B dst     Clear destination

* CLRC                  Clear carry bit

* CLRN                  Clear negative bit

* CLRZ                  Clear zero bit

CMP[.W];CMP.B src,dst     dst - src

* DADC[.W];DADC.B dst       dst + C -> dst (decimal)

DADD[.W];DADD.B src,dst   src + dst + C -> dst (decimal)

* DEC[.W];DEC.B dst     dst - 1 -> dst

* DECD[.W];DECD.B dst   dst - 2 -> dst

* DINT                  Disable interrupt

* EINT                  Enable interrupt

* INC[.W];INC.B dst     dst + 1 -> dst

* INCD[.W];INCD.B dst   dst + 2 -> dst

* INV[.W];INV.B dst     Invert destination

JC/JHS Label          Jump to Label if Carry-bit is set

JEQ/JZ Label          Jump to Label if Zero-bit is set

JGE Label             Jump to Label if (N .XOR. V) = 0

JL Label              Jump to Label if (N .XOR. V) = 1

JMP Label             Jump to Label unconditionally

JN Label              Jump to Label if Negative-bit is set

JNC/JLO Label         Jump to Label if Carry-bit is reset

JNE/JNZ Label         Jump to Label if Zero-bit is reset

MOV[.W];MOV.B src,dst     src -> dst

* NOP                   No operation

AS430 ASSEMBLER                                         PAGE H-4

MPS430 ADDRESSING MODES

* POP[.W];POP.B dst     Item from stack, SP+2 -> SP

PUSH[.W];PUSH.B src   SP - 2 -> SP, src -> @SP

RETI                  Return from interrupt

TOS -> SR, SP + 2 -> SP

TOS -> PC, SP + 2 -> SZP

* RET                   Return from subroutine

TOS -> PC, SP + 2 -> SP

* RLA[.W];RLA.B dst     Rotate left arithmetically

* RLC[.W];RLC.B dst     Rotate left through carry

RRA[.W];RRA.B dst     MSB -> MSB . ....LSB -> C

RRC[.W];RRC.B dst     C -> MSB . ......LSB -> C

* SBC[.W];SBC.B dst     Subtract carry from destination

* SETC                  Set carry bit

* SETN                  Set negative bit

* SETZ                  Set zero bit

SUB[.W];SUB.B src,dst     dst + .not.src + 1 -> dst

SUBC[.W];SUBC.B src,dst   dst + .not.src + C -> dst

SBB[.W];SBB.B src,dst     dst + .not.src + C -> dst

SWPB dst              swap bytes

SXT dst               Bit7 -> Bit8 ........ Bit15

* TST[.W];TST.B dst     Test destination

XOR[.W];XOR.B src,dst     src .xor. dst -> dst

Note: Asterisked Instructions

Asterisked (*) instructions are emulated.

They are replaced with coreinstructions

by the assembler.

APPENDIX I

ASH8 ASSEMBLER

I.1  H8/3XX REGISTER SET

The following is a list of the H8 registers used by ASH8:

r0  -  r7,sp            16-bit accumulators

r0L -  r7L,spL          8-bit accumulators

r0H -  r7H,spH          8-bit accumulators

spL,spH,sp              stack pointers

ccr                     condition code

I.2  H8/3XX INSTRUCTION SET

The  following  tables list all H8/3xx mnemonics recognized

by the ASH8 assembler.  The designation [] refers to a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by ASH8:

#xx:3           immediate data (3  bit)

#xx:8           immediate data (8  bit)

#xx:16          immediate data (16 bit)

*dir            direct page addressing

(see .setdp directive)

0xFF00 <= dir <= 0xFFFF

label           branch label

rn              registers (16 bit)

r0-r7,sp

rnB             registers (8 bit)

r0H-r7H,r0L-r7L,spH,spL

ASH8 ASSEMBLER                                          PAGE I-2

H8/3XX INSTRUCTION SET

ccr             condition code register

@rn             register indirect

@-rn            register indirect (auto pre-decrement)

@rn+            register indirect (auto post-increment)

@[offset,rn]    register indirect, 16-bit displacement

@@offset        memory indirect, (8-bit address)

ext             extended addressing (16-bit)

The  terms  data, dir, label, offset, and ext may all be expres-

sions.

Note that not all addressing modes are valid with every in-

struction, refer to the H8/3xx technical data for valid modes.

I.2.1  Inherent Instructions

eepmov

nop

sleep

rte

rts

I.2.2  Branch Instructions

bcc     label                   bcs     label

beq     label                   bf      label

bge     label                   bgt     label

bhi     label                   bhis    label

bhs     label                   ble     label

blo     label                   blos    label

bls     label                   blt     label

bmi     label                   bne     label

bpl     label                   bra     label

brn     label                   bt      label

bvc     label                   bvs     label

bsr     label

ASH8 ASSEMBLER                                          PAGE I-3

H8/3XX INSTRUCTION SET

I.2.3  Single Operand Instructions

Free Form

daa     rnB                     das     rnB

dec     rnB                     inc     rnB

neg     rnB                     not     rnB

rotxl   rnB                     rotxr   rnB

rotl    rnB                     rotr    rnB

shal    rnB                     shar    rnB

shll    rnB                     shlr    rnB

push    rn                      pop     rn

Byte / Word Form

daa.b   rnB                     das.b   rnB

dec.b   rnB                     inc.b   rnB

neg.b   rnB                     not.b   rnB

rotxl.b rnB                     rotxr.b rnB

rotl.b  rnB                     rotr.b  rnB

shal.b  rnB                     shar.b  rnB

shll.b  rnB                     shlr.b  rnB

push.w  rn                      pop.w   rn

ASH8 ASSEMBLER                                          PAGE I-4

H8/3XX INSTRUCTION SET

I.2.4  Double Operand Instructions

Free Form

add     rnB,rnB                 add     #xx:8,rnB

add     rn,rn

adds    #1,rn                   adds    #2,rn

addx    rnB,rnB                 addx    #xx:8,rnB

cmp     rnB,rnB                 cmp     #xx:8,rnB

cmp     rn,rn

sub     rnB,rnB

sub     rn,rn

subs    #1,rn                   subs    #2,rn

subx    rnB,rnB                 subx    #xx:8,rnB

and     rnB,rnB                 and     #xx:8,rnB

and     #xx:8,ccr

or      rnB,rnB                 or      #xx:8,rnB

or      #xx:8,ccr

xor     rnB,rnB                 xor     #xx:8,rnB

xor     #xx:8,ccr

Byte / Word Form

add.b   rnB,rnB                 add.b   #xx:8,rnB

add.w   rn,rn

cmp.b   rnB,rnB                 cmp.b   #xx:8,rnB

cmp.w   rn,rn

sub.b   rnB,rnB

sub.w   rn,rn

addx.b  rnB,rnB                 addx.b  #xx:8,rnB

and.b   rnB,rnB                 and.b   #xx:8,rnB

and.b   #xx:8,ccr

or.b    rnB,rnB                 or.b    #xx:8,rnB

or.b    #xx:8,ccr

subx.b  rnB,rnB                 subx.b  #xx:8,rnB

xor.b   rnB,rnB                 xor.b   #xx:8,rnB

xor.b   #xx:8,ccr

ASH8 ASSEMBLER                                          PAGE I-5

H8/3XX INSTRUCTION SET

I.2.5  Mov Instructions

Free Form

mov     rnB,rnB                 mov     rn,rn

mov     #xx:8,rnB               mov     #xx:16,rn

mov     @rn,rnB                 mov     @rn,rn

mov     @[offset,rn],rnB        mov     @[offset,rn],rn

mov     @rn+,rnB                mov     @rn+,rn

mov     @dir,rnB

mov     dir,rnB

mov     *@dir,rnB

mov     *dir,rnB

mov     @label,rnB              mov     @label,rn

mov     label,rnB               mov     label,rn

mov     rnB,@rn                 mov     rn,@rn

mov     rnB,@[offset,rn]        mov     rn,@[offset,rn]

mov     rnB,@-rn                mov     rn,@-rn

mov     rnB,@dir

mov     rnB,dir

mov     rnB,*@dir

mov     rnB,*dir

mov     rnB,@label              mov     rn,@label

mov     rnB,label               mov     rn,label

Byte / Word Form

mov.b   rnB,rnB                 mov.w   rn,rn

mov.b   #xx:8,rnB               mov.w   #xx:16,rn

mov.b   @rn,rnB                 mov.w   @rn,rn

mov.b   @[offset,rn],rnB        mov.w   @[offset,rn],rn

mov.b   @rn+,rnB                mov.w   @rn+,rn

mov.b   @dir,rnB

mov.b   dir,rnB

mov.b   *@dir,rnB

mov.b   *dir,rnB

mov.b   @label,rnB              mov.w   @label,rn

mov.b   label,rnB               mov.w   label,rn

mov.b   rnB,@rn                 mov.w   rn,@rn

mov.b   rnB,@[offset,rn]        mov.w   rn,@[offset,rn]

mov.b   rnB,@-rn                mov.w   rn,@-rn

mov.b   rnB,@dir

mov.b   rnB,dir

mov.b   rnB,*@dir

mov.b   rnB,*dir

mov.b   rnB,@label              mov.w   rn,@label

mov.b   rnB,label               mov.w   rn,label

ASH8 ASSEMBLER                                          PAGE I-6

H8/3XX INSTRUCTION SET

I.2.6  Bit Manipulation Instructions

bld     #xx:3,rnB               bld     #xx:3,@rn

bld     #xx:3,@dir              bld     #xx:3,dir

bld     #xx:3,*@dir             bld     #xx:3,*dir

bild    #xx:3,rnB               bild    #xx:3,@rn

bild    #xx:3,@dir              bild    #xx:3,dir

bild    #xx:3,*@dir             bild    #xx:3,*dir

bst     #xx:3,rnB               bst     #xx:3,@rn

bst     #xx:3,@dir              bst     #xx:3,dir

bst     #xx:3,*@dir             bst     #xx:3,*dir

bist    #xx:3,rnB               bist    #xx:3,@rn

bist    #xx:3,@dir              bist    #xx:3,dir

bist    #xx:3,*@dir             bist    #xx:3,*dir

band    #xx:3,rnB               band    #xx:3,@rn

band    #xx:3,@dir              band    #xx:3,dir

band    #xx:3,*@dir             band    #xx:3,*dir

biand   #xx:3,rnB               biand   #xx:3,@rn

biand   #xx:3,@dir              biand   #xx:3,dir

biand   #xx:3,*@dir             biand   #xx:3,*dir

bor     #xx:3,rnB               bor     #xx:3,@rn

bor     #xx:3,@dir              bor     #xx:3,dir

bor     #xx:3,*@dir             bor     #xx:3,*dir

bior    #xx:3,rnB               bior    #xx:3,@rn

bior    #xx:3,@dir              bior    #xx:3,dir

bior    #xx:3,*@dir             bior    #xx:3,*dir

bxor    #xx:3,rnB               bxor    #xx:3,@rn

bxor    #xx:3,@dir              bxor    #xx:3,dir

bxor    #xx:3,*@dir             bxor    #xx:3,*dir

bixor   #xx:3,rnB               bixor   #xx:3,@rn

bixor   #xx:3,@dir              bixor   #xx:3,dir

bixor   #xx:3,*@dir             bixor   #xx:3,*dir

ASH8 ASSEMBLER                                          PAGE I-7

H8/3XX INSTRUCTION SET

I.2.7  Extended Bit Manipulation Instructions

bset    #xx:3,rnB               bset    #xx:3,@rn

bset    #xx:3,@dir              bset    #xx:3,dir

bset    #xx:3,*@dir             bset    #xx:3,*dir

bset    rnB,rnB                 bset    rnB,@rn

bset    rnB,@dir                bset    rnB,dir

bset    rnB,*@dir               bset    rnB,*dir

bclr    #xx:3,rnB               bclr    #xx:3,@rn

bclr    #xx:3,@dir              bclr    #xx:3,dir

bclr    #xx:3,*@dir             bclr    #xx:3,*dir

bclr    rnB,rnB                 bclr    rnB,@rn

bclr    rnB,@dir                bclr    rnB,dir

bclr    rnB,*@dir               bclr    rnB,*dir

bnot    #xx:3,rnB               bnot    #xx:3,@rn

bnot    #xx:3,@dir              bnot    #xx:3,dir

bnot    #xx:3,*@dir             bnot    #xx:3,*dir

bnot    rnB,rnB                 bnot    rnB,@rn

bnot    rnB,@dir                bnot    rnB,dir

bnot    rnB,*@dir               bnot    rnB,*dir

btst    #xx:3,rnB               btst    #xx:3,@rn

btst    #xx:3,@dir              btst    #xx:3,dir

btst    #xx:3,*@dir             btst    #xx:3,*dir

btst    rnB,rnB                 btst    rnB,@rn

btst    rnB,@dir                btst    rnB,dir

btst    rnB,*@dir               btst    rnB,*dir

I.2.8  Condition Code Instructions

andc    #xx:8,ccr               andc    #xx:8

and     #xx:8,ccr               and.b   #xx:8,ccr

ldc     #xx:8,ccr               ldc     #xx:8

ldc     rnB,ccr                 ldc     rnB

orc     #xx:8,ccr               orc     #xx:8

or      #xx:8,ccr               or.b    #xx:8,ccr

xorc    #xx:8,ccr               xorc    #xx:8

xor     #xx:8,ccr               xor.b   #xx:8,ccr

stc     ccr,rnB                 stc     rnB

ASH8 ASSEMBLER                                          PAGE I-8

H8/3XX INSTRUCTION SET

I.2.9  Other Instructions

divxu   rnB,rn                  divxu.b rnB,rn

mulxu   rnB,rn                  mulxu.b rnB,rn

movfpe  @label,rnB              movfpe  label,rnB

movfpe.b  @label,rnB            movfpe.b  label,rnB

movtpe  @label,rnB              movtpe  label,rnB

movtpe.b  @label,rnB            movtpe.b  label,rnB

I.2.10  Jump and Jump to Subroutine Instructions

jmp     @rn                     jmp     @@dir

jmp     @label                  jmp     label

jsr     @rn                     jsr     @@dir

jsr     @label                  jsr     label

APPENDIX J

AS6800 ASSEMBLER

J.1  6800 REGISTER SET

The following is a list of the 6800 registers used by AS6800:

a,b     -       8-bit accumulators

x       -       index register

J.2  6800 INSTRUCTION SET

The  following  tables  list  all  6800/6802/6808 mnemonics

recognized by the AS6800 assembler.  The designation  []  refers

to  a  required  addressing  mode  argument.  The following list

specifies the format  for  each  addressing  mode  supported  by

AS6800:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255

ext             extended addressing

label           branch label

The  terms  data, dir, offset, ext, and label may all be expres-

sions.

AS6800 ASSEMBLER                                        PAGE J-2

6800 INSTRUCTION SET

Note that not all addressing modes are valid with every in-

struction, refer to the 6800 technical data for valid modes.

J.2.1  Inherent Instructions

aba                     cba

clc                     cli

clv                     daa

des                     dex

ins                     inx

nop                     rti

rts                     sba

sec                     sei

sev                     swi

tab                     tap

tba                     tpa

tsx                     txs

wai

psha                    pshb

psh a                   psh b

pula                    pulb

pul a                   pul b

J.2.2  Branch Instructions

bra     label           bhi     label

bls     label           bcc     label

bhs     label           bcs     label

blo     label           bne     label

beq     label           bvc     label

bvs     label           bpl     label

bmi     label           bge     label

blt     label           bgt     label

ble     label           bsr     label

AS6800 ASSEMBLER                                        PAGE J-3

6800 INSTRUCTION SET

J.2.3  Single Operand Instructions

asla                    aslb

asl a                   asl b

asl     []

asra                    asrb

asr a                   asr b

asr     []

clra                    clrb

clr a                   clr b

clr     []

coma                    comb

com a                   com b

com     []

deca                    decb

dec a                   dec b

dec     []

inca                    incb

inc a                   inc b

inc     []

lsla                    lslb

lsl a                   lsl b

lsl     []

lsra                    lsrb

lsr a                   lsr b

lsr     []

nega                    negb

neg a                   neg b

neg     []

rola                    rolb

rol a                   rol b

rol     []

rora                    rorb

ror a                   ror b

ror     []

tsta                    tstb

tst a                   tst b

tst     []

AS6800 ASSEMBLER                                        PAGE J-4

6800 INSTRUCTION SET

J.2.4  Double Operand Instructions

adca    []              adcb    []

adc a   []              adc b   []

adda    []              addb    []

add a   []              add b   []

anda    []              andb    []

and a   []              and b   []

bita    []              bitb    []

bit a   []              bit b   []

cmpa    []              cmpb    []

cmp a   []              cmp b   []

eora    []              eorb    []

eor a   []              eor b   []

ldaa    []              ldab    []

lda a   []              lda b   []

oraa    []              orab    []

ora a   []              ora b   []

sbca    []              sbcb    []

sbc a   []              sbc b   []

staa    []              stab    []

sta a   []              sta b   []

suba    []              subb    []

sub a   []              sub b   []

J.2.5  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

AS6800 ASSEMBLER                                        PAGE J-5

6800 INSTRUCTION SET

J.2.6  Long Register Instructions

cpx     []

lds     []              sts     []

ldx     []              stx     []

APPENDIX K

AS6801 ASSEMBLER

K.1  .hd6303 DIRECTIVE

Format:

.hd6303

The  .hd6303 directive enables processing of the HD6303 specific

mnemonics not included in  the  6801  instruction  set.   HD6303

mnemonics  encountered  without  the  .hd6303  directive will be

flagged with an 'o' error.

K.2  6801 REGISTER SET

The following is a list of the 6801 registers used by AS6801:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x       -       index register

K.3  6801 INSTRUCTION SET

The  following  tables  list all 6801/6303 mnemonics recog-

nized by the AS6801 assembler.  The designation [] refers  to  a

required addressing mode argument.  The following list specifies

the format for each addressing mode supported by AS6801:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

AS6801 ASSEMBLER                                        PAGE K-2

6801 INSTRUCTION SET

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255

ext             extended addressing

label           branch label

The  terms  data, dir, offset, ext, and label may all be expres-

sions.

Note that not all addressing modes are valid with every in-

struction, refer to  the  6801/6303  technical  data  for  valid

modes.

K.3.1  Inherent Instructions

aba             abx

cba             clc

cli             clv

daa             des

dex             ins

inx             mul

nop             rti

rts             sba

sec             sei

sev             swi

tab             tap

tba             tpa

tsx             txs

wai

K.3.2  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bhs     label

bcs     label           blo     label

bne     label           beq     label

bvc     label           bvs     label

bpl     label           bmi     label

bge     label           blt     label

bgt     label           ble     label

bsr     label

AS6801 ASSEMBLER                                        PAGE K-3

6801 INSTRUCTION SET

K.3.3  Single Operand Instructions

asla            aslb            asld

asl a           asl b           asl d

asl     []

asra            asrb

asr a           asr b

asr     []

clra            clrb

clr a           clr b

clr     []

coma            comb

com a           com b

com     []

deca            decb

dec a           dec b

dec     []

eora            eorb

eor a           eor b

eor     []

inca            incb

inc a           inc b

inc     []

lsla            lslb            lsld

lsl a           lsl b           lsl d

lsl     []

lsra            lsrb            lsrd

lsr a           lsr b           lsr d

lsr     []

nega            negb

neg a           neg b

neg     []

psha            pshb            pshx

psh a           psh b           psh x

pula            pulb            pulx

pul a           pul b           pul x

rola            rolb

rol a           rol b

rol     []

AS6801 ASSEMBLER                                        PAGE K-4

6801 INSTRUCTION SET

rora            rorb

ror a           ror b

ror     []

tsta            tstb

tst a           tst b

tst     []

K.3.4  Double Operand Instructions

adca    []      adcb    []

adc a   []      adc b   []

adda    []      addb    []      addd    []

add a   []      add b   []      add d   []

anda    []      andb    []

and a   []      and b   []

bita    []      bitb    []

bit a   []      bit b   []

cmpa    []      cmpb    []

cmp a   []      cmp b   []

ldaa    []      ldab    []

lda a   []      lda b   []

oraa    []      orab    []

ora a   []      ora b   []

sbca    []      sbcb    []

sbc a   []      sbc b   []

staa    []      stab    []

sta a   []      sta b   []

suba    []      subb    []      subd    []

sub a   []      sub b   []      sub d   []

AS6801 ASSEMBLER                                        PAGE K-5

6801 INSTRUCTION SET

K.3.5  Jump and Jump to Subroutine Instructions

jmp     []      jsr     []

K.3.6  Long Register Instructions

cpx     []      ldd     []

lds     []      ldx     []

std     []      sts     []

stx     []

K.3.7  6303 Specific Instructions

aim     #data, []       eim     #data, []

oim     #data, []       tim     #data, []

xgdx            slp

APPENDIX L

AS6804 ASSEMBLER

Requires the .setdp directive to specify the ram area.

L.1  6804 REGISTER SET

The following is a list of the 6804 registers used by AS6804:

x,y     -       index registers

L.2  6804 INSTRUCTION SET

The  following tables list all 6804 mnemonics recognized by

the AS6804 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6804:

#data           immediate data

byte or word data

,x              register indirect addressing

dir             direct addressing

(see .setdp directive)

0 <= dir <= 255

ext             extended addressing

label           branch label

The  terms data, dir, and ext may be expressions.  The label for

the short branchs beq, bne, bcc, and bcs must not be external.

Note that not all addressing modes are valid with every in-

struction, refer to the 6804 technical data for valid modes.

AS6804 ASSEMBLER                                        PAGE L-2

6804 INSTRUCTION SET

L.2.1  Inherent Instructions

coma            decx

decy            incx

incy            rola

rti             rts

stop            tax

tay             txa

tya             wait

L.2.2  Branch Instructions

bne     label           beq     label

bcc     label           bcs     label

L.2.3  Single Operand Instructions

add     []

and     []

cmp     []

dec     []

inc     []

lda     []

sta     []

sub     []

L.2.4  Jump and Jump to Subroutine Instructions

jsr     []

jmp     []

L.2.5  Bit Test Instructions

brclr   #data,[],label

brset   #data,[],label

bclr    #label,[]

bset    #label,[]

AS6804 ASSEMBLER                                        PAGE L-3

6804 INSTRUCTION SET

L.2.6  Load Immediate data Instruction

mvi     [],#data

L.2.7  6804 Derived Instructions

asla

bam     label

bap     label

bxmi    label

bxpl    label

bymi    label

bypl    label

clra

clrx

clry

deca

decx

decy

inca

incx

incy

ldxi    #data

ldyi    #data

nop

tax

tay

txa

tya

APPENDIX M

AS6805 ASSEMBLER

M.1  6805 REGISTER SET

The following is a list of the 6805 registers used by AS6805:

a       -       8-bit accumulator

x       -       index register

M.2  6805 INSTRUCTION SET

The  following tables list all 6805 mnemonics recognized by

the AS6805 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6805:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255   --- byte mode

256 <= offset <= 65535 --- word mode

(an externally defined offset uses the

word mode)

ext             extended addressing

label           branch label

AS6805 ASSEMBLER                                        PAGE M-2

6805 INSTRUCTION SET

The terms data, dir, offset, and ext may all be expressions.

Note that not all addressing modes are valid with every in-

struction, refer to the 6805 technical data for valid modes.

M.2.1  Control Instructions

clc             cli

nop             rsp

rti             rts

sec             sei

stop            swi

tax             txa

wait

M.2.2  Bit Manipulation Instructions

brset   #data,*dir,label

brclr   #data,*dir,label

bset    #data,*dir

bclr    #data,*dir

M.2.3  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bcs     label

bne     label           beq     label

bhcc    label           bhcs    label

bpl     label           bmi     label

bmc     label           bms     label

bil     label           bih     label

bsr     label

AS6805 ASSEMBLER                                        PAGE M-3

6805 INSTRUCTION SET

M.2.4  Read-Modify-Write Instructions

nega            negx

neg     []

coma            comx

com     []

lsra            lsrx

lsr     []

rora            rorx

ror     []

asra            asrx

asr     []

lsla            lslx

lsl     []

rola            rolx

rol     []

deca            decx

dec     []

inca            incx

inc     []

tsta            tstx

tst     []

clra            clrx

clr     []

M.2.5  Register\Memory Instructions

sub     []              cmp     []

sbc     []              cpx     []

and     []              bit     []

lda     []              sta     []

eor     []              adc     []

ora     []              add     []

ldx     []              stx     []

AS6805 ASSEMBLER                                        PAGE M-4

6805 INSTRUCTION SET

M.2.6  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

APPENDIX N

AS6808 ASSEMBLER

N.1  68HC08 REGISTER SET

The  following  is  a  list  of  the  68HC08  registers  used by

AS68HC08:

a       -       8-bit accumulator

x       -       index register  <H:X>

s       -       stack pointer

N.2  68HC08 INSTRUCTION SET

The  following  tables list all 68HC08 mnemonics recognized

by the AS6808 assembler.  The designation []  refers  to  a  re-

quired  addressing  mode argument.  The following list specifies

the format for each addressing mode supported by AS6808:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indexed addressing

zero offset

offset,x        register indexed addressing

0 <= offset <= 255   --- byte mode

256 <= offset <= 65535 --- word mode

(an externally defined offset uses the

word mode)

,x+             register indexed addressing

zero offset with post increment

AS6808 ASSEMBLER                                        PAGE N-2

68HC08 INSTRUCTION SET

offset,x+       register indexed addressing

unsigned byte offset with post increment

offset,s        stack pointer indexed addressing

0 <= offset <= 255   --- byte mode

256 <= offset <= 65535 --- word mode

(an externally defined offset uses the

word mode)

ext             extended addressing

label           branch label

The terms data, dir, offset, and ext may all be expressions.

Note that not all addressing modes are valid with every in-

struction, refer to the 68HC08 technical data for valid modes.

N.2.1  Control Instructions

clc             cli             daa             div

mul             nop             nsa             psha

pshh            pshx            pula            pulh

pulx            rsp             rti             rts

sec             sei             stop            swi

tap             tax             tpa             tsx

txa             txs             wait

N.2.2  Bit Manipulation Instructions

brset   #data,*dir,label

brclr   #data,*dir,label

bset    #data,*dir

bclr    #data,*dir

AS6808 ASSEMBLER                                        PAGE N-3

68HC08 INSTRUCTION SET

N.2.3  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bcs     label

bne     label           beq     label

bhcc    label           bhcs    label

bpl     label           bmi     label

bmc     label           bms     label

bil     label           bih     label

bsr     label           bge     label

blt     label           bgt     label

ble     label

N.2.4  Complex Branch Instructions

cbeqa   [],label

cbeqx   [],label

cbeq    [],label

dbnza   label

dbnzx   label

dbnz    [],label

AS6808 ASSEMBLER                                        PAGE N-4

68HC08 INSTRUCTION SET

N.2.5  Read-Modify-Write Instructions

nega                    negx

neg     []

coma                    comx

com     []

lsra                    lsrx

lsr     []

rora                    rorx

ror     []

asra                    asrx

asr     []

asla                    aslx

asl     []

lsla                    lslx

lsl     []

rola                    rolx

rol     []

deca                    decx

dec     []

inca                    incx

inc     []

tsta                    tstx

tst     []

clra                    clrx

clr     []              clrh

aix     #data

ais     #data

AS6808 ASSEMBLER                                        PAGE N-5

68HC08 INSTRUCTION SET

N.2.6  Register\Memory Instructions

sub     []              cmp     []

sbc     []              cpx     []

and     []              bit     []

lda     []              sta     []

eor     []              adc     []

ora     []              add     []

ldx     []              stx     []

N.2.7  Double Operand Move Instruction

mov     [],[]

N.2.8  16-Bit <H:X> Index Register Instructions

cphx    []

ldhx    []

sthx    []

N.2.9  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

APPENDIX O

AS6809 ASSEMBLER

O.1  6809 REGISTER SET

The following is a list of the 6809 registers used by AS6809:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x,y     -       index registers

s,u     -       stack pointers

pc      -       program counter

cc      -       condition code

dp      -       direct page

O.2  6809 INSTRUCTION SET

The  following tables list all 6809 mnemonics recognized by

the AS6809 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS6809:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

label           branch label

r,r1,r2         registers

cc,a,b,d,dp,x,y,s,u,pc

,-x     ,--x    register indexed

autodecrement

AS6809 ASSEMBLER                                        PAGE O-2

6809 INSTRUCTION SET

,x+     ,x++    register indexed

autoincrement

,x              register indexed addressing

zero offset

offset,x        register indexed addressing

-16 <= offset <= 15    ---  5-bit

-128 <= offset <= -17   ---  8-bit

16 <= offset <= 127   ---  8-bit

-32768 <= offset <= -129  --- 16-bit

128 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

a,x             accumulator offset indexed addressing

ext             extended addressing

ext,pc          pc addressing ( pc <- pc + ext )

ext,pcr         pc relative addressing

[,--x]          register indexed indirect

autodecrement

[,x++]          register indexed indirect

autoincrement

[,x]            register indexed indirect addressing

zero offset

[offset,x]      register indexed indirect addressing

-128 <= offset <= 127   ---  8-bit

-32768 <= offset <= -129  --- 16-bit

128 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

[a,x]           accumulator offset indexed

indirect addressing

[ext]           extended indirect addressing

[ext,pc]        pc indirect addressing

( [pc <- pc + ext] )

[ext,pcr]       pc relative indirect addressing

The  terms  data, dir, label, offset, and ext may all be expres-

sions.

AS6809 ASSEMBLER                                        PAGE O-3

6809 INSTRUCTION SET

Note that not all addressing modes are valid with every in-

struction, refer to the 6809 technical data for valid modes.

O.2.1  Inherent Instructions

abx             daa

mul             nop

rti             rts

sex             swi

swi1            swi2

swi3            sync

O.2.2  Short Branch Instructions

bcc     label           bcs     label

beq     label           bge     label

bgt     label           bhi     label

bhis    label           bhs     label

ble     label           blo     label

blos    label           bls     label

blt     label           bmi     label

bne     label           bpl     label

bra     label           brn     label

bvc     label           bvs     label

bsr     label

O.2.3  Long Branch Instructions

lbcc    label           lbcs    label

lbeq    label           lbge    label

lbgt    label           lbhi    label

lbhis   label           lbhs    label

lble    label           lblo    label

lblos   label           lbls    label

lblt    label           lbmi    label

lbne    label           lbpl    label

lbra    label           lbrn    label

lbvc    label           lbvs    label

lbsr    label

AS6809 ASSEMBLER                                        PAGE O-4

6809 INSTRUCTION SET

O.2.4  Single Operand Instructions

asla            aslb

asl     []

asra            asrb

asr     []

clra            clrb

clr     []

coma            comb

com     []

deca            decb

dec     []

inca            incb

inc     []

lsla            lslb

lsl     []

lsra            lsrb

lsr     []

nega            negb

neg     []

rola            rolb

rol     []

rora            rorb

ror     []

tsta            tstb

tst     []

AS6809 ASSEMBLER                                        PAGE O-5

6809 INSTRUCTION SET

O.2.5  Double Operand Instructions

adca    []              adcb    []

adda    []              addb    []

anda    []              andb    []

bita    []              bitb    []

cmpa    []              cmpb    []

eora    []              eorb    []

lda     []              ldb     []

ora     []              orb     []

sbca    []              sbcb    []

sta     []              stb     []

suba    []              subb    []

O.2.6  D-register Instructions

addd    []              subd    []

cmpd    []              ldd     []

std     []

O.2.7  Index/Stack Register Instructions

cmps    []              cmpu    []

cmpx    []              cmpy    []

lds     []              ldu     []

ldx     []              ldy     []

leas    []              leau    []

leax    []              leay    []

sts     []              stu     []

stx     []              sty     []

pshs    r               pshu    r

puls    r               pulu    r

AS6809 ASSEMBLER                                        PAGE O-6

6809 INSTRUCTION SET

O.2.8  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

O.2.9  Register - Register Instructions

exg     r1,r2           tfr     r1,r2

O.2.10  Condition Code Register Instructions

andcc   #data           orcc    #data

cwai    #data

O.2.11  6800 Compatibility Instructions

aba             cba

clc             cli

clv             des

dex             ins

inx

ldaa    []      ldab    []

oraa    []      orab    []

psha            pshb

pula            pulb

sba             sec

sei             sev

staa    []      stab    []

tab             tap

tba             tpa

tsx             txs

wai

APPENDIX P

AS6811 ASSEMBLER

P.1  68HC11 REGISTER SET

The following is a list of the 68HC11 registers used by AS6811:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x,y     -       index registers

P.2  68HC11 INSTRUCTION SET

The  following  tables list all 68HC11 mnemonics recognized

by the AS6811 assembler.  The designation []  refers  to  a  re-

quired  addressing  mode argument.  The following list specifies

the format for each addressing mode supported by AS6811:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

,x              register indirect addressing

zero offset

offset,x        register indirect addressing

0 <= offset <= 255

ext             extended addressing

label           branch label

The terms data, dir, offset, and ext may all be expressions.

AS6811 ASSEMBLER                                        PAGE P-2

68HC11 INSTRUCTION SET

Note that not all addressing modes are valid with every in-

struction, refer to the 68HC11 technical data for valid modes.

P.2.1  Inherent Instructions

aba             abx

aby             cba

clc             cli

clv             daa

des             dex

dey             fdiv

idiv            ins

inx             iny

mul             nop

rti             rts

sba             sec

sei             sev

stop            swi

tab             tap

tba             tpa

tsx             txs

wai             xgdx

xgdy

psha            pshb

psh a           psh b

pshx            pshy

psh x           psh y

pula            pulb

pul a           pul b

pulx            puly

pul x           pul y

P.2.2  Branch Instructions

bra     label           brn     label

bhi     label           bls     label

bcc     label           bhs     label

bcs     label           blo     label

bne     label           beq     label

bvc     label           bvs     label

bpl     label           bmi     label

bge     label           blt     label

bgt     label           ble     label

bsr     label

AS6811 ASSEMBLER                                        PAGE P-3

68HC11 INSTRUCTION SET

P.2.3  Single Operand Instructions

asla            aslb            asld

asl a           asl b           asl d

asl     []

asra            asrb

asr a           asr b

asr     []

clra            clrb

clr a           clr b

clr     label

coma            comb

com a           com b

com     []

deca            decb

dec a           dec b

dec     []

inca            incb

inc a           inc b

inc     []

lsla            lslb            lsld

lsl a           lsl b           lsl d

lsl     []

lsra            lsrb            lsrd

lsr a           lsr b           lsr d

lsr     []

nega            negb

neg a           neg b

neg     []

rola            rolb

rol a           rol b

rol     []

rora            rorb

ror a           ror b

ror     []

tsta            tstb

tst a           tst b

tst     []

AS6811 ASSEMBLER                                        PAGE P-4

68HC11 INSTRUCTION SET

P.2.4  Double Operand Instructions

adca    []              adcb    []

adc a   []              adc b   []

adda    []      addb    []      addd    []

add a   []      add b   []      add d   []

anda    []              andb    []

and a   []              and b   []

bita    []              bitb    []

bit a   []              bit b   []

cmpa    []              cmpb    []

cmp a   []              cmp b   []

eora    []              eorb    []

eor a   []              eor b   []

ldaa    []              ldab    []

lda a   []              lda b   []

oraa    []              orab    []

ora a   []              ora b   []

sbca    []              sbcb    []

sbc a   []              sbc b   []

staa    []              stab    []

sta a   []              sta b   []

suba    []      subb    []      subd    []

sub a   []      sub b   []      sub d   []

P.2.5  Bit Manupulation Instructions

bclr    [],#data

bset    [],#data

brclr   [],#data,label

brset   [],#data,label

AS6811 ASSEMBLER                                        PAGE P-5

68HC11 INSTRUCTION SET

P.2.6  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

P.2.7  Long Register Instructions

cpx     []              cpy     []

ldd     []              lds     []

ldx     []              ldy     []

std     []              sts     []

stx     []              sty     []

APPENDIX Q

AS6812 ASSEMBLER

Q.1  68HC12 REGISTER SET

The following is a list of the 68HC12 registers used by AS6812:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

x,y     -       index registers

sp,s    -       stack pointer

pc      -       program counter

ccr,cc  -       condition code register

Q.2  68HC12 INSTRUCTION SET

The  following  tables list all 68HC12 mnemonics recognized

by the AS6812 assembler.  The designation []  refers  to  a  re-

quired  addressing  mode argument.  The following list specifies

the format for each addressing mode supported by AS6812:

#data           immediate data

byte or word data

ext             extended addressing

pg              memory page number

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

label           branch label

r,r1,r2         registers

ccr,a,b,d,x,y,sp,pc

AS6812 ASSEMBLER                                        PAGE Q-2

68HC12 INSTRUCTION SET

-x      x-      register indexed, pre or

,-x     ,x-     post autodecrement by 1

n,-x    n,x-    register indexed, pre or

post autodecrement by 1 - 8

+x      x+      register indexed, pre or

,+x     ,x+     post autoincrement by 1

n,+x    n,x+    register indexed, pre or

post autoincrement by 1 - 8

offset,x        register indexed addressing

-16 <= offset <= 15    ---  5-bit

-256 <= offset <= -17   ---  9-bit

16 <= offset <= 255   ---  9-bit

-32768 <= offset <= -257  --- 16-bit

256 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

[offset,x]      register indexed indirect addressing

-32768 <= offset <= 32767 --- 16-bit

[,x]            register indexed indirect addressing

zero offset

a,x             accumulator offset indexed addressing

[d,x]           d accumulator offset indexed

indirect addressing

The  terms  data, dir, label, offset, and ext may all be expres-

sions.

Note that not all addressing modes are valid with every in-

struction, refer to the 68HC12 technical data for valid modes.

AS6812 ASSEMBLER                                        PAGE Q-3

68HC12 INSTRUCTION SET

Q.2.1  Inherent Instructions

aba             bgnd            cba

daa             dex             dey

ediv            edivs           emul

emuls           fdiv            idiv

idivs           inx             iny

mem             mul             nop

psha            pshb            pshc

pshd            pshx            pshy

pula            pulb            pulc

puld            pulx            puly

rev             revw            rtc

rti             rts             sba

stop            swi             tab

tba             wai             wav

wavr

Q.2.2  Short Branch Instructions

bcc     label           bcs     label

beq     label           bge     label

bgt     label           bhi     label

bhis    label           bhs     label

ble     label           blo     label

blos    label           bls     label

blt     label           bmi     label

bne     label           bpl     label

bra     label           brn     label

bvc     label           bvs     label

bsr     label

Q.2.3  Long Branch Instructions

lbcc    label           lbcs    label

lbeq    label           lbge    label

lbgt    label           lbhi    label

lbhis   label           lbhs    label

lble    label           lblo    label

lblos   label           lbls    label

lblt    label           lbmi    label

lbne    label           lbpl    label

lbra    label           lbrn    label

lbvc    label           lbvs    label

AS6812 ASSEMBLER                                        PAGE Q-4

68HC12 INSTRUCTION SET

Q.2.4  Branch on Decrement, Test, or Increment

dbeq    r,label         dbne    r,label

ibeq    r,label         ibne    r,label

tbeq    r,label         tbne    r,label

Q.2.5  Bit Clear and Set Instructions

bclr    [],#data

bset    [],#data

Q.2.6  Branch on Bit Clear or Set

brclr   [],#data,label

brset   [],#data,label

AS6812 ASSEMBLER                                        PAGE Q-5

68HC12 INSTRUCTION SET

Q.2.7  Single Operand Instructions

asla            aslb

asl     []

asra            asrb

asr     []

clra            clrb

clr     []

coma            comb

com     []

deca            decb

dec     []

inca            incb

inc     []

lsla            lslb

lsl     []

lsra            lsrb

lsr     []

nega            negb

neg     []

rola            rolb

rol     []

rora            rorb

ror     []

tsta            tstb

tst     []

AS6812 ASSEMBLER                                        PAGE Q-6

68HC12 INSTRUCTION SET

Q.2.8  Double Operand Instructions

adca    []              adcb    []

adda    []              addb    []

anda    []              andb    []

bita    []              bitb    []

cmpa    []              cmpb    []

eora    []              eorb    []

ldaa    []      <=>     lda     []

ldab    []      <=>     ldb     []

oraa    []      <=>     ora     []

orab    []      <=>     orb     []

sbca    []              sbcb    []

staa    []      <=>     sta     []

stab    []      <=>     stb     []

suba    []              subb    []

Q.2.9  Move Instructions

movb    [],[]           movw    [],[]

Q.2.10  D-register Instructions

addd    []              subd    []

cpd     []      <=>     cmpd    []

ldd     []              std     []

AS6812 ASSEMBLER                                        PAGE Q-7

68HC12 INSTRUCTION SET

Q.2.11  Index/Stack Register Instructions

cps     []      <=>     cmps    []

cpx     []      <=>     cmpx    []

cpy     []      <=>     cmpy    []

lds     []

ldx     []              ldy     []

leas    []

leax    []              leay    []

sts     []

stx     []              sty     []

Q.2.12  Jump and Jump/Call to Subroutine Instructions

call    [],pg

jmp     []              jsr     []

Q.2.13  Other Special Instructions

emacs   []

emaxd   []              emaxm   []

emind   []              eminm   []

etbl    []

maxa    []              maxm    []

mina    []              minm    []

tbl     []              trap    #data

Q.2.14  Register - Register Instructions

exg     r1,r2           sex     r1,r2

tfr     r1,r2

Q.2.15  Condition Code Register Instructions

andcc   #data           orcc    #data

AS6812 ASSEMBLER                                        PAGE Q-8

68HC12 INSTRUCTION SET

Q.2.16  M68HC11 Compatibility Mode Instructions

abx             aby             clc

cli             clv             des

ins             sec             sei

sev             tap             tpa

tsx             tsy             txs

tys             xgdx            xgdy

APPENDIX R

AS6816 ASSEMBLER

R.1  68HC16 REGISTER SET

The following is a list of the 68HC16 registers used by AS6816:

a,b     -       8-bit accumulators

d       -       16-bit accumulator <a:b>

e       -       16-bit accumulator

x,y,z   -       index registers

k       -       address extension register

s       -       stack pointer

ccr     -       condition code

R.2  68HC16 INSTRUCTION SET

The  following  tables list all 68HC16 mnemonics recognized

by the AS6816 assembler.  The designation []  refers  to  a  re-

quired  addressing  mode argument.  The following list specifies

the format for each addressing mode supported by AS6816:

#data           immediate data

byte or word data

#xo,#yo         local immediate data (mac / rmac)

label           branch label

r               register

ccr,a,b,d,e,x,y,z,s

,x              zero offset register indexed addressing

,x8

,x16

offset,x        register indexed addressing

AS6816 ASSEMBLER                                        PAGE R-2

68HC16 INSTRUCTION SET

0 <= offset <= 255   ---  8-bit

-32768 <= offset <= -1    --- 16-bit

256 <= offset <= 32767 --- 16-bit

(external definition of offset

uses 16-bit mode)

offset,x8       unsigned 8-bit offset indexed addressing

offset,x16      signed 16-bit offset indexed addressing

e,x             accumulator offset indexed addressing

ext             extended addressing

bank            64K bank number (jmp / jsr)

The  terms data, label, offset, bank, and ext may all be expres-

sions.

Note that not all addressing modes are valid with every in-

struction, refer to the 6816 technical data for valid modes.

R.2.1  Inherent Instructions

aba             abx             aby             abz

ace             aced            ade             adx

ady             adz             aex             aey

aez             bgnd            cba             daa

ediv            edivs           emul            emuls

fdiv            fmuls           idiv            ldhi

lpstop          mul             nop             psha

pshb            pshmac          pula            pulb

pulmac          rtr             rts             sba

sde             sted            swi             sxt

tab             tap             tba             tbek

tbsk            tbxk            tbyk            tbzk

tde             tdmsk           tdp             ted

tedm            tekb            tem             tmer

tmet            tmxed           tpa             tpd

tskb            tsx             tsy             tsz

txkb            txs             txy             txz

tykb            tys             tyx             tyz

tzkb            tzs             tzx             tzy

wai             xgab            xgde            xgdx

xgdy            xgdz            xgex            xgey

xgez

AS6816 ASSEMBLER                                        PAGE R-3

68HC16 INSTRUCTION SET

R.2.2  Push/Pull Multiple Register Instructions

pshm    r,...           pulm    r,...

R.2.3  Short Branch Instructions

bcc     label           bcs     label

beq     label           bge     label

bgt     label           bhi     label

bhis    label           bhs     label

ble     label           blo     label

blos    label           bls     label

blt     label           bmi     label

bne     label           bpl     label

bra     label           brn     label

bvc     label           bvs     label

bsr     label

R.2.4  Long Branch Instructions

lbcc    label           lbcs    label

lbeq    label           lbge    label

lbgt    label           lbhi    label

lbhis   label           lbhs    label

lble    label           lblo    label

lblos   label           lbls    label

lblt    label           lbmi    label

lbne    label           lbpl    label

lbra    label           lbrn    label

lbvc    label           lbvs    label

lbsr    label

R.2.5  Bit Manipulation Instructions

bclr    [],#data

bset    [],#data

brclr   [],#data,label

brset   [],#data,label

AS6816 ASSEMBLER                                        PAGE R-4

68HC16 INSTRUCTION SET

R.2.6  Single Operand Instructions

asla                    aslb

asld                    asle

aslm

asl     []              aslw    []

asra                    asrb

asrd                    asre

asrm

asr     []              asrw    []

clra                    clrb

clrd                    clre

clrm

clr     []              clrw    []

coma                    comb

comd                    come

com     []              comw    []

deca                    decb

dec     []              decw    []

inca                    incb

inc     []              incw    []

lsla                    lslb

lsld                    lsle

lslm

lsl     []              lslw    []

lsra                    lsrb

lsrd                    lsre

lsr     []              lsrw    []

nega                    negb

negd                    nege

neg     []              negw    []

rola                    rolb

rold                    role

rol     []              rolw    []

rora                    rorb

rord                    rore

ror     []              rorw    []

tsta                    tstb

tsta                    tste

tst     []              tstw    []

AS6816 ASSEMBLER                                        PAGE R-5

68HC16 INSTRUCTION SET

R.2.7  Double Operand Instructions

adca    []              adcb    []

adcd    []              adce    []

adda    []              addb    []

addd    []              adde    []

anda    []              andb    []

andd    []              ande    []

bita    []              bitb    []

cmpa    []              cmpb    []

cpd     []              cpe     []

eora    []              eorb    []

eord    []              eore    []

ldaa    []              ldab    []

ldd     []              lde     []

oraa    []              orab    []

ord     []              ore     []

sbca    []              sbcb    []

sbcd    []              sbce    []

staa    []              stab    []

std     []              ste     []

suba    []              subb    []

subd    []              sube    []

R.2.8  Index/Stack Register Instructions

cps     []              cpx     []

cpy     []              cpz     []

lds     []              ldx     []

ldy     []              ldz     []

sts     []              stx     []

sty     []              stz     []

AS6816 ASSEMBLER                                        PAGE R-6

68HC16 INSTRUCTION SET

R.2.9  Jump and Jump to Subroutine Instructions

jmp     bank,[]         jsr     bank,[]

R.2.10  Condition Code Register Instructions

andp    #data           orp     #data

R.2.11  Multiply and Accumulate Instructions

mac     #data           rmac    #data

mac     #xo,#yo         rmac    #xo,#yo

APPENDIX S

AS8051 ASSEMBLER

S.1  ACKNOWLEDGMENT

Thanks  to  John Hartman for his contribution of the AS8051

cross assembler.

John L. Hartman

jhartman at compuserve dot com

noice at noicedebugger dot com

S.2  8051 REGISTER SET

The following is a list of the 8051 registers used by AS8051:

a,b             -       8-bit accumulators

r0,r1,r2,r3     -       8-bit registers

r4,r5,r6,r7

dptr            -       data pointer

sp              -       stack pointer

pc              -       program counter

psw             -       status word

c               -       carry (bit in status word)

AS8051 ASSEMBLER                                        PAGE S-2

8051 REGISTER SET

S.3  8051 INSTRUCTION SET

The  following tables list all 8051 mnemonics recognized by

the AS8051 assembler.  The following list specifies  the  format

for each addressing mode supported by AS8051:

#data           immediate data

byte or word data

r,r1,r2         register r0,r1,r2,r3,r4,r5,r6, or r7

@r              indirect on register r0 or r1

@dptr           indirect on data pointer

@a+dptr         indirect on accumulator

plus data pointer

@a+pc           indirect on accumulator

plus program counter

addr            direct memory address

bitaddr         bit address

label           call or jump label

The terms data, addr, bitaddr, and label may all be expressions.

Note that not all addressing modes are valid with every in-

struction.  Refer to the 8051 technical data for valid modes.

S.3.1  Inherent Instructions

nop

AS8051 ASSEMBLER                                        PAGE S-3

8051 INSTRUCTION SET

S.3.2  Move Instructions

mov     a,#data         mov     a,addr

mov     a,r             mov     a,@r

mov     r,#data         mov     r,addr

mov     r,a

mov     addr,a          mov     addr,#data

mov     addr,r          mov     addr,@r

mov     addr1,addr2     mov     bitaddr,c

mov     @r,#data        mov     @r,addr

mov     @r,a

mov     c,bitaddr

mov     dptr,#data

movc    a,@a+dptr       movc    a,@a+pc

movx    a,@dptr         movx    a,@r

movx    @dptr,a         movx    @r,a

S.3.3  Single Operand Instructions

clr     a               clr     c

clr     bitaddr

cpl     a               cpl     c

cpl     bitaddr

setb    c               setb    bitaddr

da      a

rr      a               rrc     a

rl      a               rlc     a

swap    a

dec     a               dec     r

dec     @r

inc     a               inc     r

inc     dptr            inc     @r

div     ab              mul     ab

pop     addr            push    addr

AS8051 ASSEMBLER                                        PAGE S-4

8051 INSTRUCTION SET

S.3.4  Two Operand Instructions

add     a,#data         add     a,addr

add     a,r             add     a,@r

addc    a,#data         addc    a,addr

addc    a,r             addc    a,@r

subb    a,#data         subb    a,addr

subb    a,r             subb    a,@r

orl     a,#data         orl     a,addr

orl     a,r             orl     a,@r

orl     addr,a          orl     addr,#data

orl     c,bitaddr       orl     c,/bitaddr

anl     a,#data         anl     a,addr

anl     a,r             anl     a,@r

anl     addr,a          anl     addr,#data

anl     c,bitaddr       anl     c,/bitaddr

xrl     a,#data         xrl     a,addr

xrl     a,r             xrl     a,@r

xrl     addr,a          xrl     addr,#data

xrl     c,bitaddr       xrl     c,/bitaddr

xch     a,addr          xch     a,r

xch     a,@r            xchd    a,@r

S.3.5  Call and Return Instructions

acall   label           lcall   label

ret                     reti

in      data

out     data

rst     data

S.3.6  Jump Instructions

ajmp    label

cjne    a,#data,label   cjne    a,addr,label

cjne    r,#data,label   cjne    @r,#data,label

djnz    r,label         djnz    addr,label

jbc     bitadr,label

jb      bitadr,label    jnb     bitadr,label

jc      label           jnc     label

jz      label           jnz     label

jmp     @a+dptr

ljmp    label           sjmp    label

AS8051 ASSEMBLER                                        PAGE S-5

8051 INSTRUCTION SET

S.3.7  Predefined Symbols:  SFR Map

--------- 4 Bytes ----------

----    ----    ----    ----

FC                                          FF

F8                                          FB

F4                                          F7

F0      B                                   F3

EC                                          EF

E8                                          EB

E4                                          E7

E0      ACC                                 E3

DC                                          DF

D8                                          DB

D4                                          D7

D0      PSW                                 D3

CC   [  TL2     TH2                     ]   CF

C8   [  T2CON           RCAP2L  RCAP2H  ]   CB

C4                                          C7

C0                                          C3

BC                                          BF

B8      IP                                  BB

B4                                          B7

B0      P3                                  B3

AC                                          AF

A8      IE                                  AB

A4                                          A7

A0      P2                                  A3

9C                                          9F

98      SCON    SBUF                        9B

94                                          97

90      P1                                  93

8C      TH0     TH1                         8F

88      TCON    TMOD    TL0     TL1         8B

84                              PCON        87

80      P0      SP      DPL     DPH         83

[...] Indicates Resident in 8052, not 8051

AS8051 ASSEMBLER                                        PAGE S-6

8051 INSTRUCTION SET

S.3.8  Predefined Symbols:  SFR Bit Addresses

---------- 4 BITS ----------

----    ----    ----    ----

FC                                          FF

F8                                          FB

F4      B.4     B.5     B.6     B.7         F7

F0      B.0     B.1     B.2     B.3         F3

EC                                          EF

E8                                          EB

E4      ACC.4   ACC.5   ACC.6   ACC.7       E7

E0      ACC.0   ACC.1   ACC.2   ACC.3       E3

DC                                          DF

D8                                          DB

D4      PSW.4   PSW.5   PSW.6   PSW.7       D7

D0      PSW.0   PSW.1   PSW.2   PSW.3       D3

CC   [  T2CON.4 T2CON.5 T2CON.6 T2CON.7 ]   CF

C8   [  T2CON.0 T2CON.1 T2CON.2 T2CON.3 ]   CB

C4                                          C7

C0                                          C3

BC      IP.4    IP.5    IP.6    IP.7        BF

B8      IP.0    IP.1    IP.2    IP.3        BB

B4      P3.4    P3.5    P3.6    P3.7        B7

B0      P3.0    P3.1    P3.2    P3.3        B3

AC      IE.4    IE.5    EI.6    IE.7        AF

A8      IE.0    IE.1    IE.2    IE.3        AB

A4      P2.4    P2.5    P2.6    P2.7        A7

A0      P2.0    P2.1    P2.2    P2.3        A3

9C      SCON.4  SCON.5  SCON.6  SCON.7      9F

98      SCON.0  SCON.1  SCON.2  SCON.3      9B

94      P1.4    P1.5    P1.6    P1.7        97

90      P1.0    P1.1    P1.2    P1.3        93

8C      TCON.4  TCON.5  TCON.6  TCON.7      8F

88      TCON.0  TCON.1  TCON.2  TCON.3      8B

84      P0.4    P0.5    P0.6    P0.7        87

80      P0.0    P0.1    P0.2    P0.3        83

[...] Indicates Resident in 8052, not 8051

AS8051 ASSEMBLER                                        PAGE S-7

8051 INSTRUCTION SET

S.3.9  Predefined Symbols:  Control Bits

---------- 4 BITS ----------

----    ----    ----    ----

FC                                          FF

F8                                          FB

F4                                          F7

F0                                          F3

EC                                          EF

E8                                          EB

E4                                          E7

E0                                          E3

DC                                          DF

D8                                          DB

D4      RS1     F0      AC      CY          D7

D0      P               OV      RS0         D3

CC   [  TLCK    RCLK    EXF2    TF2     ]   CF

C8   [  CPRL2   CT2     TR2     EXEN2   ]   CB

C4                                          C7

C0                                          C3

BC      PS      PT2                         BF

B8      PX0     PT0     PX1     PT1         BB

B4                                          B7

B0      RXD     TXD     INT0    INT1        B3

AC      ES      ET2             EA          AF

A8      EX0     ET0     EX1     ET1         AB

A4                                          A7

A0                                          A3

9C      REN     SM2     SM1     SM0         9F

98      RI      TI      RB8     TB8         9B

94                                          97

90                                          93

8C      TR0     TF0     TR1     TF1         8F

88      IT0     IE0     IT1     IE1         8B

84                                          87

80                                          83

[...] Indicates Resident in 8052, not 8051

APPENDIX T

AS8085 ASSEMBLER

T.1  8085 REGISTER SET

The  following  is  a  list  of  the 8080/8085 registers used by

AS8085:

a,b,c,d,e,h,l   -       8-bit accumulators

m               -       memory through (hl)

sp              -       stack pointer

psw             -       status word

T.2  8085 INSTRUCTION SET

The  following  tables  list all 8080/8085 mnemonics recog-

nized by the AS8085 assembler.  The following list specifies the

format for each addressing mode supported by AS8085:

#data           immediate data

byte or word data

r,r1,r2         register or register pair

psw,a,b,c,d,e,h,l

bc,de,hl,sp,pc

m               memory address using (hl)

addr            direct memory addressing

label           call or jump label

The terms data, m, addr, and label may be expressions.

Note that not all addressing modes are valid with every in-

struction, refer to  the  8080/8085  technical  data  for  valid

modes.

AS8085 ASSEMBLER                                        PAGE T-2

8085 INSTRUCTION SET

T.2.1  Inherent Instructions

cma             cmc

daa             di

ei              hlt

nop             pchl

ral             rar

ret             rim

rrc             rlc

sim             sphl

stc             xchg

xthl

T.2.2  Register/Memory/Immediate Instructions

adc     r       adc     m       aci     #data

add     r       add     m       adi     #data

ana     r       ana     m       ani     #data

cmp     r       cmp     m       cpi     #data

ora     r       ora     m       ori     #data

sbb     r       sbb     m       sbi     #data

sub     r       sub     m       sui     #data

xra     r       xra     m       xri     #data

T.2.3  Call and Return Instructions

cc      label           rc

cm      label           rm

cnc     label           rnc

cnz     label           rnz

cp      label           rp

cpe     label           rpe

cpo     label           rpo

cz      label           rz

call    label

T.2.4  Jump Instructions

jc      label

jm      label

jnc     label

jnz     label

jp      label

jpe     label

jpo     label

jz      label

jmp     label

AS8085 ASSEMBLER                                        PAGE T-3

8085 INSTRUCTION SET

T.2.5  Input/Output/Reset Instructions

in      data

out     data

rst     data

T.2.6  Move Instructions

mov     r1,r2

mov     r,m

mov     m,r

mvi     r,#data

mvi     m,#data

T.2.7  Other Instructions

dcr     r               dcr     m

inr     r               inr     m

dad     r               dcx     r

inx     r               ldax    r

pop     r               push    r

stax    r

lda     addr            lhld    addr

shld    addr            sta     addr

lxi     r,#data

APPENDIX U

AS6500 ASSEMBLER

U.1  ACKNOWLEDGMENT

Thanks  to  Marko Makela for his contribution of the AS6500

cross assembler.

Marko Makela

Sillitie 10 A

01480 Vantaa

Finland

Internet: Marko dot Makela at Helsinki dot Fi

EARN/BitNet: msmakela at finuh

Several  additions  and modifications were made to his code

to support the following families of 6500 processors:

(1)     650X and 651X processor family

(2)     65F11 and 65F12 processor family

(3)     65C00/21 and 65C29 processor family

(4)     65C02, 65C102, and 65C112 processor family

The instruction syntax of this cross assembler contains two

peculiarities:  (1) the addressing indirection is denoted by the

square  brackets  []  and (2) the `bbrx' and `bbsx' instructions

are written `bbr0 memory,label'.

AS6500 ASSEMBLER                                        PAGE U-2

6500 REGISTER SET

U.2  6500 REGISTER SET

The following is a list of the 6500 registers used by AS6500:

a       -       8-bit accumulator

x,y     -       index registers

U.3  6500 INSTRUCTION SET

The  following tables list all 6500 family mnemonics recog-

nized by the AS6500 assembler.  The designation [] refers  to  a

required addressing mode argument.  The following list specifies

the format for each addressing mode supported by AS6500:

#data           immediate data

byte or word data

*dir            direct page addressing

(see .setdp directive)

0 <= dir <= 255

offset,x        indexed addressing

offset,y        indexed addressing

address = (offset + (x or y))

[offset,x]      pre-indexed indirect addressing

0 <= offset <= 255

address = contents of location

(offset + (x or y)) mod 256

[offset],y      post-indexed indirect addressing

address = contents of location at offset

plus the value of the y register

[address]       indirect addressing

ext             extended addressing

label           branch label

address,label   direct page memory location

branch label

bbrx and bbsx instruction addressing

The  terms data, dir, offset, address, ext, and label may all be

expressions.

Note that not all addressing modes are valid with every in-

struction, refer to the 65xx technical data for valid modes.

AS6500 ASSEMBLER                                        PAGE U-3

6500 INSTRUCTION SET

U.3.1  Processor Specific Directives

The  AS6500 cross assembler has four (4) processor specific

assembler directives which  define  the  target  65xx  processor

family:

.r6500          Core 650X and 651X family (default)

.r65f11         Core plus 65F11 and 65F12

.r65c00         Core plus 65C00/21 and 65C29

.r65c02         Core plus 65C02, 65C102, and 65C112

U.3.2  65xx Core Inherent Instructions

brk                     clc

cld                     cli

clv                     dex

dey                     inx

iny                     nop

pha                     php

pla                     plp

rti                     rts

sec                     sed

sei                     tax

tay                     tsx

txa                     txs

tya

U.3.3  65xx Core Branch Instructions

bcc     label           bhs     label

bcs     label           blo     label

beq     label           bmi     label

bne     label           bpl     label

bvc     label           bvs     label

U.3.4  65xx Core Single Operand Instructions

asl     []

dec     []

inc     []

lsr     []

rol     []

ror     []

AS6500 ASSEMBLER                                        PAGE U-4

6500 INSTRUCTION SET

U.3.5  65xx Core Double Operand Instructions

adc     []

and     []

bit     []

cmp     []

eor     []

lda     []

ora     []

sbc     []

sta     []

U.3.6  65xx Core Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

U.3.7  65xx Core Miscellaneous X and Y Register Instructions

cpx     []

cpy     []

ldx     []

stx     []

ldy     []

sty     []

AS6500 ASSEMBLER                                        PAGE U-5

6500 INSTRUCTION SET

U.3.8  65F11 and 65F12 Specific Instructions

bbr0    [],label                bbr1    [],label

bbr2    [],label                bbr3    [],label

bbr4    [],label                bbr5    [],label

bbr6    [],label                bbr7    [],label

bbs0    [],label                bbs1    [],label

bbs2    [],label                bbs3    [],label

bbs4    [],label                bbs5    [],label

bbs6    [],label                bbs7    [],label

rmb0    []                      rmb1    []

rmb2    []                      rmb3    []

rmb4    []                      rmb5    []

rmb6    []                      rmb7    []

smb0    []                      smb1    []

smb2    []                      smb3    []

smb4    []                      smb5    []

smb6    []                      smb7    []

U.3.9  65C00/21 and 65C29 Specific Instructions

bbr0    [],label                bbr1    [],label

bbr2    [],label                bbr3    [],label

bbr4    [],label                bbr5    [],label

bbr6    [],label                bbr7    [],label

bbs0    [],label                bbs1    [],label

bbs2    [],label                bbs3    [],label

bbs4    [],label                bbs5    [],label

bbs6    [],label                bbs7    [],label

bra     label

phx                             phy

plx                             ply

rmb0    []                      rmb1    []

rmb2    []                      rmb3    []

rmb4    []                      rmb5    []

rmb6    []                      rmb7    []

smb0    []                      smb1    []

smb2    []                      smb3    []

smb4    []                      smb5    []

smb6    []                      smb7    []

AS6500 ASSEMBLER                                        PAGE U-6

6500 INSTRUCTION SET

U.3.10  65C02, 65C102, and 65C112 Specific Instructions

bbr0    [],label                bbr1    [],label

bbr2    [],label                bbr3    [],label

bbr4    [],label                bbr5    [],label

bbr6    [],label                bbr7    [],label

bbs0    [],label                bbs1    [],label

bbs2    [],label                bbs3    [],label

bbs4    [],label                bbs5    [],label

bbs6    [],label                bbs7    [],label

bra     label

phx                             phy

plx                             ply

rmb0    []                      rmb1    []

rmb2    []                      rmb3    []

rmb4    []                      rmb5    []

rmb6    []                      rmb7    []

smb0    []                      smb1    []

smb2    []                      smb3    []

smb4    []                      smb5    []

smb6    []                      smb7    []

stz     []

trb     []

tsb     []

Additional addressing modes for the following core instruc-

tions are also available with the 65C02, 65C102, and 65C112 pro-

cessors.

adc     []                      and     []

cmp     []                      eor     []

lda     []                      ora     []

sbc     []                      sta     []

bit     []                      jmp     []

dec                             inc

APPENDIX V

AS740 ASSEMBLER

V.1  ACKNOWLEDGMENT

Thanks  to  Uwe  Steller  for his contribution of the AS740

cross assembler.

Uwe Stellar

Uwe dot Steller at t-online dot de

The  instruction  syntax  of  this cross assembler uses the

square brackets [] to denote addressing indirection.

V.2  740 REGISTER SET

The following is a list of the 740 registers used by AS740:

a       -       8-bit accumulator

x,y     -       index registers

V.3  740 INSTRUCTION SET

The  following  tables list all 740 family mnemonics recog-

nized by the AS740 assembler.  The designation []  refers  to  a

required addressing mode argument.  The following list specifies

the format for each addressing mode supported by AS740:

#data           immediate data byte

#data,*zp       immediate data to zero page

a               accumulator addressing

AS740 ASSEMBLER                                         PAGE V-2

740 INSTRUCTION SET

*zp             zero page addressing

(see .setdp directive)

0 &lt;= dir &lt;= 255

*zp,x           zero page x addressing

*zp,y           zero page y addressing

address = (offset + (x or y))

[*zp,x]         indirect x addressing

0 &lt;= offset &lt;= 255

address = 2 bytes at location

[(offset + (x or y)) mod 256]

[*zp],y         indirect y addressing

address = 2 byte value at offset

plus the value of the y register

abs             absolute addressing (2 byte)

abs,x           absolute x addressing (2 byte + x)

abs,y           absolute y addressing (2 byte + y)

[abs]           indirect addressing (2 byte)

label           branch label

\special        low order byte of address 0xFFnn

BIT#,*zp        bit set/clear zero page

BIT#,A          bit set/clear accumulator

BIT#,*zp,label  branch on bit set/clear in zero page

BIT#,A,label    branch on bit set/clear in accumulator

The terms data, zp, abs, BIT , special, and label may all be ex-

pressions.

Note that not all addressing modes are valid with every in-

struction, refer to the 740 technical data for valid modes.

AS740 ASSEMBLER                                         PAGE V-3

740 INSTRUCTION SET

V.3.1  Inherent Instructions

brk                     clc

cld                     cli

clt                     clv

dex                     dey

inx                     iny

nop                     pha

php                     pla

plp                     rti

rts                     sec

sed                     sei

set                     stp

tax                     tay

tsx                     txa

txs                     tya

wit

V.3.2  Branch Instructions

bcc     label           bhs     label

bcs     label           blo     label

beq     label           bmi     label

bne     label           bpl     label

bvc     label           bvs     label

bra     label

V.3.3  Single Operand Instructions

asl     []

dec     []

inc     []

lsr     []

rol     []

ror     []

AS740 ASSEMBLER                                         PAGE V-4

740 INSTRUCTION SET

V.3.4  Double Operand Instructions

adc     []

and     []

bit     []

cmp     []

eor     []

lda     []

ora     []

sbc     []

sta     []

V.3.5  Jump and Jump to Subroutine Instructions

jmp     []              jsr     []

V.3.6  Miscellaneous X and Y Register Instructions

cpx     []

cpy     []

ldx     []

stx     []

ldy     []

sty     []

V.3.7  Bit Instructions

bit     []

bbc     BIT#,[],label           bbs     BIT#,[],label

clb     BIT#,[]                 seb     BIT#,[]

V.3.8  Other Instructions

div     []                      mul     []

ldm     #imm,[]                 com     []

tst     []                      rrf     []

APPENDIX W

AS1802 ASSEMBLER

W.1  ACKNOWLEDGMENT

Thanks  to  Shujen  Chen for his contribution of the AS1802

cross assembler.

Shujen Chen

DeVry University

Tinley Park, IL

schen at tp dot devry dot edu

W.2  1802 REGISTER SET

The following is a list of the 1802 registers used by AS1802:

r0-r15  -       8-bit registers

sp      -       register r2

pc      -       register r3

call    -       register r4

return  -       register r5

argr    -       register r6

W.3  1802 INSTRUCTION SET

The  following tables list all 1802 mnemonics recognized by

the AS1802 assembler.  The designation [] refers to  a  required

addressing  mode  argument.   The  following  list specifies the

format for each addressing mode supported by AS1802:

#data           immediate data

byte or word data

expr            expression

AS1802 ASSEMBLER                                        PAGE W-2

1802 INSTRUCTION SET

Rn              register addressing

label           branch label

The terms data, expr, and label may be expressions.

Note that not all addressing modes are valid with every in-

struction, refer to the 1802 technical data for valid modes.

W.3.1  1802 Inherent Instructions

adc             add             and

dis             idl             irx

ldx             ldxa            lsdf

lsie            lskp            lsnf

lsnq            lsnz            lsq

lsz             mark            nop

or              req             ret

rshl            rshr            sav

sd              sdb             seq

shl             shlc            shr

shrc            skp             sm

smb             stxd            xor

W.3.2  1802 Short Branch Instructions

b1      label           b2      label

b3      label           b4      label

bdf     label           bge     label

bl      label           bm      label

bn1     label           bn2     label

bn3     label           bn4     label

bnf     label           bnq     label

bnz     label           bpz     label

bq      label           br      label

bz      label           nbr     label

AS1802 ASSEMBLER                                        PAGE W-3

1802 INSTRUCTION SET

W.3.3  1802 Long Branch Instructions

lbdf    label           lbnf    label

lbnq    label           lbnz    label

lbq     label           lbr     label

lbz     label           nlbr    label

W.3.4  1802 Immediate Instructions

adci    #data           adi     #data

ani     #data           ldi     #data

ori     #data           sdbi    #data

sdi     #data           smbi    #data

smi     #data           xri     #data

W.3.5  1802 Register Instructions

dec     Rn              ghi     Rn

glo     Rn              inc     Rn

lda     Rn              ldn     Rn

phi     Rn              plo     Rn

sep     Rn              sex     Rn

str     Rn

W.3.6  1802 Input and Output Instructions

inp     expr

out     expr

AS1802 ASSEMBLER                                        PAGE W-4

1802 INSTRUCTION SET

W.3.7  CDP1802 COSMAC Microprocessor Instruction Set Summary

----------------------------------------------------------------

|                                                              |

|                                                              |

|                             RCA                              |

|                                                              |

|               1     88888      000      22222                |

|              11    8     8    0   0    2     2               |

|               1    8     8   0   0 0        2                |

|               1     88888    0  0  0     222                 |

|               1    8     8   0 0   0    2                    |

|               1    8     8    0   0    2                     |

|              111    88888      000     2222222               |

|                                                              |

|    CDP1802 COSMAC Microprocessor Instruction Set Summary     |

|                                                              |

|                                                              |

|                                                              |

|                                                              |

|Written by     Jonathan Bowen                                 |

|               Programming Research Group                     |

|               Oxford University Computing Laboratory         |

|               8-11 Keble Road                                |

|               Oxford OX1 3QD                                 |

|               England                                        |

|                                                              |

|               Tel +44-865-273840                             |

|                                                              |

|Created        August 1981                                    |

|Updated        April 1985                                     |

|Issue          1.3                Copyright (C) J.P.Bowen 1985|

----------------------------------------------------------------

AS1802 ASSEMBLER                                        PAGE W-5

1802 INSTRUCTION SET

----------------------------------------------------------------

|                                                              |

|             CDP1802 COSMAC Microprocessor Pinout             |

|                                                              |

|                    _________    _________                    |

|                  _|         \__/         |_                  |

|       --> CLOCK |_|1                   40|_| Vdd             |

|            ____  _|                      |_  ____            |

|        --> WAIT |_|2                   39|_| XTAL -->        |

|           _____  _|                      |_  ______          |

|       --> CLEAR |_|3                   38|_| DMA IN <--      |

|                  _|                      |_  _______         |

|           <-- Q |_|4                   37|_| DMA OUT <--     |

|                  _|                      |_  _________       |

|         <-- SC1 |_|5                   36|_| INTERRUPT <--   |

|                  _|                      |_  ___             |

|         <-- SC0 |_|6                   35|_| MWR <--         |

|             ___  _|                      |_                  |

|         <-- MRD |_|7                   34|_| TPA -->         |

|                  _|                      |_                  |

|      <--> BUS 7 |_|8                   33|_| TPB -->         |

|                  _|                      |_                  |

|      <--> BUS 6 |_|9                   32|_| MA7 -->         |

|                  _|                      |_                  |

|      <--> BUS 5 |_|10       1802       31|_| MA6 -->         |

|                  _|                      |_                  |

|      <--> BUS 4 |_|11                  30|_| MA5 -->         |

|                  _|                      |_                  |

|      <--> BUS 3 |_|12                  29|_| MA4 -->         |

|                  _|                      |_                  |

|      <--> BUS 2 |_|13                  28|_| MA3 -->         |

|                  _|                      |_                  |

|      <--> BUS 1 |_|14                  27|_| MA2 -->         |

|                  _|                      |_                  |

|      <--> BUS 0 |_|15                  26|_| MA1 -->         |

|                  _|                      |_                  |

|             Vcc |_|16                  25|_| MA0 -->         |

|                  _|                      |_  ___             |

|          <-- N2 |_|17                  24|_| EF1 <--         |

|                  _|                      |_  ___             |

|          <-- N1 |_|18                  23|_| EF2 <--         |

|                  _|                      |_  ___             |

|          <-- N0 |_|19                  22|_| EF3 <--         |

|                  _|                      |_  ___             |

|             Vss |_|20                  21|_| EF4 <--         |

|                   |______________________|                   |

|                                                              |

|                                                              |

----------------------------------------------------------------

AS1802 ASSEMBLER                                        PAGE W-6

1802 INSTRUCTION SET

----------------------------------------------------------------

|Mnem. |Op|F|Description                 |Notes                |

|------+--+-+----------------------------+---------------------|

|ADC   |74|*|Add with Carry              |{DF,D}=mx+D+DF       |

|ADCI i|7C|*|Add with Carry Immediate    |{DF,D}=mp+D+DF,p=p+1 |

|ADD   |F4|*|Add                         |{DF,D}=mx+D          |

|ADI  i|FC|*|Add Immediate               |{DF,D}=mp+D,p=p+1    |

|AND   |F2|*|Logical AND                 |D={mx}&D             |

|ANI  i|FA|*|Logical AND Immediate       |D={mp}&D,p=p+1       |

|B1   a|34|-|Branch if EF1               |If EF1=1 BR else NBR |

|B2   a|35|-|Branch if EF2               |If EF2=1 BR else NBR |

|B3   a|36|-|Branch if EF3               |If EF3=1 BR else NBR |

|B4   a|37|-|Branch if EF4               |If EF4=1 BR else NBR |

|BDF  a|33|-|Branch if DF                |If DF=1 BR else NBR  |

|BGE  a|33|-|Branch if Greater or Equal  |See BDF              |

|BL   a|38|-|Branch if Less              |See BNF BR else NBR  |

|BM   a|38|-|Branch if Minus             |See BNF              |

|BN1  a|3C|-|Branch if Not EF1           |If EF1=0 BR else NBR |

|BN2  a|3D|-|Branch if Not EF2           |If EF2=0 BR else NBR |

|BN3  a|3E|-|Branch if Not EF3           |If EF3=0 BR else NBR |

|BN4  a|3F|-|Branch if Not EF4           |If EF4=0 BR else NBR |

|BNF  a|38|-|Branch if Not DF            |If DF=0 BR else NBR  |

|BNQ  a|39|-|Branch if Not Q             |If Q=0 BR else NBR   |

|BNZ  a|3A|-|Branch if D Not Zero        |If D=1 BR else NBR   |

|BPZ  a|33|-|Branch if Positive or Zero  |See BDF              |

|BQ   a|31|-|Branch if Q                 |If Q=1 BR else NBR   |

|BR   a|30|-|Branch                      |pl=mp                |

|BZ   a|32|-|Branch if D Zero            |If D=0 BR else NBR   |

|DEC  r|2N|-|Decrement register N        |n=n-1                |

|DIS   |71|-|Disable                     |{X,P}=mx,x=x+1,IE=0  |

|GHI  r|9N|-|Get High register N         |D=nh                 |

|GLO  r|8N|-|Get Low register N          |D=nl                 |

|IDL   |00|-|Idle (wait for DMA or int.) |Bus=m0               |

|INC  r|1N|-|Increment register N        |n=n+1                |

|INP  d|6N|-|Input (N=d+8=9-F)           |mx=Bus,D=Bus,Nlines=d|

|IRX   |60|-|Increment register X        |x=x+1                |

|LBDF a|C3|-|Long Branch if DF           |If DF=1 LBR else LNBR|

|LBNF a|C8|-|Long Branch if Not DF       |If DF=0 LBR else LNBR|

|LBNQ a|C9|-|Long Branch if Not Q        |If Q=0 LBR else LNBR |

|LBNZ a|CA|-|Long Branch if D Not Zero   |If D=1 LBR else LNBR |

|LBQ  a|C1|-|Long Branch if Q            |If Q=1 LBR else LNBR |

|LBR  a|C0|-|Long Branch                 |p=mp                 |

|LBZ  a|C2|-|Long Branch if D Zero       |If D=0 LBR else LNBR |

|LDA  r|4N|-|Load advance                |D=mn,n=n+1           |

|LDI  i|F8|-|Load Immediate              |D=mp,p=p+1           |

|LDN  r|0N|-|Load via N (except N=0)     |D=mn                 |

|LDX   |F0|-|Load via X                  |D=mx                 |

|LDXA  |72|-|Load via X and Advance      |D=mx,x=x+1           |

|LSDF  |CF|-|Long Skip if DF             |If DF=1 LSKP else NOP|

----------------------------------------------------------------

AS1802 ASSEMBLER                                        PAGE W-7

1802 INSTRUCTION SET

----------------------------------------------------------------

|Mnem. |Op|F|Description                 |Notes                |

|------+--+-+----------------------------+---------------------|

|LSIE  |CC|-|Long Skip if IE             |If IE=1 LSKP else NOP|

|LSKP  |C8|-|Long Skip                   |See NLBR             |

|LSNF  |C7|-|Long Skip if Not DF         |If DF=0 LSKP else NOP|

|LSNQ  |C5|-|Long Skip if Not Q          |If Q=0 LSKP else NOP |

|LSNZ  |C6|-|Long Skip if D Not Zero     |If D=1 LSKP else NOP |

|LSQ   |CD|-|Long Skip if Q              |If Q=1 LSKP else NOP |

|LSZ   |CE|-|Long Skip if D Zero         |If D=0 LSKP else NOP |

|MARK  |79|-|Push X,P to stack  (T={X,P})|m2={X,P},X=P,r2=r2-1 |

|NBR   |38|-|No short Branch (see SKP)   |p=p+1                |

|NLBR a|C8|-|No Long Branch (see LSKP)   |p=p+2                |

|NOP   |C4|-|No Operation                |Continue             |

|OR    |F1|*|Logical OR                  |D={mx}vD             |

|ORI  i|F9|*|Logical OR Immediate        |D={mp}vD,p=p+1       |

|OUT  d|6N|-|Output (N=d=1-7)            |Bus=mx,x=x+1,Nlines=d|

|PLO  r|AN|-|Put Low register N          |nl=D                 |

|PHI  r|BN|-|Put High register N         |nh=D                 |

|REQ   |7A|-|Reset Q                     |Q=0                  |

|RET   |70|-|Return                      |{X,P}=mx,x=x+1,IE=1  |

|RSHL  |7E|*|Ring Shift Left             |See SHLC             |

|RSHR  |76|*|Ring Shift Right            |See SHRC             |

|SAV   |78|-|Save                        |mx=T                 |

|SDB   |75|*|Subtract D with Borrow      |{DF,D}=mx-D-DF       |

|SDBI i|7D|*|Subtract D with Borrow Imm. |{DF,D}=mp-D-DF,p=p+1 |

|SD    |F5|*|Subtract D                  |{DF,D}=mx-D          |

|SDI  i|FD|*|Subtract D Immediate        |{DF,D}=mp-D,p=p+1    |

|SEP  r|DN|-|Set P                       |P=N                  |

|SEQ   |7B|-|Set Q                       |Q=1                  |

|SEX  r|EN|-|Set X                       |X=N                  |

|SHL   |FE|*|Shift Left                  |{DF,D}={DF,D,0}<-    |

|SHLC  |7E|*|Shift Left with Carry       |{DF,D}={DF,D}<-      |

|SHR   |F6|*|Shift Right                 |{D,DF}=->{0,D,DF}    |

|SHRC  |76|*|Shift Right with Carry      |{D,DF}=->{D,DF}      |

|SKP   |38|-|Short Skip                  |See NBR              |

|SMB   |77|*|Subtract Memory with Borrow |{DF,D}=D-mx-{~DF}    |

|SMBI i|7F|*|Subtract Mem with Borrow Imm|{DF,D}=D-mp-~DF,p=p+1|

|SM    |F7|*|Subtract Memory             |{DF,D}=D-mx          |

|SMI  i|FF|*|Subtract Memory Immediate   |{DF,D}=D-mp,p=p+1    |

|STR  r|5N|-|Store via N                 |mn=D                 |

|STXD  |73|-|Store via X and Decrement   |mx=D,x=x-1           |

|XOR   |F3|*|Logical Exclusive OR        |D={mx}.D             |

|XRI  i|FB|*|Logical Exclusive OR Imm.   |D={mp}.D,p=p+1       |

|      |  |-|Interrupt action            |T={X,P},P=1,X=2,IE=0 |

|------+--+-+--------------------------------------------------|

|      |??| |8-bit hexadecimal opcode                          |

|      |?N| |Opcode with register/device in low 4/3 bits       |

|      |  |-|DF flag unaffected                                |

|      |  |*|DF flag affected                                  |

----------------------------------------------------------------

AS1802 ASSEMBLER                                        PAGE W-8

1802 INSTRUCTION SET

----------------------------------------------------------------

|Arguments  |                     Notes                        |

|-----------+--------------------------------------------------|

| mn        |Register addressing                               |

| mx        |Register-indirect addressing                      |

| mp        |Immediate addressing                              |

| R( )      |Stack addressing (implied addressing)             |

|-----------+--------------------------------------------------|

| D         |Data register (accumulator, 8-bit)                |

| DF        |Data Flag (ALU carry, 1-bit)                      |

| I         |High-order instruction digit (4-bit)              |

| IE        |Interrupt Enable (1-bit)                          |

| N         |Low-order instruction digit (4-bit)               |

| P         |Designates Program Counter register (4-bit)       |

| Q         |Output flip-flop (1-bit)                          |

| R         |1 of 16 scratchpad Registers(16-bit)              |

| T         |Holds old {X,P} after interrupt (X high, 8-bit)   |

| X         |Designates Data Pointer register (4-bit)          |

|-----------+--------------------------------------------------|

| mn        |Memory byte addressed by R(N)                     |

| mp        |Memory byte addressed by R(P)                     |

| mx        |Memory byte addressed by R(X)                     |

| m?        |Memory byte addressed by R(?)                     |

| n         |Short form for R(N)                               |

| nh        |High-order byte of R(N)                           |

| nl        |Low-order byte of R(N)                            |

| p         |Short form for R(P)                               |

| pl        |Low-order byte of R(P)                            |

| r?        |Short form for R(?)                               |

| x         |Short form for R(X)                               |

|-----------+--------------------------------------------------|

| R(N)      |Register specified by N                           |

| R(P)      |Current program counter                           |

| R(X)      |Current data pointer                              |

| R(?)      |Specific register                                 |

----------------------------------------------------------------

AS1802 ASSEMBLER                                        PAGE W-9

1802 INSTRUCTION SET

----------------------------------------------------------------

|Arguments  |                     Notes                        |

|-----------+--------------------------------------------------|

| a         |Address expression                                |

| d         |Device number (1-7)                               |

| i         |Immediate expression                              |

| n         |Expression                                        |

| r         |Register (hex digit or an R followed by hex digit)|

|-----------+--------------------------------------------------|

| +         |Arithmetic addition                               |

| -         |Arithmetic subtraction                            |

| *         |Arithmetic multiplication                         |

| /         |Arithmetic division                               |

| &         |Logical AND                                       |

| ~         |Logical NOT                                       |

| v         |Logical inclusive OR                              |

| .         |Logical exclusive OR                              |

| <-        |Rotate left                                       |

| ->        |Rotate right                                      |

| { }       |Combination of operands                           |

| ?         |Hexadecimal digit (0-F)                           |

| -->       |Input pin                                         |

| <--       |Output pin                                        |

| <-->      |Input/output pin                                  |

----------------------------------------------------------------

APPENDIX X

AS61860 ASSEMBLER

X.1  ACKNOWLEDGMENT

Thanks  to  Edgar  Puehringer  for  his contribution of the

AS61860 cross assembler.

Edgar Peuhringer

edgar_pue at yahoo dot com

X.2  61860 REGISTER SET

The  SC61860  from  Sharp has 96 bytes of internal RAM which are

used as registers and hardware stack.  The last  four  bytes  of

the  internal  RAM  are  special  purpose registers (I/O, timers

...).  Here is a list of the 61860 registers:

Reg     Address         Common use

---     -------         ----------

i, j    0, 1            Length of block operations

a, b    2, 3            Accumulator

xl, xh  4, 5            Pointer for read operations

yl, yh  6, 7            Pointer for write operations

k - n   8 - 0x0b        General purpose (counters ...)

-     0x0c - 0x5b     Stack

ia      0x5c            Inport A

ib      0x5d            Inport B

fo      0x5e            Outport F

cout    0x5f            Control port

AS61860 ASSEMBLER                                       PAGE X-2

61860 REGISTER SET

Other  parts  of  the  61860 are the 16 bit program counter

(pc) and 16 bit data pointer (dp).  The ALU has a carry flag (c)

and  a  zero  flag  (z).   There is an internal register d which

can't be accessed with machine instructions.  It is filled  from

i or j when executing block operations.

In  addition  there  are  three 7 bit registers p, q, and r

which are used to address the  internal  RAM  (r  is  the  stack

pointer, p and q are used for block operations).

X.3  PROCESSOR SPECIFIC DIRECTIVES

The  AS61860 cross assembler has two (2) processor specific

assembler directives which are used for the etc mnemonic  (which

is a kind of a built-in switch/case statement):

.default        A 16 bit address (same as .dw)

.case           One byte followed by a 16 bit address

Here is an example how this should be used (cut from a lst

file)::

022B 7A 05 02 18         614         PTC     0x05,   CONT16

022F 69                  615         DTC

0230 4C 01 25            616         .CASE   0x4C,   SLOADI

0233 4D 01 2F            617         .CASE   0x4D,   SMERGI

0236 51 01 D2            618         .CASE   0x51,   QUITI

0239 53 00 CD            619         .CASE   0x53,   LLISTI

023C 56 01 D5            620         .CASE   0x56,   VERI

023F 01 D1               621         .DEFAULT        CONT9

X.4  61860 INSTRUCTION SET

The following tables list all 61860 family mnemonics recog-

nized by the AS61860 assembler.  Most of the mnemonics are  con-

verted  into  8  bit  machine instructions with no argument or a

one- or two-byte argument.  There are some exceptions for this:

Mnemonic        Description

--------        -----------

jp              2 bit instruction, 6 bit argument

cal             3 bit instruction, 13 bit argument

ptc *)          1 byte instruction, 3 byte argument

dtc *)          1 byte instruction, n bytes argument

*) Not mentioned in the CPU specification from Sharp

AS61860 ASSEMBLER                                       PAGE X-3

61860 INSTRUCTION SET

X.4.1  Load Immediate Register

LII n           (n --> I)

LIJ n

LIA n

LIB n

LIP n

LIQ n

LIDP nm

LIDL n          (DL is the low byte of DP)

LP              (One byte version of LIP)

RA              (Same as LIA 0, but only one byte)

CLRA            (synonym for RA)

X.4.2  Load Accumulator

LDP             (P --> A)

LDQ

LDR

LDM             ((P) --> A)

LDD             ((DP) --> A)

X.4.3  Store Accumulator

STP             (A --> P)

STQ

STR

STD             (A --> (DP))

X.4.4  Move Data

MVDM            ((P) --> (DP))

MVMD            ((DP) --> (P))

AS61860 ASSEMBLER                                       PAGE X-4

61860 INSTRUCTION SET

X.4.5  Exchange Data

EXAB            (A <--> B)

EXAM            (A <--> (P))

X.4.6  Stack Operations

PUSH            (R - 1 --> R, A --> (R))

POP             ((R) --> A, R + 1 --> R)

LEAVE           (0 --> (R))

X.4.7  Block Move Data

MVW             ((Q) --> (P), I+1 bytes)

MVB             ((Q) --> (P), J+1 bytes)

MVWD            ((DP) --> (P), I+1 bytes)

MVBD            ((DP) --> (P), J+1 bytes)

DATA            ((B,A) --> (P), I+1 bytes,

reads CPU ROM also)

X.4.8  Block Exchange Data

EXW             ((Q) <--> (P), I+1 bytes)

EXB             ((Q) <--> (P), J+1 bytes)

EXWD            ((DP) <--> (P), I+1 bytes)

EXBD            ((DP) <--> (P), J+1 bytes)

AS61860 ASSEMBLER                                       PAGE X-5

61860 INSTRUCTION SET

X.4.9  Increment and Decrement

INCP            (P + 1 --> P)

DECP

INCI

DECI

INCJ

DECJ

INCA

DECA

INCB

DECB

INCK

DECK

INCL

DECL

IX              (X + 1 --> X, X --> DP)

DX

IY

DY

INCM *)

DECM *)

INCN *)

DECN *)

*) Not mentioned in the CPU specification from Sharp

X.4.10  Increment/Decrement with Load/Store

IXL             (Same as IX plus LDD)

DXL

IYS             (Same as IY plus STD)

DYS

X.4.11  Fill

FILM            (A --> (P), I+1 bytes)

FILD            (A --> (DP), I+1 bytes)

AS61860 ASSEMBLER                                       PAGE X-6

61860 INSTRUCTION SET

X.4.12  Addition and Subtraction

ADIA n          (A + n --> A)

SBIA n

ADIM n          ((P) + n --> (P))

SBIM n

ADM n           ((P) + A --> (P))

SBM n

ADCM n          ((P) + A --> (P), with carry)

SBCM

ADB             (like ADM, but 16 bit)

SBB

ADN             (like ADM, BCD addition, I+1 bytes)

SBN

ADW             ((P) + (Q) --> (P), BCD, I+1 bytes)

SBW

X.4.13  Shift Operations

SRW             (shift I+1 bytes in (P) 4 bits right)

SLW

SR              (shift A 1 bit, with carry)

SL

SWP             (exchange low and high nibble of A)

X.4.14  Boolean Operations

ANIA n          (A & n --> A)

ORIA n

ANIM n          ((P) & n --> (P))

ORIM n

ANID n          ((DP) & n --> (DP))

ORID n

ANMA            ((P) & A --> (P))

ORMA

AS61860 ASSEMBLER                                       PAGE X-7

61860 INSTRUCTION SET

X.4.15  Compare

CPIA n          (A - n --> c,z)

CPIM n          ((P) - n --> c,z)

CPMA            ((P) - A --> c,z)

TSIA n          (A & n --> z)

TSIM n          ((P) & n --> z)

TSID n          ((DP) & n --> z)

TSIP            ((P) & A --> z)

X.4.16  CPU Control

SC              (Set carry)

RC

NOPW            (no op)

NOPT

WAIT n          (wait 6+n cycles)

WAITJ           (wait 5+4*I cycles)

CUP             (synonym for WAITJ)

X.4.17  Absolute Jumps

JP nm

JPZ nm          (on zero)

JPNZ nm

JPC nm

JPNC nm

PTC/DTC         (see 'Processor Specific Directives')

PTJ/DTJ         (synonym for PTD/DTC)

CPCAL/DTLRA     (synonym for PTC/DTC)

CASE1/CASE2     (synonym for PTC/DTC)

SETT/JST        (synonym for PTC/DTC)

AS61860 ASSEMBLER                                       PAGE X-8

61860 INSTRUCTION SET

X.4.18  Relative Jumps

These  operations  handle a jump relative to PC forward and

back with  a  maximum  distance  of  255  byte.   The  assembler

resolves 16 bit addresses to to 8 bit relative adresses.  If the

target address is to far away, an error will be generated.  Note

that relative jumps need 1 byte less than absolute jumps.

JRP nm

JRZP nm

JRNZP nm        (jump relative non zero plus direction)

JRCP nm

JRNCP nm

JRM nm

JRZM nm

JRNZM nm

JRCM nm         (jump relative on carry minus direction)

JRNCM nm

LOOP nm         (decrements (R) and makes a JRNCM)

X.4.19  Calls

CALL nm

CAL nm          (nm must be <= 0x1fff,

1 byte less code than CALL)

RTN

X.4.20  Input and output

INA             (IA --> A)

INB

OUTA

OUTB

OUTF            (A --> FO)

OUTC            (control port)

TEST n          (timers, pins & n --> z)

AS61860 ASSEMBLER                                       PAGE X-9

61860 INSTRUCTION SET

X.4.21  Unknown Commands

READ            ((PC+1) -> A)

READM           ((PC+1) -> (P))

WRIT            (???)