----------------------------------------------------------------------------

The following lists the keyboard keystrokes and the scan code/character code

combinations returned through Int 16h (values in hex):

Shift               Ctrl                Alt

+-----------------+-------------------+-------------------+------------------+

Esc   01-1B        Esc   01-1B         Esc   01-1B (ESC)   Esc   -- [01-00]

1     02-31        !     02-21         1     --            1     78-00

2     03-32        @     03-40         2     03-00 (NUL)   2     79-00

3     04-33        #     04-23         3     --            3     7A-00

4     05-34        $     05-24         4     --            4     7B-00

5     06-35        %     06-25         5     --            5     7C-00

6     07-36        ^     07-5E         6     07-1E (RS )   6     7D-00

7     08-37        &     08-26         7     --            7     7E-00

8     09-38        *     09-2A         8     --            8     7F-00

9     0A-39        (     0A-28         9     --            9     80-00

0     0B-30        )     0B-29         0     --            0     81-00

-     0C-2D        _     0C-5F         -     0C-1F (US )   -     82-00

=     0D-3D        +     0D-2B         =     --            =     83-00

Bsp   0E-08        Bsp   0E-08         Bsp   0E-7F (DEL)   Bsp   -- [0E-00]

Tab   0F-09        Tab   0F-09         Tab   -- [94-00]    Tab   -- [A5-00]

q     10-71        Q     10-51         q     10-11 (DC1)   q     10-00

w     11-77        W     11-57         w     11-17 (ETB)   w     11-00

e     12-65        E     12-45         e     12-05 (ENQ)   e     12-00

r     13-72        R     13-52         r     13-12 (DC2)   r     13-00

t     14-74        T     14-54         t     14-14 (DC4)   t     14-00

y     15-79        Y     15-59         y     15-19 (EM )   y     15-00

u     16-75        U     16-55         u     16-15 (NAK)   u     16-00

i     17-69        I     17-49         i     17-09 (HT )   i     17-00

o     18-6F        O     18-4F         o     18-0F (SI )   o     18-00

p     19-70        P     19-50         p     19-10 (DLE)   p     19-00

[     1A-5B        {     1A-7B         [     1A-1B (ESC)   [     -- [1A-00]

]     1B-5D        }     1B-7D         ]     1B-1D (GS )   ]     -- [1B-00]

Rtn   1C-0D        Rtn   1C-0D         Rtn   1C-0A (LF )   Rtn   -- [1C-00]

Ctl   **           Ctl   **            Ctl   **            Ctl   **

a     1E-61        A     1E-41         a     1E-01 (SOH)   a     1E-00

s     1F-73        S     1F-53         s     1F-13 (DC3)   s     1F-00

d     20-64        D     20-44         d     20-04 (EOT)   d     20-00

f     21-66        F     21-46         f     21-06 (ACK)   f     21-00

g     22-67        G     22-47         g     22-07 (BEL)   g     22-00

h     23-68        H     23-48         h     23-08 (BS )   h     23-00

j     24-6A        J     24-4A         j     24-0A (LF )   j     24-00

k     25-6B        K     25-4B         k     25-0B (VT )   k     25-00

l     26-6C        L     26-4C         l     26-0C (FF )   l     26-00

;     27-3B        :     27-3A         ;     --            ;     -- [27-00]

'     28-27        "     28-22         '     --            '     -- [28-00]

`     29-60        ~     29-7E         `     --            `     -- [29-00]

Sft   **           Sft   **            Sft   **            Sft   **

\     2B-5C        |     2B-7C         \     2B-1C (FS )   \     2B-00

z     2C-7A        Z     2C-5A         z     2C-1A (SUB)   z     2C-00

x     2D-78        X     2D-58         x     2D-18 (CAN)   x     2D-00

c     2E-63        C     2E-43         c     2E-03 (ETX)   c     2E-00

v     2F-76        V     2F-56         v     2F-16 (SYN)   v     2F-00

b     30-62        B     30-42         b     30-02 (STX)   b     30-00

n     31-6E        N     31-4E         n     31-0E (SO )   n     31-00

m     32-6D        M     32-4D         m     32-0D (CR )   m     32-00

,     33-2C        <     33-3C         ,     --            ,     -- [33-00]

.     34-2E        >     34-3E         .     --            .     -- [34-00]

/     35-2F        ?     35-3F         /     --            /     -- [35-00]

*     37-2A        *     37-2A         *     -- [96-00]    *     -- [37-00]

Alt   **           Alt   **            Alt   **            Alt   **

Sp    39-20        Sp    39-20         Sp    39-20         Sp    39-00

Cap   **           Cap   **            Cap   **            Cap   **

Shift               Ctrl                Alt

+-----------------+-------------------+-------------------+------------------+

F1    3B-00        F1    54-00         F1    5E-00         F1    68-00

F2    3C-00        F2    55-00         F2    5F-00         F2    69-00

F3    3D-00        F3    56-00         F3    60-00         F3    6A-00

F4    3E-00        F4    57-00         F4    61-00         F4    6B-00

F5    3F-00        F5    58-00         F5    62-00         F5    6C-00

F6    40-00        F6    59-00         F6    63-00         F6    6D-00

F7    41-00        F7    5A-00         F7    64-00         F7    6E-00

F8    42-00        F8    5B-00         F8    65-00         F8    6F-00

F9    43-00        F9    5C-00         F9    66-00         F9    70-00

F10   44-00        F10   5D-00         F10   67-00         F10   71-00

F11   85-00        F11   87-00         F11   -- [89-00]    F11   -- [8B-00]

F12   86-00        F12   88-00         F12   -- [8A-00]    F12   -- [8C-00]

Num   **           Num   **            Num   **            Num   **

Scr   **           Scr   **            Brk   00-00         Scr   **

Hom   47-00        Hom   47-37         Hom   77-00         Hom    ###

Up    48-00        Up    48-38         Up    -- [8D-E0]    Up     ###

PgU   49-00        PgU   49-39         PgU   84-00 [E0]    PgU    ###

-     4A-2D        -     4A-2D         -     -- [8E-00]    -     -- [4A-00]

Lft   4B-00        Lft   4B-34         Lft   73-00         Lft    ###

Cen   -- [4C-00]   Cen   4C-35         Cen   -- [8F-00]    Cen    ###

Rgt   4D-00        Rgt   4D-36         Rgt   74-00         Rgt    ###

+     4E-2B        +     4E-2B         +     -- [90-00]    +     -- [4E-00]

End   4F-00        End   4F-31         End   75-00         End    ###

Dwn   50-00        Dwn   50-32         Dwn   -- [91-00]    Dwn    ###

PgD   51-00        PgD   51-33         PgD   76-00         PgD    ###

Ins   52-00        Ins   52-30         Ins   -- [92-00]    Ins    ###

Del   53-00        Del   53-2E         Del   -- [93-00]    Del   --

SyR   **           SyR   **            SyR   **            SyR   **

K45   56-5C        K45   56-7C         K45   --            K45   --

Ent   -- [E0-0D]   Ent   -- [E0-0D]    Ent   -- [E0-0A]    Ent   -- [A6-00]

/     -- [E0-2F]   /     -- [E0-2F]    /     -- [95-00]    /     -- [A4-00]

PSc   **           PSc   **            PSc   72-00         PSc   **

Pse   **           Pse   **            Pse   **            Pse   **

Hom   47-00        Hom   47-00 [E0]    Hom   77-00 [E0]    Hom   -- [97-00]

Up    48-00        Up    48-00 [E0]    Up    -- [8D-E0]    Up    -- [98-00]

PgU   49-00        PgU   49-00 [E0]    PgU   84-00 [E0]    PgU   -- [99-00]

Lft   4B-00        Lft   4B-00 [E0]    Lft   73-00 [E0]    Lft   -- [9B-00]

Rgt   4D-00        Rgt   4D-00 [E0]    Rgt   74-00 [E0]    Rgt   -- [9D-00]

End   4F-00        End   4F-00 [E0]    End   75-00 [E0]    End   -- [9F-00]

Dwn   50-00        Dwn   50-00 [E0]    Dwn   -- [91-E0]    Dwn   -- [A0-00]

PgD   51-00        PgD   51-00 [E0]    PgD   76-00 [E0]    PgD   -- [A1-00]

Ins   52-00        Ins   52-00 [E0]    Ins   -- [92-E0]    Ins   -- [A2-00]

Del   53-00        Del   53-00 [E0]    Del   -- [93-E0]    Del   -- [A3-00]

Double starred entries perform an action, but do not provide a keystroke

in Int 16h.

Double dashed entries are ignored.

The bracketed numbers apply to the 101/102 Extended-Function keyboard,

but not the 101/102 or the 83/84 standard keyboards.

All keys are 'make/break', except the Pause key.  The make scan code of

a key is sent to the keyboard controller when the key is pressed, the

break when it is released.  In addition, all keys except the pause

are 'typematic', sending repeats if held down at a setable rate.

PS/2 keyboards can accepts commands from the system (0EDH to 0FFH)

which selects keyboard scan translation, typematic rate/delay,

combinations of typematic/make/break for any key, requests its ID, or

resets the keyboard.  For details, see information under Ports 60H-64H.

The pause key causes the keyboard interrupt routine to loop, waiting

for any other key to be pressed.  This suspends further processing of

the current program, such as a listing or printing.  The key stroke

used to resume operation is discarded.

When a system request (Alt-PrtSc) key is pressed, 8500H is placed in

AX by the keyboard interrupt routine, and an Int 15h is executed.

Release of system request produces a 8501H in AX.  This provides a

way to interrupt a program from the keyboard.  All registers must

be saved except AX by the Int 15h AH=85h service routine.

The keyboard routine suppresses the typematic action of the shift keys

and Num Lock, Caps Lock, Scroll Lock, and Ins.

During each Int 9 from the keyboard, an Int 15h, AH=4Fh is produced

after the scan code is read from the keyboard controller.  The

scan code is passed in AL with the carry flag set.  This allows

programs to intercept each scan code prior to its being handled

by the Int 9h routine.  If the carry flag is cleared on return

from Int 15h, the scan code will be ignored by the handler.