------------------------------------------------------------------

RX50 Diskette Driver for the IBM PC-AT

------------------------------------------------------------------

8 December 1984

RX50DRVR.SYS is an MS-DOS loadable device driver which enables an

IBM PC-AT to read and write DEC RX50 diskettes on its high

capacity diskette drive.  To use the driver, copy it into the

root directory of your system disk and include the following line

in your CONFIG.SYS file:    DEVICE=RX50DRVR.SYS

When the system is booted, the driver is loaded and it displays

its name, version number, load address and the drive letter to be

used to refer to RX50 diskettes.  This is normally D: but may be

different if you have a second hard disk drive or other block

devices installed.

Once the driver is installed, all the normal disk commands can be

used EXCEPT the FORMAT command.  RX50 media cannot be formatted

by the IBM controller, and attempting to do so will make the RX50

diskette unusable.  Needless to say, references to drives A: and

D: should not be intermixed:  no good will come of it.

The DOS direct disk read and write interrupts (25h and 26h) may

also be used.  Block numbers 0 through 19 are mapped to tracks 0

and 1 and the sectors are addressed in physical sequence.  Block

numbers 20 through 799 (which cover the area used by MS-DOS) are

mapped to tracks 2 through 79.  On each track the sectors are

interleaved 2:1 such that the logical sector sequence of 0..9 is

mapped to the physical sequence 1, 3, 5, 7, 9, 2, 4, 6, 8, 10.

This is consistent with the behaviour of MS-DOS on the DECmate

and the Rainbow.

RX50INIT.COM is a program which initializes an RX50 for use with

MS-DOS.  It writes a FAT and directory in the format used by the

DECmate and Rainbow versions of MS-DOS.  The command format is:

RX50INIT  [d:]  [volume-name]

The drive letter d: is optional; D: is the default value.  The

volume name is optional; if it is omitted, no volume entry is

made in the directory.  If a volume name is given, it may contain

only letters, numerals, and hyphens.  Letters are converted to

upper case.

NOTE:  After initializing a diskette, remove it from the drive and

then reinsert it before using it.  This ensures that DOS recognizes

it as a "new" diskette.

RX50DRVR.SYS and RX50INIT.COM are copyrighted by Robert F. Morse.

Permission is granted for private non-commerical use only.

Questions, comments and suggestions may be sent to:

Robert F. Morse

17 Bowdoin Street

Cambridge, MA  02138

(617) 874-4250

DTN 223-6720

MLO5-2/B6

LEWS2::MORSE