/Geotronics/sys		operating system modifications

The *.e files in this directory and its subdirectories are ed scripts

for the corresponding /usr/sys files.  These are the minimum changes

we felt we could get by with:

1)  Support for MOS parity memory (M7850-style controllers).  This is

compatible with systems that have non-parity memory installed.

2)  DC3/DC1 "stall" support for terminals (needed especially for VT100).

3)  Full support for the DZ11 async line multiplexer, including dialup.

The "getty" and "stty" mods in ../s1 and ../s2 are also required.

Considerable effort has been made to support all DZ configurations.

4)  Support for FP11 floating-point in m40.s (11/34).

There is a related patch for "cdb" in ../s1 also.

5)  Support for Emulex SC11/A2 controller with 80Mb disk.

We use 57000-block "rpa" & "rpb", with "rpc" swap and "rpd" scratch.

This patch is compatible with standard RP11 controllers too.

6)  Simplify and improve line printer driver.

7)  Various minor bug fixes (not a complete set, by any means).

"mkunix" contains the commands we run to rebuild our system.