1

/*** LPTQUE.PAL, APRIL 14, 1970*****

/

/

/

/	PETER LEMKIN

/	NATIONAL INSTITUTES OF HEALTH

/	BETHESDA, MD. 20014

/

/

/INTRODUCTION

/------------

/"LPTQUE" IS A PDP8 UTILITY PROGRAM WHICH IS USED TO BUFFER ASCII

/INPUT FROM A PT08 SERIAL TO PARALLEL INTERFACE ONTO AN A.B. DICK

/940 (USING THE ECLECTIC COMPUTER COMPANY'S INTERFACE) LINEPRINTER.

/THE  ASR33 TELETYPE CAN BE USED TO SEND COMMANDS TO A COMPUTER

/ON THE OTHER END OF THE PT08 ( WHICH IN TURN IS SENDING INFORMATION TO

/THE PDP8 TO BE PRINTED ON THE LINEPRINTER.).

/I.E. PDP8'S ASR33 TTY ==> PT08 ==>PDP10

/    PDP8'S 940 LPT <=== PT08 <== PDP10.

/

/

/MINIMUM HARDWARE:

/------------------

/PDP8 WITH PT08, 4K MEMORY, AND A.B. DICK 940 LINEPRINTER

/WITH ECLECTIC COMPUTER COMPANY INTERFACE TO PDP8. TO RUN "LPTQUE"

/IN ANOTHER FIELD, SET DATFLD=10 TIMES THE DESIRED FIELD. IE.E 00, 10

/ETC. THEN REASSEMBLE "LPTQUE". "LPTQUE" WAS ASSEMBLED USING PAL10 ON A

/PDP10.

/

/

/CORE REQUIREMENTS AND STARTING ADDRESS:

/--------------------------------------

/	<200,577> PROGRAM

/	<600,4577>  LPT BUFFER.

/	STARTING ADDRESS IS 200

/

/

/

/ERROR CONDITION:

/---------------

/NOTE: ALTHOUGH THE 940 LINEPRINTER CAN PRINT AT 2400 BAUD, ITS

/EFFECTIVE DATA RATE IS A FUNCTION OF LENGTH OF THE LINE PRINTED.

/FOR SHORT LINES, THIS CAN GO BELOW 1200 BAUD. THEREFORE IF YOU

/ARE USING A 1200 BAUD LINE (AS WE ARE), OCCASIONALLY YOU WILL OVERFLOW

/THE LPT BUFFER DUE TO PTHE FACT THAT YOU ARE ACCUMULATING DATA FASTER THAN

/YOU CAN PRINT IT. WHEN THIS HAPPENS, THE PROGRAM HALTS. TO RESTART IT

/DO A LOAD ADDRESS-START 200.

/NOTE: THE NUMBER OF LINES IN THE BUFFER QUEUE WAITING TO BE PRINTED

/ARE DISPLAYED IN THE MQ REGISTER. IF YOU DO NOT HAVE EAE, THIS IS A NOP.

/

/

/PROBLEMS:

/-------

/TABS AND FORM FEEDS ARE NOT IMPLEMENTED.

/

2

/

/

/

/

/

/

/PT08 OP CODES

6411	PTSF=6411

6401	PKSF=6401

6416	PTLS=6416

6406	PKRB=6406

6412	PTCF=6412

/AB DICK 940 LINEPRINTER OP CODES

6656	LTLS=6656

6651	LTSF=6651

6652	LTCF=6652

/LPT BUFFER MIN AND MAX POINTERS: MAY BE CHANGED BEFORE ASSEMBLE.

0601	TQMIN=601

4577	TQMAX=4577

/FIELD DEFINITION - I.E. RUNNING FIELDS.

0000	DATFLD=00

0000	INSTFD=DATFLD

/

/

/

/

/

0000		FIELD INSTFD

0200		*200

/MONITOR LOOP

0200	7200		CLA /DISABLE THE EMB

0201	1377		TAD (-62 /SET LINE COUNTER

0202	3350		DCA LINCNT

0203	3347		DCA EMB

0204	3346		DCA BUFCTR /ZERO THE BUFFER COUNTER

0205	6656		LTLS /START IT UP

0206	4776'	START,	JMS INITQU /INIT QUEUE

0207	4775'		JMS INITBF

0210	6031	INPUT,	KSF /TTY? IF SO TYPE IT AND SEND TO PDP10.

0211	5215		JMP .+4 /NO

0212	6036		KRB

0213	6046		TLS

0214	6416		PTLS /SEND TO PDP10 SO AND TELL IT OFF.

0215	1374		TAD (-215 /IS IT CR?

0216	7640		SZA CLA

0217	5224		JMP .+5 /NO

0220	1373		TAD (212 /PRINT A LINE FEED

0221	6041		TSF

0222	5221		JMP .-1

0223	6046		TLS

0224	7200		CLA /PUT THE # OF BUFFERS IN THE MQ

0225	1346		TAD BUFCTR

7421		MQL=7421

0226	7421		MQL

0227	6401		PKSF /INPUT?

3

0230	7410		SKP

0231	5234		JMP .+3

0232	4314		JMS OUTPUT /NO, CHECK OUTPUT

0233	5210		JMP INPUT

0234	6406		PKRB

0235	0372		AND (177

0236	1371		TAD (200

0237	3351		DCA LPTCHR

0240	2770'		ISZ N /INCREMENT THE CHARACTER COUNTER.

/TEST IT  TO SEE IF FORM FEED.

0241	1351		TAD LPTCHR

0242	1367		TAD (-214 /FF

0243	7640		SZA CLA

0244	5272		JMP NOFF /NO

0245	1373	LOOP1,	TAD (212 /YES, PUT THE REQUIRED # LF TO

/FINISH UP THE PAGE.

0246	4766'		JMS INSRQ

0247	7402		HLT

0250	4765'		JMS CLOSBF /PUT ON QUEUE, AND CLOSE BUFFER..

0251	4775'		JMS INITBF /FIX UP BUFFER

0252	2350		ISZ LINCNT /DONE?

0253	5245		JMP LOOP1 /NO

/

/NOW GO 20 20 SPACES TO NEXT PAGE.

0254	7200	LOOP2,	CLA

0255	1364		TAD (-20

0256	3350		DCA LINCNT

0257	1373	LOOP3,	TAD (212

0260	4766'		JMS INSRQ

0261	7402		HLT

0262	4314		JMS OUTPUT /TRY OUTPUT

0263	4765'		JMS CLOSBF

0264	4775'		JMS INITBF

0265	2350		ISZ LINCNT

0266	5257		JMP LOOP3

0267	1377		TAD (-62 /RESET LINE PER PAGE COUNTER

0270	3350		DCA LINCNT

0271	5210		JMP INPUT

/

0272	1351	NOFF,	TAD LPTCHR

0273	4766'		JMS INSRQ /PUT ON QUEUE

0274	7402		HLT

0275	1351		TAD LPTCHR

0276	1363		TAD (-212 /IS IT A LF?

0277	7640		SZA CLA

0300	7410		SKP

0301	5304		JMP .+3

0302	4314		JMS OUTPUT /NO, GO CHECK THE OUTPUT

0303	5210		JMP INPUT

/IT WAS A LINEFEED, BOP THE LINE COUNTER

0304	2350		ISZ LINCNT

0305	5310		JMP NORST /DON'T RESET IT

0306	1377		TAD (-62 /RESET IT

0307	3350		DCA LINCNT

0310	4765'	NORST,	JMS CLOSBF /YES, GO CLOSE THE BUFFER

4

0311	4775'		JMS INITBF /OPEN THE NEXT BUFFER

0312	4314		JMS OUTPUT /GO CHECK THE OUTPUT.

0313	5210		JMP INPUT

/

/

/

/

/

/

0314	0000	OUTPUT,	0

0315	6651		LTSF /LINE PRINTER READY?

0316	5714		JMP  I OUTPUT /NOT YET

0317	7200		CLA

0320	1347		TAD EMB

0321	7640		SZA CLA

0322	5334		JMP GETOUT /ENABLED, GET THE OUTPUT

0323	7200		CLA /TEST IF ANY BUFFERS

0324	1346		TAD BUFCTR /THE NUMBER OF ACTIVE BUFFERS

0325	7650		SNA CLA

0326	5714		JMP  I OUTPUT /NO BUFFERS

0327	7240		CLA CMA

0330	1346		TAD BUFCTR /SET BUFCTR=BUFCTR-1

0331	3346		DCA BUFCTR

0332	7240		CLA CMA

0333	3347		DCA EMB /YES, SET BUFFER FLAG = -1

0334	4762'	GETOUT,	JMS TTYFQ /GET THE CHAR.

0335	5714		JMP  I OUTPUT /NEVER SHOULD GET HERE.

0336	7500		SMA /SEE IF THE CHAR IS 7777, END OF BUFFER.

0337	5343		JMP PUTOUT /NO, GO OUTPUT IT.

0340	7200		CLA

0341	3347		DCA EMB /YES, CLEAR THE ENABLE BUFFER FLAG AND LEAVE

0342	5714		JMP  I OUTPUT

0343	6656	PUTOUT,	LTLS /OUTPUT IT

0344	7200		CLA

0345	5714		JMP  I OUTPUT

0346	0000	BUFCTR,	0

0347	0000	EMB,	0

0350	0000	LINCNT,	0

0351	0000	LPTCHR,	0

/

/

/

0362	0514

0363	7566

0364	7760

0365	0400

0366	0466

0367	7564

0370	0456

0371	0200

0372	0177

0373	0212

0374	7563

0375	0432

0376	0457

5

0377	7716

0400		PAGE

/SUBROUTINE CLOSBF

/PUTS AN EM, N/4+19 NULLS, 7777 AT THE END OF THE BUFFER.

/CALLING SEQUENCE:

/	JMS CLOSBF

/	NORMAL RETURN

0400	0000	CLOSBF,	0

0401	7200		CLA

0402	1377		TAD (31 /EM

0403	4266		JMS INSRQ

0404	7402		HLT /BUFFER OVERFLOW

0405	4776'		JMS OUTPUT /TRY TO DO OUTPUT

0406	1256		TAD N /COMPUTE THE N/4 # OF NULLS

0407	7110		CLL RAR

0410	7110		CLL RAR /DIVIDE BY 4

0411	1375		TAD (23 /19

0412	7041		CIA

0413	3231		DCA IBMM

0414	3256		DCA N /ZERO THE CHARACTER COUNTER.

0415	4266		JMS INSRQ

0416	7402		HLT

0417	4776'		JMS OUTPUT /TRY TO DO OUTPUT

0420	2231		ISZ IBMM

0421	5215		JMP .-4

0422	7240		CLA CMA /PUT EOF AT END OF BUFFER.

0423	4266		JMS INSRQ

0424	7402		HLT

0425	4776'		JMS OUTPUT /TRY TO DO OUTPUT

0426	2774'		ISZ BUFCTR /INCREMENT THE BUFFER COUNTER.

0427	5600		JMP I CLOSBF /RETURN

0430	7402		HLT /OVERFLOW

0431	0000	IBMM,	0

/

/SUBROUTINE INITBF

/PUTS A SYN,SOH,8 NULLS IN FRONT OF THE BUFFER.

/CALLING SEQUENCE:

/	JMS INITBF

/	NORMAL RETURN

0432	0000	INITBF,	0 /INIT THE BUFFER.

0433	7200		CLA

0434	3256		DCA N /ZERO THE WORKING CHAR COUNTER.

0435	1373		TAD (26 /SYN

0436	4266		JMS INSRQ

0437	7402		HLT /OVERFLOW

0440	4776'		JMS OUTPUT /TRY OUTPUT

0441	1372		TAD (1 /SOH

0442	4266		JMS INSRQ

0443	7402		HLT /OVERFLOW

0444	4776'		JMS OUTPUT

0445	1371		TAD (-10 /8

0446	3255		DCA IBM

0447	4266		JMS INSRQ

0450	7402		HLT

0451	4776'		JMS OUTPUT /TRY OUTPUT

6

0452	2255		ISZ IBM

0453	5247		JMP .-4 /PUT 8 NULLS IN BUFFER

0454	5632		JMP I INITBF /RETURN

0455	0000	IBM,	0

0456	0000	N,	0 /WORKING CHARACTER COUNTER FOR USE IN COMPUTING NULLS AT EOB.

/

/

/

/

/

/

/

/SUBROUTINE INITQU

/IS USED TO INIT THE QUEUE

/RESETS THE QUEUE.

/CALLING SEQUENCE:

/	JMS INITQU

/	NORMAL RETURN

0457	0000	INITQU,	0

0460	7200		CLA

0461	1370		TAD (TQMIN

0462	3345		DCA KFQ

0463	1345		TAD KFQ

0464	3344		DCA KRQ

0465	5657		JMP I INITQU

/

/

/

/

/

/

/

/

/

/SUBROUTINE INSRQ

/INSERTS A DATA WORD IN THE

/AC--> THE TTY OUTPUT BUFFER

/AT THE REAR OF THE QUEUE.

/CALLING SEQUENCE1G

/    JMS INSRQ /DATA IN AC

/    ERROR RETURN /BUFFER OVERFLOW

/     NORMAL RETURN /DATA ENTERED

/                  /RQ POINTER ADVANCED.

/

0466	0000	INSRQ,	 0

0467	3346	    DCA TQDAT /SAVE DATA

/A.1 COMPUTE NEW KRQ

0470	1342	    TAD ITQMAX

0471	7041	    CIA

0472	1344	    TAD KRQ

0473	7700	    SMA CLA

0474	5277	    JMP .+3

0475	2344	    ISZ KRQ /KRQ<--KRQ+1

0476	5301	    JMP .+3

0477	1343	    TAD ITQMIN /KRQ<--TQMIN

7

0500	3344	    DCA KRQ

/A.2 TEST FOR OVERFLOW

0501	1344	    TAD KRQ

0502	7041	    CIA

0503	1345	    TAD KFQ

0504	7650	    SNA CLA

0505	5666	    JMP I INSRQ /ERROR RETURN

0506	2266	    ISZ INSRQ

/A.3 INSERT DATA INTO THE REAR OF THE QUEUE.

0507	1346	    TAD TQDAT

0510	6201	    CDF DATFLD

0511	3744	    DCA I KRQ

0512	6201	    CDF INSTFD

0513	5666	    JMP I INSRQ /RETURN

/.........................................

/

/SUBROUTINE TTYFQ

/GET A DATA WORD FROM THE REAR

/OF THE TTY QUEUE-->AC.

/CALLING SEQUENCE:

/    JMS TTYFQ /DATA-->AC

/    ERROR RETURN /BUFFER EMPTY

/    NORMAL RETURN /DATA IN AC.

0514	0000	TTYFQ,	 0

/A.1 TEST IF KFQ=KRQ

/IF SO BUFFER EMPTY

0515	7200	    CLA

0516	1345	    TAD KFQ

0517	7041	    CIA

0520	1344	    TAD KRQ

0521	7650	    SNA CLA

0522	5714	    JMP I TTYFQ /ERROR RETURN

0523	2314	    ISZ TTYFQ /SET UP NORMAL RETURN.

/A.2 ADVANCE THE KFQ POINTER

0524	7200	    CLA

0525	1342	    TAD ITQMAX

0526	7041	    CIA

0527	1345	    TAD KFQ

0530	7700	    SMA CLA

0531	5334	    JMP .+3 /YES, SET KFQ=TQMIN

0532	2345	    ISZ KFQ /NO, SET KFQ=KFQ+1

0533	5336	    JMP .+3

0534	1343	    TAD ITQMIN

0535	3345	    DCA KFQ

/A.3 GET DATA FROM THE QUEUE.

0536	6201	    CDF DATFLD

0537	1745	    TAD I KFQ

0540	6201	    CDF INSTFD

0541	5714	    JMP I TTYFQ /RETURN

/...CONSTANTS...

0542	4577	ITQMAX,	 TQMAX

0543	0601	ITQMIN,	 TQMIN

0544	0601	KRQ,	 TQMIN

0545	0601	KFQ,	 TQMIN

8

0546	0000	TQDAT,	 0

/..........................................

/P. LEMKIN, APRIL 14, 1970...

$

0570	0601

0571	7770

0572	0001

0573	0026

0574	0346

0575	0023

0576	0314

0577	0031