XMODEM47.DOC

(revised 10/20/81)

XMODEM47 is based on the last known reliable working

copy of xmodem, XMODEM43.  XMODEM47 incorporates John

Mahr's modifications for CRC (cyclic redundancy check)

verification of the transmitted data.  Versions 44 and

45 were known to have incorrect coding modifications

made to them, and XMODEM46 was an experimental copy

which never should have been released.  Its operation

for all conditionals and options was never verified.

Below is some modification history of XMODEM.

******************************************************

[John Mahr's XMODMX41.ASM as modified by Keith Peter-

sen.  The modifications, according to Keith, are

relatively minor.  But the difference between this

and XMODEM41.ASM is fundamental and parallels the

difference between MODEM527 and its successor,

MODEM206 -- XMODEM42 and MODEM206 both use the new

extended version of the Christensen protocol, and

have now been worked into relatively final form.]

This program is an extension of Keith Petersen's

XMODEM program.  It adds checking for framing errors,

overruns, and parity errors.  These checks will only

be made if a PMMI modem is being used.  These checks

were added because sometimes the checksum does not

catch all transmission errors.  Hopefully, by checking

for framing errors in particular, all transmission

errors will be caught.  This program does not get rid

of the checksum, but rather adds the framing error,

overrun, and parity error check to that function.  The

parity error check is only meaningful if parity is being

used.  If parity is not being used, the check will still

be made, but no parity errors will ever be found.

Whenever a framing error and/or overrun is detected on a

file receive, the following takes place:

1. Purge the line (receive rest of sector and throw

it away).

2. If error limit has not been exceeded, send a

NAK so that the sender will resend the sector.

3. If error limit has been exceeded, abort the

transmission.

The capability to display the status of the PMMI on a front

panel has also been added.  The default for the front panel

is port FFh.  If you do not have a front panel, then set

the FRNTPNL EQU to FALSE.  If you have a front panel at a

port other than FFh, then change the PANEL EQU to your port

address.  The status is only displayed when data is being

by the PMMI modem.  See your PMMI manual for the meaning of

the PMMI status bits.

- bit 0 is transmit buffer empty

- bit 1 is input data is available

- bit 3 is a parity error has been detected

- bit 4 is an overrun has occurred

- bit 5 is a framing error has occurred

---  END  ---