NEW CARD READER HANDLER

TO ALL & SUNDRY:

I HAVE WRITTEN A NEW CARD-READER HANDLER TO TAKE CARE

OF INPUTS OF ASCII TEXT. IT WILL HANDLE ALL ASCII CODES AND SHOULD

BE INSTALLED VIA THE FOLLOWING STEPS:

1. IF CR11.TSK IS ALREADY IN [3,30] OMIT THE NEXT STEPS AND GO TO

THE INSTALL PHASES.

2. OTHERWISE GET A COPY OF [3,30]CRBR.MAC IN A UIC ON YOUR DISK

FROM THE SYSTEM DEVELOPMENT AND INTERFACE TEST DISK [3,30] AREA

AND A COPY OF CR11.CMD FROM THE SAME PLACE AND PUT ON YOUR DISK.

3. ASSEMBLE AS FOLLOWS:

MAC CR=CRBR

4. TASK BUILD AS FOLLOWS:

TKB @CR11.CMD

5. THIS IS THE INSTALLATION PROCEDURE. ASSUMING YOU BUILT

TASK CR11.TSK USING CR11.CMD WHILE LOGGED IN AS [3,30]:

TYPE THE FOLLOWING WHILE LOGGED IN [1,1]:

UNL CR:

REM CR....

UNL MT:

LOA MT:

DMO SY:

FIX F11ACP

MOU SY:

UNF F11ACP

INS [3,30]CR11

LOA CR:

THE NEW CARD READER WILL NOT HANDLE BINARY (REASSEMBLE WITH A FILE

CONTAINING THE LINE

BINARY=0

TO PERMIT IT TO)

BUT WHEN A CONTROL CARD WITH A MULTIPUNCH WITH COLUMNS 12,0,2,4, AND 8

IN COLUMN 1 IS PUT AHEAD OF THE DECK, THE HANDLER WILL TRANSLATE ALL

CARD CODES TO THEIR ASCII EQUIVALENTS INCLUDING CR, LF AND OTHER

SPECIAL CHARACTERS. WITHOUT THIS CONTROL CARD, THE HANDLER WILL ACT

EXACTLY AS YOUR OLD ONE DID. THE CONTROL FEATURE SHOULD REMAIN SET

ACROSS ENDFILES. NOTE THAT MOST 029 CODES ARE IDENTICAL TO ASCII

CODES WHERE THE TWO OVERLAP. CARD MULTIPUNCHES THAT HAVE NO ASCII

EQUIVALENTS ARE TRANSLATED TO NULLS (0) ON INPUT. I WILL ALTER THAT

WHEN ANYONE GIVES ME A GOOD REASON TO DO SO.

GLENN EVERHART

10/22/1976