VTDRV -- VIRTUAL TERMINAL DRIVER

The origional artical on the virtual terminal  driver(VTDRV)

appeared  in  the Small Buffer, Vol.  389, by Dave Elderkin(DEC).

RSX11M-PLUS will have a virtual terminal driver, but RSX11M  will

have  no DEC supplied and supportd virtual terminal drivers for a

while(if ever).  This is interesting  since  starting  with  V3.1

both  MCR and ...AT.  recognize the device VT:  as a virtual ter-

minal.  Since all users may not have ready access  to  the  Small

Buffer, the following description of Dave Elderkin is included in

RUNOFF format.

The procedure for allowing a privileged task to submit  com-

mand  lines  to  the  MCR is well known for RSX-11M systems.  The

problem of carrying on a dialog with the system (both  input  and

output)  from  a  nonprivleged  tast  is a little more difficult.

This article describes a virtual terminal driver for  RSX-11M  V3

system  which  can perform this function until the subject is ad-

dressed in a future release.

The driver allows the user to establish a  virtual  terminal

(VT:)  on RSX-11M V3 systems.  Using this driver, a nonpriviliged

task may supply input to the system and receive output  from  the

system  as if the input and output had been done at a system ter-

minal.  In other words, the user task sends a  line  to  the  VT:

(via  a QIO IO.WLB) and the VT sends it to the system just as the

TT driver would.  The system will then send its output to what it

thinks  is  a  TI,  and  the  user task can receive it (via a QIO

IO.RLB).  The task's input to the system may be solicited or  un-

solicited.

PAGE 2

The driver is easy for the task to use.  A general  schematic  of

the required task function is:

START:

Prepare an input line to the system

Issue a QIO IO.WLB to VT1:

LOOP:

Issue a QIO IO.RLB to VT1:

Was an EOF received?

YES -- GOTO START:

NO  -- CONTINUE#

Process output line received from the system.

Was the input line an MCR prompt?

YES -- GOTO START:

NO  -- GOTO LOOP:

Preparation of the input line for the system  might  consist

of reading input from a foreign block mode terminal or of reading

a record from a file, for example.   Processing  output  received

from  the system might involve sending output to a foreign termi-

nal or writing a record to a file.  As written, the  driver  does

have some restrictions that must be delt with:

1) The driver establishes one virtual  terminal  on  the  system.

This  terminal  may be used by only one task at a time.  This one

task may have only a single stream of terminal I/O in progress at

a  time.  If this is an unaceptable restriction, you could rewite

the driver to handle multiple units or build  several  copies  of

the driver with different device mnemonics.

2)The driver is primarily record orientated(as opposed  to  char-

acter  oriented)  and  does not allow for the functions of the TT

driver used to edit an input line.  That is there is  no  support

for  (CTRL)R,  RUBOUT,  and the like.  The user task must provide

completely edited lines of input to VT:

3)The driver assumes that all lines of input to be  sent  to  the

system  are terminated by a carriage return.  Thus, there will be

difficulties using the VT  with  programs  that  terminate  their

reads on characters count or altmode(like ODT and TECO).

4)There is no support in the VT for using (CTRL)C to get the MCR.

MCR  input may only be entered in response to the standard system

prompt ">".  Furthermore, the task which  is  dealing  with  VT1:

must  examine  each output string from the system to determine if

an MCR prompt has been received.  This  is  caused  by  the  fact

that,  unlike  all  other  tasks in the system, tthe MCR does not

issue a QIO IO.RLB to  the  terminal  after  it  has  issued  its

prompt.

5)The buffer of the user's task QIO IO.WLB should(MUST) contain a

trailing carriage return.

PAGE 3

6) IO.RPR is not handled.

As written , the VT driver does allow for some special processing

to take place in addition to its normal transfer of data.

1)On output of data from the system to the user task(user  task's

IO.RLB  to  VT1:), the driver places the carriage control byte of

the system output in the first byte of the  user  task's  buffer.

This  will  allow the user task to correctly format system output

for display.

2)On input of data from the user task to the  system(user  task's

IO.WLB  to  VT:), the driver checks for the presence of a (CTRL)Z

as the first character of the buffer.  If this is  the  case,  an

EOF  notification is sent to the system.  This allows you to ter-

minate system utilities normallly.  The driver  also  checks  for

the presence of a charrige return as the firt character.  If this

is the case the system receives  a  notification  of  zero  bytes

transfered.   This makes EDI terminate input mode normally.  This

kludge is necessary because RSX will not accept  QIO's's  with  a

byte count of zero.

The VT driver works by simulating two units, VT0:  and VT1:.  The

data  base  consists  of  one DCB, Two UCB's, and two SCB's.  The

user task does its communicating with unit 1 via Qio's while  the

system  sees  a normal terminal on unit 0.  The driver cordinates

activity on these two units and performs transfers between  them.

In  the  case  in which unit 1 receives an IO.WLB and unit 0 does

not have any I/O operations outstanding, the driver performs  un-

solictied  input  to the MCR on behalf of the user.  This is done

by allocating a buffer from the DSR(pool), flling it with the UCB

address  of unit 0 and the buffer contents from unit 1 write, and

queueing it to MCR.