1.0  INTRODUCTION

This is a discription of the operation  of  the  list  processing

function of the RSX11M CAMAC driver.  It has the following sections:

1.  HOW TO GENERATE A LIST PROCESSING DRIVER

2.  INPUT TO THE LIST PROCESSING PART

3.  ERROR RETURNS

4.  INTERNAL DATA STRUCTURES

5.  DESIGN OF THE PROCESSING PART

2.0  HOW TO GENERATE A LIST PROCESSING DRIVER

3.0  INPUT TO THE LIST PROCESSING PART

The purpose of the list  processing  CAMAC  driver  is  to  allow

higher  speed  operations  then  is possable with the standard driver.

This is done by providing a list of operations for the driver  to  do.

This  list  must  go  to a DMA unit (an odd numbered unit) and as such

must  be  legal  DMA  operations.   Nondata  transfer  operations  are

accomplished by using a operation count of zero.

The list processing part of  the  CAMAC  driver  works  with  two

buffers.   These  buffers  are the data buffer and the command buffer.

Any operation in that occures compleatly in the data buffer is  legal.

Both  the  data  buffer  and  the address buffer are addressed checked

before the operation packet is queued to the driver.

3.1  Initeating a list operation

To initeate a list  operation,  the  following  QIO  is  executed

(standard system paramiters have been ommited).

QIO$  IO.LPR,dma,,,,,<dbufad,dbufsz,,,cbufad,cbufsz>

Where:

PAGE 2

1.  dbufad is the data buffer address

2.  dbufsz is the data buffer size in bytes

3.  cbufad is the controll buffer address

4.  dbufsz is the control buffer size in bytes

5.  dma is a DMA CAMAC unit

6.  IO.LPR is 3001 octal

The status return from  this  operation  is  described  in  the  error

section.   If  either buffer violates the address check, the operation

is aborted, and an error is returned in the $DSW word and  carry  will

be set.

3.2  Format of the CAMAC Operation Packit List

The CAMAC Operation Packit List (COP List) is a table of COPs which is

terminated by a word of zero.

Each COP in the COP List is made up of  two  parts;   a  mandtory

part, and a optional part.

3.2.1  The mandatory part - Each COP must have the following format:

****************************************

* BYTE COUNT   ! FLAG BYTE             *

****************************************

* CRATE SELECT WORD                    *

****************************************

* EXTENDED WORD COUNT WORD             *

****************************************

* FNA WORD                             *

****************************************

****************************************

* OPTIONAL DATA                        *

****************************************

1.  The byte count is used to index to the next COP as  COPS  are

of variable length.

2.  The type byte indicates the options that are included in  the

optinol part of the COP.

3.  The crate select  is  the  word  that  will  be  put  in  the

corrosponding interface register.

4.  The extended word count is  the  word  that  is  put  in  the

PAGE 3

corrosponding interface register

5.  The fna word is the word that is placed in the  corrosponding

interface register.

3.2.2  Optional part - The optional part is  controlled  by  the  type

byte  in  the  mandatory part.  This part contains information that is

needed to do special operations on the list.  The type  byte  has  the

following bit encodings:

1.  bit 0= buffer offset requested

2.  bit 1= do an  station  subaddress  check  on  the  compleated

operation

3.  bit 2= execute this COP some number of times.

4.  bit 6= ignore CAMAC timout (T02)

5.  bit 7= ignore crate overflow indication

This byte is examined from  the  LSB  to  the  MSB,  and  if  the

encoding  exists,  the optinal part is assumed to have the apreopreate

data.  The format for the data in the optinal part with respect to the

type byte encodings is given below:

1.  bit 0= byte in buffer to place first word of  data  read  in.

If  this  bit  is not set, the data will be appended onto the

data just finished.  Initally, data will be  placed  starting

at  the  first  word  of the buffer.  If this bit is set, the

word value must be even and less then the size  of  the  data

buffer.

2.  bit 1= expected crate, station, and subaddress of the  driver

after  the  operation.   This word has the following format (

where <field size:  starting bit> indicate the number of bits

in the field and the starting bit of the field.  i.e.  <16:0>

is a word, <8:0> is even byte, <8:8> is odd byte):

subaddress <4:0>, station <5:4> end crate mask <7:9>

3.  bit 2= request for  repeat  count.   The  corrosponding  word

contains the number of repatitions.

4.  bit 6= ignore CAMAC time out .  If this bit is set, no errors

will be produced from a CAMAC timeout (T02).

5.  bit 7= ignore crate overflow indication.  If this bit is  not

set and a crate overflow occures, an error will occure.

PAGE 4

PAGE 5

4.0  ERROR RETURNS

f an error occures and if an IO status block was specified in the QIO,

an  error  indication  will  be returned.  The IO status block has the

following format:

****************************************

*  PKT CNT    ! RETURN CODE            *

****************************************

* USER SELECTED VALUE                  *

****************************************

The return codes are:

1.  IE.VER

2.  IE.BAD

3.  IE.ONP

4.  IE.END= end fault, where end CNA was not the one expected.

5.  IE.OVR= an attempt was made to access outside of the buffer

6.  IE.CVF= unexpected crate overflow occured

7.  IE.IMP= 18  bit  address  calculation  error  (imp  indicates

impossable).

8.  IE.CT2= CAMAC time out error.

9.  IE.JER=  Jorway  interface  error  (detected  by   check   on

calculated  number of bytes transfered vs.  actuall number of

bytes transfered.) If this occures the UCB is marked  offline

as the interface is not working.

The PKT CNT is the number of the packet that was being  processed

at  the time of the error.  The user selected value is any provided by

the standard driver.

PAGE 6

5.0  INTERNAL DATA STRUCTURES

The following is the data structure for the list processing driver  in

the UCB above thos already used by the standard driver:

****************************************

U.EEND    * EXPECTED END LOCATION                *

****************************************

U.FLAG    * FLAG WORD (LOW BYTE COP FLAG)        *

****************************************

U.DBBR    * BYTES REMAINING IN DATA BUFFER       *

****************************************

U.LNPR    * LAST 18 BIT ADDRESS OF               *

****************************************

* JORWAY ADDRESS POINTER               *

****************************************

U.CBSZ    * BYTES IN CONTROLL BUFFER             *

****************************************

U.CBRL    * RELOCATED ADDRESS OF CONTROL BUF     *

****************************************

*                                      *

****************************************

U.COPC    * COP COUNT                            *

****************************************

U.REPT    * REPATITION COUNT FOR COP             *

****************************************

U.BYTR    * BYTES TO BE TRANSFERED               *

****************************************

PAGE 7

6.0  DESIGN OF THE PROCESSING PART

The initalization part for the first trnasfer of a COPL.

MODULE LISTINI

BEGIN

FIRST=TRUE;

MAP TO CONTROLL BUFFER;

SETUPL();

INTERUPT ENABLE=TRUE;

EXCTFN();

UNMAP FROM CONTROL BUFFER;

END;

ELUDOM LISTINI

MODULE SETUPL

BEGIN

IF (BUFFOFSETREQ OR FIRST)

THEN BEGIN

IF (BUFFOFSET)

THEN BEGIN

IF (BUFFOFSET GT U.CNT) THEN (ERROR);

U.LNPR=U.BUF+BUFFOFSET;

U.DBBR=U.CNT-BUFFOFSET;

END

ELSE BEGIN

U.LNPR=U.BUF;

U.DBBR=U.CNT;

END;

FIRST=FALSE;

LOADAD=TRUE;

END;

IF (ENDCK) THEN (U.EEND=ENDNA);

IF (REPT AND (U.REPT EQ 0)) THEN (U.REPT=REPTCNT);

IF (U.REPT EQ 0) THEN (U.REPT=1);

IF (EXWC<12:0>  NE 0)

THEN BEGIN

U.BYTR= -(170000 OR EXWC)*2;

IF (24BITMODE) THEN (U.BYTR=U.BYTR*2);

U.DBBR=U.DBBR-U.BYTR;

IF (U.DBBR LT 0) THEN (ERROR);

END;

U.FLAG<8:0>=FLAG;

END;

ELUDOM SETUPL

MODULE EXCTFN

BEGIN

PAGE 8

IF (LOADAD) THEN (J411ADD=U.LNPR; LOADAD=FALSE);

J411CS=CRATESELECT;

J411EXWC=EXTENDEDWORDCOUNT;

J411FNA=FNA

!

! AT THIS POINT THE CAMAC OPERATION STARTS

!

U.REPT=R.REPT-1;

IF (U.REPT EQ 0)

THEN BEGIN

ADVANCE CONTROLL BUFFER POINTER

U.COPC=U.COPC+1;

END;

END;

ELUDOM EXCTFN

MODULE INTERUPT

BEGIN

MAP TO CONTROLL BUFFER;

WHILE (OPERATION COMPLEATE)

DO BEGIN

CLEAR J411 INTERUPT ENABLE AND SEQUEUCER

ERRCK();

IF (DONE) THEN ( U.COPC=U.COPC-1; FORK);

SETUPL();

INTERUPT ENABLE=TRUE;

EXCTFN();

END;

UNMAP FROM CONTROL BUFFER;

INTEXIT;

END;

ELUDOM INTERUPT

MODULE ERRCK

BEGIN

IF (J411CROV AND NOT CRTOVOK) THEN (ERROR);

IF (J411T02 AND NOT T02OK) THEN (ERROR);

IF (ENDCK)

THEN BEGIN

IF ((U.EEND AND 777) NE (J411FNA AND 777)) THEN (ERROR);

IF ((U.EEND AND 177000) NE (J411CS AND 177000))

THEN (ERROR);

END;

IF ((J411EXWC AND 7777) EQ 0) THEN (NBYTES=0)

ELSE BEGIN

NBYTES= -(170000 OR J411EXWC)*2;

IF (MD24) THEN

(NBYTES=NBYTES*2);

END;

U.DBBR=U.DBBR+NBYTES;

U.BYTR=U.BYTR-NBYTES;

U.LNPR=U.LNPR+U.BYTR;

PAGE 9

IF (U.LNPR NE J411MADR)

THEN BEGIN

SET UCB OFFLINE;

ERROR;

END;

END

ELUDOM ERRCK