THIS COMMAND FILE GENERATES A CAMAC DRIVER FOR THE EG&G  CAMAC  BRANCH

HIGHWAY  OR  THE  JORWAY  J411  EQUIVELANT.   THE  DRIVER  CAN BE MADE

LOADABLE OR RESIDENT.  THE GTWRD AND PTWRD OPTIONS MUST  BE  SPECIFIED

DURING  SYSGEN TO INSTALL THE DRIVER.  IF YOU GET ASSEMBLY ERRORS THIS

IS PROBABLY THE CAUSE.

BEFORE CONTINUING WITH  THE  DRIVER  INSTALATION  YOU  MUST  EDIT  THE

RSXMC.MAC FILE THAT IS ON THIS AREA TO ADD SOME OPTIONS.

THE OPTIONS ARE:

1.  JORW=0 IF A JORWAY DIRVER IS DESIRED.  IF NOT DEFINED A  EG&G

DRIVER IS GENERATED.

2.  C$$M11= THE NUMBER OF CONTROLLERS TO BE  USED.   IF  THIS  IS

GREATER  THEN 1, YOU MUST SPECIFY THE CSR AND INTERUPT VECTOR

ADDRESS IN RSXMC.MAC (SEE 4).

3.  LD$CM=0 IF THE DRIVER IS TO BE LOADABLE.

4.  IF YOU HAVE MULTIPL INTERFACES, YOU MUST DEFINE THE  CSR  AND

INTERUPT  VECTOR FOR EACH ADITIONAL INTERFACE.  IF YOU DO NOT

WANT THE DEFAULTS FOR THE  FIRST  INTERFACE,  YOU  MUST  ALSO

DEFINE ITS CSR AND INTERUPT VECTOR.

TO DO THIS:

1.  CMCSR$=CSR ADDRESS WHERE $ GOES FROM 0-7 AND  0  DEFAULTS

TO 166000

2.  CMINT$=INTERUPT ADDRESS WHERE  $  GOES  FROM  0-7  AND  0

DEFAULTS TO 274

5.  LSTPDV=0 IF YOU WANT  THE  LIST  PROCESSING  PORTION  OF  THE

DRIVER.   THIS  OPTION  DOUBLES  THE  SIZE  OF THE DRIVER BUT

ALLOWS A LIST OF CAMAC OPERATIONS TO BE SENT  TO  THE  DRIVER

WITH  ONLY  ONE  QIO.   IT  ALSO REDUCES SYSTEM OVERHEAD BY A

FACTOR OF 2.  HOWEVER, YOU MUST HAVE A MAPPED SYSTEM  TO  USE

THIS OPTION.