RSX11M/DECNET VIRTUAL TERMINAL SUPPORT

Greg Thompson (Informatics/PMI)

4/16/78

Version 1.0 for RSX11M V3.0 and DECNET V1.2

This  document  describes the virtual terminal support available on

RSX11M  systems with DECNET which allows a terminal on one DECNET host

to  logically be connected as a terminal on another DECNET host.  Once

a  connection  is  established  the initiating terminal appears on the

remote  host  just  like  any other remote terminal and is required to

log-in  to  the remote system using HELLO as usual.  The connection is

as  fully  transparent  as  possible  including the support of running

RMDEMO or EDT over the virtual terminal connection.

The  support consists of two tasks and one loadable driver entitled

CON,  TELNET,  and  RTDRV.   It  requires the DECNET software and many

functions  of  the  full  user-oriented  terminal  driver.   The RTDRV

requires  loadable  device driver support but can be easily changed to

be  a resident driver if desired.  The software, as provided, supports

up  to  eight  virtual  terminals on each host, although the number of

terminals  supported  is  simply  an  assembly parameter in TELNET and

RTDRV  and  may  be modified if desired.  Only RTDRV need be built for

the  particular host and the software does not require a new sysgen as

long as the required functions are already available.

The  CONNECT task is the user side of the virtual terminal software

and  allows  a user to connect to the virtual terminal server software

on  a  remote host.  Even though the CON task is a privileged task (so

write-break-throughs  will  work), it is based on APR0 and need not be

rebuilt  for  each system.  The connection is initiated by issuing the

command  "CON  <name  of host>" where <name of host> is a valid DECNET

host  name for the local host.  When the connection is established the

virtual   terminal  driver  on  the  remote  host  returns  a  message

indicating to which  virtual terminal the user is connected.  The  CON

task  has  two  local commands which allow the user to disconnect from

the  remote  host  and  to temporarily suspend a connection inorder to

issue commands to the local host after which he (or she) may reconnect

using  the  RESUME  command.  Local commands are preceeded by a number

sign  "#"  as the first character of a line.  If it is desired to send

to  the remote host a line with the first character a number sign, two

number  signs  should  be  used  instead.   The local commands;  #END,

#CLOSE,  #QUIT  or #EXIT close the connection while #SUSPEND or #PAUSE

suspend  the  connection.   If  a  user  forgets  to  issue BYE before

disconnecting, a BYE is automatically issued for him.

Even though DECNET does not support route through, issuing a CON on

the  remote  host  inorder  to  connect  to  a  host  local  to  it is

explicitly  allowed.  Of course the user must still have an account on

the  remote  hosts  inorder  to  log-in and each remote host must have

allowed virtual terminal connections by installing the TELNET task and

loading  the  virtual  terminal  driver  (RT:).   Some caution must be

observed  when using CON from the operators console since CON attaches

the terminal (with unsolicited input AST) and any messages intended to

the  terminal  are held up until the connection is ended or suspended.

This  is particularly a problem on our systems since our HELLO and BYE

tasks report the logging in and out of users to the operators console.

TELNET  (from  ARPANET's  TELecommunications NETwork protocol) is a

non-privileged  server  task  that  interfaces  between DECNET and the

virtual  terminal  driver.   Since it is a non-privileged task it also

does  not need to be rebuilt for each host.  It contains 24 buffers of

142  bytes  each  (both  assembly  parameters)  which  are dynamically

allocated  to connections as necessary inorder to keep the exchange of

messages between CONNECT and RTDRV as rapid as possible.

RTDRV  is  the  loadable  device  driver  for  the virtual terminal

support.   It  implements  additional  terminal type devices under the

device  name  "RTx:".  where x ranges from 0 to one less the number of

virtual  terminals  supported.   It  accept all of the functions codes

supported  by the TT: driver along with IO.CON, IO.XMT, IO.RCV, IO.DIS

and  IO.CTL  which  the TELNET task uses to communicate to the driver.

To  disable  virtual  terminal connections to a node simply unload the

RT: driver on that node.  When no user is connected to a given virtual

terminal  number  the  DISABLED  bit  for  that terminal is set and is

cleared as soon as someone is connected to that virtual terminal.

The   sizes   of   the  software  modules  (with  current  assembly

parameters) are 2.5K words for the CONNECT task, 2.9K words for TELNET

and  .9K  words for the virtual terminal driver.  The virtual terminal

software  was  written under contract to NASA/Ames Research Center and

is therefore in the public domain.

The only currently known bug which will be fixed in the ungrade to

DECNET V2 is the fact that when a task attaches RTx: the unsolicited

input is not tossed away and this can cause problems when used with

EDT if you type to fast.