THE FILES ON THIS TAPE ARE INTENDED TO BE USED

TO GENERATE SPEECH VIA A VOTRAX VSK UNIT.  WE HAVE

INTERFACED THIS UNIT TO AN ASYNCHRONOUS LINE AT

9600 BAUD.  INCLUDED IN OUR INTERFACE IS A 160 BYTE

FIFO BUFFER.  A CHARACTER CAN BE REQUESTED TO INDICATE WHETHER

OR NOT THIS BUFFER IS 3/4 FILLED.  THIS CONSIDERABLY

SIMPLIFIES THE TIMING OF THE OUTPUT STREAM.

THESE ROUTINES RUN UNDER RSX-11M V3.1 ON A PDP-11/55.

FORTRAN IV+ AND MACRO-11 ARE USED.

RULES.VSK	THIS IS MOSTLY CROWLEY'S ADAPTATION OF NRL RULES

WITH THESE CHANGES:

1. ! SYMBOL USED TO ALLOW UNSEPARATED

PUNCTUATION

2. SOME PRONUNCIATION OF PUNCTUATION

3. A VERY FEW ADDED RULES

4. SEVERAL TYPOS CORRECTED

RULES.CMP	CMP LISTING BETWEEN MY RULES AND CROWLEY'S

RULES.FTN	THIS PROGRAM CONDENSES THE ASCII VERSION OF

THE RULES TO AN INTERNAL FORM

RULES.CMD	BUILD FILE FOR RULES.FTN

RULES.PAR	CONDENSED VERSION OF THE RULES

RULIST.FTN	THIS PROGRAM PRINTS TRANS.PAR IN A READABLE

FORM (MOSTLY FOR DEBUGGING)

RULIST.CMD	BUILD FILE FOR RULIST.FTN

TRANST.FTN	A TEST PROGRAM TO DRIVE TRANS.  IT ACCEPTS A FILE

SPECIFIER (INCLUDING OF COURSE TI:) THEN TRANSMITS

THE FILE CONTENTS TO TRANS.  REFERENCES TO NCELL

AND CELL CODE REFER (ESSENTIALLY) TO THE

IDENTIFICATION OF THE VSK SERIAL PORT.

TRANST.CMD	BUILD FILE FOR TRANST.FTN

TRANS.FTN	MOSTLY CROWLEY'S VERSION CONVERTED TO FOR+.

IT READS THE CONDENSED VERSION OF THE RULES

ON ENTRANCE AND CALLS VSK SUB FOR OUTPUT.

BCOMP.FTN	BYTE COMPARE FUNCTION USED BY TRANS AND RULES

BAND.MAC	BYTE "IAND" FUNCTION USED BY TRANS AND RULES

VSKS.FTN	SUB TO SEND RECORD ON TO SERIO IF THERE'S ROOM

IN THE FIFO

SERIOS.FTN	INTERCHANGES DATA WITH VSK (VIA DL-TYPE INTERFACE)

VSP.MAC		MACRO PROGRAM TO INITIALIZE COMMON BLOCK FOR

TRANSLATION OF VOTRAX PHONEMES FROM ASCII TO

INTERNAL FORM.