RSX-11M V3.2 MODIFICATIONS

This is a varied assortment of changes to RSX-11M V3.2.

not  every  one  will  be  suitable and/or desirable for any

given installation.  They are presented here as a base  from

which your facility may extract, modify, and use to your own

particular advantage.

Many of the executive changes have  to  do  with  power

fails.   This  is important to us because we use on-line UN-

IBUS switches which cause simulated power fails during their

operation.  The cache change works with modified Minntronics

and modified ABLE models.

IMPORTANT NOTE:  Any discs which  were  initialized  by

RSX-11M  prior to V3.0 (i.e.  V2.0) had some volume defaults

stored in different offsets in the  home  block.   V3.0  and

V3.1  MOU  had  hooks to provide "default" defaults if these

values were 0.  V3.2 MOU no longer has these hooks!   So  if

you  want to make use of these (i.e.  EDI which uses default

file extension), you must either a)  use  /EXT  switch  with

MOU,  b) process the disc with CVL (from the N.O.  tape), or

c) use ZAP on INDEXF.SYS (very carefully or  you  may  never

again mount that disc).

EXECUTIVE:

CRASH   Reduced size for our combination (CDA with

console stack dump)

EXDBT   Corrected stack overflow check

Allow CRASH access to M.M. fault registers

INITL   Modify 11/70 memory CSR table

Remove device offline messages

PANIC   Remove unused code

PARTY   Allow TKTN to display memory parity CSR'S

Modify 11/70 memory CSR table

POWER   Re-verify online devices on power-up

Modify 11/70 memory CSR table

Enable cache and stack limit registers

SSTSR   Save M.M. fault registers for TKTN

DRIVERS:

DKDRV   Modify not-ready wait technique to wait up

to 10 seconds in 1 second increments

rather than up to 15 seconds in 5 second

increments

PAGE 2

MTDRV   Lengthen timeout periods for reads, writes,

spaces

Unimplement powerfail "feature"

FULL DUPLEX TERMINAL DRIVER:

TTDAT   Added terminal types: ADM3A, ADDS, Tektronix

Added cursor positioning types

TTSUB   Use S2.HFF to choose "VT" expansion (like

IAS)

Added cursor positioning types

TTYL    Modified DL-11 record startup procedure

(required for use with ABLE Quadrasync)

MCR:

LDFIN   Allow error logging drivers to have two

vectors

UNLCTL  Allow error logging drivers to have two

vectors

Unload data base with driver (see caution in

source code)

DEVOV   Display UCB address in device listing

(useful for on-line patching of "LOA"ded

device tables by XDT

FLAOV   Correct 3 problems in FLA command

INDOPN  J G Downward's patch for 2 consecutive

.OPENs or .TESTFILEs

INIMAG  Eliminate TKB error messages

MCROV   Allow STOP/UNSTOP commands in single user

systems

M11OV   Record mounts in error log for non-M-plus

systems (at last you can identify which

disc had the errors)

PAROV   Provide a BUSY status of a partition (tells

you whether or not you loaded a common

partition)

SAVSUB  Modify memory size calculation

Remove device offline messages

Modify 11/70 memory CSR table

SDSFDT  Added terminal types to FDT

SPRFDT  Added terminal types to FDT

TASOV   Add optional taskname (from 3.1 patch on

N.O. tape)

TKTN    Display memory parity CSR's if parity error

Print KT11 registers on segment faults

Correct sense of time stamp conditional

FORTRAN IV-PLUS V2.5:

FIO     Use unsigned branch for address check

FNBST   Use unsigned branch for address check

PATOTS  Patch command file for above with checksums

RMDEMO:

REMAP   Use proper EIS conditional

PAGE 3

RMDRIV  Use write pass all

Allow split binary/decimal addressing

between row and column (required for ADDS

terminals)

V1HCHA  New decimal flag for use with RMDRIV

HLINIT  Reduce listing size

MDCOM   Modified device list

TERMID  Added terminal types

V52CHA  Replaced bad comment

ADDCHA  ADDS terminal routines

ADDHLM

ADDMDM

ADMCHA  ADM3A terminal routines

ADMHLM

ADMMDM

T06CHA  Tektronix 4006 routines

T06HLM

T06MDM

LPTCHA  Line printer routines

LPTHLM

LPTMDM

RMDBLD  ODL file to include these new routines

SYE:

M1145P  Correct output of memory parity CSR address

and contents

MUNDEF  Revised output for non-DEC devices (.OBJ

also supplied for use without SYE.MAC from

source kit)

BROAD:

BROAD   Provide global def for START, CMPBLK to

allow TKB GBLPAT's

BROBLD  GBLPAT to cause broadcast to slaved

terminals

SYSGEN COMMAND FILES:

SYSGEN  Install MAC with increment to speed phase

one on baseline

SGNEXC  Some conditionals added for 11/45's and

better

SYGEN2  Install TKB with increment to speed phase

two on baseline

Correct size of LDRPAR after LOADR

correction

SYGEN3  Transfer to system disc with /CO

Cleanup at end of SYSGEN3

Allow maps on disc as in SYSGEN2

BLDUMD  Allow colon after map device name

MISCELLANEOUS:

STNLON  DOC file describing how to modify default

vectors and CSRs for standalone BAD, DSC,

PAGE 4

and PRESRV

XDTHNT  Hint for easily making a loadable driver

trap to XDT