#HP67 Programmable Calculator Emulator -- Instruction Set

Instruction	Code #			Description

CLX		-128		Clear X register, disables stack lift on next

numeric entry.

CLSTK		-127		Clears entire stack.

CLREG		-126		Clears all registers.

^		-125		Pushes X register into Y register.

RU		-124		Rolls stack up; X to Y...Z to X.

RD		-123		Rolls stack down; T to Z...X to T.

><		-122		Interchange X and Y registers on stack.

LSTX		-121		Moves contents of LAST X register to X lifting

rest of stack registers.

STK		-120		Displays current stack contents.

CHS		-119		Changes sign of # in X register.

ABS		-118		Replaces # in X with absolute value of # in X.

INT		-117		Truncates # in X to integer.

FRAC		-116		Replaces # in X with fractional portion of # in

X.

PI		-115		Places pi (3.1415...) in X register.

+		-114		Adds X and Y registers.

-		-113		Subtracts X from Y register.

*		-112		Multiplies X and Y registers.

/		-111		Divides Y by X register.

%		-110		Replaces # in X with X percent of Y register.

%CH		-109		Computes percent of change from # in Y register

to # in X register ( (X-Y)*100/Y ).

N!		-108		Computes factorial of integer portion of # in

X register.

/X		-107		Computes reciprocal of # in X register.

SQRE		-106		Computes square of # in X register.

SQRT		-105		Computes square root of # in X register.

Y^X		-104		Raise # in Y register to power of # in X

register.

^X		-103		Raise 10 to power of # in X register; common

anti-logarithm.

E^X		-102		Raise "e" to power of # in X register; natural

anti-logarithm.

LN		-101		Computes natural (base "e") logarithm of # in

X register.

LOG		-100		Computes common (base 10) logarithm of # in X

register.

DEG		-99		Sets calculator to degree mode; all angular

operands assumed to be in decimal degrees.

RAD		-98		Sets calculator to radians mode; all angular

operands assumed to be in radians.

GRD		-97		Sets calculator to grads mode; all angular

operands assumed to be in grads.

COS		-96		Computes cosine of # in X register.

SIN		-95		Computes sine of # in X register.

TAN		-94		Computes tangent of # in X register.

ACOS		-93		Computes arccosine of # in X register.

ASIN		-92		Computes arcsine of # in X register.

ATAN		-91		Computes arctangent of # in X register.

>HMS		-90		Converts decimal hours or degrees to

hours, minutes, seconds in form HH.MMSSSS.

>DMS		-89		Identical to ">HMS".

HMS+		-88		Adds Y register to X register assuming both

registers are in form of HH.MMSSSS.

DMS+		-87		Identical to "HMS+".

>R		-86		Converts # in X register from degrees to

radians.

D<		-85		Converts # in X register from radians to

degrees.

H<		-84		Converts # in X register from "HH.MMSSS" form

to decimal degrees.

>P		-83		Converts x,y coordinates in X and Y registers

to polar magnitude (r in X) and angle (theta

in Y).

R<		-82		Converts polar magnitude and angle in X and Y

registers to rectangular x,y coordinates.

Following relational tests are only executed when under program control.

NEY		-81		Executes next program step if # in X register

is not equal to # in Y register; otherwise

next program step is skipped.

NE0		-80		Executes next program step if X not equal to 0;

else skip step.

EQY		-79		Executes next program step if X = Y; else skip

step.

EQ0		-78		Executes next program step if X = 0; else skip

step.

GTY		-77		Executes next program step if X > Y; else skip

step.

GT0		-67		Executes next program step if X > 0; else skip

step.

GEY		-75		Executes next program step if X > or = Y; else

skip step.

GE0		-74		Executes next program step if X > or = 0; else

skip step.

LTY		-73		Executes next program step if X < Y; else skip

step.

LT0		-72		Executes next program step if X < 0; else skip

step.

LEY		-71		Executes next program step if X < or = Y; else

skip step.

LE0		-70		Executes next program step if X < or = 0; else

skip step.

-69 to -21	Currently unassigned

Following instructions only accepted when in "programming" mode.

CLPRGM		-20		"Clears" program memory; sets all steps to

R/S instruction.

BST		-19		Backsteps program to previous step.

DEL		-18		Deletes current program step, moves all

remaining steps up, bringing in R/S instructions

at end of memory.

INS		-17		Inserts a "NOP" instruction by pushing current

and trailing steps down, losing last

instruction (currently force a R/S to be last

program step).

RUN		-16		Exits programming mode, returns to normal

keyboard control, i.e. run mode.

Following instruction only accepted when in "run" mode.

PRGM		-15		Places calculator into programming mode.

-14 to -6	Currently unassigned

Following instructions accepted at all times.

SST		-5		When in run mode, executes next step from

program memory and returns control to keyboard.

When in program mode, skips over current program

step and displays next program step.

-4		Currently unassigned.

PAUSE		-3		When executed from keyboard, acts as "NOP".

When executed from program memory, displays

current # in X register, then resumes execution.

NOP		-2		No operation.

RTN		-1		When executed from the keyboard, resets

program counter to program step #1.  When

encountered during program execution as the

result of a "GSB" instruction, program execution

resumes at step immediately following the GSB

instruction; otherwise, program counter is set

to step # 1 and control returned to keyboard.

R/S		0		When encountered during program execution,

returns control to the keyboard.  When

entered from the keyboard, resumes program

execution at current program step.

LBLa		1		Alphabetic (A-Z, a-z) label designator, defines

beginning of a program section.

LBLn		2		Numeric (0-255) label designator, defines

beginning of a program section.

3		Reserved for future use.

DSPn		4		Selects display of n digits to right of decimal

point.  Program automatically shifts to

scientific notation for numbers which can not

be represented with n decimal digits.

DSP(n)		5		Uses the absolute value of integer portion of

contents of register n (0-255) to select #

displayed digits to right of decimal point.

CFn		6		Clear flag # n (0-3).

SFn		7		Set flag # n (0-3).

TFn		8		Test flag # n (0-3).  When encountered as

program step, program execution continues

with next sequential step if flag n is set,

otherwise, next step is skipped.  TF2 and TF3

automatically clear flag 2 or 3 after

execution.  Flag 3 is set following

each numeric entry.  Executes as NOP when

entered from keyboard.

DSZn		9		Decrement register n (0-255) and skip if 0.

Subtracts 1 from register n and skips 1 program

step if contents = 0.

DSZ(n)		10		Indirect decrement and skip if 0.  Subtracts

1 from register pointed to by contents of

register n and skips 1 program step if result

is 0.

ISZn		11		Increment register n and skip if 0.  Adds 1 to

contents of register n and skips 1 program step

if contents = 0.

ISZ(n)		12		Indirect increment and skip if 0.  Adds 1 to

register pointed to by contents of register n

and skips 1 program step if result is 0.

GTOa		13		Go to alpha (A-Z, a-z) label designator.  If

executed from keyboard, positions program

counter to program step immediately following

the "LBLa" instruction, and returns control to

keyboard.  If executed as program step, finds

"LBLa" instruction, and resumes execution with

next program step.

GTOn		14		Go to numeric (0-255) label designator.  If

executed from keyboard, positions program

counter to program step immediately following

the "LBLn" instruction, and returns control

to the keyboard.  If executed as a program

step, finds "LBLn" instruction, and resumes

execution with next program step.

GTO(n)		15		Indirect go to numeric label.  Searches for

numeric label entry corresponding to contents

of register n, then proceeds as GTOn.

GSBa		16		Go to subroutine labeled "a".  Executes as

GTOa when entered from keyboard.  When

executed as a program step, program counter

of next sequential step is loaded into a

subroutine return point register, and program

control is transfered to the program step

immediately following "LBLa" instruction.

GSBn		17		Go to subroutine labeled "n".  Executes as

GTOn when entered from keyboard.  When

executed as a program step, program counter

of next sequential step is loaded into a

subroutine return point register, and program

control is transfered to the program step

immediately following "LBLn" instruction.

GSB(n)		18		Indirect go to subroutine.  Executes as

GTO(n) when entered from keyboard.  When

encountered as a program step, program counter

of next sequential step is loaded into a

subroutine return point register, and program

control is transferred to the program step

immediately following a numeric label entry

corresponding to the contents of register n.

19		Currently unassigned.

REGn		20		Displays current contents of register n.

REG(n)		21		Displays current contents of register pointed

to by contents of register n.

CLREGn		22		Clears register n.

CLREG(n)	23		Clears register pointed to by contents of

register n.

><n		24		Interchanges contents of stack register X

and register n.

><(n)		25		Interchanges contents of stack register X

with contents of register pointed to by

contents of register n.

RCLn		26		Recalls contents of register n to stack

register X, lifting previous stack elements.

RCL(n)		27		Recalls contents of register pointed to by

contents of register n to stack register X.

Previous stack elements are lifted.

STOn		28		Stores contents of stack register X in register

n; stack remains unchanged.

STO(n)		29		Stores contents of stack register X in register

pointed to by contents of register n; stack

remains unchanged.

STO+n		30		Adds contents of stack register X to register

n; stack remains unchanged.

STO+(n)		31		Adds contents of stack register X to register

pointed to by contents of register n; stack

remains unchanged.

STO-n		32		Subtracts contents of stack register X from

register n; stack remains unchanged.

STO-(n)		33		Subtracts contents of stack register X from

register pointed to by contents of register n;

stack remains unchanged.

STO*n		34		Multiplies contents of stack register X by

register n; stack remains unchanged.

STO*(n)		35		Multiplies contents of stack register X by

register pointed to by contents of register n;

stack remains unchanged.

STO/n		36		Divides register n by contents of stack register

X, result placed in register n; stack remains

unchanged.

STO/(n)		37		Divides register pointed to by contents of

register n by contents of stack register X;

stack remains unchanged.

38 to 63	Currently unassigned.

Following instruction only allowed under "programming" mode

GTO.i		64		Steps program to absolute step # i (1-1000).

Program steps do not necessarily correlate to

program memory locations, e.g. LBLa instruction

requires 2 memory locations.

65 to 119	Currently unassigned.

d		120		Dummy instruction used for storing constants in

program memory.  When in programming mode,

any numeric string not associated with an

instruction is interpretted as a constant and

prefixed by this code #.  When executed as a

program step, lifts stack and loads stack

register X from program memory.

121-123		Currently unassigned.

"prompt"	123		Types the character string enclosed by "'s

for programmed prompting of user.

LOADfilespec	124		When entered from keyboard, loads program

memory from "filespec" and returns to keyboard

control pointing at first program step.  When

encountered as a program step, loads program

memory from "filespec" and begins execution at

first program step.

Following instruction only valid in "programming" mode

SAVEfilespec	125		From programming mode, starts decoding current

program memory into analogous character

instructions and writing to "filespec".  Control

returns to keyboard in programming mode.

GETREGfilespec	126		Loads registers 0 to 224 from specified file

using list directed read format.

PUTREGfilespec	127		Writes current contents of registers 0-224 to

specified file, 3 #'s per line separated by

commas in "E24.17" format.

a = alphabetic character A-Z, a-z

n = unsigned integer number in range 0-255

i = integer number (optionally signed) in range -32768 to 32767

d = double precision floating point number in range .29E-38 to .17E39

filespec = valid operating system file specification

prompt = any valid ASCII string