The TAPE command                                       28-MAY-81

--- ---- -------

The TAPE command displays the current status  of  the  tape

drive.  It is useful to save you walking in to the computer room

to verify that the tape is at a desired position.   To  use  the

TAPE command, simply type

TAPE       [or just TAP]

The display produced by this command  informs  you  of  the

drive status by means of a sense characteristics function.  Note

that if the tape is engaged in a lengthy function when you issue

the  TAPE  command,  it  may  take  a while for the output to be

produced.  The information that can be returned by  TAPE  is  as

follows:

1.  Drive is Offline.  This means that  there  is  no  tape

mounted  or  that  the  drive is not turned online.  To

turn the drive online, press the "ONLINE" button.

2.  Tape is past EOT.  The  tape  us  positioned  past  the

end-of-tape (EOT) marker.

3.  Writing  is   prohibited.    The   tape   is   software

write-locked.

4.  Tape is physically write-locked.  The tape was  mounted

without a write-enable ring.

5.  Current  density  is  800  BPI  (or  1600  BPI).   Self

explanatory.

6.  Tape is at load point (BOT).  The  tape  is  positioned

right at the beginning.

7.  Tape is at end-of-volume (EOV).  The tape is positioned

right at the end.

8.  Drive is currently rewinding.  Self explanatory.

9.  Last tape command encountered EOF.  The last  operation

on  the  tape  encountered and end-of-file mark (unless

the last operation was a backspace).

10. Writing  with  extended  inter-record  gap  prohibited.

Just  what  is  says:  no recovery will be attempted by

the driver if a write error  occurs,  such  as  if  the

drive encounters a patch of bad tape.