Program TALK

Documentation and User's Guide

J. Yambor and N. Bewtra

Computer Sciences Corporation

Silver Spring, Maryland

Prepared for

Goddard Space Flight Center

under Technical Direction of

Bob Turkelson

PAGE 2

1.0  Introduction

TALK is a terminal emulation program which enables a PDP-11

user  running  under the RSX-11M operating system to communicate

with a remote host computer.  The program  usually  operates  in

"terminal"  mode which allows the user to talk to the host as if

the PDP-11 were invisible.  The program also has the  capability

of bi-directional file transfers.

Throughout this document the "host" refers to the  computer

with  which  the  TALK program is communicating - not the one on

which TALK is running.

1.1  System Requirements

The PDP-11 must be running under  (at  least)  version

3.2 of RSX-11M with the full duplex terminal driver.

Program TALK Documentation                                PAGE 3

12/2/81

2.0  Operation Details

Once TALK has been initiated, the user is prompted to enter

the  channel  number.   This  is  the  number  of  the interface

connected to the host computer.  A control-Z input at this  time

will terminate the program.

At this point, TALK is in "terminal"  mode - that  is,  any

characters typed will be sent directly to the host computer, and

all characters from the host computer will be displayed  on  the

terminal.

There are a number of  control  characters  which  are  not

transmitted  from  the  keyboard  to  the  host  computer.   The

control-S (^S) and control-Q (^Q) characters are used to suspend

and  resume (respectively) the processing of characters from the

host.  The control-B (^B) character  instructs  the  program  to

transmit   a   "break"  character  to  the  host.   During  file

transfers, a control-O (^O) character alternately  suspends  and

resumes  the  terminal  echoing of characters being transmitted.

The control-W (^W) character  instructs  the  program  to  enter

command input mode.  In this mode, the user is prompted to enter

an alphabetic character specifying one of a  number  of  program

commands  which  are not specified by control characters.  These

commands are outlined in the next section.

2.1  Command Descriptions

Commands  entered  from  "command  input  mode"  are   used

primarily to control file transfers.  There are also commands to

display  the  command  menu,  send  to  the  host  the   control

characters  which  are intercepted by TALK in terminal mode, and

exit the TALK program.  Commands are accepted in both upper  and

lower case.  A carriage return gets the user out of this mode if

he does not wish to enter a command at this time.

2.1.1   A -  Initiate file transfer to host

This  command  coordinates  a  number  of   operations

necessary  to  transfer  a  text file to the host computer.

The user is prompted for the desired host file name as well

as  the  local file name.  Redirected input mode is entered

and a command is  sent  to  the  host  to  begin  the  file

transfer.   When  the  end-of-file  is  reached,  the <EOF>

character is transmitted  to  the  host  to  terminate  the

transfer and redirected input is cancelled.  TALK currently

Program TALK Documentation                                PAGE 4

12/2/81

supports file transfers in this manner to  one  of  several

host  configurations:   the  Sigma 9 CP-V Operating System,

the  PDP-11  RSX-11M  Operating  System,  or  the   VAX/VMS

Operating   System.   The  desired  host  configuration  is

specified when the program is assembled,  by  defining  the

appropriate  symbol  in  the TALK prefix file ( see section

3.1).

2.1.2   B -  Initiate file transfer from host

This  command  performs  a  function  similar  to  the

previous  command,  but  is used to transfer files from the

host computer.  The same general procedure is used,  except

that  the transfer is terminated by the receipt of the host

computer's monitor prompt as the  first  character  in  any

line.  Although this may occasionally cause problems, files

can always be transfered in a "manual" mode  if  necessary.

During file transfers, <CR>, <LF>, and <DEL> characters are

not transfered to the output file.

2.1.3  C -  Transmit the break command character to the host

This command will transmit to  the  host  the  control

character  defined as the TALK break character for entering

command mode.

2.1.4  D -  Pause

This command allows the user  to  interrupt  TALK  and

communicate  with  the  Monitor  Console  Routine (MCR).  A

message is displayed on  the  terminal  indicating  how  to

restart  TALK and a "STOP" directive is issued to the task.

The program resumes execution  when  the  user  issues  the

"UNSTOP" command to MCR.

2.1.5  E -  Change EOR character

The user is prompted to enter  a  new  EOR  character.

The  EOR  character  is used to delimit records in both the

redirected input and output modes.  If an EOR character  is

recieved  from  the  host  and the input is redirected, the

Program TALK Documentation                                PAGE 5

12/2/81

next record from the input file is sent to the host.  Also,

in  redirected  input mode, an EOR character is appended to

each record sent to the  host.   If  an  EOR  character  is

recieved  from  the  host and the output is redirected, the

buffer  containing  all  characters  recieved   after   the

previous EOR is written to the output file.

The default EOR character is a carriage return.

2.1.6   I -  Enter redirected input mode

In this mode, the terminal is disconnected as an input

device,  and  data  is  transmitted  to  the  host  from  a

specified file.  The redirected file is transmitted  record

by record to the host on an EOR prompt basis.

Redirected input mode continues until the end of  file

is reached, a read error occurs (including reading a record

longer than INBSIZ, a symbol defined  in  the  TALK  source

file), or it is cancelled explicitly by breaking to command

mode and entering the command to cancel redirect input.

The default filename, TLKIN.,  is  used  if  the  user

enters no filename when prompted.

If the input is already redirected, this command  will

be ignored.

2.1.7  J -  Exit redirected input mode

If the input is currently redirected, the  input  file

is  closed,  and the terminal is reconnected to the host as

an input device.  This command will be ignored if the input

is not redirected.

2.1.8  K -  Transmit the "break" control character to the host

This command simply transmits  the  control  character

which  is  used as the command to transmit a "break" to the

host.

Program TALK Documentation                                PAGE 6

12/2/81

2.1.9  M -  Display the command menu

This  command  will  display  the  menu  of  available

commands on the terminal and reissue the prompt for command

input.

2.1.10  O -  Enter redirected output mode

In this mode, everything sent to the terminal from the

host  computer  will  also  be  copied to a specified file.

Records are terminated by EOR characters  or  by  exceeding

the buffer size, OFBSIZ ( defined in the TALK source file).

Redirected output files have the  "list"  carriage  control

attribute and can be editted normally.

Redirected  output  mode   continues   until   it   is

explicitly  cancelled  by  breaking  to  command  mode  and

entering the command to cancel redirected output.

The default filename, TLKOUT., is  used  if  the  user

enters no filename when prompted.

If the output is already redirected, this command will

be ignored.

2.1.11  P -  Exit redirected output mode

If the output is  currently  redirected,  any  partial

record  is  written  to  the  output  file  and the file is

closed.  This command will be ignored if the output is  not

redirected.

2.1.12  Q -  Transmit a control-Q (^Q) to the host

This command simply transmits the ^Q character to  the

host,  since  this character would otherwise be intercepted

by TALK as a command to  restart  the  processing  of  host

characters.

Program TALK Documentation                                PAGE 7

12/2/81

2.1.13  S -  Transmit a control-S (^S) to the host

This command simply transmits the ^S character to  the

host,  since  this character would otherwise be intercepted

by TALK as  a  command  to  halt  the  processing  of  host

characters.

2.1.14  X -  Exit redirected input and/or output mode

This command exits any redirected mode  (input  and/or

output) currently in effect.

2.1.15  Z or ^Z -  Exit from TALK

This command will close any  redirected  input  and/or

output  files  and terminate the program.  In addition, the

port will be disconnected.  If desired, TALK  may  then  be

rerun,  and  communication  with  the  host  will continue.

Exiting from TALK will allow local file preparation without

dropping  the  communications  line or logging off the host

computer.

Program TALK Documentation                                PAGE 8

12/2/81

3.0  Program Notes

3.1  Parameter and Code Modifications

Program command codes and  I/O  buffer  sizes  may  be

easily  changed.   These parameters are defined in the data

definition area of the macro source code and  in  the  TALK

prefix and system definition files.

The vector and CSR addresses for  each  port  must  be

defined  in  the  appropriate  tables.  See the TALK system

definition file (TALKSYS.MAC) for instructions on how to do

this.   The  system  definition  file also contains symbols

which define the host computer  for  which  TALK  is  being

used.   If a version of TALK is desired to communicate with

a PDP-11 RSX-11M system or a VAX/VMS  system,  comment  out

the  definition of the symbol SIGMA9 and remove the comment

indicator (;;;) from the definition of the symbol RSX11  or

VAX.

The QSIZET or QSIZEP parameters should be increased if

the  queue  overflow  message is displayed on the terminal.

Currently, the port queue size is 600  characters  and  the

terminal queue size is 200 characters.

The INBSIZ and OFBSIZ parameters  are  the  input  and

output buffers sizes for the redirected files.  The current

buffer sizes are 133 bytes for  input  and  133  bytes  for

output.

In addition to the system definition  file,  there  is

also  a  TALK prefix file (TALKPRE.MAC).  This file defines

some macros and symbols which  are  also  needed  by  TALK.

When  assembling  TALK,  the  prefix  file  and  the system

definition file should be assembled along with  the  source

modules  into the resulting object modules.  A command file

(TALK.CMD) is provided with TALK to perform  the  necessary

assembly and taskbuild operations.

3.2  General

Should  TALK  be  abnormally  terminated,   the   user

terminal  will  have  the  no-echo,  binary input, and full

duplex characteristics set.

Program TALK Documentation                                PAGE 9

12/2/81

4.0  Program Design

On an abstract level, TALK is a simple  program.   Separate

queues  are  used  for  characters  from  the  port and from the

terminal.  Characters from the terminal are handled  by  an  AST

routine, which queues the character and indicates that something

is in the queue.  Port  characters  are  handled  in  a  similar

manner through an "interrupt service routine" (ISR) via hardware

interrupts.  The "connect to interrupt vector" directive (CINT$)

is used.

When both of the queues are empty, TALK waits for an  event

flag  to be set by one of the interrupt routines.  This prevents

the program from  competing  for  the  CPU  when  there  are  no

characters  to be processed.  Once a character appears in one of

the queues, both queues are processed until empty.

Most prompted input from the terminal (EOR and file  names)

are  entered  when the program is in a special mode.  Instead of

being sent to the host, characters received from the terminal in

this mode are buffered to create an input string.  By reading in

this fashion, the unsolicited input feature need not  be  turned

off, and host characters may be processed while typing input.

The following sections contain the Program Design  Language

(PDL) code for the TALK program modules.

Program TALK Documentation                               PAGE 10

TALK PDL Source Code                                     12/2/81

4.1  PDL Source Code

TALK

initialize file i/o package

CALL CNPRT - connect port terminal

if

connect error

then

EXIT

else

CALL ATTDEV( TERMINAL ) - AST attach user terminal - read no echo

CALL CONTRL - handle all characters on queue

CALL DCNPRT - disconnect the port

CALL DETDEV( TERMINAL ) - detach user terminal - read with echo

if

redirected input

then

CALL CANINP - cancel redirected input

fi

if

redirected output

then

CALL CANOUT - cancel redirected output

fi

fi

stop

Program TALK Documentation                                               PAGE 11

TALK PDL Source Code                                                     12/2/81

AST -- unsolicited character interrupt handler

if

queue full

then

indicate queue overflow

else

put character on queue

if

pointer off queue

then

reset back of queue pointer

fi

fi

return from interrupt

Program TALK Documentation                                               PAGE 12

TALK PDL Source Code                                                     12/2/81

PTINTR -- port interrupt routine for character receive

disable interrupts

if

queue has overflowed

then

set error flag

else

move character to back of port queue

if

"back of queue" pointer is at its limit

then

reset it to the front

fi

fi

set flag to indicate port data

if

event flag needs to be set

then

set event flag indicating presence of something in the queue

fi

enable interrupts

return

Program TALK Documentation                                               PAGE 13

TALK PDL Source Code                                                     12/2/81

CONTRL

while

.TRUE.

do /* Loop forever */

if

port queue processing has not been suspended

then

wait for a signal that there is something in a queue

if

the port queue is not empty

then

if

the port queue has overflowed

then

WRITE 'QUEUE OVERFLOW' message to terminal

else

CALL PRTIN( CHARACTER ) - process a port character

if

the port queue is getting full

then

CALL PSLEEP to put the host to sleep

fi

fi

fi

fi

while

there is something in the terminal queue

do

if

terminal queue has overflowed

then

WRITE 'QUEUE OVERFLOW' message to terminal

else

CALL TRMIN( CHARACTER ) - process a user terminal character

if

wish to exit program

then

return

fi

fi

od

if

port queue processing is not suspended

then

if

port queue is empty

then

if

host is asleep

then

CALL PAWAKE - to wake up the host

fi

Program TALK Documentation                                               PAGE 14

TALK PDL Source Code                                                     12/2/81

if

there is a record to be sent to port

then

CALL XFERP to transmit it

fi

fi

fi

od

Program TALK Documentation                                               PAGE 15

TALK PDL Source Code                                                     12/2/81

PRTIN( CHARACTER )

if

a command is being sent to the host

then

echo the character

if

the character is a <cr>

then

reset the command flag

fi

else

if

input is redirected

then

if

character is a prompt for next file read

then

indicate a record is to be sent to port

fi

fi

echo the character

if

output is redirected

then

if

character is not <DEL>, <CR>, or <LF>

then

if

TALK is helping with file transfer

then

if

the first character in the line is a monitor prompt

then

CALL CANOUT - to cancel redirected output

return

fi

fi

move character to output file buffer

else

if

CHARACTER is EOR

then

CALL WRTBUF - to write a record to the output file

fi

fi

fi

fi

return

Program TALK Documentation                                               PAGE 16

TALK PDL Source Code                                                     12/2/81

TRMIN( CHARACTER )

case in the process of

/ building a file name /

CALL BUILD( CHARACTER ) - build file name

/ changing the EOR character

CALL ENTEOR( CHARACTER ) - change EOR character

/ entering a command /

CALL CMDIN( CHARACTER )

else

case CHARACTER of

/ enter "command input mode" command /

CALL BREAK

/ transmit break command /

CALL SNDBRK

/ suspend port queue processing command /

CALL PSLEEP

set "suspend" flag

/ resume port queue processing command /

clear "suspend" flag

/ suspend/resume terminal echo command /

if

TALK is helping with file transfer

then

toggle the echo flag

fi

else

if

not redirected input

then

WRITE CHARACTER to port

fi

esac

esac

return

Program TALK Documentation                                               PAGE 17

TALK PDL Source Code                                                     12/2/81

CMDIN

if

character is control-Q or control-S

then

return

fi

if

character is lower case

then

convert character to upper case

fi

if

not legal command character

then

report error to user

CALL BREAK - to prompt for new command

else

case  command to

/ transmit control character /

CALL SENDPR( control character )

/ initiate file transfer to host /

CALL RDIHLP

/ initiate file transfer from host /

CALL RDOHLP

/ display the command menu /

CALL MENU - to display the menu

CALL BREAK - to prompt for command

/ cancel redirected input and/or output /

CALL PAWAKE to wake up the host

CALL CANINP

CALL CANOUT

/ cancel redirected input /

CALL CANINP

/ cancel redirected output /

CALL CANOUT

/ change EOR character /

CALL ASKEOR - to prompt for new EOR character

/ exit program /

set exit flag

/ redirect input /

CALL REDINP

/ redirect output /

CALL REDOUT

esac

clear command flag

CALL PAWAKE - to wake up the host

fi

RETURN

Program TALK Documentation                                               PAGE 18

TALK PDL Source Code                                                     12/2/81

BREAK

CALL PSLEEP - to put the host to sleep

indicate that port queue processing is suspended

indicate command mode is in effect

issue the prompt to enter a command

RETURN

MENU

WRITE the command menu to the terminal

RETURN

Program TALK Documentation                                               PAGE 19

TALK PDL Source Code                                                     12/2/81

RDIHLP

prompt for destination file name

set flag to indicate that a redirected input command is being built

initialize the command length, file name length,

and their respective pointers

set flag to indicate that TALK is helping with a file transfer

RETURN

RDOHLP

prompt for source file name

set flag to indicate that a redirected output command is being built

initialize the command length, file name length,

and their respective pointers

set flag to indicate that TALK is helping with a file transfer

return

Program TALK Documentation                                               PAGE 20

TALK PDL Source Code                                                     12/2/81

PSLEEP

if

port output not already suspended

then

case

/ host is SIGMA9 /

send an <ESC> H to suspend output

/ host is RSX11M system /

send a ^S to suspend output

esac

fi

return

PAWAKE

clear flag indicating suspended output

case

/ host is SIGMA9 /

send an <ESC> to resume host output

/ host is an RSX11M system /

send a ^Q to resume host output

esac

return

Program TALK Documentation                                               PAGE 21

TALK PDL Source Code                                                     12/2/81

SNDBRK

set the "transmit break" bit in the transmit CSR

issue "mark time" directive to wait for one tick (1/60 second)

and then execute AST routine CLRBRK

if

directive status word indicates failure

then

clear the "transmit break" bit in the CSR

fi

return

CLRBRK

clear the "transmit break" bit in the transmit CSR

pop the stack

return from AST

Program TALK Documentation                                               PAGE 22

TALK PDL Source Code                                                     12/2/81

REDINP

if

not redirected input

then

indicate that we are building an input file name

initialize file name pointer

CALL ASKNM - prompt user to enter a file name

fi

return

REDOUT

if

not redirected output

then

indicate that we are building an output file name

initialize file name pointer

CALL ASKNM - prompt user to enter a file name

fi

return

Program TALK Documentation                                               PAGE 23

TALK PDL Source Code                                                     12/2/81

CANINP

if

redirected input

then

CALL CLSFLE( type = INPUT ) - close redirected input file

turn off redirected input indicator

fi

if

TALK is helping with file transfer

then

clear terminal echo flag

CALL SENDPR( <EOF> )

fi

clear help flag

return

CANOUT

if

redirected output

then

if

output file buffer is not empty

then

CALL WRTBUF - write output buffer to redirected output file

fi

CALL CLSFLE( type = OUTPUT ) - close redirected output file

turn off redirected output indicator

clear help flag and terminal echo flag

fi

return

Program TALK Documentation                                               PAGE 24

TALK PDL Source Code                                                     12/2/81

ASKNM

WRITE 'Enter file name:' to user terminal

initialize file name length to zero

return

Program TALK Documentation                                               PAGE 25

TALK PDL Source Code                                                     12/2/81

BUILD( CHARACTER )

if

CHARACTER is a DEL

then

erase last character written on the terminal

delete last character placed in file name buffer

else

if

CHARACTER is a CR

then /* End of file name */

send a <CR> to the terminal

if

building a host file name

then

append any trailer to the command string

if

transferring file to host

then

CALL REDINP

else

CALL REDOUT

fi

else /* building a local file name */

CALL OPNFLE - to open file

turn off build file name indicator

if

TALK is helping with file transfer

then

CALL SNDCMD( command string )

fi

fi

else

echo CHARACTER on user terminal

if

file name is longer than legal name

then /*Error, stop building file name */

WRITE 'File name is too long' to user terminal

turn off build file name indicator

else

place CHARACTER at the end of file name buffer

fi

fi

fi

return

Program TALK Documentation                                               PAGE 26

TALK PDL Source Code                                                     12/2/81

OPNFLE

prepare file name buffer

if

want to open an input file

then

open input file

if

not open error

then

turn on redirected input indicator

WRITE message indicating that input is redirected

if

TALK is not helping with a file transfer

then

indicate that there is a record to be transfered

fi

else

WRITE ' File open error' to user terminal

clear help flag

fi

else

open output file

if

not open error

then

turn on redirected output indicator

WRITE message indicating that output is redirected

else

WRITE 'FILE OPEN ERROR' to user terminal

clear help flag

fi

fi

return

Program TALK Documentation                                               PAGE 27

TALK PDL Source Code                                                     12/2/81

SNDCMD

get address and length of command

until

all characters are sent

do

CALL SENDPR( command character )

od

return

ASKEOR

WRITE 'Enter new EOR character' to user terminal

turn on change EOR character indicator

return

ENTEOR( CHARACTER )

echo CHARACTER to user terminal

replace EOR by CHARACTER

turn off change EOR character indicator

return

CLSFLE( TYPE )

close file of type = INPUT or OUTPUT

return

Program TALK Documentation                                               PAGE 28

TALK PDL Source Code                                                     12/2/81

XFERP

READ RECORD from redirected input file

if

not end of file and not read error

then

append an EOR character to RECORD

until

end of record

do

send a character to port

if

host is RSX11M system

then

wait for host to echo the character

fi

od

else

CALL CANINP - stop redirected input

fi

return

WRTBUF

WRITE output buffer to redirected output file

return

Program TALK Documentation                                               PAGE 29

TALK PDL Source Code                                                     12/2/81

CNPRT

set up DPB for terminal read/write

until

legal channel specified

do

if

channel not specified in the TALK invocation

then

prompt for and read in channel number

if

error on read

then

indicate error

return

fi

fi

od

connect to interrupt vectors

if

no errors on connecting

then

report successful connect

else

report failure to connect

set carry to indicate failure

fi

return

Program TALK Documentation                                               PAGE 30

TALK PDL Source Code                                                     12/2/81

ATTDEV( DEVICE )

save DEVICE characteristics

set new DEVICE characteristics

attach DEVICE with unsolicited character AST

return

DETDEV( DEVICE )

detach DEVICE

set DEVICE characteristics

return

Program TALK Documentation                               PAGE 31

TALK PDL Source Code                                     12/2/81

5.0  Building TALK

A command file  (TALK.CMD)  is  provided  with  TALK  which

performs  the  assembly and/or taskbuild operations necessary to

build the program.  This command file  should  be  used,  rather

than  performing  the  operations  manually,  to insure that the

required options are specified.