DDT-11 - Dynamic Debugging Technique on the PDP-11

DDT is a powerful debugging tool on the PDP11 that can

control execution of a program or display program or data in a

variety of formats. It can display or allow entry of data as

MACRO-11 instructions, octal or decimal constants, floating

point munbers, ASCII, RAD50, or, in some limited cases, hex

or other radices, or bytes of arbitrary length. It can single step

through programs, allow execution until breaks, or execute until

one or more memory locations changes. It allows users to define

symbols and displays addresses either absolutely or relative to

symbols. It can read symbol table files to automatically define

global symbols too, and caches these symbols internally to speed

searches.

Where DDT is too big to fit into a task's context,

it may be assembled and linked to run in a separate task under

RSX11M, RSX11D, RSX11M+, or IAS and to debug another task

or a collection of tasks from a debug task, with all facilities

intact and available and only minor speed loss. This makes DDT

suitable for all PDP11 debugging chores.

Under certain conditions, DDT may need to be sure tasks are

not shuffled too much, but it can operate in a mode ("pure send/

receive") that does not need any such constraint. It is able to

address a task directly, however, considerably faster than it

can pseudo-address the task by communicating with a "debug kernel",

and in this mode needs to be sure the task does not move until

a breakpoint is fully processed. These constraints apply only to

DDT debugging one task from another.

It should be noted that DDT can operate using pure send

and receive data directives between tasks, and in this mode is

not sensitive to shuffling/checkpointing, etc. However, single

stepping with a watchpoint was measured about 3 times slower in

"pure send/receive" mode than in "DDT mapping" mode (50 instructions

per second vs. 16 instructions per second, single stepping and

examining 1 memory location per step; representative of the speed

of handling normal breakpoints, but worse than normal single

stepping.

Each  command in DDT/11  is identical to the  PDP-10 version whenever

possible.   In some cases,  DDT/10 commands have been  omitted in the

interest  of saving core and  making good sense.  The  following is a

list of the DDT/11 commands:

The $ used in the examples below is an ESCape character. For ODT

compatibility, the ; (semicolon) character has the same effect as escape.

n/	Open word n.  Type contents in current mode

/	Open word pointed to by last typed value

n<CR>	Modify currently open word to be n.  Close word.

n<LF>	Modify, close, open next word

n^	Modify, close, open previous word

n<TAB>	Modify, close, open word pointed to by last typed address

n\	Modify, close, examine word n

\	Examine word pointed to by last typed value

n?	Type word n in RAD50 format (forces even address)

@	Types out location absolutely addressed by the open

location (the same convention as <tab>, and the

same as the DEC ODT convention for @ sign.

====> Important !!!! <====

Note that the following 9 commands are usually used with 2 escapes

(e.g. $$S) to set the modes "permanently" rather than only until the

next carriage return.

====> ************** <====

$S	Change current mode to instruction type out ("Symbolic")

$A	Addresses will be typed as absolute numbers ("Absolute")

$R	Addresses will be typed relative to symbols ("Relative")

$C	Change current mode to numeric type out	    ("Constant")

$nR	Change current radix for $C to n (n read in decimal)

$T	Change current mode to ASCII text output    ("Text")

$5T	Change current mode to Radix-50 text output

$nO	Change current mode to n bit byte typeout

$H	Change to halfword (byte) mode (reset by $5T and $S)

Note that the initial modes are $S and $8R.  These can be

temporarily changed by the above commands and will return

to the permanent setting by a carriage return.  The settings

can be permanently changed by using two altmodes before the

mode setting command ($$T sets permanent ASCII mode)

n[	Open location n, type as a numeric

n]	Open location n, type as instruction

n!	Open location n, do not type value

;	Retype currently open word

=	Retype currently open word as number

n=	Type value of n as a number

_	Retype currently open word as instruction

$_	Retype currently open word symbolically

i$X	Execute instruction i (cannot execute branches or jumps)

Note the instruction executes in the context of the

target task if multi-task DDT is in use.

n$G	Start executing program at location n.  Default is to

start executing at address in JOBSA (start of program)

v$nB	Set breakpoint n at address v (n from 1 to 8 or can be omitted)

a,v$nB	Same as above but open address a at breakpoint

If two altmodes are used, automatic proceding is enabled.  The

breakpoint will cause the message to be typed and the program

resumed.  If you type a character, the automatic proceding will stop.

NOTE: If the address a is less than 8 (the number of breakpoints

allowed), then macro a will be executed (0 to 8). These macros

are defined by the Y command and may consist of any characters

to be used as commands. There is however no conditional testing

built into DDT commands. These macros are additions to those defined

using the $UF, etc. commands and they never echo.

0$nB	Remove breakpoint n

$B	Remove all breakpoints

$P	Procede from breakpoint

n$P	Procede from breakpoint n times (without halting)

If two altmodes are used, the breakpoint will be proceded

from automatically.

n$UW	Set offset limit to n (used when typing addresses)

$1UT	Turn on single stepping (break at each instruction)

a$1UT	Turn on single stepping and open location a at each break

If two altmodes are used, the single stepping will be

done automatically.

NOTE: the same restriction about a less than 8 applies as

for breakpoints.

$UT	Turn off single stepping

a:	Make current location have symbolic name a

v<a:	Make symbolic name a have value v

a$$K	Delete symbolic name a

a$K	Delete symbolic name a from typeout only

$D	Perform an $K on the last symbol coloned (:)

l<u$$Z	Zero memory from l to u ;Location D.FILV Contains number filled in

1$nY& --- any text   &<space>

Define macro n as the characters between the &. To enter & use 2 &'s.

0$nY	Delete contents of macro n.

$nY	Execute macro n. Does not echo.

n$M	Set mask for searches (below) to n

l<u>s$W	Search core from l to u for word s.

l<u>s$E	Search core from l to u for effective address s.

l<u>s$N	Search core from l to u for word not equal to s.

addr$UInumber	Insert 32-bit floating-point number at addr

addr$UJnumber	Insert 64-bit floating point number at addr

addr$UX	EXamine floating point number at addr. Assumed to have

precision in D.PRCN (2 or 4 for 32 or 64 bits) which is

left over from last $UI or $UJ or set from console.

Note that floating AC's are saved/restored only if FPCTL is

set nonzero. If so, FPSTAT is FPU status and AC0-AC% are floating

AC's.

addr$J	Display contents of (addr, addr+2) as a 32 bit integer in current

radix.

addr$1Jnumber	Insert number at addr and addr+2 as a 32 bit integer.

number is delimited by any character not legal in the current

radix. The 32 bit integers are assumed stored as (low order,

high order) in memory (addr=l.o., addr+2=h.o.).

addr$2Jnumber	Insert number (in current radix) at addr as a 16 bit number.

This is provided in addition to the usual DDT methods since the

DDT parser cannot handle bases higher than 10. Thus, this

command gives a means to enter, as well as display, numbers in

higher bases, such as hex.

$L	Close open core image file. Must be done prior to exit from DDT.

$1L	Prompt for and open core image file to use instead of virtual

memory in $UM mode. CD.BLK is (lo,hi) base block number in file

(initially 1,0) and CD.BAS is lowest address in file. DDT converts

addresses to file relative form and reads or writes. If the

/RO switch is used, the file is read only, else read/write, and

it must be on SY:. If the /AB switch is used, DDT tries to do

read/write logical block on a device directly.

$UO	Prompt for name of symbol table file. If a file is specified and

exists, set to use it to resolve symbols. If file spec is null

or does not exist, close existing symbol table file.

$1UO will close any open symbol table file too but will erase

DDT's internal "cache" of symbols. $UQ leaves that cache intact.

$UQ	Close symbol table file if any is open.

$UM	Turn on use of virtual map. Variables PAR0 through PAR7 will be used

as APR's for all number entry or displays in any format.

Where I/D space support is used, $UM refers to data space

annd $1UM refers to instruction space.

$UU	Turn on virtual mapping and copy current User APR's into maps

used by DDT. All addressing will be done via those virtual

APR's.

$UK	Turn on virtual mapping and copy current Kernel APR's into

maps used by DDT. All mapping will be done via these APR's.

Note that if Kernel I and D space exists, $UK gets kernel D

APRs and $1UK gets I space kernel APRs.

$UV	Use standard DDT virtual mapping (no pseudo maps). This differs from

the other commands in that no APR's are used; the others use the

APR's that DDT is intialized with, which may be inadequate if the

task is checkpointed and moved. DDT initializes in this mode and

can, in it, access whatever a normal DDT can access (and conversely).

brk#<addr$UZ set memory break at addr. If brk# is higher than 8, clears

out memory break. Done with single step mode and infinite

proceed...slow. Note MEMCTL may be set 0 to turn off too.

If MEMCTL is nonzero, the default proceed count is made

infinite for single step, to allow these "watchpoints" to

work without excess user effort. Note further that the

watchpoint may be cleared EITHER by setting the address to

0 OR by a $UZ command with brk# out of range (10 or over

octal as normally assembled. A negative number will be

out of range too.) The method of setting the memory address

to 0 clears the one watchpoint but leaves the rest; using

an illegal brk# will clear ALL watchpoints out.

brk#<0$UZ Turn off memory watchpoint numbered brk#, leaving others on.

11<0$UZ Turn off all memory watchpoints. Does not automatically

leave single step mode however. Any number from 10 up, or

a negative number, will suffice to turn off watchpoints.

This effect can be obtained by putting 0 in the MEMCTL

variable in DDT also. This form clears all old watchpoint

addresses; zeroing MEMCTL will leave them intact for

later use if desired.

; The following are controls for DDT macros, that is, sets of

; DDT commands stored and executed by the single $UC command.

$UE	Turn on echoing of macros. Macros are stored in DDT space

at address D.KBBF as ASCII text using the standard DDT

text storage syntax, up to 80 characters. Macros should

end with negative bytes of data or the $US command to return

to standard input, though they should eventually return

with possible junk commands if this is omitted.

$UN	No echo of macros at the console.

$UC	Use core for input. Enables macros at D.KBBF to be read and

executed as though they came from the console.

$US	Use Standard input device (console) for command input.

$UF<delim>text<delim> Fill in text of macro (up to 80 char) with any

characters. Text is always read from console and the command

terminates, filling in a legal end sentinel, after buffer

full or recognition of the second delimiter character, which may

be any ASCII character not in the text string.

These macros are enabled by the D.MCRO assembly conditional

defined.

NOTE

DDT mapping does not affect program execution; only displays

are affected. Modifications to memory will use the current

APR's, but breakpoints will not be put into arbitrary memory

locations.

The $UM, $UV, $UU and $UK commands only exist where the

assembly conditional $VMEM was defined and DDT is linked to

a common in the I/O page at APR 7.

; The following command is used to point DDT at a task in the system.

; It is automatically used in cases where DDT is debugging another

; task.

$UL<delim>TSKNAM<delim>  Read a task name (6 characters maximum) from

the console (or macro) and locate its header and mapping

information inside RSX11M. Then compute the APR mappings for

that task and move them into the locations used by DDT for

its pseudo mapping. This option requires that R$XMDB be defined

and that DDT be assembled with $VMEM defined and with the

appropriate RSX macros (normally [1,1]EXEMC) available for

header, TCB, and window block offset definitions. Also, the

task using such a version of DDT must have access to the I/O

page via APR 7. This option only makes sense on fixed tasks

or in systems without checkpointing, but may be useful in

those systems for examining or modifying other tasks.

The $UM command is needed to actually use the virtual APRs

set up by the $UL command; $UL only sets them up for later

use.

Note that numbers can be as any combination of the following:

<Octal number>

<Decimal number> .

<Program symbol>

" <Delimeter> <Two ASCII Characters> <Delimeter>

"$ <Delimeter> <Three Radix-50 Characters> <Delimeter>

<Instruction>

The operators +  (addition), - (subtraction), * (multiplication), and

' (division) are evaluated  from left to right unless parenthesis are

included.  The space is treated like a plus except after instructions

when  it is merely a  separator.   All input is  accepted in lower or

upper case and is folded (except for text.)

In addition to program symbols, the following are also defined for you:

R0-R5	are the user registers.  R1/ would open register 1.

SP	is the user's stack pointer

PC	is the user's program counter

$I	is the user status

$2I	is the status of DDT

$M	is the mask for searches

$Q	is the value at the current address

$V	is the value at the current address with the bytes switched

$nB	is the address of breakpoint n

JOBSA	is the starting address of your program.

In multi-task DDT, U.PSW and U.DSW are the target task's PSW and $DSW

respectively. Then D.R0 thru D.PC are the DDT registers.

AC0-AC5 are the FPU registers where $D$FPU was defined at assembly

(Note however that instructions still print the symbols R0

through R5 instead of AC0-AC5.)

FPSTAT  is the FPU status register

FPCTL   controls FP save/restore. If 0, no save/restore of

floating registers occurs. if nonzero, all FPU status is

saved across calls. Initially the value of $D$FPU at assembly.

FPFMT   2 bytes for number decimals and width fo FP print field

D.PRCN  is the precision of FP print conversions. It may be 2 or 4

for single precision or double precision outputs.

RSXEXT IS RSX-11-M or -D EXIT$S call

$$DOS is DOS-11 EXIT call

DDTODD is DDT odd addr SST table entry (RSX DDT only)

DDTIOT is DDT IOT table entry

DDTTRP is TRAP entry (This is frequently assembled as 0 for FORTRAN OTS use)

DDTRES is RESERVED INST entry

DDTEMT is non-RSX EMT entry

D.FILV is number filled in by $Z command (initially 0)

(TASK NAME) is start address of task

BYE is equivalent to RSXEXT in RSX DDT

(in RSX, a control Z also exits DDT.)

DDTBGN is the start address of DDT itself

PAR0 Is the current virtual memory APR 0. (PDR is always 4K R/W)

PAR1	is the current virtual APR 1

PAR2	is current APR 2

PAR3	is current APR 3

PAR4	is current APR 4 (all these are in virtual ($UM) space

PAR5	is current APR 5

PAR6	is current APR 6

PAR7	is current APR 7 (must be I/O page for this to work)

MEMCTL  is the control for memory breakpoints. MEMBPA and MBPVAL

are addresses and last seen values for watchpoints which

are used if MEMCTL is nonzero. Note D.CT should be -1

and DDT should be in single step mode to use memory

watchpoints. The $UZ command establishes those

conditions.

SR.CTL	if 0 allows normal DDT mapping to arbitrary memory areas,

and if 1 allows send/receive data directives to be used

for communication with a target task (if MU$DBG defined only!)

OVRBRK	If present, may contain 0 or the address of a special

breakpoint. If nonzero, then if a breakpoint occurs from

the address in OVRBRK, only the data from OVRBRK will

be restored with the original instruction. It is expected

that a break at $ALBP2 will be used here (the autoload

completion point) so breakpoints may be redefined after

an overlay load.

Thus, to use this facility, one would load the

address of $ALBP2 into OVRBRK, then put a breakpoint at

$ALBP2. Then one would insert breakpoints into an overlay

and debug normally. When a new overlay occurs, and one

reaches the break at $ALBP2, remove all breaks that were

defined in the old overlay and insert new ones in the

new overlay. Then continue.

PRFCTL	If 0, turns off profiler (default) or if nonzero turns

profiler on. If the profiler is on, then execution will

be histogrammed. The address of each breakpoint (typically

a single step with many proceeds in the proceed count; -1

is infinite proceed count) will be compared with PRFBAS

and if above it will have PRFBAS subtracted from it. The

offset will then be shifted RIGHT by PRFSHF bits, and the

result converted into a word offset. The word of PRFHST

so found will be incremented, provided the offset is

within the bound of PRFSIZ. Breakpoint then proceeds

normally. Note that watchpoints also enable single step

mode and permit execution profiles to be done. Only

addresses of instructions will be monitored in this way

and the PRFHST array, internal to DDT, may be examined

at any time with the normal DDT commands. Since the PC

at break is always available, no intertask message overhead

occurs with profiler above that of single step overhead.

D.BMSK and

D.BTST	Used to support conditional breakpoints, in the following

sense: if the D.BMSK array word corresponding to breakpoint

number is nonzero, it is ANDed with the word opened at

the breakpoint and if the result is different from the

corresponding word in the D.BTST array (both are word arrays

generally 9 words long), the breakpoint will be ignored.

This is the correct way to allow a "stack" break, by entering

the current value of SP into D.BTST and opening SP at

each break, then single stepping until you hit the break

(D.BMSK should have 177777 in it, of course). This allows

one to step over a set of subroutines easily.

Up to 160. user symbols (as presently defined) may be entered.

.	represents the current location.  Thus .$1B puts

breakpoint 1 at the currently opened location.

When  your program is  executing, the following  unexpected traps may

occur.  NXM! is  caused when  your  program requests  a  non-existant

memory or does a word operation on an odd address.  ILG! is caused by

an  illegal instruction.  BPT! is  caused by an entry  to DDT that is

unexpected (i.e. executing the BPT instruction or setting the T-bit).

Also TRP! means a TRAP, EMT! means an EMT was seen, and a few others

indicate the type of trap. (SEG! means memory protect error.)

To include DDT with your task, just include on the input side

of the taskbuild. thus, whn your normal TKB would be

TKB MYFILE,MYFILE=MYFILE

make it look like this:

TKB MYFILE,MYFILE=MYFILE,[1,1]DDT/DA

DDT will gain control at startup. To start your program, type

<ESC>G where you type the ESC key, not the 5 characters "<ESC>".

To exit prematurely, type  BYE<ESC>G (again the ESC key).

A type-in of control Z will also exit DDT.

Note that DDT does not prompt with anything!

Where DDT is built to do its own echo, it echoes ESC as $. If not built

this way, DDT may not look like it echoes everything on VT52 or VT100

terminals, but will still function correctly.

Some instruction mnemonics in DDT are nonstandard to reduce

the size of the instruction database. The following are the

nonstandard mnemonics and their standard counterparts:

Nonstd	Standard

HLT	HALT

WAT	WAIT

RST	RESET

MK	MARK

FA	FADD

FS	FSUB

FM	FMUL

FD	FDIV

GD	MFPD

PD	MTPD

CC	CFCC

F	SETF

I	SETI

D	SETD

L	SETL

LS	LDFPS

SS	STFPS

S	STST

C	CLRF

T	TSTF

AB	ABSF

NG	NEGF

ML	MULF

MD	MODF

AD	ADDF

LD	LDF

SB	SUBF

CM	CMPF

ST	STF

DV	DIVF

SX	STEXP

SI	STCFI

SD	STCFD

LX	LDEXP

LI	LDCFI

LF	LDCDF

This completes the list of nonstandard DDT instruction mnemonics.

DDT was derived from the original program of S. Rubin @CMU for stand-

alone GDP PDP11s, and extensively modified and extended by G. Everhart

for DEC environments, bugfixes, floating point, multi-task use, symbol

table reads, hex radix, watchpoints, kernel mode dual mode support,

macros, and a few other things. There is a comment in the original

that credits an old version of ODT as the starting point S. Rubin used;

I haven't been able to find any of that old ODT code!

G. Everhart, 1/12/81

A command file DDTBLD.CMD is provided to build DDT more or less reasonably

for all operating systems of the RSX/IAS/VMS family. If you need more

help, there is some info about the conditionals of DDT assembly

in the front of the DDT22.MAC source file, and also a copy of most

of this file.