RUNOFF

BONNER LAB version

- i -

TABLE OF CONTENTS

CHAPTER I  INTRODUCTION                                                1

CHAPTER Ii  RSX/IAS OPERATING PROCEDURES                               2

Ii.1  INITIATING RUNOFF                                        2

Ii.2  RNO COMMAND STRING                                       2

CHAPTER Iii  SOURCE FILE FORMAT                                        6

Iii.1  CASE INFORMATION                                        7

Iii.2  SPECIAL CHARACTERS                                      8

CHAPTER Iv  INTRODUCTION TO THE COMMANDS                              11

CHAPTER V  RUNOFF COMMANDS                                            15

V.1  TEXT FORMATTING                                          16

V.2  FOOTNOTE / NOTE                                          18

V.3  PAGE FORMATTING                                          20

V.4  PAGE HEADERS                                             22

V.5  LISTS                                                    25

V.6  CHAPTER/APPENDIX FORMATTING                              26

V.7  SECTION HEADERS                                          28

V.8  DISPLAY COMMANDS                                         31

V.9  MODE SETTING                                             33

V.10  PARAMETER SETTING                                       37

V.11  TAB STOPS                                               39

V.12  FLAGS                                                   41

V.13  ENABLE/DISABLE                                          46

V.14  DEFINE / RESET                                          48

V.15  MISC COMMANDS                                           51

V.16  INDEXING                                                52

V.17  TABLE OF CONTENTS                                       53

V.18  EQUATION FORMATTING                                     55

V.19  CONDITIONAL TEXT                                        56

APPENDIX A  RUNOFF LIMITATIONS                                        58

APPENDIX B  FILL/JUSTIFY                                              59

APPENDIX C  RNO ERROR MESSAGES                                        60

APPENDIX D  BUILDING RUNOFF                                           66

APPENDIX E  SPECIAL CHARACTERS                                        69

APPENDIX F  LIST OF COMMANDS (ALPHABETICAL)                           70

APPENDIX G  NON STANDARD COMMANDS                                     74

APPENDIX H  SPECIAL PRINTER SUPPORT                                   75

APPENDIX I  INDEX                                                     77

- ii -

RUNOFF

Revision Date:  May 11 1983

Program Version:  BL3.6

Document Revision :  May 1983

Document Printed :  13 May 83 10:52:24

NOTICE:   This  document describes a program which the Digital Equipment

Corporation has no committment to offer or support at this  time.   This

document is, however, believed to be an accurate description of RUNOFF.

This version of RUNOFF is a major revision of the RNO working group ver-

sion.  The revisions essentially add a large number of features from DSR

to RUNOFF while maintaining compatability.  There are 3 known incompati-

ble changes, to the .CENTER , .NUMBER, and .FOOTNOTE commands.  The  ad-

ditions  include the .REQUIRE command, overstrike capability, and escape

sequence handling.  A macro facility via the SUBSTITUTE  flag  has  been

added and permanent margins are also implemented.  The permanent margins

can cause incompatible output with respect to the headers  if  they  are

not  set.   In  addition  all  FLAGS  are  redefinable  and  many can be

enabled/disabled.  The escape sequence handling  has  been  designed  to

give  the  user  the ability to define arbitrary sequences, and intermix

them with standard RNO text.  This is capable of handling any  arbitrary

printer.   Since both this version of RNO and the previous versions used

ASCII non printing characters internally, including such  characters  in

your  input  file  could  be disastrous.  To prevent this all such char-

acters with the exception of CR,LF,TAB are ignored.

The philosophy of this version of RNO has been to create a program which

can handle virtually any printer with minimal support for  any  specific

printer.   As a result of this decision general mechanisms have been in-

cluded for printer control, but only suggested escape  sequence  defini-

tions  have  been  added as part of the permanent escape sequence table.

The necessity for added pre or post processors has been avoided  by  ad-

ding  the  /CR  switch, and by adding features which allow RNO to handle

the table of contents directly.  New features such  as  multiple  column

output would directly conflict with the general escape sequence handling

mechanisms.  For example font changes would spill over onto adjacent co-

lumns  rather than continue within a column.  As a bonus this version of

RNO can be used in a multiple pass manner.  The output text may  be  fed

back into RNO as input text, after being edited.

If you have any problems with this version I will be glad to answer your

questions.  I do not guarantee that all errors can be patched.   I  will

try  to  maintain  this  version and release new versions with most bugs

fixed and some enhancements added.  If there are other users who need to

use this on RSTS or RT-11 I will answer any questions you have about the

conversion, and I will consider including the added code to conditional-

ize this version to other operating systems.

- iii -

John Clement

T. W. Bonner Nuclear Lab

Rice University

P.O. Box 1892

Houston, Tx, 77251

Tel (713) 527-4018

- iv -

CHAPTER I

INTRODUCTION

RUNOFF  (RNO) is a PDP-11 program to facilitate the preparation of typed

or printed manuscripts, such as memos, manuals, etc.  The user  prepares

his  material  on any regular PDP-11 terminal, and writes it onto a file

using an appropriate editor.  The user includes not only textual materi-

al,  but  also  case  and formatting information.  RUNOFF then takes the

file and reproduces it onto the line printer, terminal or other file  to

produce  a  final  copy or final file image.  It performs the formatting

and case shifting as directed, and will also perform line justification,

page numbering and titling, etc., as desired.

The  principal  benefit of such a program is that files prepared for use

with it may be easily edited and corrected.  Small or large  amounts  of

material may be added or deleted, and unchanged material need not be re-

typed.  After a set of changes, the program may be operated to produce a

new  copy which is properly paged and formatted.  Documentation may thus

be updated as necessary without requiring extensive retyping.

- 1 -

CHAPTER Ii

RSX/IAS OPERATING PROCEDURES

Ii.1  INITIATING RUNOFF

RUNOFF can be initiated in several ways.  The general formats for RSX-11

users are:

General and Privileged Users

MCR>RNO CR>

MCR>RNO filespec

Privileged User Only

MCR>RUN ...RNO/UIC=[uic]

MCR>RUN $RNO/UIC=[uic]

Ii.2  RNO COMMAND STRING

System  programs usually require a command string to specify such things

as input files and various options.  A standard command string  has  one

of the following forms:

output-file,toc-file=input-file

or

@command-file

The  output-file, toc-file, and input-file are strings comprising a file

specification.  The toc-file may be omitted if no table of  contents  is

desired.

The  second form of command string (@command-file) is used to specify an

indirect command file.  In this case each record of the file is used  as

a  command string before any commands following the indirect command are

accepted.

The  command  string  syntax  is described more completely in the User's

Guide for the relevant operating system.

- 2 -

RSX/IAS OPERATING PROCEDURES

RNO COMMAND STRING

Ii.2.1  File specifications

An RSX-11 file specification has the following form:

Dev:[UIC]Filename.Typ;Ver

In  this  specification  Dev  must  be a legal device code.  The default

value is SY:, the system disk.  UIC must be a valid User  Identification

Code.  Filename is any 1- to 9-character alphanumeric string which spec-

ifies a file name.  Typ is a 1- to 3-character alphanumeric  file  type.

System  programs  default to an appropriate standard type so the typical

user will not have to explicitly specify it.  Finally, Ver is  a  number

which  is  the version number of the file.  This field also normally can

be omitted since it defaults on input to the  highest  existing  version

number for the file and on output to the highest existing version number

plus one.

Default file specifications (any part of which may be overridden):

Input:   SY:RUNOFF.RNO

Output:  SY:RUNOFF.DOC

Table of contents:  SY:RUNOFF.RNT

Editing RUNOFF Output

RUNOFF  output files may not be edited normally with the stan-

dard RSX/IAS file editors.  This version may produce  editable

output  by  using the /-CR switch, when the text is processed.

If You do this /UL:L will not work but the output  may  easily

be  edited.   If  this switch is used the ouput text/line must

not exceed 256 characters, the output  buffer  size.   If  you

need  to  use  this feature with more than 256 characters/line

see BUILDING RNO on how to change the output buffer size.

Ii.2.2  RNO SWITCHES

A  file  specification may have one or more switches associated with it.

A switch is an indicator, consisting of  a  slash  and  a  two-character

ASCII  name, used to select program options.  The switch itself may have

three forms.  If the switch designator, for example, is SW, then:

/SW sets the switch action;

/-SW negates the switch action, and

/NOSW also negates the switch action.

In  addition  the switch identifier may be followed by any number of va-

lues.  The permitted values  are  ASCII  strings  numbers,  and  decimal

numbers.   Any  numeric value may be preceded with a + or a - sign;  The

following are valid switch specifications.

/SW:27:XXX:29

/-SW

- 3 -

RSX/IAS OPERATING PROCEDURES

RNO COMMAND STRING

/NOSW:NOSWITCH:-50

- 4 -

RSX/IAS OPERATING PROCEDURES

RNO COMMAND STRING

Eleven  switches  control  the  RNO  program.  These switches, described

below, may be specified on either the input or  output  file  specifica-

tion.  the defaults may be set by the system manager at Task Build time.

/SP     -- Spool the output file

/-SP    -- Inhibit spooling of output file

/UC     -- Force ALL uppercase output

/-UC    -- Allow normal case conversion

/FF     -- Force Form-feeds (instead of multi-LFs)

/-FF:n  -- Simulate  Form-feeds  with Line-feeds n is paper length,

default set by your system manager.

/WA     -- Wait for CR input at beginning of each page

/-WA    -- Continue printing each page with no wait

/PA:l:h -- Output only indicated page range

l = lowest page to print

h = highest page to print

default = print all

This switch should be used with the /AP,/CH switches for

chapter oriented text.

/CH:l:h -- Output only indicated chapter range.

/AP:l:h -- Output only indicated appendix range.

/PS:H:W -- Set page size to H,W (height,width)

Default is /PS:58:60 (See PAGE SIZE command)

/HY     -- Hyphenate output

/-HY    -- Do not hyphenate output

/RI:n   -- Right shift output.  n=amount of shift

/UL:B   -- Underline with a spacing-backspace

/UL:L   -- Underline via line overprint with underline character.

/UL:S   -- Simulate underline with hyphen character on next line.

/UL:N   -- Underlining suppressed.

/CR     -- Output CR,LF in file.

/-CR    -- Output  regular  text  (No CR,LF).  This doesn't work if

the output line exceeds 256 characters.

- 5 -

CHAPTER Iii

SOURCE FILE FORMAT

The  source  file contains the textual material which will appear on the

final copy, plus information to specify formatting.   Most  importantly,

upper  and  lower case information also may be supplied so that copy can

be prepared on the terminal or other such device which  can  input  only

upper  case  letters.  All command information consists of regular ASCII

printing characters so that a listing of the source file may be examined

if the final copy is not exactly as desired.

All  material in the source file is taken to be source text except those

lines beginning with a period.  A line beginning with a  period  is  as-

sumed  to be a command, and must match one of those listed in chapter 4.

The commands provide the formatting information, and control various op-

tional modes of operation.

Usually  the  text is filled and justified as it is processed.  That is,

the program FILLS a line by adding successive words from the source text

until  one  more  word would cause the right margin to be exceeded.  The

line is then JUSTIFIED by making the word spacings larger until the last

word in the line exactly meets the right margin.

The  user  may  occasionally  wish to reproduce the source text exactly,

which is done by disabling filling and justification or by  use  of  the

LITERAL  command.   The  program  may be set to fill but not justify, in

which case the output will be normal except that lines will not be  jus-

tified  to the right margin.  The program may also be set to justify but

not fill, although this would probably produce peculiar results  and  is

not recommended.

When  the fill mode is on, spaces, and carriage returns occurring in the

source text are treated only as word  separators.   Multiple  separators

are  ignored.  In addition an extra space is added to punctuation occur-

ring at the end of a sentence.

Some  of  the  commands cause a BREAK in the output.  A break means that

the current line is output without justification, and the next word goes

at  the  beginning  of  the  next line.  This occurs at the end of para-

graphs.

The  program  will  advance to new pages as necessary, placing the title

(if given) and the page number at the top of each page.   The  user  may

call  explicitly  for  a page advance where desired, and may inhibit the

occurrence of a page advance within specified material.

- 6 -

SOURCE FILE FORMAT

CASE INFORMATION

Iii.1  CASE INFORMATION

NOTE

The  following  section explains how to change the case of the

input text.  If you have a terminal with a shift  key  you  do

not need to read this section.

Specification  of  case  for files prepared on the terminal is done with

two characters, circumflex (^), and back-slash (\).  The appearance of a

circumflex  causes the letter immediately following to be transmitted in

upper case.  The appearance of a back-slash causes the letter immediate-

ly following to be converted to lower case.  Any letter not preceeded by

one of these characters is transmitted in the current mode.  The mode is

initially upper case, and is changed by the occurrance of two successive

case control characters.  Two circumflexes (^^) cause the mode to be set

to  upper  case,  and  two back-slashes (\\) cause the mode to be set to

lower case.

When  the text is in upper case mode (^^) the text is printed exactly as

you typed it in.  This means that no case  conversion  occurrs.   If  in

lower  case  mode  (\\) all text is converted to lower case except where

explicitly specified as upper case by a  single  upper  shift  character

(^).

The following shows the uses of the case control characters:

H\\ERE IS A ^SAMPLE ^SENTENCE IN ^^UPPER CASE\\ AND LOWER CASE.

becomes:

Here is a Sample Sentence in UPPER CASE and lower case.

An  additional  character, less-than (<), capitalizes the entire word it

precedes.  It then returns the file to the current case.  This character

is  not engaged unless preceded by the FLAGS CAPITALIZE command.  If the

capitalization character is engaged, you may capitalize a whole  section

of  text  by  inserting  ^<  in front of the text and ^^ after the text.

Similarly to the lower case lock a single character  may  be  forced  to

lower case rather than upper case by preceeding it with (\).

These case change control symbols always override the actual case of the

input text and also the implied case of a command.  For example the com-

mand  .HEADER LEVEL would normally produce a capitalized title for level

number 1.  This may be changed by preceeding the title with \\ to  force

it to lower case.

- 7 -

SOURCE FILE FORMAT

SPECIAL CHARACTERS

Iii.2  SPECIAL CHARACTERS

&  Ampersand     UNDERLINE

The  character  ampersand  (&) is used to specify underlining.  The

ampersand will cause the character following it to  be  underlined,

e.g. &f&o&o  becomes  foo.  To begin underlining a whole section of

---

text you preceed the section by ^& and follow the  section  by  \&.

Any  spaces  in the section will not be underlined.  To underline a

single space preceed it by &.  Text with  imbedded  spaces  may  be

underlined spaces and all by issuing the command .UNDERLINE SPACES.

#  Number Sign   Quoted or explicit SPACE

It  is  occasionally  necessary to include spaces in the text which

should not be treated as word separators.  For this purpose, RUNOFF

treats  the  number-sign character (#) as a quoted space;  i.e.  it

will print as exactly one space in the output.  Normal  spaces  may

be  expanded  to  multiple  spaces  to justify the text, but quoted

spaces may not be expanded.  Also normal spaces mark the end of the

previous  word  and  the beginning of the next.  Text with imbedded

quoted spaces may not be broken at the quoted spaces and split into

separate lines.

_  Underline     ACCEPT or Quote character

To  allow  the  appearance  of  the  special characters (ampersand,

number-sign, circumflex, or back-slash) in the output,  the  under-

line character (_) is used as a quote character.  The character im-

mediately following an underline will be transmitted to the  output

with  no  formatting  effect.  The underline itself is thus another

case requiring quoting.  The following five cases occur:   _&,  _^,

_\, _, _$, _|, _=, _%, _<, and_#.

If  normal spaces or tabs are preceeded by an ACCEPT character they

are converted to quoted spaces.

^  Circumflex    UPPERCASE shift or mode lock

As described above, the circumflex character (^) is used to convert

the letter following to upper-case.  It is also used  to  lock  the

case  mode  in upper case, and the underline mode to "underline all

text".  If it is to appear in the printed text, it must be preceded

by the quote character (_^).

\   Backslash    LOWERCASE shift or mode unlock

As  described  above, the backslash character (\) is used to output

the letter following in lower-case.  It is also used  to  lock  the

case  mode  in lower-case, and to disable underlining.  If it is to

appear in the printed text, it must be preceded by the quote  char-

acter (_\).

- 8 -

SOURCE FILE FORMAT

SPECIAL CHARACTERS

<  Less-than     CAPITALIZE next word

If  .FLAGS CAPITALIZE has been engaged, the less-than character (<)

is a special character used to capitalize the entire word  it  pre-

cedes.  If it is to appear in the printed text, it must be preceded

by the quote character (_<).  An appearance of  a  circumflex  fol-

lowed  by  less  than (^<) permanently locks Runoff into capitalize

mode.  This may be cancelled by a double circumflex (^^).

=  Equals-sign   HYPHENATE character

If .FLAGS HYPHENATE has been engaged, the equals character (=) used

to disable automatic hyphenation for the word it preceeds.   If  it

is  to appear in the printed text, it must be preceded by the quote

character (_=).  If it is used in the middle of a word,  the  auto-

matic  hyphenation  is  not used, and instead the equals sign marks

where hyphenation may occurr.

%  Percent sign   OVERSTRIKE character

If  .FLAGS  OVERSTRIKE  has been engaged, the percent character (%)

will generate a backspace to allow overstriking the previous  char-

acter  to  form new composite characters.  When overstriking is en-

abled, this character must be preceeded  by  the  quote  character.

The  default  is  .NO FLAGS OVERSTRIKE.  Multiple backspaces may be

generated, but you should be careful since the characters following

the  multiple backspace can cause unpredictable results if they are

not printable characters.

.  Period   CONTROL character

This  flag  marks  the rest of a line as a control or command if it

appears at the beginning of a line.

$  Dollar sign   SUBSTITUTE character

If  .FLAGS  SUBSTITUTE  has been enabled then the dollar sign marks

the beginning of a substitution label.  Runoff will then substitute

some  specified  text  for the label.  The text may be specified by

the .DEFINE SUBSTITUTE command.

|  Vertical bar   BREAK character

If .FLAGS BREAK is enabled this character marks where a word or ex-

pression may be broken.  A word containing a break will not be  hy-

phenated,  but  will  be broken if necessary.  This flag also marks

where a string is justified if .TAB RIGHT is enabled.

NOTE

All  of  these  flags may be disabled by a .NO FLAGS com-

mand.  They may be enabled and  changed  to  a  different

character  by  a  .FLAGS command.  For example if you use

the equals sign frequently you may change it to the  dou-

ble  quotes which you never use by entering the following

line.

- 9 -

SOURCE FILE FORMAT

SPECIAL CHARACTERS

.FLAGS HYPHENATE "

- 10 -

CHAPTER Iv

INTRODUCTION TO THE COMMANDS

When  you  first  use  RUNOFF  the wealth of available commands may seem

confusing , but this wealth may be used to speed the creation  of  docu-

ments.  The first thing you must learn is how to create simple documents

such as letters.  To do this you must learn the following commands.

1.  .RIGHT MARGIN to set the right margin.

2.  .LEFT MARGIN sets the left margin.

3.  .TOP MARGIN sets the top margin on the paper.

4.  .PAGE  SIZE combines all of the above and sets the page length.

This command should always be used if you wish to produce docu-

ments with the pages numbered.

5.  .PARAGRAPH starts paragraphs.

6.  .CENTER centers a line of text.

7.  .RIGHT right justifies a line of text.

8.  .INDENT indents the next line.

9.  .SPACING sets the spacing to single, double, triple ....

10.  .HALF  SPACING  sets  RNO to produce in between spacing such as

11/2, 21/2 ....

-     -

11.  .SKIP  skips  a  line (or 2 if double spacing, 3 etc.) In other

words skip skips lines according to the spacing selected.

12.  .BLANK skips multiple lines independent of spacing.

13.  .FIGURE  reserves  n lines independent of spacing for inserting

drawings, or other artwork later.

14.  .PAGE starts a new page.

You should read the sections on how .FILL, .JUSTIFY, .BREAK, and .ENABLE

HYPHENATION all work.  These commands form the  basic  set  of  commands

which  automatically  place the text on the page in a "pleasing manner".

For example .FILL will automatically make sure each line has enough text

to go all the way to the right margin.

The  task of setting up paragraphs may be simplified by using the .AUTO-

PARAGRAPH command.  Once this command has been given any line  preceeded

by  a  blank,tab,  or a blank line automatically starts a new paragraph.

By using this command the input text may be formatted to "look" like the

final  document.   The .SET PARAGRAPH command may be used in conjunction

with the .AUTOPARAGRAPH to control the format of each paragraph.

One  of  the  most frequently asked question is:  How do I do bibliogra-

phies or tables?  This is usually a matter of setting up a negative  in-

dentation and adding the same extra extra to the left margin.  For exam-

ple you issue .LM +5 .SET PARAGRAPH -5.  Every time you  begin  a  para-

graph  the  line will start on the old left margin, and the text will be

indented.  The text will look like the following.

.SET PARAGRAPH [-indent],[-skip],[test page]

this  command  sets the format produced by the .PARAGRAPH

command, or the .AUTOPARAGRAPH feature.  The  indentation

may be either positive or negative.

- 11 -

INTRODUCTION TO THE COMMANDS

Next  you  should  learn  how  to control the overall look of the pages.

This   is   controlled   by   the   .HEADER   commands,   .LAYOUT,   and

.TITLE/.SUBTITLE  commands.  The basic page contains a title/subtitle at

the top with a page number.  The location of the title/subtitle and  the

page number may be varied using the .LAYOUT command.  This allows 8 dif-

ferent placements.  The title and subtitle are 2 lines of text which ap-

pear  at  the top of each page.  The text may be specified by the .TITLE

or .SUBTITLE command.  If no title is specified none appears.  In  addi-

tion  the  .NO  TITLE  command will suppress both the title and subtitle

without suppressing the page number.  The  .NO  HEADER  command  on  the

other  hand  will  suppress  title,subtitle,  and  page number.  The .NO

NUMBER PAGE command may be used to suppress the page number without  af-

fecting  the  title/subtitle.   Normally the page number is preceeded by

the word "Page".  If you want only a number and no extra word the  .DIS-

PLAY  NUMBER command can be used to suppress the word "Page".  The .DIS-

PLAY NUMBER command can substitute some other text for the  word  "Page"

and  can  add extra text after the page number.  For example, this could

be used to number the pages of a 4  page  document  as  1/4,2/4,3/4,4/4.

The  .DISPLAY command can also switch page numbers to other forms.  Nor-

mally the first page of a document  will  not  have  the  title/subtitle

printed on it.  This may be changed by using the .FIRST TITLE command.

A  common problem with text is arranging it on each page.  Often you may

wish to keep a section of text all on the same page  without  having  it

split between 2 pages.  This may be achieved by using the .TEXT and .END

TEXT commands.  An alternate way to guarantee a fixed  number  of  lines

all  on  the  same page is by use of the .TEST TEXT command or the .TEST

PAGE command.  You should be aware that the .PARAGRAPH  command  has  an

automatic .TEST TEXT built into it.

Once  you are able to basically format a letter or simple few page docu-

ment, you should begin to learn some of the shortcuts.  Often  you  need

to  produce  lists  such as the list above of simple commands.  The easy

way to do this is by using 3 commands .LIST,  .LIST  ELEMENT,  and  .END

LIST.  These commands make creation and modification of lists simple and

perhaps even enjoyable.

1.  .LIST is the command to begin a list.

2.  .LIST  ELEMENT  starts  each entry in the list.  If you need to

insert more entries in the list you  need  only  insert  a  new

.LIST  ELEMENT command with the added text and the list will be

appropriately numbered.

3.  .END LIST ends the list.

The  file LIST.TST contains an example of a properly formatted list.  If

you need to produce lists preceeded by Roman numerals or letters  rather

than numbers the .DISPLAY ELEMENTS command allows you to change the list

format.  This is illustrated in the file DISLIST.RNO.

You  may  wish  to format a large document.  This is most easily done by

breaking it up into sections.  Each section may be combined by using the

.REQUIRE command.  You would make a master file containing just the com-

mand .REQUIRE "filename".  For each file you wish to combine you  insert

the  .REQUIRE  with  the finlename being the name of each file with text

you wish to print.  If you later on decide that you must  reshuffle  the

various  sections  it is easily done by rearranging the various .REQUIRE

statements.

Another  thing you may do in a large document is divide it into chapters

and appendices.  The best way to do this is by using  the  .CHAPTER  and

- 12 -

INTRODUCTION TO THE COMMANDS

.APPENDIX  commands.   These commands automatically format the beginning

of each chapter in identical fashion and the chapters  are  numbered  in

the  order  in  which  they  occurr.   If later you need to insert a new

chapter this is easily done because the chapters are then  automatically

renumbered.   The  "look"  of the chapters may be modified by the .STYLE

CHAPTER command.  This sets up the margins, and placement of the chapter

number  and  title.   Some  documents may need the page number placed in

different locations depending on whether the current page is  the  first

page  of  a chapter.  The .CHAPTER LAYOUT command may be used to control

the first page of each chapter independently of subsequent pages.  .DIS-

PLAY CHAPTER and .DISPLAY APPENDIX may be used to select either letters,

ruman numerals, or normal Arabic numerals for numbering the chapters and

appendices.   Whenever  a doucument is chapter oriented the page numbers

are then printed as chapter-page.  For example the 5'th page in  chapter

2  would  be  numbered  2-5.   If  sequential page numbering without the

chapter numbers is desired, then .NO NUMBER CHAPTER should be used.

Often  you  wish  to  divide  a document into sections in addition to or

instead of chapters.  Each section is started with a section header con-

sisting  of  a number followed by a title.  A section number is a string

of numbers separated by periods.  For example 3.2.5 or 1.2 and so on.  A

string of 3 numbers is called the third header level, 2 numbers the sec-

ond and 1 the first.  RUNOFF can automatically generate headers by using

the .HEADER LEVEL command.  This is illustrated by the file .HEADER.TST.

Using the header level command allows you to easily insert more sections

because  RUNOFF  will then renumber all the remaining sections automati-

cally.  Headers can be reformatted by the .STYLE HEADERS command.   This

controls the spacing and placement of the headers.  In addition the .IN-

DENT LEVELS command indents the left and right margins on the headers as

desired.  If you need to use letters or Roman numerals instead of normal

decimal numbers you may use the .DISPLAY LEVELS command  to  change  the

type of numeral.

The  next page number or chapter number and so on may be assigned by the

.NUMBER commands.  These assign numbers for the document.   In  addition

to pages subpages may be numbered.  This allows insertion of extra pages

without reformatting and reprinting the  whole  document.   For  example

after  page  12  you  may insert 12A, 12B, and 12C for a total of 3 more

subpages.  This is done by issuing the .SUBPAGE and  .END  SUBPAGE  com-

mands.   The actual subpage number may be selected with the .NUMBER SUB-

PAGE command.  The .NUMBER commands make skipping pages  possible.   You

should  also  be aware that it is possible to format only selected pages

and chapters by using the CH,AP, and PA switches.

Text  may be underlined by using the underline flag.  This is controlled

by the .FLAGS UNDERLINE, .ENABLE UNDERLINING  and  .DISABLE  UNDERLINING

commands.   Likewise  a whole cornicopia (or Pandoras box) of extra fea-

tures may be controlled by the .FLAGS commands.  These allow you to  en-

able and redefine special symbols to perform various functions as under-

lining, capitalization, hyphenation, overstriking, breaking, and substi-

tuting.   The  .ENABLE  or  .DISABLE commands let you control when these

features are used.  Most of these features are self explanatory.   Three

of them are explained a little further here.

1.  OVERSTRIKING is where more than 1 symbol is printed in the same

spot.  For example overstriking an oh "O" with a minus sign "-"

can produce the Greek letter theta.

2.  BREAK  is  a special way of telling RUNOFF where it may break a

line as opposed to  hyphenating  it.   This  is  also  used  to

- 13 -

INTRODUCTION TO THE COMMANDS

control tabulation.

3.  SUBSTITUTION  is  a way of defining abbreviations for long com-

monly used lines of text, or symbols.  This is most convenient-

ly  used to create complex symbols which may be printed on spe-

cial line printers.  See .DEFINE SUBSTITUTE, .RESET SUBSTITUTE,

.FLAGS SUBSTITUTE, and .ENABLE SUBSTITUTION

Sometimes  you  need to type a table in column format.  This can be par-

ticularly onerous even with the tab stops on a typewriter.  RUNOFF  con-

tains  tab stops settable by the .TAB STOPS command.  In addition RUNOFF

can right justify columns or even  center  justify  columns  using  .TAB

STOPS,  .TAB  RIGHT,  and  .FLAGS  BREAK.  An example of some of this is

shown in the file TAB.TST.  You may want to connect items in a  list  by

ellipses  to guide the eye.  The command .ELLIPSES will fill in ellipses

rather than spaces whenever you tab to the next item.

You  should  be  aware  that RUNOFF can automatically take care of foot-

notes.  The .FOOTNOTE command allows you to type  the  footnote  at  the

same  point  it  is  referenced, and it will appear at the bootom of the

page.  In addition RUNOFF can produce an INDEX to your document and also

a  table  of  contents.  To do this you should have some experience with

the aforementioned features.  The .RNO files included with this document

give examples of most of these features.

Since  this version of RUNOFF is almost the same as Digital Standard Ru-

noff (DSR), the DSR manual can be used a guide to most of the basic fea-

tures.   VAX/VMS  users  may  prefer  this version to DSR because of the

extra features available to customize documents.  RSX or IAS users  have

no  choice  but  to  use one of the versions of RUNOFF available through

DECUS.

- 14 -

CHAPTER V

RUNOFF COMMANDS

The  following  commands will be recognized if they are at the beginning

of a line started with a period.  Any line in the source file  beginning

with  a period is assumed to be one of these commands.  If it is not, an

error diagnostic will be typed and the line will be ignored.  Some  com-

mands  take  one or more decimal numeric arguments.  These are separated

from the command by a space.  Multiple arguments  may  be  separated  by

spaces,tabs,  or commas.  If the parameters are separated by commas, any

number of tabs or spaces may appear before or after the comma.   If  the

argument  is  preceeded  by a minus sign (-) then it may be entered as a

negative value.  If it is preceeded by +- then it may be a relative  ar-

gument.   A  relative  argument  changes the old value by the + or - the

value specified.  Square are [] brackets are  used  to  denote  optional

syntax elements.

More than one command may be entered on a single line.  The commands may

be optionally separated by a semicolon (;).  If the semicolon is used as

a separator the next command must immediately follow with no intervening

spaces.  Commands which are followed by text may be the last command  on

a  line,  but  may  not have other commands after them on the same line.

For example the following line is a legal set of commands all on 1 line:

.TP 10.Fill;.EH .Title GONE WITH THE WIND

Multi-word  commands may appear in any form.  Thus, .NO HEADER and .NOH-

EADER are both legal.  Many commands may be abbreviated.   Standard  ab-

breviations are given below each command.

Commands  may  have  text  following them provided the text is separated

from the command by a semicolon (;).  For  example  the  following  line

will produce a centered title.

.C;  THIS IS A TITLE

The  semicolon  is essentially a separator which indicates that the rest

of the line is to be treated just as if a new line had begun.

For  example  the  following  forms  of  the .PAGE SIZE command area all

legal.

.PAGE SIZE 20,20

.PAGE SIZE 20 , 20

.PAGESIZE 20 20

.PS ,,1,2

.ps , , 1 2

- 15 -

RUNOFF COMMANDS

V.1  TEXT FORMATTING

.BREAK

.BR

causes  a  break, i.e. the current line will be output with no jus-

tification, and the next word of the source text will be placed  at

the beginning of the next line.

.SKIP [-n]

.S [-n]

causes  a BREAK after which n is multiplied by the number of spaces

between lines.  The result is the number of lines skipped.   Output

is  advanced to the top of the next page if there is no room on the

current page.  If the current page is  empty,  SKIP  does  nothing.

The  n  can  also  have a negative value.  Negative values indicate

skips to n lines from the bottom of the page.   Negative  skips  or

skips greater than 6 are not allowed during footnotes or .TEXT sec-

tions.

.BLANK [-n]

.B -[n]

causes the current line to be output with no justification, skips n

line spaces, and then starts output of  the  current  source  text.

BLANK is like SKIP, except that the space to be left is independent

of line spacing.  If the page is empty, BLANK does nothing.  n  can

be  negative to move to n lines from the end of the page.  Negative

n or n greater than 6 are not allowed in footnotes  or  .TEXT  sec-

tions.

.FIGURE [n]

.FG n

leaves  n  lines  blank  to  make room for a figure or diagram.  If

fewer than n lines remain on the current page, the page is advanced

and  n blank lines are left at the top of the next page.  This com-

mand guarantees your having n blank lines for a figure, while blank

will  only  guarantee  n or fewer blank lines.  This command is not

allowed during footnotes or .TEXT sections.  This command  has  the

unfortunate  side  effect  that  if  the  current page doesn't have

enough space for the figure, the page is  left  with  a  number  of

blank lines at the bottom of it.  The default is .FIGURE 1

.FIGURE DEFERRED [n]

.FGD [n]

This  command is similar to figure.  It will reserve n lines on the

current page or if there is not enough room n lines at the  top  of

the  next  page.   If there is not enough room on the next page for

all the lines requested an error message is produced and  the  next

page will be entirely blank.  This differs from .FIGURE in that the

current page will be filled with text right to the bottom  line  if

there is no room for the figure on this page.  This command can not

- 16 -

RUNOFF COMMANDS

TEXT FORMATTING

have the figure caption follow the .FIGURE command, unlike .FIGURE.

NOTE

.BLANK, .SKIP, and .FIGURE are very similar commands.  If

you are intending to leave 1 inch of space .BLANK 6 would

be  appropriate.  If you need 3 lines of space .SKIP 3 is

the correct command.  If you need to leave space  at  the

top  of  a page, or need a guaranteed amount of space use

.FIGURE or .FIGURE DEFERRED.  When choosing between .SKIP

and  .BLANK remember that .SKIP is appropriate if you are

concerned with the number of text lines  to  skip,  where

text  lines  may be more than 1 physical line.  .BLANK is

appropriate when the space to  be  left  is  an  absolute

value, for example .5 inches which is normally 3 physical

lines.

.INDENT [-n]

.I [-n]

| .LEFT [-n]

| .L [-n]

causes  a  BREAK  and  sets  the next line to begin n spaces to the

right of the left margin.  The n can be negative to allow beginning

a  line  to  the  left  of the left margin.  However, a line cannot

begin to the left of column 0.  If n is not supplied,  the  current

paragraph indent is used.

.RIGHT [-n]

.R [-n]

places  the following text flush with the right margin if n is zero

or unspecified.  If n is specified the text is placed n  spaces  to

the left of the margin (n positive) or n to the right (n negative).

|      If .RIGHT is issued after .CENTER TEXT, but before .END CENTER, the

|      text will be centered not right justified.

.PARAGRAPH [-n], [v], [t]

.P [-n], [v], [t]

CAUSES A BREAK and formats the output paragraphs.  The n is option-

al and, if present, sets the number of spaces the paragraph  is  to

be  indented.  (n can also have a negative value).  v is the verti-

cal spacing or number of lines between  paragraphs.   v  can  range

from 0 to 5.  T is the TEST TEXT value.  If there is not room for t

lines remaining on the page then a new page is started.  The action

of  both  t  and  v are changed by the .SPACING command.  Initially

n,v,t are 1,5,2 as the default.  If  you  specify  n,v,t  different

from the defaults, you only need to do this once.  Every .PARAGRAPH

command that follows will assume the same n,v,t.  This  command  is

the same as issuing the following sequence of commands.

.SKIP v .TEST TEXT t .INDENT n

- 17 -

RUNOFF COMMANDS

TEXT FORMATTING

.CENTER [LINE][+-n]

.CENTRE [+-n]

.C [LINE] [+-n]

causes a BREAK and centers the following line of text in the source

file.  The centering is halfway between the right and left  margins

if  n  is not specified.  If n is specified as an absolute argument

(no sign) then the centering is over column n.  If n is a  relative

argument the centering is over the default center plus the relative

value of n.

NOTE

The  definition of n differs from the previous version of

RUNOFF.

.CENTER TEXT [+-n]

.C TEXT [+-n]

centers  the  following  lines of text until a .END CENTER command.

This differs from .CENTER LINE in that it centers more than 1  line

of  text.   Centering automatically prevents filling and justifica-

tion.  The argument n has the same meaning as for CENTER LINE.

.END CENTER

.ECN

stops  centering the text.  This must be given only after a .CENTER

TEXT command.

V.2  FOOTNOTE / NOTE

.FOOTNOTE

.FN

saves lines at the bottom of the current page for a footnote.  When

the bottom of the page is reached the text following the  .FOOTNOTE

command  will be printed.  If insufficient room remains on the cur-

rent page, space is allocated at the bottom of the following  page.

The  text  of  the  footnote should begin on the line following the

FOOTNOTE command.  Indentation, case lock, justify, margins,  spac-

ing,  bar,  and fill are preserved around footnotes.  However, com-

mands that affect page formatting are illegal in a  footnote.   Tab

stops are illegal because they are not preserved.  A footnote with-

in a footnote is also illegal.  If the footnote overflows from  the

current  page  onto  the  next  page,  it is possible that the line

referring to the footnote will also be printed on  the  next  page.

To  prevent  this either preface the .FOOTNOTE command by a command

that causes a break, or place enough text between the reference and

the command to guarantee a full line of text.

|      Normally  this command does not draw a line between the body of the

|      text and the footnotes.  If you want it  to  automatically  draw  a

- 18 -

RUNOFF COMMANDS

FOOTNOTE / NOTE

|      line you must use .PERMANENT FOOTNOTE.

.END FOOTNOTE

.EFN

ends the footnote.  The previous version of RUNOFF used an exclama-

tion point ( ! ) to do this.  The exclamation point is not support-

ed  in this version.  Each .FOOTNOTE or .PERMANENT FOOTNOTE command

must be followed by a .END FOOTNOTE command.

| .PERMANENT FOOTNOTE

| .PFN

|      This command may only be issued before other footnote commands.  If

|      this command is issued and a  footnote  command  has  already  been

|      given  for  the  current  page,  the  command will be flagged as an

|      error.  The text following this command is taken a permanent  foot-

|      note  which is then printed between the body of text on a page, and

|      the footnotes at the bottom.  This also sets up permanent  margins,

|      spacing,  and  fill/justify mode for footnotes.  The left and right

|      margins, and the spacing may be set during the permanent  footnote,

|      and  they  will be restored for every subsequent .FOOTNOTE command.

|      For example you may wish to separate the footnote from the text  by

|      a  blank line, and a divider.  To do this you might use the follow-

|      ing sequence of instructions.

|           .PERMANENT FOOTNOTE

|           .SKIP 2;------- footnote -------

|           .END FOOTNOTE

|      The  permanent  footnote  will  not be printed at the bottom of the

|      page if no regular footnotes are declared.

.NOTE [title text]

.NT [title text]

This  command is used to format notes.  A note is text set off from

the rest of the text by reduced margins and a centered title.   The

text  of  the  note is follows the .NOTE command, and is ended by a

.END NOTE command.  First this command issues a .TEST TEXT n  where

|      n  is 4 added to the paragraph test page count.  This command SKIPs

|      2, reduces both margins, centers the title  text  (if  no  text  is

|      given,  it centers the word "NOTE"), and then SKIPS 1.  The text of

|      the note is then printed.  An indentation is applied  to  both  the

|      left  and  right margins.  The indentation is either 20,15,10, or 5

|      whichever produces no more than a 25% reduction in the  text  line.

|      If  the  indentation produces a text line less than the minimum, no

|      indentation is used, and an error message is typed.

| .END NOTE [n]

| .EN [n]

|      terminates  the  NOTE command, SKIPs n, and reverts the margins and

spacing modes to their settings before the last NOTE command.  If n

is not specified it will be 2 as the default.

- 19 -

RUNOFF COMMANDS

PAGE FORMATTING

V.3  PAGE FORMATTING

.PAGE

.PG

causes  a  BREAK and an advance to a new page.  If the current page

is empty, this command does not advance the page.  Just like an au-

tomatic  page advance, this command prints the title (if given) and

page numbers on every page.

.SUBPAGE

.SPG

executes  a PAGE with page numbering suspended.  The page number is

unchanged, but letters are appended to the page number.  This  per-

mits  insertion  of  additional  pages  within an existing document

without changing the existing page numbering.

.END SUBPAGE

.ES

disengages  the SUBPAGE command and starts a new page.  Normal page

numbering is resumed.

.NUMBER [PAGE] [+-n]

.NMPG [+-N]

starts  page  numbering.  This is the default so there is no reason

to issue this command unless page numbering is disengaged.  If  re-

sumption  of page numbering is desired at a certain page, specify n

as the next page number.  n may  also  be  a  string  of  up  to  3

letters.   If  n  is  a letter A cooresponds to 1, B to 2, Z to 26.

Similarly AA is 27, AB 28 and so on.  The maximum value  for  n  is

4000.   If  you  wish to skip several pages for inserting diagrams,

artwork or any other items which require a full pages, you  may  do

so by issuing the following commands:

.NUMBER PAGE +n .PAGE The next page number will be the current

page number +n.  The page number is normally right  justified  with

respect to the permenant right margin.  See the .PAGE SIZE command.

.NO NUMBER

.NNM

disengages  page numbering.  However, pages continue to be counted,

so that the normal page number can  appear  if  page  numbering  is

re-entered with the NUMBER command.

.NUMBER SUBPAGE [+-n]

.NMSPG [+-n]

starts the next page as subpage number n or letter n where n is the

subpage letter.  The maximum value for n  is  255  or  the  largest

letter is JT.

- 20 -

RUNOFF COMMANDS

PAGE FORMATTING

| .TEXT

| .TX

|      This  command begins a section of text that you wish to keep all on

|      one page.  The .END TEXT or another .TEXT command ends the section.

|      With  this  command  you can guarantee that a given section of text

|      will not be split between 2 pages unless it is too long to fit on 1

|      page.   This  command is illegal inside a footnote, or note.  Like-

|      wise footnotes and .TEST PAGE, .TEST TEXT, or .FIGURE  are  illegal

|      inside this command.

| .END TEXT

| .ETX

|      This ends a section of text you wish to keep all on the same page.

.TEST PAGE n

.TP n

causes a BREAK followed by a conditional page advance.  It skips to

the next page if fewer than n lines are left on the page.  This ca-

pability  is to ensure that the following n lines are all output on

the same page.

| .IMMEDIATE TEST PAGE n

| .ITP n

|      This is almost the same as test page except that it doesn't cause a

|      break.  If fewer than n lines remain on this page  a  new  page  is

|      started  and  the  current  line is the first line on the new page.

|      This is useful if you wish to test  for  sufficient  space  without

|      breaking the line.

| .TEST TEXT n

| .TT n

|      First this command causes a break, then tests to see if enough room

|      is on the current page for n lines of text.  If not a new  page  is

|      started.  This differs from .TEST PAGE in that the actual space re-

|      quired depends on the current spacing.

| .IMMEDIATE TEST TEXT n

| .ITT n

|      This  command is the same as test text except that no break occurrs

|      before the test.  This command is very useful when you are  filling

|      and  justifying  text.   It  allows you to "see" if there is enough

|      room on the page for the rest of the text and start a new  page  if

|      there  is  not.  This may be used to prevent single lines from dan-

|      gling on the top of the next page.

- 21 -

RUNOFF COMMANDS

PAGE HEADERS

V.4  PAGE HEADERS

The  page header consists of a title, and subtitle printed at the top of

each page, as well as a page number.  This title is printed on the first

line  below the top margin, and the subtitle on the next line.  The page

number will normally appear right justified on the same  line  with  the

title.   The  page number is normally preceeded by the word "Page".  The

title and subtitle are normally left justified.  The actual location  of

title,subtitle,  and  page number are controlled by the .LAYOUT command.

In addition to the following commands the .NUMBER PAGE, .DISPLAY  NUMBER

.NO NUMBER, and .NO NUMBER CHAPTER commands control the page header.

.TITLE [text]

.T [text]

takes  the remaining text as the title and outputs it on every page

at line 0.  The title is normally left justified  with  respect  to

the  permanent  left margin.  See the .PAGE SIZE command.  The per-

manent margins allow different margins for the headers than for the

text.  Titles and subtitles may not contain escape flags.  In addi-

tion tabs are converted to single spaces.  Some of  these  restric-

tions  may be removed in future releases.  Titles are also produced

by these commands:  .FIRST TITLE, .CHAPTER, or .APPENDIX.

.NO TITLE

prevents  printing  either  the title or subtitle at the top of the

page.  The title print is reinstated by the .TITLE  command.   This

command  also  prevents  titles from being produced by the .CHAPTER

command.  The chapter title will still be printed after the chapter

number , but it will not appear as part of the page header.

.FIRST TITLE [text]

.FT [text]

Same  as TITLE , but used to specify the title to be printed on the

first page of the document.  This command must precede all text  in

the source file.  Use of the FIRST TITLE command is the only way to

print a title line on the first page of the document.  If the  page

number  appears  on the same line as the title it will also not ap-

pear on the first page unless a .FIRST TITLE command is issued.  If

the  title  is  blank only the page number will be printed.  If the

page number is at the bottom of the page it will be printed on  the

first  page  whether  or  not a .FIRST TITLE command is used.  This

command is equivalent to:

.HEADER .TITLE [ text ]

.SUBTITLE [text]

.ST [text]

takes  the  remaining  text as the subtitle and outputs it on every

page.  It appears directly under the title.  The  subtitle  is  al-

igned on the same margin as the title.  The subtitle is not printed

if the .NO TITLE command is in force.

- 22 -

RUNOFF COMMANDS

PAGE HEADERS

changes the layout of the title and page numbers on the page.  This

command doesn't have any affect on the current page.  The next page

will  have  the  new  layout.  If you want to change only the first

page of each chapter use the .CHAPTER LAYOUT command.  The  spacing

is  the number of blank lines between the last line on the page and

the page number.  If the code is 0 then the spacing is  the  number

of  blank  lines-1 between the bottom of the text and the bottom of

the page.  Pages with different layouts but the same  spacing  will

have  the  same number of lines on a page regardless of whether the

page number is at the top or bottom of the page.  If the spacing is

omitted  it is assumed to be 2 except for code 0.  For code 0 it is

|      assumed to be -1 if not specified.  Normally if layout  0  is  used

|      the  first  page  of a document will not have a page number, unless

|      .FIRST TITLE  or  .HEADER  commands  are  used  at  the  beginning.

|      However  all other layouts will number every page.  This is consis-

|      tent with DSR usage.  The codes are:

CODE       PAGE LAYOUT

----       ---- ------

0  Title flush left, page number flush right

1  Title centered ,Page number centered at bottom

2  Title  flush  left  odd  page ,Title flush right even page ,Page

number centered at bottom

3  Title flush left, Page number centered at bottom

4  Title top left, Page bottom right

5  Title top center, page number bottom right

6  Title top center, page number bottom right for odd numbers, left

for even

7  Title  top,  page  bottom,  both right for odd numbers, left for

even

The default is code 0.

.CHAPTER LAYOUT code [,spacing]

.CHLO [code] [,spacing]

This  command changes the layout for the first page of a chapter or

appendix without having any effect on any other pages.  The parame-

ters  are  specified in the same manner as for the .LAYOUT command.

If this command is not used, or used with no parameters, the layout

will be the same all pages of a document.

.HEADER

.HD

causes  the  page  header  (title, subtitle, and page number) to be

printed.

- 23 -

RUNOFF COMMANDS

PAGE HEADERS

.NO HEADER

.NHD

causes the page header (title, subtitle, and page number) to be om-

itted.  If a layout other than 0 is used, then the page  number  is

still printed at the bottom of the page and only the title and sub-

title are suppressed.  The header lines are completely omitted,  so

that  text begins at the top margin where the header would normally

start.  The first page of output from RNO is formatted  as  if  the

.NO HEADER command had been issued, while subsequent pages are for-

matted as if the .HEADER command had  been  issued.   This  may  be

changed  by issuing the .FIRST TITLE command before any text on the

first page.

.HEADER PAGE

sets  RUNOFF  to  print  the word PAGE in front of the page number.

This is the initial or default option.

.HEADER NO PAGE

set  RUNOFF  to print only the page number without the word PAGE in

front of it.  This essentially supresses the pre-header on the page

number.

.HEADER UPPER

.HEADER MIXED

.HEADER LOWER

These  3 commands enable the header and determine the format of the

pre-header.  UPPER,MIXED or LOWER  cause  the  word  "PAGE"  to  be

printed  before  the page number in all upper case, capitalized, or

all lower case respectively.

NOTE

The  .DISPLAY  NUMBER  command should be used rather than

.HEADER UPPER .HEADER LOWER, .HEADER  MIXED,  .HEADER  NO

PAGE,  and  .HEADER PAGE.  It is a more versatile command

and it can perform all the same functions as these  other

HEADER commands.

.HEADER SPACING [n]

sets the spacing between the title and the first line of text.  The

value n is the number of lines to skip.  If n is not specified then

the  current  spacing per line of text is used as the default.  The

initial setting is 3.  The minimum value for n is 1  and  if  0  is

specified the value of 1 is assumed.

- 24 -

RUNOFF COMMANDS

LISTS

V.5  LISTS

The following commands format lists of items.  Say you wish to make up a

list of the colors of the rainbow.  To do this you could issue the  fol-

lowing commands.

.list 0

.le RED

.le ORANGE

.le YELLOW

.le GREEN

.le BLUE

.le VIOLET

.end list

The result would be the following:

1.  RED

2.  ORANGE

3.  YELLOW

4.  GREEN

5.  BLUE

6.  VIOLET

It is possible to preceed the list with letters as A,B,C ...  and so on,

or by Roman numerals.  To do this see the .DISPLAY  LIST  command.   The

following commands control lists:

.LIST n

.LS n

starts  an  indented  numbered  list with n spacing, moves the left

margin 9 spaces to the right for the first LIST command, and 4 more

spaces  for  each  subsequent  nested LIST.  The n specified in the

.LIST command skips n lines before each list element in the current

list.  If the spacing is set to other than single spacing the actu-

al number of blank lines is n times the spacing.  If n  is  omitted

the current paragraph spacing is used.

.LIST ELEMENT

.LE

starts  an  item in the list, used in conjunction with the LIST co-

mand.  If you want to type the text on the same line  as  the  com-

mand,  you  must separate the text from the command with any number

of intervening spaces or tabs, or (optionally) one semicolon.  Each

list  element  produces  a  number followed by the text.  The first

list element is numbered 1 the second 2 and so on.  Each number  is

normally  followed  by  a decimal point ( . ) and 2 spaces.  If you

prefer letters, roman numerals or some character other than the de-

cimal point see .DISPLAY ELEMENTS.  The text lines up with the cur-

rent left margin and the numbers are placed 4 spaces to the left of

the new left margin.  The numbers will not be placed to the left of

the permanent left margin.  If you want the numbers lined up on the

left  margin with the text indented, you may do this by issuing .LM

after the .LS command to reset the left margin back to the  perman-

ent  left  margin.  A .TEST PAGE is taken for each list element ac-

cording to the value set in the last .PARAGRAPH or  .SET  PARAGRAPH

command.

- 25 -

RUNOFF COMMANDS

LISTS

|      This  command may be used without preceeding it by a .LIST command.

|      The numbers will again be indented -4 from the current  margin  but

|      not  to the left of the permanent margin.  This might be considered

|      to be the 0'th or base list.  All of the other commands except .END

|      LIST apply to this list also.

.END LIST n

.ELS n

Terminate the .LIST command and returns the settings to the same as

for the last .LIST command.  The optional value n sets the  spacing

at  the  end  of the current list.  If none is supplied the current

paragraph spacing is used.  The actual number of lines skipped is n

times the current spacing.

.NUMBER LIST [+-n]

.NMLS [+-n]

sets the next number in the list to n.  If n is preceeded by + or -

then the next list element number will be the current one + or - n.

You may also specify a letter instead of n.  If this is done A cor-

responds to 1, B to 2 and so on.  The maximum value for n is 4000.

V.6  CHAPTER/APPENDIX FORMATTING

.CHAPTER [text]

.CH [text]

starts  a  new  chapter using the text as the title of the chapter.

The text may not contain escape sequences, and tabs within the text

are  treated as ordinary spaces.  This command declares the text as

being chapter oriented, which has effects on the page  numbers  and

the section headers.  This command acts as if the following command

string were entered:

.NO HEADER .PAGE SIZE .PAGE

.FIGURE 12 .CENTER;  CHAPTER n

The  n  is  incremented by 1 automatically.  After the CHAPTER n is

typed on the page the following are executed:

.BLANK 2 .CENTER;  text

.BLANK 3 .TITLE text

.FILL .HEADER .SUBTITLE

The result of this command is a chapter heading with centered title

and chapter number.  The chapters of this document are  an  example

of  this  format.   This command resets the case, margins, spacing,

and justify/fill modes.  It also clears any subtitles and sets  the

chapter name as the title.  The title will appear in CAPS unless it

contains a shift down (\\).  If the .NO AUTOTITLE or .NO TITLE com-

mand  is used, the page title will not automatically be the same as

the chapter title.  To retain control over the margins and  spacing

you  should use the .PAGE SIZE command to set the permanent margins

and spacing before the .CHAPTER command is executed.

- 26 -

RUNOFF COMMANDS

CHAPTER/APPENDIX FORMATTING

It  is  possible  to  reformat the chapters with the .STYLE CHAPTER

command.  The chapter numbers may be  selected  to  be  letters  or

Roman  numerals with the .DISPLAY CHAPTER command.  In addition the

.DISPLAY CHAPTER command may also  change  the  word  "CHAPTER"  to

another  word, or just blank space.  The STYLE and DISPLAY commands

when used together can completely alter the chapter header.

.APPENDIX [text]

.AX [text]

starts  a new appendix using the text as the title of the appendix.

This command performs the same functions as .CHAPTER except that an

appendix  results.   This  command acts as if the following command

string were entered:

.NO HEADER .PAGE SIZE .PAGE

.FIGURE 12 .CENTER  APPENDIX a

.BLANK 2 .CENTER; text

.BLANK 3 .TITLE text

.FILL .HEADER .SUBTITLE

The  letter a following APPENDIX is incremented before it is print-

ed.  Both .STYLE CHAPTER and .DISPLAY APPENDIX alter  the  look  of

the appendix header.

.NUMBER CHAPTER [+-n]

.NMCH [+-N]

supplies  a  number (n) to be used in a subsequent CHAPTER command.

NUMBER CHAPTER would be used when a chapter of a document  occupies

a  source file of its own.  In such a case, NUMBER CHAPTER would be

the first command of the source file.  n may also be a letter.  The

maximum value for n is 255 or the largest letter is JT.  This would

also be used if you wish to skip a chapter for insertion later  on.

To  skip  chapters  specify  +n.  This command also enables chapter

numbering.  When chapter numbering is in effect  each  .CHAPTER  or

.APPENDIX  command  resets  the page number to 1 and the chapter or

appendix number are printed with the page number  as  part  of  the

page header.  This is the normal RUNOFF mode.

.NO NUMBER CHAPTER

disables  chapter numbering.  When it is disabled, the page numbers

are  not  reset  with   each   chapter   or   appendix,   and   the

chapter/appendix number is not printed as part of the header.  This

command must be issued before the first .CHAPTER or .APPENDIX  com-

mand if you wish to disable chapter-page numbering.  In addition it

must be reissued after each .NUMBER CHAPTER if you intend  to  con-

trol chapter numbering but wish running page numbers.

.NUMBER APPENDIX [+-n]

supplies  a  number or letter to be used as the letter for a subse-

quent APPENDIX command.  n may also be a letter.  See NUMBER  PAGE.

The maximum value for n is 255 or the largest letter is JT.

- 27 -

RUNOFF COMMANDS

CHAPTER/APPENDIX FORMATTING

| .STYLE CHAPTER [n1],[-n2],[-n3],[-n4],[-n5]

| .STCH [n1],[-n2],[-n3],[-n4],[-n5]

|      changes the style of the chapter headers.

|      n1 = The number of lines to skip before printing CHAPTER.

|      n2 = Number of lines to skip after the word CHAPTER.  If n2=-1 then

|           the title and word CHAPTER will appear on the same line.

|      n3 = Number  of  lines from title to text.  If n3=-1 then the title

|           and any following text will be on the same line.

|      n4 = The number of spaces to indent the word CHAPTER.

|      n5 = The  number of spaces to indent the title.  If n4 or n5 are -1

|           then the line will be centered.  If n4 or n5 are -2  then  the

|           lines are right justified.

|      The default values are 12,1,3,-1,-1.

V.7  SECTION HEADERS

A  section  header consists of a string of numbers ( n1.n2.n3.... ) fol-

lowed by a title.  For example this section is preceeded  by  a  section

header  with  the  title "Section Headers".  There are a variety of com-

mands to control section headers.  If you are using either the  .CHAPTER

or .APPENDIX command then the first number n1 is the same as the chapter

or appendix number.  The number of numbers ( n1.n2...) which preceed the

section  header  is call the level of the header.  If you have a chapter

oriented document, the chapter number is not included in the  numbering.

For example the following header is at level 4

1.2.3.4 Example of a header

The following is at level 6:

3.1.1.1.1.1 Example of a header

Section  headers  always  consist of at least 2 numbers so that in a non

chapter orinted document header s at level 1 are printed  as  n.0.   The

following commands control the header levels.

.HEADER LEVEL [+-n] text

.HL [+-n] text

starts  a  section  at  the level specified and takes the following

text as the header or title of the section.  n can range from 1  to

6.  The sections are incremented by 1 automatically, and the number

is output in the form i.j.k.l.m.  If this is a chapter oriented do-

cument,  the  i is the chapter number.  Otherwise, it is the number

of the .HL 1 level.  This command acts as a

.BREAK;.TEST PAGE 7;.BLANK 2

followed  by  the section number, two spaces, and the section name.

if the level number is preceeded by + or - the current level number

is the previous one + or - the value specified.

LEVEL  1 capitalizes the section name and separates it from the the

rest of the text by putting it on a separate line.

LEVEL  2  only separates the section name from the rest of the text

- 28 -

RUNOFF COMMANDS

SECTION HEADERS

by a break.

LEVELS  3,4,5  run  the  title  and following text together and the

title and text are separated by a single dash.

If  the  title is too long to fit on 1 line it is filled,

justified, and continued on the  next  line  indented  to

line  up  with the first part of the title.  Autohyphena-

tion is not performed on header titles.

If  the  title is omitted, only the number is printed, and the next

line of text follows it separated by 2 spaces.  For  this  case  no

distinction is made between different level numbers.

| .NUMBER LEVEL [+-n1],[+-n2],[+-n3],[+-n4],[+-n5],[+-n6]

| .NMLV [+-n1],[+-n2],[+-n3],[+-n4],[+-n5],[+-n6]

|      sets the next header to the number specified.  If n is preceeded by

|      + or - the n is added or subtracted from the  current  value.   The

|      next  time  .HEADER  LEVEL is used the number specified will be the

|      header number.  If the next .HEADER LEVEL command specifies a level

|      different from the one implied in the .NUMBER LEVEL command the re-

|      sult will be undefined.  Instead of a number  n  may  be  up  to  2

|      letters.   The  maximum value for n is 255 or the largest letter is

|      JT.  For example the following command sequence is specified:

|                .NUMBER HEADER 5,4,3,2 .HEADER LEVEL

|      The result of this would be:

|                5.4.3.2

.STYLE HEADERS [n1],[n2],[n3],[n4],[n5],[n6],[n7],[n8],[n9]

.STHL [n1],[n2],[n3],[n4],[n5],[n6],[n7],[n8],[n9]

controls  the formats of the header levels.  The initial default is

in ().

n1  =  The lowest level at which text starts on same line as title.

(3)

n2 = The lowest level with title capitalized.  (1)

n3 = The higest level with first letter of title in CAPS.  (6)

n4 = Lowest level with no level number.  (7)

n5 = Lowest level with centered title and number.  If the title and

text are on the same line no centering occurrs.  (7)

n6 = The number of blank lines before the header.  (2)

n7 = The number of blank lines after the title.  (1)

n8 = The implicit test page value (7)

n9 = Number spaces between section number and title.  (2)

| .INDENT LEVELS n1,n2

|      this  causes  the  header levels to be indented by n1 from the left

|      margin and indented n2 from the right margin.  A positive value  of

|      n1  causes indentation to the right of the left margin.  A positive

|      value of n2 causes indentation to the left of the right margin.

| .ENABLE LEVELS [+-n1],[+-n2]

|      enables  output of header levels up to n1, and TOC output up to n2.

|      In other words the .DOC file will have header levels 1  to  n1  and

|      the  .RNT file will have levels up to n2.  If n2 is greater than n1

- 29 -

RUNOFF COMMANDS

SECTION HEADERS

|      the .RNT file will only contain levels up to n1.  The default is 6.

|      If n is not specified the default is assumed.

- 30 -

RUNOFF COMMANDS

V.8  DISPLAY COMMANDS

| .DISPLAY NUMBER ["pre-header",] [format] [,"post-header"]

| .DNM ["pre-header",] [format] [,"post-header"]

controls  the format of the page numbers as they are printed at the

|      top of the page, in the index and in the table  of  contents.   The

|      pre  and  post  headers specify text to be printed before and after

|      the page number.  The pre-header may consist  of  up  to  10  char-

|      acters.  The post headr may be up to 4 characters long.  This extra

|      text is only printed on the individual pages, and do not appear  in

|      the  index  or table of contents page numbers.  Any flag characters

|      included in the pre or post header text are printed, and do not act

|      as  flags.   In  addition  no case conversion is made on the header

|      text.  The pre or post headers may  be  enclosed  in  either  apos-

|      trophes  (')  or quotes (").  The format determines what format the

page number is in.  It can be either regular decimal numbers, Roman

numerals,  or  a  sequence  of  letters.   See the table of formats

below.  You may specify only the pre-header and omit the format and

post-header.   However  if  you wish to specify the post-header you

must specify the format.  The pre-header is always optional.

|      Assume  you want pages to be numbered in lower case roman numerals.

|      You would use the following command:

|           .DISPLAY NUMBER RL

|      As another example say you wish to print a 4 page memorandum and on

|      each page you want the page number and the total  number  of  pages

|      printed as n/4.  To do this you issue the following command.

|           .DISPLAY NUMBER "", D, "/4"

|      The first page is numbered 1/1 the second 1/2 and so on.

|      This document is numbered with:

|           .DISPLAY "- ",D," -".

|      The default is:

|           .DISPLAY NUMBER "Page" , D , ""

format        Description

------        -----------

D         Decimal numbers

(1,2,3......)

LU      Letters uppercase

(A,B,C.....Z,AA,AB,AC....)

LL      Letters lowercase

(a,b,c.....)

LM      Letters mixed (first is uppercase)

(A,B....,Aa,Ab,...)

RU      Roman numerals uppercase

(I,II,....)

RL      Roman lowercase

(i,ii.....)

RM      Roman mixed

(I,Ii,Iii......)

- 31 -

RUNOFF COMMANDS

DISPLAY COMMANDS

| .DISPLAY CHAPTER ["pre-header",] [format] [,"post-header"]

| .DCH ["pre-header",] [format] [,"post-header"]

controls the format of the chapter numbers as they are printed when

|      the .CHAPTER command is used.  The pre-header is  up  to  10  char-

|      acters  of  text to be printed in front of the chapter number.  The

|      format determines what type of number  is  printed.   See  .DISPLAY

|      NUMBER.   The post-header is up to 4 characters to be printed after

|      the chapter number.

|      For  example  rather than chapters you want sections with numbering

|      by Roman numerals, so you include the following in your .RNO file:

|           .DISPLAY 'Section (' , RU , ')'

|      When  you  issue the .CHAPTER command for the 6'th time you get the

|      following in the chapter header:

|           Section (VI)

The default is:

.DISPLAY CHAPTER "CHAPTER " , D , ""

| .DISPLAY APPENDIX ["pre-header",] [format] [,"post-header"]

| .DAX ["pre-header",] [format] [,"post-header"]

controls  the  format  of the appendix number.  This works the same

way as .DISPLAY CHAPTER.

The default is:

.DISPLAY APPENDIX "APPENDIX " , LU , ""

.DISPLAY SUBPAGE [format]

.DSP [format]

controls  the  format of the subpage number.  It is normally upper-

case letters.  See .DISPLAY NUMBER for a table of possible formats.

.DISPLAY LEVELS [fm1],[fm2],[fm3],[fm4],[fm5],[fm6]

.DHL [fm1],[fm2],[fm3],[fm4],[fm5],[fm6]

sets  the header level numbers to be displayed in the selected for-

mat.  fm1 to fm6 are format codes for LEVELS 1 to 6.  See  .DISPLAY

NUMBER for a table of possible formats.

The default is:

.DISPLAY LEVELS D,D,D,D,D,D

.DISPLAY ELEMENTS ["l"] [,format] [,"r"]

.DLE ["l",] [format] [,"r"]

this  command sets the format of the current LIST element.  The "l"

and "r" are the left and right characters to put  around  the  list

number.   See  .LIST,  .LIST ELEMENT, .END LIST for explanations of

the list elements.  See .DISPLAY NUMBER for  a  table  of  possible

formats.  Once you have specified the display characteristics for a

given list it will remain specified even after more .LIST  or  .END

LIST  commands.  The optional characters may be delimited by either

quotes ["] or a pair of apostrophes ['].  The default format is:

.DISPLAY ELEMENTS "" , D , "."

If you wish to setup your lists at the beginning of the document to

- 32 -

RUNOFF COMMANDS

DISPLAY COMMANDS

normal outline form you issue the following commands.

.LIST 0 .LIST 0 .DISPLAY ELEMENTS LU

.LIST 0 .DISPLAY ELEMENTS LL

.END LIST 0 .END LIST 0 .END LIST 0

V.9  MODE SETTING

| .ENABLE BAR [n1] [,n2]

| .EBB [n1] [,n2]

The  change bar is a vertical bar printed in the in the left margin

of a document.  It is generally used to note sections of the  docu-

ment  that  have  been changed since the last printing of the docu-

ment.  This command enables the printing of change bars.  Bars will

not  actually  be  printed  unless  a .BEGIN BAR command is issued.

|      This command is normally issued at the  beginning  of  a  document.

|      Each  line is shifted right by n1 before the change bar and by n2-1

|      after the change bar.  If no change bar  is  printed  the  line  is

|      shifted  right by n1+n2.  In effect if n1 is 4 and n2 is 3 the text

|      line is always indented by 7 but the change bar appears 2 spaces to

|      the  left of the text when it is turned on.  The defaults for n1,n2

|      are 0,3.  If you set a left margin the n1,n2 are in addition to the

|      left margin.

.DISABLE BAR

.DBB

Causes  change bar mode to be disabled.  If this is done at all, it

is usually only at page breaks.  This is the default setting.

.BEGIN BAR

.BB

Causes  the  change  bar to be printed in the document.  It will be

printed until a .END BAR command is encountered.

.END BAR

.EB

This command stops the printing of change bars.

.JUSTIFY

.J

Causes  a  break  and  sets subsequent output lines to be justified

(initial setting).  The command increases the spaces between  words

until the last word exactly meets the right margin.

.NO JUSTIFY

.NJ

Causes  a  break  and  prevents  justification of subsequent output

lines to make a ragged right margin.

- 33 -

RUNOFF COMMANDS

MODE SETTING

.FILL

.F

Causes a break and specifies that subsequent output lines be filled

(inital setting).  Sets the justification mode to be that specified

by  the last appearance of JUSTIFY or NOJUSTIFY.  FILL adds succes-

sive words from the source text until the adding of one  more  word

will  exceed  the  right  margin.  It stops before putting the last

word in.  (If hyphenation has not been disabled, RNO  will  attempt

|      to  break words which cause line overflow into syllables.) Normally

|      the end of an input line is treated as a space unless  the  command

|      .NO SPACE is used.

.NO FILL

.NF

disengages  the  FILL  and  JUSTIFY modes.  This command is used to

permit typing a table.

NOTE

1. The  nofill-nojustify  mode  need  be  used only where

there are several lines of material to be copied exactly.

A  single  line example will not require using these com-

mands if there are breaks before and after.

2. Normally FILL and NOFILL are used to turn both filling

and justification on and off.  It is usually desirable to

do both.  A subsequent appearance of a justification com-

mand will override the fill command however.

3. Because  of the action of FILL, a single occurrance of

NOJUSTIFY will cause the remainder of the file to be  un-

justified, with filling as specified.  In order to justi-

fy but not fill (not recommended), a JUSTIFY command must

follow every NOFILL command.

| .NO SPACE

| .NSP

|      This  command may be used to join 2 filled lines without the normal

|      space in the output between them.  If  .FILL  is  engaged  and  you

|      issue  .NO  SPACE after a line, the end of that line will not auto-

|      matically generate a space in the output .DOC file.

.UPPER CASE

.UC

sets  the  output mode to upper case.  This comand acts the same as

typing two circumflexes (^^).  This is the default mode.  There  is

no need to type this command unless the mode was previously altered

to lower case.

- 34 -

RUNOFF COMMANDS

MODE SETTING

.LOWER CASE

.LC

sets the typeout mode to lower case.  This command acts the same as

typing two backslashes (\\).

.PERIOD

.PR

enables printing of two spaces after every punctuation that is fol-

lowed by at least one separator character (initial  setting).   The

legal punctuations are:

.  ;  :  ?  !

If  you  wish  to prevent 2 spaces from being inserted after one of

these punctuation marks you may insert a SPACE flag after the punc-

tuation rather than a space.

.NO PERIOD

.NPR

disengages  conversion  of punctuation/separator to punctuation/two

spaces.

.LITERAL

.LT

turns off all flags and all features to permit printing of text ex-

actly as it appears in the source file.  In addition  all  commands

are  disabled except for .END LITERAL.  Commands appear in the out-

put as if they were normal text.  Blank lines are output instead of

being  ignored.  This command is almost equivalent to the following

sequence of commands.

.NO FILL .NO FLAGS ALL .NO FLAGS CONTROL

Unlike the sequence above the .LITERAL command may be turned off by

the .END LITERAL command.

.END LITERAL

.ELI

is  used  at  the  end of a LITERAL to allow normal text processing

again.  This is the only command recognized after the .LITERAL com-

mand.  It is equivalent to the following:

.FLAGS CONTROL .FLAGS ALL

In  addition the fill mode is restored if it was in effect when the

.LITERAL command was issued.

.UNDERLINE SPACES

sets  RUNOFF  to underline the spaces imbedded in a line of text to

be underlined.

.UNDERLINE NO SPACES

set  RUNOFF  to  not  underline  spaces imbedded in a line of text.

This is the normal option.

- 35 -

RUNOFF COMMANDS

MODE SETTING

.AUTOPARAGRAPH

.AP

causes  any  blank line or any line starting with a space or tab to

be considered as the start of a new paragraph.  This command allows

normally  typed  text to be justified without special commands.  It

does not cause a paragraph if blank lines are followed  by  a  com-

mand.

.NO AUTOPARAGRAPH

.NAP

disengages the AUTOPARAGRAPH mode.

.AUTOHYPHENATE

.AH

turns  on  automatic hyphenation.  When it is on RNO will automati-

cally hyphenate words wherever necessary.  If .DISABLE  HYPHENATION

is  used  all  hyphenation  including  autohyphenation is disabled,

until .ENABLE HYPHENATION is issued.  This is the default  so  this

command need not be used if you want autohyphenation.

.NO AUTOHYPHENATE

.NAH

turns off autohyphenation.  When it is off user selectable hyphena-

tion may still be used.  Note that autohyphenation  may  be  turned

off  temporarily  for  1  word  by preceeding it with the hyphenate

flag.

| .AUTOTITLE

| .AT

|      This  command  enables  automatic  titles.   Everytime  you  give a

|      .CHAPTER or .APPENDIX command the chapter or appendix title will be

|      used  as  the title at the top of the page.  This is the normal de-

|      fault so it is no necessary to give this command.

| .NO AUTOTITLE

| .NAT

|      turns  off  the autotitle mode.  When off the .CHAPTER or .APPENDIX

|      commands will not automatically generate titles.

| .AUTOSUBTITLE [+-n]

| .AST [+-n]

|      Turns on autosubtitling.  When this is on each header level command

|      lower than n sets the subtitle to the current header  level  title.

|      The default for n is 1.

| .NO AUTOSUBTITLE

| .NAST

|      Turns off autosubtitling.  This is the normal default.

- 36 -

RUNOFF COMMANDS

PARAMETER SETTING

V.10  PARAMETER SETTING

.LEFT MARGIN [+-n]

.LM [+-n]

sets  the left margin to n.  The n must be less than the right mar-

gin but not less than 0.  The initial setting is 0.  If  n  is  not

supplied,  the  permanent margin is used.  You should note that tab

stops are relative to the margin at 0.  In other words if  you  set

.LEFT  MARGIN  10 .TAB STOPS 10,15,20,25 the tab stop at 10 will be

ineffective because it is already at the left margin.

.RIGHT MARGIN [+-n]

.RM [+-n]

sets  the right margin n.  The n must be greater than the left mar-

gin.  The initial setting is 60.  If n is not supplied, the current

page width (set with the PAGE SIZE command) is used.

.TOP MARGIN [+-n]

.TM [+-n]

sets  the top margin or the number of blank lines at the top of the

page to n.  The top margin must be less than the  number  of  total

lines  in the page.  The default value is 0.  If n is not specified

the top margin is reset to the default  or  permanent  value.   See

also .PAGE SIZE command below.

| .PAPER SIZE [+-h],[+-w],[+-l],[+-t],[s]

| .PAGE SIZE [+-h],[+-w],[+-l],[+-t],[s]

.PS [+-h],[+-w],[+-l],[+-t],[s]

sets the size of the page to h by w for the height and width.  Also

|      sets the LEFT MARGIN to l, the TOP MARGIN to t and the  spacing  to

|      s.  The initial setting is 58,60,0,0,1.  These settings are perman-

ent since they are now the new default for the duration of the  do-

cument.   Every  .CH command will reinstate the PAGE SIZE settings.

In addition the other margin commands without any margins specified

will  reset  the  requested margin to the permanent value.  You may

reset all of these values to the permanent  value  by  issuing  the

.PAGE  SIZE  command  without any parameters.  The permanent values

are used to determine the margins of the heading on each page.  The

older  versions  of  RUNOFF used only the temporary right margin to

set the header margins.  If the margins are set so that  there  are

fewer  than 16 lines on a page or fewer than 16 spaces per line the

.PAGE SIZE command will be rejected with an  error  message.   This

restriction also applies to the margin commands.

NOTE

If  you  wish this command or any other margin command to

set the margins for the  entire  document  including  the

first page, you must put the command before all text, and

spacing commands.  A spacing command is .BLANK  ,  .SKIP,

.FIGURE,  or any other command that generates extra blank

lines.

- 37 -

RUNOFF COMMANDS

PARAMETER SETTING

.SET PARAGRAPH [indent],[spacing],[test-page-lines]

.SPR

sets  the paragraph parameters without actually causing a break, or

starting a new paragraph.  This is similar to the  .PARAGRAPH  com-

mand.   The parameters specifed in this command also have an effect

on the following commands:

.LIST               .LIST ELEMENT       .END LIST

.NOTE               .PRINT INDEX        .DO INDEX

.SPACING [n]

.SP [n]

sets the number of spaces between lines.  The n can range from 1 to

|      5.  If n is omitted the permanent spacing (normally 1) is restored.

|      The  permanent spacing is set by the .PAGE SIZE command.  This com-

mand influences .PARAGRAPH, .SET PARAGRAPH, and .SKIP.  It  has  no

effect on .TEST PAGE, .FIGURE, or .BLANK.  SPACING 1 is like single

spacing on a typewriter and  SPACING  2  is  like  double  spacing.

SPACING 2 puts one blank line between lines of text.

| .HALF SPACING [n]

|      This  command is very similar to the spacing command except that it

|      turns on half spaing.  The n will be the number of half  lines  per

|      line  of  text.   Once half spacing is enabled, every command which

|      specifies spacing now works in increments of half lines.   Commands

|      which are affected by half spacing are:

|           .BLANK              .FIGURE             .FIGURE DEFERRED

|           .PAGE SIZE          .TOP MARGIN         .SPACING

|           .HEADER SPACING     .LAYOUT             .STYLE LEVELS

|           .STYLE CHAPTER

| .NO HALF SPACING

|      This turns off the half spacing option.  All commands which specify

|      a spacing will work in increments  of  1  space  only.   Previously

|      specified spacing will however remain the same.

.STANDARD n

.SD n

returns  all parameters to their initial settings and sets n as the

page width.  This command has been retained for compatability  with

previous  versions  of  RNO and is NOT recommended for general use.

---

The .PAGE SIZE command with no parameters will reset margins to the

permanent  values,  while this command resets them to "installation

standard".  If .STANDARD 70 is specified, margins are reset

.PAGE SIZE 58,70,0,3 .SPACING 1 .FILL

.SET PARAGRAPH 5,1,2

The  actual  parameters  may  differ according to the "installation

standard".

- 38 -

RUNOFF COMMANDS

PARAMETER SETTING

.LOCK

this  command  locks a number of page formatting parameters.  While

locked these parameters can not be changed.  Essentially  all  com-

mands  that  change the page format are locked.  The following com-

mands will give error messages if issued after a .LOCK command:

.HEADER UPPER            .HEADER LOWER       .HEADER MIXED

.HEADER PAGE             .HEADER NO PAGE     .HEADER SPACING

.NO HEADER               .HEADER             .INDENT LEVELS

.PAGE SIZE               .TOP MARGIN         .STYLE (all commands)

.STANDARD                .NO NUMBER          .NO NUMBER CHAPTER

.NO TITLE                .LAYOUT

.DISPLAY (all commands)

In  addition the .NUMBER CHAPTER will not reinstate chapter number-

ing, and .NUMBER PAGE will not reinstate page  numbering.   If  you

are  careful  to  put  all page formatting commands into a separate

file, this command will probably not be necessary.  If however  you

have  been foolish enough to sprinkle them throughout your text, or

you need to merge several documents into a  single  coherent  tome,

this command can be a valuable short cut.

This  is  intended to facilitate merging several files into 1 docu-

ment.  You first set up all the parameters mentioned above and then

lock  them.  If you still have page formatting commands in the rest

of the document they will not be obeyed.  This will produce a  uni-

form document with all the same format.

V.11  TAB STOPS

Lists  of  items may be conveniently expressed in tabular form using the

tab key and the .TAB STOPS command.  Every time you press the "tab"  key

you  have entered a tab into the text.  Just as on a typewriter the next

text will begin at the next tab stop.  The tab stops are set  using  the

.TAB STOP command.  You can right justify or center justify the items by

using the .TAB RIGHT command.  When tabbing it is suggested that you use

the  .NO  JUSTIFY command or the .NO FILL command.  Having both fill and

justify modes on will either give you error messages, or  cause  unpred-

ictable  results.   You  should also .DISABLE HYPHENATION when using the

tab feature.  You may wish to set up a list of items to go into  regular

columns, but where any item could go into any column.  This can be achi-

eved by putting a "tab" at the end of each item, and  setting  the  left

margin to the first column, with .FILL enabled.

.TAB STOPS [n1],[n2], .  .  .  ,[n32]

.TS [n1],[n2], .  .  .  ,[n32]

sets  tabs.  The n must be listed in ascending order.  The value of

a tab stop (n) must be between 1 to 255.  If  tabs  already  exist,

the  issuing  of another TAB STOPS command clears all previous tabs

before setting new ones.  Initially tabs are  set  at  eight-column

intervals  to  match the Digital hardware standard.  These tabs are

at columns 8, 16, 24, 32, 40, 48, 56, 64, 72, and 80.  The tabs are

converted  to  the appropriate number of non-expandable spaces.  If

there are no regular spaces to the left  of  the  tabs,  they  will

print  out  at the appropriate position, even if FILL is on.  Up to

- 39 -

RUNOFF COMMANDS

TAB STOPS

32 tab stops may be specified.

.TAB RIGHT

this  causes  the  text following a tab to be right justified.  The

text is right justified at the next tab terminator.   The  text  is

considered  to  be terminated by either a space, tab, end of line,,

or break character.  If the break character is used to  define  the

right  side  of the text, then the .FLAGS BREAK command must be is-

sued.  This allows typing tables with the text right justified,  or

with an arbitrary point (defined by the break) lined up.

The first example of this is listing a column of figures right jus-

tified.  You type in the following text.

.NO FILL .TAB STOPS 30,40,50,60 .TAB RIGHT

NAME      Jan       Feb       Mar       April

Jones     152.75    0.00      3.78      1000.50

Smith     4.95      300.22    5.75      54.95

Which produces the following text:

NAME             Jan       Feb       Mar     April

Smith         152.75      0.00      3.78   1000.50

Jones           4.95    300.22      5.75     54.95

The second example is columns of figures lined up along the decimal

points.  You input the following lines:

.NO FILL .FLAGS BREAK .TAB STOPS 20,30,40,50 .TAB RIGHT

1|.23     3|.1415   75|.2

789|.0    55|.2     100|.98765

Which produces the following text.

1.23      3.1415   75.2

789.0      55.2     100.98765

.TAB LEFT

this sets the tabs back to the normal action of left justifying the

text on the next tab stop.  This is the default.  The following ex-

ample illustrates the .TAB LEFT command.  You type in the following

text.

.NO FILL .TAB STOPS 30,40,50,60 .TAB LEFT

NAME      Jan       Feb       Mar       April

Jones     152.75    0.00      3.78      1000.50

Smith     4.95      300.22    5.75      54.95

Which produces the following text:

NAME                Jan       Feb       Mar       April

Smith               152.75    0.00      3.78      1000.50

Jones               4.95      300.22    5.75      54.95

- 40 -

RUNOFF COMMANDS

TAB STOPS

.ELLIPSES

causes  tabbed  text to be filled with ellipses rather than spaces.

In other words if you tab and this feature is on the text  will  be

padded  by  ellipses  ( . . . .)  rather than spaces to justify the

text.

For example you type in the following text:

.NO FILL .TAB STOPS 30,40,50,60 .TAB RIGHT

NAME      Jan       Feb       Mar       April

.ELLIPSES;Jones     152.75    0.00      3.78      1000.50

Smith     4.95      300.22    5.75      54.95

Which produces the following text:

NAME             Jan       Feb       Mar     April

Smith  . . .  152.75 . .  0.00 . .  3.78 . 1000.50

Jones  . . . .  4.95 .  300.22 . .  5.75 . . 54.95

.NO ELLIPSES

turns off the ELLIPSES feature.  This is the default.

V.12  FLAGS

A  flag  is  a character which performs some special action and does not

appear in the final document.  An example of a  flag  is  the  ampersand

(&).   It  causes  the next character to be underlined.  Each flag char-

acter can be defined by the .FLAG command.  This  command  defines  what

the  character  does  and can optionally define which character does it.

For example the following command will redefine the double quotes as the

underline flag.

.FLAGS UNDERLINE "

After  issuing this command any character preceeded by double quotes (")

will be underlined.  The ampersand will then be merely a simple printing

character.   The .NO FLAGS command disables the selected flag character.

It is then a normal printable character.  If you issue a .FLAGS  command

with  a  character  specified  which is already in use as a flag, RUNOFF

will reject the command and give you an error message.

.FLAGS ACCEPT [new flag]

.FL ACCEPT [new flag]

enables  recognition  of the underscore (_) to allow printing other

characters that are reserved to flags.  This is  normally  enabled,

but may be disabled by .NO FLAGS ACCEPT.  The underscore may be re-

placed by another character to use for this flag.

.NO FLAGS ACCEPT

.NFL ACCEPT

disables  the  use of (_) as an accept flag, so that it may be used

as a normal character in the text.

- 41 -

RUNOFF COMMANDS

FLAGS

.FLAGS ALL

.FL ALL

.FL

turns  on all flags that have been previously enabled.  This is an-

alogous to a master switch which turns on all other switches.

.NO FLAGS ALL

.NFL ALL

.NFL

turns  off all flags.  They can only be enabled again by .FLAGS ALL

This will not turn off the CONTROL FLAG.

.FLAGS BREAK [new flag]

.FL BREAK [new flag]

enables  the  break flag (|) or vertical bar.  This flag is used to

mark where a word or expression may be broken.  The expression will

only be broken there if it would otherwise exceed the right margin.

Normally this flag is not enabled.

.NO FLAGS BREAK

.NFL BREAK

disables the break flag.

.FLAGS CAPITALIZE [new flag]

.FL CAPITALIZE [new flag]

enables  the  less-than (<) character to capitalize the entire word

it precedes.  It then returns the file to the  current  case  mode.

This special character is usually off and must be typed at the very

beginning of the source text to enable this  character.   Typing  a

space  returns the file to the current case lock.  The flag command

can be followed by a character to be used instead of the  less-than

(<).   (^<)  may  be  used as a permanent shift to upper case only.

(^^) will then cancel this mode.

.NO FLAGS CAPITALIZE

.NFC

disengages the FLAG CAPITALIZE command (inital setting).

.FLAGS CONTROL [new flag]

.FL CONTROL [new flag]

allows  you to specify a new CONTROL flag.  Normally it is a period

(.).  This is the code which appears at the start of a line to sig-

nal a command.

.NO FLAGS CONTROL

.NFL CONTROL

disables  recognition  of  the CONTROL flag.  Once this command has

been issued you can no longer give any  commands  including  .FLAGS

CONTROL.  So this is a non reversable command.

- 42 -

RUNOFF COMMANDS

FLAGS

.FLAGS ESCAPE [escape flag]

.FL ESCAPE

enables  the  escape sequence flags.  This allows you to issue spe-

cial control codes (escape sequences) to the printer to  perform  a

variety  of functions.  In addition you must ENABLE ESCAPE to actu-

ally output the control codes.  Normally the escape  flag  is  dis-

abled.   The  escape  flag defines a flag symbol for the escape se-

quence.  Escape sequences can be defined for  the  circumflex  (^),

back  slash (\), and an escape character of your choice.  To define

escape sequences you must use .DEFINE ESCAPE.

.NO FLAGS ESCAPE

.NFL ESCAPE

disables the recognition of the escape sequence flags.  This is the

normal default.

| .FLAGS EQUATION

| .FL EQUATION

|      Turns  on  the  equation  formatting flags.  This sets the left and

|      right braces { } as the flags to use in formatting equations.

| .NO FLAGS EQUATION

| .NFL EQUATION

|      Turns off the equation flags.

.FLAGS HYPHENATE [new flag]

.FL HYPHENATE [new flag]

enables the equals character (=) to disengage hyphenization for the

word it preceeds, or specify where rno may hyphenate inside a word.

This  special  character  is initially off and must be typed at the

beginning of the source file to enable this character.   The  FLAGS

HYPHENATE  character  is  used to disengage hyphenization for words

improperly hyphenated by the hyphenation algorithm.

.NO FLAGS HYPHENATE

.NFL HYPHENATE

Disengages the FLAGS HYPHENATE command (initial setting).

.FLAGS OVERSTRIKE [new flag]

.FL OVERSTRIKE [new flag]

enables  the  overstrike flag (%).  This flag generates a backspace

so you may overstrike the previous character.  This  allows  forma-

tion  of  composite characters to approximate scientific symbols or

add diacritical marks for foreign languages.  A different character

other than % may be assigned to this flag.

- 43 -

RUNOFF COMMANDS

FLAGS

.NO FLAGS OVERSTRIKE

.NFL OVERSTRIKE

disables the overstrike flag (%).  This is the mode RUNOFF normally

starts in, so if you wish to use this flag it must be enabled.

.FLAGS SPACE [new flag]

.FL SPACE [new flag]

enables  the  number  (#)  sign  as  a quoted non expandable space.

Every time the # appears in the text RUNOFF changes it to a  space,

but  only  1  space even if FILL is enabled.  A different character

may be assigned to this function when it is enabled.  This flag  is

normally enabled.

.NO FLAGS SPACE

.NFL SPACE

disables  the space flag (#), thus allowing the # to be used in the

text without explicitly quoting or accepting it.

.FLAGS SPECIAL [flag]

.FL SPECIAL [flag]

This sets up a single character to trigger an escape sequence.  The

flag is the character desired.  The escape sequence must be defined

with  the first character equal to the circumflex (^).  For example

you wish the left square braket ([) to output  an  escape  sequence

for  superscripting  on  most  printers.   You  enter the following

commands:

.FLAGS SPECIAL <

.DEFINE ESCAPE /^</ 33,"D"

Once  this  is done, the left square bracket is no longer printable

unless preceeded by the accept flag (_).  If you do not  include  a

flag  character  then  it  turns on processing of all special flags

previously defined.

.NO FLAGS SPECIAL [flag]

.NFL SPECIAL [flag]

This  command  turns  off a special flag.  If the flag had not been

previously defined by a .FLAGS SPECIAL command Runoff will give you

an  error message.  If the flag character is omitted, then all spe-

cial flags are turned off, until a .FLAGS SPECIAL command  is  exe-

cuted.

.FLAGS SUBSTITUTE [new flag]

.FL SUBSTITUTE

enables  the  dollar  sign  ($) as a substitution flag.  The dollar

|      sign then marks the start of a label which defines a  substitution.

|      Eight  substitutes are defined when Runoff begins.  These allow you

|      to output the current date or time as part of the text.

- 44 -

RUNOFF COMMANDS

FLAGS

|      Assuming the current date and time is

|                           January 17,1983 15:23:51

|      The substitutions are:

|           Symbol                   Resulting output

|           $$DATE                   17 Jan 83

|           $$TIME                   15:23:51

|           $$YEAR                   1983

|           $$MONTH                  January

|           $$DAY                    17

|           $$HOURS                  15

|           $$MINUTES                23

|           $$SECONDS                51

|      The  pair of dollar signs ($$) which denote the permanent substitu-

|      tions are both redefined if the substitute flag is redefined.   For

|      example  you redefine the substitute flag to be " then to print the

|      date you must type:  ""DATE .  The permanent  substitutins  may  be

|      typed  in  either  lower  or upper case letters.  The following are

|      equaly valid:  $$DATE $$date $$DaTe and so on.

.NO FLAGS SUBSTITUTE

.NFL SUBSTITUTE

disables  the  dollar sign as a substitute flag.  This flag is nor-

mally disabled.

.FLAGS UNDERLINE [new flag]

.FL UNDERLINE [new flag]

enables recognition of the underline flag (&).  This is the default

so you do not normally have to do this.

.NO FLAGS UNDERLINE [new flag]

.NFL UNDERLINE [new flag]

disables recognition of the underline flag.

.FLAGS UPPERCASE [new flag]

.FL UPPERCASE [new flag]

enables  the  circumflex (^) to engage upper case shift.  When this

is enabled the ^ will cause the next character to  be  capitalized,

and  ^^  turns  off the lower case lock.  If the circumflex is used

extensively in the text an alternate character  may  be  specified.

This  FLAG  is  normally  enabled when RUNOFF starts.  This flag is

also used as a function lock.  For example putting a circumflex  in

front of an ampersand (^&) will lock underlining on to allow under-

lining lines of text.

.NO FLAGS UPPERCASE

.NFL UPPERCASE

disables the UPPERCASE FLAG.  The ^ will then not shift text to up-

percase.

- 45 -

RUNOFF COMMANDS

ENABLE/DISABLE

V.13  ENABLE/DISABLE

The following commands can be confused with the FLAGS commands, but they

are very different.  The FLAGS command enables  recognition  of  special

characters  or  combinations  of characters.  The enable command enables

the action which that character requests.  For example  you  may  enable

the  overstrike  flag by .FLAGS OVERSTRIKE.  The percent sign ( % ) will

then cause overstriking and will not appear in the text.   If  you  then

issue  the command .DISABLE OVERSTRIKING the percent sign will still not

appear in the text, unless quoted, but no overstriking will occurr.

.ENABLE ESCAPE

.EES

enables output of escape sequences.  The escape sequence flags will

be recognized, but not executed while disabled.  This  is  the  de-

fault.

.DISABLE ESCAPE

.DES

disables output of escape sequences.

| .ENABLE EQUATION n

| .EEQ n

|      This  enables  the  equation  formatting.   The  parameter n is the

|      number of half lines to allocate above and  below  each  character.

|      The  default  is .EEQ 0.  Sometimes if you are formatting equations

|      with many subscripts or superscripts n=1 will look better by adding

|      more blank space to the fractions.

| .DISABLE EQUATION

| .DEQ

|      Disables  the  equation  writing mode.  When disabled, but with the

|      equation flags set via the .FLAGS EQUATION command, left and  right

|      braces { } are translated to be normal parentheses ( ).

.ENABLE HYPHENATION

.EHY

enables  hyphenation.  While hyphenation is enabled RUNOFF will au-

tomatically to hyphenate words at the end of an output line.   User

defined hyphenation via the hyphenate flag character [ = ] may also

be used.  RUNOFF will never hyphenate a word containing escape  se-

quences  or  unknown  punctuation.   Currently the only punctuation

marks considered valid within a word are:

/ \ .  , ( ) " @

For  example  words  containing hyphens are never hyphenated.  Such

words may be optionally broken by using the break character.

The  hyphenation routines are not ideal and there are certain prob-

lems which you must overcome manually.  The hyphenation routine has

no  warning when a paragraph is ending, consequently it may hyphen-

ate the last word in a paragraph.  This may be avoided by using the

.FLAGS  HYPHENATE  command and marking the last word in a paragraph

- 46 -

RUNOFF COMMANDS

ENABLE/DISABLE

for no hyphenation.  The hyphenation routine will never hyphenate a

word  in  the  last  line  on a page.  If you use footnotes and the

footnote overflows to the next page, the  hyphenation  routine  may

hyphenate  the  last word on a page.  This may be avoided by use of

the hyphenation flag, or by picking the page breaks with the  .PAGE

command.   It  is always a good idea to avoid footnotes which over-

flow to the next page.

.DISABLE HYPHENATION

.DHY

disables  hyphenation of text.  Neither automatic nor user selected

hyphenation will occurr.

.ENABLE INDEXING

.EIX

enables  collecting and printing of index entries.  This is the de-

fault.

.DISABLE INDEXING

.DIX

disables or stops indexing.

.ENABLE OVERSTRIKING

.EOV

enables  overstriking  by  means of the overstrike character [ % ].

Overstriking is normally enabled, but the flag is disabled.

.DISABLE OVERSTRIKING

.DOV

disables overstriking.

.ENABLE SUBSTITUTION

enables substitutions.  This is the normal default.

.DISABLE SUBSTITUTION

disables  output  of  substitutions.  If the substitute flag is en-

gaged, the substitution will be  recognized,  but  no  substitution

will be made.

| .ENABLE TABS

| .ETB

|      This  enables the use of the tab key to produce tabulated or colum-

|      nar output.  Tabs are normally enabled so you do not need to enable

|      them unless you have previously disabled them.

| .DISABLE TABS

| .DTB

|      This  disables  the  use  of tabs.  When tabs are disabled they are

|      treated as ordinary spaces.

- 47 -

RUNOFF COMMANDS

ENABLE/DISABLE

| .ENABLE TOC

| .ETC

|      this  allows  all CHAPTER commands, and HEADER LEVEL commands to be

|      output to the table of contents file.  This is normally enabled, so

|      you  do  not have to give this command unless you have disabled the

|      TOC output.

| .DISABLE TOC

| .DTC

|      prevents  any  TOC output.  Neither CHAPTER, .HEADER LEVEL, .APPEN-

|      DIX, nor .SEND TOC commands will output anything to the .RNT  file.

|      By using this command you can control which items are in the TOC.

.ENABLE UNDERLINING

.EUN

enable  underlining  using  the underline character [ & ].  This is

the default.

.DISABLE UNDERLINING

.DUN

disables underlining.

V.14  DEFINE / RESET

.DEFINE ESCAPE "escape label" [,modifiers] definition

defines  non  printing escape sequences for control of a variety of

printers.  Each escape sequence is indicated in the text by  either

\  or ^ or a symbol of your choice followed by another symbol.  For

example ^* can enable bolding by issuing  a  sequence  to  turn  on

bolding  and \* may disable bolding by issuing a turn off sequence.

The escape sequence is entered as follows.  First you select  which

2 character label.  The first character must be either ^,\, or your

flag character, while the second one may be  any  other  character.

They are entered as a literal.  For example the label ^* may be en-

|      tered as "^*" or '^*' or "^","*".  Your flag character  is  defined

|      by the .FLAGS ESCAPE command.

|      Next  the  modifiers  are entered.  These define how the escape se-

|      quence is handled.  The modifiers are:

|           LCK  -  escape sequence is a lock/unlock pair.  For example if

|                ^* and \* are defined as a  lock/unlock  pair,  then  the

|                first  appearance of ^* will "lock" this feature on and a

|                subsequent appearance of ^* will be ignored if \* is  not

|                used in between them.  Effectively the lock is ^* and un-

|                lock is \*.  In addition this allows RNO to control these

|                sequences  to  prevent  them  from acting on the headers.

|                They will only change the text, much in the same way  the

|                underline flag works.  If a line contains only the "lock"

|                sequence then the unlock sequence will  be  automatically

|                output  at the end off the line.  The next line will then

|                automatically be preceeded by the  lock  sequence.   This

- 48 -

RUNOFF COMMANDS

DEFINE / RESET

|                continues  till  the input text contains the "unlock" se-

|                quence flag.

|           VSP,spacing  -  escape  sequence  changes vertical position by

|                spacing.  This allows you  to  define  a  sequence  which

|                changes  the vertical position.  The spacing parameter is

|                specified in half lines with 1 being 1/2 line advance, -1

|                1/2 line backwards.

|           HSP,spacing  -  sequence  modifies  the horizontal position by

|                spacing.  You may define a sequence  which  modifies  the

|                horizontal position.

|           PSP  -  The  horizontal spacing specified by HSP is applied to

|                all printable characters following the  escape  sequence.

|                This  attribute  modifies the HSP attribute.  You can de-

|                fine a sequence which changes the  character  width,  and

|                let  RNO  know what the new width is.  The width may only

|                be 0,1,2,...  or multiples of the normal width.   If  the

|                HSP  is  specified as a number other than 1, certains as-

|                pects of fill are disabled.  Spaces will not be  expanded

|                if necessary to justify.  Only "normal" spaces with HSP=1

|                are expanded.  This allows imbedding extra wide text  in-

|                side  normal text, or centering extra wide text.  A whole

|                line of extra wide text can not be justified, but a mixed

|                line  of normal and wide can be justified.  This restric-

|                tion may be removed in future releases of RNO.

|           CHR  - The lock/unlock pair is used to change only 1 printable

|                character.   This  attribute  is   used   only   if   the

|                lock/unlock pair is designed to function similarly to the

|                underline flag.  You must also declare the  escape  char-

|                acter in a .FLAGS SPECIAL command.

|      Then  following  this  the  escape  sequence is entered.  Printable

|      characters may be entered as literals ('This is a  literl'),  while

|      non  printable  must  be entered as a decimal value.  The following

|      example shows the definition of an escape sequence to turn on bold-

|      ing, and turn off bolding for a FLORIDA DATA printer.

|           .DEFINE ESCAPE '^*',ES.CHR,ES.LCK,33,'W'

|           .DEFINE ESCAPE '\*',ES.CHR,ES.LCK,33,'&'

|           .FLAGS SPECIAL *

|      The  net effect of these definitions is to have ^* turn on bolding,

|      \* turn off bolding and * will bold a  single  character  following

|      it.   Bolding will never occurr for titles, page numbers and subti-

|      tles.

The  literals  may  be  enclosed in either single (') or double (")

quotes.  If a literal is started by a single quote it must be  ter-

minated by a single quote, but it may contain a double quote.  Sim-

ilarly a literal enclosed in double quotes  may  contain  a  single

quote.   The  numbers and literals may be separated by blanks,tabs,

and or commas.  Two commas in a row constitute a null and are ille-

gal.   If you define an escape sequence twice in your text only the

first definition will be obeyed unless you reset escape before  the

|      second one.  The second definition will cause an error message.

|      If you use a large number of different escape sequences frequently,

|      you should define the most commonly used ones first.  This will in-

|      crease the speed with which RUNOFF can find them.

- 49 -

RUNOFF COMMANDS

DEFINE / RESET

NOTE

Escape  sequences  are output without any interpretation.

If the sequence leaves the carriage of your printer on  a

different  column  than  before, and you have not defined

the HSP attribute, the output line will not fill or  jus-

tify properly.  Similarly any net vertical movement while

not as serious, is not accounted for if VSP  is  not  de-

|           fined.   If an escape sequence is defined which conflicts

|           with a flag, the flag will be obeyed and the  escape  se-

|           quence  ignored  if  the flag recognition is enabled by a

|           .FLAGS command.

.RESET SUBSTITUTE

resets the table of substitutions so that all the previous ones are

no  longer  defined.    The   permanently   defined   substitutions

($$DATE...) are also no longer defined.

.DEFINE SUBSTITUTE /label/ text

defines  a  substitution to be made in the text.  The label is pre-

ceeded and followed by a delimiter of your choice.  The label  must

be  no  more  than 10 characters long and may not contain spaces or

tabs.  The command and the first delimiter must be separated by one

or  more spaces or tabs, but no spaces are necessary after the sec-

ond delimiter unless you wish to include them in  the  text.   When

the  substitute  flag is encountered in your text followed by a de-

fined label,  the  specified  text  will  be  substituted  for  the

flag+label.   The  text  may contain another substitution.  Up to 3

levels of substitution may be nested.  The rules for  multiple  de-

finitions of escape sequences also apply to substitutions.

EXAMPLE:  You wish to abbreviate the phrase:

The party of the first part

So you define and enable substitution:

.DEFINE SUBSTITUTE /P1$/The party of the first part

.FLAGS SUBSTITUTE

Then  every  time  you  type into the text $P1$ you get instead the

substitute phrase:

The party of the first part

The  example  of define substitute could have used different delim-

iters if you needed to used the delimiters in the text.  For  exam-

ple it could be done as follows:

.DEFINE SUBSTITUTE :P1$:The party of the first part

...  or

.DEFINE SUBSTITUTE "P1$"The party of the first part

|      The  substitution  label or name may consist of any printable char-

|      acter.  Upper case or lowercase letters may be  used,  however  the

|      case  of the letter will be ignored except for the first character.

|      In other words the following commands are equivalent.

|           .DEFINE SUBSTITUTE /ABC/

|           .DEFINE SUBSTITUTE /Abc/

- 50 -

RUNOFF COMMANDS

DEFINE / RESET

|           .DEFINE SUBSTITUTE /AbC/

|      But the following are not equivalent.

|           .DEFINE SUBSTITUTE /aBC/

|           .DEFINE SUBSTITUTE /ABC/

|      A  special  convention  is  used to define DSR compatible substitu-

|      tions.  If the first character of the substitution name is a dollar

|      sign,  the dollar sign stands for the substitute flag.  If the sub-

|      stitute flag is changed to a character other than the  dollar  sign

|      then  the  defined substitution is considered to begin with the new

|      substitute flag.  When the substitution is searched for,  the  case

|      of the input characters is ignored.  An example of such a substitu-

|      tion is the $$DATE permanently defined substitution.

|            For example the following input:

|                 .FLAGS SUBSTITUTE @

|                 .DEFINE SUBSTITUTE /$TEST/This is a test

|                @@test !!

|            Will produce the following line:

|                 This is a test !!

The substitution is a really invaluable aid for defining complicat-

ed mathematical symbols.  All you need to enter in the text is  the

simple abbreviation and the complicated symbol will be substituted.

The substitution may contain all kinds of  escape  sequences  over-

strike symbols and so on to construct the special symbol needed.

V.15  MISC COMMANDS

.REQUIRE 'filename'

.REQ 'filename'

takes the input text from the specified file.  The .REQUIRE command

may be imbedded in files which have been  required.   The  required

files  may  be nested until 5 input files are open at once.  An at-

tempt to require a sixth level of input will cause an  error.   The

default  filetype  is .RNO.  The filename may be enclosed in either

single (') or double (") quotes.  If any commands follow this  com-

mand on the same line, they will be ignored.

This  command makes it easy for you to break up your text into rea-

sonable chunks.  For example you may wish to put each section in  a

separate  file and then have a master file which requires each sec-

tion in order.  If you need to rearrange the order of the  sections

you only need to change the .REQUIRE statements in the master file.

You also should put the overall formatting commands such  as  .PAGE

SIZE,  .DISPLAY, .HEADER ...  into a separate file which may be re-

quired at the beginning of each section.

You  may also wish to build up files of commonly used substitutions

and escape sequences.  These can then be easily required at the be-

ginning of any file that needs them.

Another  example is generation of a series of letters with the same

body but different inside addresses  and  salutations.   You  would

make a master file containing the inside addresses and salutations.

- 51 -

RUNOFF COMMANDS

MISC COMMANDS

After each salutation you might put:

.REQUIRE "LETTER"

You would then create a file LETTER.RNO with the actual text of the

letter.  By running the master file through RNO  you  would  get  a

string  of  personalized letters.  Later on you may change the con-

tents of LETTER.RNO to get another set of personalized letters  for

mass mailings.

.COMMENT text

text

causes the line to be ignored.  The text is not printed in the out-

put .  Comments may contain useful information which you need  when

re-editing the file.  For example you may wish to add comments to a

string of .DEFINE ESCAPE and .DEFINE SUBSTITUTE to remind you  what

each escape sequence and substitution does.

V.16  INDEXING

Runoff  can produce an index to your document.  To do this you must

use the .INDEX command for each occurrence of a keyword you wish to

index.  This does not automatically cause all occurrences of a key-

word to be indexed.  If you wish to automatically index all  occur-

rences of a keyword you may do this by using an appropriate editor.

The editor must search for the keyword, form a new line  after  the

keyword  and insert .X keyword on the new line.  The index facility

is intended to produce indexes not  concordances.   Then  when  you

wish to print the index, you use .PRINT INDEX or .DO INDEX.

.INDEX [text]

.X [text]

takes  the  remaining  text  on  the line as a keyword and adds it,

along with the current page number, to the internal  index  buffer.

The command does not cause a break.  If in FILL mode the index com-

mand must appear immediately after the item to be indexed.  If  you

are in NO FILL mode the index command must appear immediately after

the line containing the index item.  If the line is broken  because

it  is too long the index page number may be incorrect.  If you de-

clare a .TEXT section then the .INDEX command should be at the  end

of  the  section before the next .TEXT or .END TEXT.  Overstriking,

substitution, and escape sequences are not allowed  in  index  key-

words.   A given keyword may be indexed more than once.  If a kewy-

ord is indexed more than one time on a page the extra times are ig-

nored.

SEE also the commands .ENABLE INDEXING and .DISABLE INDEXING.

.DO INDEX [text]

.DX [text]

First  the  permanent margins, and line spacing are restored.  Then

this command starts a new page, centers the text, if given,  other-

wise  it  centers the word " INDEX ".  This command then prints the

entire contents of the index buffer.  Entries are printed in alpha-

betic  order  and  are  set  against the left margin.  Regular line

- 52 -

RUNOFF COMMANDS

INDEXING

spacing is used, except that a new paragraph is formed between  en-

tries  with different first letter.  The entries of different first

letter may be separated by non zero paragraph spacing,  and  orphan

lines  at  the  end of the page are prevented by the paragraph test

page value.  The paragraph indentation is used to indent extra page

numbers which can not fit on the line with the index entry.

If  both  FILL and JUSTIFY are enabled the entry is followed by el-

lipses ( .  .  .) and the page numbers are printed right  justified

with  respect to the current right margin.  If FILL is disabled the

ellipses are omitted.  If JUSTIFY is disabled the page numbers will

start 5 spaces after the index entry, and no justification will oc-

curr.

.PRINT INDEX

.PX

This  prints  the entire contents of the index buffer.  The current

margins, and spacing are not changed, nor is a  new  page  started.

The  format  of  the index is the same as for .DO INDEX except that

parameters are not reset and no header is formed.

V.17  TABLE OF CONTENTS

| There  is  now a table of contents feature.  A table of contents file is

| automatically output if a second  output  filespec  is  specified.   The

| table of contents file is labeled .RNT by default.  For example you wish

| to get a table of contents and regular runoff file.  You type:

|           >RNO EXAMPLE,EXAMPLE=EXAMPLE

| The  EXAMPLE.RNT  file  must then be passed through RNO to get the final

| TOC.  You may edit the .RNT file if it is not satisfactory.   You  would

| do the following:

|           >RNO EXAMPLE.TOC=EXAMPLE.RNT

| You  now  have the final EXAMPLE.DOC and EXAMPLE.TOC files.  They may be

| printed separately or a new composite file may be created:

|           >COPY EXAMPLE.TOC,EXAMPLE.DOC EXAMPLE.TXT

|                or

|           >PIP EXAMPLE.TXT=EXAMPLE.TOC,EXAMPLE.DOC

| Now the composite file EXAMPLE.TXT may be printed as a whole document.

| The following commands control the TOC output.

| .SEND TOC text

| .STC text

|      sends the "text" to the .RNT file.  The text may be used to control

|      the TOC output.  The text may actually be a set of commands for ex-

|      ample  .REQUIRE "FORMAT" to insert a set of commands to control the

|      format of the TOC.  If the same command appears in  the  text  then

|      you  can make sure the TOC and documentation all have the same for-

|      mat.

- 53 -

RUNOFF COMMANDS

TABLE OF CONTENTS

| .TOC

|      You  do  not normally use this command.  It is automatically placed

|      at the head of a .RNT file.  It enables formatting for the table of

|      contents.  It does the following:

1.  New pages for each chapter are inhibited.

2.  All tab stops are cleared.

3.  .NO NUMBER CHAPTER

4.  .STYLE CHAPTER 1,-1,-1,0,2

5.  .STYLE HEADERS ,,,,,0,0

6.  .INDENT LEVELS 8,8

| .END TOC

|      You  would  not normally use this command unless you wish to append

|      more text to the .RNT  file  for  processing  with  the  same  page

|      numbering as the table of contents.  This command is the complement

|      to .TOC.  It ends the table of contents and allows normal text pro-

|      cessing.  It does the following:

1.  New pages are allowed at the start of each chapter.

2.  Tab stops are NOT changed.

---

3.  .NUMBER CHAPTER

4.  .STYLE CHAPTER 12,1,3,-1,-1

5.  .STYLE HEADERS ,,,,,2,1

6.  .INDENT LEVELS 0,0

| The  format of the final table of contents may be varied either by edit-

| ing the .RNT file or by using .STC to add extra commands.  The following

| hints may help you in designing your own format.

1.  The  .RNT file needs .FILL to operate properly so do not edit a

|          .NO FILL command into it.

|      2.  The  page  numbers  are  right justified using tabs without el-

|          lipses enabled.  If you want ellipses used in the table of con-

|          tents  you should .SEND TOC .ELLIPSES.  le .ENABLE LEVELS n1,n2

|          may be used to automatically determine which header levels  are

|          printed in the table of contents.  The parameter n2 is the max-

|          imum header level to output to the .RNT file and hence the max-

|          imum to appear in the table of contents.

|      3.  A title page may be placed in front of the table of contents by

|          putting it into a separate file called TITLE and inserting .STC

|          .REQUIRE "TITLE" at the front of the .RNO file.

4.  You  may  wish  to  number the table of contents in lower roman

|          numerals by using the command .STC .DISPLAY NUMBER RL.

5.  The  .STYLE commands may be inserted into the .RNT file to con-

|          trol the actual format of the final table.  By doing  this  you

|          can  vary  the spacing between the entries, the spacing between

|          items in the entries, the Capitalization of the  header  levels

|          and so on.

6.  .INDENT  LEVELS can also be used to change the HEADER LEVEL in-

|          dentation.

7.  Finally  the foreward or preface may be inserted into the table

|          of contents file bye .STC .REQUIRE "PREFACE" and it will follow

|          the  same  numbering as the table of contents.  You may wish to

|          .END TOC at the beginning of the PREFACE.RNO file.  This should

- 54 -

RUNOFF COMMANDS

TABLE OF CONTENTS

|          only be necessary if the preface contains section headers.

V.18  EQUATION FORMATTING

| An  equation  formatting  routine has been added to RUNOFF.  This is de-

| signed to format multiple nested fractions.  To use  this  facility  the

| equation  flag  must  be  enabled via the .FLAGS EQUATION command.  Sub-

| scripts or superscripts are normally taken care of in  formatting  frac-

| tions, but you may wish to add extra space between the line and the num-

| erator or denominator.  To do this give the command .ENABLE EQATION 1 to

| add  1/2  line  or  .ENABLE  EQUATION 2 to add 1 whole line.  With these

| preliminary commands out of the way the braces { } are now  flags  which

| are  used in formatting equations.  They must be preceeded by the under-

| score _ if they are to appear in the text.

| Fractions  are  formatted in the following manner.  You express the fac-

| tion as:

|           {NUMERATOR}/{DENOMINATOR}

| and RNO will output:

|            NUMERATOR

|           ___________

|           DENOMINATOR

| Fractions       may      be      nested      inside      each      other

| {{A}/{B} + {C}/{D}}/{DENOMINATOR}.  The macimum nesting is about 6 deep.

| If the nesting is all in numerators then it is limited to 12, all in de-

| nominators is limited to 6.  Spaces or tabs may be freely  used  between

| numerators  and  denominators  outside  of the braces for clarity.  They

| will not appear in the final output.  For example  {A}/{B}  will  format

| the  same  as  {A} / {B}.   Certain  features will no longer work inside

| fractions.  Normal spaces are treated as non expandable,  and  tabs  are

| likewise  treated  as  non expandable spaces.  As a result fractions are

| never expanded to right justify the text.

| One  warning  is  in order.  Fractions may not be split between 2 output

| lines.  If this occurrs because the text is too long,  you  will  get  a

| string of error messages and the output will be messed up.  You certain-

| ly may use more than 1 input line to format the fraction.  You may  wish

| to use the command .NO SPACE to prevent extra spaces from being automat-

| ically inserted.

| The commands used to control equation formatting are:

1.  .FLAGS EQUATION

2.  .NO FLAGS EQUATION

3.  .ENABLE EQUATION n

4.  .DISABLE EQUATION

NOTE

|      This  equation  routine will only work properly with word pro-

|      cessing style hard copy printers which support sub/superscript

|      escape     sequences    and    backspacing.     In    addition

|      sub/superscripting escape sequences must be defined  with  VSP

- 55 -

RUNOFF COMMANDS

EQUATION FORMATTING

|      parameters.

V.19  CONDITIONAL TEXT

| Sometimes  you may wish to have 2 different copies of the same document.

| There are many ways to achieve this goal.  Only 3 methods will be consi-

| dered.

1.  .REQUIRE  may  be  used to achieve this goal.  You can break up

|          the document into small sections, and  then  set  up  2  master

|          files  which  you  might  call  DOCUMENT1.RNO or DOCUMENT2.RNO.

|          Each master file would then require only those sections  appro-

|          priate to generating each different document.

2.  SUBSTITUTIONS  can  be  used to change a document.  For example

|          you may wish to generate different documents each personalized.

|          You  could  define different substitutions which would generate

|          different output.

3.  Finally  you  could  use the .IF, .IF NOT, .ENDIF, and .VARIANT

|          statements to conditionlize sections of a document.  These  are

|          shown below.

NOTE

|      The  .IF,  .IF NOT, and .ENDIF commands must be the first com-

|      mand on a line.

| .IF aaa

|      This  statement causes the text following it to be ignored until an

|      .ENDIF aaa statement is encountered.  The aaa is a label which con-

|      trols the action of the .IF.  If aaa has been previously defined in

|      a .VARIANT statement then the text is not skipped.  If not  defined

|      the text is skipped.

| .IF NOT aaa

|      This statement causes the text following it to be ignored if aaa is

|      undefined.  In other words if a .VARIANT statement preceeds the .IF

|      NOT with the label aaa then the text is skipped.

| .ENDIF aaa

|      This  statement  terminates  the range of the .IF or .IF NOT state-

|      ments.  The label aaa must be the same as the previous .IF which is

|      terminated.

- 56 -

RUNOFF COMMANDS

CONDITIONAL TEXT

| .VARIANT aaa

|      This statement defines a label aaa which is used to control the ac-

|      tion of subsequent .IF commands.  aaa may be up to 10 characters in

|      either upper or lower case.  The case is ignored and ABC or abc are

|      equivalent labels.  If the label is defined in the  VARIANT  state-

|      ment  then  .IF  aaa  will cause text to be printed and .IF NOT aaa

|      will suppress text.

|           For example:

|                .VARIANT NOTES

|                .IF notes

|                This line is printed

|                .ENDIF notes

|                .IF NOT notes

|                This line is not printed

|                .ENDIF notes

- 57 -

APPENDIX A

RUNOFF LIMITATIONS

Runoff  has  many limitations as a general text formatting program.  One

of the main limitations is the inability to look ahead  and  format  the

text  based  on what will be happening in the next few lines.  Here is a

partial list of some of the implications of this inability, and what you

can do to alleviate this.

1.  RUNOFF has the annoying habit of leaving single lines of a par-

agraph dangling on the next page.  This can be fixed by hinting

that  the  paragraph is about to end by issuing .IMMEDIATE TEST

TEXT 2 about 2 lines before the next paragraph begins.

2.  Sometimes  a  word  can  be hyphenated between pages.  This can

only happen on a page with a footnote.  Hyphenation is  usually

inhibited  for the last line of a page.  However a footnote can

confuse RUNOFF if the end of the page  is  reached  during  the

.FOOTNOTE  command.   The  only solution to this is to .DISABLE

HYPHENATION or use the hyphenation flag to declare the word in-

elegible for hyphenation.

3.  Similarly  RUNOFF  can  hyphenate the last word of a paragraph.

This is easily fixed by using the hyphenate flag  in  front  of

the last word of each paragraph.

4.  Footnotes  may  appear on the page before the line that referrs

to them.  This may be fixed by starting the  footnote  about  1

line after the reference.  This "fix" may make the footnote ap-

pear on the next page after the reference, if the reference  is

too  near  to the bottom of the page.  The .IMMEDIATE TEST TEXT

command may then be used before the reference to force both the

reference and footnote onto the same page.

- 58 -

APPENDIX B

FILL/JUSTIFY

The commands .FILL and .JUSTIFY allow the user a large number of options

for the final text format.  To understand how these work  the  following

rules may clarify their operation.

FILL

1.  When  .FILL  is  enabled,  multiple or reduncant spaces are re-

moved.  This includes the following:

(A)  Spaces following spaces.

(B)  Spaces following non expandable spaces " "

(C)  Spaces following "tabs"

(D)  Spaces at the beginning of a line.

2.  When .FILL is enabled the end of a line is treated exactly like

a space.

3.  If both .FILL and .PERIOD are enabled a punctuation followed by

a space is converted to "punctuation", "space", "non expandable

space".

4.  If  .NO  FILL  is used, the end of a line is treated as a break

("|").

5.  Regardless  of  whether .FILL or .NO FILL is used, if the input

line is longer than the available page size, and it contains no

"normal"  spaces,  it  will  be  truncated to fit the available

space.  The only exception to this is when the line contains  a

"break" character ("|").  Then the line is divided at the break

character to make it fit.

JUSTIFY

1.  If .JUSTIFY is enabled then all "normal" spaces are expanded or

padded to right justify the final output.

2.  If a break occurrs between lines, then the previous line is NOT

---

justified.

3.  If  an  input  line is shorter than the output line size and it

contains no "normal" spaces, the line  can  not  be  justified.

This is indicated by an error message:

"Can't justify line"

4.  If  .NO JUSTIFY is used then the output will not be padded with

extra spaces , but if .FILL is enabled it will be  missing  re-

dundant spaces.

- 59 -

APPENDIX C

RNO ERROR MESSAGES

RUNOFF produces error messages in the following format:

RNO -- error message

This appendix tabulates the error messages produced by RNO.  Most of the

error messages are self-explanatory.  In some cases, RNO prints the line

in which the error has occurred, so that the user can examine the source

line which caused the problem and correct it.  If  the  source  line  is

printed  it is printed only up to the point at which the error occurred.

The error is generally the last character, or word in the source.   Fol-

lowing the source line is the line and page number of the output text on

which the error will cause problems.

1.  RNO -- Bad parameters

This  message  is given if any of the parameters which follow a

command are not "correct".  The command is listed up to the bad

parameter.   An  example of a bad parameter would be .INDENT 25

when you only have 20 characters per line.  Remember  that  the

minimum  number  of  characters  per line is 16 and the minimum

number of lines/page is also 16.  The following  is  a  partial

list of commands which generate this error.

(A)  .DEFINE  ESCAPE  accepts only decimal or literal input, and

any other input constitutes an error.

(B)  .DEFINE  SUBSTITUTE  must have a label surrounded by delim-

iters and this error will occurr if the delimiters are  not

present.

(C)  .DISPLAY either the format is not one of the following:

D, LU, LL, LM, RU, RL, or RM.

or The pre-header is more than 10 characters

or The post-header is more than 4 characters

or  The  post-header  is specified, but no format is speci-

fied.  You may however specify a pre-header with no  format

or post-header.

(D)  Commands  which  take literals as input will generally give

this error if the apostrophe (') or quotes (") are missing.

(E)  Most  commands  check input numbers and if they are too big

or too small this error will result.  For  example  .HEADER

LEVEL 7 is nonsensical and is flagged as an error.

2.  RNO -- Can't indent list element

The  user  has nested lists that cause RNO to attempt indenting

beyond the right margin.

3.  RNO -- Can't justify line:

RNO found a line of text containing no spaces (other than quot-

ed spaces), so the line could not be justified.

- 60 -

RNO ERROR MESSAGES

4.  RNO -- Can't reduce margins

A  .NOTE  command occurred after the margins were set to values

that prevent the .NOTE command from reducing margins by 5.  The

margins can't be reduced to less than 16 characters/line.

5.  RNO -- Character already defined as flag

In attempting to enable a character as a flag you have used the

same same character as a previous  .FLAGS  command.   The  same

character  may  not be used as 2 different flags.  You must de-

cide which flag you want and either disable the  previous  flag

via  a  .NO FLAGS command, or use a different character for the

second flag.

6.  RNO -- Character specified is not special flag

You  have  attempted to disable an individual special flag, but

the flag character you specified was not previously defined  as

a special flag in a .FLAGS SPECIAL command.

7.  RNO -- Command string syntax error

You  have  entered  a command string in a format that RNO can't

decipher.  The general form of the RNO command string is:

Documentfile,Tocfile=Inputfile/Switches

The  switches  or  Tocfile  may be omitted, but the other items

must be specified.  See the section RNO COMMAND STRING.

8.  RNO -- Extraneous chars in command

One  or more extraneous characters appear after a legal command

and before the next command.  These  are  either  typographical

errors or you used the command improperly.  The command is exe-

cuted in either case.

9.  RNO -- Illegal command

This  message encompasses a large range of errors.  The follow-

ing is a partial listing of the commands which can be illegal.

(A)  .END FOOTNOTE if no .FOOTNOTE command preceeds it.

(B)  .END LIST if no .LIST command preceeds it or if the spacing

is greater than 5 or less than 0.

(C)  .END LITERAL if no .LITERAL command preceeds it.

(D)  .END NOTE if no .NOTE command preceeds it.

10.  RNO -- Illegal command after .LOCK command

You used .LOCK at the head of the document and subsequent ille-

gal commands were encountered.  You  can  probably  ignore  the

error  message if your document is satisfactory.  The inclusion

of the .LOCK command was designed  to  exclude  the  statements

which are now illegal.

11.  RNO -- Illegal command after a .TEXT command

You  probably  forgot to end the text section with an .END TEXT

command.

- 61 -

RNO ERROR MESSAGES

12.  RNO -- Illegal command during footnote

You  probably  forgot  to include an .END FOOTNOTE statement at

the end of your footnote.  You should be aware that  statements

which change the overall page format are forbidden during foot-

notes.

13.  RNO -- Illegal command during note

You probably forgot to end your note with an .END NOTE command.

14.  RNO -- Illegal flag character

Only special characters may be used as flags.  This includes:

^ !  " # $ % & ' ( ) * = [ ] { } ~ + ` | < > ?  _ \ / , .

No numbers or letters may be used as flag characters.

15.  RNO -- Input file error

A read error has occurred on an input file.  One of the follow-

ing conditions may exist:

(A)  The  file  contains a line of text with more than 512 char-

acters.  You should edit the file and shorten all such long

lines to less than 512 characters.

(B)  A  problem  exists with a physical device (e.g.  device cy-

cled down).

(C)  The file is corrupted or the format is wrong.

(D)  The file is a nonsource file.  (Object file, etc.)

16.  RNO -- Input file open error

The  file  system,  while attempting to open an input file, has

detected an error.  One of the following conditions may exist.

(A)  The user directory file is protected against an open.

(B)  A problem exists with the physical device.

(C)  The volume is not mounted.

(D)  The specified file directory does not exist.

(E)  The file does not exist as specified.

17.  RNO -- Input file specification error

An  error  exists in the input file specification.  Correct the

specification and re-enter the command.

18.  RNO -- Insufficient dynamic memory

One of the following commands filled up the dynamic memory.

(A)  .FOOTNOTE

(B)  .TEXT

(C)  .DEFINE ESCAPE - permanent

(D)  .DEFINE SUBSTITUTE - permanent

(E)  .INDEX - permanent

All  index  entries, escape sequences, and substitutions reduce

dynamic memory permenantly until the end of the  current  docu-

ment while footnotes and .TEXT only use it temporarily.  If you

define many substitutions or use .INDEX frequently you must cut

down  on these statements.  If it occurred during a footnote or

.TEXT you can  reduce  the  amount  of  text  before  the  .END

- 62 -

RNO ERROR MESSAGES

statement.   Instead  of  .TEXT and .END TEXT use .TEST PAGE or

.TEST TEXT.  If you are currently using the non  overlaid  ver-

sion of RNO switch to the overlaid version.

19.  RNO -- Internal error - Contact your system manager

This  message  is  caused  by  an  internal program error.  You

should never get this message.  If you get this message contact

the  system  manager.   This  indicates that RNO is not working

properly and should be fixed.  It is  also  possible  that  you

have  a hardware malfunction if the same version of RNO used to

work on the same identical text.  Identical means no changes at

all!!

20.  RNO -- Maximum list level exceeded

You have attempted to create more than 6 nested lists.  That is

you issued the command .LIST more than 6 times without any .END

LIST  statements.  The 7'th and subsequent .LIST statements are

ignored.

21.  RNO -- Missing parameter

This command requires a parameter.  Look up the command and add

the parameter to the command.

22.  RNO -- Missing right braces in equation

One of the following errors have been made:]

(A)  You  forgot  to close a fraction with a right brace }.  The

number of right braces must be the same as  the  number  of

left braces in all fractions.

(B)  The fraction was too wide to fit on the current line and it

overflowed to the next one.

23.  RNO -- Output file error

A write error has occurred on the output file.  One of the fol-

lowing conditions exists:

(A)  The device is full or is write protected.  The hardware has

failed.

(B)  The user does not have write access privileges to the file.

24.  RNO -- Output file open failure

The  file system, while attempting to open the output file, has

detected an error.  One of the following conditions may exist:

(A)  The user's directory file is protected against an open.

(B)  A problem exists with the physical device.

(C)  The volume is not mounted.

(D)  The specified file directory does not exist.

(E)  The device is full of write protected.

25.  RNO -- Output file specification error

An  error exists in the output file specification.  Correct the

specification and re-enter the command.

- 63 -

RNO ERROR MESSAGES

26.  RNO -- Permanent footnote error

You  specified a permanent footnote, while a footnote was pend-

ing for the current output page.  You must specify the  perman-

ent footnote first.

27.  RNO -- Symbol already defined

This is one of the following errors.

(A)  .DEFINE  ESCAPE - The escape sequence you are attempting to

define is already  defined.   Either  change  the  sequence

name, or .RESET ESCAPE.

(B)  .DEFINE  SUBSTITUTE - The substitution name or abbreviation

has already been defined.  You  must  change  the  name  or

.RESET SUBSTITUTE.

28.  RNO -- Syntax error in equation

You  have incorrectly formatted a fraction.  Fractions are for-

matted as:

{Numerator}/{Denominator}

You have made one of the following mistakes.

(A)  There  are  extraneous characters between the numerator and

denominator.   For  example  the  following  is  an   error

}/Denominator

(B)  You  have  started  a numerator or denominator with a right

brace } when you should use a left brace {.

(C)  You  have omitted the slash / between the numerator and de-

nominator.  The following expression is an example of  this

error:

{Numerator}{Denominator}

29.  RNO -- Too many nested fractions

The  equation  flag  has been engaged, and an attempt to format

fractions has been made using the braces { }.  Too  many  frac-

tions  have  been nested on top of one another.  For example if

you input the following expression:

{{A}/{B}}/{Denominator}

You have nested fractions up to a depth of 2.  When the nesting

exceeds a depth of 6 the equation formatting may fail.  See the

section on EQUATION FORMATTING.

30.  RNO -- Too many nested .REQ

More  than  5  nested levels of .REQUEST have been found.  This

means that the original file contained  a  .REQ  for  a  second

file,  which contained a .REQ for a third file and so on.  When

the .REQ for a sixth file was issued, it was ignored  and  this

error message was issued.  To fix this error you must rearrange

your files to prevent more than 5 input files being in  use  at

one time.

31.  RNO -- Too many nested substitutions

More  than  3 levels of substitution have been attempted.  This

means that the first substitution contained a second, the  sec-

ond  contained  a  third and the third contained a fourth.  The

- 64 -

RNO ERROR MESSAGES

fourth level is ignored.

32.  RNO -- Too many tabs specified

The  maximum  number  of tabs stops you may specify is 32.  Any

specified beyond the thirty-second one are ignored.

33.  RNO -- Undefined sustitution

The  substitution  flag  ($)  was  followed  by an unrecognized

label.  Either you made a typographical error in the  label  or

you  wished to print the substitute flag character in the text,

but forgot to preceed it with an Accept flag character(_).

34.  RNO -- Unrecognized command

You  probably  misspelled  the  command  or  abbreviation.  You

should be aware that any input line is considered to begin with

a  command  if  the  first  character  in column 1 is a decimal

point.  If you wish to begin a line  of  text  with  a  decimal

point  insert  a  space before it to prevent its being confused

with a command.

- 65 -

APPENDIX D

BUILDING RUNOFF

RSX-11M MAPPED SYSTEMS

Procedure:

1.  Use UFD to set up the proper UFDs, if necessary.

2.  Select the proper task build file:

(A)  RNOBLD  Overlayed

(B)  RNOBLDFCS  Overlayed, resident library

(C)  BIGRNOBLD  Non overlayed, resident FCS library

(D)  RNOIASBLD  IAS overlayed

3.  Select  the  options you require by editing the RNPRE.MAC.  For

example the conditional $FLORD may be enabled by  deleting  the

semicolon.   This  will enter a default set of escape sequences

for the FLORIDA DATA printer.

4.  Edit  the  Task  Build  command file to reflect your individual

system needs.  The Task Build command file contains  parameters

to  set the default switches, default paper size (for /-FF) and

default underline mode.  If you use lots of index terms or oth-

erwise make heavy demands on dynamic memory RNO will run faster

by increasing EXTSK parameter.  The overlayed version has  lots

of room for dynamic memory extension and runs nearly as fast as

the non overlayed version.

5.  Assemble

MAC @RNOASM

6.  Build the task

TKB @RNOBLD .....or

TKB @RNOBLDFCS ..or

TKB @BIGRNOBLD

7.  The  final RNO.TSK should be copied to LB:[1,54] if you wish to

make it available to all users.

RSX-11M UNMAPPED SYSTEMS

Users  of  unmapped  systems  should  follow  the procedure outlined for

mapped systems, but selecting UICs as appropriate.  File RNOBLD.CMD will

have  to be edited to delete the /MM switch and to change the PAR direc-

tive to match system requirements.

RSX-11D/IAS

Users  of  RSX-11D/IAS should follow the procedure outlined for users of

RSX-11M Mapped systems.  Since the conventions  for  source  and  object

- 66 -

BUILDING RUNOFF

files  are installation dependent, all .CMD and .ODL files should be ed-

ited  accordingly.   Additionally,  RSX-11D/IAS   users   should   alter

RNOBLD.CMD  to change the /MM switch to /MU and to change the PAR direc-

tive as needed.

Alternatively, IAS users may use command file RNOIAS.CMD to assemble and

build RUNOFF.  The command file contains parameters to set  the  default

switches, default paper size (for /-FF) and default underline mode.

VAX/VMS users

This program must be built under compatibility mode using MCR.  The com-

mands are:

MCR MAC @RNOASM

MCR TKB @RNOBLD

Otherwise  the 11-M proceedure should be used.  The final RNO.EXE should

be copied to SYS$SYSTEM with W:r protection if all users are to be  able

to use it via the $ MCR RNO command.  For convenience the following sym-

bol may be defined:

$ RNO :== MCR RNO

This makes RNO work with the same commands as on an 11/M system.

RESTRICTIONS

This version of RNO uses the SOB instruction.  If your machine (11/20 or

11/10) doesn't  have  this  instruction  it  may  be  simulated  in  the

RNPRE.MAC file.  Since almost every PDP-11 has this instruction you will

probably not have to worry about this.  In addition the prefix file  has

a symbol $FLORD to define a set of default escape sequences for the Flo-

rida data printer.  If you wish to define your own internal set  of  de-

fault escape sequences the default table is in routine INIT.

|                               BUFFER SIZES

| The  maximum  input line is currently set to 512 characters.  If this is

| too short you may redefine the symbol IBFSZ in RNPRE.MAC.  You should be

| aware  that  if you do this the program will grow in size at the expense

| of dynamic memory.  The maximum output line  is  256.   characters.   If

| this  is  not  correct  you  may redefine it by changing symbol OBFSZ in

| RNPRE.MAC.  This should only be necessary if you need shorter  lines  to

| pass to another program, or if you need longer lines in conjunction with

| the /-CR switch.  Normally each line of printed text will occupy 1  out-

| put  record, unless you have a large number of imbedded escape sequences

| which cause it to grow beyond 256.  If it is longer than 256 it is split

| into several records.  This causes no problems as long as you do not use

| /-CR.  If you need to print more than 150 characters on a line  and  you

| use  /UL:S  or /UL:L the underline buffer size may not be adequate.  The

| current definition is symbol ULNSZ found in RNPRE.MAC.

|                                EFFICIENCY

| The  overlay  structure places all of the commonly used routines and the

| hyphenation routine in  the  same  overlay.   The  .LIST,.LIST  ELEMENT,

| .CHAPTER,  .APPENDIX,  .NOTE,  .HEADER  LEVEL commands are in a separate

| overlay.  The least used commands such as .STYLE, .LAYOUT, .DISPLAY  are

| in  a third overlay.  This structure should be only slightly slower than

| a non overlaid program, and it contains  a  maximum  of  dynamic  memory

| space  (16+  Kbytes).   With  this document the non overlaid version was

- 67 -

BUILDING RUNOFF

| only about 10% slower.  Anyone contemplating using the  SUBSTITUTE  com-

| mands,  or  indexing will probably need the overlaid version.  If no in-

| dexing is done, and a limited number of escape sequences, and  substitu-

| tions  are  defined,  the non overlaid version will also probably do the

| job.  One way of increasing the efficiency of RUNOFF is to turn off  au-

| tohyphenation.  This will prevent excessive overlay switching as well as

| eliminating some code.  If you still want autohyphenation for most  text

| you  can  still  achieve greater efficiency by turning it off only while

| processing lists with many items.  For this document turning off hyphen-

| tation  only  decreased  the time by about 6%.  One more way to increase

| efficiency is to increase the the task extension.  This only has an  ef-

| fect  if  large  amounts  of  dynamic memory are used.  Large footnotes,

| large .TEXT sections, or many large definitions may make this option ne-

| cessary.   In  general most users will find existing dynamic memory ade-

| quate.

NOTE

This  program  has  been successfully run on RSX-11M, VAX/VMS,

and IAS systems.  It has not been tried on  unmapped  systems.

It  has  1  major  known bug.  Indirect command files will not

work as input to RUNOFF when executing on IAS.  They work fine

for RSX or VMS systems.

- 68 -

APPENDIX E

SPECIAL CHARACTERS

Below  is a list of RUNOFF's special characters.  To appear in the text,

each must be preceeded by the underscore  character  (itself  a  special

character).

^ UPPERCASE shift or LOCK function

\ LOWERCASE shift or UNLOCK function

< CAPITALIZE word flag character *

# SPACE character (non expandable or quoted space)

= HYPHENATE specify or temporary autohyphenate disable. *

& UNDERLINE the following character or ^

_ ACCEPT the following character.

% OVERSTRIKE the previous char by the next char *

$ SUBSTITUTE defined text for abbreviation *

| BREAK text with no spaces *

{ EQUATION flag  1 *

} EQUATION flag  2 *

-------- footnote --------

* Only becomes a special character if the FLAG is engaged.

- 69 -

APPENDIX F

LIST OF COMMANDS (ALPHABETICAL)

* Indicates the command causes a line BREAK.

$ Indicates a command forbidden during a FOOTNOTE.

D Indicates the default setting

~ Indicates not available in DSR

Command                                      Abbreviation   Opt.

-------                                      ------------   ----

APPENDIX [title] . . . . . . . . . . . . . . AX  . . . . .  *$

AUTOHYPHENATE  . . . . . . . . . . . . . . . AH  . . . . .   $D~

AUTOPARAGRAPH  . . . . . . . . . . . . . . . AP  . . . . .   $

AUTOSUBTITLE . . . . . . . . . . . . . . . . AST . . . . .   $

AUTOTITLE  . . . . . . . . . . . . . . . . . AT  . . . . .   $D~

BEGIN BAR  . . . . . . . . . . . . . . . . . BB

BLANK [-space] . . . . . . . . . . . . . . . B . . . . . .  *

BREAK  . . . . . . . . . . . . . . . . . . . BR  . . . . .  *

CENTER [LINE][+-n] . . . . . . . . . . . . . C [LINE]  . .  *

CENTER TEXT [+-n]  . . . . . . . . . . . . . C TEXT  . . .  *  ~

CENTRE [+-n] . . . . . . . . . . . . . . . . C . . . . . .  *

CHAPTER [title]  . . . . . . . . . . . . . . CH  . . . . .  *$

CHAPTER LAYOUT [code][,spacing]  . . . . . . CHLO  . . . .   $ ~

COMMENT [comment text] . . . . . . . . . . . ;

DEFINE ESCAPE escape definition  . . . . . . . . . . . . .   $ ~

DEFINE SUBSTITUTE /label/text  . . . . . . . . . . . . . .   $ ~

DISABLE BAR  . . . . . . . . . . . . . . . . DBB . . . . .    D

DISABLE ESCAPE . . . . . . . . . . . . . . . DES . . . . .     ~

DISABLE EQUATION . . . . . . . . . . . . . . DEQ . . . . .     ~

DISABLE HYPHENATION  . . . . . . . . . . . . DHY

DISABLE INDEXING . . . . . . . . . . . . . . DIX

DISABLE TABS . . . . . . . . . . . . . . . . DTB . . . . .     ~

DISABLE TOC  . . . . . . . . . . . . . . . . DTC

DISABLE OVERSTRIKING . . . . . . . . . . . . DOV

DISABLE UNDERLINING  . . . . . . . . . . . . DUN

DISPLAY APPENDIX ["pre",][form][,"post"] . . DAX . . . . .   $

DISPLAY CHAPTER ["pre",][form][,"post"]  . . DCH . . . . .   $

DISPLAY ELEMENTS ["pre",][form][,"post"] . . DLE . . . . .   $

DISPLAY LEVELS [f1],[f2],... . . . . . . . . DHL . . . . .   $

DISPLAY NUMBER ["pre",][form][,"post"] . . . DNM . . . . .   $

DISPLAY SUBPAGE [form] . . . . . . . . . . . DSP . . . . .   $

(form = D,LU,LL,LM,RU,RL,RM)

DO INDEX [title] . . . . . . . . . . . . . . DX  . . . . .  *$

ELLIPSES . . . . . . . . . . . . . . . . . . . . . . . . .   $ ~

ENABLE BAR . . . . . . . . . . . . . . . . . EBB . . . . .    D

ENABLE ESCAPE  . . . . . . . . . . . . . . . ESC . . . . .    D~

ENABLE EQUATION n  . . . . . . . . . . . . . EEQ . . . . .    D~

ENABLE HYPHENATION . . . . . . . . . . . . . EHY . . . . .    D

ENABLE INDEXING  . . . . . . . . . . . . . . EIX . . . . .    D

- 70 -

LIST OF COMMANDS (ALPHABETICAL)

ENABLE LEVELS [+-out],[+-toc]  . . . . . . . . . . . . . .   $ ~

ENABLE TABS  . . . . . . . . . . . . . . . . ETB . . . . .    D~

ENABLE TOC . . . . . . . . . . . . . . . . . ETC . . . . .    D

ENABLE OVERSTRIKING  . . . . . . . . . . . . EOV . . . . .    D

ENABLE SUBSTITUTION  . . . . . . . . . . . . . . . . . . .    D

ENABLE UNDERLINING . . . . . . . . . . . . . EUN . . . . .    D

END BAR  . . . . . . . . . . . . . . . . . . EB

END CENTER . . . . . . . . . . . . . . . . . ECN . . . . .     ~

END FOOTNOTE . . . . . . . . . . . . . . . . EFN

ENDIF aaaa

END LIST [space] . . . . . . . . . . . . . . ELS . . . . .  *$

END LITERAL  . . . . . . . . . . . . . . . . EL  . . . . .  *$

END NOTE . . . . . . . . . . . . . . . . . . EN  . . . . .  *$

END SUBPAGE  . . . . . . . . . . . . . . . . ES  . . . . .  *$

END TEXT . . . . . . . . . . . . . . . . . . ETX . . . . .  *$ ~

FIGURE [space] . . . . . . . . . . . . . . . FG  . . . . .  *$

FIGURE DEFERRED [space]  . . . . . . . . . . FGD . . . . .  *$

FILL . . . . . . . . . . . . . . . . . . . . F . . . . . .    D

FIRST TITLE  . . . . . . . . . . . . . . . . FT  . . . . .   $

FLAGS ACCEPT [new flag]  . . . . . . . . . . FL ACCEPT . .   $D

FLAGS [ALL]  . . . . . . . . . . . . . . . . FL [ALL]  . .   $

FLAGS BREAK [new flag] . . . . . . . . . . . FL BREAK  . .   $

FLAGS CAPITALIZE [new flag]  . . . . . . . . FL CAPITALIZE   $

FLAGS CONTROL [new flag] . . . . . . . . . . FL CONTROL  .   $D

FLAGS ESCAPE [new flag]  . . . . . . . . . . FL ESCAPE . .   $ ~

FLAGS EQUATION . . . . . . . . . . . . . . . FL EQUATION .     ~

FLAGS HYPHENATE [new flag] . . . . . . . . . FL HYPHENATE    $

FLAGS INDEX [new flag] . . . . . . . . . . . FL INDEX  . .   $

FLAGS LOWERCASE [new flag] . . . . . . . . . FL LOWERCASE    $D

FLAGS OVERSTRIKE [new flag]  . . . . . . . . FL OVERSTRIKE   $

FLAGS SUBSTITUTE [new flag]  . . . . . . . . FL SUBSTITUTE   $

FLAGS UNDERLINE [new flag] . . . . . . . . . FL UNDERLINE    $D

FLAGS UPPERCASE [new flag] . . . . . . . . . FL UPPERCASE    $D

FOOTNOTE . . . . . . . . . . . . . . . . . . FN  . . . . .   $

HALF SPACING [n] . . . . . . . . . . . . . . . . . . . . .     ~

HEADER . . . . . . . . . . . . . . . . . . . HD  . . . . .   $D

HEADER LEVEL [+-level] [title] . . . . . . . HL  . . . . .  *$

HEADER LOWER . . . . . . . . . . . . . . . . . . . . . . .   $

HEADER MIXED . . . . . . . . . . . . . . . . . . . . . . .   $D

HEADER NO PAGE . . . . . . . . . . . . . . . . . . . . . .   $ ~

HEADER PAGE  . . . . . . . . . . . . . . . . . . . . . . .   $D~

HEADER SPACING [spacing] . . . . . . . . . . . . . . . . .     ~

HEADER UPPER . . . . . . . . . . . . . . . . . . . . . . .   $

IF aaaa

IF NOT aaaa

IMMEDIATE TEST PAGE n  . . . . . . . . . . . ITP . . . . .   $ ~

IMMEDIATE TEST TEXT n  . . . . . . . . . . . ITT . . . . .   $ ~

INDENT [-n]  . . . . . . . . . . . . . . . . I . . . . . .  *

INDENT LEVELS [-l],[-r]  . . . . . . . . . . . . . . . . .   $ ~

INDEX entry  . . . . . . . . . . . . . . . . X

JUSTIFY  . . . . . . . . . . . . . . . . . . J . . . . . .    D

LAYOUT [code] [,spacing] . . . . . . . . . . LO  . . . . .  *$

(0=all at top 1-7=Header top,number bottom)

- 71 -

LIST OF COMMANDS (ALPHABETICAL)

(0=default 1=C 2=LR,C 3=L,C 4=L,R 5=C,R 6=C,LR 7=RL)

LEFT MARGIN [+-n]  . . . . . . . . . . . . . LM  . . . . .  *

LIST . . . . . . . . . . . . . . . . . . . . LS  . . . . .  *$

LIST ELEMENT text  . . . . . . . . . . . . . LE  . . . . .  *$

LITERAL  . . . . . . . . . . . . . . . . . . LT  . . . . .  *

LOCK . . . . . . . . . . . . . . . . . . . . . . . . . . .   $ ~

LOWER CASE . . . . . . . . . . . . . . . . . LC

NO AUTOHYPHENATE . . . . . . . . . . . . . . NAH . . . . .   $

NO AUTOPARAGRAPH . . . . . . . . . . . . . . NAP . . . . .   $D

NO AUTOSUBTITLE  . . . . . . . . . . . . . . NAST  . . . .   $D

NO AUTOTITLE . . . . . . . . . . . . . . . . NAT . . . . .   $ ~

NO ELLIPSES  . . . . . . . . . . . . . . . . . . . . . . .   $D~

NO FILL  . . . . . . . . . . . . . . . . . . NF  . . . . .  *

NO FLAGS [flag name] . . . . . . . . . . . . NFL . . . . .   $

NO HALF SPACING  . . . . . . . . . . . . . . . . . . . . .    D~

NO JUSTIFY . . . . . . . . . . . . . . . . . NJ

NO PAGING  . . . . . . . . . . . . . . . . . NPA . . . . .   $

NO NUMBER  . . . . . . . . . . . . . . . . . NNM . . . . .   $

NO NUMBER CHAPTER  . . . . . . . . . . . . . . . . . . . .   $ ~

NO SPACE . . . . . . . . . . . . . . . . . . NSP

NO TITLE . . . . . . . . . . . . . . . . . . . . . . . . .   $ ~

NOTE . . . . . . . . . . . . . . . . . . . . N . . . . . .  *$

NUMBER APPENDIX [+-n]  . . . . . . . . . . . NMAX  . . . .   $

NUMBER CHAPTER [+-n] . . . . . . . . . . . . NMCH  . . . .   $D

NUMBER LEVEL [+-n1],....,[+-n6]  . . . . . . NMLV  . . . .   $

NUMBER LIST [+-n]  . . . . . . . . . . . . . NPLS  . . . .   $

NUMBER [PAGE] [+-n]  . . . . . . . . . . . . NMPG  . . . .   $D

NUMBER SUBPAGE [+-n] . . . . . . . . . . . . NMSPG . . . .   $

PAGE . . . . . . . . . . . . . . . . . . . . PG  . . . . .  *$

PAGE SIZE [+-h],[+-w],[+-l],[+-t],[s]  . . . PS  . . . . .   $

PAPER SIZE [+-h],[+-w],[+-l],[+-t],[s] . . . PS  . . . . .   $

PAGING . . . . . . . . . . . . . . . . . . . PA  . . . . .   $

PARAGRAPH [-n],[v],[t] . . . . . . . . . . . P . . . . . .  *$

PERIOD . . . . . . . . . . . . . . . . . . . PR  . . . . .   $D

PERMANENT FOOTNOTE . . . . . . . . . . . . . PFN . . . . .   $ ~

PRINT INDEX  . . . . . . . . . . . . . . . . PX  . . . . .  *$

RESET ESCAPE . . . . . . . . . . . . . . . . . . . . . . .   $ ~

RESET SUBSTITUTE . . . . . . . . . . . . . . . . . . . . .   $ ~

REQUIRE 'file spec'  . . . . . . . . . . . . REQ . . . . .   $

RIGHT [+-indent] . . . . . . . . . . . . . . R . . . . . .   $

RIGHT MARGIN [+-n] . . . . . . . . . . . . . RM  . . . . .  *

SEND TOC text  . . . . . . . . . . . . . . . STC

SET PARAGRAPH [-n],[v],[t] . . . . . . . . . STP . . . . .   $

SKIP [lines] . . . . . . . . . . . . . . . . S . . . . . .  *

SPACING [s]  . . . . . . . . . . . . . . . . SP  . . . . .   $

STYLE CHAPTER [n1],...,[n5]  . . . . . . . . STCH  . . . .   $ ~

(1-3 = spacing, 4,5=indent (-1 centered,-2 right just)

STYLE HEADERS [n1],...,[n9]  . . . . . . . . STHL  . . . .  *$

(line,CAPS,Cap,number,center,skip before,after,test,indent)

SUBPAGE  . . . . . . . . . . . . . . . . . . SPG . . . . .  *$

SUBTITLE [subtitle]  . . . . . . . . . . . . ST  . . . . .   $

TAB LEFT . . . . . . . . . . . . . . . . . . . . . . . . .   $D~

TAB RIGHT  . . . . . . . . . . . . . . . . . . . . . . . .   $ ~

- 72 -

LIST OF COMMANDS (ALPHABETICAL)

TAB STOPS [+-n1,n2...] . . . . . . . . . . . TS  . . . . .   $

TEST PAGE n  . . . . . . . . . . . . . . . . TP  . . . . .  *$

TEST TEXT n  . . . . . . . . . . . . . . . . TT  . . . . .  *$ ~

TEXT . . . . . . . . . . . . . . . . . . . . TX  . . . . .  *$ ~

TITLE [title]  . . . . . . . . . . . . . . . T . . . . . .   $D

TOP MARGIN [+-n] . . . . . . . . . . . . . . TM  . . . . .  *$ ~

UNDERLINE SPACES . . . . . . . . . . . . . . . . . . . . .   $ ~

UNDERLINE NO SPACES  . . . . . . . . . . . . . . . . . . .   $D~

UPPER CASE . . . . . . . . . . . . . . . . . UC  . . . . .    D

VARIANT aaaa . . . . . . . . . . . . . . . . . . . . . . .      ~

- 73 -

APPENDIX G

NON STANDARD COMMANDS

Command                       Standard equivalent

-------                       -------- ----------

ELI                           END LITERAL

HY                            ENABLE HYPHENATION

HYPHENATION                   ENABLE HYPHENATION

LIT                           LITERAL

NFC                           NO FLAGS CAPITALIZE

NFH                           NO FLAGS HYPHENATE

NHY                           DISABLE HYPHENATION

NM                            NUMBER PAGE

NO HYPHENATION                DISABLE HYPHENATION

SK                            SKIP

SUBTTL                        SUBTITLE

In  addition to the non standard commands and the commands marked as not

available in DSR some commands have additional functions  not  available

in  DSR.   The  permanent  margins are not available in DSR and the TOP,

LEFT margins parameters for .PAGE SIZE will not be recognized when tran-

sporting  text  to  DSR.   The  layout  codes  4-7  are not in DSR.  The

pre/post headers in the .DISPLAY NUMBER, .DISPLAY CHAPTER, and, .DISPLAY

APPENDIX  are  not available in DSR.  The indentation for the .NOTE com-

mand is slightly different from DSR.  The parameter for the .CENTER com-

mand  is  non standard.  Other than these exceptions this version of RNO

is a superset of other versions of RNO and a subset of DSR.

- 74 -

APPENDIX H

SPECIAL PRINTER SUPPORT

A  number of features have been added to support special printers.  Some

of these require that the printer respond to specific special  code  se-

quences or escape sequences.  Three general commands are involved.

1.  .HALF  SPACING requires an escape sequence to move the paper up

by 1/2 line.  The escape sequence must be:

ESC U

2.  .FLAGS  EQUATION needs the escape sequence ESC U for moving the

paper up.  In addition it need the ability to backspace via the

BS  code  and  the escape sequence ESC D to move the paper down

1/2 line.

3.  .FLAGS ESCAPE may be used to enable a variety of special escape

sequences.  The following escape sequences permanently  defined

when  RNO  is  built  by  enabling the line defining the symbol

$FLORD  found  in  RNPRE.for  Florida   data   printers.    For

spinwriter  model 5515/25 and various Diablo models, enable the

symbol $DIAB.  The following escape sequences will then be ava-

ilable.

(A)  ^>  Causes  the text to be shifted up 1/2 space for writing

superscripts, or cancelling subscripts.

(B)  ^<  Shifts the text down 1/2 space for writing subsripts or

cancelling superscripts This escape sequence conflicts with

the CAPITALIZE FLAG if the flag is enabled.  If the flag is

enabled, it  takes  precedence  over  the  escape  sequence

definition.

(C)  ^( Causes the text to shift up 1 whole line.

(D)  ^) Shifts text down 1 whole line.

FONT SELECTION for FLORIDA DATA only.

(E)  ^n  Where  n is a number between 1 and 9 selects a font.  3

is currently a 10 pitch ROMAN font and 8 will be GREEK  and

special symbols.

SHADOW PRINT for FLORIDA DATA only

(F)  ^* Begins shadow printing for boldface text.  This normally

ends at the end of a line or  until  turned  off  whichever

comes first.

(G)  \*  End  shadow  print If .FLAGS SPECIAL * is used then you

may boldface 1 character by preceeding it with *.

ELONGATED PRINT for FLORIDA DATA only

(H)  ^= Begin elongated double width printing.

(I)  \= End elongated print.  If FLAGS SPECIAL = is used, then a

single occurrence of = will  elongate  only  the  character

following it.

GRAPHICS MODE for DIABLO/SPINWRITER only

(J)  ^!  Begin graphics mode

(K)  \!  End graphics mode

The  following  are the .DEFINE ESCAPE statements equivalent to

the permanently defined sequences.

- 75 -

SPECIAL PRINTER SUPPORT

(A)  .DEFINE ESCAPE "^" VSP 1, 33, "U"

(B)  .DEFINE ESCAPE "^>" VSP -1, 33, "D"

(C)  .DEFINE ESCAPE "^(" VSP -2, 33, 13

(D)  .DEFINE ESCAPE "^)" VSP 2, 13

(E)  .DEFINE ESCAPE "^1", 33, 2

(F)  .DEFINE ESCAPE "^2", 33, 3

(G)  .DEFINE ESCAPE "^3", 33, 4

(H)  .DEFINE ESCAPE "^4", 33, 5

(I)  .DEFINE ESCAPE "^5", 33, 6

(J)  .DEFINE ESCAPE "^8", 33, 9

(K)  .DEFINE ESCAPE "^*", LCK, CHR, 33, "W"

(L)  .DEFINE ESCAPE "\*", LCK, CHR, 33, "&"

(M)  .DEFINE ESCAPE "^=", LCK, CHR, HSP, 2, PSP, 16

(N)  .DEFINE ESCAPE "\=", LCK, CHR, HSP, 1, PSP, 17

(O)  .DEFINE ESCAPE "^!", LCK, 33, "3"

(P)  .DEFINE ESCAPE "\!", LCK, 33, "4"

- 76 -

APPENDIX I

INDEX

(RUNOFF commands appear in upper case.)

Accept character  . . . . . . . . . . . . . . . . . . . . . . . . . .  8

APPENDIX  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27

AUTOHYPHENATE   . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

AUTOPARAGRAPH   . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

AUTOSUBTITLE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

AUTOTITLE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

BEGIN BAR   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

BLANK   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

BREAK   . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  6, 16

Case  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

Case information  . . . . . . . . . . . . . . . . . . . . . . . . . .  7

CENTER LINE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

CENTER TEXT   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

CENTRE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

CHAPTER   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

CHAPTER LAYOUT  . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

COMMAND FORMAT  . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

Command string  . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

COMMANDS, FORMATTING  . . . . . . . . . . . . . . . . . . . . . . . . 16

COMMANDS, MODE SETTING  . . . . . . . . . . . . . . . . . . . . . . . 33

COMMANDS, PAGE FORMATTING   . . . . . . . . . . . . . . . . . . . . . 20

COMMENT   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 52

Define / Reset  . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

DEFINE ESCAPE   . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

DEFINE SUBSTITUTE   . . . . . . . . . . . . . . . . . . . . . . . . . 50

DISABLE BAR   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

DISABLE EQUATION  . . . . . . . . . . . . . . . . . . . . . . . . . . 46

DISABLE ESCAPE  . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

DISABLE HYPHENATION   . . . . . . . . . . . . . . . . . . . . . . . . 47

DISABLE OVERSTRIKING  . . . . . . . . . . . . . . . . . . . . . . . . 47

DISABLE SUBSTITUTION  . . . . . . . . . . . . . . . . . . . . . . . . 47

DISABLE TABS  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47

DISABLE TOC   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

DISABLE UNDERLINING   . . . . . . . . . . . . . . . . . . . . . . . . 48

DISPLAY APPENDIX  . . . . . . . . . . . . . . . . . . . . . . . . . . 32

DISPLAY CHAPTER   . . . . . . . . . . . . . . . . . . . . . . . . . . 32

DISPLAY ELEMENTS  . . . . . . . . . . . . . . . . . . . . . . . . . . 32

DISPLAY LEVELS  . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

DISPLAY NUMBER  . . . . . . . . . . . . . . . . . . . . . . . . . . . 31

DISPLAY SUBPAGE   . . . . . . . . . . . . . . . . . . . . . . . . . . 32

DO INDEX  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 52

- 77 -

INDEX

ELLIPSES  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

Enable / Disable  . . . . . . . . . . . . . . . . . . . . . . . . . . 46

ENABLE BAR  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

ENABLE EQUATION   . . . . . . . . . . . . . . . . . . . . . . . . . . 46

ENABLE ESCAPE   . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

ENABLE HYPHENATION  . . . . . . . . . . . . . . . . . . . . . . . . . 46

ENABLE INDEXING   . . . . . . . . . . . . . . . . . . . . . . . . . . 47

ENABLE LEVELS   . . . . . . . . . . . . . . . . . . . . . . . . . . . 29

ENABLE OVERSTRIKING   . . . . . . . . . . . . . . . . . . . . . . . . 47

ENABLE SUBSTITUTION   . . . . . . . . . . . . . . . . . . . . . . . . 47

ENABLE TABS   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47

ENABLE TOC  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

ENABLE UNDERLINING  . . . . . . . . . . . . . . . . . . . . . . . . . 48

END BAR   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

END CENTER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

END FOOTNOTE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

END LIST  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

END LITERAL   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

END NOTE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

END SUBPAGE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

END TEXT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

END TOC   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 54

ENDIF   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 56

ERROR MESSAGES  . . . . . . . . . . . . . . . . . . . . . . . . . . . 60

FIGURE DEFERRED   . . . . . . . . . . . . . . . . . . . . . . . . . . 16

FIGURE n  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

File Specifications   . . . . . . . . . . . . . . . . . . . . . . . .  3

FILL  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 34, 59

Filling of text   . . . . . . . . . . . . . . . . . . . . . . . . . .  6

FIRST TITLE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

Flags   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

FLAGS ACCEPT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

FLAGS ALL   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

FLAGS BREAK   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

FLAGS CAPITALIZE  . . . . . . . . . . . . . . . . . . . . . . . . . . 42

FLAGS CONTROL   . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

FLAGS EQUATION  . . . . . . . . . . . . . . . . . . . . . . . . . . . 43

FLAGS ESCAPE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 43

FLAGS HYPHENATE   . . . . . . . . . . . . . . . . . . . . . . . . . . 43

FLAGS OVERSTRIKE  . . . . . . . . . . . . . . . . . . . . . . . . . . 43

FLAGS SPACE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 44

FLAGS SPECIAL   . . . . . . . . . . . . . . . . . . . . . . . . . . . 44

FLAGS SUBSTITUTE  . . . . . . . . . . . . . . . . . . . . . . . . . . 44

FLAGS UNDERLINE   . . . . . . . . . . . . . . . . . . . . . . . . . . 45

FLAGS UPPERCASE   . . . . . . . . . . . . . . . . . . . . . . . . . . 45

FOOTNOTE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

Form-feeds  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

FORMATTING  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

HALF SPACING  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

HEADER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

HEADER LEVEL  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 28

HEADER LOWER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

HEADER MIXED  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

HEADER NO PAGE  . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

HEADER PAGE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

HEADER SPACING  . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

- 78 -

INDEX

HEADER UPPER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

Hyphenation   . . . . . . . . . . . . . . . . . . . . . .  5, 36, 43, 46

IF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 56

IF NOT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 56

IMMEDIATE TEST PAGE   . . . . . . . . . . . . . . . . . . . . . . . . 21

IMMEDIATE TEST TEXT   . . . . . . . . . . . . . . . . . . . . . . . . 21

INDENT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

INDENT LEVELS   . . . . . . . . . . . . . . . . . . . . . . . . . . . 29

INDEX   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 52

Justification of text   . . . . . . . . . . . . . . . . . . . . . . .  6

JUSTIFY   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33, 59

LAYOUT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

LEFT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

LEFT MARGIN   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

LIST  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

LIST ELEMENT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

LITERAL   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

Literals  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  6

LOCK  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 39

LOWER CASE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

MODE SETTING  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

NO AUTOHYPHENATE  . . . . . . . . . . . . . . . . . . . . . . . . . . 36

NO AUTOPARAGRAPH  . . . . . . . . . . . . . . . . . . . . . . . . . . 36

NO AUTOSUBTITLE   . . . . . . . . . . . . . . . . . . . . . . . . . . 36

NO AUTOTITLE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

NO ELLIPSES   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

NO FILL   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 34

NO FLAGS ALL  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

NO FLAGS BREAK  . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

NO FLAGS CAPITALIZE   . . . . . . . . . . . . . . . . . . . . . . . . 42

NO FLAGS EQUATION   . . . . . . . . . . . . . . . . . . . . . . . . . 43

NO FLAGS ESCAPE   . . . . . . . . . . . . . . . . . . . . . . . . . . 43

NO FLAGS HYPHENATE  . . . . . . . . . . . . . . . . . . . . . . . . . 43

NO FLAGS SPECIAL  . . . . . . . . . . . . . . . . . . . . . . . . . . 44

NO FLAGS SUBSTITUTE   . . . . . . . . . . . . . . . . . . . . . . . . 45

NO FLAGS UNDERLINE  . . . . . . . . . . . . . . . . . . . . . . . . . 45

NO HALF SPACING   . . . . . . . . . . . . . . . . . . . . . . . . . . 38

NO HEADER   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

NO JUSTIFY  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

NO NUMBER CHAPTER   . . . . . . . . . . . . . . . . . . . . . . . . . 27

NO PERIOD   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

NO SPACE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 34

NO TITLE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

NONUMBER  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

NOTE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

NUMBER APPENDIX   . . . . . . . . . . . . . . . . . . . . . . . . . . 27

NUMBER CHAPTER  . . . . . . . . . . . . . . . . . . . . . . . . . . . 27

NUMBER LEVEL  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 29

NUMBER LIST   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

NUMBER PAGE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

NUMBER SUBPAGE  . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

- 79 -

INDEX

Operation   . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

PAGE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

PAGE FORMATTING   . . . . . . . . . . . . . . . . . . . . . . . . . . 20

Page Numbering  . . . . . . . . . . . . . . . . . . . . . . . . . . .  6

PAGE SIZE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

PAPER SIZE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

PARAGRAPH   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

Parameter setting   . . . . . . . . . . . . . . . . . . . . . . . . . 37

PERIOD  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

PERMANENT FOOTNOTE  . . . . . . . . . . . . . . . . . . . . . . . . . 19

PRINT INDEX   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 53

Quote character   . . . . . . . . . . . . . . . . . . . . . . . . . .  8

Quoted Space  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  8

REQUIRE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 51

RESET SUBSTITUTE  . . . . . . . . . . . . . . . . . . . . . . . . . . 50

RIGHT   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

RIGHT MARGIN  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

Right shift   . . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

RUNOFF COMMANDS   . . . . . . . . . . . . . . . . . . . . . . . . . . 15

SEND TOC  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 53

SET PARAGRAPH   . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

SKIP  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

Source File Format  . . . . . . . . . . . . . . . . . . . . . . . . .  6

Space   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6, 8

SPACING   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

SPECIAL CHARACTERS  . . . . . . . . . . . . . . . . . . . . . . .  8, 69

Spooling  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  5

STANDARD  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

STYLE CHAPTER   . . . . . . . . . . . . . . . . . . . . . . . . . . . 28

STYLE HEADERS   . . . . . . . . . . . . . . . . . . . . . . . . . . . 29

SUBPAGE   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

Substitute character  . . . . . . . . . . . . . . . . . . . . . . . .  9

SUBTITLE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

Switches  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

TAB LEFT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

TAB RIGHT   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

TAB STOPS   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 39

Table of contents   . . . . . . . . . . . . . . . . . . . . . . . . . 53

TEST PAGE n   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

TEST TEXT   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

TEXT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

Title   . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  6, 22

TOC   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 54

TOP MARGIN  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

UNDERLINE NO SPACES   . . . . . . . . . . . . . . . . . . . . . . . . 35

UNDERLINE SPACES  . . . . . . . . . . . . . . . . . . . . . . . . . . 35

Underlining   . . . . . . . . . . . . . . . . . . . . . . . . . . . 5, 8

UPPER CASE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 34

VARIANT   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 57

- 80 -

INDEX

Word spacing  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  6

- 81 -