UVT100 User's Guide

AUTHOR:  GLEN HOFFING

DATE:    Feb.  24, 1981

The UVT100 subroutine interface is designed to allow convenient  access  to  the

VT100  ANSI  Mode  Control  Sequences as defined in chapter 3 of the VT100 Users

Guide.  The calling sequence for each function is described below, together with

a description of the function.

In order for  cursor  control  of  the  VT100  to  perform  satisfactorily,  the

wraparound  feature  of  the  terminal  driver should be disabled by issuing the

MCR command "SET /NOWRAP=TI:"

The VT100 must have its ANSI/VT52 setup toggle switch  set  to  ANSI  for  these

commands  to  work.   This  can  be done manually, or a CALL UVT100(ANSI) can be

issued to ensure the terminal is set to ANSI.

Programs using UVT100 to write to a terminal must open the logical unit  of  the

terminal  with  CARRIAGECONTROL='NONE'  to  disable  Fortran  carriage  control.

Standard  Fortran  CR/LF  carriage  control  can  be  emulated  by   issuing   a

CALL UVT100(NEL) between each Fortran write.

The format of a UVT100 call is CALL UVT100(cmd,num1,num2), where:     cmd is the

mode  control  command  to  be executed,    num1 and num2 are numeric parameters

required for certain commands.

Symbolics are defined for the cmd  parameter  in  the  file  UVT100.DAT.   These

symbolics  match  those  provided in the VT100 Users Guide, and are provided for

ease of operation.

The following commands are implemented.

CALL UVT100(CUP,line,column)    position cursor to coordinates given by line

and column. lines are numbered 1-24 and

columns 1-80. a zero is treated as a one.

CUP,0,0 is the equivalent of a HOME CURSOR.

CALL UVT100(CUB,num)            backspace the cursor num columns. a zero is

treated as a one. cursor will not backspace

beyond left margin.

CALL UVT100(CUD,num)            move the cursor down num lines. a zero is

treated as a one. cursor will not move below

bottom margin.

CALL UVT100(CUF,num)            forward the cursor num columns. a zero is

treated as a one. cursor will not forward

beyond right margin.

CALL UVT100(CUU,num)            move the cursor up num lines. a zero is

treated as a one. cursor will not move up

beyond top margin.

CALL UVT100(DECDHL,num)         define active line as top or bottom of a

double-height line. num=0 is the top half,

and num=1 is the bottom half.

Page 2

UVT100 User's Guide

CALL UVT100(DECDWL)             define active line as a double width line.

CALL UVT100(DECSWL)             define active line as a single-width,

single-height line.

CALL UVT100(DECSC)              save the active cursor coordinates.

CALL UVT100(DECRC)              restore the cursor to the saved coordinates.

CALL UVT100(ED,num)             erase the display. if num=0, erase from active

cursor position to end of display. if num=1,

erase from top of display to active cursor

position. if num=2, erase entire display.

CALL UVT100(EL,num)             erase the line. if num=0, erase from active

cursor position to the end of line. if num=1,

erase from the beginning of line to the active

cursor position. if num=2, erase entire line.

CALL UVT100(SGR,num)            set graphic rendition. the only attribute

selectable for a VT100 without the advanced

video option is reverse image, num=7.

num=0 turns all attributes off.

CALL UVT100(NEL)                position cursor to beginning of next line.

CALL UVT100(SCS,num1,num2)      select character set. num1=0 indicates G0 set.

num1=1 indicates G1 set. num2=0 indicates

United Kingdom set. num2=1 indicates standard

ASCII set. num2=2 indicates special graphics

set. in subsequent I/O, the G0 set is selected

by a shift in <control O>, and the G1 set is

selected by a shift out <control N> embedded

in the data.

CALL UVT100(SM,num)             set DEC private mode. num is mode to be set.

1 = DECCKM (cursor key)

2 = DECANM (ANSI/VT52)

3 = DECCOLM (column)

4 = DECSCLM (scrolling)

5 = DECSCNM (screen)

6 = DECOM (origin)

7 = DECAWM (auto wrap)

8 = DECARM (auto repeating)

9 = DECINLM (interlace)

CALL UVT100(RM,num)             reset DEC private mode. num is the mode to be

reset. see codes listed above for set mode.

CALL UVT100(ANSI)               this is a VT52-compatible command to ensure

the terminal is set to ANSI mode. this should

be the first command issued in your program

if you are not sure the target terminal is set

to ANSI mode. this command will be ignored if

the terminal is already in ANSI mode.