Transmission of documents on Magnetic tape with FLX

B. Z. Lederman  I.T.T. World Communications

When preparing documents for trasmission by magnetic  tape  in  DOS  format

(such  as  with FLX) it is important that the file have the "implicit" or "list"

carriage control attribute;  that is, the type of file normally produced by  the

various  editors  on  RSX  systems  where  a  carriage-return  line-feed pair is

"implied" to exist between  records,  but  where  there  are  normally  no  such

characters  actually in the file.  The reason for this is that FLX converts such

files to files with the carriage-return / line-feed  present  when  placing  the

file  on  tape,  and  does the reverse when restoring the file to disk.  This is

done only when the file is determined to be "formatted ASCII", which is  a  file

which  does  not have an extension of ".TSK", ".OBJ", ".SAV", or other extension

which would flag the use of other than "formatted ASCII" file type.  If the file

is  of the type with "no" carriage control (which is a file which has all of the

carriage-return and line-feed characters within the file) it may be garbled when

transmitted  by  tape  unless  an  image transfer is done every time the file is

placed on or removed from the tape.  During processes such as the Symposium tape

collection,  not  every file can receive this individualized treatment, and when

transmitting other tapes the receiver may  not  know  that  an  individual  file

requires  special treatment, so it is better to convert the file to a form which

will be correctly treated by default.  Simalarly, FLX can handle files with  the

"Fortran"  carriage  control  attribute  (where  the  first  byte in each record

determines the carriage control for that record), but will only do so  correctly

if  the  appropriate  switch  is  used on every transfer.  Note that this switch

would apply only to files created by a Fortran  task,  not  the  Fortran  source

created by the editor.

The greatest source of trouble is the output of RUNOFF, as the ".DOC" files

have  internal  carriage  control,  and there are other such files as well (task

builder maps and such).  The carriage control type for any  particular  file  is

most easily determined with the RMS DSP utility, which is supplied with RSX (you

need not have RMS-11K, plain RMS will suffice).   When  invoked,  the  5th  line

displayed  is  "RECORD  ATTRIBUTES:":  if this is followed by "CARRIAGE RETURN",

the file has the desired carriage control type.  If the attribute is blank, then

the  file  has  internal  carriage  control,  and "FORTRAN" is of course Fortran

carriage control.  The easiest way to get around the problem is to use  TECO  to

convert  the file to one with "implied" or "list" carriage control.  It does not

matter if you have never used TECO before, and do  not  expect  to  use  it  for

anything  else.   I  have  never  been  able  to do any normal editing with TECO

regardles of which manual I read, but I  can  convert  from  one  file  type  to

another and have it work every time.  The clue was in one of the RUNOFF manuals,

and is very simple.

First invoke TECO by the appropriate command for your system, which may be:

TEC             (if installed under the normal name TEC)

RUN $TEC or RUN $TECO (if not installed)

EDIT/TECO       (if installed and you use DCL)

or such command as is appropriate.  You should receive an asetrisk  prompt  "*".

There  is  a possibility that a video-teco is installed in your system, in which

case you may receive a screen display showing the keypad layout:  ignore it  and

enter two "control-Z"s and you should receive the astrisk prompt.  When you have

Transmission of documents on Magnetic tape with FLX                       PAGE 2

B. Z. Lederman  I.T.T. World Communications

the asterisk, enter the single command:

*ERinfile/-CR$EWoutfile/CR$EX$$

where the "*" is the prompt on the screen, ER means read this  file,  infile  is

the  file  which  is going to be converted (a normal RSX file specification with

name and extension), /-CR means the file has no  carriage  control,  the  dollar

sign  is  an  ESCAPE character created by pressing the ESCAPE or ESC key on your

terminal  which  will  echo  as  "$",  EW  means  write  this  file,  the   file

specification  to  which  the converted file is written (again a normal RSX file

spec.), /CR means use implied carriage control, another escape, EX  means  write

all  of  the infile to the outfile and leave, and two escapes, which signify the

end of the command screen.  TECO will now go to work.  You  may  see  a  message

such as

[14K bytes]

[15K bytes]

[16K bytes]

and so on, which is an indication of how much memory TECO  is  using.   It  will

eventually  exit,  and  send  you  back  to the MCR prompt level ">" (or DCL, or

whatever).

In most cases, the converted file is ready to go, but there are still a few

precautions.   One is that underlining may not come through properly, especially

if you use the "/UL:L" switch in RUNOFF, as this generates a very  long  record.

I  reccomend  you  try  to  get  along  without underlining if the file is to be

distributed widely, but if you must have it, the "/UL:B"  attribute  is  better,

and  the simulated underlining "/UL:S" is probably safer still.  If you have any

tables or illustrations, they should be checked, and if neccessary, "touched up"

with an editor before placing the file on tape.

If you have received a file which is already garbled, all is not lost.  You

may  be able to retrieve it with TECO.  If some of the lines are garbled, or you

cannot edit the file because the records are too long, look at the file with DMP

or the RMS DSP utility.  If you have all 512 byte records, the file was probably

transferred incorrectly.  You may be able to recover by using the following  two

commands.   First,  write  it  back  to  a file with internal carrage control by

reversing the command given before:

*ERinfile/CR$EWtemporaryfile/-CR$EX$$

and then try to put it back with the original sequence

*ERtemporaryfile/-CR$EWoutfile/CR$EX$$

You might now have a file which is mostly  correct,  but  there  are  still  two

problems  you  may encounter.  First, the last record may be very long, too long

to be accepted by editors, BASIC, etc.   which  will  give  a  message  such  as

"illegal  record  length",  or  "record  too  long".  This is usually due to the

remains of a 512 byte tape block being appended to the  last  record  as  nulls.

The  solution  (if  you have a recent version of RSX) is to find the last record

with DMP:  first list the file directory with PIP to find the last  block,  then

Transmission of documents on Magnetic tape with FLX                       PAGE 3

B. Z. Lederman  I.T.T. World Communications

examine that block with:

DMP TI:=filespec/AS/BL:block:block

to see the last block in ASCII.  You should see text seperated  by  one  or  two

strange  bytes  (these  are  the record length indicators).  If the block is all

nulls ("@") move back through the file one block at a time until  you  find  the

last  block  with  data.  Once you have found the last good byte, set the end of

the file to the next byte using the PIP /EOF switch:

PIP filespec/EOF:block:byte

remembering that in MCR decimal numbers must be followed by a  ".".   This  will

terminate  the  file  at  the proper location, so it can be "digested" normally.

The next problem is that a few lines may be truncated and continued on the  next

line:  I believe that this happens where a line crossed a block boundry on tape.

The easiest way around this is to use the EDT editor in "change" or keypad mode.

When  a truncated line is found, simply press the "EOL" key to put the cursor at

the end of that line, and then "DELCHR" to remove the carriage-return (which  is

not  visible),  and  the remainder of the line will move up from the line below.

Doubtless other editors could be used also, but I happen to like EDT, and it  is

supposed to be standard accross operating systems.

This last technique is also useful in transferring files from RT-11 to RSX.

I  have read a number of tapes with BASIC and other programs which were on RT-11

tapes by mounting them as ANSI tapes (the headers are 512 bytes vice 80, but RSX

will  usually  take  them  anyway),  PIPing the files over, then using the above

technique to correct any garbled lines.