Contents ( and functionality ) of CPMGENERI.ASM

This is a "first" attempt, to construct a "generic" version of KERMIT.

It is in NO WAY finished yet! "Generic" KERMIT uses only CP/M calls.

This brings one disadvantage and some advantages:

1. It should ( after more modifications ) run on "most" CP/M systems

without major modifications.

2. It only will run at speeds up to (but NOT exceeding) 1200 Baud on

the Rainbow; perhaps faster on other systems (it is known to run at

4800 buad on the VT180).

How did it get done:

The "standard" CP/M system allows limited assignment of "logical"

devices to physical devices. This is handled by the I/O byte. The I/O

byte allows one to "assign" 4 "physical" devices to 4 "logical" devices.

This allows one to use the CP/M call CONSTAT ( is there a Input at the

"logical" CONSOLE-device ? ) to be used for any of the "other" four

assignable devices for CONSOL.  One of these devices is BAT:, the

BATCH device ( Input comes from "logical" device READER, Output goes

to "logical" device LIST ).

We use in GENERIC Kermit during CONNECT-state (terminal-emulation)

and SEND/RECEIVE state the CONSOLE=BAT: assignment, to be able to "poll"

for pending character-input from the physical device READER=RDR: , which

is for the DEC-VT180 (aka ROBIN) and the DEC-RAINBOW the COMM-Port.

To circumvent BDOS-implementation differences, we call the BIOS

directly. This is accomplished in routine INIADR, which grabs location

1 and 2 ( at 0 is a JMP WARMSTART ), adds increments of 3 ( we got the

second entry of the BIOS-JMP vector -- the next three entries are JMP CONSTAT,

JMP CONIN and JMP CONOUT ) and plugs them into our BIOS-CALL routines.

-	Bernie Eiben