KERMIT USER GUIDE

Fifth Edition, Revision 1

Frank da Cruz, editor

Columbia University Center for Computing Activities

New York, New York 10027

27 July 1984

Copyright (C) 1981,1982,1983,1984

Trustees of Columbia University in the City of New York

Permission is granted to any individual or institution to copy or

use this document and the programs described in it, except for

explicitly commercial purposes.

Preface to the 5th Edition                                        Page 1

Preface to the 5th Edition (March 1984)

Since  the  4th  Edition  of the KERMIT Users Guide was produced in July

1983, the KERMITs have been flying thicker and faster than anyone  could

keep  up with.  Old versions have improved, and implementations for many

new systems have appeared.  It is no longer practical to even attempt to

cover all the implementations in  a  single  manual.    Therefore,  this

manual  will try to describe a sort of "ideal" KERMIT program, one which

has most of the features specified in the KERMIT Protocol Manual.   Most

real  KERMIT  programs will fall short of this description in some ways.

After the main, system-independent part of the manual there are sections

for several particular KERMIT programs,  emphasizing  their  differences

from  the ideal, at the time of this writing.  The system-dependent por-

tions of this manual will  rapidly  become  dated;  current  information

about  any  particular  KERMIT  program can be found in the accompanying

on-line help or documentation files, or built-in internal help text.

5TH EDITION, REVISION 1 (July 1984)

The major sections of the manual are relatively unchanged.  The chapters

describing DECSYSTEM-20, MS-DOS, CP/M-86, CP/M-80, and Apple DOS Kermits

have been updated to reflect new releases since last March.   Meanwhile,

a 2-part article  describing the  Kermit protocol was  published in  the

June and July 1984 issues of BYTE Magazine, which is recommended reading

for anyone who wants to know the  reasons why a protocol like Kermit  is

necessary.

HISTORY AND ACKNOWLEDGEMENTS

The KERMIT file transfer protocol was designed at the  Columbia  Univer-

sity  Center  for Computing Activities (CUCCA) in 1981-82 mainly by Bill

Catchings and Frank da Cruz.  Bill wrote the first two programs, one for

the DECSYSTEM-20 and one for a CP/M-80 microcomputer.

The initial objective was to allow users of our DEC-20 and IBM timeshar-

ing systems to archive their files on microcomputer floppy disks.    The

design  owes  much  to  the ANSI and ISO models, and ideas were borrowed

from similar projects at Stanford University and the University of Utah.

The protocol was designed to accommodate the "sensitive"  communications

front  end of the full-duplex DEC-20 system as well as the peculiarities

of half-duplex IBM mainframe communications.  The protocol was soon  im-

plemented  successfully  on  our IBM 4341 systems under VM/CMS by Daphne

Tzoar of CUCCA.

Meanwhile it was becoming apparent that KERMIT was useful for more  than

just file archiving; IBM PCs were beginning to appear in the offices and

departments,  and there arose a general need for file transfer among all

our systems.  Daphne soon had prepared an IBM PC implementation.

After our initial success with KERMIT, we presented it at conferences of

user groups like DECUS and SHARE, and we began to get  requests  for  it

from  other  sites.    Since  we  had  written down a description of the

protocol, some sites wrote their own implementations for new  computers,

or  adapted one of our implementations to run on additional systems, and

sent back these new versions to us so that  we  could  share  them  with

Preface to the 5th Edition                                        Page 2

others.    In  this  way, KERMIT has grown to support about 50 different

systems; it has been sent on magnetic tape from Columbia to hundreds  of

sites in dozens of countries, and has reached hundreds or thousands more

through various user groups and networks.

To  date,  contributions  to  the  KERMIT  effort  have been made by in-

dividuals at the following institutions: Stevens Institute  of  Technol-

ogy,  Cornell  University,  Rutgers  University,  Cerritos  College, the

University of Toronto, the University of  Tennessee  at  Knoxville,  the

University  of  California  at  Berkeley,  the University of Toledo, the

University of Texas at  Austin,  the  University  of  Michigan,  Oakland

University,  the University of Wisconsin, University College Dublin, the

University of Washington, ABC-Klubben Stockholm, the Helsinki University

of Technology, the US National Institutes of Health,  Digital  Equipment

Corporation,  The  SOURCE  Telecomputing,  Hewlett-Packard Laboratories,

Litton Data Systems, RCA Laboratories, Atari Computer, the University of

Chicago, SPSS Inc, and others.   The list grows constantly.

The  Kermit  protocol  was  named  after  Kermit  the Frog, start of the

television series THE MUPPET SHOW, and is used by permission  of  Henson

Associates, Inc.

CUSTOMIZING THIS MANUAL

Although  this  manual was produced at Columbia University, all attempts

have been made to keep it free of site-specific information.    However,

due  to the large number of KERMIT implementations, descriptions of each

one would make the manual unnecessarily thick.  Therefore, the manual is

sent from Columbia with specific documentation about a selection of sys-

tems.  Some of these descriptions may not be of interest at  your  site,

while others that are may be lacking.

Each  site,  upon  receiving a KERMIT tape, may decide which versions of

KERMIT are important to it, and include the appropriate documentation in

this manual.  This is most conveniently done if your site has the Scribe

text formatting system (from UNILOGIC Ltd in Pittsburgh PA,  USA),  with

which  this  manual was produced.  Scribe runs on a wide variety of sys-

tems.  There are also Scribe subsets, such as Perfect Writer  and  Final

Word, that run on various microcomputers.

The  system-specific  parts  of  the KERMIT User Guide are included with

"@INCLUDE" statements at the end of the  Scribe  source  file  for  this

manual,  whose  filename  is  USER.MSS.   You may add or delete @INCLUDE

statements to suit your needs, and run the result through the text  for-

matter to produce a customized manual.

Not all system-specific documentation is provided in .MSS (Scribe input)

format,  since  some  KERMIT  contributors  do  not have Scribe at their

sites.  In that case, you will either have to add Scribe formatting com-

mands, or else enclose the whole subfile in @VERBATIM brackets.

If you do not have SCRIBE, you may still use an editor to delete or  add

sections to the finished documentation file, though the results will not

be  as  satisfactory  --  the table of contents, index, and page numbers

will not be automatically adjusted.

Preface to the 5th Edition                                        Page 3

If  you are running a version of KERMIT for which adequate documentation

has not been provided (after all, this is a distributed,  volunteer  ef-

fort!),  please feel free to write some, preferably in Scribe input for-

mat, and send it back to Columbia so that others may  benefit  from  it.

Likewise if you produce a new implementation of KERMIT.

Ordering Information                                              Page 4

Ordering Information

The  KERMIT software is free and available to all.  Columbia University,

however, cannot afford to distribute free software on the scale required

for KERMIT.  Therefore, to defray our costs for  media,  printing,  pos-

tage,  materials,  labor,  and  computing  resources,  we must request a

moderate distribution fee from sites that request KERMIT  directly  from

Columbia.  The schedule is as follows:

Complete KERMIT Distribution            $100.00

(Tape, Users Guide, and Protocol Manual)

Printed Documents                       $5.00 each

(Users Guide, Protocol Manual, or Any Source Listing)

Other  sites are free to redistribute KERMIT on their own terms, and are

encouraged to do so, with the following stipulations: KERMIT should  not

be  sold  for  profit;  credit  should be given where it is due; and new

material should be sent back to Columbia University at the address below

so that we can maintain a definitive and comprehensive set of KERMIT im-

plementations for further distribution.

To order KERMIT from Columbia University, send a letter  requesting  ei-

ther:

(a)  The manuals or source listings you desire (specify each one), or

(b)  A 9-track magnetic tape in one of the following formats:

System            Tape Format                     Densities

TOPS-10           BACKUP/Interchange, Unlabeled   1600

TOPS-20           DUMPER, Unlabeled               1600

IBM VM/CMS        EBCDIC, CMS Format              1600, 6250

or EBCDIC, OS Standard Label 1600, 6250

UNIX              TAR                             1600

Other             ASCII, ANSI Label, Format ``D'' 1600

(Specify  system,  format,  and density.)  The "Kermit collection"

has grown so large that we can no longer fit it on a 2400' reel of

magnetic tape at 800bpi.  One copy of each manual will be included

with the tape.  We will supply the tape, packaging, and postage.

We can only make tapes in the formats listed above.  We  cannot  produce

floppy  disks;  bootstrapping  procedures  are  provided  to  allow  the

microcomputer versions to be downloaded from the mainframe for which the

tape is produced.  The tape includes all source programs, documentation,

and, when practical,  binaries  or  hex.    Unfortunately,  our  limited

resources  to  not  allow  us  to  provide  automatic  updates to KERMIT

recipients when new implementations, documentation, or bug fixes appear.

Send your letter to:

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

Ordering Information                                              Page 5

New York, N.Y.  10025

Please  list the machines and operating systems you expect to run KERMIT

on, specify the tape format or the listings desired, and mention whether

there are additional systems for which you  require  KERMIT  or  if  you

might be interested in attempting your own implementation for a new sys-

tem.    Make  checks payable to Columbia University Center for Computing

Activities.

KERMIT is available to users of the BITNET network via a server at  host

CUVMA.    BITNET users may type ``SMSG RSCS MSG CUVMA KERMSRV HELP'' for

further information.  KERMIT is also available to users of ARPANET,  via

anonymous  FTP from host COLUMBIA-20, in the area PS:<KERMIT>.  And KER-

MIT is distributed regularly by various computer  user  groups  such  as

DECUS and SHARE.

Since  new KERMIT programs are added -- and old ones improved -- so fre-

quently, sites that use KERMIT heavily are encouraged to contact  Colum-

bia two or three times a year for news.

No  warranty  of  the  software nor of the accuracy of the documentation

surrounding it is expressed or implied,  and  neither  the  authors  nor

Columbia  University acknowledge any liability resulting from program or

documentation errors.

Introduction                                                      Page 6

1. Introduction

Everyone  wants to get computers talking to one another.  There are many

ways to do this, and most of them are very expensive.  But there is  one

way that is cheap and relatively easy: connect the two computers through

their terminal (TTY) ports, tricking one computer (or both) into believ-

ing  that the other is a terminal.  This can be expected to work because

the standard for connecting computers to terminals is almost universally

followed, in both hardware (plug and signal: EIA  RS-232)  and  software

(character  code: ASCII).  Once two computers are connected in this way,

cooperating programs can be run on each  to  achieve  the  desired  com-

munication by means of a communication protocol.

Why is a protocol necessary at all?  Three major problems occur when you

try to connect two computers via TTY line:

1. Noise  --  It  is rarely safe to assume that there will be no

electrical interference on a line; any long or switched  data

communication  line  will  have  occasional  interference, or

noise, which typically results in garbled  or  extra  charac-

ters.  Noise corrupts data, perhaps in subtle ways that might

not be noticed until it's too late.

2. Synchronization  --  Data  must  not  come in faster than the

receiving machine can handle it.  Although line speeds at the

two ends of the connection may match, the  receiving  machine

might not be able to process a steady stream of input at that

speed.   Its central processor may be too slow or too heavily

loaded, or its buffers too full or too small.    The  typical

symptom  of  a  synchronization  problem  is  lost data; most

operating systems will simply discard incoming data they  are

not prepared to receive.

3. Line Outages -- A line may stop working for short periods be-

cause  of  a  faulty  connector,  loss  of  power, or similar

reason.  On dialup or switched connections, such intermittent

failures will cause carrier to drop and the connection to  be

closed, but for any connection in which the carrier signal is

not used, the symptom will be lost data.

To   prevent  corruption  of  data  and  to  synchronize  communication,

cooperating computers can send control information to one another at the

same time that they are transferring data.  This intermingling  of  con-

trol  information  with  data,  and  the resulting actions, constitute a

"protocol".

KERMIT is such a protocol.  It is specifically designed for transfer  of

sequential  files  over ordinary serial telecommunication lines.  KERMIT

is not necessarily better than many other terminal-oriented file  trans-

fer protocols but it is free, it is well documented, and it has been im-

plemented compatibly on a variety of microcomputers and mainframes.

KERMIT transfers data by encapsulating it in "packets" of control infor-

mation.    This  information includes a synchronization marker, a packet

number to allow detection of lost packets, a  length  indicator,  and  a

"checksum" to allow verification of the data.  Lost or corrupt packets

Introduction                                                      Page 7

are  detected,  and retransmission is requested.  Duplicated packets are

discarded.  In addition, various special control packets allow cooperat-

ing KERMITs to connect and disconnect from each other  and  to  exchange

various  kinds  of information.  Very few assumptions are made about the

capabilities of either computer, so the KERMIT protocol can work between

many different kinds of systems.

ORGANIZATION OF THIS MANUAL

Section 2, How to Use KERMIT, tells all you need  to  know  to  transfer

text files in most cases, and shows some specific examples.

If  you  follow  the examples in Section 2 but you can't make a terminal

connection or you can't transfer files successfully, consult Section  3,

When Things Go Wrong.

If  you  expect to be a heavy user of KERMIT, you should read Section 4,

KERMIT Commands, which describes all the features of KERMIT  in  detail.

You  may  find  that  familiarity with the material in this section will

help you get past difficulties that can crop up when you are making  new

kinds  of  connections or transferring unusual kinds of files.  You will

also find descriptions of some advanced file  management  features  that

have been omitted from the earlier sections.

Section  5,  KERMIT Implementations, briefly lists the systems for which

KERMIT is available  as  of  this  writing.    The  subsequent  chapters

describe  selected  particular implementations.  You should read the ap-

propriate section for each system with which you are using KERMIT;  each

section  describes the file naming conventions and other system features

that are important to KERMIT users, and lists the  KERMIT  commands  for

that system mainly in terms of their differences from the "ideal" KERMIT

described in section 4.

How to Use KERMIT                                                 Page 8

2. How to Use KERMIT

KERMIT  is  a protocol for reliable file transfer between computers over

the ordinary serial telecommunication lines that  are  used  to  connect

terminals  to  computers.    The mechanics of using KERMIT to get a file

transferred can be confusing until you get the hang of  it.    A  little

background material might make the process a bit easier to understand.

KERMIT is probably the cheapest way to put two computers into communica-

tion.   The required hardware is usually already available, the software

is free, and all components run as ordinary user programs, with no  sys-

tem  modifications.    This is in sharp contrast to a communication net-

work, where there are dedicated high-speed communications  channels  and

drivers,  expensive  software,  and so forth.  The network provides more

services than KERMIT, usually at higher speed,  and  with  greater  con-

venience,  because  the  network  is usually part of the system.  When a

network is not available, KERMIT can fill in.  But since KERMIT  is  not

integrated with any particular system, but rather grafted on top of many

different systems, it requires some extra work from those who use it.

2.1. The KERMIT Program

KERMIT  embodies  a set of rules for transferring files reliably between

computers.  In general, one computer is a large system (a host, for  in-

stance  a  timesharing  system  with many terminals), and the other is a

1

personal computer (PC) .  The host believes that the PC is  an  ordinary

terminal.    In order for the KERMIT protocol to occur, a KERMIT program

must be running on each end of the communication  line  --  one  on  the

host, one on the PC.

The  two  Kermit  programs  exchange  messages in a special language all

their own, the Kermit protocol.  The dialog runs  something  like,  "Hi!

I'm  going  to  be  sending files to you.  When you send messages to me,

please don't make them more than 80 characters long, and  if  you  don't

hear anything from me for 15 seconds, wake me up, OK?"  "OK." "Now, here

comes  a  file  called  FOO.TXT, OK?"  "OK." "Here's the first piece..."

"Got it." "Good, here's the second piece..."   "That  second  piece  was

junk."  "Well,  then here it is again..."  Et cetera.  You don't see any

of this.  It's all packed into a concise code which the two Kermits  can

understand; they do all the worrying about transmission, error checking,

character  set  translation,  and  so  forth.   Each message is called a

packet, and each packet is in a special format that all Kermits can  un-

derstand.

_______________

1

Host-to-host and PC-to-PC connections are also possible.

How to Use KERMIT                                                 Page 9

2.2. Talking to Two Computers at Once

Your  task is just to get the two Kermits started.  The confusion arises

because you have to use a single keyboard and screen to talk to two dif-

ferent computers, two different programs.  Let's  talk  about  a  common

2

case:   you are sitting at a personal computer (PC ), which has a serial

communication port.  The serial port is connected  to  a  host  computer

3

using, say, a dialup modem .

Normally,  when  you use your PC, you are "talking" directly to it; your

commands are interpreted directly by the PC's  operating  system  (CP/M,

MS-DOS,  UNIX,  whatever),  or  by  some program that runs on the PC (an

editor, a text formatter, space invaders...).  The version of Kermit  on

your PC is a program like any other, but it has a special ability to ei-

ther  interpret  your commands directly, like other programs, or to pass

everything you type through to the  host.    When  you  tell  Kermit  to

CONNECT, it sends every character you type out the serial port, and it

will put every character that comes in the serial port onto the  screen.

This is called virtual terminal service -- one computer acts "virtually"

as  though  it were a terminal on another.  You are now "talking" to the

host, and the PC is ignoring you.

Kermit, like most programs, has a prompt.  The prompt  is  a  symbol  it

types  on the left margin to indicate that it is ready for you to type a

command.  Kermit's prompt is normally "Kermit-xx>".  The  xx  identifies

the  implementation  of  Kermit;  the  Kermit that runs on the DEC-20 is

called "Kermit-20" and its prompt is "Kermit-20>"; the Kermit that  runs

on  Z80  and  8080-based  microcomputers  is  called "Kermit-80" and its

4

prompt is "Kermit-80>"; the Kermit on the IBM PC is "Kermit-86" , and so

forth.  If you become confused about who you are talking to, the  prompt

should  provide  a clue.  In addition, most Kermits print an informative

message like

[Connecting to remote host, type CTRL-]C to return]

when you CONNECT, and type another message like

[Connection closed, back at PC]

_______________

2

The terms PC, micro, microcomputer, and workstation will all be  used

loosely in this document to denote a single-user system.

3

The  actual  means  of  connection isn't important in this case -- it

also could be a direct line to the host, some  kind  of  switched  line,

etc.

4

Although  the processor in the IBM PC is an 8088, it is programmed as

though it were an 8086.

How to Use KERMIT                                                Page 10

when you return.

Having  "connected" to the host, there must be a way for you to get back

to the PC.  This is accomplished by  an  escape  sequence.    As  Kermit

passes your characters through to the host, it checks each one to see if

it's  a  special  predefined  escape  character.   When the PC sees this

character, it stops ignoring you -- you are once again "talking" to  the

PC,  not  the  host.   The escape character is normally chosen to be one

that you will not need to type while talking to the host, and  one  that

is hard to type by accident -- it's usually a control character, such as

Control-],  which is accomplished by holding down the key marked CTRL or

CONTROL and typing the  indicated  character  (in  this  case,  a  right

bracket  "]").    The  CTRL  key  works  just like a SHIFT key.  Control

characters are written either as CTRL-A or ^A, where A is the  character

to be typed while holding down CTRL.

2.3. Transferring a File

To transfer a file, you must first get the attention of the PC's operat-

ing  system.  This is normally done by starting the PC, possibly insert-

ing your system floppy disk first.  Once you're at command level on your

PC, you run Kermit.  Then you tell Kermit to CONNECT you  to  the  host.

Now  you're  talking  to  the host -- at this point you must log in, and

then run Kermit on the host.

Now you have a Kermit on each end of the wire.  The next step is to tell

each Kermit what to do.  Suppose you want to transfer a  file  from  the

host  to  the PC; you would first tell the host Kermit to SEND the file,

then "escape" back to the PC Kermit and tell it  to  receive  the  file.

The transfer begins -- you can sit back and watch, or go make yourself a

sandwich.   The PC Kermit will continuously show packet and retry counts

on your screen, and will notify you when the transfer is complete.

The desired file is now on your PC's disk.  The Kermit protocol has  en-

sured  that  the  file  arrived  correctly and completely.  Now you must

clean up after yourself: CONNECT back to the host, exit from  Kermit  on

the  host,  log  out  from the host, "escape" back to PC Kermit and exit

from it.  Now you can do whatever you had planned for your file --  edit

it, print it on your PC printer, etc.

The  KERMIT protocol, and most Kermit programs, allow you to send a file

reliably from the host to the PC, from the PC to the host, from host  to

host,  or PC to PC, usually without any special regard for the nature of

the particular machines involved.  Most implementations also allow files

to be sent in groups, with a  single  command,  such  as  "Send  all  my

Fortran  files!"    The scenario for each of these is always the same as

above -- only the details of how to establish the actual connection dif-

fer.

KERMIT works best with "printable" files -- files composed only of  let-

ters,  digits,  punctuation  marks, carriage returns, tabs, and so forth

-- since these can be represented on almost any kind of computer.   KER-

MIT  is also able to transfer "binary" files -- files such as executable

programs -- composed of arbitrary bit patterns, but  binary  files  nor-

mally  are  meaningful  only  to  the kind of computer on which they are

How to Use KERMIT                                                Page 11

generated.  Nevertheless, KERMIT can usually move such files from system

A  to  system  B  (where  they are not much use) and back to system A in

their original condition, although in some cases some special care  must

be taken to accomplish this.

Now  that  we  have a basic understanding of what KERMIT does and how it

works, let's look at some more concrete examples.   First  you  need  to

know what the basic Kermit commands are.

2.4. Basic KERMIT Commands

These  are  generic  descriptions  of  the  most  basic Kermit commands.

Detailed  descriptions  will  come  later.    In   these   descriptions,

local refers to the system that you are using directly, remote refers to

the system to which you are CONNECTed via Kermit.  Commands may take one

or  more  operands  on  the  same line, and are terminated by a carriage

return.

SEND filespec   Send the file or file group specified by  filespec  from

this  Kermit  to  the  other.   The name of each file is

passed to the other Kermit in a special control  packet,

so  it  can  be stored there with the same name.  A file

group    is    usually    specified     by     including

"wildcard"  characters  like  "*" in the file specifica-

tion.  Examples:

send foo.txt

send *.for

Some implementations of Kermit may not support  transfer

of  file groups; these versions would require a separate

SEND command for each file to be transferred.

RECEIVE         Receive a file or file group from the other Kermit.   If

an  incoming  file  name  is  not legal, then attempt to

transform it to a similar legal name,  e.g. by  deleting

illegal  or  excessive characters.  The name thus formed

cannot  be  guaranteed  to  be  unique,  in  which  case

previously  existing  files  could be overwritten.  Some

versions of Kermit attempt to prevent  this  by  warning

you  of filename collisions and taking, or allowing for,

evasive action.

CONNECT         Make a "virtual terminal" connection to the remote  sys-

tem.    On a PC or micro, this usually means to send all

keyboard input out the serial port, and display all  in-

put  from  the  serial  port on the screen.  To "escape"

from a virtual terminal connection, type Kermit's escape

character (e.g. CTRL-], control-rightbracket),  followed

by the letter "C" for "Close Connection".

SET             Establish  various nonstandard settings, such as CONNECT

escape character,  file  characteristics,  communication

line number, parity, or flow control.

How to Use KERMIT                                                Page 12

SHOW            Display the values of SET options.

HELP            Type a summary of KERMIT commands and what they do.

EXIT            Exit from KERMIT back to the host operating system.

?               Typed  anywhere  within a KERMIT command:  List the com-

mands, options, or operands that are  possible  at  this

point.    This command may or may not require a carriage

return, depending on the host operating system.

2.5. Real Examples

Kermit can be used in several ways: from a PC that  is  connected  to  a

larger host computer; from a host computer which is connected to another

host; from one PC to another.

2.5.1. PC to Host

In  this  example, the user is sitting at an IBM Personal Computer (PC),

which is connected through its serial port to a DECSYSTEM-20  host  com-

puter.    The  IBM PC is local, the DEC-20 is remote.  This example will

also apply almost literally to any other microcomputer implementation of

Kermit.

You have started up your PC and have the Kermit program  on  your  disk.

Begin  by running Kermit on the PC.  Use Kermit's CONNECT command to be-

come a terminal to the DEC-20.  In fact, the  PC  emulates  the  popular

Heath-19  (or VT52) terminal, so it is desirable to tell the DEC-20 that

your terminal is one of these.  Login  on  the  DEC-20  and  run  Kermit

there.   Here is an example of this procedure with commands you type un-

derlined:

5

A>kermit              ! Run Kermit on the PC.

Kermit V1.20

Kermit-86>            ! This is the Kermit prompt for the PC.

Kermit-86>connect     ! Connect to the DEC-20.

[Connecting to host, type control-] to return to PC.

Baud rate is 9600, connecting over COM1.]

! You are now connected to the DEC-20.

CU20B                 ! The system prints its herald.

@terminal heath-19    ! Set your terminal type (optional).

@login my-id password ! Login using normal login method.

(At this point, the DEC-20 prints various messages.)

_______________

5

Everthing from a "!" mark to the end of line is commentary, not  sys-

tem typeout or part of a command.

How to Use KERMIT                                                Page 13

@kermit               ! Run Kermit on the DEC-20.

Kermit-20>            ! This is Kermit-20's prompt.

You are now ready to transfer files between the two machines.

The  following  example illustrates how to send files from the DEC-20 to

the PC.  Note the use of the "*" wildcard character  to  denote  a  file

group.

Kermit-20>send *.for  ! Send all my FORTRAN files.

^]c                   ! Now return back to the PC by

! typing the escape sequence, in this case

! ^]C (Control-] followed by "C")

[Back at PC.]

Kermit-86>receive     ! Tell the PC files are coming.

If you take more than about 5 seconds to get back to Kermit-86 and issue

the  RECEIVE command, the first packets from Kermit-20 may arrive prema-

turely and appear on your screen, but no harm will be done  because  the

packet will be retransmitted by the DEC-20 until the PC acknowledges it.

Once the connection is established, the PC will show you what is happen-

ing  --  it  first  clears the screen and waits for incoming packets; as

packets arrive, the current file name and packet  number  will  be  con-

tinuously  displayed  on  the screen.  When the PC's "Kermit-86>" prompt

returns to your screen, the transfer is done.  During file transfer, the

microcomputer screen looks something like this:

IBM PC Kermit-86 V1.20

Number of Packets:  294                       Receiving...

Number of Retries:    2

File Name:   FOO.TXT

The packet and retry counts are continuously updated, and  the  word  in

the  upper right tells the status of the transfer -- receiving, sending,

complete, interrupted, or failed.

When the transfer is complete (most versions of KERMIT sound a  beep  to

wake you up), you must CONNECT back to the DEC-20 host, EXIT from Kermit

there, logout, and "escape back" to the PC as you did previously.

Kermit-86>connect     ! Get back to the DEC-20.

[Connecting to host. Type CTRL-]C to return to PC.]

Kermit-20>            ! Here we are.

Kermit-20>exit        ! Get out of Kermit-20.

@logout               ! Logout from the DEC-20.

Logged out Job 55, User MY-ID, Account MY-ACCOUNT, TTY 146,

at 24-Jan-84 15:18:56,  Used 0:00:17 in 0:21:55

How to Use KERMIT                                                Page 14

^]c                   ! Now "escape" back to the PC,

[Back at PC.]

Kermit-86>exit        ! and exit from the PC's Kermit.

The files you transferred should now be on your PC disk.

To  send  files  from  the PC to the DEC-20, follow a similar procedure.

First follow the instructions in the previous section to log in  to  the

DEC-20  through  the  PC.    Then  in  response  to  the  host  Kermit's

"Kermit-20>" prompt you type RECEIVE rather than SEND.  Now escape  back

to  the PC and use the SEND command to send the local PC files to DEC-20

host.  The PC will show you the progress  of  the  transmission  on  its

screen.

When the "Kermit-86>" prompt indicates that the transmission is complete

you  should  follow  the procedure shown above to logout from the DEC-20

host, except that you may first wish to confirm that the files have been

stored correctly in your directory on the DEC-20.

2.5.2. Host to Host

This section describes use of Kermit between two hosts.    A  "host"  is

considered  to  be  a  large  or multi-user system, whose distinguishing

characteristic is that it has multiple terminals.   Use  of  Kermit  for

host-to-host file transfers differs from the PC-to-host case in that the

line  your  terminal  is  connected  to is not the same as the line over

which the data is being transferred, and that some special commands  may

have to be issued to allow one Kermit to conform to unusual requirements

of the other host.

In  this  example, you are already logged in to a DEC-20, and you use an

autodialer  to  connect  to   an   IBM   370-series   system   running

VM/CMS  through DEC-20 TTY port 12.  The autodialer, in this example, is

invoked from program called DIAL (idealized here,  for  simplicity),  to

which you merely supply the phone number.

@dial 765-4321/baud:1200

765-4321, baud 1200

[confirm]

Dialing your number, please hold...

Your party waiting is on TTY12:

@

Other  methods exist for connecting two hosts with a serial line.  Dedi-

cated hookups can be made simply by running an  EIA  cable  between  TTY

6

ports  on  the  two  systems.   For connecting to remote systems when no

_______________

6

Such a connection, by the  way,  usually  requires  the  receive  and

transmit  leads  (pins  2 and 3) be swapped in one of the RS-232 connec-

tors; this is called a "null modem" cable.

How to Use KERMIT                                                Page 15

autodialer  is  available,  a manual dialup connection is also possible,

7

but tricky.  If you have a microcomputer that supports KERMIT,  you  may

find it easier to first transfer from host A to the micro, then from the

micro to host B.

The following procedure would be the same in any case, once a connection

is made.

@

@kermit               ! Run Kermit on the DEC-20.

Kermit-20>set ibm     ! Turn on handshaking, parity, local echo.

Kermit-20>set line (to tty) 12 ! Indicate the line we'll use.

Kermit-20>connect     ! And connect to it.

[KERMIT-20: Connecting over TTY12:, type <CTRL-Y>C to return.]

VM/370 ONLINE         ! The IBM system prints its herald.

.login myuserid mypassword                  ! Login to IBM system.

LOGON AT 20:49:21 EST THURSDAY 01/20/84

CUVMB SP/CMS PUT 8210 01/19/84

.

.kermit

KERMIT-CMS>.send profile exec ! Send a file.

^Yc                   ! KERMIT-20's escape sequence typed here.

[KERMIT-20: Connection Closed. Back at DEC-20.]

Kermit-20>receive     ! Tell Kermit-20 to RECEIVE.

The transfer takes place now; Kermit-20 will print the names of incoming

files,  followed  by  dots or percents to indicate the packet traffic (a

dot for every 5 packets successfully transferred, a  percent  for  every

timeout  or retransmission).  It is complete when when you see "[OK]", a

beep is sounded, and the Kermit-20 prompt next appears.  At  that  point

we  connect  back  to the remote IBM system, exit from the remote Kermit

and log out.

.

PROFILE.EXEC.1 ..%%.[OK]

Kermit-20>connect     ! Get back to IBM and clean up.

[KERMIT-20: Connecting over TTY12:, type <CTRL-Y>C to return.]

_______________

7

Here's one way: log in on port x on your system, and  assign  another

port,  y,  to  which you have physical access.  Unplug the terminal from

port y, and connect the terminal to a dialup modem.  Dial up the  remote

computer  and  log in on it.  Now, using a null modem cable, connect the

modem directly to port y. Go back to your terminal on port x, run Kermit

from it, and CONNECT to port y.

How to Use KERMIT                                                Page 16

KERMIT-CMS>.

KERMIT-CMS>.exit

R;

.

SP/CMS

.logout

CONNECT= 00:03:01 VIRTCPU= 000:00.12 TOTCPU= 000:00.60

LOGOFF AT 20:52:24 EST THURSDAY 01/20/84

^Yc                   ! Type Kermit-20's escape sequence

[KERMIT-20: Connection Closed. Back at DEC-20.]

Kermit-20>exit        ! All done with Kermit.

That's  the whole procedure.  The file is in your DEC-20 directory, com-

pletely readable, as PROFILE.EXEC --  note  that  KERMIT-CMS  translated

from  the  IBM  EBCDIC  character encoding into standard ASCII, and con-

verted the space between the file name and file type to a dot.

To send a file from the local host to the remote host, we  would  merely

have reversed the SEND and RECEIVE commands in the example above.

2.5.3. Micro to Micro

Kermit   also   works   between   personal   computers  (microcomputers,

workstations).  The difference here is that commands are  typed  on  two

keyboards,  rather  than  a single one.  This is because a personal com-

puter normally only accepts commands from its own keyboard.  If  one  PC

Kermit  CONNECTs  to  another,  there will normally be no program on the

other side to listen.

Making the physical connection between two micros is tricky.  If the two

8

units are in close proximity , you can connect their serial ports with a

null modem cable.  However, different micros have different requirements

-- some may want a male connector on their serial port, others a female;

9

many require that certain of the RS-232 signals be held  high  or  low .

In  any  case,  you  must also make sure the port speeds are the same at

both ends.

_______________

8

Why would you want to run Kermit between two PCs  that  are  next  to

each other?  One good reason is that if they are different models, their

floppy disks are probably incompatible.

9

By  wiring  certain  of  the  pins in the connector together; for in-

stance, some micros want DTR (Data Terminal Ready, pin 20)  to  be  held

high,  and  this might be accomplished by connecting it to CTS (Clear To

Send, pin 5).  See EIA Standard RS-232-C, and  the  appropriate  manuals

for your micro.

How to Use KERMIT                                                Page 17

Connections  at  longer  distances can be made via dialup, providing the

required modems are available (one side needs autoanswer capability), or

using any kind of dedicated or switched circuit that  may  be  available

--  PBX,  port contention unit, almost anything you can plug an EIA con-

nector into.

In this example, a DEC VT180 "Robin" CP/M microcomputer is connected  to

a  Intertec  "SuperBrain"  CP/M micro, using a female-to-male null modem

cable.  Getting the cable right is the hard part.  The connection can be

tested by running Kermit and issuing the CONNECT command on  both  ends:

typein from each micro should appear on the screen of the other.

Suppose  you  want  to  send a file FOO.HEX from the Robin to the Super-

Brain.  Proceed as follows:

1. Run Kermit on the SuperBrain, and give the RECEIVE command:

A>kermit

Intertec SuperBrain Kermit-80 - V3.7

Kermit-80>receive

2. Run Kermit on the  Robin,  and  give  the  SEND  command  for

FOO.HEX.

A>kermit

DEC VT18X Kermit-80 - V3.7

Kermit-80>send foo.hex

Watch  the  packets  fly.    When you get the next Kermit-80>

prompt, the transfer is done, and you can EXIT from both Ker-

mits.

The key point is to start the receiving end first -- most  microcomputer

Kermits  do  not  include a timeout facility, and if the receiver is not

ready to receive when the sender first sends, there will be  a  protocol

deadlock.

2.6. Another Way -- The KERMIT Server

So  far,  we  have  been describing the bare-bones version of the KERMIT

protocol.  An optional extension to the protocol includes the concept of

a Kermit server.  A KERMIT server is a Kermit program that does not  in-

teract  directly  with  the  user, but only with another Kermit program.

You do not type commands to a Kermit server, you merely start it at  one

end  of  the connection, and then type all further commands at the other

end.

Not all implementations of Kermit can be servers, and not all  know  how

to talk to servers -- but most of the major ones can and do.  The server

is  run  on  the  remote computer, which would normally be a large host,

such as the DEC-20.  You must still connect to the remote host to log in

and start the server, but you no longer have to tell one  side  to  SEND

and  the  other to RECEIVE, nor must you connect back to the remote side

to clean up and log out when you're done.  Using the server, you can  do

as  many  send and receive operations as you like without ever having to

How to Use KERMIT                                                Page 18

connect  back  to the remote host.  Some servers also provide additional

services, such as directory listings, file deletion, or disk  usage  in-

quiries.

A  Kermit server is just a Kermit program running in a special mode.  It

acts much like ordinary Kermit does after you give it a RECEIVE  command

--  it  waits  for a message from the other Kermit, but in this case the

message is a command telling what to do, normally to send or to  receive

a  file or group of files.  After escaping back to the local system, you

can give as many SEND and GET commands as  you  like,  and  when  you're

finished transferring files, you can give the BYE command, which sends a

message  to  the remote Kermit server to log itself out.  You don't have

to connect back to the remote host and clean up.  However, if  you  want

to  connect  back to the host, you can use the FINISH command instead of

BYE, to shut down the Kermit server on the remote host  without  logging

it off, allowing you to CONNECT back to your job there.

Here's an example of the use of a Kermit server.  The user is sitting at

a CP/M-80 microcomputer and a DEC-20 is the remote host.

A>kermit              ! Run Kermit on the micro.

Kermit V3.9A

Kermit-80>            ! This is the micro Kermit's prompt.

Kermit-80>connect     ! Connect to the DEC-20.

[Connecting to remote host.  Type CTRL-]C to return to micro.]

CU20E                 ! The DEC-20 prints its herald.

@login my-id password ! Log in normally.

(The DEC-20 prints various login messages here.)

@kermit               ! Run Kermit-20 normally

Kermit-20>server      ! Tell it to be a server.

Kermit Server running on DEC-20 host.  Please type your escape

sequence to return to your local machine.  Shut down the server by

typing the Kermit BYE command on your local machine.

^]c                   ! Now escape back to the micro.

[Connection closed, back at micro.]

Kermit-80>get *.pas   ! Get all my DEC-20 Pascal programs.

Kermit-80>send foo.*  ! Send all the "foo" files from my micro.

Kermit-80>exit        ! Exit from Kermit back to CP/M.

A>

(Here you can do some work on the micro, edit files, whatever you like.)

A>kermit              ! Run Kermit-80 some more.

Kermit-80>send file.pas                     ! Send another file.

Kermit-80>bye         ! That's all.  Shut down the Kermit server.

A>                    ! Back at CP/M automatically.

This  is  much simpler.  Note that once you've started the Kermit Server

on the remote end, you can run Kermit as often as you like on the  micro

without having to go back and forth any more; just make sure to shut the

How to Use KERMIT                                                Page 19

server down when you're done by typing the BYE command.

Here are basic the commands available for talking to servers.

SEND filespec   Sends  a  file  or file group from the local host to the

remote host in the normal way.

GET filespec    Ask the remote host to send a file or file group.    Ex-

ample:

get *.c

This  command is exactly equivalent to typing "send *.c"

at the remote host followed by "receive"  on  the  local

host.    Note  that the local Kermit does not attempt to

validate the filespec.  If the server cannot  parse  it,

or  cannot  access  the  specified file(s), it will send

back an appropriate error message.

BYE             Shut down the remote server and exit from Kermit.   This

will  cause the job at the remote end to log itself out.

You need not connect back and clean up unless you get an

error message in response to this command (for instance,

if your logged-out disk quota is exceeded on the  remote

host).

FINISH          Shut  down  the server without having it log itself out,

and don't exit from Kermit.  A subsequent  CONNECT  com-

mand  will  put you back at your job on the remote host,

at system command level.

When Things Go Wrong                                             Page 20

3. When Things Go Wrong

Connecting  two  computers can be a tricky business, and many things can

go wrong.  Before you can transfer files at all, you must  first  estab-

lish  terminal  communication.   But successful terminal connection does

not necessarily mean that file transfer will also work.  And  even  when

file transfer seems to be working, things can happen to ruin it.

3.1. Communication Line Problems

If  you  have a version of KERMIT on your microcomputer, but the CONNECT

command doesn't seem to work at all, please:

- Make sure all the required physical connections have been made

and have not wiggled loose.  If you are using  a  modem,  make

sure the carrier light is on.

- If  you  have more than one connector on your micro, make sure

you are using the right one.

- Make sure that the port is  set  to  the  right  communication

speed,  or  baud  rate.  Some versions of KERMIT have a built-

SET BAUD command, others require that you set  the  baud  rate

using a system command or setup mode before you start the KER-

MIT  program.   Use the SHOW command to find out what the cur-

rent baud rate is.

- Make sure that the other communication line  parameters,  like

parity,  bits  per  character, handshake, and flow control are

set correctly.

You must consult the appropriate manuals for the systems  and  equipment

in question.

If  all settings and connections appear to be correct, and communication

still does not take place, the fault may be in  your  modem.    Internal

modems  (i.e.  those  that  plug  in  to a slot inside the microcomputer

chassis) are not recommended for use with KERMIT.    Many  microcomputer

KERMIT  programs  are  written to control the communication hardware ex-

plicitly; internal modems can interfere with that control.

KERMIT normally expects to have full control of the communication  port.

However,  it  is  sometimes  the case that some communications equipment

controls the line between the two computers on either end.  Examples in-

clude modems (particularly "smart" modems), port contention or selection

units, multiplexers, local networks, and wide-area networks.  Such equi-

pment can interfere with the KERMIT file transfer  protocol  in  various

ways:

- It  can impose parity upon the communication line.  This means

that the 8th bit of each character is used by the equipment to

check for correct transmission.  Use of parity will:

* Cause  packet  checksums  to  appear  incorrect  to   the

receiver  and foil any attempt at file transfer.  In most

When Things Go Wrong                                             Page 21

cases, not even the first packet will get through.

* Prevent the use of the 8th bit for binary file data.

If  terminal  connection  works  but  file  transfer does not,

parity is the most likely culprit.  To overcome  this  impedi-

ment,  you  should find out what parity is being used, and in-

form the KERMITs on each side (using the SET  PARITY  command)

so that they can:

* Compose and interpret the checksums correctly.

* Employ  a  special  encoding  to allow 8-bit data to pass

through the 7-bit communication channel.

Many packet-switched networks, such as  GTE  TELENET,  require

10

parity to be set.

- Communications equipment can also interpret certain characters

in  the data stream as commands rather than passing them along

to the other side.  For instance, you might find your  "smart"

modem  suddenly  disconnecting  you and placing a call to Tas-

mania.  The only way to work around such problems  is  to  put

the  device  into  "transparent"  or "binary" mode.  Most com-

munication devices have a way to  do  this;  consult  the  ap-

propriate  manual.   In some cases, transparent mode will also

cancel the parity processing and allow the use of the 8th  bit

for data.

3.2. The Transfer is Stuck

There  are various ways in which Kermit file transfers can become stuck,

but since many hosts are capable of generating timeout  interrupts  when

input  doesn't  appear  quickly enough, they can usually resend or "NAK"

(negatively acknowledge) lost packets.    Nevertheless,  if  a  transfer

seems to be stuck, you can type RETURN on the keyboard of most micros to

simulate a timeout.

An interesting exception is the IBM mainframe (VM/CMS) Kermit -- it can-

not  time  out its "virtual console" (i.e. the user's terminal), so when

using Kermit from a micro to an IBM host, occasional manual wakeups  may

be necessary.

The  following  sections  discuss  various  reasons  why  a  transfer in

progress could become stuck.  Before examining these,  first  make  sure

that you really have a Kermit on the other end of the line, and you have

issued the appropriate command: SEND, RECEIVE, or SERVER.  If the remote

side  is  not a server, remember that you must connect back between each

transfer and issue a new SEND or RECEIVE command.

_______________

10

TELENET uses MARK parity.

When Things Go Wrong                                             Page 22

3.3. The Micro is Hung

The micro itself sometimes becomes hung for reasons beyond Kermit's con-

trol,  such  as  power fluctuations.  If the micro's screen has not been

updated for a long time, then the micro may be hung.   Try  these  steps

(in the following order):

- Check  the  connection.   Make sure no connectors have wiggled

loose from their sockets.  If you're using a modem, make  sure

you  still have a carrier signal.  Reestablish your connection

if you have to.

- Press RETURN to wake the micro up.  This should clear  up  any

protocol deadlock.  Several RETURNs might be necessary.

- If  the problem was not a deadlock, restart the micro and then

restart Kermit, CONNECT back to the host, get back to your job

or login again, and restart the transfer.   You  may  have  to

stop and restart Kermit on the remote host.

3.4. The Remote Host Went Away

If  your  local  system  is  working but the transfer is hung, maybe the

remote host or the remote KERMIT program crashed.  Get back  to  command

level  on the local KERMIT (on microcomputer implementations, you may be

able  to  do  this  by  typing  about  five  RETURNs,  or  one  or  more

Control-C's).    Issue the CONNECT command so that you can see what hap-

pened.  If the remote system has crashed then you will have to wait  for

it to come back, and restart whatever file that was being transferred at

the time.

3.5. The Disk is Full

If  your  local  floppy disk or remote directory fills up, the Kermit on

the machine where this occurs will inform you  and  then  terminate  the

transfer.    You can continue the transfer by repeating the whole proce-

dure either with a fresh floppy or after  cleaning  up  your  directory.

Some  KERMIT  programs  allow you to continue the sequence where it left

off, for instance on the DEC-20 by using the SEND command and  including

the name of the file that failed in the "(INITIAL)" field:

Kermit-20>send *.for (initial) foo.for

See  the Kermit-20 command summary for further information about the in-

itial filespec.

When Things Go Wrong                                             Page 23

3.6. Message Interference

You  may  find  that  file transfers fail occasionally and upredictably.

One explanation could be that terminal messages  are  being  mixed  with

your  file  packet  data.  These could include system broadcast messages

(like "System is going down in 30 minutes"), messages from  other  users

("Hi   Fred,  what's  that  KERMIT  program  you're  always  running?"),

notifications that you have requested ("It's 7:30,  go  home!"  or  "You

have  mail from...").  Most KERMIT programs attempt to disable intrusive

messages automatically, but not all can be guaranteed to do so.  It  may

be necessary for you to "turn off" such messages before starting KERMIT.

3.7. Host Errors

Various  error conditions can occur on the remote host that could effect

file transmission.  Whenever any such error occurs,  the  remote  Kermit

normally attempts to send an informative error message to the local one,

and  then  breaks transmission, putting you back at Kermit command level

on the local system.

3.8. File is Garbage

There are certain conditions under which Kermit can  believe  it  trans-

ferred a file correctly when in fact, it did not.  The most likely cause

has  to  do with the tricky business of file attributes, such as text vs

binary, 7-bit vs 8-bit, blocked vs stream, and so forth.    Each  system

has its own peculiarities, and each KERMIT has special commands to allow

you to specify how a file should be sent or stored.  However, these dif-

ficulties usually crop up only when sending binary files.  Textual files

should normally present no problem between any two KERMIT programs.

3.9. Junk after End of File

When transferring a text file from a microcomputer to a mainframe, some-

times  you  will find extraneous characters at the end of the file after

it arrives on the target system.  This is  because  many  microcomputers

don't  have a consistent way of indicating the end of a file.  CP/M is a

good example.  The minimum unit of storage on a CP/M floppy is a "block"

of 128 bytes.  Binary files always consist of a whole number of  blocks,

but  a  text  file can end anywhere within a block.  Since CP/M does not

record a file's byte count, it uses the convention of  marking  the  end

with  an imbedded Control-Z character.  If your microcomputer version of

KERMIT is not looking for this character, it will send the  entire  last

block,  which  may  contain  arbitrary  junk after the "real" end of the

file.  To circumvent this problem, most microcomputer KERMITs have  com-

mands  like  SET  FILE ASCII or SET FILE TEXT to instruct KERMIT to obey

the CTRL-Z convention.  Some microcomputer  KERMITs  operate  in  "text"

mode by default, others in "binary" or "block" mode.

KERMIT Commands                                                  Page 24

4. KERMIT Commands

An  "ideal" KERMIT program will be described here, which has most of the

features specified in the KERMIT Protocol Manual.    No  KERMIT  program

will  have  all  these commands or support all these options.  The exact

form of some of the commands may differ from version to version.    Some

KERMIT programs may support system-dependent options not described here.

The  intention  of  this  description  is  to  provide a base from which

specific KERMIT programs can be described in terms of their  differences

from the "ideal."

4.1. Remote and Local Operation

Some KERMIT programs can be run in two ways, remote and local.  A remote

Kermit  is  usually  running on a mainframe, which you have CONNECTed to

through a PC or other computer.  When KERMIT  runs  remotely,  all  file

transfer  is  done  over the job's controlling terminal line -- the same

line over which you logged in, and to which you would  type  interactive

commands.    What  the  system thinks is your terminal is really another

computer, usually a microcomputer, running its own copy of Kermit.

When KERMIT is in "local mode", file transfer is done over  an  external

device,  such  as a microcomputer's serial communication port, or an as-

signed terminal line on a mainframe.  The local Kermit is  connected  in

some  way  (like a dialout mechanism) to another computer, again running

its own copy of Kermit.  A local Kermit is in control of the  screen,  a

remote Kermit has no direct access to it.  Microcomputer KERMITs usually

run  in local "mode", whereas mainframe Kermits usually need to be given

some special command to run in local mode.   Some  commands  make  sense

only for remote Kermits, others only for local, still others can be used

with  either.    Local and remote operation of KERMIT is shown schemati-

cally here:

PC is Local, Mainframe is Remote:

KERMIT Commands                                                  Page 25

Communication

Line                             (Packets)

+---------------/  /-----------------+  Other terminals

|                                    |  |  |  |

|                                    |  |  |  |

PC        |     LOCAL              Mainframe   |  |  |  |  REMOTE

+----------+----------+            +------------+--+--+--+--------+

|   Serial Port       |            |            |                 |

|                     |            |            |                 |

|                     |            |            |                 |

|  +---------------+  |            |            Your job's        |

|  | Packets: 724  |  |            |            terminal line     |

|  | Retries:   7  |  |            |                              |

|  | File: FOO.BAR |  |            |                              |

|  +---------------+  |            |                              |

|   Screen            |            |                              |

|                     |            |                              |

+---------------+-----+            +------------------------------+

|

| (Commands)

|

+------------+---------+

\       Keyboard       \

+----------------------+

You

The  KERMIT  program  on  the  PC is a local Kermit.  It can control the

screen, the keyboard, and the port separately, thus it  can  update  the

screen  with  status  information,  watch for interrupt signals from the

keyboard, and transfer packets on the communications port,  all  at  the

same time.

The  KERMIT  program  running  on the mainframe is a remote Kermit.  The

user logs in to the mainframe through a terminal port.   The  host  com-

puter  cannot tell that the user is really coming in through a microcom-

puter.  The keyboard, screen, and port functions  are  all  combined  in

user's  mainframe  terminal  line.  Therefore a remote Kermit is cut off

from your screen and keyboard during file transfer.

A KERMIT server is always remote, and must get its commands from a local

KERMIT.  The following descriptions will indicate when a command must be

remote or local.

4.2. Command Interface

Most implementations (the UNIX version is the major exception)  have  an

interactive  keyword-style  command interface, modeled after that of the

DECSYSTEM-20,  which  is  roughly  as  follows:  In  response   to   the

"Kermit-xx>"  prompt  you  may type a keyword, such as SEND, RECEIVE, or

EXIT, possibly followed by additional  keywords  or  operands,  each  of

which  is  called  a  field.   You can abbreviate keywords (but not file

names) to any length that makes  them  distinguishable  from  any  other

KERMIT Commands                                                  Page 26

keyword  valid for that field.  You can type a question mark at any time

to get information about what's expected or valid at that  point.    The

ESC  and "?"  features work best on full duplex systems (all but the IBM

mainframe, so far), where the program can "wake up" immediately and per-

form the required function.  On half duplex or record-oriented  systems,

the  ESC  feature  is  not  available,  and the "?"  requires a carriage

return to follow.

In this example, the user types "set" and then a question mark  to  find

out  what  the  SET options are.  The user then continues the command at

the point where the question mark was typed, adding a  "d"  and  another

question  mark  to  see  what set options start with "d".  The user then

adds a "u" to select "duplex" (the only  SET  option  that  starts  with

"du")  followed  by an ESC (shown here by a dollar sign) to complete the

current field and issue the guide word "(to)" for  the  next  one,  then

another  question  mark to see what the possibilities are, and so forth.

The command is finally terminated by a carriage return.  Before carriage

return is typed, however, the command can  be  edited  using  RUBOUT  or

other  command editing keys.  Finally, the same command is entered again

with a minimum  of  keystrokes,  with  each  field  abbreviated  to  its

shortest  unique  length.   In the example, the parts the user types are

underlined; all the rest is system typeout:

Kermit-20>set ? one of the following:

debugging        delay             duplex            escape

file             handshake         IBM               line

parity           receive           send

Kermit-20>set d? one of the following:

debugging   delay        duplex

Kermit-20>set du$plex (to) ? one of the following:

full   half

Kermit-20>set duplex (to) h$alf

Kermit-20>set du h

4.3. Notation

In the command descriptions, the following notation is used:

anything    A parameter - the symbol in italics is replaced  by  an  ar-

gument of the specified type (number, filename, etc).

[anything]  An  optional  field.    If  omitted,  it  defaults to an ap-

propriate value.

number      A whole number, entered in prevailing notation of  the  sys-

tem.

character   A  single  character,  entered  literally,  or  as  a number

(perhaps octal or hexadecimal) representing the ASCII  value

of the character.

floating-point-number

A  "real"  number, possibly containing a decimal point and a

fractional part.

KERMIT Commands                                                  Page 27

filespec    A  file specification, i.e. the name of a file, possibly in-

cluding a search path, device or directory  name,  or  other

qualifying  information,  and possibly containing "wildcard"

or pattern-matching characters to denote a group of files.

^X          A control  character  may  be  written  using  "uparrow"  or

"caret" notation, since many systems display control charac-

ters  this  way.  Control characters are produced by holding

down the key marked CTRL  or  Control  and  typing  the  ap-

propriate character, e.g. X.

Commands  are shown in upper case, but can be entered in any combination

of upper and lower case.

KERMIT Commands                                                  Page 28

4.4. Summary of KERMIT Commands

Here  is a brief list of KERMIT commands as they are to be found in most

KERMIT programs.  The following sections will describe these commands in

detail.

For exchanging files:

SEND, RECEIVE, GET

For connecting to a remote host:

CONNECT, SET LINE, SET PARITY, SET DUPLEX, SET  HANDSHAKE,  SET  ES-

CAPE, SET FLOW-CONTROL

For acting as a server:

SERVER

For talking to a server:

BYE, FINISH, GET, SEND, REMOTE

Setting nonstandard transmission and file parameters:

SET BLOCK-CHECK, SET DEBUG, SET DELAY, SET FILE, SET INCOMPLETE, SET

PARITY, SET RETRY;

SET  SEND  (or RECEIVE) END-OF-LINE, START-OF-PACKET, PACKET-LENGTH,

PAUSE, TIMEOUT, PADDING

For defining "macros" of SET commands:

DEFINE

For interrupting transmission:

Control-X, Control-Z, Control-C, Control-E

Getting information:

HELP, STATISTICS, SHOW

Executing command files:

TAKE

For recording the history of a file transfer operation:

LOG TRANSACTIONS

For non-protocol file capture or transmission:

LOG SESSION, TRANSMIT

For closing log files:

CLOSE

Leaving the program:

EXIT, QUIT

If you have a file called KERMIT.INI in your default or home disk,  KER-

MIT  will  execute an automatic TAKE command on it upon initial startup.

KERMIT.INI may contain any KERMIT commands, for instance  SET  commands,

or DEFINEs for SET macros to configure KERMIT to various systems or com-

munications  media.   Note: Your particular implementation of KERMIT may

use a different name for this file.

KERMIT Commands                                                  Page 29

4.5. The SEND Command

Syntax:

Sending a single file:

SEND nonwild-filespec1 [filespec2]

Sending multiple files:

SEND wild-filespec1 [filespec2]

The  SEND  command  causes  a file or file group to be sent to the other

system.  There are two  forms  of  the  command,  depending  on  whether

filespec1 contains "wildcard" characters.  Use of wildcard characters is

the  most  common method of indicating a group of files in a single file

specification.  For instance if FOO.FOR is  a  single  file,  a  FORTRAN

program named FOO, then *.FOR might be a group of FORTRAN programs.

Sending a File Group

If  filespec1  contains wildcard characters then all matching files will

be sent, in directory-listing order (according to  the  ASCII  collating

sequence)  by  name.  If a file can't be opened for read access, it will

be skipped.  The initial file in a wildcard group can be specified  with

the  optional  filespec2.  This allows a previously interrupted wildcard

transfer to continue from where it left off, or it can be used  to  skip

some files that would be transmitted first.

Sending a Single File

If  filespec1  does not contain any wildcard characters, then the single

file specified by filespec1 will be sent.  Optionally, filespec2 may  be

used  to specify the name under which the file will arrive at the target

system; filespec2 is not parsed or validated locally in  any  way.    If

filespec2 is not specified, the file will be sent with its own name.

SEND Command General Operation

Files  will  be  sent  with  their  filename  and filetype (for instance

FOO.BAR,  no  device  or  directory  field,  no  generation  number   or

attributes).    If  communication  line  parity  is  being used (see SET

PARITY), the sending KERMIT will request that the other KERMIT accept  a

special  kind  of prefix notation for binary files.  This is an advanced

feature, and not all KERMITs have it; if the other KERMIT does not agree

to use this feature, binary files cannot be sent correctly.

The sending KERMIT will also ask the other KERMIT whether it can  handle

a  special  prefix  encoding  for  repeated characters.  If it can, then

files with long strings of repeated characters will be transmitted  very

efficiently.   Columnar data, highly indented text, and binary files are

the major beneficiaries of this technique.

KERMIT Commands                                                  Page 30

SEND Remote Operation

If you are running KERMIT remotely (for instance, from a microcomputer),

you should "escape back" to your local Kermit within a reasonable amount

of  time and give the RECEIVE command.  Don't take more than a minute or

two to complete the switch, or KERMIT may "time out"  and  give  up  (in

that  case, you'll have to CONNECT back to the remote system and reissue

the SEND command).

SEND Local Operation

If you're running KERMIT locally, for instance on a  microcomputer,  you

should  have already run KERMIT on the remote system and issued either a

RECEIVE or a SERVER command.

Once you give KERMIT the SEND command, the name of  each  file  will  be

printed  on  your screen as the transfer begins, and information will be

displayed to indicate the packet traffic.  When the specified  operation

is complete, the program will sound a beep, and the status of the opera-

tion  will  be indicated by a message like OK, Complete, Interrupted, or

Failed.

If you see many packet retry indications,  you  are  probably  suffering

from a noisy connection.  You may be able to cut down on the retransmis-

sions  by  using  SET  SEND PACKET-LENGTH to decrease the packet length;

this will reduce the probability that a given packet will  be  corrupted

by noise, and reduce the time required to retransmit a corrupted packet.

If  you  notice  a file being sent which you do not really want to send,

you may cancel the operation immediately by typing either  Control-X  or

Control-Z.    If your are sending a file group, Control-X will cause the

current file to be skipped, and KERMIT will go  on  to  the  next  file,

whereas Control-Z will cancel sending the entire group and return you to

KERMIT-20 command level.

4.6. The RECEIVE Command

Syntax:  RECEIVE [filespec]

The  RECEIVE command tells KERMIT to wait for the arrival a file or file

group sent by a SEND command from the other system.  If only one file is

being received, you may include the optional filespec  as  the  name  to

store the incoming file under; otherwise, the name is taken from the in-

coming  file header.  If the name in the header is not a legal file name

on the local system, KERMIT will attempt to  transform  it  to  a  legal

name.

If  an  incoming file has the same name as an existing file, KERMIT will

either overwrite the old file or else try to create a new  unique  name,

depending on the setting of FILE WARNING.

If  you  have  SET PARITY, then 8th-bit prefixing will be requested.  If

the other side cannot do this, binary files cannot be  transferred  cor-

rectly.  The sending KERMIT may also request that repeated characters be

KERMIT Commands                                                  Page 31

compressed.

If  an  incoming  file does not arrive in its entirety, KERMIT will nor-

mally discard it; it will not appear in your directory.  You may  change

this behavior by using the command SET INCOMPLETE KEEP, which will cause

as much of the file as arrived to be saved in your directory.

RECEIVE Remote Operation

If  your  are  running  KERMIT  remotely, you should escape back to your

local Kermit and give the SEND command.  You should do this within about

two minutes, or KERMIT may time out and give up; if  this  happens,  you

can CONNECT back to the remote system and reissue the RECEIVE command.

RECEIVE Local Operation

If you are running KERMIT locally, you should already have issued a SEND

11

command   to the remote KERMIT, and then escaped back to DEC-20 Kermit.

As  files arrive, their names will be shown on your screen, along with a

continuous display the packet traffic.

If a file begins to arrives that you don't really want, you can  attempt

to  cancel  it by typing Control-X; this sends a cancellation request to

the remote Kermit.  If the remote Kermit understands this  request  (not

all  implementations  of  Kermit  support this feature), it will comply;

otherwise it will continue to send.  If a file group is being sent,  you

can request the entire group be cancelled by typing Control-Z.

4.7. GET

LOCAL ONLY -- Syntax: GET [remote-filespec]

The GET command requests a remote KERMIT server to send the file or file

group  specified  by  remote-filespec.  Note the distinction between the

RECEIVE and GET commands: RECEIVE puts KERMIT into a passive wait state,

whereas GET actively sends a command to a server.

The GET command can be used only when KERMIT is  local,  with  a  KERMIT

server on the other end of the line.  This means that you must have CON-

NECTed  to  the  other  system,  logged in, run KERMIT there, issued the

SERVER command, and escaped back to the local KERMIT.

The remote filespec is any string that can be a legal file specification

for the remote system; it is not parsed or validated locally.  As  files

arrive,  their names will be displayed on your screen, along with a con-

_______________

11

not SERVER -- use the GET command to receive  files  from  a  KERMIT

server.

KERMIT Commands                                                  Page 32

tinuous  indication  of  the packet traffic.  As in the RECEIVE command,

you may type Control-X to request that the current incoming file be can-

celled, Control-Z to request that the  entire  incoming  batch  be  can-

celled.

If  the  remote KERMIT is not capable of server functions, then you will

probably get an error message back from it like "Illegal  packet  type".

In this case, you must connect to the other Kermit, give a SEND command,

escape back, and give a RECEIVE command.

Optional  Syntax:  If you are requesting a single file, you may type the

GET command without a filespec.  In  that  case,  Kermit  programs  that

implement  the  optional  GET  syntax  will  prompt  you  for the remote

filespec on the subsequent line, and the name to store it under when  it

arrives on the line after that:

Kermit-MS>get

Remote Source File: aux.txt

Local Destination File: auxfile.txt

4.8. SERVER

REMOTE ONLY -- Syntax: SERVER

The  SERVER  command  instructs KERMIT to cease taking commands from the

keyboard and to receive all further instructions in the form  of  KERMIT

packets  from  another system.  A KERMIT server must be remote; that is,

you must be logged in to the system through another computer, such as  a

microcomputer.   In addition, your local KERMIT should have commands for

communicating with remote servers; these include GET, FINISH, and BYE.

After issuing this command, escape back to your local system  and  issue

SEND,  GET,  BYE,  FINISH, or other server-oriented commands from there.

If your local KERMIT does not have a BYE command, then it does not  have

the  full ability to communicate with a KERMIT server and you should not

put the remote KERMIT into SERVER mode.  If your local KERMIT does  have

a  BYE  command,  use it to shut down and log out the KERMIT server when

you are done with it.

Any nonstandard parameters should be selected with SET  commands  before

putting  KERMIT  in  server mode, in particular the block check type and

special file modes.

4.9. BYE

LOCAL ONLY -- Syntax: BYE

When running as a local Kermit talking to a KERMIT server, use  the  BYE

command  to  shut down and log out the server.  This will also close any

debugging log files and exit from the local KERMIT.

KERMIT Commands                                                  Page 33

4.10. FINISH

LOCAL ONLY -- Syntax: FINISH

When running as a local Kermit talking to a remote KERMIT server use the

FINISH  command  to  shut down the server without logging out the remote

job, so that you can CONNECT back to it.  Also, close any  local  debug-

ging log file.

4.11. REMOTE

LOCAL ONLY -- Syntax: REMOTE command

When  running  in local mode, talking to a remote KERMIT server send the

specified command to the remote server.  If the server does  not  under-

stand  the  command  (all of these commands are optional features of the

KERMIT protocol), it will reply with  a  message  like  "Unknown  KERMIT

server command".  If does understand, it will send the results back, and

they will be displayed on the screen.  The REMOTE commands are:

CWD [directory] Change  Working  Directory.    If  no  directory name is

provided, the server will change to the  default  direc-

tory.    Otherwise, you will be prompted for a password,

and the server will attempt to change to  the  specified

directory.    If  access is not granted, the server will

provide a message to that effect.

DELETE filespec Delete the specified file or files.  The  names  of  the

files that are deleted will appear on your screen.

DIRECTORY [filespec]

The  names  of  the  files  that  match  the  given file

specification will be displayed on your screen.   If  no

file  specification is given, all files from the current

directory will be listed.

DISK [directory]

Provide information about  disk  usage  in  the  current

directory,  such  as the quota, the current storage, the

amount of remaining free space.

HELP            Provide a list of the functions that are available.

HOST [command]  Pass the given command  to  the  server's  host  command

processor,  and  display  the  resulting  output on your

screen.

KERMIT [command]

Pass the given command, which is expressed in the server

KERMIT's own interactive-mode  command  syntax,  to  the

server  for execution.  This is useful for changing set-

tings, logging, and other functions.

RUN program-name [command-line-argument]

Have the remote KERMIT run the  indicated  program  with

KERMIT Commands                                                  Page 34

the  indicated  command  line;  send the results back to

your screen.

PROGRAM [command]

Send the command to the program started by  most  recent

REMOTE  RUN  program,  and  display  the  results on the

screen.  If no command is given, send newline character.

TYPE filespec   Display the contents  of  the  specified  file  on  your

screen.

4.12. LOCAL

Syntax: LOCAL command

Execute the specified command on the local system -- on the system where

KERMIT to which your are typing this command is running.  These commands

provide  some  local  file management capability without having to leave

the KERMIT program, which is particularly useful on microcomputers.

CWD [directory] "Change Working Directory" to the specified directory.

DELETE filespec Delete the specified file or files.

DIRECTORY [filespec] Provide a directory listing of the specified files.

Some KERMIT programs may provide commands for these or  other  functions

in  the  syntax of their own system, when this would cause no confusion.

For instance, CP/M KERMIT may use ERA in place of LOCAL DELETE.

4.13. CONNECT

LOCAL ONLY -- Syntax: CONNECT [terminal-designator]

Establish a terminal connection to the system at the other  end  of  the

communication  line.    On  a microcomputer, this is normally the serial

port.  On a mainframe, you will have to specify a terminal  line  number

or  other  identifier, either in the CONNECT command itself, or in a SET

LINE command.  Get back to the local KERMIT by typing the escape charac-

ter followed by a single character "command".  Several  single-character

commands are possible:

C   Close the connection and return to the local KERMIT.

S   Show status of the connection.

B   Send a BREAK signal.

0   (zero) Send a NUL (0) character.

P   Push  to  the  local system command processor without breaking the

connection.

Q   Quit logging session transcript.

R   Resume logging session transcript.

?   List all the possible single-character arguments.

^] (or whatever you have set the escape character to be)

Typing the escape character twice sends one copy of it to the con-

nected host.

KERMIT Commands                                                  Page 35

You  can use the SET ESCAPE command to define a different escape charac-

ter, and SET PARITY, SET DUPLEX, SET FLOW-CONTROL, SET HANDSHAKE to  es-

tablish or change those parameters.

4.14. HELP

Syntax: HELP

Typing HELP alone prints a brief summary of KERMIT and its commands, and

possibly  instructions  for  obtaining  more detailed help on particular

topics.  Most KERMIT implementations also allow the use of "?" within  a

command to produce a short help message.

4.15. TAKE

TAKE filespec

Execute  KERMIT  commands from the specified file.  The file may contain

contain any valid KERMIT commands, including other TAKE commands.

4.16. EXIT, QUIT

EXIT

Exit from KERMIT.

QUIT is a synonym for EXIT.

4.17. The SET Command

Syntax: SET parameter [option] [value]

Establish or modify various parameters for  file  transfer  or  terminal

connection.

When a file transfer operation begins, the two KERMITs automatically ex-

change  special  initialization messages, in which each program provides

the other with certain information about itself.  This  information  in-

cludes the maximum packet size it wants to receive, the timeout interval

it wants the other KERMIT to use, the number and type of padding charac-

ters  it  needs,  the  end-of-line  character it needs to terminate each

packet (if any), the block check type, the desired prefixes for  control

characters, characters with the "high bit" set, and repeated characters.

Each  KERMIT  program  has  its  own  preset  "default" values for these

parameters, and you normally need not concern yourself with them.    You

can  examine  their  values  with  the  SHOW command; the SET command is

provided to allow you to change them in order to adapt to  unusual  con-

ditions.

The following parameters may be SET:

BAUD-RATE       Set the speed of the current communications port

KERMIT Commands                                                  Page 36

BLOCK-CHECK     Packet transmission error detection method

DEBUGGING       Mode or log file

DELAY           How long to wait before starting to send

DUPLEX          For  terminal  connection,  full  (remote  echo) or half

(local echo)

ESCAPE          Character for terminal connection

FILE            For setting file parameters  like  name  conversion  and

byte size

FLOW-CONTROL    Selecting flow control method, like XON/XOFF

HANDSHAKE       For turning around half duplex communication line

IBM             Set things up for communicating with an IBM mainframe

INCOMPLETE      What to do with an incomplete file

LINE            Terminal  line  to  use  for terminal connection or file

transfer

PARITY          Character parity to use

PORT            For switching communication ports

PROMPT          For changing the program's command prompt

RECEIVE         Various parameters for receiving files

RETRY           How many times to retry a packet before giving up

SEND            Various parameters for sending files

The DEFINE command may be used to compose "macros" by combining SET com-

mands.  The SET commands are now described in detail.

SET BAUD-RATE

Set or change the baud rate (approximate translation: transmission speed

in bits per second) on the  currently  selected  communications  device.

The  way  of  specifying  the baud rate varies from system to system; in

most cases, the actual number (such as 1200 or 9600) is typed.   Systems

that  do not provide this command generally expect that the speed of the

line has already been set appropriately outside of KERMIT.

SET BLOCK-CHECK

KERMIT normally uses a 1-character block check, or "checksum",  on  each

packet.   The sender of the packet computes the block check based on the

other characters in the packet, and the receiver recomputes it the  same

way.    If these quantities agree, the packet is accepted and the trans-

mission proceeds.  If they disagree, the packet is rejected  and  trans-

mitted again.

However,  the  block check is not a foolproof method of error detection.

The normal single-character KERMIT block check is only a 6-bit  quantity

(the  low  order 8 bits of the arithmetic sum folded upon itself).  With

6

only six bits of accuracy, the chances are one in 2  --  that  is,  1/64

--  that  an error can occur which will not be detected in the checksum,

assuming that all errors are equally likely.

You can decrease the probability that an error can slip through, at  the

expense of transmission efficiency, by using the SET BLOCK-CHECK command

to  select  more  rigorous  block  check  methods.   Note that all three

methods will detect any single-bit error, or any error in an odd  number

KERMIT Commands                                                  Page 37

of bits.  The options are:

1-CHARACTER-CHECKSUM:

The normal single-character 6-bit checksum.

2-CHARACTER-CHECKSUM:

A 2-character, 12-bit checksum.  Reduces the probability

of  an error going undetected to 1/4096, but adds an ex-

tra character to each packet.

3-CHARACTER-CRC:

A 3-character, 16-bit  Cyclic  Redundancy  Check,  CCITT

format.    In  addition  to  errors in any odd number of

bits, this method detects double bit errors,  all  error

bursts of length 16 or less, and more than 99.99% of all

possible  longer  bursts.   Adds two extra characters to

each packet.

The single character checksum has proven to be quite adequate  in  prac-

tice,  much more effective than straightforward analysis would indicate,

since all errors are not equally likely, and a simple checksum  is  well

suited  to  catching  the  kinds  of errors that are typical of telecom-

munication lines.  The other methods should be requested only  when  the

connection is very noisy.

Note  that  the 2- and 3-character block checks are not available in all

versions of KERMIT; if the other KERMIT is not capable of performing the

higher-precision block checks, the transfer will automatically  use  the

standard single-character method.

SET DEBUG

Syntax: SET DEBUG options

Record  the  packet  traffic, either on your terminal or in a file.  Op-

tions are:

STATES          Show  Kermit  state  transitions  and   packet   numbers

(brief).

PACKETS         Display each incoming and outgoing packet (lengthy).

LOG-FILE        Log  the selected information (STATES or PACKETS) to the

specified file.  If log file not specified, then use the

terminal if local.

OFF             Don't  display  debugging  information  (this   is   the

default).    If debugging was in effect, turn it off and

close any log file.

KERMIT Commands                                                  Page 38

SET DELAY

Syntax: SET DELAY number

Specify how many seconds to wait before sending the first packet after a

SEND command.  Use when remote and SENDing files back to your local Ker-

mit.   This gives you time to "escape" back and issue a RECEIVE command.

The normal delay is 5 seconds.  In local mode  or  server  mode,  KERMIT

does not delay before sending the first packet.

SET DUPLEX

Syntax: SET DUPLEX keyword

For use when CONNECTed to a remote system.  The keyword choices are FULL

and  HALF.  FULL means the remote system echoes the characters you type,

HALF means the local system echoes them.  FULL is the  default,  and  is

used  by  most  hosts.    HALF  is  necessary  when  connecting  to  IBM

mainframes.  Half duplex is also called "local echo".

SET ESCAPE

Syntax: SET ESCAPE character

Specify or change the character you want to use to "escape" from  remote

connections  back  to  KERMIT.    This would normally be a character you

don't expect to be using on the remote system, perhaps a control charac-

ter like ^\, ^], ^^, or ^_.  Most versions of KERMIT use one of these by

default.  After you type the escape character, you must follow it  by  a

single-character  "argument", such as "C" for Close Connection.  The ar-

guments are listed above, under the description of the CONNECT command.

SET FILE

Syntax: SET FILE parameter keyword

Establish file-related parameters.  Depending on the characteristics  of

the  system, it may be necessary to tell KERMIT how to fetch an outbound

file from the disk, or how to  store  an  incoming  file.    The  actual

parameters you can specify in this command will vary from system to sys-

tem,  and  you should consult the documentation for your particular ver-

sion of KERMIT.  Some examples would be byte size (PDP-10 architecture),

record length or  block  size  (record  oriented  systems),  end-of-file

detection method (on microcomputers).

This  can  be  a very important command if you intend to transfer binary

files, but is normally unecessary for transmitting textual files.

KERMIT Commands                                                  Page 39

SET FLOW-CONTROL

Syntax: SET FLOW-CONTROL option

For  communicating  with full duplex systems.  System-level flow control

is not necessary to the KERMIT protocol, but it can help to  use  it  if

the  same  method is available on both systems.  The most common type of

flow control on full duplex systems is XON/XOFF.

SET HANDSHAKE

Syntax: SET HANDSHAKE option

For communicating with half duplex systems.  This lets you  specify  the

line  turnaround  character  sent by the half duplex host to indicate it

has ended its transmission and is granting you permission  to  transmit.

When  a  handshake  is set, KERMIT will not send a packet until the half

duplex  host  has  sent  the  specified  character  (or  a  timeout  has

occurred).  The options may include:

NONE    No handshake; undo the effect of any previous SET HANDSHAKE.

XOFF    Control-S.

XON     Control-Q.

BELL    Control-G.

CR      Carriage Return, Control-M.

LF      Linefeed, Control-J.

ESC     Escape, Control-[.

SET INCOMPLETE

Syntax: SET INCOMPLETE option

Specify  what  to  do when a file transfer fails before it is completed.

The options are DISCARD (the default) and KEEP.   If  you  choose  KEEP,

then  if  a transfer fails to complete successfully, you will be able to

keep the incomplete part that was received.

SET LINE

Syntax: SET LINE [terminal-designator]

Specify the terminal line to use for file transfer  or  CONNECT.    This

command  is  found  on  mainframe KERMITs, which normally run in "remote

mode" using their own controlling terminal for file transfer.   Specify-

ing  a  separate  line  puts the program in "local mode."  If no line is

specified, revert to the job's controlling terminal,  i.e.  go  back  to

"remote mode."

KERMIT Commands                                                  Page 40

SET PORT

Syntax: SET PORT terminal-designator

Specify the communications port for file transfer or CONNECT.  This com-

mand  is  found  on microcomputer KERMITs that run in "local" mode.  SET

PORT does not change the remote/local status but simply selects  a  dif-

ferent port for local operation.

SET PARITY

Syntax: SET PARITY keyword

Parity is a technique used by communications equipment for detecting er-

rors on a per-character basis; the "8th bit" of each character acts as a

check  bit for the other seven bits.  KERMIT uses block checks to detect

errors on a per-packet basis, and it  does  not  use  character  parity.

However,  some  systems  that KERMIT runs on, or equipment through which

these systems communicate, may be using character  parity.    If  KERMIT

does  not know about this, arriving data will have been modified and the

block check will appear to be wrong, and packets will be rejected.

If parity is being used on the communication line, you must inform  both

KERMITs,  so the desired parity can be added to outgoing characters, and

stripped from incoming ones.  SET PARITY should be used for  communicat-

ing with hosts that require character parity (IBM mainframes are typical

examples)  or  through  devices  or networks (like GTE TELENET) that add

parity to characters that pass through them.  Both KERMITs should be set

to the same parity.  The specified parity is used both for terminal con-

nection (CONNECT) and file transfer (SEND, RECEIVE, GET).

The choices for SET PARITY are:

NONE  (the default) eight data bits and no parity bit.

MARK  seven data bits with the parity bit set to one.

SPACE seven data bits with the parity bit set to zero.

EVEN  seven data bits with the parity bit  set  to  make  the  overall

parity even.

ODD   seven  data  bits  with  the  parity bit set to make the overall

parity odd.

NONE means no parity processing is done, and the 8th bit of each charac-

ter can be used for data when transmitting binary files.

If you have set parity to ODD, EVEN, MARK, or SPACE, then advanced  ver-

sions of KERMIT will request that binary files will be transferred using

8th-bit-prefixing.    If  the  KERMIT  on the other side knows how to do

8th-bit-prefixing (this is an optional feature of the  KERMIT  protocol,

and not all implementations of KERMIT have it), then binary files can be

transmitted  successfully.  If NONE is specified, 8th-bit-prefixing will

not be requested.

KERMIT Commands                                                  Page 41

SET PROMPT

This  allows  you  to change the program's prompt.  This is particularly

useful if you are using KERMIT to transfer files between two systems  of

the  same  kind,  in which case you can change the prompts of the KERMIT

programs involved to include appropriate distinguishing information.

SET SEND

SET SEND parameter

Parameters for outgoing packets, as follows:

END-OF-LINE character

The ASCII character to be used as a line terminator for packets,

if one is required by  the  other  system,  carriage  return  by

default.    You  will  only have to use this command for systems

that require a line terminator other than carriage return.

PACKET-LENGTH number

Maximum packet length to  send  between  10  and  94  (decimal).

Shortening  the  packets might allow more of them to get through

through without error on noisy communication lines.  Lengthening

the packets increases the throughput on clean lines.

TIMEOUT number

How many seconds to wait for a packet before trying again.

PAUSE floating-point-number

How many seconds to pause before sending each data packet.  Set-

ting this to a nonzero value may allow some slow systems  enough

time  to  consolidate  itself  packet before the next packet ar-

rives.  Normally, no per-packet pausing is done.

PADDING number, PADCHAR character

How much padding to send before a  packet,  if  the  other  side

needs  padding, and what character to use for padding.  Defaults

are no padding, and NUL (0) for the padding character.

QUOTE character

What printable character to use for quoting of  control  charac-

ters,  "#" (43) by default.  There should be no reason to change

this.

START-OF-PACKET character

The start-of-packet character is the only control character used

"bare" by the KERMIT protocol.  It is Control-A by default.   If

a bare Control-A causes problems for your communication hardware

or software, you can use this command to select a different con-

trol  character  to  mark  the start of a packet.  You must also

issue the reciprocal command (SET  RECEIVE  START-OF-PACKET)  to

the  KERMIT  on  the  other  system  (providing  it  has  such a

command).

KERMIT Commands                                                  Page 42

SET RECEIVE

Syntax: SET RECEIVE parameter

Parameters to request or expect for incoming packets, as follows:

END-OF-LINE character

Carriage return (15) by default.

PACKET-LENGTH number

Maximum  length  packet for the other side to send, decimal num-

ber, between 10 and 94, decimal.

TIMEOUT number

How many seconds the other Kermit should wait for a  packet  be-

fore asking for retransmission.

PAUSE floating-point-number

How  many  seconds to pause before acknowledging a packet.  Set-

ting this to a nonzero value will slow down the  rate  at  which

data  packets  arrive,  which  may be necessary for systems that

have "sensitive" front ends and cannot accept input  at  a  high

rate.

PADDING number, PADCHAR character

How  many  padding  characters  to  request before each incoming

packet, and what the padding character should be.    No  KERMITs

are  known  to need padding, and if one did, it would request it

without your having to tell it to do so.    This  command  would

only be necessary, therefore, under very unusual circumstances.

QUOTE character

What  printable  character to use for quoting of control charac-

ters, "#" (43) by default.  There should be no reason to  change

this.

START-OF-PACKET character

The control character to mark the beginning of incoming packets.

Normally SOH (Control-A, ASCII 1) (see SET SEND START-OF-PACKET,

above).

SET RETRY

SET RETRY option number

Set the maximum number of retries allowed for:

INITIAL-CONNECTION

How  many  times  to  try  connecting before giving up, normally

something like 15.

PACKETS How many times to try sending a particular packet before  giving

up,  normally 5.  If a line is very noisy, you might want to in-

crease this number.

KERMIT Commands                                                  Page 43

4.18. DEFINE

DEFINE macroname [set-parameters]

Define  a "SET macro" to allow convenient association of one or more SET

parameters with a mnemonic keyword of your choice.  The  SET  parameters

are  a list of one or more SET options, separated by commas.  If you use

KERMIT to communicate with several different kinds of systems,  you  may

set up a macro for each, for instance:

DEFINE IBM PARITY MARK, DUPLEX HALF, HANDSHAKE XON

DEFINE UNIX PARITY NONE, DUPLEX FULL, HANDSHAKE NONE

DEFINE TELENET PARITY MARK, RECEIVE TIMEOUT 20

You may then type SET IBM, SET UNIX, and so forth to set all the desired

parameters  with  a  single  command.  It is convenient to include these

definitions in your KERMIT.INI file.

Another other handy use for SET macros would be for rapid adaptation  to

different conditions of line noise:

DEFINE CLEAN BLOCK-CHECK 1, SEND PACKET-LENGTH 94, RETRY PACKET 5

DEFINE NOISY BLOCK-CHECK 2, SEND PACKET-LENGTH 60, RETRY PACKET 10

DEFINE VERY-NOISY BLOCK 3, SEND PACKET 40, RETRY PACKET 20

You may redefine an existing macro in the same manner as you defined it.

You can undefine an existing macro by typing an empty DEFINE command for

it, for instance:

DEFINE IBM

You  can list all your macros and their definitions with the SHOW MACROS

command.

4.19. SHOW

Syntax: SHOW [option]

The SHOW command displays the values of the parameters settable  by  the

SET  command.   If a particular option is not requested, a complete dis-

play will be provided.

4.20. STATISTICS

Give statistics about the most recent file transfer, such as  the  total

number of characters transmitted, the effective baud rate, and so forth.

KERMIT Commands                                                  Page 44

4.21. LOG

Syntax: LOG [option] [filespec]

Log the specified entity to the specified log file.

TRANSACTIONS    Direct  KERMIT  to  log transactions, such as files suc-

cessfully sent or received or files that  could  not  be

successfully  sent or received.  A transaction is useful

recording the progress of a long,  unattended  multifile

transfer.

SESSION         Create a transcript of a CONNECT session, when running a

local  KERMIT  connected  to  a  remote  system,  in the

specified file.  The log is closed  when  connection  is

closed.     In  some  implementations,  logging  can  be

"toggled" by typing the connect  escape  character  fol-

lowed  by  Q  (Quit  logging)  or  R (Resume logging) or

similar single-character commands.   Session-logging  is

useful  for recording dialog with an interactive system,

and for "capturing" from systems that don't have KERMIT.

No guarantee can be made that the file will arrive  cor-

rectly  or  completely,  since  no  error checking takes

place.

DEBUGGING       Record debugging  information  in  the  specified  file.

There  may  be several options to select the desired in-

formation --  entire  packets,  state  transitions,  etc

-- available via the SET DEBUGGING command.

4.22. TRANSMIT

Syntax: TRANSMIT filespec

Send  the  contents  of  the  specified file to the other system "bare",

without protocol, packets, error checking, or retransmission.  This com-

mand is useful for sending standard logon or connection  sequences,  and

for  sending  files to systems that don't have KERMIT.  No guarantee can

be made that the target system will receive the file correctly and  com-

pletely.    When  receiving  a file, the target system would normally be

running a text editor in text collection mode.

KERMIT Implementations                                           Page 45

5. KERMIT Implementations

Kermit  has  been written for a wide variety of systems, both mainframes

and microcomputers.  Kermit is  not  written  in  a  portable  language;

rather,  each  implemenation  is written in the language best suited for

the particular machine.  The specification, given in the Kermit Protocol

Manual, is  quite  general  and  allows  implementation  on  almost  any

machine.

Here's a brief table summarizing the known Kermit implementations, as of

this  writing.    This list is constantly growing, and may be far out of

date by the time you read it.

Machine             Operating System    Language

DECsystem-10,20     TOPS-10,20          MACRO-10,20

IBM 370 Series      VM/CMS, MVS/TSO     IBM Assembler

IBM 370 Series      MTS                 Pascal

CDC Cyber 170       NOS                 Fortran-77

Sperry/Univac-1100  EXEC                EXEC Assembler

Honeywell           MULTICS             PL/I

DEC VAX-11          VMS                 Bliss-32, Macro-32

DEC PDP-11          RT,RSX,RSTS,MUMPS   MACRO-11 & others

DEC Pro-300 Series  P/OS                Bliss-16, Macro-11

VAX,PDP-11,SUN,etc  UNIX                C

PRIME               PRIMOS              PL/P

HP3000, Univac, etc Software Tools      Ratfor

HP1000              RTE                 Fortran

Apollo              Aegis               Fortran

Terak, HP-98x6, IBM PC                  UCSD p-System

8080, 8085, or Z80  CP/M-80             ASM

8086, 8088          PC-DOS, MS-DOS      MS MASM-86

8086, 8088          CP/M-86             DR ASM86

Apple II 6502       Apple DOS           DEC-10/20 CROSS

TRS80 I, III        TRSDOS              Z80 Assembler

Atari               DOS                 Action!

The 8080 version runs on  the  DEC  VT180,  DECmate  II  (CP/M),  Heath/

Zenith-89  and 100, Superbrain, Apple II/Z80, TRS-80 II (CP/M), Osborne,

Kaypro, and others.  There are 8086 MS DOS versions for the IBM PC,  DEC

Rainbow,  Wang  PC, Compaq, Heath/Zenith-100, HP-150, Tandy 2000, Victor

9000, and others.  The 8086 CP/M-86 version runs on the DEC Rainbow  and

the NEC APC.

The  remainder  of  the  KERMIT User Guide is devoted to descriptions of

selected KERMIT implementations.  If a description of  your  version  of

KERMIT does not appear, look in the KERMIT area on your mainframe for an

on-line  documentation  file.   Even if your version is described below,

the version of the manual you are reading may be out of date and the on-

line information may be more current.

DECSYSTEM-20 KERMIT                                              Page 46

6. DECSYSTEM-20 KERMIT

Authors:    Frank da Cruz, Bill Catchings, Columbia University

Language:   MACRO-20

Version:    4.1(236)

Date:       July 3, 1984

KERMIT-20 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      (Uses generation numbers)

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes

IBM communication:                 Yes

Transaction logging:               Yes

Session logging:                   Yes

Raw transmit:                      No

Act as server:                     Yes

Talk to server:                    Yes

Advanced commands for servers:     Yes

Local file management:             Yes

Handle file attributes:            No

Command/init files:                Yes

KERMIT-20 is a program that implements the KERMIT file transfer protocol

for  the  Digital Equipment Corporation DECSYSTEM-20 mainframe computer.

It is written in MACRO-20 assembly language and should run on any DEC-20

system with version 4 of TOPS-20 or later.

The KERMIT-20 section will describe the things you should know about the

DEC-20 file system in order to make effective use of KERMIT, and then it

will describe the special features of the KERMIT-20 program.

6.1. The DEC-20 File System

The features of the DEC-20 file system of greatest  interest  to  KERMIT

users  are the form of the file specifications, and the distinctions be-

tween text and binary files.

DECSYSTEM-20 KERMIT                                              Page 47

DEC-20 File Specifications

DEC-20 file specifications are of the form

DEVICE:<DIRECTORY>NAME.TYPE.GEN;ATTRIBUTES

where  the  DIRECTORY, NAME, and TYPE may each be up to 39 characters in

length, GEN is a generation (version number), and various attributes are

possible (protection code, account, temporary, etc).  Generation and at-

tributes are normally omitted.   Device  and  directory,  when  omitted,

default  to  the  user's  own (or "connected") disk and directory.  Thus

NAME.TYPE is normally sufficient to specify a file, and only this infor-

mation is sent along by KERMIT-20 with an outgoing file.

The device, directory, name, and type fields may contain uppercase  let-

ters,  digits,  and the special characters "-" (dash), "_" (underscore),

and "$" (dollar sign).  There are no imbedded or trailing spaces.  Other

characters may be included by prefixing them (each)  with  a  Control-V.

The fields of the file specification are set off from one another by the

punctuation indicated above.

The device field specifies a physical or "logical" device upon which the

file is resident.  The directory field indicates the area on the device,

for  instance  the  area  belonging to the owner of the file.  KERMIT-20

does not transmit the device or directory fields to the  target  system,

and does not attempt to honor device or directory fields that may appear

in  incoming  file  names;  for  instance, it will not create new direc-

tories.

The name is the primary identifier for the file.  The type, also  called

the  "extension",  is an indicator which, by convention, tells what kind

of file we have.  For instance  FOO.FOR  is  the  source  of  a  Fortran

program  named  FOO;  FOO.REL  might  be  the  relocatable object module

produced by compiling  FOO.FOR;  FOO.EXE  could  an  executable  program

produced by LOADing and SAVing FOO.REL, and so forth.

The  DEC-20  allows  a  group  of files to be specified in a single file

specification by including the special "wildcard"  characters,  "*"  and

"%".  A "*" matches any string of characters, including no characters at

all; a "%" matches any single character.  Here are some examples:

*.FOR   All  files  of  type  FOR (all Fortran source files) in the con-

nected directory.

FOO.*   Files of all types with name FOO.

F*.*    All files whose names start with F.

F*X*.*  All files whose names start with F and contain at least one X.

%.*     All files whose names are exactly one character long.

*.%%%*  All files whose types are at least three characters long.

Wildcard notation is used on many computer systems in similar ways,  and

it  is  the  mechanism  most  commonly used to instruct KERMIT to send a

DECSYSTEM-20 KERMIT                                              Page 48

group of files.

Text Files and Binary Files

The  DEC-20,  like  most  computers,  has  a  file  system  with its own

peculiarities.  Like many other systems, the DEC-20 makes a  distinction

between  text  files  and  binary files.  Text files are generally those

composed only of printing characters (letters, digits, and  punctuation)

and  "carriage  control"  characters  (carriage  return, line feed, form

feed, tab).  Text files are designed to be read by people.  Binary files

are designed to be read by a computer program, and may have any contents

at all.  If you use the DEC-20 TYPE command to display a  text  file  on

your  terminal,  the  result will be intelligible.  If you type a binary

file on your terminal, you will probably see mainly gibberish.  You  can

not  always tell a text file from a binary file by its name or directory

information, though in general files with types like  .TXT,  .DOC,  .HLP

are  textual (as are "source files" for computer programs like text for-

matters and programming language compilers), and files with  types  like

.EXE, .REL, .BIN are binary.

The  DEC-20  has  an  unusual  word size, 36 bits.  It differs from most

other systems by storing text in 7-bit, rather than 8-bit, bytes.  Since

text is encoded in the 7-bit ASCII character set, this allows  more  ef-

ficient use of storage.  However, the word size is not a multiple of the

normal byte size.  The DEC-20 therefore stores five 7-bit characters per

word, with one bit left over.

It  is  also  possible to store files with other byte sizes.  The common

layouts of bytes within a word are:

7  Text Files: Five 7-bit bytes per word.

+------+------+------+------+------++

|      |      |      |      |      ||

+------+------+------+------+------++

0      7      14     21     28     35

Normally, bit 35 is unused and set to zero.   However,  in  EDIT

(or  SOS,  or OTTO) line-numbered files, bit 35 is set to 1 when

the word contains a line number.

8  "Foreign" binary files:  Four 8-bit bytes per word.

+-------+-------+-------+-------+---+

|       |       |       |       |   |

+-------+-------+-------+-------+---+

0       8       16      24      32  35

Bits 32-35 are unused.

DECSYSTEM-20 KERMIT                                              Page 49

36  "Native" binary files:  One 36-bit byte per word.

+-----------------------------------+

|                                   |

+-----------------------------------+

0                                   35

All bits are used.

The  minimum unit of disk allocation on the DEC-20 is a page, 512 36-bit

words, or 2560 7-bit characters, or 2048 8-bit bytes.    Any  file  that

contains  at  least one bit of information occupies at least a full page

on the disk.  The directory information for a file includes  the  number

of  pages occupied on the disk, the bytesize of the file, and the number

of bytes of that size which are in the file.  This  information  can  be

seen by using the DEC-20 VDIRECTORY command, for instance

@vdir foo.*

PS:<MY-DIRECTORY>

Name     Protection    Pages Bytes(Size) Creation

FOO.COM.1;P774242          1 384(8)     27-Dec-83

MAC.1;P774242           1 152(7)     27-Dec-83

.REL.1;P774242           1 39(36)     27-Dec-83

.EXE.1;P774242           2 1024(36)   27-Dec-83

Total of 5 pages in 4 files

In  this  example, FOO.MAC occupies 1 page, and is composed of 152 7-bit

bytes.  This file is textual (program source for the  MACRO  assembler),

152  characters  long.    Programs  which  read text files (such as text

editors, program compilers, the TYPE command, etc) determine the end  of

a file from the byte count specified in the directory.  KERMIT-20 deter-

mines  the  end of file in the same way, so although FOO.MAC occupies an

entire 2560-byte page of storage, only  the  first  152  characters  are

transmitted.    Binary  files,  such  as  FOO.EXE  (an executable DEC-20

program), tend to occupy full pages.  In this case too,  KERMIT-20  uses

the byte count to determine the end of file.

Why do you need to know all this?  In most cases, you don't.  It depends

on whether you are using the DEC-20 as your "home base".

Using a Microcomputer to Archive DEC-20 Files

Most  computers  (other  than the DEC-10 and DEC-20) store characters in

8-bit bytes.  Let's call any such system an  8-bit-byte  system.    This

discussion  applies  to  all  8-bit-byte  systems, including all popular

microcomputers,  minicomputers  like  the  DEC  PDP-11  and   VAX,   and

mainframes  like  the IBM 370.  For simplicity, we'll focus on microcom-

puters.

KERMIT can send any "native" DEC-20 sequential file, text or binary,  to

an  8-bit-byte  system  and  bring it back to the DEC-20 restored to its

original form.  If you are using a microcomputer to archive your  DEC-20

files, you need never concern yourself with details of byte size or file

DECSYSTEM-20 KERMIT                                              Page 50

format.    The  same  holds  true between two DEC-20s, or a DEC-10 and a

DEC-20.

There is, however, one special  complication  of  which  you  should  be

aware.    Certain  microcomputer operating systems, notably CP/M, do not

have an entirely satisfactory way of indicating the end of  file.    The

file  length  is  recorded in blocks rather than bytes.  For text files,

the end of file is marked within a block by inserting a Control-Z  after

the  last  data  character.  Binary files, however, might easily contain

Control-Z characters as data.  Therefore, in order  not  to  lose  data,

these systems must transmit binary files in complete blocks.  If the bi-

nary file is of foreign origin (for instance, from a DEC-20), and it did

not  happen  to  fill  up  the last block when it was transferred to the

micro, then when that file is sent back  to  the  system  of  origin  in

"binary  mode," junk will appear at the end (if it is sent back in "text

mode," it might be truncated by any data  byte  that  happened  to  cor-

respond  to  Control-Z).    For  DEC-20  programs  in  .EXE format, this

generally has no effect on the runnability or behavior of  the  program.

But  for other binary files, particularly internal format numerical data

or relocatable program object (.REL) files, the junk could have bad  ef-

fects.    Extraneous data at the end of a .REL file will generally cause

LINK to fail to load the file.

Using the DEC-20 to Archive Microcomputer Files

You can use KERMIT to send textual files from  a  microcomputer  or  any

8-bit  system  to the DEC-20 with no special provisions, since KERMIT-20

stores incoming characters in 7-bit bytes as text unless you  explicitly

instruct  it  otherwise.   But KERMIT-20 has no automatic way of distin-

12

guishing an incoming binary file from an incoming  text  file.    Binary

files  from 8-bit-byte systems generally contain significant data in the

8th bit, which would be lost if the incoming characters were  stored  in

7-bit  bytes,  rendering the file useless when sent back to the original

system.  Thus if you want to use KERMIT to store  foreign  8-bit  binary

data on the DEC-20, you must tell it to store such files with a bytesize

of 8 rather than 7.  This can be the source of much confusion and incon-

venience.    In  particular, you cannot use a "wildcard send" command to

send a mixture of text and binary files from an 8-bit-byte system to the

DEC-20; rather, you must send  all  text  files  with  KERMIT-20's  file

bytesize  set  to 7, and all 8-bit binary files with the bytesize set to

8.

Once you get the foreign binary file into the DEC-20,  stored  with  the

correct  bytesize  (as FOO.COM is stored in the example above), you need

take no special measures to send it back to its system of origin.   This

is  because KERMIT-20 honors the bytesize and byte count from the direc-

tory.  For instance, if you told KERMIT-20 to SEND FOO.*, every file  in

the  example above would be transmitted in the correct manner, automati-

cally.

_______________

12

Unless the incoming file has an "ITS Binary Header"; see below.

DECSYSTEM-20 KERMIT                                              Page 51

Files KERMIT-20 Cannot Handle

The  KERMIT  protocol can only accommodate transfer of sequential files,

files which are a linear sequence of bytes (or words).

Some files on the DEC-20 are not sequential, and cannot be  successfully

sent  or  received  by  KERMIT-20.  These include directory files, files

with holes (missing pages), ISAM files, and RMS files.  These files  re-

quire  external  information (kept in the DEC-20's file descriptor block

and/or index table) in order to be reconstructed;  when  sending  files,

KERMIT-20 presently transmits only the file name and the contents of the

file.    External  control  information (file attributes) are not trans-

mitted.

6.2. Program Operation

Kermit-20's prompt is "Kermit-20>".  Kermit-20 will accept a single com-

mand on the Exec command line, like this:

@

@kermit send foo.bar

the file is sent

@

or you can run the program interactively to issue several commands, like

this:

@

@kermit

TOPS-20 KERMIT version 4.1(236)

Kermit-20>send foo.*

files are sent

Kermit-20>statistics

performance statistics are printed

Kermit-20>receive

files are received

Kermit-20>exit

@

During interactive operation, you may use the  TOPS-20  help  ("?")  and

recognition  (ESC)  features freely while typing commands.  A question

mark typed at any point in a command displays the options  available  at

that  point;  typing  an  ESC  character  causes  the current keyword or

filename to be completed (or default value to be supplied), and a "guide

word" in parentheses to be typed, prompting you for the next field.   If

DECSYSTEM-20 KERMIT                                              Page 52

you have not typed sufficient characters to uniquely specify the keyword

or  filename  (or  if  there  is  no  default value) then a beep will be

sounded and you may continue typing.

Command keywords may be abbreviated to their shortest prefix  that  sets

them apart from any other keyword valid in that field.

If  you have a file called KERMIT.INI in your login directory, KERMIT-20

will execute an automatic TAKE  command  on  it  upon  initial  startup.

KERMIT.INI  may  contain  any  KERMIT-20 commands, for instance SET com-

mands, or DEFINEs for SET macros to configure KERMIT-20 to various  sys-

tems or communications media.

KERMIT-20  provides  most of the commands possible for an "ideal" KERMIT

program, as described in the main part of the KERMIT User  Guide.    The

following  sections  will  concentrate  on  system-dependent  aspects of

KERMIT-20.

KERMIT-20 disables terminal links, advice, and system messages in  order

to minimize interference with data transfer (and restores these to their

previous  value  upon  completion of a transfer).  However, certain mes-

sages cannot be disabled by KERMIT because they are issued  by  KERMIT's

superior  controlling  process,  the  TOPS-20  EXEC.  These include mail

notifications and alerts.  Before running KERMIT-20 to  transfer  files,

you should SET NO MAIL-WATCH and SET NO ALERT.

6.3. Remote and Local Operation

KERMIT-20  normally  runs in remote mode, with the user sitting at a PC.

But KERMIT-20 can also run in local mode.  Local operation of  KERMIT-20

is  useful if the DEC-20 has an autodialer, or a hardwired connection to

another computer.  When in local mode, file transfer takes place over an

assigned TTY line, and KERMIT-20 is free  to  update  your  screen  with

status information, and to listen to your keyboard for interrupt charac-

ters.

Local Operation of KERMIT-20:

DECSYSTEM-20 KERMIT                                              Page 53

DECSYSTEM-20

+---------------------------------------+

|                                       |

|   +--------------------+              |

|   | Your Job           |              |

|   |                    |              |

|   |   +------------+   | <--Commands  | Your Job's

|   |   | KERMIT-20  +---+--------------+-----------------O You

|   |   |            |   | Display--->  | Controlling TTY

|   |   |            |   |              |

|   |   |            |   |              |

|   |   |            |   | <--Packets   | Kermit's

|   |   |            +---+--------------+-----------------> Remote

|   |   +------------+   | Packets-->   | Assigned TTY      System

|   |                    |              |

|   +--------------------+              |

|                                       |

+---------------------------------------+

KERMIT-20 enters local mode when you issue a SET LINE n command, where n

is the octal TTY number of any line other than your own controlling ter-

minal.

6.4. Conditioning Your Job for Kermit

Kermit-20  does as much as it can to condition your line for file trans-

fer.  It saves all your terminal and link settings,  and  restores  them

after  use.   However, there are some sources of interference over which

Kermit-20 can have no control.  In particular, messages issued by  supe-

rior or parellel forks could become mingled with Kermit packets and slow

things  down  or  stop  them  entirely.    For this reason, before using

Kermit-20 for any extended period, you should:

- Type the Exec commands SET NO MAIL-WATCH and SET NO ALERTS

- Make sure you don't have any print or batch jobs pending  that

were submitted with the /NOTIFY option.

After running Kermit, you can restore your mail-watch and alerts.

6.5. KERMIT-20 Commands

This section describes the KERMIT-20 commands, in detail where they dif-

fer from the "ideal" KERMIT, briefly where they coincide.

THE SEND COMMAND

Syntax:

Sending a single file:

SEND nonwild-filespec1 (AS) [filespec2]

DECSYSTEM-20 KERMIT                                              Page 54

Sending multiple files:

SEND wild-filespec1 (INITIAL) [filespec2]

The  SEND command causes a file or file group to be sent from the DEC-20

to the other system.  There are two forms of the command,  depending  on

whether  filespec1 contains wildcard characters ("*" or "%").  KERMIT-20

automatically recognizes the two cases and issues the appropriate  guide

word, (AS) or (INITIAL), depending on the form of filespec1.

Sending a File Group

If  filespec1  contains wildcard characters then all matching files will

be sent,  in  alphabetical  order  (according  to  the  ASCII  collating

sequence)  by  name.  If a file can't be opened for read access, it will

be skipped.  The initial file in a wildcard group can be specified  with

the  optional  filespec2.  This allows a previously interrupted wildcard

transfer from where it left off, or it can be used to  skip  some  files

that would be transmitted first.

Sending a Single File

If  filespec1  does not contain any wildcard characters, then the single

file specified by filespec1 will be sent.  Optionally, filespec2 may  be

used  to specify the name under which the file will arrive at the target

system; filespec2 is not parsed or validated in any  way  by  KERMIT-20,

but   lower   case  letters  are  raised  to  upper  case,  and  leading

"whitespace" (blanks and tabs) are  discarded.    If  filespec2  is  not

13

specified, KERMIT-20 will send the file with its own name.

SEND Command General Operation

Files will be sent with their DEC-20 filename and filetype (for instance

FOO.BAR,   no  device  or  directory  field,  no  generation  number  or

attributes).  If you expect to be  sending  files  whose  names  contain

characters  that would be illegal in filenames on the target system, and

you know that the KERMIT on the target system does not have the  ability

to  convert  incoming  filenames,  you  can  issue  the  SET FILE NAMING

NORMAL-FORM command to have KERMIT-20 replace suspect characters by X's.

Each file will be sent according to its bytesize and byte count from the

directory unless you specify otherwise using SET FILE BYTESIZE,  or  un-

less  the  file  has an "ITS Binary" header.  If the bytesize is 8, then

four 8-bit bytes will be sent from each DEC-20 36-bit word, and the  low

order four bits will be skipped.  If other than 8, then five 7-bit bytes

will  be  sent from each word, with the 8th bit of the 5th character set

_______________

13

Control-V's, which are used to quote otherwise illegal characters in

DEC-20 file specifications, are stripped.

DECSYSTEM-20 KERMIT                                              Page 55

14

to the value of the remaining bit ("bit 35") from the word.

If  communication  line parity is being used (see SET PARITY), KERMIT-20

will request that the other KERMIT accept a special kind of prefix nota-

tion for binary files.  This is an advanced feature, and not all KERMITs

have it; if the other KERMIT does not agree to use this feature,  binary

files cannot be sent correctly.  This includes executable programs (like

DEC-20  .EXE  files,  CP/M .COM files), relocatable object modules (.REL

files), as well as text files with line sequence numbers.

KERMIT-20 will also ask the other KERMIT whether it can handle a special

prefix encoding for repeated characters.  If it  can,  then  files  with

long  strings  of  repeated  characters  will  be  transmitted  very ef-

ficiently.  Columnar data, highly indented text, and  binary  files  are

the major beneficiaries of this technique.

If  you're  running KERMIT-20 locally, for instance dialing out from the

DEC-20 to another system using an autodialer, you  should  have  already

run  KERMIT on the remote system and issued either a RECEIVE or a SERVER

command.  Once you give KERMIT-20 the SEND command,  the  name  of  each

file will be displayed on your screen as the transfer begins; a "." will

be  displayed  for  every 5 data packets sucessfully sent, and a "%" for

every retransmission or timeout that occurs (you may  also  elect  other

typeout  options  with  the SET DEBUG command).  If the file is success-

fully transferred, you will see "[OK]", otherwise there will be an error

message.  When the specified operation is  complete,  the  program  will

sound  a beep.  If you see many "%" characters, you are probably suffer-

ing from a noisy connection.  You  may  be  able  to  cut  down  on  the

retransmissions  by  using SET SEND PACKET-LENGTH to decrease the packet

length; this will reduce the probability that a  given  packet  will  be

corrupted  by  noise,  and reduce the time required to retransmit a cor-

rupted packet.

During local operation, you can type Control-A at any point  during  the

transfer  to  get a brief status report.  You may also type Control-X or

Control-Z to interrupt the current file or file group.

THE RECEIVE COMMAND

Syntax:  RECEIVE [filespec]

The RECEIVE command tells KERMIT-20 to receive a file or file group from

the other system.  If only one file is being received, you  may  include

the  optional  filespec  as  the  name to store the incoming file under;

otherwise, the name is taken from the incoming file header.  Even if the

_______________

14

This is the same method used by the DEC-20 to encode 36-bit data  on

"ANSI-ASCII"  tapes.    It allows not only DEC-20 binary files, but also

the line-sequence-numbered files produced by EDIT, SOS, or  OTTO,  which

use  bit  35  to  distinguish  line  numbers  from  text, to be sent and

retrieved correctly.

DECSYSTEM-20 KERMIT                                              Page 56

name  in  the  header  is  not a legal TOPS-20 file name, KERMIT-20 will

store it under that name, in which case you can refer to it  later  only

by quoting each illegal character (spaces, control characters, etc) with

Control-V.    If  for  some reason an incoming filename simply cannot be

converted   to   legal   form,   the   file    will    be    saved    as

-UNTRANSLATABLE-FILENAME-.KERMIT (new generation).  You may also use SET

FILE NAMING NORMAL-FORM to have KERMIT-20 choose more conventional names

for incoming files.

If  an  incoming  file  has the same name as an existing file, KERMIT-20

just creates a new generation of the same name and  type,  for  instance

FOO.BAR.3,  FOO.BAR.4.    The  oldest  generation  will be automatically

deleted, but you can still UNDELETE it.

Incoming files will all be stored with the  prevailing  bytesize,  7  by

default,  which  is  appropriate  for  text  files.    If you are asking

KERMIT-20 to receive binary files from a microcomputer  or  other  8-bit

system, you must first type SET FILE BYTESIZE 8.  Otherwise, the 8th bit

of each byte will be lost and the file will be useless when sent back to

the system of origin.

If  you  have  SET PARITY, then 8th-bit prefixing will be requested.  If

the other side cannot do this, binary files cannot be  transferred  cor-

rectly.    In all cases, KERMIT-20 will request the other KERMIT to com-

press repeated characters; if the other side can do this (not  all  KER-

MITs  know  how)  there may be a significant improvement in transmission

speed.

If an incoming file does not arrive in its entirety, KERMIT-20 will nor-

mally discard it; it will not appear in your directory.  You may  change

this behavior by using the command SET INCOMPLETE KEEP, which will cause

as much of the file as arrived to be saved in your directory.

If  you  are running KERMIT-20 locally, you should already have issued a

15

SEND command   to the remote KERMIT, and then  escaped  back  to  DEC-20

Kermit.   As files arrive, their names will be displayed on your screen,

along with "." and "%" characters to indicate the  packet  traffic;  you

can type Control-A during the transfer for a brief status report.

If  a file arrives that you don't really want, you can attempt to cancel

it by typing Control-X; this sends a cancellation request to the  remote

Kermit.    If  the  remote  Kermit understands this request (not all im-

plementations of Kermit support this feature), it will comply; otherwise

it will continue to send.  If a file group is being sent,  you  can  re-

quest the entire group be cancelled by typing Control-Z.

_______________

15

not  SERVER  --  use  the GET command to receive files from a KERMIT

server.

DECSYSTEM-20 KERMIT                                              Page 57

THE GET COMMAND

Syntax: GET [remote-filespec]

The GET command requests a remote KERMIT server to send the file or file

group  specified by remote-filespec.  This command can be used only when

KERMIT-20 is local, with a KERMIT server on the other end  of  the  line

specified  by  SET LINE.  This means that you must have CONNECTed to the

other system, logged in, run KERMIT there, issued  the  SERVER  command,

and escaped back to the DEC-20.

The remote filespec is any string that can be a legal file specification

for  the  remote  system; it is not parsed or validated locally.  If you

need to include otherwise illegal characters such as  "!"  or  ";"  (the

normal  command  comment  delimeters), "?" (the command help character),

"@" (the indirect command file indicator), or  certain  control  charac-

ters,  then  you  should  precede  each  such  character by a Control-V.

Kermit-20 will discard these Control-V quoting prefixes  before  sending

the file specification to the remote host.

If  you  want to store the incoming file name with a different name than

the remote host sends it with, just type GET alone on a line;  Kermit-20

will  prompt  you  separately  for  the  source (remote) and destination

(local) file specification.  If more than one  file  arrives,  only  the

first  one  will be stored under the name given; the rest will be stored

under the names they are sent with.  Example:

Kermit-20>get

Remote Source File: profile exec a1

Local Destination File: profile.exec

As files arrive, their names will be displayed  on  your  screen,  along

with  "."  and "%" characters to indicate the packet traffic.  As in the

RECEIVE command, you may type Control-A to get a brief status report, ^X

to request that the current incoming file be cancelled,  ^Z  to  request

that the entire incoming batch be cancelled.

If  the  remote KERMIT is not capable of server functions, then you will

probably get an error message back from it like "Illegal  packet  type".

In this case, you must connect to the other Kermit, give a SEND command,

escape back, and give a RECEIVE command.

THE SERVER COMMAND

The  SERVER command puts a remote KERMIT-20 in "server mode", so that it

receives all further commands in packets from the  local  KERMIT.    The

KERMIT-20  server  is  capable  (as  of  this writing) of executing many

remote server commands, including SEND, GET, FINISH, BYE, REMOTE  DIREC-

TORY, REMOTE CWD, REMOTE SPACE, REMOTE DELETE, REMOTE TYPE, REMOTE HELP.

Any  nonstandard  parameters should be selected with SET commands before

putting KERMIT-20 into server mode, in  particular  the  file  bytesize.

The  DEC-20  Kermit  server  can  send  most files in the correct manner

automatically, by recognizing the DEC-20 file bytesize.  However, if you

need to ask the DEC-20 KERMIT server to receive  binary  files  from  an

DECSYSTEM-20 KERMIT                                              Page 58

8-bit-byte  system  (that is, from almost any system that's not a DEC-10

or DEC-20) you must issue the SET FILE BYTESIZE 8 command before putting

it into server mode, and then you must only  send  8-bit  binary  files.

You  cannot  send  a  mixture  of text files and 8-bit binary files to a

KERMIT-20 server.

COMMANDS FOR SERVERS

When running in local mode, KERMIT-20 allows you to give a wide range of

commands to a remote KERMIT server,  with  no  guarantee  the  that  the

remote  server can process them, since they are all optional features of

the protocol.  Commands for servers include the standard SEND, GET, BYE,

and FINISH commands, as well as the REMOTE command,  which  has  various

options.

Syntax: REMOTE command

Send the specified command to the remote server.  If the server does not

understand  the  command (all of these commands are optional features of

the KERMIT protocol), it will reply with a message like "Unknown  KERMIT

server command".  If does understand, it will send the results back, and

they will be displayed on the screen.  The REMOTE commands are:

CWD [directory] Change  Working  Directory.    If  no  directory name is

provided, the server will change to the default or  home

directory.    Otherwise,  you  will  be  prompted  for a

password, and the server will attempt to change  to  the

specified  directory.    The  password  is  entered on a

separate line, and does not echo as you type it.  If ac-

cess is not granted, the server will provide  a  message

to that effect.

DELETE filespec Delete  the  specified  file or files.  The names of the

files that are deleted will appear on your screen.

DIRECTORY [filespec]

The names  of  the  files  that  match  the  given  file

specification  will be displayed on your screen, perhaps

along with size and date information for each file.   If

no  file specification is given, all files from the cur-

rent directory will be listed.

HELP            Provide a list of the functions that are available  from

the server.

HOST [command]  Pass  the  given  command  to  the server's host command

processor, and display  the  resulting  output  on  your

screen.

SPACE           Provide  information  about  disk  usage  in the current

directory, such as the quota, the current  storage,  the

amount of remaining free space.

TYPE filespec   Display  the  contents  of  the  specified  file on your

screen.

DECSYSTEM-20 KERMIT                                              Page 59

THE LOCAL COMMAND

Syntax: LOCAL [command]

Execute the specified command on the local system -- on the DEC-20 where

KERMIT-20 is running.  These commands provide some local file management

capability without having to leave the KERMIT-20 program.

CWD [directory] Change  working  directory,  or,  in DEC-20 terminology,

CONNECT to the specified directory.

DELETE filespec Delete the specified file or files, but do  not  expunge

them.

DIRECTORY [filespec]

Provide a directory listing of the specified files.

RUN [filespec]  Attempts  to  run  the  specified file, which must be in

".EXE" format (.EXE is the default filetype), in an  in-

ferior  fork.    Control  returns  to KERMIT-20 when the

program terminates.  Once you have  used  this  command,

you  can  restart the same program by issuing a RUN com-

mand with no arguments.  If you  RUN  SYSTEM:EXEC,  then

you will be able to issue TOPS-20 commands without leav-

ing  KERMIT; you can get back to KERMIT from the EXEC by

typing the EXEC POP command.  You can also use  the  RUN

command  to  supply  new  functions to KERMIT by writing

little programs in the language of your choice, for  in-

stance  to  conduct a signon dialog with a remote system

when dialing out.

SPACE           Show how much space is used and remaining in the current

directory.

TYPE            Display the contents of the specified file or  files  at

your terminal.  This works like the DEC-20 TYPE command,

except  that  if  a  file has a bytesize of 8, KERMIT-20

will do 8-bit input from it rather than  7-bit.    Also,

the  DEC-20  Control-O command discards output only from

the file currently being displayed;  if  multiple  files

are  being  typed, then output will resume with the next

file.

The LOCAL commands may also be used without the "LOCAL" prefix.

THE CONNECT COMMAND

Syntax: CONNECT [number]

Establish a terminal connection to the system connected to the octal TTY

number specified here or in the most recent SET LINE command, using full

duplex echoing and no parity unless otherwise specified in previous  SET

commands.  Get back to KERMIT-20 by typing the escape character followed

by  the  letter  C. The  escape  character  is Control-Backslash (^\) by

default.  When you type the escape character,  several  single-character

DECSYSTEM-20 KERMIT                                              Page 60

commands are possible:

C   Close the connection and return to KERMIT-20.

S   Show status of the connection; equivalent to SHOW LINE.

P   Push  to a new Exec.  POP from the Exec to get back to the connec-

tion.

Q   If a session log is active, temporarily Quit logging.

R   Resume logging to the session log.

B   Send a simulated BREAK signal.

?   List all the possible single-character arguments.

^\ (or whatever you have set the escape character to be)

Typing the escape character twice sends one copy of it to the con-

nected host.

You can use the SET ESCAPE command to define a different escape  charac-

ter,  and SET PARITY, SET DUPLEX, SET HANDSHAKE, SET FLOW, and SET SPEED

to change those communication-line-oriented parameters.   In  order  for

the  simulated  BREAK signal to work, TOPS-20 must know the speed of the

terminal.  If it does not, you may use the SET SPEED command.

KERMIT-20 does not have any special autodialer interface.    It  assumes

that the connection has already been made and the line assigned.

THE HELP COMMAND

Syntax: HELP [topic [subtopic]]

Typing  HELP alone prints a brief summary of KERMIT-20 and its commands.

You can also type

HELP command

for any Kermit-20 command, e.g. "help send" or "help set parity" to  get

more detailed information about a specific command.  Type

HELP ?

to see a list of the available help commands.

THE TAKE COMMAND

Syntax: TAKE filespec

Execute  KERMIT-20  commands from the specified file.  The file may con-

tain contain any valid KERMIT-20 commands,  including  other  TAKE  com-

mands; command files may be nested up to a depth of 20.

DECSYSTEM-20 KERMIT                                              Page 61

THE EXIT AND QUIT COMMANDS

Syntax: EXIT

Exit  from  KERMIT-20.    You  can CONTINUE the program from the TOPS-20

Exec, provided you haven't run another program on top of it.    You  can

also exit from KERMIT-20 by typing one or more control-C's, even if it's

in  the  middle  of  transferring a file.  KERMIT-20 will always restore

your terminal to its original condition, and you will be  able  to  CON-

TINUE the program to get back to "KERMIT-20>" command level with current

settings intact.

QUIT is a synonym for EXIT.

THE SET COMMAND

Syntax: SET parameter [option [value]]

Establish  or  modify  various  parameters for file transfer or terminal

connection.  You can examine their values with the SHOW  command.    The

following parameters may be SET:

BREAK  Adjust the BREAK simulation parameter

BLOCK-CHECK  Packet transmission error detection method

DEBUGGING  Record or display state transitions or packets

DELAY  How long to wait before starting to send

DUPLEX  For terminal connection, FULL or HALF

ESCAPE  Character for terminal connection

FILE  For setting file parameters like byte size

FLOW-CONTROL  For enabling or disabling XON/XOFF flow control

HANDSHAKE  For turning around half duplex communication line

IBM  For communicating with an IBM mainframe

INCOMPLETE  What to do with an incomplete file

ITS-BINARY  For recognizing a special 8-bit binary file format

LINE  TTY line to use for file transfer or CONNECT

PARITY  Character parity to use

PROMPT  Change the program's command prompt

RECEIVE  Various parameters for receiving files

RETRY  How many times to retry a packet before giving up

SEND  Various parameters for sending files

SPEED  Baud rate of communication line

TVT-BINARY  For negotiating binary mode on ARPANET

The DEFINE command may be used to compose "macros" by combining SET com-

mands.   Those SET commands which differ from the "ideal" KERMIT are now

described in detail.

SET BREAK

Syntax: SET BREAK n Specify the number of nulls to be sent at 50 baud to

simulate a BREAK signal when connected to a remote host via SET LINE and

CONNECT.

DECSYSTEM-20 KERMIT                                              Page 62

SET DEBUG

Syntax: SET DEBUG options

Record  the  packet traffic, either on your terminal or in a file.  Some

reasons for doing this would be to debug a version of  KERMIT  that  you

are  working  on, to record a transaction in which an error occurred for

evidence when reporting bugs, or simply to vary the display you get when

running KERMIT-20 in local mode.  Options are:

STATES          Show  Kermit  state  transitions  and   packet   numbers

(brief).

PACKETS         Display each incoming and outgoing packet (lengthy).

OFF             Don't  display  or record debugging information (this is

the normal mode).  If debugging was in effect,  turn  it

off and close any log file.

The  debugging information is recorded in the file specified by the most

recent LOG DEBUGGING command.

SET ESCAPE

SET ESCAPE octal-number

Specify the control character you want to use to  "escape"  from  remote

connections back to KERMIT-20.  The default is 34 (Control-\).  The num-

ber is the octal value of the ASCII control character, 1 to 37 (or 177),

for  instance  2 is Control-B.  After you type the escape character, you

must follow it by a one of the  single-character  "arguments"  described

under the CONNECT command, above.

SET EXPUNGE

SET EXPUNGE ON or OFF

Tell  whether you want a DELETE command (either the LOCAL DELETE command

or a REMOTE DELETE command sent to a KERMIT-20 server) to expunge  files

as  it deletes them.  On the DEC-20, a deleted file continues to take up

space, and may be "undeleted" at a later time in the same session.    To

expunge  a  deleted  file means to remove it completely and irrevocably,

freeing its space for further use.  EXPUNGE is OFF by  default;  deleted

files are not automatically expunged.  SET EXPUNGE applies only to files

that  are deleted explicitly by KERMIT-20, and not to files that are im-

plicitly deleted when new generations of existing files are created.

SET FILE

Syntax: SET FILE parameter keyword

Establish file-related parameters:

DECSYSTEM-20 KERMIT                                              Page 63

BYTESIZE keyword or number

Byte  size  for DEC-20 file input/output.  The choices are SEVEN

(7), EIGHT (8), and AUTO.

SEVEN   (or 7) Always store or retrieve  five  7-bit  bytes  per

word.  When sending a file, ignore the file bytesize and

do  7-bit input from the file.  There would be no reason

to use this option except to explicitly force  an  8-bit

file to be treated as a 7-bit file.

EIGHT   (or  8)  Always  store  or retrieve four 8-bit bytes per

word.  When sending a file, ignore the file bytesize and

do 8-bit input from the file.  This command is necessary

when receiving binary  files  from  8-bit-byte  systems,

such as most microcomputers.

AUTO    Equivalent to SEVEN for incoming files, and for outgoing

files means to use EIGHT if the DEC-20 file bytesize (as

shown  by  the  Exec  VDIR  command) is 8, otherwise use

SEVEN.  The default is AUTO.

The DEC-20 can send any mixture of file types in the correct way

automatically, but you must set the file bytesize to 8  for  any

incoming 8-bit binary files, and to AUTO (i.e. 7) for any incom-

ing text files or DEC-20 binary files.

NAMING UNTRANSLATED or NORMAL-FORM

If  NORMAL-FORM  the names of incoming or outgoing files will be

converted to contain only uppercase letters, digits, and at most

one period; any other characters will be translated to "X".   If

UNTRANSLATED,  filenames  will  be sent and used literally.  UN-

TRANSLATED is the default.

SET IBM

Syntax: SET IBM ON or OFF

SET IBM is really a predefined SET macro rather than a  "hardwired"  SET

command;  it  can be redefined or undefined (see DEFINE); as distributed

from Columbia, KERMIT-20 defines IBM to be "parity mark, handshake  XON,

duplex half".

SET  IBM  should be used when running KERMIT-20 in local mode, connected

to an IBM or similar mainframe.  If  you  have  redefined  the  SET  IBM

macro, then your parameters will be used instead.

SET ITS-BINARY

Syntax: SET ITS-BINARY ON or OFF

Specify whether ITS-Binary file headers are to be recognized or ignored.

By default, they are recognized.  ITS binary format is a way (devised at

MIT)  of  storing foreign 8-bit binary data on a 36-bit machine to allow

automatic recognition of these files when sending  them  out  again,  so

DECSYSTEM-20 KERMIT                                              Page 64

that  you  don't  have  to depend on the file byte size, or to issue ex-

plicit SET FILE BYTESIZE commands to KERMIT.

An  ITS  format  binary  file  contains  the  sixbit  characters  "DSK8"

left-adjusted  in  the  first  36-bit  word.   If ITS-BINARY is ON, then

KERMIT-20 will send any file starting  with  this  "header  word"  using

8-bit  input  from the file even if the file bytesize is not 8, and will

not send the header word itself.  KERMIT-20 will also store any incoming

file that begins with that header word using 8-bit bytesize, again  dis-

carding  the  header word itself.  If ITS-BINARY is OFF, then the header

word, if any, will be sent or kept, and i/o will  be  according  to  the

setting of FILE BYTESIZE.

This  facility  is provided for compatibility with the file formats used

on certain public-access CP/M libraries.

SET LINE

Syntax: SET LINE [octal-number]

Specify the octal TTY number to use for file transfer or  CONNECT.    If

you  issue  this command, you will be running KERMIT-20 locally, and you

must log in to the remote system and run Kermit on that side in order to

transfer a file.  If you don't issue this command, KERMIT-20 assumes  it

is  running  remotely, and does file transfer over its job's controlling

terminal line.  You can also select the line  directly  in  the  CONNECT

command; the command

CONNECT 12

is equivalent to

SET LINE 12

CONNECT

If you type SET LINE with no number argument, you will deassign any pre-

vious assigned line and revert to remote mode.

The  SHOW LINE command will display the currently selected communication

line and its charactistics, including parity,  duplex,  handshake,  flow

control,  the  speed  if  known,  whether carrier is present (if it is a

modem-controlled line), and whether KERMIT-20  is  in  local  or  remote

mode.

SET RECEIVE

In  addition to the full complement of SET RECEIVE commands described in

the main part of the manual, you may also SET RECEIVE SERVER-TIMEOUT  to

a  value between 0 and 94.  This specifies the number of seconds between

timeouts during server command wait, 0 specifies that no timeouts should

occur during server command wait.  When a KERMIT server  times  out,  it

sends  a NAK packet.  Some systems cannot clear piled-up NAKs from their

input buffers; if you're using such  a  system  to  communicate  with  a

KERMIT-20  server, and you expect to be leaving the server idle for long

DECSYSTEM-20 KERMIT                                              Page 65

periods of time, you should use this command to turn off server command-

wait timeouts.

SET SPEED

Syntax: SET SPEED n

Set  the  baud  rate  of  the currently selected communication to n, the

decimal baud rate, for instance 300, 1200,  4800.    When  operating  in

local mode, it may be necessary to issue this command in order to enable

BREAK simulation.

SET TVT-BINARY

Syntax: SET TVT-BINARY ON or OFF

Only  for  users running KERMIT-20 on an ARPANET DEC-20, signed on to an

ARPANET virtual terminal (TVT) from another host or through  an  ARPANET

TAC.   SET TVT ON causes KERMIT-20 to negotiate binary mode (8-bit) com-

munication with the ARPANET during file transfer.  Without this command,

file transfer through a TVT would not work in most cases.

TVT-BINARY is OFF by default.  If you normally use KERMIT-20 through the

ARPAnet, you can put the command SET TVT-BINARY ON into your  KERMIT.INI

file.

CAUTION:  This  facility  requires  certain  features  in  the Release 5

TOPS-20 ARPANET monitor, which may  not  be  present  in  releases  dis-

tributed by DEC.  See the KERMIT-20 source code for details.

THE DEFINE COMMAND

Syntax: DEFINE macroname [set-option [, set-option [...]]]

The  DEFINE  command  is available in KERMIT-20 for building "macros" of

SET commands.  The macro name can be any keyword-style character string,

and the set options are anything you would type after SET in a SET  com-

mand;  several  set options may be strung together, separated by commas.

Example:

define notimeout send timeout 0, receive timeout 0

Macro definitions may not include macro names.  You can  list  all  your

macros and their definitions with the SHOW MACROS command.  You can list

a particular macro definition with HELP SET macroname.

DECSYSTEM-20 KERMIT                                              Page 66

THE SHOW COMMAND

Syntax: SHOW [option]

The SHOW command displays various information:

DAYTIME         Current date, time, phase of moon.

DEBUGGING       Debugging mode in effect, if any.

FILE-INFO       Byte  size for DEC-20 file i/o, incomplete file disposi-

tion.

LINE            TTY line, parity, duplex, flow control,  handshake,  es-

cape character, speed (if known), and session loggin in-

formation.  Note that before release 6.0 of TOPS-20, the

DEC-20 does not keep a record of the actual baud rate of

a modem-controlled or "remote" TTY line.

MACROS          Definitions for SET macros.

PACKET-INFO     For  incoming and outbound packets.  Items under RECEIVE

column show parameters for packets KERMIT-20 expects  to

receive,  under SEND shows parameters for outgoing pack-

ets.

TIMING-INFO     Delays, retries, server NAK intervals.

VERSION         Program version of KERMIT-20.  This  is  also  displayed

when KERMIT-20 is initially started.

ALL             (default) All of the above.

THE STATISTICS COMMAND

Give  statistics  about  the  most  recent file transfer.  For instance,

here's what KERMIT-20 displayed after transmitting a short binary  file,

using repeated-character compression:

Maximum number of characters in packet:  80 received; 80 sent

Number of characters transmitted in 2 seconds

Sent:      34           Overhead:       34

Received:  107          Overhead:       -408

Total received:  141          Overhead:       -374

Total characters transmitted per second:       70

Effective data rate:   2570 baud

Efficiency:            214.1667 per cent

Interpacket pause in effect: 0 sec

Timeouts: 0

NAKs:     0

Note that the data compression allowed the effective baud rate to exceed

the  actual speed of the communication line, which in this case happened

DECSYSTEM-20 KERMIT                                              Page 67

to  be  1200  baud.  The efficiency is displayed only if the actual baud

rate is known.

THE LOG COMMAND

Syntax: LOG [option [filespec]]

Log the specified option to the specified file:

SESSION         During CONNECT, log all characters that  appear  on  the

screen  to  the  specified file.  The session log can be

temporarily turned off  during  the  remote  session  by

typing  the  escape  character  followed  by Q (for Quit

logging), and turned  on  again  by  typing  the  escape

character  followed  by R (for Resume logging).  Default

log is SESSION.LOG in the current directory.

TRANSACTIONS    During file transfer, log the  progress  of  each  file.

The DEC-20 transaction log file looks like this:

KERMIT-20 Transaction Log File, Monday 27-Feb-1984

18:40:13: Opened Log: PS:<TIMREK>SAMPLE.LOG.1

18:40:31: -- Send Begins --

8th bit prefixing: Off

Block check type: 1

18:40:31: Opened File: PS:<SY.FDC>LOGIN.CMD.6

Sending As "LOGIN.CMD"

Sent: 547 7-bit bytes

18:40:34: Closed PS:<SY.FDC>LOGIN.CMD.6

18:40:34: Send Complete

18:40:50: -- Receive Begins --

8th bit prefixing: Off

Block check type: 1

18:40:50: Opened: PS:<TIMREK>AUTOEXEC.BAT.1

Written: 186 7-bit bytes

18:40:51: Closed: PS:<TIMREK>AUTOEXEC.BAT.1

18:40:56: Closed Transaction Log

Transaction  logging  is  recommended  for  long  or un-

attended file transfers, so that you don't have to watch

the screen.  The log may be inspected after the transfer

is complete to see what files were transferred and  what

errors    may   have   occurred.      Default   log   is

TRANSACTION.LOG in the current directory.

DEBUGGING       Log STATES or PACKETS, as specified in the  most  recent

SET  DEBUGGING  command,  to the specified file.  If log

file  not  specified,  then  use  TTY   if   local,   or

DEBUGGING.LOG in the current directory if remote.  If no

SET  DEBUGGING command was previously issued, log STATES

to the specified file.    Also  allow  specification  of

bytesize  for  the  log  file, 7 (normal, default), or 8

DECSYSTEM-20 KERMIT                                              Page 68

(for  debugging  binary transfers when the parity bit is

being used for data), for instance

LOG DEBUGGING BINARY.LOG 8

A 7-bit log file can be typed, printed, or examined with

a text editor or searching program.  An 8-bit  log  file

can  only be examined with a system utility like FILDDT.

When logging packets,  each  packet  is  preceded  by  a

timestamp,  the  current timeout interval (preceded by a

slash), and "R:" or "S:" to indicate data being received

and sent, respectively.  Packet format is  described  in

the KERMIT Protocol Manual.

SESSION  is the default option.  Thus the command "LOG" alone will cause

CONNECT sessions to be logged in SESSION.LOG in the  current  directory.

Any  log  files  are  closed  when you EXIT or QUIT from KERMIT, and are

reactivated if you CONTINUE the program.  You may explicitly close a log

file and terminate logging with the CLOSE command.

THE CLOSE COMMAND

Syntax: CLOSE [option]

Close the specified log file, SESSION, TRANSACTION,  or  DEBUGGING,  and

terminate logging.

6.6. Examples

Here  are  a  few examples of the use of KERMIT-20.  Text entered by the

user is underlined.

Remote Operation

The following example shows use of KERMIT-20 as a server from an IBM PC.

In this example, the user runs KERMIT on the PC, connects to the DEC-20,

and starts KERMIT-20 in server mode.  From that point on, the user  need

never  connect  to  the  DEC-20 again.  In this example, the user gets a

file from the DEC-20, works on it locally at the PC, and then sends  the

results  back  to  the DEC-20.  Note that the user can leave and restart

KERMIT on the PC as often as desired.

A>kermit

Kermit-86>connect

@

@kermit

TOPS-20 KERMIT version 4.1(236)

Kermit-20>server

DECSYSTEM-20 KERMIT                                              Page 69

Kermit Server running on DEC-20 host.  Please type your escape

sequence to return to your local machine.  Shut down the server by

typing the Kermit BYE command on your local machine.

^[C

Kermit-86>get foo.txt

The transfer takes place.

Kermit-86>exit

A>

A>edit foo.txt ; (or whatever...)

A>

A>kermit

Kermit-86>send foo.txt

The transfer takes place.

Kermit-86>bye

A>

The  next  example  shows the special procedure you would have to use in

order to send a mixture of text and binary files from a PC (or an 8-bit-

byte system) to the DEC-20.  Note that in  this  case,  it's  more  con-

venient to avoid server mode.

Kermit-86>connect

@

@kermit

TOPS-20 KERMIT version 4.1(236)

Kermit-20>receive

^]C

Kermit-86>send *.txt

Textual files are sent.

Kermit-86>connect

Kermit-20>set file bytesize 8

Kermit-20>receive

^]C

Kermit-86>send *.exe

Binary files are sent.

Kermit-86>connect

Kermit-20>exit

@logout

^]C

Kermit-86>exit

A>

DECSYSTEM-20 KERMIT                                              Page 70

Local Operation

In  this example, a program DIAL is used to direct an autodialer to call

another computer (a DECSYSTEM-10); once the  connection  is  made,  DIAL

starts  KERMIT with an implicit CONNECT command for the appropriate com-

munication line.  DIAL is not part of KERMIT;  if  your  system  has  an

autodialer, there will be some site-specific procedure for using it.

@dial

Dial>dial stevens

STEVENS, 1-(201) 555-1234, baud:1200

[confirm]

Dialing your number, please hold...

Your party is waiting on TTY11:.

@

@kermit

TOPS-20 KERMIT version 4.1(236)

Kermit-20>connect 11

[KERMIT-20: Connecting over TTY11:, type <CTRL-\>C to return]

CONNECTING TO HOST SYSTEM.

Stevens T/S 7.01A(10) 20:20:04 TTY41 system 1282

Connected to Node DN87S1(101) Line # 57

Please LOGIN or ATTACH

.log 10,35

JOB 51 Stevens T/S 7.01A(10) TTY41

Password:

20:20   15-Dec-83       Thur

.r new:kermit

TOPS-10 KERMIT version 2(106)

Kermit-10>server

[Kermit Server running on the DEC host.  Please type your escape

sequence to return to your local machine.  Shut down the server by

typing the Kermit BYE command on your local machine.]

^YC

[KERMIT-20: Connection Closed.  Back at DEC-20.]

DECSYSTEM-20 KERMIT                                              Page 71

Kermit-20>set file bytesize 8

Kermit-20>get setdtr.cmd

^A for status report, ^X to cancel file, ^Z to cancel batch.

SETDTR.CMD.7 ^A

Receiving SETDTR.CMD.7, file bytesize 8

(repeated character compression)

At page 1

Files: 0, packets: 1, chars: 66

NAKs: 0, timeouts: 0

.[OK]

Kermit-20>bye

Job 51  User F DA CRUZ [10,35]

Logged-off TTY41  at 20:22:58  on 15-Dec-83

Runtime: 0:00:01, KCS:33, Connect time: 0:02:39

Disk Reads:72, Writes:4, Blocks saved:160

....

Hangup? y

Click. Call duration was 193 seconds to area 201.

Dial>exit

Note the use of Control-A to get a status report during the transfer.

6.7. Installation

KERMIT-20 is built from a single MACRO-20 source file, 20KERMIT.MAC.  It

requires the standard DEC-distributed tools MONSYM, MACSYM, and CMD; the

following files should be in SYS: -- MONSYM.UNV, MACSYM.UNV, MACREL.REL,

CMD.UNV,  and CMD.REL. The program should work on all TOPS-20 systems as

distributed, but many customizations are possible.  The site manager may

wish to change various default parameters on a site-wide basis; this may

be done simply by changing the definitions of the desired symbols, under

"subttl Definitions", and reassembling.

The most notable site dependency is the definition of  "SET  IBM".    As

distributed from Columbia, KERMIT-20 defines "SET IBM" in a built-in SET

macro  definition  as  "parity  mark, duplex half, handshake xon".  This

definition may be found at MACTAB+1, near the end  of  the  impure  data

section.  It may be changed or deleted, and the program reassembled.

Sites that do not have ARPANET may wish to delete the TVT-BINARY entries

from SET command tables, SETABL and SETHLP.

VAX/VMS KERMIT                                                   Page 72

7. VAX/VMS KERMIT

Authors:    Bob McQueen, Nick Bush, Stevens Institute of Technology

Language:   Bliss-32, Common Bliss

Version:    2.0

Date:       November 1983

KERMIT  for  the  Digital Equipment Corporation VAX/VMS system is called

"KERMIT-32" since the VAX is DEC's 32-bit line of computers.   KERMIT-32

can be run from SYS$SYSTEM:.  It will prompt for input from SYS$COMMAND:

.

Kermit-32  can  be run in either local or remote modes.  In remote mode,

transfers take place over the  controlling  terminal  line.    Ususally,

Kermit-32 is used in remote mode as a "server", meaning that it will ac-

cept commands from the other Kermit.  In local mode, Kermit-32 will per-

form transfers over a terminal line other than the controlling terminal.

In  local  mode,  Kermit-32  is capable of giving commands to a "server"

Kermit.  Note that in order to use Kermit-32 in local mode, the  protec-

tion  code for the terminal to be used must allow the user access.  This

is set by the system manager.  Kermit-32 is put into local mode by using

the SET LINE TTcnn: command.

Currently, VMS Kermit does not allocate the terminal line you are  using

for  CONNECT  or  transfers.   Therefore, when going between CONNECT and

SEND or RECEIVE, VMS hangs up the phone for you.  This is easily  solved

by  using  the DCL ALLOCATE command to allocate the terminal line before

entering Kermit.

VMS KERMIT implements a large subset of "ideal" KERMIT, for both  remote

and local operation.

Here is a summary of the commands of KERMIT-32:

CONNECT [dev:]

The  CONNECT command will allow you to connect as a virtual ter-

minal over the line that was specified by the SET LINE  command,

or  to the terminal line specified in the command.  The terminal

line must be one which is accessible to the users process.  This

means that the applicable protection code for the terminal  must

have  been  set  to allow your process to access it (done by the

system manager).  The format of the CONNECT command is:

Kermit-32>CONNECT

or

Kermit-32>CONNECT TTcn:

where TTcn: is the terminal line name to be used.

HELP [keyword [keyword...]]

Give VMS-style help on KERMIT commands.

EXIT, QUIT

Exit from Kermit-32.

VAX/VMS KERMIT                                                   Page 73

RECEIVE The  RECEIVE  command  is used to put Kermit-32 into remote mode

waiting for a single file transfer transaction,  or  to  have  a

local  Kermit-32  request  a file from the remote Kermit.  If no

file specification is given, Kermit-32  will  wait  for  a  file

transfer initialization sequence from the other Kermit.  This is

most  useful  if  the other Kermit does not support local server

commands.  In order  for  a  file  specification  to  be  given,

Kermit-32  must  be  running  as a local Kermit (i.e. a SET LINE

command must have been done).  Kermit-32 will then  request  the

other  Kermit (which must be running in server mode) to transfer

the specified file (or set of files) to  Kermit-32.    The  file

specification  must  be in the format of the system on which the

server Kermit is running.  The format of the command is:

Kermit-32>RECEIVE

or

Kermit-32>RECEIVE file-specification

where "file-specification" is any valid  file  specification  on

the system on which the server Kermit is running.

GET filespec

This  command  is identical to the RECEIVE filespec command.  It

is now the preferred command to cause  the  other  Kermit  (when

running in server mode) to transmit a file to Kermit-32.

BYE     This  command  will cause Kermit-32 (when in local mode) to tell

the other Kermit (which should be in server mode) to  exit  from

Kermit and, if applicable, terminate its job (or process, etc.).

When  Kermit-32  receives the acknowledgement that this is being

done, it will exit to VMS.

FINISH  This command will cause Kermit-32 (when in local mode)  to  tell

the  other  Kermit (which should be in server mode) to exit from

Kermit.  After receiving the acknowledgement that this is  being

done, Kermit-32 will prompt for another command.

LOGOUT  This  command  will cause Kermit-32 (when in local mode) to tell

the other Kermit (which should be in server mode) to  exit  from

Kermit and, if applicable, terminate its job (or process, etc.).

When  Kermit-32  receives the acknowledgement that this is being

done, it will prompt for another command.

SEND    The SEND command will allow you to send a file(s) to  the  other

Kermit.    The SEND command will allow file wild card processing

as is found in VMS.  If Kermit-32 is running in remote mode, the

file will be sent on the controlling terminal line after waiting

the number of seconds specified by the SET DELAY command.   This

gives the user time to escape back to the other Kermit and issue

a  receive  command.  If Kermit-32 is running in local mode, the

file will be sent immediately on the terminal line specified  by

the SET LINE command.  The command format is:

Kermit-32>SEND file-specification

VAX/VMS KERMIT                                                   Page 74

Where "file-specification" is any normal VAX/VMS file specifica-

tion.

SERVER  This  command  will  cause  Kermit-32 to enter server mode.  The

other Kermit can then issue server commands to send and  receive

files  without  having  to  give  SEND  or  RECEIVE  commands to

Kermit-32.  Kermit-32 may be put into SERVER mode while  running

as  either  a  remote  Kermit (transmitting over the controlling

terminal line), or as a local Kermit (transmitting over  a  ter-

minal  specified  by a SET LINE command).  Note that in order to

correctly receive binary files  while  in  SERVER  mode,  a  SET

FILETYPE  BINARY  must  be done first.  At this time there is no

way for Kermit-32 to determine whether an incoming file is ASCII

or binary.  The format of the command is:

Kermit-32>SERVER

STATUS  The current status of Kermit-32 will be  displayed.    This  in-

cludes the number of characters that have been sent and received

from the remote Kermit.  Also included is an estimate of the ef-

fective baud rate of

THE SET COMMAND

The SET command is used to set various parameters in Kermit.

BLOCK_CHECK_TYPEkeyword

where keyword is one of:

1_CHARACTER_CHECKSUM  or ONE_CHARACTER_CHECKSUM

2_CHARACTER_CHECKSUM  or TWO_CHARACTER_CHECKSUM

3_CHARACTER_CRC_CCITT or THREE_CHARACTER_CRC_CCITT

DEBUGGING       The  SET DEBUGGING command is used to set the debug type

out on the user's terminal.  The command will accept ei-

ther the keywords ON or OFF.    Kermit-32  can  only  do

debugging  type  out when running as a local Kermit (SET

LINE command done).  This is because the debugging  type

out  would  interfere  with the file transfer if it were

sent to the controlling terminal line in remote mode.

Kermit-32>SET DEBUGGING state

Where state is either 'ON' or 'OFF'.

DELAY           The DELAY parameter is the number of seconds to wait be-

fore sending data after a SEND command is given.    This

is  used when Kermit-32 is running in remote mode to al-

low the user time to escape back to the other Kermit and

give a RECEIVE command.

Kermit-32>SET DELAY number-of-seconds

Where number of  seconds  is  the  (decimal)  number  of

seconds to wait before sending data.

VAX/VMS KERMIT                                                   Page 75

ESCAPE          This  command will set the escape character for the CON-

NECT processing.  The command will take the octal  value

of  the  character to use as the escape character.  This

is the character which  is  used  to  "escape"  back  to

Kermit-32  after using the CONNECT command.  It defaults

to CTRL-] (octal 35).  It is usually a good idea to  set

this  character  to  something  which is not used (or at

least not used very much) on the system being  to  which

Kermit-32 is CONNECTing.

Kermit-32>SET ESCAPE octal-character-value

where  octal-character-value  is  the ASCII value of the

character to use as the escape character (in octal).

FILE_TYPE       This command will set  the  file  type  that  Kermit  is

receiving.    A  file  type  of  ASCII should be used to

receive text files which are to be used as text files on

the VMS system.  The file type BINARY should be used for

binary files, such as CP/M .COM files, which need to  be

kept  in  a  format  that allows the file to be returned

without any changes.

Kermit-32>SET FILE_TYPE type

where type is one of:

ASCII           File type ASCII is for text files.

BINARY          File type BINARY is for non-text  files.

Note   that   binary   files  which  are

generated on  a  VMS  system  cannot  be

transferred  to another VMS system with-

out losing file attributes.  This  means

that  (for example), an .EXE file cannot

be transmitted with  Kermit-32.    (This

problem  should  be resolved in a future

version of Kermit).

IBM_MODE        For communicating with IBM mainframes; sets parity MARK,

handshake XON, and local echo during CONNECT.

Kermit-32>SET IBM_MODE keyword

where keyword is either ON or OFF.

INCOMPLETE_FILE_DISPOSITION

The SET INCOMPLETE_FILE_DISPOSITION allows the  user  to

determine  what  is  done  with  a file that is not com-

pletely received.

Kermit-32>SET INCOMPLETE_FILE_DISPOSITION keyword

where keyword is either DISCARD or KEEP.

LINE            This will set the terminal line that you are using.  The

VAX/VMS KERMIT                                                   Page 76

terminal  line  must  be  one which is accessible to the

user's process.  This means that the applicable  protec-

tion  code  for the terminal must have been set to allow

your process to access it (done by the system  manager).

You should also ALLOCATE the line from DCL before giving

this command.

Kermit-32>SET LINE device:

The device must be a terminal line (e.g. TTA0:).

LOCAL_ECHO      The  SET LOCAL_ECHO command specifies whether characters

should be echoed locally when CONNECTing to another sys-

tem.

Kermit-32>SET LOCAL_ECHO keyword

where keyword is either ON (local  echo)  or  OFF  (full

duplex, normal case).

MESSAGE         This  command sets the type of typeout Kermit-32 will do

during transfers in local mode.  Kermit-32 can type  out

the  file  specification  being  transferred, the packet

numbers being sent an received, both or  neither.    The

default  is  to  type file specifications but not packet

numbers.

Kermit-32>SET MESSAGE type keyword

Where type is either FILE or PACKET, and keyword is  ei-

ther ON or OFF.

PARITY          This command determines the type of parity to use on the

transmission  line.   Kermit-32 normally uses characters

which consist of eight data bits with no parity bit.

Kermit-32>SET PARITY keyword

where keyword is NONE (default), MARK, SPACE,  EVEN,  or

ODD.    If  any  parity  other  than  NONE is specified,

8th-bit-prefixing will be requested for transmission  of

binary files.

RETRY           This  command sets the maximum number of times Kermit-32

should try to send a specific packet.

Kermit-32>SET RETRY keyword number

where keyword is either INITIAL_CONNECTION (for  initial

connection  packet)  or  PACKET (for all other packets),

and number is the decimal number of retries to attempt.

RECEIVE         It is possible to set various parameters associated with

the receiving of the data from the remote Kermit.    SET

RECEIVE  will  enable  you  to  set  the various receive

parameters.

VAX/VMS KERMIT                                                   Page 77

PACKET_LENGTH   This will set the receive packet length,

between 10 and 96.  The default value is

80.

Kermit-32>SET REC PACKET_LEN 60

PADDING         This command will set the number of pad-

ding characters that will be sent to the

other Kermit.  The default value is 0.

Kermit-32>SET RECEIVE PADDING n

Where n is the decimal number of padding

characters to use.

PADCHAR         This  parameter is the padding character

that is sent to the remote Kermit.   The

parameter must be an octal number in the

range  of  0  to  37  or 177.  All other

values are illegal.  The  default  value

is 0 (an ASCII NUL).

Kermit-32>SET RECEIVE PADCHAR nnn

where  nnn  is  the  ASCII  value of the

character to be used as a pad  character

(in octal).

START_OF_PACKET This  command  will  set  the  start  of

packet character for Kermit.  The  start

of packet character must be in the range

of 0 to 36 octal. The default value is 1

(ASCII  SOH, CTRL-A).  This value should

only be changed if absolutely necessary.

It must be set the same in both Kermits.

Kermit-32>SET REC START_OF_PACK 3

TIMEOUT         This will set the number of seconds  be-

fore Kermit-32 will time out the attempt

to  receive a message.  This time out is

used to handle transmission errors which

totally lose a  message.    The  default

value is 15 seconds.

Kermit-32>SET RECEIVE TIMEOUT n

where n is the number of seconds to wait

for a message (in decimal).

END_OF_LINE     This  will set the end of line character

the Kermit-32 expects  to  receive  from

the  remote Kermit.  This is the charac-

ter which  terminates  a  packet.    The

default  value is 15 (ASCII CR, CTRL-M).

VAX/VMS KERMIT                                                   Page 78

Kermit-32>SET REC END_OF_LINE nnn

where  nnn  is  the  ASCII  value of the

character to use for  the  end  of  line

character (in octal).

QUOTE           This will set the quoting character that

Kermit-32  will  expect on incoming mes-

sages.  This is the  character  used  to

quote  control  characters.  The default

value is 43 (ASCII "#").

Kermit-32>SET RECEIVE QUOTE nnn

where nnn is  the  ASCII  value  of  the

quoting character (in octal).

SEND            It is possible to set various parameters associated with

the  receiving  of the data from the remote Kermit.  SET

SEND will enable you to set the various SEND parameters.

These parameters should not normally be  set,  since  as

part  of  the  transfer  initialization  process the two

Kermit's  exchange  their  RECEIVE  parameters.      The

capability  of  setting  these parameters is provided so

that the transfer initialization can be  completed  even

if the default parameters are not correct.

PACKET_LENGTH   This  will  set  the SEND packet length.

The value for this parameter must be be-

tween 10 and 96.  Packet lengths outside

of this range are illegal.  The  default

value is 80.

PADDING         This command will set the number of pad-

ding characters that will be sent to the

other Kermit.  The default value is 0.

PADCHAR         This  parameter is the padding character

that is sent to the remote Kermit.   The

parameter must be an octal number in the

range  of  0  to  37  or 177.  All other

values are illegal.  The  default  value

is 0 (an ASCII NUL).

START_OF_PACKET This  command  will  set  the  start  of

packet character for Kermit.  The  start

of packet character must be in the range

of 0 to 36 octal. The default value is 1

(ASCII  SOH, CTRL-A).  This value should

only be changed if absolutely necessary.

It  must  be  set  the  same   in   both

Kermit's.

TIMEOUT         This  will set the number of seconds be-

VAX/VMS KERMIT                                                   Page 79

fore  Kermit-32  will time out a message

it has sent to the other Kermit.    mes-

sage.    This time out is used to handle

transmission errors which totally lose a

message.    The  default  value  is   15

seconds.

END_OF_LINE     This  will set the end of line character

the Kermit-32 will send  to  the  remote

Kermit.    This  is  the character which

terminates a packet.  The default  value

is 15 (ASCII CR, CTRL-M).

QUOTE           This will set the quoting character that

Kermit-32  will  expect on incoming mes-

sages.  This is the  character  used  to

quote  control  characters.  The default

value is 43 (ASCII "#").

The SHOW Command

The SHOW command will allow you to show the various parameters that  are

set with the SET command.

ALL             The SHOW ALL command will cause all of the parameters to

be listed.

BLOCK_CHECK_TYPE

This  command  will type out what type of block check is

being requested.

COMMUNICATIONS  This command will type out the  communcations  line  re-

lated parameters.  This includes the terminal line being

used, the parity type, etc.

DEBUGGING       The  SHOW  DEBUGGING command will print the state of the

debugging flag.

DELAY           This will display the number of seconds delay that  Ker-

mit  will  use  before  attempting  to send or receive a

file.

ESCAPE          This will display the current escape character  for  the

CONNECT processing.

FILE_PARAMETERS This  will display the parameters related to files being

used.  This includes the file type  and  the  incomplete

file disposition.

FILE_TYPE       This  will display the current file type that is used in

sending the file to or receiving the from the micro com-

puter.

INCOMPLETE_FILE_DISPOSITION

This  will  display  the  disposition  of   incompletely

VAX/VMS KERMIT                                                   Page 80

received files.

LOCAL_ECHO      This will display the status of the local echo flag.

PACKET          This  will  display the current settings of the send and

receive packet parameters.

PARITY          This will display the current parity setting.

SEND            All of the send parameters  will  be  displayed  on  the

user's terminal.

RECEIVE         The  current  values  of  the RECEIVE parameters will be

displayed on the user's terminal.   Only  the  parmeters

that can be set will be displayed.

RETRY           This  command  will show the maximum retry attempts that

Kermit will attempt to send a message the remote.

IBM VM/CMS KERMIT                                                Page 81

8. IBM VM/CMS KERMIT

Author:     Daphne Tzoar, Columbia University

Version:    ( unnumbered )

Date:       February 1983

Written  in  IBM  370  assembly language to run under VM/CMS on IBM 370-

series mainframes (System/370, 303x, 43xx, 308x, ...).  These  are  half

duplex  systems;  the  communication  line must "turn around" before any

data can be sent to it.  The fact that a packet has been  received  from

the  IBM  system is no guarantee that it is ready for a reply.  Thus any

Kermit talking to this system must wait for the line turnaround  charac-

ter (XON) before transmitting the next character.

IBM  systems  talk to their terminals through a communications front end

(IBM 3705, 3725, COMTEN 3670, etc).  These front ends  generally  insist

on  using the 8th bit of each character for parity.  This means that bi-

nary files  (files  containing  other  than  ordinary  letters,  digits,

punctuation,  carriage returns, tabs, and so forth) can not be correctly

sent or received by these systems with Kermit (protocol version 1).

The IBM system under VM/CMS  is  unable  to  interrupt  a  read  on  its

"console".    This  means that the IBM version of Kermit cannot timeout.

The only way to "timeout" CMS Kermit is from the other side -- typing  a

carriage  return to the micro's Kermit causing it to retransmit its last

packet, or an automatic timeout as provided  by  Kermit-20.    For  this

reason,  CMS  Kermit  waits  ten seconds before sending its first packet

when sending files from VM/CMS.  This gives the user sufficient time  to

return  to the local Kermit and issue the Receive command.  Otherwise, a

protocol deadlock would arise requiring manual intervention by the user.

Also, VM/CMS stores files as records rather byte streams.  VM/CMS Kermit

has to worry about assembling incoming data  packets  into  records  and

stripping  CRLFs from incoming lines, and also appending CRLFs to -- and

stripping trailing blanks from -- outgoing records.

The VM/CMS file specification is in the form

FILENAME FILETYPE FILEMODE

(abbreviated FN FT FM).  FM is equivalent to a device  specification  on

DEC  or  microcomputer  systems  (FN FT FM would translate to FM:FN.FT).

FILENAME and FILETYPE are at most 8  characters  in  length,  each,  and

FILEMODE  at  most  2.    When  FILEMODE is omitted from a filespec, the

user's own  disk  is  assumed.    Kermit-CMS  sends  only  FILENAME  and

FILETYPE,  and  converts  the  intervening  blank  to  a period for com-

patibility with most other operating systems.  Kermit-CMS Commands:

SEND fn ft [fm]

Send the specified file(s), using * or % as the wildcard charac-

ters (* will match any number of characters while % matches only

one).  Kermit-CMS assumes the file is located on the A disk, and

sets the filemode to A1.  If, however, the file is located on  a

different  disk, the filemode must be cited.  Also, note that if

you use * for the filemode, Kermit-CMS will send only the  first

file that matches.  Examples:

IBM VM/CMS KERMIT                                                Page 82

The  command  SEND  CEN  SPSS will send CEN SPSS A1.  To

send the same file located on  your  B  disk,  you  must

specify:  SEND CEN SPSS B.  SEND * FORTRAN will send all

fortran files on your A disk.  SEND ABC% EXEC will  send

all  exec  files  with  a four letter filename beginning

with ABC. If you have the file PLOT SAS on your  A  disk

and your B disk, SEND PLOT SAS * will send PLOT SAS A1.

RECEIVE [fn ft [fm]]

Receive  the  file(s)  sent  from  the  other Kermit.  If a file

specification is not included, Kermit-CMS will use  the  name(s)

provided by the other Kermit.  Use the file specification to in-

dicate  a different filename or a disk other than the A disk (in

this case, the file name and  type  must  also  be  supplied  or

= = FM can be used.)  Examples:

To  receive  files  using  the  filename(s)  sent by the

micro, use:  RECEIVE.  To save a file under a  different

name,  specify:   RECEIVE ABC FORTRAN.  To save the file

under the same name but on the B disk, specify:  RECEIVE

ABC FORTRAN B, or RECEIVE = = B.

SET parameter value

Set  the  parameter  to the specified value.  Legal Set commands

are:

RECFM option

Denotes the record format to be used when  creating  the

file.    Only  fixed and variable length records are al-

lowed, where variable is  the  default.    Indicate  the

desired  record  format  by  either  an F (fixed) or a V

(variable).

LRECL decimal-number

Indicates the logical record length.  The default is 80,

and the maximum allowed is 256.

QUOTE decimal-number

The ASCII value of the control prefix character you wish

to use in place of the  default  (#).    It  must  be  a

single,  printable  character  from among the following:

33-62, 96, or 123-126 (decimal).

END decimal-number

Indicates the ASCII value of the  end-of-line  character

you  choose  to  send.    The  default  is  CR (ASCII 13

decimal), but can be set to any two digit number between

00 and 31 (decimal).

PAC decimal-number

Allows the user to specify the  packet  size  the  micro

should  use  when  sending  to Kermit-CMS.  The range is

26-94, where 94 is the default.

SHOW parameter

Displays the current value of any variable that can  be  changed

IBM VM/CMS KERMIT                                                Page 83

via the SET command.

STATUS  Returns  the  status  of  the  previous execution of Kermit-CMS.

Therefore, STATUS will either display the message  "Kermit  com-

pleted  successfully",  or  the  last error encountered prior to

aborting.

CMS     Issues a CMS command from within Kermit-CMS.

CP      Issues a CP command from within Kermit-CMS.

?       Lists all legal Kermit-CMS commands.

This is a list of other salient facts about Kermit-CMS:

1. The commands are supplied with a help option, so  a  question

mark  can be typed to get the appropriate format or a list of

options.  The question mark, however, must be followed  by  a

carriage  return;  Kermit-CMS  will  respond  and display the

prompt again.  For instance, SET ? will list  all  valid  op-

tions for the SET command.

2. When  receiving  files,  if  the  record format is fixed, any

record longer than the logical record length  will  be  trun-

cated.    If the record format is variable, the record length

can be as high as 256.  For sending files, the maximum record

length is 256.

3. Before connecting to the IBM  mainframe  from  other  systems

(like  the  various microcomputer and PC Kermits, DEC-20 Ker-

mit, etc), you should set the IBM flag ON  so  that  echoing,

parity,  and  handshaking  can be done the way the IBM system

likes.

4. Note that "(" and ")" act as word  separators  on  the  input

line.    Therefore,  if you try to set the quote character to

"(*" or "*(", for example, only the first character  will  be

used.

5. Since  some  Kermits  do  not  send an error packet when they

"abort", Kermit-CMS  does  not  always  know  the  micro  has

stopped  sending it information.  Therefore, when you connect

back to the IBM, Kermit-CMS  may  still  be  sending  packets

(they  will  appear on the screen).  The user must hit a car-

riage return until Kermit-CMS has sent the maximum number  of

packets allowed and aborts.  The error message, however, will

not  indicate  that  communication  stopped because the micro

aborted, but rather that no start  of  header  character  was

found.

6. The  minimum  send  packet  size Kermit-CMS will allow is 26.

This is  necessary  to  avoid  an  error  while  sending  the

filename  or  an error packet.  If the micro tries to set the

value to be less than 26, Kermit-CMS will  immediately  abort

with an error of "Bad send-packet size."

IBM VM/CMS KERMIT                                                Page 84

7. While  the IBM's communication front end processor translates

all incoming  characters  from  ASCII  terminals  to  EBCDIC,

Kermit-CMS  translates  the  data  it  reads  back  to  ASCII

(characters not representable in  ASCII  are  replaced  by  a

null).   Not only is it easier to work with ASCII characters,

but it makes things more consistent throughout the many  ver-

sions  of  Kermit.    When the packets are sent to the micro,

Kermit-CMS converts all data back to EBCDIC.   The  ASCII  to

EBCDIC translation table can be found in the Appendix.

8. If  a transfer becomes stuck, you can CONNECT back to the CMS

system and type a lot of carriage returns --  each  one  will

cause  KERMIT-CMS to retransmit the current packet, until the

retransmission limit is reached, and  you  will  be  back  at

"KERMIT-CMS>" command level.

9. Kermit-CMS  supplies the micro and the user with numerous er-

ror messages.  If  the  execution  must  be  abnormally  ter-

minated,  an  error  packet  is  sent  to  the  micro  before

Kermit-CMS stops.  The same message can be retrieved via  the

STATUS  command  when  Kermit-CMS  returns  and  displays the

prompt.  If Kermit-CMS aborted because the maximum amount  of

retries  was  exceeded (20 on initialization packets and 5 on

others), the error message will display the most recent error

(i.e. the last NAK Kermit-CMS  encountered).    If  execution

stops  because the micro gave up, the error message will con-

vey that to the user, but it is the micro's responsibility to

pinpoint the error.  The messages  Kermit-CMS  gives  are  as

follows:

"Bad send-packet size"

Sent  when  the  micro attempts to set its receive buffer

size to a value that is less than 26  (the  minimum  that

Kermit-CMS  will  accept) or larger than 94, the maximum.

It will also occur if Kermit-CMS tries to send  a  packet

that is larger than the maximum specified.

"Bad message number"

This and the following messages flag inconsistencies in a

Kermit packet.

"Illegal packet type" -- This can be caused by sending server

commands.

"Unrecognized State"

"No SOH encountered"

"Bad Checksum"

"Bad character count"

"Micro sent a NAK"

"Lost a packet"

"Micro aborted"

The micro abnormally terminated the transfer.

"Illegal file name"

When  receiving  the  name  of  the  file from the micro,

Kermit-CMS   expects   it   to   be   in    the    format

'filename.filetype'.    If the filename, filetype, or dot

is missing, Kermit-CMS  will  reject  (NAK)  the  packet.

Also,  if  either  the filename or filetype exceeds eight

characters, it will be truncated.

IBM VM/CMS KERMIT                                                Page 85

"Invalid lrecl"

Kermit-CMS  will  abort  on  any file-system error it en-

counters when reading from the file it is to  send.    It

can  only send files with variable or fixed length record

formats, therefore, Wylbur Edit or  Packed  format  files

will cause an error.

"Permanent I/O error"

This  signifies  a  permanent I/O error that occured when

reading from an existing file.  Execution is aborted  im-

mediately.

"Disk is read-only"

Attempt to write on a read-only disk.

"Recfm conflict"

If a filename conflict arises, Kermit-CMS will append the

received  file  to  the existing one, provided the record

formats of the two are the same.  Otherwise,  this  error

will cause a halt of the execution.

"Disk is full"

Refers  to  any  error  regarding limitations on a user's

storage space.    Most  likely,  it  signifies  that  the

receiving  disk is full, but the error can also mean that

the maximum number of files allowed has been reached,  or

virtual storage capacity has been exceeded, and so on.

"Err allocating space"

Kermit-CMS  keeps a table of all files it has sent to the

micro, allocating extra space if more than ten files  are

sent  at  one  time.  If there is an error obtaining more

space, Kermit-CMS will abort with this error.

Work on VM/CMS Kermit continues.  Planned future enhancements include:

1. 8-bit quoting, to allow binary files  to  pass  through  com-

munication  front  ends  that insist on using the 8th bit for

parity.

2. Ability to act as a Kermit Server.

3. Ability to SET LINE, so that Kermit-CMS  can  be  used  as  a

local  Kermit,  connecting to a remote host over another com-

munication port.

UNIX KERMIT                                                      Page 86

9. UNIX KERMIT

Authors:    Bill Catchings, Bob Cattani, Chris Maio, Columbia University

with fixes and contributions from many others.

Documentation:

Walter Underwood, Ford Aerospace (Palo Alto, CA)

Version:    (unnumbered)

Date:       October 1983

A  sample,  working implementation of the Kermit "kernel" was written in

the C language, and widely distributed in the  Kermit  Protocol  Manual.

This  kernel was intended merely to illustrate the protocol, and did not

include a "user interface", nor some of the fancy features  like  server

support, 8-bit quoting, file warning, timeouts, etc.  Several sites have

added  the necessary trappings to make this a production version of Ker-

mit, usually under the UNIX operating system.

The keyword style of user/program interaction favored by Kermit (program

types prompt, user types command followed  by  operands,  program  types

another  prompt,  etc)  is contrary to the UNIX style, so UNIX implemen-

tations have a style more familiar to UNIX users.  C versions of  Kermit

are  running  successfully  on VAX and PDP-11 UNIX systems, IBM 370-com-

patible mainframes under Amdahl UTS, and the SUN  Microsystems  MC68000-

based and other workstations.

UNIX filespecs are of the form

dir1/dir2/dir3/ ... /filename

where  the  tokens delimited by slashes form a path name, and by conven-

tion are each limited to 14 characters in length.  The final token in  a

path  is  the  actual  file  name.    By  convention,  it is of the form

name.type, but there is nothing special about the  dot  separating  name

and  type;  to  UNIX  it's just another character, and there may be many

dots in a filename.

In the tradition of UNIX, here's the UNIX KERMIT "man page".

NAME            kermit - file transfer, virtual terminal over tty link

SYNOPSIS        kermit c[lbe] [line] [baud] [esc]

kermit r[ddilb] [line] [baud]

kermit s[ddilb] [line] [baud] file ...

DESCRIPTION     Kermit provides reliable  file  transfer  and  primitive

virtual terminal communication between machines.  It has

been  implemented on many different computers, including

microprocessors (see below).  The files transferred  may

be arbitrary ASCII data (7-bit characters) and may be of

any  length.   The file transfer protocol uses small (96

character) checksummed packets, with ACK/NACK  responses

and  timeouts.    Kermit  currently  uses  a five second

timeout and ten retries.

UNIX KERMIT                                                      Page 87

The  arguments  to  kermit are a set of flags (no spaces

between the flags), three optional args (which,  if  in-

cluded, must be in the same order as the flags which in-

dicate their presence), and, if this is a Send operation

a list of one or more files.  (It is similar in some way

to the tar command structure).

Kermit has three modes, Connect, Send, and Receive.  The

first  is  for  a virtual terminal connection, the other

two for file transfer.  These modes are specified by the

first flag, which should be c, s,  or  r,  respectively.

Exactly one mode must be specified.

The d flag (debug) makes kermit a bit more verbose.  The

states  kermit goes through are printed along with other

traces of its operation.  A second  d  flag  will  cause

kermit to give an even more detailed trace.

The  i  flag (image) allows slightly more efficient file

transfer between Unix machines.   Normally  (on  Kermits

defined  to  run  on  Unix systems) newline is mapped to

CRLF on output, CR's are discarded on input,  and  bytes

are  masked  to  7  bits.  If this is set, no mapping is

done on newlines, and all eight bits of  each  byte  are

sent or received.  This is the default for non-Unix ker-

mits.

The  l  flag  (line)  specifies the tty line that kermit

should use to communicate with the other machine.   This

is  specified  as a regular filename, like "/dev/ttyh1".

If no l option is specified, standard input is used  and

kermit assumes it is running on the remote host (ie. NOT

the machine to which your terminal is attached).

The  b  flag  (baud)  sets  the  baud  rate  on the line

specified by the l flag.  No changes are made if  the  b

flag  is  not  used.    Legal speeds are: 110, 150, 300,

1200, 2400, 4800, 9600.  Note that this version of  ker-

mit supports this option on Unix systems only.

The  e  flag  (escape)  allows the user to set the first

character of the two character escape sequence for  Con-

nect  mode.   When the escape character is typed, kermit

will hold it and wait for the next character.    If  the

next  character is c or C, kermit will close the connec-

tion with the remote host.  If the second  character  is

the  same  as the escape character, the escape character

itself is passed.  Any character other  than  these  two

results  in a bell being sent to the user's terminal and

no characters passwd to the  remote  host.    All  other

typed  characters  are  passed  through  unchanged.  The

default escape character is '^'.

The file arguments are only meaningful to a Send kermit.

The Receiving kermit will attempt to store the file with

the same name that was used to send it.    Unix  kermits

UNIX KERMIT                                                      Page 88

normally  convert  outgoing  file names to uppercase and

incoming ones to lower case (see the  f  flag).    If  a

filename  contains a slash (/) all outgoing kermits will

strip off the leading part of the name through the  last

slash.

EXAMPLE         For  this  example we will assume two Unix machines.  We

are logged onto "unixa" (the local machine), and want to

communicate with "unixb" (the remote machine).  There is

a modem on "/dev/tty03".

We want to connect to "unixb", then transfer "file1"  to

that machine.

We type:

kermit clb /dev/tty03 1200

Kermit answers:

Kermit: connected...

Now  we  dial  the remote machine and connect the modem.

Anything typed on the  terminal  will  be  sent  to  the

remote  machine and any output from that machine will be

displayed on  our  terminal.    We  hit  RETURN,  get  a

"login:" prompt and login.

Now  we  need to start a kermit on the remote machine so

that we can send the file over.  First we start  up  the

remote, (in this case receiving) kermit, then the local,

(sending)  one.    Remember that we are talking to unixb

right now.

We type:

kermit r

(there is now a Receive kermit on unixb)

We type ^ (the escape character) and then the  letter  c

to kill the local (Connecting) kermit:  ^C

Kermit answers:

Kermit: disconnected.

We type:

kermit slb /dev/tty03 1200 file1

Kermit answers:

Sending file1 as FILE1

When  the transmission is finished, kermit will type ei-

UNIX KERMIT                                                      Page 89

ther  "Send  complete",  or "Send failed.", depending on

the success of the transfer.  If we now wanted to trans-

fer a file from unixb  (remote)  to  unixa  (local),  we

would use these commands:

kermit clb /dev/tty03 1200

(connected to unixb)

kermit s file9

^c (up-arrow c not control-c)

(talking to unixa again)

kermit rl /dev/tty03 1200

After  all  the  transfers  were done, we should connect

again, log off of unixb, kill  the  Connect  kermit  and

hang up the phone.

FEATURES        Kermit  can  interact strangely with the tty driver.  In

particular, a tty with "hangup on last close" set  (stty

hup),  will  reset  to 300 Baud between kermit commands.

It will also hang up a modem at that time.  It is better

to run with "stty -hup", and use "stty 0" to  explicitly

hang up the modem.

The KERMIT Protocol uses only printing ASCII characters,

Ctrl-A,  and  CRLF.    Ctrl-S/Ctrl-Q flow control can be

used "underneath" the Kermit protocol (TANDEM line  dis-

cipline on Berkeley Unix).

Since  BREAK  is  not  an ASCII character, kermit cannot

send a BREAK to the remote machine.  On some systems,  a

BREAK will be read as a NUL.

This  kermit  does have timeouts when run under Unix, so

the protocol is stable when  communicating  with  "dumb"

kermits (that don't have timeouts).

DIAGNOSTICS     cannot open device

The file named in the line argument did not exist or had

the wrong permissions.

bad line speed

The baud argument was not a legal speed.

Could not create file

A Receive kermit could not create the file being sent to

it.

nothing to connect to

A Connect kermit was started without a line argument.

MS-DOS KERMIT                                                    Page 90

10. MS-DOS KERMIT

Program:    Daphne Tzoar and Jeff Damens, Columbia University; contribu-

tions by many others.

Language:   Microsoft Macro Assembler (MASM)

Documentation:

Frank  da  Cruz,  Columbia  University; Herm Fischer, Litton

Data Systems, Van Nuys CA.

Version:    2.26

Date:       July 1984

Kermit-MS Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            Yes

Alternate block checks:            Yes

Terminal emulation:                Yes

Communication settings:            Yes

Transmit BREAK:                    Yes

IBM mainframe communication:       Yes

Transaction logging:               No

Session logging:                   Yes

Raw transmit:                      No

Act as server:                     Yes

Talk to server:                    Yes

Advanced server functions:         No

Advanced commands for servers:     Yes

Local file management:             Yes

Handle file attributes:            No

Command/init files:                Yes

Command macros:                    Yes

Kermit-MS is a program that implements the KERMIT file transfer protocol

for the IBM PC and several  other  machines  using  the  same  processor

family  (Intel  8088  or  8086)  and  operating system family (PC-DOS or

MS-DOS, henceforth referred to collectively  as  MS-DOS,  versions  1.1,

2.0,  and  2.1,  and thereafter).  This section will describe the things

you should know about the MS-DOS file system in order to make  effective

use of Kermit, and then it will describe the Kermit-MS program.

Version  2  of MS-DOS Kermit runs on a variety of systems, including the

IBM PC and XT, the HP-150, the DEC Rainbow 100 and 100+ (MS-DOS 2.05 and

above), the Wang PC, and there is a "generic" MS-DOS version.  Version 1

was adapted at various stages of development to run on other systems  as

well,   including   the   Heath/Zenith  100,  Tandy  2000,  Victor  9000

(Sirius-1), and Seequa Chameleon, and is still available for those  sys-

tems until support for them and others is added to version 2.

MS-DOS KERMIT                                                    Page 91

The program operates under version 1.1 or 2.0 and above of DOS, although

some  features  require  the  functionality  of  2.0.    It  runs in ap-

proximately 80K of memory -- over and  above  the  memory  used  by  DOS

--  which means that your system should have at least 128K of RAM to use

version 2 of MS-DOS Kermit; smaller systems may still use Version 1.

10.1. The MS-DOS File System

The features of the MS-DOS file system of greatest  interest  to  KERMIT

users  are  the form of the file specifications, and the distinction be-

tween pre-MS-DOS 2.0 file names and newer file names which allow  direc-

tory paths.

10.1.1. File Specifications

MS-DOS 2.x file specifications are of the form

DEVICE:\PATHNAME\NAME.TYPE

where  the  DEVICE is a single character identifier (for instance, A for

the first floppy disk, C for the first fixed disk,  D  for  a  RAM  disk

emulator),  PATHNAME  is  up  to 63 characters of identifier(s) (up to 8

characters each) surrounded by reverse slashes, NAME is an identifier of

up to 8 characters, and TYPE is an identifier of up to 3  characters  in

length.  Device and pathname may be omitted.  The first backslash in the

pathname may be omitted if the specified path is relative to the current

directory.    In the path field, "." means current directory, ".." means

parent directory.  Some DOS implementations (like Wang)  may  use  slash

"/" rather than backslash in the path field.

Pathname  is normally omitted, and cannot be specified for MS-DOS 1.x or

with those commands which allow MS-DOS 1.x use.   Device  and  directory

pathnames,  when  omitted, default to either the user's current disk and

directory, or to the current directory search path as specified  in  the

DOS  PATH  environment  variable,  depending on the context in which the

file name appears.

When this manual says that a file is searched for "in  the  cur-

rent path," it means that the PATH is searched first, and if the

file  is not found, then Kermit-MS looks on the current disk and

directory.  If the PATH environment variable  is  empty,  Kermit

looks only at the current disk and directory.

NAME.TYPE is normally sufficient to specify a file, and only this infor-

mation is sent along by Kermit-MS with an outgoing file.

The  device,  path, name, and type fields may contain uppercase letters,

digits, and the special characters "-" (dash), "_" (underscore), and "$"

(dollar sign).   (For  use  only  among  MS-DOS  processors,  additional

filename  special  characters  allowed are "#&!%'`(){}".  DOS 1.x allows

others as well.).  There are no imbedded  or  trailing  spaces.    Other

characters  may  be not be included; there is no mechanism for "quoting"

otherwise illegal characters in filenames.    The  fields  of  the  file

specification  are set off from one another by the punctuation indicated

MS-DOS KERMIT                                                    Page 92

above.

The  name  field is the primary identifier for the file.  The type, also

called the extension or suffix, is an indicator  which,  by  convention,

tells  what kind of file we have.  For instance FOO.BAS is the source of

a BASIC program named FOO;  FOO.OBJ  might  be  the  relocatable  object

module  produced  by  compiling  FOO.BAS; FOO.EXE could be an executable

program produced by linking FOO.OBJ, and so forth.  .EXE  and  .COM  are

the normal suffixes for executable programs.

The  MS-DOS  allows  a  group  of files to be specified in a single file

specification by including the special "wildcard" characters, "*" and "?

".  A "*" matches any string of characters from the current position  to

the  end of the field, including no characters at all; a "?" matches any

single character.  Here are some examples:

*.BAS   All files of type BAS (all BASIC source files) in the  current

directory.

FOO.*   Files of all types with name FOO.

F*.*    All files whose names start with F.

F?X*.*  All  files whose names start with F and contain X in the third

position, followed by zero or more characters.

?.*     All files whose names are exactly one character long.

Wildcard notation is used on many computer systems in similar ways,  and

it  is  the  mechanism  most  commonly used to instruct Kermit to send a

group of files.

Note: Kermit-MS uses the "?" character for help  while  commands

are  being  typed, so the single-character wildcard in Kermit-MS

commands is "=" rather than "?".  For example

Kermit-MS>send =.*

would send files of all  types  whose  names  were  exactly  one

character long.

Kermit-MS  users should bear in mind that other (non-MS-DOS) systems may

use different wildcard characters.  For instance the DEC-20 uses "%" in-

stead of "?" as the single character wildcard; when using  Kermit-MS  to

request a wildcard file group from a KERMIT-20 server, the Kermit-MS "="

must be replaced by the DEC-20 "%".

10.1.2. File Formats

MS-DOS  systems  store files as bulk collections of 8 bit bytes, with no

particular differences between text, program  code,  and  binary  files.

ASCII  text files consist of lines separated by carriage-return-linefeed

sequences (CRLFs), which conforms exactly to the way  Kermit  represents

text  files  during  transmission.   Since a non-MS-DOS receiving system

might need to make distinctions as to file type, you  may  need  to  use

MS-DOS KERMIT                                                    Page 93

various  SET functions on the remote system to inform it that the incom-

ing file is of some particular (non-default) type, such as binary.    In

transmitting files between Kermit-MS's, regardless of file contents, the

receiving MS-DOS system is equally capable of processing text, code, and

data,  and  in  fact  has  no knowledge of how the bytes in the file are

used.

MS-DOS (unlike CP/M) is capable of pinpointing  the  end  of  file  with

precision  by keeping a byte count in the directory, so one would expect

no particular  confusion  in  this  regard.    However,  certain  MS-DOS

programs  continue to use the CP/M convention of terminating a text file

with a Control-Z character, and won't operate correctly unless this ter-

minating byte is present.  Therefore, Kermit-MS users should be aware of

a special SET EOF option for both incoming and outbound files, described

below.

Non-MS-DOS systems may well be confused by nonstandard ASCII files  from

Kermit-MS.   Files produced by Easywriter or Word Star, for example, may

need to be converted to conventional ASCII format prior to  transmission

by  commonly  available  "exporter"  programs.   Spreadsheet or database

files usually need special formatting to  be  meaningful  to  non-MS-DOS

recipients  (though  they can be transmitted between MS-DOS systems with

Kermit-MS).  Furthermore, files created by word processors (such as BLUE

or Easy Writer) that store formatting data at the end of the file, after

the control-Z and before physical end, will require  special  processing

via  SET  EOF to strip the formatting data, lest they confuse non-MS-DOS

recipients.

10.2. Program Operation

Kermit-MS can be  run  interactively,  from  a  batch  file,  or  as  an

"external"  DOS  command.    Commands  consist  of  one  or more fields,

separated by "whitespace" -- one or more spaces or tabs.

Upon initial startup, the program executes any  commands  found  in  the

file  MSKERMIT.INI  in  the  current path.  This initialization file may

contain command macro definitions, communications settings  for  one  or

more  ports,  or  any  other  Kermit-MS  commands.    Here  is  a sample

MSKERMIT.INI file:

set warning on  ; Enable filename collision avoidance.

;

; Define some macros

;

define unix set local-echo off, set flow xon, set timer off

def ibm set parity odd, set local on, set handsh xon, set timer on

def modem set port 2, set baud 1200

def noisy set block-check 3, set send packet-length 40

;

; Select a port

;

set port 1      ; Select COM1 for communications,

set baud 4800   ; setting the speed to 4800 baud,

connect         ; and make a terminal connection.

MS-DOS KERMIT                                                    Page 94

Note  that  comments may be included by prefixing them with a semicolon.

The program can be run in several ways.

Interactive Operation:

To run Kermit-MS interactively, invoke  the  program  from  DOS  command

level by typing its name.  When you see the command's prompt,

Kermit-MS>

you  may type Kermit commands repeatedly until you are ready to exit the

program, for example:

A>

A>kermit

IBM PC Kermit-MS V2.26

Type ? for help

Kermit-MS>send foo.*

informational messages about the files being sent

Kermit-MS>get bar.*

informational messages about the files being received

Kermit-MS>exit

A>

During interactive operation, you may edit the command you're  currently

typing  to  erase  the character most recently typed (BACKSPACE or DEL),

the most recent field (CTRL-W), or the entire command (CTRL-U).  In  ad-

dition, you may use the help ("?") and recognition (ESC) features freely

while  typing  Kermit-MS  commands.  A question mark typed at almost any

point in a command produces a brief description of what is  expected  or

possible  at  that  point;  for  this reason, Kermit-MS uses "=" for the

single-character match wildcard in local filenames.  ESC  typed  at  any

point,  even  in  a  local  filename, will cause the current field to be

filled out if what you have typed so far is sufficient to  identify  it,

and  will leave you in position to type the next field (or to type a "?"

to find out what the next field is); otherwise, the program will beep at

you and wait for you to type further characters.

Some Kermit-MS commands, like GET, SHOW KEY, SET KEY, may prompt for ad-

ditional information on subsequent lines.  If you have  reached  one  of

these  prompts  and  then  wish  to  cancel  the  command,  you may type

Control-C.

Summary of Kermit-MS Command Characters:

BACKSPACE     Delete the character most recently typed.  May be  typed

repeatedly  to  delete  backwards.    You  may  also use

DELETE, RUBOUT, or equivalent keys.

MS-DOS KERMIT                                                    Page 95

CTRL-W        Delete  the most recent "word", or field, on the command

line.  May be typed repeatedly.

CTRL-U        Delete the entire command line.

CTRL-C        Cancel  the  current   command   and   return   to   the

"Kermit-MS>" prompt.

?             Type a brief message describing what you are expected to

type in the current field.

ESC           If  enough  characters have been supplied in the current

field (keyword or file name) to  uniquely  identify  it,

supply  the  remainder  of the field and position to the

next field of the command.  Otherwise, sound a beep.

=             Wildcard character for  matching  single  characters  in

filenames, equivalent to MS-DOS "?".

Command Line Invocation:

Kermit-MS  may also be invoked with command line arguments from DOS com-

mand level, for instance:

A>kermit send foo.bar

or

A>kermit set port 1, set baud 9600, connect

In this case, help  and  recognition  are  not  available  (because  the

program  won't  start  running  until  after you type the entire command

line), and Kermit-MS will exit after completing the specified command or

commands.    Therefore,  when  invoked  with  command  line   arguments,

Kermit-MS  will behave as if it were an external DOS command, like MODE.

Note that several commands may be given on the command  line,  separated

by commas.

Batch Operation:

Like  other  MS-DOS programs, Kermit-MS may be operated under batch with

either command line arguments and/or TAKE files; Kermit  will  also  run

interactively  if invoked from batch, but it will read commands from the

keyboard and not the batch file.

10.3. Kermit-MS Commands

MS-DOS Kermit implements a large subset of the commands of "ideal"  Ker-

mit.  Here's a brief summary:

BYE  to remote server.

CLOSE  log file and stop logging remote session.

CONNECT  as terminal to remote system.

MS-DOS KERMIT                                                    Page 96

DEFINE  macros of Kermit-MS commands.

DELETE  local files.

DIRECTORY  listing of local files.

DO  a macro expansion.

EXIT  from Kermit-MS.

FINISH  Shut down remote server.

GET  remote files from server.

HELP  about Kermit-MS.

LOCAL  prefix for local file management commands.

LOG  remote terminal session.

LOGOUT  remote server.

PUSH  to MS-DOS command level.

QUIT  from Kermit-MS

RECEIVE  files from remote Kermit.

REMOTE  prefix for remote file management commands.

RUN  an MS-DOS program.

SEND  files to remote Kermit.

SERVER  mode of remote operation.

SET  various parameters.

SHOW  various parameters.

SPACE  inquiry.

STATUS  inquiry.

TAKE  commands from file.

The  remainder  of  this  section concentrates on the commands that have

special form or meaning for MS-DOS Kermit.  Not  all  of  the  following

commands  are  necessarily  available on all MS-DOS systems, and some of

the commands may work somewhat differently between DOS versions.

10.3.1. Commands for File Transfer

The file transfer commands are SEND, GET, and RECEIVE.

THE SEND COMMAND

Syntax:  SEND filespec1 [filespec2]

The SEND command causes a file or file group to be sent from  the  local

MS-DOS system to the Kermit on the remote system.  The remote Kermit may

be running in either server or interactive mode; in the latter case, you

should  already have given it a RECEIVE command and escaped back to your

PC.

filespec1 may contain a device designator, like "A:"  and  the  wildcard

characters  "*"  and/or "=".  The current release of Kermit-MS, however,

does not allow pathnames in the SEND command file specification.

If filespec1 contains wildcard characters then all matching  files  will

be  sent,  in  the same order that MS-DOS would show them in a directory

listing.  If filespec1 specifies a single file, you may direct Kermit-MS

to send that file with a different name, given in filespec2.    For  in-

stance, in the command

Kermit-MS>send foo.bar framus.widget

MS-DOS KERMIT                                                    Page 97

filespec2  begins  with the first nonblank character after filespec1 and

ends with the carriage return; thus it may contain blanks or  other  un-

usual  characters  that may be appropriate on the target machine.  Lower

case letters in filespec2 are raised to upper case for transmission.

If a file can't be opened for  read  access,  standard  MS-DOS  recovery

procedures will take place.  For example:

Not ready error reading drive A

Abort, Retry, Ignore?

If you select "Abort," you will be returned to DOS.

Files will be sent with their MS-DOS filename and filetype (for instance

FOO.TXT,  no device or pathname).  Each file is sent as is, with no con-

versions done on the data, except for possibly adding or deleting a ter-

minating Control-Z character (see the SET EOF command).

Once you give Kermit-MS the SEND command, the name of each file will  be

displayed  on  your  screen  as  the transfer begins; packet, retry, and

other counts will be displayed along with informational messages  during

the  transfer.    If  the file is successfully transferred, you will see

"Complete", otherwise  there  will  be  an  error  message.    When  the

specified operation is done, the program will sound a beep.

Several  single-character commands may be given while a file transfer is

in progress:

^X  (Control-X) Stop sending the current file and go on  to  the  next

one, if any.

^Z  Stop sending this file, and don't send any further files.

^C  Return  to Kermit-MS command level immediately without sending any

kind of notification to the remote system.

^E  Like ^C, but send an Error packet to the remote Kermit in  an  at-

tempt to bring it back to server or interactive command level.

CR  Simulate a timeout: resend the current packet, or NAK the expected

one.

Control-X  and Control-Z send the proper protocol messages to the remote

Kermit to bring it gracefully to the desired state.    Control-C  leaves

the  remote  Kermit  in  whatever  state it happens to be in.  Control-E

"aborts" any protocol that is taking place.

THE RECEIVE COMMAND

Syntax: RECEIVE [filespec]

The RECEIVE command tells Kermit-MS to receive a file or file group from

the other system.  Kermit-MS simply waits for the file to  arrive;  this

command  is  not to be used when talking to a Kermit server (use GET for

that).  You should already have issued a SEND command to the remote Ker-

MS-DOS KERMIT                                                    Page 98

mit and escaped back to Kermit-MS before issuing the RECEIVE command.

If the optional filespec is provided, store the incoming file under that

name.    The filespec may include a device designator, or may consist of

only a device designator.  The incoming file is stored on the default or

specified device (current directory in DOS 2.0 and thereafter).   If  no

name  was  specified,  the  name from the incoming file header packet is

used; if that name is not a  legal  MS-DOS  file  name,  Kermit-MS  will

delete  excessive characters from it, and will change illegal characters

to the letter X.

If the optional filespec was provided, but more than one  file  arrives,

the  first  file  will  be  stored  under  the  given  filespec, and the

remainder will be stored under their own names,  but  on  the  specified

device.

If an incoming file does not arrive in its entirety, Kermit-MS will nor-

mally  discard it; it will not appear in your directory.  You may change

this behavior by using the command SET INCOMPLETE KEEP, which will cause

as much of the file as arrived to be saved in your directory.

The same single-character commands are available as during SEND:

^X  Request that the remote Kermit stop sending the current file,  and

proceed  to  the  next one immediately.  Since this is an optional

feature of the Kermit protocol, the remote Kermit might not  honor

the request.

^Z  Request that the remote Kermit terminate the entire transfer; this

is  also  an  optional feature that may or may not be supported by

the remote Kermit.

^C, ^E, and CR operate in the same way as they do during SEND.

If the incoming file has the same name as a file  that  already  exists,

and  WARNING is set ON, Kermit-MS will change the incoming name (and in-

form you how it renamed it) so as not  to  obliterate  the  pre-existing

file.   If WARNING is OFF, the original file will be overwritten; if you

type ^X or ^Z to interrupt the transfer, you'll either get a partial new

file, or else both the old and the new file of that name will  be  lost,

depending  on  SET  INCOMPLETE.  In any case, when WARNING is off, files

with the same name as incoming files will not survive.

Caution: If an incoming file's name  (the  part  before  the  dot)  cor-

responds  to an MS-DOS device name, such as NUL, COM1, CON, AUX, or PRN,

output will go to that device, rather than to a  file  with  that  name.

This is a feature of MS-DOS.

MS-DOS KERMIT                                                    Page 99

THE GET COMMAND

Syntax: GET remote-filespec

The GET command requests a remote KERMIT server to send the file or file

group  specified by remote-filespec.  This command can be used only when

Kermit-MS has a KERMIT server on the other end of the connection.   This

means  that  you must have CONNECTed to the other system, logged in, run

KERMIT there, issued the SERVER command, and escaped back (e.g. ^]C)  to

the  local  Kermit-MS.  If the remote Kermit does not have a SERVER com-

mand, then you should use SEND and RECEIVE as described above.

You may use the GET command to specify a different name for storing  the

incoming.    Just  type  GET  alone  on a line, and you will be prompted

separately for the remote filespec and the local filespec:

Kermit-MS>get

Remote Source File: com1.txt

Local Destination File: xcom1.txt

If more than one file arrives, only the first will be renamed.

The remote filespec is any string that can be a legal file specification

for the remote system; it is not parsed or validated locally.    It  can

contain  whatever wildcard or file-group notation is valid on the remote

system.  As files arrive, their names will be displayed on your  screen,

along  with packet traffic statistics and status messages.  You may type

^X to request that the current incoming file be cancelled, ^Z to request

that the entire incoming batch be cancelled, and ^C or ^E to return  im-

mediately to the Kermit-MS> prompt, exactly as described for the RECEIVE

command.

10.3.2. Commands for Connecting and Disconnecting

The CONNECT command connects your PC as a terminal to the remote system,

so that you can start up Kermit there.  The BYE, FINISH, and LOGOUT com-

mands allow you to shut down a remote Kermit server.

BYE             When  communicating with a remote KERMIT server, use the

BYE command to shut down the server, log  out  its  job,

and exit from Kermit-MS to DOS.

FINISH          Like BYE, FINISH shuts down the remote server.  However,

FINISH  does not log out the server's job.  You are left

at Kermit-MS prompt level so that you can  connect  back

to the job on the remote system.

LOGOUT          The  LOGOUT command is identical to the BYE command, ex-

cept you will remain at Kermit-MS prompt  level,  rather

than exit to DOS, so that you can establish another con-

nection.

MS-DOS KERMIT                                                   Page 100

The CONNECT Command

Establish  an interactive terminal connection to the system connected to

the currently selected communications port (e.g.  COM1  or  COM2)  using

full duplex (remote) echoing and no parity unless otherwise specified in

previous  SET  commands.    Get  back  to Kermit-MS by typing the escape

character followed by the letter C. The escape character is Control-] by

default.

You can use the SET ESCAPE command to define a different escape  charac-

ter,  and  on some systems (including the PC and XT) you can SET BAUD to

change the baud rate, and SET PORT to switch between ports.

Terminal emulation is described in greater detail in section 10.4 below.

10.3.3. Commands for File Management

Kermit-MS provides commands or managing both local and remote files.

THE REMOTE COMMANDS

The REMOTE keyword is a prefix for a number of commands.   It  indicates

that  the command is to be performed by the remote Kermit, which must be

running as a server.  Note that not all Kermit servers  are  capable  of

executing  all  these  commands,  and some Kermit servers may be able to

perform functions for which Kermit-MS does not yet have the  correspond-

ing  commands.  In case you send a command the server cannot execute, it

will send back a message stating that the command is unknown to it.   If

the  remote  server can execute the command, it will send the results to

your screen.  Here are the REMOTE commands which Kermit-MS may issue:

CWD [directory] Change Working Directory on the remote host.  Change the

default source and destination area  for  file  transfer

and  management.    You will be prompted for a password,

which will be erased as you type it.    If  you  do  not

supply  a  password  (i.e.  you  type  only  a  carriage

return), the server will attempt to access the specified

directory without a password.  If you do  not  supply  a

directory  name,  your default or login directory on the

remote system will be assumed.

DELETE filespec Delete the specified file or files on the remote system.

In response, the remote host should display  a  list  of

the files that were or were not successfully deleted.

DIRECTORY [filespec]  The remote system will provide a directory listing

of the specified files.  If no files are specified, then

all  files  in  the  default  area  (the current working

directory) will be listed.

HELP            The remote  host  tells  what  server  functions  it  is

capable of.

HOST [command]  Send  the command to the remote system's command proces-

MS-DOS KERMIT                                                   Page 101

sor for execution.

SPACE [directory]

Provide  a  brief summary of disk usage in the specified

area on the remote host.  If none specified, the default

or current area will be summarized.

TYPE filespec   Display the contents of the  specified  remote  file  or

files on the screen.

THE LOCAL COMMAND

The  LOCAL  keyword  is a prefix for a number of commands.  It indicates

that the specified command is to be executed on the local MS-DOS system.

The LOCAL prefix may be omitted.  The local commands available are:

DELETE filespec Deletes the specified file or files.   As  in  DOS,  the

names of the deleted files are not listed, only the mes-

sage  "file(s)  deleted"  or "file(s) not found", and if

you give the command "delete *.*", Kermit-MS will prompt

"Are you sure?", like DOS.

DIRECTORY [filespec]  Lists the names,  sizes,  and  creation  dates  of

files  that  match  the given file specification.  If no

filespec is given, the command is equivalent to DIR *.*.

SPACE           Performs the  MS-DOS  CHKDSK  function  by  running  the

CHKDSK  program  from  the current path, or default disk

under DOS 1.1.

RUN filespec    Runs the specified file, which must be in .EXE  or  .COM

format,  from  the  specified  path  or according to the

value of the PATH variable if no path  was  included  in

the  filespec.    This  command  requires  MS-DOS 2.0 or

higher.

PUSH            Invokes an MS-DOS command processor  "under"  Kermit-MS,

either  COMMAND.COM or whatever shell you have specified

with COMSPEC.  When you return  to  Kermit-MS  (for  in-

stance,  by  typing  the  MS-DOS EXIT command), you will

find Kermit-MS as you left it, with all settings intact.

This command only works in MS-DOS 2.0 or higher.

The local RUN command has various uses, one of which  is  to  supplement

the  features  of Kermit-MS.  For instance, suppose there is an involved

procedure that you regularly perform on a certain remote system --  this

might include giving commands to a modem to dial the system, looking for

a particular herald or prompt, performing a login command sequence, run-

ning a selected application, and then running Kermit to send the results

back  to your PC.  You could write a program in the compiled language of

your choice, say C or BASIC, to send the desired commands to your  modem

and  the  remote  system and to look for the appropriate responses.  You

could put all this in a Kermit-MS TAKE command file (see below), like

MS-DOS KERMIT                                                   Page 102

run update.com

receive

The  program,  called UPDATE in this case, does everything up to and in-

cluding starting Kermit sending  from  the  remote  system.    When  the

program  terminates,  the next Kermit-MS command, "receive," is executed

from the command file.  When the end of the command file is reached, in-

teractive operation is resumed.

THE TAKE COMMAND

Syntax: TAKE filespec

Execute Kermit commands from the specified file, which  may  include  an

explicit  path;  if no path is specified, the value of the PATH variable

is used; if PATH has no value, then the current disk and  directory  are

searched.  The command file may include TAKE commands, but it cannot in-

clude  characters  to be sent to a remote host during terminal emulation

(i.e. after a CONNECT command).  A command  file  may  include  comments

prefixed by semicolons.

THE LOG COMMAND

Syntax: LOG filespec

Specifies  that all characters that appear on your screen during CONNECT

will be recorded in the specified file.  This allows  you  to  "capture"

files  from  a  remote  system  that  doesn't have Kermit, as well as to

record remote command typescripts.  The log is closed when you EXIT from

Kermit-MS or when you issue an explicit CLOSE command.

10.3.4. The SERVER Command

Kermit-MS is capable of acting as a Kermit server, providing file trans-

fer for users coming in through one of the  communication  ports.    The

current  version  of Kermit-MS can send files (the user on the other end

types the GET command), receive files (the user types  SEND),  and  ter-

minate, giving control back to the console (user types BYE).

To  put Kermit-MS into server mode, first issue any desired SET commands

to select and configure the desired port, and then type the SERVER  com-

mand.   Kermit-MS will await all further instructions from the user Ker-

mit on the other end of the connection, which may be hardwired  or  con-

nected through an autoanswer modem.  For example:

Kermit-MS>set port 1

Kermit-MS>set baud 1200

Kermit-MS>set timer on

Kermit-MS>set warning on

Kermit-MS>server

MS-DOS KERMIT                                                   Page 103

10.3.5. The SET Command

Syntax: SET parameter [value]

Establish  or  modify  various  parameters for file transfer or terminal

connection.  You can examine their values with the SHOW command.    Note

that  there  is  no  "set  ibm"  command;  IBM  mainframe  communication

parameters may be selected with a command macro (see below).   The  fol-

lowing SET commands are available in Kermit-MS:

BAUD  Communications port line speed

BELL  Whether to beep at the end of a transaction

BLOCK-CHECK-TYPE  Level of error checking for file transfer

DEBUG  Display packet contents during file transfer

DEFAULT-DISK  Default disk drive for file i/o

DESTINATION  Default destination device for incoming files

END-OF-LINE  Packet terminator

EOF  Method for determining or marking end of file

ESCAPE  Escape character for CONNECT

FLOW-CONTROL  Enable or disable XON/XOFF

HANDSHAKE  Half-duplex line turnaround option

HEATH19  Heath/Zenith-19 terminal emulation

INCOMPLETE  What to do with an incompletely received file

KEY  Specify key redefinitions, or "keystroke macros"

LOCAL-ECHO  Specify which host does the echoing during CONNECT

PARITY  Character parity to use

PORT  Select a communications port

PROMPT  Change the "Kermit-MS>" prompt to something else

RECEIVE  Request remote Kermit to use specified parameters

REMOTE  For running Kermit-MS interactively from back port

SEND  Use the specified parameters during file transfer

TAKE-ECHO  Control echoing of commands from TAKE files

TIMER  Enable/disable timeouts during file transfer

WARNING  Specify how to handle filename collisions

The SET commands that are peculiar to MS-DOS Kermit are now described in

greater detail.  The others behave as in "ideal" Kermit.

SET BAUD

Syntax: SET BAUD rate

Set  the  speed  of  the currently selected terminal communications port

(COM1 by default) to 300, 1200, 1800, 2400, 4800, 9600 or  other  common

baud  rate.    Some implementations do not support this command.  In any

case, Kermit-MS leaves the current communication port settings alone un-

less you issue explicit SET commands to change them.

SET BELL

Syntax: SET BELL ON or OFF

Specifies whether bell (beeper) should sound upon completion of  a  file

transfer operation.

MS-DOS KERMIT                                                   Page 104

SET DEBUG

Syntax: SET DEBUG ON or OFF

ON      Display  the  Kermit  packet  traffic on your screen during file

transfer.  If the debugger is loaded,  transfer  control  to  it

when CTRL-C is typed.  In Heath-19 terminal emulation on the IBM

PC, display unusual control characters in uparrow notation.

OFF     Don't  display  debugging information (this is the default).  If

debugging was in effect, turn it off.

SET DEFAULT-DISK

Syntax: SET DEFAULT-DISK x:

Specify the default disk drive to use for file transfer, directory list-

ings, and so forth.  Equivalent to typing the DOS command  for  changing

disks.

SET DESTINATION

Syntax: SET DESTINATION device

Specify the device for incoming files, DISK or PRINTER.  SET DESTINATION

PRINTER will cause incoming files to be spooled directly to the printer.

The normal destination is DISK.

END-OF-LINE

Syntax: SET END-OF-LINE number

If  the  remote  system needs packets to be terminated by anything other

than carriage return, specify the decimal value  of  the  desired  ASCII

character.

SET EOF

Syntax: SET EOF option

Controls how the end of file is handled.  The options are:

CTRL-Z          Append  a  Control-Z character to the end of an incoming

file, unless it already ends with a Control-Z.   Certain

MS-DOS text editors and other applications require files

to  be  in  this  format.  For outbound files, treat the

first Control-Z as the end of file, and do not  send  it

nor any characters following it.

NOCTRL-Z        (Default)  Store  incoming files exactly as is, and send

MS DOS files exactly as  is  (according  to  their  byte

count).

MS-DOS KERMIT                                                   Page 105

SET ESCAPE

Syntax: SET ESCAPE character

Specify  the  control  character you want to use to "escape" from remote

connections back to Kermit-MS.  The default  is  normally  ^]  (Control-

Rightbracket).  The  character is entered literally, and should normally

be chosen from the ASCII control range.

SET FLOW-CONTROL

Syntax: SET FLOW-CONTROL option

Specify the full duplex  flow  control  to  be  done  on  the  currently

selected  port.    The  current  options  are  XON/XOFF  and  NONE.  The

specified type of flow control will be done during both terminal  emula-

tion  and file transfer.  If set to XON/XOFF, HANDSHAKE is automatically

set to OFF.

SET HANDSHAKE

Syntax: SET HANDSHAKE option

Specify any half-duplex handshaking to be done on the currently selected

port.  The options are BELL, CR, LF, NONE, XOFF, or XON.  The  specified

handshaking will be done during file transfer only.  If HANDSHAKE is set

to anything other than NONE, FLOW-CONTROL is automatically set to OFF.

SET HEATH19

Syntax: SET HEATH19 ON or OFF

Specify  whether Kermit-MS should use its built-in software facility for

emulating a Heath/Zenith-19 (H19) terminal.

ON      During CONNECT, incoming characters are to be examined  for  H19

terminal  screen control commands (escape sequences), and if en-

countered, the commands are to be emulated  on  the  PC  screen.

The  H19  codes are a superset of the popular DEC VT52 codes, so

if your system does not support the Heath-19, you  may  tell  it

that  your  terminal  type  is  VT52  (or  one  of the many VT52

compatibles).  The Heath-19 codes are listed in  section  10.10,

below.

OFF     All  incoming  characters  will  be  sent  to the screen "bare",

through DOS.  If you have loaded a device driver  into  DOS  for

the CON: device, such as ANSI.SYS, then that driver will be able

to  interpret  the codes itself. Most non-IBM systems have their

own screen control code interpreter built into DOS or  firmware,

or available as a loadable device driver.

See section 10.4 for details about terminal emulation.

MS-DOS KERMIT                                                   Page 106

SET KEY

Syntax: SET KEY key-specifier

Specifies  that when the designated key is struck during terminal emula-

tion, the associated character string is sent.  The key-specifier is one

of the keywords F1, F2, ..., or SCAN followed by a scan code.    Systems

that have a BACKSPACE key also include BACKSPACE as a keyword.

If SCAN is used, it is followed by a decimal number to indicate the scan

code  of  the  key, which you would ascertain from your system reference

manual, or else by using the  Kermit-MS  SHOW  KEY  command.    SET  KEY

prompts  you  on  a new line for the definition string.  Certain charac-

ters, like ESC and CR, may not be entered literally into the string, but

can be included by inserting escape codes of the form \ooo, a  backslash

followed  by  a  2-  or  3-digit octal number corresponding to the ASCII

value of the desired character.  If some other key redefinition package,

like  ProKey,  has  been  loaded,  then  its  redefinitions  will   take

precedence over Kermit's.

The  SET  KEY  command is illustrated in the terminal emulation section,

10.4, below.

SET LOCAL-ECHO

Syntax: SET LOCAL-ECHO option

Specify how characters are echoed during terminal emulation on the  cur-

rently  selected port.  ON specifies that characters are to be echoed by

Kermit-MS (because neither the remote computer  nor  the  communications

circuitry  has  been  requested  to  echo), and is appropriate for half-

duplex connections.  LOCAL-ECHO is  OFF  by  default,  for  full-duplex,

remote echo operation.

When  you SET LOCAL-ECHO ON, the current HANDSHAKE (if any) is automati-

cally enabled and full-duplex FLOW-CONTROL is automatically turned  off.

When you SET LOCAL-ECHO OFF, HANDSHAKE is also disabled, and the current

mode  of  FLOW-CONTROL  (if  any)  is  enabled.  If this behavior is un-

desired, you may override it by typing explicit  SET  HANDSHAKE  or  SET

FLOW commands after entering the SET LOCAL-ECHO command.

SET PARITY

Syntax: SET PARITY keyword

Specify  the character parity to be used on the currently selected port.

The choices for SET PARITY are NONE (the default), ODD, EVEN, MARK,  and

SPACE.  NONE means no parity processing is done, and the 8th bit of each

character can be used for data when transmitting binary files.

You  will  need to SET PARITY to ODD, EVEN, MARK, or possibly SPACE when

communicating with a system, or over a network, or through modems,  con-

centrators, multiplexers, or front ends that require or impose character

parity  on  the  communication line.  For instance, GTE Telenet requires

MS-DOS KERMIT                                                   Page 107

MARK parity.  If you neglect to SET PARITY when the communications equi-

pment requires it, the symptom may be that terminal emulation works par-

tially, and file transfer does not work at all.

If you have set parity to ODD, EVEN, MARK, or SPACE, then Kermit-MS will

request  that  binary files will be transferred using 8th-bit-prefixing.

If the other side knows how to do 8th-bit-prefixing (this is an optional

feature of the KERMIT protocol, and not all  implementations  of  KERMIT

have it), then binary files can be transmitted successfully.  If NONE is

specified,  8th-bit-prefixing will not be requested.  Note that there is

no advantage to using parity; it only slows Kermit file  transfer  down.

The  SET  PARITY  command  is  provided only to allow Kermit to adapt to

hardware that insists upon using parity.

SET PORT

Syntax: SET PORT number

On machines with more than one communications port, select the  port  to

use  for  file  transfer  and CONNECT.  This command lets you use a dif-

ferent asynchronous adapter, or switch between two or more  simultaneous

remote  sessions.    Subsequent  SET  BAUD, PARITY, HANDSHAKE, FLOW, and

LOCAL-ECHO commands will apply to this port only.  SET  PORT  1  selects

COM1, SET PORT 2 selects COM2.

SET REMOTE

Syntax: SET REMOTE ON or OFF

If  you  wish  to run Kermit-MS interactively through the back port, for

instance after the operator has done CTTY COM1, you must give  the  com-

mand SET REMOTE ON; this suppresses the file transfer display screen, so

that the display won't interfere with the file transfer itself.

SET RECEIVE

Syntax: SET RECEIVE parameter value

At  the  beginning of a protocol operation, request the remote Kermit to

use the given value specified parameter, or inform  Kermit-MS  that  the

remote Kermit will be using it.

PACKET-LENGTH   Ask  the  remote  Kermit  to  use  the specified maximum

length for packets that it sends to Kermit-MS.  The nor-

mal (and maximum) length is 94.   Use  this  command  to

shorten packets if the communication line is noisy; this

will  decrease  the probability that a particular packet

will be corrupted, and will  reduce  the  retransmission

overhead  when  corruption  occurs, but it will increase

the protocol overhead.

PADCHAR         Ask the remote Kermit to use the given character for in-

terpacket padding.  Kermit-MS should never  require  any

MS-DOS KERMIT                                                   Page 108

padding.

PADDING         Ask the remote Kermit to insert the given number of pad-

ding  characters  before  each  packet  it  sends.  This

should never be necessary.

START-OF-PACKET The remote Kermit will be marking the beginning of pack-

ets with something other than Control-A.  This  will  be

necessary  only  if the hosts or communication equipment

involved cannot pass a Control-A through as data.

TIMEOUT         Ask the remote Kermit to time out after the given number

of seconds if a packet expected from Kermit-MS  has  not

arrived.   Use this command to change the normal timeout

interval.

SET SEND

Syntax: SET SEND parameter value

PACKET-LENGTH   Use the specified maximum length for  outbound  packets.

Normally,  Kermit-MS uses whatever length the other Ker-

mit requests.

PADCHAR         Use the specified  character  for  interpacket  padding.

Some hosts may require some padding characters (normally

NUL or DEL) before a packet.

PADDING         How many padding characters to use between packets, nor-

mally zero.

QUOTE           Use  the  indicated  printable  character  for prefixing

(quoting) control characters and  other  prefix  charac-

ters.  The only reason to change this would be for send-

ing a very long file that contains very many "#" charac-

ters (the normal control prefix) as data.

START-OF-PACKET Mark the beginning of outbound packets with some control

character  other than Control-A.  This will be necessary

only if the remote host or the communication channel in-

volved cannot accept a Control-A as data.    The  remote

host  must have been given the corresponding SET RECEIVE

START-OF-PACKET command.

TIMEOUT         Change Kermit-MS's normal timeout interval; this command

is effective only if TIMER is set to be ON; it  is  nor-

mally   OFF  so  that  the  remote  KERMIT  can  control

timeouts.

MS-DOS KERMIT                                                   Page 109

SET TAKE-ECHO

Syntax: SET TAKE-ECHO ON or OFF

Specifies  whether  screen  display  should occur during implicit or ex-

plicit TAKE operations on MSKERMIT.INI or other Kermit-MS command files,

and during evaluation of macro definitions.  Handy for finding errors in

command files.

SET TIMER

Syntax: SET TIMER ON or OFF

Enable or disable the timer that is used during file transfer  to  break

the  deadlock  that  occurs when an expected packet does not arrive.  By

default, the timer is OFF, because Kermit-MS is usually used in conjunc-

tion with a mainframe that is doing its own timeouts.    During  a  file

transfer,  it  is  sufficient  for one side to do the timing out and the

mainframe is usually better equipped to adjust timeout  intervals  based

on  system  load or other conditions.  The timer should be set ON if you

are communicating with a system that cannot do  timeouts,  such  as  IBM

VM/CMS Kermit.

SET WARNING

Syntax: SET WARNING option

Specify  what to do when an incoming file has the same name as an exist-

ing file in the default directory of the default device.  If ON,  Kermit

will  warn  you  when  an incoming file has the same name as an existing

file, and automatically rename the incoming file (as  indicated  in  the

warning  message) so as not to destroy (overwrite) the pre-existing one.

If OFF, the pre-existing file is destroyed, even if  the  incoming  file

does not arrive completely.

10.3.6. The SHOW Command

Syntax: SHOW option

Currently,  most  parameters  that  may be altered with SET commands are

displayed by the STATUS command.  The SHOW command is used for  display-

ing macro definitions and key redefinitions.

The  SHOW  MACROS  command  displays  the  definitions  of all currently

defined macros.

The SHOW KEY command allows you to determine the scan code  produced  by

pressing  a  given  key,  so that you can construct a SET KEY command to

redefine the key.  If the key already has a redefinition in effect, that

too will be displayed.  In this example, a DEC Rainbow  user  determines

the  scan  code for the accent grave key, and then redefines that key to

send ESC:

MS-DOS KERMIT                                                   Page 110

Kermit-MS>show key

Press a key: `

Scan Code:  96

Definition:

Kermit-MS>set key scan 96

Definition string: \33

Kermit-MS>show key

Press a key: `

Scan Code:  96

Definition: \33

Kermit-MS>

The SHOW KEY command only works on certain systems.

10.3.7. Command Macros

Kermit-MS  provides  a  facility  for  combining commands into "macros."

Command macro definitions may be included  in  your  MSKERMIT.INI  file,

TAKEn  explicitly from a specified file, or typed interactively, and may

be invoked with the DO command.

THE DEFINE COMMAND

Kermit-MS command macros are constructed with the DEFINE command.    The

syntax is

DEFINE macro-name  [command [, command [, ...]]]

Any Kermit-MS commands may be included.  Example:

define telenet set parity mark, set baud 1200, connect

THE DO COMMAND

A  Kermit-MS  command  macro  is  invoked using the DO command.  For in-

stance, Kermit-MS comes with a  predefined  macro  to  allow  convenient

setup for IBM communications; to invoke it, you would type

do ibm

The  IBM macro is defined as "parity mark, handshake xon, local-echo on,

timer on".  You can delete or replace this definition by  adding  a  new

(perhaps null) definition, such as

define ibm parity even, handshake cr, local-echo on, timer on

or

define ibm

MS-DOS KERMIT                                                   Page 111

10.4. Terminal Emulation

When you issue the CONNECT command, your PC acts as a terminal connected

to  a  remote computer through the currently selected port.  The charac-

ters you type are sent out the port, and characters that arrive  at  the

port  are displayed on your screen.  If you have not previously issued a

SET PORT command, COM1 is used.  If you have SET LOCAL-ECHO ON  for  the

selected  port,  then Kermit-MS will display characters on the screen as

you type them.  If LOCAL-ECHO is OFF, then XON/XOFF flow control will be

done unless you have SET FLOW-CONTROL OFF.  If you have  SET  PARITY  to

anything  other  than NONE, Kermit-MS will add the appropriate parity to

each outbound character, and strip any parity from incoming ones.  While

CONNECTed, you can also communicate with an autodialer or "smart  modem"

to  control  the  communications line, hang it up, and the like; for in-

stance, typing +++ to a Hayes-like modem will allow you to  follow  that

by dialing or hangup commands.

If  Heath-19  emulation  is  being  done,  incoming  characters  will be

monitored for H19/VT52 escape sequences.  These will be interpreted  ac-

cording to the table in section 10.10.  In addition, keys on the numeric

keypad  will  send H19/VT52 sequences unless you disable this feature in

some way, for instance by pressing Num Lock on the IBM PC  keyboard,  or

with key redefinitions.

Caution:  On  some systems, such as the IBM PC and XT, Kermit-MS

accesses the screen memory memory directly  to  perform  certain

H19  emulation  functions  such  as  character insert/delete and

screen scroll.  Without direct screen memory access, these func-

tions would be painfully slow.    Although  Kermit-MS  has  been

tested  successfully  on a variety of monochrome and color adap-

ters and monitors, there may  be  combinations  for  which  this

method  could  cause  video problems, such as snow.  Should this

occur, you can alleviate the problem by setting  HEATH19  emula-

tion  OFF.    In  that  case,  however,  you remove not only the

problems, but also the desirable features  of  emulation.    But

Kermit-MS  does  permit  you  to load an external console device

driver, such as IBM's ANSI.SYS, to provide  any  desired  screen

control.

Here  are  the terminal emulation options for the systems presently sup-

ported by Kermit-MS:

System         EscChar   Cabilities   Terminal Service

IBM PC, XT       ^]      R M P K      Heath19 emulation

DEC Rainbow      ^]      R   P K      VT102 firmware

HP-150           ^]      R            HP-2623 firmware

Wang PC          ^A                   Wang firmware

Generic DOS      ^]                   Depends on system

Under Capabilities, R means rollback, M means mode line, P means printer

control, and K means key redefinition.

IBM PC/XT Kermit can disable Heath-19 emulation and use an external con-

sole device driver like ANSI.SYS instead.

When you first issue the CONNECT command, a message (on some systems, an

MS-DOS KERMIT                                                   Page 112

inverse  video  "mode line") will display the most important facts about

the connection you've just established, so that you can quickly diagnose

any problems.  The items displayed in the mode line include  the  escape

character,  port number, the baud rate, the parity, the echo, and how to

get help, for instance:

+---------------------------------------------------------------------+

|EscChar:^],Port:1,Baud:9600,Parity:None,Echo:Remote,Type ^]? for Help|

+---------------------------------------------------------------------+

The escape character is used to regain the attention of Kermit-MS.  When

you type the escape character, Kermit-MS waits for you to follow it with

a single character command.  For instance, the  single-character-command

"?"  produces  a  list  of  available single character commands, such as

this:

?   Help -- prints the available single-character commands.

C   Close the connection and return to Kermit-MS prompt level.

S   Show the status of the connection.

B   Send a BREAK signal to the port.

0   (the digit zero) Send a NUL (ASCII 0) to the port.

Q   Temporarily quit logging the remote session.

R   Resume logging the remote session.

M   Toggle the mode line, i.e. turn it off if it is on & vice versa.

^]  (or whatever you have set the escape character to be)

Typing the escape character twice sends one copy of it to the con-

nected host.

Typing any other character (except the space bar,  which  is  the  "null

command")  after  the escape character will cause Kermit-MS to beep, but

will do no harm.  The escape character can be changed to something other

than Control-Rightbracket by using the SET ESCAPE command.

Kermit-MS includes several advanced features  for  use  during  terminal

emulation,  including  screen scroll, printer control, and key redefini-

tions.

Screen Scroll

Kermit-MS provides several pages of screen memory, which may be scrolled

up and down using keys as follows:

Function            IBM PC/XT   Rainbow           HP-150

Screen Down         PgDn        PrevScreen        Prev

Line Down           Ctrl-PgDn   Ctrl-PrevScreen   Shift-UpArrow

Screen Up           PgUp        NextScreen        Next

Line Up             Ctrl-PgUp   Ctrl-NextScreen   Shift-DownArrow

Top of Memory       Home

Bottom of Memory    End

There is presently no way to assign these functions to other keys.

MS-DOS KERMIT                                                   Page 113

Printer Control

A  locally  attached  printer may be controlled in the normal manner, on

most systems.  Pushing the "Print Screen" key (shifted on some  systems)

will  cause the current contents of the screen to be printed or spooled;

holding down CTRL while depressing Print Screen will start or  stop  the

spooling  of  incoming  characters to the printer.  ^P or ^N are sent to

the host during terminal emulation, and do not toggle printing, as  they

do when you're talking directly to DOS.

CTRL-Print-Screen  can be simulated with the Kermit-MS LOG PRN and CLOSE

commands.

Key Redefinitions

Key redefinitions are useful for defining "keystroke  macros"  of  login

sequences,  frequently issued commands, and so forth, and for setting up

the terminal for use with host resident software designed to  work  with

terminals  that send predefined sequences from their function keys.  For

instance, here's a key redefinition file for arranging the  DEC  Rainbow

keyboard into the normal ASCII keyboard layout:

; Make shift-comma send a left angle bracket

set key scan 556

<

; Shift-period sends a right angle bracket

set key scan 558

>

; Accent grave is where ESC is supposed to be

set key scan 96

\33

; Put accent grave on the ESC function key

set key f11

`

The  SET  KEY  facility may be used provide the PC with a "meta" key for

use with editors like EMACS or TVEDIT that can use "meta characters"  as

commands.   A meta key is a shift key whose effect is to turn on the 8th

(parity) bit of the character.  For instance, on the  IBM  PC  the  scan

codes  produced  by  holding  down  ALT  together with other keys can be

determined using SHOW KEY, and then 8-bit ASCII equivalents with the 8th

bit turned on can be defined using SET KEY; if the scan code produced by

typing ALT-a, i.e. the letter "a" (ASCII 141, octal) with  the  ALT  key

held  down,  is  2078  (decimal),  you  would set the META equivalent to

141+200=341 (octal), or "\341" in octal SET KEY notation:

Kermit-MS>sho key

Press a key: ALT-a

Scan Code: 2078

Definition:

Kermit-MS>set key scan 2078

Definition String: \341

Whenever you type ALT-a with this definition in effect,  Kermit-MS  will

transmit octal 341, rather than 141.

MS-DOS KERMIT                                                   Page 114

10.5. Installation of Kermit-MS

by Bill Catchings, Columbia University

If you already have Kermit on your PC, you can use it to obtain new ver-

sions  of Kermit-MS when they appear on the central system at your site.

If you do not have Kermit or any other reliable file capture facility on

your PC, and there is no one from whom you can borrow a floppy  disk  to

copy  Kermit,  then  you  should read the following instructions for in-

itially "bootstrapping" Kermit-MS from a mainframe where  it  is  stored

onto your microcomputer.

There  are  at  least  three methods of initially getting Kermit-MS onto

your PC:

1. Try again to find a copy on diskette.

2. Use another file capture facility to get it.

3. Type in and run a bootstrapping program.

10.5.1. Try Again To Find A Kermit Disk

Before explaining how to bootstrap Kermit onto  your  PC,  a  disclaimer

must  be made.  Although a fair amount of thought and time has gone into

these procedures, they are far from error free.  If they were foolproof,

there would be no need for a protocol such as Kermit.   There  are  many

places  where  things can go wrong, from something as simple as a typing

mistake to something as unavoidable and probably inevitable  as  a  com-

munications  line  failure.   By far the easiest and best way to install

Kermit is from a floppy disk.  Before you embark on any of the following

procedures it is a good idea to check once again for a diskette to copy,

even it it contains an old version  of  Kermit.    The  time  you  spend

searching  is  likely to be far less frustrating than the time you spend

trying to bootstrap Kermit by the methods described below.

10.5.2. Bootstrapping From the Communication Line

If you can't find a diskette with Kermit on  it,  there  are  two  other

methods  available  for  bootstrapping  MS-DOS Kermit onto your PC.  The

first method is to use a file capture  method  or  other  file  transfer

protocol  to  transfer  the file to your PC.  Some systems come supplied

with facilities like this,  and  various  public  domain  or  commercial

packages  are available.  The second method requires you to type in your

own downloading program.

In either case, you must transmit the file  from  the  system  where  it

resides  over a communication line and into your PC.  Since version 2 of

MS-DOS Kermit is much larger  than  version  1,  it  comes  with  a  new

bootstrapping  procedure in which the executable program is encoded much

more compactly than in the earlier "fix" files.  The new encoding  packs

3 .EXE file bytes into 4 printable characters in the MSxxx.BOO file, and

also compresses adjacent zero bytes (of which there are many).  The .BOO

file  contains only printable characters, to ensure that downloading can

MS-DOS KERMIT                                                   Page 115

take place regardless of parity or other peculariaries of the communica-

tion channel.

10.5.2.1. Use An Existing File Capture Facility

In  the rest of this discussion of bootstrapping, the host-resident boot

.BOO file will be referred to as MSKERMIT.BOO.  In fact, the actual name

will depend on which MS-DOS system you are using -- MSIBMPC.BOO for  the

IBM PC or XT, MSRB100.BOO for the Rainbow 100, etc.

Use  your  file  capture  facility,  whatever it may be, to get the file

MSKERMIT.BOO onto your PC's disk, but first make sure  you  have  enough

room  for  it.    Once  the file is on your disk, you must run the BASIC

program MSPCTRAN.BAS to decode the file  back  into  KERMIT.EXE.    This

program can be downloaded by the same method you used with MSKERMIT.BOO.

The  program  looks  on  your  current  disk  and directory for the file

MSKERMIT.BOO and outputs KERMIT.EXE to the same place.    KERMIT.EXE  is

about 80K bytes, so make sure there is space for it on your disk or else

you  will  have  to start the program over.  Since the program will take

about twenty minutes to completely translate the file you will  want  to

avoid running it more than once.

10.5.2.2. Type In Your Own Bootstrap

If  you  can't  find  some  method for downloading the .BOO file and the

BASIC program, the second way of bootstrapping  Kermit  is  to  use  the

programs   MSPCBOOT.BAS   and  MSBOOT.FOR  to  download  via  your  PC's

asynchronous port from your host and  translate  it  directly,  "on  the

fly."    You  run  the  program MSBOOT.FOR on your host and then run the

program MSPCBOOT.BAS in BASIC on your PC.  The FORTRAN program sends the

encoded .EXE file to the BASIC program, which decodes it and  stores  it

in  executable  form  on  your  current directory as KERMIT.EXE.  A very

rudimentary form of error checking is done to allow obviously  corrupted

records to be retransmitted.  Follow this procedure:

1. First,  you  must  establish a connection from your PC to the

host system.   A  high  speed  connection  is  preferable;  a

"clean"  line is preferable to a noisy one.  In fact, a clean

line is essential for this procedure.  You must  be  able  to

log  in  to the host system over this connection.  If your PC

already has a terminal emulation facility, use that.  If not,

you might need to put your PC next to a real terminal and use

that for logging in, then switch the connector to the  PC  at

the  crucial  moment.  If you are using a terminal, make sure

the terminal and PC have their communication ports set to the

same speed.

2. Ensure that the files MSBOOT.FOR and MSKERMIT.BOO are present

on the host system.  MSBOOT.FOR is listed below, in case  you

need to type it in.

3. Get  back  to  your PC and type in MSPCBOOT.BAS on your PC; a

listing appears below.  There is no need to type in the  com-

ments (anything following an apostrophe); they are only there

MS-DOS KERMIT                                                   Page 116

to  clarify  what the program is doing.  Check very carefully

for errors.  You should check line 80 in the program  to  see

that  it reflects the way your system is actually set up.  If

necessary, substitute the correct baud rate for the  supplied

rate of 9600, and if you are not using COM1: make that change

as  well.   If you are downloading from an IBM or other half-

duplex mainframe,  leave  line  1000  as  it  is;  otherwise,

replace it by a RETURN statement.  If you type it in directly

to BASIC make sure you save the program before you run it, so

you won't have to type it in again in case of error.

4. Get  back  to  your host system and compile MSBOOT.FOR, if it

needs compiling.  Define logical unit numbers 5 and 6  to  be

the  controlling  terminal, and logical unit 7 to be the file

MSKERMIT.BOO.  On VAX/VMS systems,  for  example,  use  these

commands:

$assign sys$input for005

$assign sys$output for006

$assign mskermit.boo for007

On a DECSYSTEM-20, do:

@define 5: tty:

@define 6: tty:

@define 7: mskermit.boo

On a DECsystem-10, do something like this:

.assign tty: 5:

.assign tty: 6:

.assign dsk: 7:

.rename for007.dat=mskerm.boo

On an IBM system under VM/CMS, do this:

.filedef 5 term ( lrecl 80 recfm v

.filedef 6 term ( lrecl 80 recfm v

.filedef 7 disk mskermit boo ( lrecl 77 recfm f perm

5. Set your host system up for downloading:

- Ensure  that  your terminal does not automatically pause

at the end of a screenful of output.  For instance, on a

DEC-20 you would issue the command  "terminal  no  pause

end-of-page".

- Do  whatever  you can to disable messages from appearing

at your terminal while these programs are running.  This

would include messages from other users, mail  notifica-

tion,  alarms  or alerts, system messages, and so forth.

Such messages will interfere  with  the  procedure,  and

probably render the result useless.

- You  should  put  your  host terminal in "local echo" or

"half duplex" mode, if possible.

MS-DOS KERMIT                                                   Page 117

6. Start the MSBOOT program on your host system.

7. Get  back  to  the  PC.    If you have been using a terminal,

switch the connector to the PC.

8. Now run the BASIC program, MSPCBOOT.BAS.  This procedure will

take at least twenty minutes and possibly longer depending on

line speed.  Watch your modem and/or disk  lights  for  reas-

surance that something is happening.

By  using one of these installation methods, you should now have a work-

ing version of Kermit.  If you experience any  problems  or  quirky  be-

havior  with  the  program,  it's possible that some part of it was cor-

rupted during the downloading procedure.   Perhaps  enough  usable  code

remains  to  allow  you to transfer MSKERMIT.EXE from the host.  If not,

you will have to repeat the downloading procedure.

Once you have Kermit-MS on your disk, you should make the disk available

to other users for copying, so that they can be  spared  the  tedium  of

this bootstrap procedure.

Here  is  a  listing  of MSPCBOOT.BAS.  The "outdented" PRINT statements

with line numbers ending in 5 may  be  included  if  you  want  incoming

records  to  be  displayed on the screen.  You don't need to include the

comments.

1    'Run this program on the PC in conjunction with a Fortran program

2    '(MSBOOT.FOR) on the mainframe to download Kermit to the PC.  This

3    'program will run for about thirty minutes, depending on line speed.

4    ' Bill Catchings, June 1984

5    ' Columbia University Center for Computing Activities

10   t$ = time$                         ' Save the time.

20   defint a-z                         ' All integer to gain some speed.

30   n$ = chr$(0)

40   z = asc("0")

50   t = asc("~")-z

60   def fnuchr%(a$)=asc(a$)-z

70   open "com1:9600,s,7,1,cs,ds,cd" as #1

100  print#1,"O ,"                      ' Char constants "O", " " and ","

110  input#1,f$

120  if len(f$) < 5 then goto 110       ' In case the host echos the ACK.

130  input#1,n

135 print f$+" "+str$(n)

140  if n > 20 then goto 900

150  open f$ for output as #2

160  print "Outputting to "+f$

170  goto 300                           ' Correct version of the file.

200  gosub 1000                         ' Do turnaround char processing

210  print#1,"NO"                       ' Tell host data was incorrect.

220  goto 320

MS-DOS KERMIT                                                   Page 118

300  gosub 1000                         ' Do turnaround char processing

310  print#1,"OK"                       ' Say the line was all right.

320  input#1,x$

330  if len(x$) < 5 then goto 320       ' In case the host echos ACK/NAK

340  input#1,n

345 print x$+" "+str$(n)

350  if len(x$) <> n then goto 200      ' Length doesn't match, NAK it.

360  if x$ = "&&&&&&&&&&" then goto 800 ' End of file?

370  y$ = ""                            ' Set output string to null.

380  goto 500

400  print#2,y$;                        ' Print the output string.

410  goto 300                           ' Go get another line.

500  if len(x$) = 0 goto 400            ' Done with input string?

510  a = fnuchr%(x$)

520  if a = t then goto 700             ' Null repeat character?

530  q$=mid$(x$,2,3)                    ' Get the quadruplet to decode.

540  x$=mid$(x$,5)

550  b = fnuchr%(q$)

560  q$ = mid$(q$,2)

570  c = fnuchr%(q$)

580  q$ = mid$(q$,2)

590  d = fnuchr%(q$)

600  y$ = y$ + chr$(((a * 4) + (b \ 16)) and 255) ' Decode the quad.

610  y$ = y$ + chr$(((b * 16) + (c \ 4)) and 255)

620  y$ = y$ + chr$(((c * 64) + d) and 255)

630  goto 500                           ' Get another quad.

700  x$ = mid$(x$,2)                    ' Expand nulls.

710  r = fnuchr%(x$)                    ' Get the number of nulls.

715 print " Null: ",r

720  x$ = mid$(x$,2)

730   for i=1 to r                      ' Loop, adding nulls to string.

740   y$ = y$ + n$

750   next

760  print#2,y$;                        ' Print the nulls.

770  y$ = ""                            ' Clear the output buffer.

780  goto 500

800  print "Processing complete, elapsed time: "+t$+" to "+time$

810  print "Output in "+f$

820  close #1,#2

830  goto 9999

900  print "?The format of the BOO file is incorrect"

910  goto 820

1000 x$ = input$(1,#1)          ' Make this line RETURN for full-duplex

1010 if x$ <> chr$(17) then goto 1000   ' Loop for a turn around char.

1020 return

9999 end

MS-DOS KERMIT                                                   Page 119

Here  is a listing of MSBOOT.FOR, in case you can't find it on your host

system:

C     This Fortran program should be run on the mainframe in conjunction

C     with a Basic program (MSPCBOOT.BAS) on the PC to transfer

C     MSKERMIT.BOO to the PC and translate it into KERMIT.EXE.  This

C     program uses a very rudimentary technique to try to insure that

C     the characters it sends arrive correctly.  It just sends a count

C     of the number of characters sent after each line.  In this way any

C     errors of character loss or insertion will be caught.  If a

C     character is just corrupted it will not be caught.  Hopefully if

C     this happens it will be in a non-critical part of the KERMIT.EXE

C     file.  The reason a simple checksum was not used was so that this

C     program could run on machines using either EBCIDIC or ASCII

C     characters.  This program should take about thirty minutes to run.

C     This program assumes that 5 and 6 are directed to the terminal and

C     7 is directed to the file MSKERMIT.BOO.

INTEGER LINE(77), ACK(4), CHECK, OK, SPACE, COMMA

WRITE(6,100)

100   FORMAT(' Ready to transfer data, now run MSPCBOOT.BAS on the PC.')

C     Get characters for constants (character constants are rough in

C     some FORTRANs!)

READ (5,200) OK, SPACE, COMMA, ACK

200   FORMAT(4A1)

GO TO 30

C     Get terminal handshake.

10    READ (5,200)ACK

C     Did the other side like it?  (Did they send OK?)

IF (ACK(1) .NE. OK) GO TO 50

C     Yes, get new line from file.

20    READ (7,300,END=99)LINE

300   FORMAT(77A1)

C     Count the characters as some rudimentary check for noise.

I = 1

30    IF (LINE(I) .EQ. SPACE) GO TO 40

I = I + 1

GO TO 30

C     Put in a comma followed by the count.

40    LINE(I) = COMMA

C     Write to TTY.

50    WRITE (6,400)LINE,I-1

400   FORMAT(' ',77A1,I2)

GOTO 10

MS-DOS KERMIT                                                   Page 120

C     Send good-bye message.

99    WRITE (6,500)

500   FORMAT(' ',10('&'),',10')

STOP

END

10.6. Compatibility with Older Versions of MS-DOS Kermit

MS-DOS  Kermit  supports  many  different systems.  Like CP/M-80 KERMIT,

this support was added to the program piecemeal, at  many  sites,  using

conditional assembly.  However, before allowing the program to grow into

a  complicated  monolith like CP/M-80 KERMIT, we have broken the program

up into separate modules,  with  system  dependencies  isolated  into  a

single  module consisting of compact collections of low-level primitives

for console and port i/o.

The last monolithic (single source file) release of  MS-DOS  Kermit  was

1.20.    To this and earlier versions was added support for systems like

the Seequa Chameleon, the Tandy 2000, the Victor 9000, the  Heath/Zenith

100,  and  others.    Eventually,  support  for these systems may be in-

tegrated with the new modular version.  Meanwhile, implementations based

on these old versions will have at least the  following  incompatibilies

from the version described here:

- RECEIVE filespec is used instead of GET filespec.  There is no

GET  command  in  older  versions, and no way to specify a new

name for an incoming file.

- No LOCAL or REMOTE commands.

- No 8th-bit  prefixing,  repeat  counts,  CRCs  or  2-character

checksums.

- No TAKE or initialization files.

- No command macros or command line arguments.

- No terminal session logging.

and others, depending on the specific version.

10.7. What's Missing

Kermit-MS  has plenty of room for improvement.  Features that need to be

improved or added include:

- A built-in facility for sending files "raw" to the remote sys-

tem, obeying current settings for parity, flow control,  hand-

shake,  and so forth.  This might include a script interpreta-

tion  facility  to  allow  remote  sessions  to  be  conducted

automatically.  For the present, this can be accomplished with

a  user-supplied  program  invoked with the Kermit-MS RUN com-

mand.

- Additional  functionality  when   running   in   server   mode

--  directory  listings,  file deletion, execution of DOS com-

mands, etc.

MS-DOS KERMIT                                                   Page 121

- More commands when talking to remote servers -- REMOTE RENAME,

COPY, STATUS, WHO, etc.

- Filename  conversion  options  (normal form, handling of fully

qualified filespecs, etc.).

- Transaction file logging.

- Improved command parsing; for instance, accept default  values

for omitted trailing fields.

- A better built-in help facility.

- Support for Kermit file attribute packets.

- The Kermit-MS program is quite large.  Much of the size is due

to  the deliberate decision to provide support for versions of

MS-DOS prior to 2.0.  At some point, this  support  should  be

removed.    This  will not only reduce the size of the program

considerably, but also provide much more flexibility.

10.8. Program Organization

Kermit-MS version 2 is composed of separate assembler source files,  as-

sembled separately, and linked together.  The modules are:

System/Device Independent:

MSKERM.ASM  Main program

MSSEND.ASM  File sender

MSRECV.ASM  File receiver

MSSERV.ASM  Server operation

MSFILE.ASM  File i/o

MSCMD.ASM   Command parser

MSTERM.ASM  CONNECT command

MSCOMM.ASM  Communications port buffering & flow control

MSSET.ASM   SET, SHOW, and STATUS commands

MSDEFS.H    Data structure definitions and equates

System/Device Dependent:

MSXxxx.ASM  System-dependent code for system xxx

MSYxxx.ASM  System-dependent screen and keyboard code

MSZxxx.ASM  Modem control (modem-dependent)

The  modular  organization  allows  easier  modification of the program,

quicker transfer  of  modified  portions  from  system-to-system.    The

modules  are  designed  to be well-defined and self-contained, such that

they can be easily replaced.  For instance, someone who prefers  windows

and  mice  to  typing  commands could replace the command parsing module

without having to worry about the effect on the other modules.

To assemble any of the kermit modules, file  MSDEFS.H  must  be  on  the

default disk.

MS-DOS KERMIT                                                   Page 122

All  the  Kermit  implementations  require  the  modules  MSCMD, MSCOMM,

MSFILE, MSKERM, MSRECV, MSSEND, MSSERV, MSSET, MSTERM.

The IBM PC version requires MSXIBM and MSYIBM as well.

The Rainbow version requires MSXRB and MSXDMB; MSXDMB must be the  first

object  file  given  to  the  linker for Kermit to link properly for the

Rainbow.

The HP150 version requires MSXHP150, the Wang version  requires  MSXWNG,

and the generic version requires MSXGEN.

Once  all the required object modules exist, they may be linked together

to produce Kermit.  For example, on the Rainbow:

A>link

Microsoft Object Linker V2.00

(C) Copyright 1982 by Microsoft Inc.

Object Modules [.OBJ]: msxdmb mskerm msxrb mscomm msset mssend +

msrecv msserv msfile msterm mscmd

Run File [MSXDMB.EXE]: kermit

List File [NUL.MAP]: kermit

A>

10.9. Adding Support For New Systems

You can bring Kermit-MS to systems that are not explicitly supported  in

one  of two ways -- attempt to run the "generic" MS-DOS Kermit on it, or

add explicit code to support your system.

10.9.1. Generic MS-DOS Kermit

To get started with Kermit on a new system, try running "generic" MS-DOS

Kermit; in many cases, it will run as  is.    The  generic  version  ac-

complishes  all  its  port and console i/o through DOS calls, and during

terminal connection does not attempt to emulate any particular  kind  of

terminal.    In  some  cases, the generic version may still require some

fiddling to run on a new system; for instance, different  systems  refer

to  their  communication  ports in different ways -- COM1, AUX, etc.  It

attempts to do this automatically by trying various DOS file handles for

the communication port, and asking you to supply  one  if  it  does  not

succeed.

Generic MS-DOS Kermit will probably run no faster than 1200 baud, and it

only works with DOS 2.0 or later.

MS-DOS KERMIT                                                   Page 123

10.9.2. Adding System-Dependent Code

The following is a guide to the system dependent module of Kermit-MS.

SPECIFICATION FOR KERMIT SYSTEM-DEPENDENT MODULES

by Jeff Damens, Columbia University

All  the system-independent global data structures used in Kermit-MS are

defined in the file MSDEFS.H.

The routine MSXxxx.ASM contains system-dependent support for system xxx,

except for terminal emulation, which is in MSXxxx.ASM, described below.

The routines in the MSX module may change any registers  but  the  stack

pointer  and  segment registers, unless otherwise noted.  A routine that

returns via a RET instruction is said to return normally; a routine that

skip returns is one that returns to three bytes past the  normal  return

address.

Global variables that must be defined in the system-dependent module:

XOFSNT          byte.   This should be set to a non-zero value if we are

doing flow control and have sent an  XOFF  character  to

the remote host, zero otherwise.

MACHNAM         byte.    A  $-terminated  string identifying the machine

this version of Kermit is for; it is printed when Kermit

starts up.

SETKTAB         byte.  A keyword table associating terminal key names to

16-bit scan code values, used in the  set  key  command.

If  the  kermit  version  can  accept  arbitrary decimal

values as scan codes, the word "SCAN" should  appear  in

the  table with a scan value of -1.  If key redefinition

is not implemented, the first byte of the  table  should

be a zero.

SETKHLP         byte.    A  $-terminated  string to be printed when ? is

typed in the SET KEY command.  This is usually simply  a

list  of  the  key  names  in  SETKTAB.  SETKHLP must be

defined even if key redefinition is not implemented,  to

satisfy  the  linker;  if key redefinition is not imple-

mented, SETKHLP will never be displayed.

COUNT           word.  The number of  characters  in  the  serial  input

buffer,  if  known.  This is how Kermit knows to send an

XON if the serial handler has sent an XOFF.  If the num-

ber of characters  in  the  buffer  isn't  known,  COUNT

should be 0.

These  are  the required entry points for the system dependent dependent

module MSXxxx.ASM.

MS-DOS KERMIT                                                   Page 124

SERINI

Parameters      None.

Returns         Normally, no return value.

Description     Perform  any initialization that must be done before the

serial port can be used, including  setting  baud  rate,

interrupt  vectors, etc.  Parity and baud rate should be

set according to the values in the  PORTINFO  structure.

The  external  variable  PORTVAL  points to the PORTINFO

structure for the current port.    Calling  SERINI  more

than  once  without an intervening call to SERRST should

have no effect.

SERRST

Parameters      None.

Returns         Normally, no return value.

Description     Undoes any  initialization  done  by  SERINI,  including

resetting  the serial port, restoring any interrupt vec-

tors changed by SERINI, etc.   Calling  this  more  than

once  without  an  intervening  call to SERINI should be

harmless.

CLRBUF

Parameters      None.

Returns         Normally, no return value.

Description     Remove and discard from the serial port's  input  buffer

any characters sent by the remote host that have not yet

been  read  by Kermit, and set COUNT to 0.  This is used

before a file transfer to flush NAK's that accumulate in

the buffer when the remote host is in server mode.

MS-DOS KERMIT                                                   Page 125

OUTCHR

Parameters      A character in AH.

Returns         Skip  returns  if  the  character  has been transmitted;

returns normally if the character can not be transmitted

because of a hardware error.

Description     Sends the character in AH  out  the  currently  selected

serial  port.    OUTCHR can assume that SERINI will have

been called previously. OUTCHR should call the  external

routine  DOPAR to set the parity of the character if the

communications  hardware   doesn't   automatically   set

parity.    Flow  control should be honored; the external

variable PORTVAL contains a pointer to a PORTINFO struc-

ture (as defined in  MSDEFS.H)  containing  the  current

flow control definitions.

COMS

Parameters      None.

Returns         Normally  if  a parse error is encountered, skip returns

otherwise.

Description     Called by the SET PORT command.  On a machine with  mul-

tiple  serial  ports,  COMS should parse for the name or

number of a serial port and make that the port  used  by

succeeding  calls to SERINI, PRTCHR, OUTCHR, and SERRST.

It should set the external variable PORTVAL to point  to

one  of the external port structures PORT1 or PORT2, and

set COMFLG in the FLAGS structure to 1 for port  one,  0

for  port  2.    For  implementations  that use only one

serial port, COMS should print a message to that  effect

and skip return.

VTS

Parameters      None.

Returns         Normally  if  a parse error is encountered, skip returns

otherwise.

Description     Parses for an ON or OFF, sets HEATH-19  emulation  while

in terminal emulation appropriately.  The VTFLG field of

the  FLAGS  structure should be set non-zero if HEATH-29

emulation is on, zero otherwise.  If HEATH-19  emulation

is not done, VTS should print a message and skip return.

MS-DOS KERMIT                                                   Page 126

DODEL

Parameters      None.

Returns         Normally, no return value.

Description     Erases the character immediately to the left of the cur-

sor from the screen, then backs up the cursor.

CTLU

Parameters      None.

Returns         Normally, no return value.

Description     Move the cursor to the left margin, then clear the line.

CMBLNK

Parameters      None.

Returns         Normally, no return value.

Description     Clears the screen and homes the cursor.

LOCATE

Parameters      None.

Returns         Normally, no return value.

Description     Homes the cursor.

LCLINI

Parameters      None.

Returns         Normally, no return value.

Description     Performs any system-dependent initialization required by

this implementation.

MS-DOS KERMIT                                                   Page 127

PRTCHR

Parameters      None.

Returns         Normally,  with  the  next  character from the currently

selected serial port in AL.  Skip returns if no  charac-

ter is available.

Description     Reads  the  next character from the current serial port.

PRTCHR can assume SERINI has been called previously, and

should handle flow control correctly.

DOBAUD

Parameters      None.

Returns         Normally, no return value.

Description     Sets the baud rate for the current port.  The baud  rate

should  be  obtained from the BAUD field of the PORTINFO

structure, pointed to by the external variable PORTVAL.

CLEARL

Parameters      None.

Returns         Normally, no return value.

Description     Clears from the cursor to the end of the current line.

DODISK

Parameters      None.

Returns         Normally, no return value.

Description     Sets the external variable DRIVES to the number of  disk

drives attached to the machine.

GETBAUD

Parameters      None.

Returns         Normally, no return value.

Description     Store  current  baud rate of the currently selected port

in the BAUD field of  the  current  PORTINFO  structure,

which  is pointed to by PORTVAL.  If the baud rate is to

default to a particular value, this  routine  can  store

that value into the BAUD field instead.

MS-DOS KERMIT                                                   Page 128

BEEP

Parameters      None.

Returns         Normally, no return value.

Description     Rings the terminal bell.

PUTHLP

Parameters      A pointer to a string in AX.

Returns         Normally, no return value.

Description     Writes  the  null-terminated  string  given in AX to the

terminal.  This is used to display help and status  mes-

sages.  The IBM and Rainbow versions write the string in

a reverse video box.

PUTMOD

Parameters      A pointer to a string in AX.

Returns         Normally, no return value.

Description     Writes  the  null-terminated  string  given in AX to the

last line of the screen, in inverse video if possible.

CLRMOD

Parameters      None.

Returns         Normally, no return value.

Description     Clears the line written by PUTMOD.

POSCUR

Parameters      Row in DH, column in DL.

Returns         Normally, no return value.

Description     Positions the cursor to the row and column given in  DX.

Rows and columns both originate at 0 (not 1!).

MS-DOS KERMIT                                                   Page 129

SENDBR

Parameters      None.

Returns         Normally, no return value.

Description     Send a break to the current serial port.

SHOWKEY

Parameters      Pointer  to  a  terminal  argument block in AX (see TERM

below).

Returns         Normally, with a string pointer in AX and the length  of

the string in CX.

Description     Called  by  the  SHOW KEY command.  Reads a key from the

terminal    and    returns    a    string     containing

implementation-dependent  information  about the key. In

the  usual  case,  the   string   contains   the   key's

(machine-dependent)  scan code, and the key's definition

(if any) from the terminal argument block.   The  length

of  the  returned  string should be returned in CX.  The

string may contain any characters;  unprintable  charac-

ters  will be quoted when the string is printed.  If the

implementation does not support key redefinition,  SHOW-

KEY may return a static string saying so.

TERM

Parameters      Pointer to terminal argument block in AX.

Returns         Normally, no return value.

Description     Do terminal emulation, based on argument block described

below...

The  terminal emulator is supplied in the file MSYxxx.ASM.  The terminal

argument block passed to the terminal emulator has the following fields:

FLGS            Byte containing flags.  Flags are:

SCRSAM (80H)    If on, the terminal  emulator  shouldn't

re-display the screen when entered.

CAPT (40H)      Capture  output.    If  on,  the routine

passed in field  CAPTR  is  called  with

each character sent to the screen.

EMHEATH (20H)   Emulate a Heath-19 terminal if on.

MS-DOS KERMIT                                                   Page 130

HAVTT (10H)     A key redefinition table is present.

TRNCTL (08H)    Print  control character X as ^X (useful

for debugging).

MODOFF (04H)    Do not display emulator mode line if on.

LCLECHO (01H)   Echo keyboard characters on  the  screen

in addition to sending them to the port.

PRT             Port  to  use  for terminal emulation, used only in mode

line.  This is just a copy of COMFLG in FLAGS.

COLS            Number of columns on screen.

ROWS            Number of rows on screen.

CAPTR           Routine to call to  with  each  character  sent  to  the

screen if CAPT flag is on.  Characters are passed in AL.

BELLD           Bell divisor (used only on IBM).

KLEN            Number  of keys in key redefinition table, if HAVTT flag

is on.

KTAB            Address of key redefinition table.  The key redefinition

table is a table  of  KLEN  16-bit  scan  codes.    Each

(machine  dependent)  scan code represents a key that is

redefined.

KRPL            Address of key replacement table.  The  key  replacement

table  parallels  the  key  redefinition  table given in

KTAB.  Entries  in  the  replacement  table  are  16-bit

pointers  to  redefinitions.    Each  redefinition has a

one-byte length, followed by the definition.

ESCC            Escape character (single byte).  When this character  is

typed to the emulator, it should return.

BAUDB           byte.    Bits  describing  the  baud  rate  so it can be

printed on the mode line.  This is a copy  of  the  BAUD

field in the PORTINFO structure.  Currently used only on

the IBM.  See MSDEFS.H for possible values.

PARITY          byte.    Current parity to print on the mode line.  This

is a copy of PARFLG in the  PORTINFO  structure.    Cur-

rently  used only on the IBM.  See MSDEFS.H for possible

values.

MS-DOS KERMIT                                                   Page 131

10.10. Heath/Zenith-19 Control Codes

The  Heath/Zenith-19  terminal is equivalent to the DEC VT52 with exten-

sions for line and character insertion and deletion.  Items marked  with

an asterisk are not currently supported by Kermit-MS H19 emulation.

Cursor Functions

Sequence    Mnemonic    Definition

ESC H      HCUH        Cursor Home

ESC C      HCUF        Cursor Forward

ESC D      HCUB        Cursor Backward

ESC B      HCUD        Cursor Down

ESC A      HCUU        Cursor Up

ESC I      HRI         Reverse Index

*ESC n      HCPR        Cursor Position Report

*ESC j      HSCP        Save Cursor Position

*ESC k      HRCP        Set Cursor to Previously Saved Position

ESC Y      HDCA        Direct Cursor Addressing, 1-based:

31+line# 31+col# (same as VT52)

Erasing and Editing

Sequence    Mnemonic    Definition

ESC E      HCD         Clear Display (Shift Erase)

ESC b      HBD         Erase Beginning of Display

ESC J      HEOP        Erase to End of Page (Erase Key)

ESC l      HEL         Erase Entire Line

ESC o      HEBL        Erase Beginning of Line

ESC K      HEOL        Erase to End of Line

ESC L      HIL         Insert Line

ESC M      HDL         Delete Line

ESC N      HDCH        Delete Character

ESC @      HEIM        Enter Insert Character Mode

ESC O      HERM        Exit Insert Character Mode

Configuration

Sequence    Mnemonic    Definition

*ESC z      HRAM        Reset to Power-Up Configuration

*ESC r Bn   HMBR        Modify Baud Rate: Bn=

A=110,  B=150,  C=300, D=600,  E=1200,

F=1800, G=2000, H=2400, I=3600, J=4800,

K=7200, L=9600, M=19200

*ESC x Ps   HSM         Set Mode(s): Ps=

1 = Enable 25th line

2 = No key click

3 = Hold screen mode

4 = Block cursor

5 = Cursor off

MS-DOS KERMIT                                                   Page 132

6 = Keypad shifted

7 = Alternate keypad mode

8 = Auto line feed on CR

9 = Auto CR on line feed

*ESC y Ps   HRM         Reset mode(s): Ps=

1 = Disable 25th line

2 = Enable key click

3 = Exit hold screen mode

4 = Underscore cursor

5 = Cursor on

6 = Keypad unshifted

7 = Exit alternate keypad mode

8 = No auto line feed

9 = No auto CR

*ESC <      HEAM        Enter ANSI Mode

Modes of Operation

Sequence    Mnemonic    Definition

*ESC [      HEHS        Enter Hold Screen Mode

*ESC \      HXHS        Exit Hold Screen Mode

ESC p      HERV        Enter Reverse Video Mode

ESC q      HXRV        Exit Reverse Video Mode

*ESC F      HEGM        Enter Graphics Mode

*ESC G      HXGM        Exit Graphics Mode

*ESC t      HEKS        Enter Keypad Shifted Mode

*ESC u      HXKS        Exit Keypad Shifted Mode

*ESC =      HAKM        Enter Alternare Keypad Mode

*ESC >      HXAM        Exit Alternate Keypad Mode

Additional Operations

Sequence    Mnemonic    Definition

*ESC }      HDK         Keyboard Disable

*ESC {      HEK         Keyboard Enable

ESC v      HEWA        Wrap Around at End of Line

ESC w      HXWA        Discard at End of Line

ESC Z      HID         Identify as VT52 (ESC / K)

*ESC ]      HX25        Transmit 25th Line

*ESC #      HXMP        Transmit Page

The  Heath-19  transmits  the  following  sequences,  but  it will not

respond to them if they are received.  Kermit-MS  will  transmit  them

only if they are programmed with SET KEY.

ESC S      HF1         Function Key #1

ESC T      HF2         Function Key #2

ESC U      HF3         Function Key #3

ESC V      HF4         Function Key #4

ESC W      HF5         Function Key #5

MS-DOS KERMIT                                                   Page 133

ESC P      HF7         Function Key #7

ESC Q      HF8         Function Key #8

ESC R      HF9         Function Key #9

CP/M-80 KERMIT                                                  Page 134

11. CP/M-80 KERMIT

Program:    Bill Catchings, Columbia University, with contributions from

Bernie   Eiben   (DEC),   Nick  Bush  (Stevens),  John  Bray

(University of Tennessee), Bruce Tanner (Cerritos  College),

Greg  Small  (University  of  California at Berkeley), Kimmo

Laaksonen (Helskini  University  of  Technology),  and  many

others.

Language:   8080 Assembler or MAC80

Version:    3.9A

Date:       6 June 1984

Documentation:

Frank da Cruz, Columbia University

KERMIT-80 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  No

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            Yes

Terminal emulation:                Yes, VT52 and others

Communication settings:            Yes; duplex, parity

Transmit BREAK:                    Yes; some versions

IBM communication:                 Yes

Transaction logging:               No

Session logging (raw download):    Yes

Raw upload:                        Yes

Act as server:                     No

Talk to server:                    Yes; SEND, GET, FIN, BYE

Advanced commands for servers:     No

Local file management:             Yes; DIR, ERA, SET DEFAULT disk

Handle file attributes:            No

Command/init files:                No

Printer control:                   Yes, limited

SUMMARY OF CP/M

CP/M-80 (version 2.2) has only five built-in commands, and they all deal

with files; other functions are done by invoking programs.

CP/M file specifications are of the form DEV:XXXXXXXX.YYY, where

DEV:            is  a  device  name,  normally  the A: or B: floppy.  If

omitted, the device name defaults to your connected dis-

kette.

XXXXXXXX        is a filename of up to 8 characters.

CP/M-80 KERMIT                                                  Page 135

YYY             is the file type, up to 3 characters.

File  names and file types may contain letters, digits, and some special

characters, including dash, dollar sign, and underscore, but no imbedded

spaces.  Upper and lower case letters are equivalent.

"Wildcard" file-group specifications are permitted  in  file  names  and

file types (but not device names) within certain contexts; a "*" matches

a  whole  field, a "?" matches a single character, including space.  Ex-

amples: "*.F??" specifies all files whose types start with F and are  1,

2,  or  3  characters long; "F?.*" specifies all files whose names start

with F and are no more than two characters  long  (before  the  trailing

spaces).

The five CP/M commands are:

DIR file        Lists the the names of the specified files.  The default

file specification is "*.*".  Example: "DIR B:*.FOR".

ERA file        Erases  (deletes)  the  specified file(s); wildcards al-

lowed.

REN new old     Changes the name of a file from old to new, e.g.

"REN NEW.FOR=OLD.FOR".

SAVE            Saves the specified number of memory blocks into a file.

TYPE file       Types  the  specified   file   on   the   screen,   e.g.

"TYPE FOO.TXT".

The most important programs are:

STAT            Gives statistics on disk usage.

PIP             Peripheral  Interchange  Program.    Copies  files.   In

response to the "*" prompt, give a command of the form

disk:outfile=disk:infile

Wildcards ("*" for a whole field or "?"  for  a  letter)

can be used.  Examples: "A:=B:*.*" to copy a whole disk,

"A:=B:*.FOR"  to copy all the Fortran programs from disk

B to disk A. If the disk specification is omitted,  your

"connected" disk is assumed.  Command line arguments are

also accepted, e.g. "PIP A:=B:*.*".

For  further information on CP/M, consult your microcomputer manual or a

CP/M handbook.

CP/M-80 KERMIT                                                  Page 136

KERMIT-80 DESCRIPTION

Implementations  of Kermit-80 presently exist for the DEC VT180 (Robin),

the DECmate II, the Intertec Superbrain, the Heath/Zenith 89  and  Z100,

the  Apple   II with the Z80 SoftCard, the Osborne 1, the TRS-80 II with

CP/M, the Telcon Zorba, the Kaypro, the Vector Graphics CP/M system, the

Morrow Decision I, the Nokia MikroMikko, and others.  There  is  also  a

"generic"  CP/M version that should run on most 8080-compatible CP/M 2.2

systems, but which may provide less performance, and  another  for  CP/M

3.0.

Since  Kermit-80  runs on a standalone micro, it is always in control of

the screen -- it is always local.  Thus, it always keeps the screen  up-

dated  with  the  file  name  and  the packet number, whether sending or

receiving.  Kermit-80 is capable of an imprecise or "fuzzy"  timeout  on

an input request, and can break deadlocks automatically.  In most cases,

this  is  not  important,  because  the KERMIT on the other side is most

likely able to handle the timeouts.  The timeouts done by Kermit-80  are

fuzzy  because  they  depend on the speed of the Z80 processor and other

factors that can vary from system to system.

If despite the timeout capability, the transmission appears to be  stuck

(and  you  can tell that this has happened if the screen fails to change

for a while) you can type carriage return to have the micro do  what  it

would have done on a timeout, namely NAK the expected packet to cause to

foreign  host  to  send  it  again  (or,  if  the  micro  is sending, to

retransmit the last packet).  Micro/micro or micro/IBM-mainframe  trans-

fers could require this kind of manual intervention.

File transfers may be interrupted in several ways.

Control-C       This  will  return  you  to  Kermit-80 command level im-

mediately, so that you can connect back  to  the  remote

system, or take any other desired action.

Control-X       When  sending a file, this will terminate the sending of

the current file with a signal  to  the  KERMIT  on  the

other  side to discard what it got so far.  If there are

more files to be sent, KERMIT-80 will go on to the  next

one.   When receiving a file, KERMIT-80 will send a sig-

nal to the remote KERMIT to stop sending this file.   If

the  remote  KERMIT understands this signal (not all im-

plementations of KERMIT do), it will  comply,  otherwise

the file will keep coming.  In any case, the remote KER-

MIT will go on to the next file in the group, if any.

Control-Z       Like  Control-X,  except if a file group is being trans-

mitted, this will stop the transmission  of  the  entire

group.    If only a single file is being transmitted, it

works exactly like Control-X.

Carriage Returns

If you type carriage return  repeatedly  Kermit-80  will

retry   the   current  packet  up  to  its  retry  limit

(somewhere between 5 and 16 times) and then, if no valid

response  was  received,  return  to  Kermit-80  command

CP/M-80 KERMIT                                                  Page 137

level.

KERMIT-80 COMMANDS

KERMIT-80  uses  the  DECSYSTEM-20 keyword style command language.  Each

keyword may be abbreviated to its minumum unique length.    "?"  may  be

typed  to  request a menu of the available options for the current field

at any point in a command.  ESC may be typed at any point in  a  command

to  fill  out  the current keyword or filename; if sufficient characters

have not been typed to identify the current  field  uniquely,  KERMIT-80

will sound a beep and allow you to continue from that point.

CONNECT Establish  a  "virtual terminal" connection to any host that may

be connected to the serial port, i.e. pass  all  typein  to  the

serial  port  and  display all input from the serial port on the

screen.  Also, emulate a  DEC  VT52  to  allow  cursor  control,

screen  clearing,  etc., if VT52-EMULATION is ON (see below), in

which case you should also set your terminal type on the  remote

host to VT52.  (Some versions emulate other terminals.)  The es-

cape  character  differs from micro to micro; when you issue the

CONNECT command, the micro will print a message telling you  how

to  get  back.   The escape sequence is generally an uncommonly-

used    control    character,     like     CTRL-backslash     or

CTRL-rightbracket, followed by a single letter "command".

C   Close Connection, return to Kermit-80> command level.

S   Display  Status  of  connection, but maintain remote connec-

tion.

?   List available single-character commands.

0   (zero) Send a null (0) character.

B   Send a BREAK signal.  Only some systems provide  this  func-

tion.

^]  (or  whatever  - a second copy of the escape character) Send

the escape character itself to the remote host.

SEND filespec

Send file(s) specified by filespec to the remote  Kermit.    The

filespec may contain CP/M wildcards.

RECEIVE Receive  file(s)  from  the remote Kermit.  Store them under the

names provided in the file headers supplied by the remote  host.

If the names aren't legal, use as many legal characters from the

name  as  possible  (see  the  description  of  SET FILE-WARNING

below).  If there's a conflict, and FILE-WARNING is ON, warn the

user and try to build a unique name for the file by  adding  "&"

characters to the name.

GET filespec

When  Kermit-80  is  talking to a Kermit Server on the host, you

should use the GET command to request the server to  send  files

to  you,  for example: get hlp:k*.hlp Limitation: If you request

an alternate block check type using the SET BLOCK  command,  the

GET  command  will  not communicate it to the remote server.  If

you want to have type 2 or 3  block  checks  done  when  getting

files  from  the  server,  you have to issue the appropriate SET

CP/M-80 KERMIT                                                  Page 138

BLOCK  command  to the remote KERMIT before putting it in server

mode.

LOG filespec

When CONNECTed to a foreign host as a terminal, log the terminal

session to the specified  diskette  file.    This  functionality

depends  to  some  extent  on  the  remote  host's ability to do

XON/XOFF flow  control,  and  does  not  guarantee  a  complete

transcript  (after all, that's what the KERMIT protocol is for).

The log file is closed when the connection is closed  by  typing

the  escape  character  followed by the single-character command

"C".

TRANSMIT filespec

Send the specified file to the system on the other  end  of  the

connection  as  though  it were being typed at the terminal, one

line at a time.  No KERMIT  protocol  is  involved.    You  must

manually confirm each line.  This is useful for sending files to

systems  that don't have a KERMIT program.  During transmission,

you may type the escape  character  followed  by  one  of  these

single-character commands:

C   Cease transmission

R   Re-transmit the previous line

BYE     When  talking to a remote Kermit Server, this command shuts down

the server and logs it out, and also  exits  from  Kermit-80  to

CP/M command level.

LOGOUT  Like BYE, but leaves you at Kermit-80 command level.

FINISH  Like LOGOUT, but shuts down the remote server without logging it

out.   Leaves you at Kermit-80 command level; subsequent CONNECT

commands will put you back at host system command level.

SET parameter [value]

Set the specified parameter to the specified  value.    Possible

settings:

WARNING ON (or OFF)

Warn  user  of  filename  conflicts when receiving files

from remote host, and attempt to generate a unique  name

by  adding  "&"  characters  to  the  given name.  ON by

default.

VT52-EMULATION ON (or OFF)

When connected as a terminal to a foreign host, controls

whether the micro emulates a VT52  or  runs  in  "native

mode".    VT52  emulation  is  ON  by default, except on

micros that already have  terminal  functionality  built

in,  such  as  the  DEC  VT180 and DECmate (these act as

VT100-series terminals).  Some  systems  emulate  other

terminals, like the ADM3A.

LOCAL-ECHO ON (or OFF)

When  you  CONNECT to a remote host, you must set LOCAL-

CP/M-80 KERMIT                                                  Page 139

ECHO  ON if the host is half duplex, OFF if full duplex.

OFF by default.

ESCAPE  Change the escape character for virtual terminal connec-

tions.  Kermit-80 will prompt you  for  the  new  escape

character, which you enter literally.

BAUD    Change  the  baud rate of the communications port.  This

command only works  on  some  systems,  and  its  actual

operation can vary from system to system.  Type SET BAUD

followed  by a question mark, and follow the directions.

On systems that do not support this  command,  you  must

set  the  port  baud  rate  from  CP/M  or  other  setup

mechanism outside of KERMIT-80.

PARITY  Sets parity for outgoing characters to one of  the  fol-

lowing:  NONE,  SPACE, MARK, EVEN, or ODD.  On input, if

parity is NONE, then the 8th  bit  is  kept  (as  data),

otherwise  it  is stripped and ignored.  The parity set-

ting applies to both terminal connection and file trans-

fer.  If you set parity to  anything  other  than  none,

KERMIT-80  will  attempt  to  use "8th bit prefixing" to

transfer binary files.  If  the  other  KERMIT  is  also

capable  of  8th bit prefixing, then binary files can be

transferred successfully; if not, the 8th  bit  of  each

data  byte  will be lost (you will see a warning on your

screen if this happens).

TIMER ON (or OFF)

Enable or disable the "fuzzy timer".  The timer  is  off

by default, because in the normal case KERMIT-80 is com-

municating  with  a  mainframe  KERMIT  that has its own

timer.  Mainframe KERMIT timers tend to be more  precise

or  adaptable  to  changing  conditions.  You should SET

TIMER ON if you are communicating  with  a  KERMIT  that

does  not have a timer.  You should SET TIMER OFF if you

are communicating over a network with long delays.

IBM ON (or OFF)

Allow the transfer of files to and from an IBM mainframe

computer.  This makes Kermit-80 wait for  the  IBM  tur-

naround character (XON), ignore parity on input, add ap-

propriate parity to output, and use local echoing during

CONNECT.  As distributed, KERMIT-80 uses MARK parity for

IBM  communication.  If you don't give this command, IBM

mode is OFF.  Since IBM  VM/CMS  KERMIT  does  not  have

timeout  capability, SET IBM ON also turns on the "fuzzy

timer" automatically.

BLOCK-CHECK-TYPE

The options are:

1-CHARACTER-CHECKSUM

Normal, default, standard 6-bit checksum.

2-CHARACTER-CHECKSUM

A 12-bit checksum encoded as two characters.

CP/M-80 KERMIT                                                  Page 140

3-CHARACTER-CRC-CCITT

A  16-bit  CCITT-format Cyclic Redundancy Check,

encoded as 3 characters.

The 2 and 3 character options should only be used  under

conditions  of extreme line noise.  Many implementations

of KERMIT only support the single character checksum.

FILE-MODE

Tells KERMIT-80 what kind of file it is sending, so that

KERMIT can correctly determine the end of the file.  SET

FILE BINARY means to send all the 128-byte blocks of the

file, including the last block in its entirety; SET FILE

ASCII is used for text  files,  and  transmission  stops

when  the first Control-Z is encountered anywhere in the

file (this is the CP/M convention for marking the end of

a text file).  If binary transmission is used on a  text

file, some extraneous characters (up to 127 of them) may

appear  at the end of the file on the target system.  If

ASCII transmission is used on a binary file, the  entire

file  will not be sent if it happens to contain any data

bytes that correspond to Control-Z.

DEFAULT-DISK

This allows you to set the default disk  as  source  and

destination  of  file  transfers.   In addition, issuing

this command causes you to switch to the specified  disk

and log it in, write-enabled.  The selected disk appears

in your KERMIT-80 prompt, for instance

Kermit-80 A:>

PORT    Allows  you  to  switch  between different communication

ports.  This command is not available on all systems.

PRINTER ON or OFF.  Turns copying of CONNECT session to  printer

on  and off.  No attempt is made to do buffering or flow

control; it is assumed printer can keep up.

DIR     This provides a directory listing of the specified files.  If no

files are specified, all files on the default disk  are  listed.

File  sizes,  in K, are included.  You may interrupt the listing

at any time by typing any  character.    The  listing  (even  if

interrupted)  concludes  with  a  display  of the amount of free

storage left on the disk.

ERA     This executes the CP/M ERA command  on  the  specified  file(s).

The names of the files being erased are not displayed.

CP/M-80 KERMIT                                                  Page 141

11.1. Generic KERMIT-80

"Generic Kermit-80" is a implementation of Kermit that should run on any

8080-compatible  CP/M  system  no modification at all, or perhaps only a

minor one.  Unlike  other  Kermit-80  implementations,  it  contains  no

system-dependent  manipulation of the serial port or screen.  All I/O is

done with standard CP/M BIOS calls, and I/O redirection  is  done  using

the  CP/M IOBYTE function, which, according to the Digital Research CP/M

Operating System Manual, is an optional feature of any  particular  CP/M

implementation.    If  your system does not provide the IOBYTE function,

Generic Kermit-80 will not  work;  furthermore,  not  all  systems  that

implement IOBYTE do so in the same way.

The  reason  all Kermit-80 implementations aren't generic is that a good

deal of speed is sacrificed by getting all services from  the  operating

system.    While  a  specific implementation of Kermit-80 may be able to

operate at 4800, 9600, or even 19200 baud, Generic Kermit will  fail  to

work on some systems at speeds in excess of 1200 baud.

Generic Kermit also differs from other Kermit-80 implementations in that

it  does  not  do  fancy  screen control during file transfer; it simply

types the file names, packet numbers, and messages  in  sequence  across

and  down the screen.  This works best if you can put your micro or ter-

minal in "autowrap" mode; otherwise the packet numbers will pile  up  in

the rightmost column; the filenames and messages will always appear on a

new  line,  however.   Neither does generic Kermit-80 do termimal emula-

tion; thus  a  generic  Kermit-80  acts  either  as  a  "dumb  terminal"

(sometimes  called  a  "glass  TTY"),  or else its own built in terminal

firmware provides cursor control functions  independent  of  the  Kermit

program.

Note  that VT180 and DECmate-II Kermit are simply adaptations of Generic

Kermit that do VT100 (ANSI) screen control during file transfer.

11.2. Installation

Kermit-80 was written originally for the Intertec SuperBrain in  lowest-

common-denominator  8080  code with the standard assembler, ASM (no mac-

ros, no advanced instructions), so that  it  can  be  assembled  on  any

16

CP/M-80  system  .  It has since been modified to run on many other sys-

tems as well.  Kermit-80 should be able to run on any  8080-,  8085-  or

Z80-based  microcomputer  under  CP/M with only minor modifications (see

below).

All versions of Kermit-80 are assembled from the same source, with  sys-

tem  dependencies taken care of by assembly-time conditionals.  The most

important system dependencies are terminal emulation (when CONNECTed  to

_______________

16

The  8080  assembler  is  distributed as a standard part of CP/M-80,

whereas the fancier Z80 or macro assemblers are  normally  a  commercial

product

CP/M-80 KERMIT                                                  Page 142

the  remote  host)  and  screen handling, which are dependent on the in-

dividual micro's escape codes  (these  features  are  table  driven  and

easily  modified  for  other  CP/M  systems),  and  the lowest level i/o

routines for the serial communications port.  The port routines are best

done only with BDOS calls, but some systems do not allow this, primarily

because the BDOS routines strip the parity bit during port i/o, and  the

parity bit is used for data when transmitting binary files.  Also, using

BDOS  calls, there's no way to poll the serial port; you must hang until

input appears.

Kermit-80's i/o routines must check the port status and go elsewhere  if

no  input  is  available;  this  allows for virtual terminal connection,

keyboard interruption of stuck transmissions,  etc.    On  systems  that

fully  implement  i/o redirection via the optional CP/M IOBYTE facility,

this may be done  by  switching  the  IOBYTE  definition.    On  others,

however,  IN/OUT  instructions  explicitly  referencing  the port device

registers must be used.

CP/M-80 KERMIT version 3.8 and above includes a "fuzzy timer"  that  al-

lows  a  timeout to occur after an interval ranging from 5 to 20 seconds

(depending upon the speed of the  processor  and  the  operating  system

routines)  during  which expected input does not appear at the port.  In

this case, retransmission occurs automatically.  In any  case,  you  may

type  a  carriage  return during transmission to simulate a timeout when

the transfer appears to be stuck.

11.2.1. Downloading Kermit-80

If you already have a version of Kermit on your micro and  you  want  to

install  a  new  version, simply use your present version to get the new

one.  If it's stored in the  form  of  a  .COM  file,  you  can  run  it

directly.  If it's stored as a .HEX file, you must first LOAD it on your

micro to produce a .COM file.

If you do not have a copy of KERMIT on your micro, and you cannot borrow

a  Kermit  floppy  but you do have access to a mainframe computer with a

copy of the Kermit-80 distribution, you should read this section.

There are several ways to get Kermit from a host system to  your  micro.

The  easiest  is  to  "download"  the  precompiled  "hex" file into your

micro's memory and then save it on the disk.  The following is a  proce-

dure which, though far from foolproof, should allow you to get a version

of Kermit to your CP/M based micro.  It depends upon the host prompt, or

at  least  the  first character of the host prompt, being some character

that cannot appear in a hex file (the valid characters for hex files are

the digits 0-9, the upper case letters A-F, the  colon  ``:'',  carriage

return,  and line feed).  As soon as any other character is encountered,

the transfer will terminate.  If your host does not issue a prompt  that

will  accommodate this scheme, you can achieve the same effect by adding

an atsign ``@'' to the very end of the hex file before sending  it  from

the  host.    The  program  below looks for an atsign (the normal DEC-20

prompt, hex 40).  DECSYSTEM-10 users would look for a dot, hex 2E.

1. Look for the appropriate hex file in the host's KERMIT  area.

The  name  will be something like CPMROBIN.HEX, CPMHEATH.HEX,

CP/M-80 KERMIT                                                  Page 143

CPMOSBORN.HEX,  etc.  If you don't find it, but you do find a

corresponding .ASM or .M80 file, you'll either have to  build

a new hex file on the host using a cross assembler (see below

for  how to do this on a DEC-10 or DEC-20), or else bring the

M80 source file to your micro and assemble it there.

2. Connect to your host using a terminal or a terminal emulation

facility.  Ensure that your host does not have your  terminal

in  "page  mode".   E.g. on the DEC-20, give the Exec command

TERMINAL NO PAUSE END-OF-PAGE.

3. Tell the host to display the hex file at your terminal.  E.g.

on the DEC-20, give the Exec command TYPE KERMIT.HEX, without

a terminating carriage return.

4. Return to your micro.  Connect to a floppy disk  with  plenty

of free space.  Make sure your IOBYTE is set so that RDR: and

PUN:  correspond  to  the  I/O  port that is connected to the

DEC-20 (this would normally be the case unless you have  done

something special to change things).  Run DDT and type in the

following  (the  comments  should  not  be typed in; they are

there just to tell you what's happening):

-ikermit.hex            ;Setup FCB for file KERMIT.HEX.

-a100                   ;Begin assembling code at 100.

0100 lxi h,ffe          ;Where to put HEX file.

0103 shld 300           ;Save the address.

0106 mvi e,d            ;Get a CR.

0108 mvi c,4            ;Output function.

010A call 5

010D mvi c,3            ;Input function.

010F call 5

0112 ani 7f             ;Turn off the parity.

0114 cpi 40             ;Our DEC-20 prompt atsign?

0116 jz 124             ;Yes, we have whole file.

0119 lhld 300           ;Get the pointer.

011C mov m,a            ;Else, store the char.

011D inx h              ;Increment the pointer.

011E shld 300           ;Save the pointer.

011F jmp 10d            ;Go around again.

0124 mvi a,1a           ;Get a control-Z.

0126 lhld 300           ;Get the pointer.

0129 mov m,a            ;Store the char.

012A shld 300           ;Save the pointer.

012D lxi h,1000         ;Pointer to file.

0130 shld 310           ;Save the pointer.

0133 mvi c,16           ;Make file.

0135 lxi d,5c

0138 call 5

013B lhld 310           ;Get the file pointer.

013E xchg               ;Put it in DE.

013F mvi c,1a           ;Set DMA.

0141 call 5

0144 mvi c,15           ;Write DMA to file.

0146 lxi d,5c

0149 call 5

CP/M-80 KERMIT                                                  Page 144

014C lhld 310           ;Get the file pointer.

014F lxi d,80           ;Get the DMA size.

0152 dad d              ;Adjust file pointer.

0153 shld 310           ;Save it.

0156 lda 301            ;Get high order byte.

0159 cmp h              ;Have we passed the end?

015A jm 170             ;Yes.

015D jz 163             ;Maybe.

0160 jmp 13b            ;No.

0163 lda 300            ;Get low order byte.

0166 cmp l              ;Passed the end?

0167 jm 170             ;Yes.

016A jz 170             ;Yes.

016D jmp 13b            ;Not quite.

0170 mvi c,10           ;Close file.

0172 lxi d,5c

0175 call 5

0178 ret

0179

-g100,179               ;Execute the program.

-                       ;Reboot

Now there should be a file KERMIT.HEX on your connected disk.

5. Load  this using the CP/M command LOAD to produce KERMIT.COM.

This should be a runnable version of Kermit.  Note that  CP/M

hex  files  have  checksums  on each line.  If there were any

transmission errors during the downloading process, the  CP/M

loader  will  notice  a bad checksum and will report an error

(something like "Illegal Format").  If  you  get  any  errors

during  loading,  either  fix  the  hex  file locally with an

editor, or repeat the previous step.

You now should have a running version of Kermit-80.

11.2.2. Building KERMIT.HEX

The source for Kermit-80 should be available on your host computer.   It

is  written  using  8080  assembler mnemonics, so it can be assembled on

most 8080s and Z80s using the  standard  8080  assembler  provided  with

CP/M.    If you have KERMIT.ASM on your CP/M system, you can assemble it

directly on the micro using ASM, setting the desired  assembly  switches

as  explained  below.  If you don't have it, you can attempt to download

the source file from the host using the procedure outlined above.

A cross assembler is provided that runs on the DEC-10 and DEC-20, called

MAC80, contributed by Bruce Tanner at Cerritos College, that may be used

for cross assembling KERMIT-80 as shown in this example (for TOPS-20):

1. Copy PS:<KERMIT>CPMKxx.M80 to your directory, as  KERMIT.M80.

TOPS-10  filename  rules  must be followed.  This is the 8080

assembler source file; xx is the current version number;  for

instance version 3.8 would be stored as CPMK38.M80.

2. Edit  KERMIT.M80  to  set the conditional assembly switch for

CP/M-80 KERMIT                                                  Page 145

your machine.  These are right near the top of the file.  The

switch  for  your  machine  should be set to TRUE and all the

others to FALSE.

3. The "ibm-flag" setting is site dependent.   As  shipped  from

Columbia,  it  turns  on  half duplex line handshaking, using

CTRL-Q as the turnaround character, sets LOCAL-ECHO ON, TIMER

ON, and PARITY MARK.  Make  any  changes  required  for  your

site.

4. The  default  FILE-MODE  is  ASCII  as shipped from Columbia.

This means that when text files are sent from the  CP/M  sys-

tem, no extraneous characters will be sent after the end, but

that  outgoing  binary  files may be truncated erroneously if

the user forgets to SET FILE BINARY.    You  can  change  the

default  to  BINARY,  so  that  no data is ever lost from any

file, but text files will usually have extraneous junk at the

end.

5.   Run MAC80:

@mac80

*kermit=kermit

*^Z

The result will be in your directory as KERMIT.HEX.

6. Use Kermit, MODEM, or  any  other  downloading  mechanism  to

transfer  KERMIT.HEX  to  the micro, or download it using the

DDT program shown above.

7. On the micro, load the hex file:

A>load kermit

KERMIT.COM will appear on the floppy.

8. The new Kermit should be ready to run.

11.2.3. Generic Kermit-80

If your CP/M 2.2 system implements i/o redirection  via  the  (optional)

IOBYTE  mechanism,  you  can  probably  run Generic Kermit on it, either

without modification, or by a very simple change to the  program.    The

standard CP/M IOBYTE is set up as follows:

I/O Byte assignments (four 2-bit fields for 4 devices at location 3)

:

bits 6+7        LIST field

0           LIST is Teletype device (TTY:)

1           LIST is CRT device (CRT:)

2           LIST is Lineprinter (LPT:)

3           LIST is user defined (UL1:)

CP/M-80 KERMIT                                                  Page 146

bits 4+5        PUNCH field

0           PUNCH is Teletype device (TTY:)

1           PUNCH is high speed punch (PUN:)

2           PUNCH is user defined #1 (UP1:)

3           PUNCH is user defined #2 (UP2:)

bits 2+3        READER field

0           READER is Teletype device (TTY:)

1           READER is high speed reader (RDR:)

2           READER is user defined #1 (UR1:)

3           READER is user defined #2 (UR2:)

bits 0+1        CONSOLE field

0           CONSOLE is console printer (TTY:)

1           CONSOLE is CRT device (CRT:)

2           CONSOLE in Batch-mode (BAT:);READER=Input,LIST=Output

3           CONSOLE is user defined (UC1:)

(Here, bit zero is the least significant, "rightmost", bit).

I/O  redirection  is  accomplished  by  switching the IOBYTE between two

values, "batch i/o" and "normal i/o".  In normal i/o mode, the  keyboard

is  defined  to  be  the console.  In batch i/o mode, the serial port is

defined to be the console.  This switching is necessary because the con-

sole is the only device that can be tested to see if input is available,

but KERMIT must shuttle back and forth  between  the  keyboard  and  the

serial  port looking for input.  Here are the batch and default i/o mode

definitions used in "standard" Generic KERMIT-80:

batio   EQU     056H            ;I/O byte CON=BAT,LIST=CRT,READER=RDR

defio   EQU     095H            ;I/O byte CON=CRT,LIST=LPT,READER=RDR

Other systems may have other logical devices that point  to  the  serial

port,  in  which  case you'll need to redefine these symbols to point to

those devices and then reassemble  the  program  (with  assembly  switch

GENER set to TRUE, all others FALSE).

If  your system runs CP/M 3.0, then the CPMPLUS version of KERMIT should

run on your system without modification, except perhaps for screen  con-

trol or baud rate setting.

CP/M-86 KERMIT                                                  Page 147

12. CP/M-86 KERMIT

Authors:    Bill  Catchings,  Columbia University; Ron Blanford, Univer-

sity of Washington; Richard Garland, Columbia University.

Language:   Digital Research ASM86

Version:    2.7

Date:       May 1984

Documentation:

Frank da Cruz, Columbia

This version of KERMIT is designed to support any CP/M-86  system.    So

far  it  supports the DEC Rainbow-100 and the NEC Advanced Personal Com-

puter (APC).  It is very similar to CP/M-80 and MS DOS KERMIT.

CP/M-86 KERMIT-86 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  No

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     Yes

^X/^Y interruption:                Yes

Filename collision avoidance:      Yes

Can time out:                      Yes

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes, uses PC firmware (VT100)

Communication settings:            Yes; duplex, parity

Transmit BREAK:                    Yes

IBM communication:                 Yes

Transaction logging:               No

Session logging (raw download):    Yes

Raw upload:                        No

Act as server:                     No

Talk to server:                    Yes; SEND, GET, FIN, BYE

Advanced commands for servers:     No

Local file management:             No

Handle file attributes:            No

Command/init files:                Yes

Printer control:                   No

CP/M-86 Kermit closely resembles CP/M-80 Kermit, just as CP/M-86 is very

similar to  CP/M-80.    In  some  respects,  KERMIT-86  is  superior  to

KERMIT-80: the program is better modularized to facilitate easy addition

of  support  for  new systems, alternate style user interfaces, etc; the

port i/o is fully buffered and XON/XOFF is done to allow high-speed com-

munication and accurate session logging.  On the other hand, the current

version does not include the CP/M-80 Kermit's local file management com-

mands.

CP/M-86 KERMIT                                                  Page 148

CP/M-86 KERMIT DESCRIPTION

Since  Kermit-86  runs on a standalone micro, it is always in control of

the screen -- it is always local.  Thus, it always keeps the screen  up-

dated  with  the  file  name  and  the packet number, whether sending or

receiving.  Kermit-86 is capable of timing out an input request, and can

thus break deadlocks automatically.  In most cases, however, this is not

desirable because the KERMIT on the other side  is  most  likely  better

able  to  handle  the timeouts; therefore, Kermit-86's timer is normally

not used.

If despite the timeout capability, the transmission appears to be  stuck

(and  you  can tell that this has happened if the screen fails to change

for a long while) you can type carriage return to have the micro do what

it would have done on a timeout, namely NAK the expected packet to cause

to foreign host to send it again  (or,  if  the  micro  is  sending,  to

retransmit  the last packet).  Micro/micro or micro/IBM-mainframe trans-

fers could require this kind of manual intervention.

File transfers may be interrupted in several ways.

Control-C       This will return you  to  Kermit-86  command  level  im-

mediately,  so  that  you can connect back to the remote

system, or take any other desired action.

Control-X       When sending a file, this will terminate the sending  of

the  current  file  with  a  signal to the KERMIT on the

other side to discard what it got so far.  If there  are

more  files to be sent, KERMIT-86 will go on to the next

one.  When receiving a file, KERMIT-86 will send a  sig-

nal  to the remote KERMIT to stop sending this file.  If

the remote KERMIT understands this signal (not  all  im-

plementations  of  KERMIT do), it will comply, otherwise

the file will keep coming.  In either case,  the  remote

KERMIT will go on to the next file in the group, if any.

Control-Z       Like  Control-X,  except if a file group is being trans-

mitted, this will stop the transmission  of  the  entire

group.    If only a single file is being transmitted, it

works exactly like Control-X.

Carriage Returns

If you type carriage return  repeatedly  Kermit-86  will

retry   the   current  packet  up  to  its  retry  limit

(somewhere between 5 and 16 times) and then, if no valid

response  was  received,  return  to  Kermit-86  command

level.

When  KERMIT-86 is started, it looks for the file KERMIT.INI.  If found,

it executes KERMIT-86 commands from it before  prompting  you  for  com-

mands.

CP/M-86 KERMIT                                                  Page 149

12.1. Kermit-86 Commands

KERMIT-86  uses  the  DECSYSTEM-20 keyword style command language.  Each

keyword may be abbreviated to its minumum unique length.    "?"  may  be

typed  to  request a menu of the available options for the current field

at any point in a command.  ESC may be typed at any point in  a  command

to  fill  out  the current keyword or filename; if sufficient characters

have not been typed to identify the current  field  uniquely,  KERMIT-86

will sound a beep and allow you to continue from that point.

CONNECT Establish  a  "virtual terminal" connection to any host that may

be connected to the serial port, i.e. pass  all  typein  to  the

serial  port  and  display all input from the serial port on the

screen, using the system's own built-in support for ANSI (VT100-

like) screen control.  When you issue the CONNECT  command,  the

PC will print a message telling you how to get back by typing an

an  escape  sequence, an uncommonly-used control character, nor-

mally CTRL-backslash, followed by a single letter "command".

C   Close Connection, return to Kermit-86> command level.

?   List available single-character commands.

B   Send a BREAK signal.

Q   Quit logging the remote session.

R   Resume logging the remote session.

L   Toggle logging.

^\  (or whatever - a second copy of the escape  character)  Send

the escape character itself to the remote host.

SEND filespec

Send  file(s)  specified by filespec to the remote Kermit, using

the prevailing file mode  (ASCII  or  BINARY;  see  SET).    The

filespec may contain CP/M wildcards.

RECEIVE Receive  file(s)  from  the remote Kermit.  Store them under the

names provided in the file headers supplied by the remote  host.

If the names aren't legal, use as many legal characters from the

name  as  possible  (see  the  description  of  SET FILE-WARNING

below).  If there's a conflict, and FILE-WARNING is ON, warn the

user and try to build a unique name for the file by  adding  "&"

characters to the name.

GET filespec

When  Kermit-86  is  talking to a Kermit Server on the host, you

should use the GET command to request the server to  send  files

to you, for example: get hlp:k*.hlp

BYE     When  talking to a remote Kermit Server, this command shuts down

the server and logs it out, and also  exits  from  Kermit-86  to

CP/M command level.

LOGOUT  Like BYE, but leaves you at Kermit-86 command level.

FINISH  Like LOGOUT, but shuts down the remote server without logging it

out.    Leaves you at Kermit-86 command level; a subsequent CON-

NECT command should put you back at host system command level.

CP/M-86 KERMIT                                                  Page 150

EXIT    Exit from KERMIT-86 back to CP/M.

QUIT    Synonym for EXIT.

SET parameter [value]

Set  the  specified  parameter to the specified value.  Possible

settings:

BAUD    Change the baud rate of the communications port.    This

command  only  works  on  some  systems,  and its actual

operation can vary from system to system.  Type SET BAUD

followed by a question mark, and follow the  directions.

On  systems  that  do not support this command, you must

set  the  port  baud  rate  from  CP/M  or  other  setup

mechanism outside of KERMIT-86.

DEBUG   ON  or OFF.  If ON, displays incoming and outbound pack-

ets during file transfer.  OFF by default.

ESCAPE  Change the escape character for virtual terminal connec-

tions.  Select a character in the control range that you

will not be likely to need at the remote host; type  the

new  character  literally.    Certain  characters,  like

Control-X, cannot be specified.

FILE-TYPE

Tells KERMIT-86 what kind of file it is sending, so that

KERMIT can correctly determine the end of the file.  SET

FILE BINARY means to send all the 128-byte blocks of the

file, including the last block in its entirety; SET FILE

ASCII is used for text  files,  and  transmission  stops

when  the first Control-Z is encountered anywhere in the

file (this is the CP/M convention for marking the end of

a text file).  If binary transmission is used on a  text

file, some extraneous characters (up to 127 of them) may

appear  at the end of the file on the target system.  If

ASCII transmission is used on a binary file, the  entire

file  will not be sent if it happens to contain any data

bytes that  correspond  to  Control-Z.    ASCII  is  the

default.

FLOW-CONTROL

Select  the  desired  type of flow control to be used on

the communication  line.    The  choices  are  NONE  and

XON/XOFF.   XON/XOFF is the default.  If the remote sys-

tem is not full duplex or cannot do XON/XOFF, you should

use NONE.

IBM ON (or OFF)

Allow the transfer of files to and from an IBM mainframe

computer.  This makes Kermit-86 wait for  the  IBM  tur-

naround character (XON), ignore parity on input, add ap-

propriate parity to output, and use local echoing during

CONNECT.  As distributed, KERMIT-86 uses MARK parity for

IBM  communication.  If you don't give this command, IBM

mode is OFF.  Since IBM  VM/CMS  KERMIT  does  not  have

CP/M-86 KERMIT                                                  Page 151

timeout capability, SET IBM ON also turns on the timeout

facility  automatically,  as if you had typed "SET TIMER

ON".

LOCAL-ECHO ON (or OFF)

When you CONNECT to a remote host, you must  set  LOCAL-

ECHO  ON if the host is half duplex, OFF if full duplex.

OFF by default.

LOG     Specify a log file on the current CP/M disk  into  which

to  record  incoming  characters during CONNECT.  If the

remote host can do XON/XOFF, then the log file will nor-

mally capture every character shown on the screen.  When

connected to the remote system, several single-character

arguments to the connect escape character can be used to

control logging -- Q (quit), R (resume), L (toggle).  If

you use R or L during connect without having  previously

specified  a log file name, then KERMIT.LOG is used.  An

open log is closed when you escape back to the PC.

PARITY  Sets parity for outgoing characters to one of  the  fol-

lowing:  NONE,  SPACE, MARK, EVEN, or ODD.  On input, if

parity is NONE, then the 8th  bit  is  kept  (as  data),

otherwise  it  is stripped and ignored.  The parity set-

ting applies to both terminal connection and file trans-

fer.  If you set parity to  anything  other  than  NONE,

Kermit-86  will  attempt  to  use "8th bit prefixing" to

transfer binary files.  If  the  other  KERMIT  is  also

capable  of  8th bit prefixing, then binary files can be

transferred successfully; if not, the 8th  bit  of  each

data  byte  will be lost (you will see a warning on your

screen if this happens).

PORT    Allows you to  switch  between  different  communication

ports  on  the PC.  This command is not available on all

systems.

TIMER ON (or OFF)

Enable or disable the timeout facility.   The  timer  is

off  by default, because in the normal case KERMIT-86 is

communicating with a mainframe KERMIT that has  its  own

timer.   Mainframe KERMIT timers tend to be more precise

or adaptable to changing conditions.    You  should  SET

TIMER  ON  if  you are communicating with another KERMIT

that does not have a timer.  You should SET TIMER OFF if

you are communicating over a network with long delays.

WARNING ON (or OFF)

Warn user of filename  conflicts  when  receiving  files

from  remote host, and attempt to generate a unique name

by adding "&" characters to the  given  name.    OFF  by

default.

SHOW    Show the current settings of the SET parameters.

TAKE    Take  KERMIT-86  commands  from  the  specified  file.  The file

CP/M-86 KERMIT                                                  Page 152

should  not  contain  any TAKE commands; nested command files do

not work.

12.2. Installation:

CP/M-86 KERMIT is broken up into several source modules:

86KERCMD.A86        Command parser

86KERFIL.A86        File handler

86KERIO.xxx         System Dependent I/O

86KERMIT.A86        Main Program

86KERPRO.A86        Protocol Module

86KERTRM.A86        Terminal Emulation

86KERUTL.A86        Utilities

The main program module, 86KERMIT.A86, contains INCLUDE  directives  for

the other files.  The 86KERIO module is stored with suffixes that denote

the  machine for which the program is being built -- RB for Rainbow, APC

for NEC APC.  The program may be built on the CP/M-86 system by  obtain-

ing  all the source files listed above, storing them on the current disk

with the names indicated, renaming the appropriate 86KERIO.xxx  file  to

be 86KERIO.A86, and then doing:

ASM86 86KERMIT $PZ  (takes about 6 minutes on the Rainbow)

GENCMD 86KERMIT     (takes less than a minute)

and, if desired,

REN KERMIT.CMD=86KERMIT.CMD

12.3. DEC Rainbow 100 Support

KERMIT-86  runs on the DEC Rainbow 100 or 100+ under CP/M-86/80, version

1 or 2, on the 8088 side.  It uses the built-in firmware  to  emulate  a

VT102  ANSI  terminal during CONNECT, and runs well at speeds up to 9600

baud.

You should be able to download the program using the old KERMIT  on  the

Z80  side  (Rainbow Kermit, VT180 Kermit, or generic CP/M-80 Kermit will

do the job, but only under DEC CP/M-86/80 version 1.0),  or  an  earlier

version of KERMIT-86.

If  you  don't have an earlier version of KERMIT, then follow the direc-

tions for installing KERMIT-80 (yes, KERMIT-80) in the KERMIT-80 section

of the KERMIT User Guide, but send the KERMIT-86 hex file instead.  This

works because the Rainbow can run CP/M-80 programs like DDT.

Another way to get KERMIT onto your Rainbow for the first time would  be

from  a  DEC  VT-180  diskette.  A VT-180 can use its own KERMIT to load

RBKERMIT onto its disk, which can then be read directly  by  a  Rainbow.

Also,  note that VT-180 KERMIT-80 can actually run on the Rainbow on the

Z80 side under DEC CP/M-86/80 version 1 (but not version 2  or  higher),

at speeds of 1800 baud or lower.

CP/M-86 KERMIT                                                  Page 153

12.4. NEC Advanced Personal Computer Support

(Contributed by Ron Blanford, University of Washington)

Currently  only  the  standard serial port is supported, and not the H14

auxiliary port.  The SET PORT command is not implemented.

While in Kermit's terminal emulation mode, local commands are  initiated

by  a  two-character  sequence consisting of the "escape character" fol-

lowed by one other character identifying the command.  (Make the  second

character  a  '?' to see a list of the valid commands.)  As distributed,

the standard Kermit-86 uses the control-backslash character as  the  es-

cape  character  in terminal mode.  The trouble is that the CP/M-86 BIOS

in the APC ignores a keyboard entry of Control-\ (i.e. holding down  the

CTRL  key  while striking the '\' key), making it difficult (impossible)

to use this method to get out of terminal mode.

One solution is to perform a "SET ESCAPE ^" command before entering ter-

minal mode to change the escape character  to  a  caret  (or  any  other

character the APC keyboard will generate).  This command could be placed

in  your  KERMIT.INI  file  for automatic execution every time Kermit is

started.

The simpler solution is  to  realize  that  the  character  code  for  a

Control-\  is  a  hexadecimal 1C, and that this is the code generated by

the INS key on the numeric keypad.  Once you  can  remember  that  every

reference  to  Control-\ should be interpreted as a reference to the INS

key, this is actually easier to use than the two-key Control-\ sequence.

In the  standard  CP/M-86  BIOS,  the  unshifted  DEL  key  generates  a

Control-X character (hexadecimal 18).  This is the CP/M command to erase

the  current  input line, and is very useful for local processing.  Most

mainframes do not use the Control-X character at all, so it becomes much

less useful during terminal emulation.  The DEL  character  (hexadecimal

7F),  on  the  other  hand,  is often used by mainframes and can only be

generated on the APC by holding down the SHIFT key  while  striking  the

DEL   key   (this   capability   is   not   mentioned  anywhere  in  the

documentation).

Because the Control-X character is so seldom used while the DEL  charac-

ter is commonly used, the initialization procedure in Kermit-86 modifies

the CP/M-86 BIOS so that the DEL key generates the DEL character whether

shifted  or not.  Control-X can still be generated if necessary by hold-

ing down the CTRL key while striking the 'X' key.  The CP/M-86  BIOS  is

returned to its original state when Kermit terminates.

The  APC  uses  escape  sequences  which  have  been standardized by the

American National Standards Institute (ANSI) to control cursor movement,

screen erasing, and character attribute manipulation.  Perhaps the best-

known other terminal which follows ANSI guidelines  is  the  DEC  VT100.

The  APC  only recognizes a few of the more important ANSI commands, and

not the complete set which the VT100 supports.

The ANSI/VT100 features that the NEC APC supports are:

- direct cursor addressing (by row and column)

CP/M-86 KERMIT                                                  Page 154

- relative cursor addressing (up, down, left, right)

- line erasing (cursor to end, beginning to cursor, entire line)

- screen  erasing  (cursor  to  end, beginning to cursor, entire

screen)

- character attributes (underline, reverse video, blink, but not

bold)

In addition, the first four grey function keys (unshifted) generate  the

escape  sequences associated with PF1 through PF4 on the VT100 keyboard.

The arrow keys and numeric keypad  DO  NOT  generate  the  corresponding

VT100 sequences.

These  functions  are  enough  to support simple command line editing on

most systems, and allow mailers or paged file display programs to  clear

the  screen before each display.  Underlining and reverse video are also

useful in some applications.  This is not enough  to  support  the  more

sophisticated screen control required by screen editors such as EMACS or

KED.    In  addition,  due to a bug in the implementation of the CP/M-86

BIOS, the sequence ordinarily used to home the cursor (esc [ H) does not

work correctly; a patch for CP/M to correct this problem is  distributed

with APC Kermit-86.

APPLE-DOS KERMIT                                                Page 155

13. APPLE-DOS KERMIT

Authors:    Antonino N. J. Mione, Stevens Institute of Technology

Peter Trei, Columbia University

Documentation:

Antonino N.J. Mione, Stevens Institute of Technology

Peter Trei, Columbia University

Version:    2.1(45)

Date:       July 1984

Kermit-65 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  Yes

Transfers text files:              Yes

Transfers binary files:            Yes

Wildcard send:                     No

^X/^Y interruption:                No

Filename collision avoidance:      Yes

Can time out:                      No

8th-bit prefixing:                 Yes

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes (VT52)

Communication settings:            Yes; local echo, parity

Transmit BREAK:                    Yes

IBM communication:                 Yes

Transaction logging:               No

Session logging (raw download):    No

Raw upload:                        No

Act as server:                     No

Talk to server:                    Yes

Advanced commands for servers:     No

Local file management:             No

Handle file attributes:            No

Command/init files:                No

Printer control:                   No

KERMIT-65 is a program that implements the KERMIT file transfer protocol

for  the  Apple ][ micro computer system. It is written in 6502 assembly

language and should run on any Apple ][ or Apple ][ Plus system  running

DOS  3.3.    This section will describe the things you should know about

the DOS 3.3 file system in order to make effective use  of  KERMIT,  and

then it will describe the special features of the KERMIT-65 program.

13.1. The DOS 3.3 File System

Items  of  importance  which  will  be discussed in this section include

Filenames and File Characteristics.

APPLE-DOS KERMIT                                                Page 156

Apple DOS Filenames

Filenames  under  Apple  DOS  may  contain  almost  any  ASCII character

(including space). It is not recommended that special characters,  (i.e.

control characters or spaces) be used in a filename to be transferred by

Kermit-65 since they may cause problems when parsing the filename.

Filenames  may  be  up to 40 characters in length. No wildcarding of any

kind can be done in KERMIT-65.

Apple DOS File Characteristics

All files in Apple DOS have a file type associated with  them  which  is

contained  in  the  directory  entry for the file but is not part of the

filename itself. There are four types of files in DOS 3.3. They are:

- APPLESOFT BASIC

- INTEGER BASIC

- BINARY

- TEXT

All file types have their data stored in eight-bit  bytes  although  not

all  of  them  need  the eighth bit. The two file types containing basic

programs required the eighth bit due to the nature  of  the  data  being

stored.  BINARY  files  are  images of memory copied into a file. Often,

these are machine code programs. These files  require  all  eight  bits.

TEXT  files  normally contain only printable or carriage control charac-

ters. They are stored in the form of seven-bit ASCII characters but  the

eighth  bit should always be set since Apples manipulate all text inter-

nally as 'NEGATIVE ASCII'.

When transmitting text files, the byte size should be set to  seven-bit.

When  transmitting anything else, the user must insure that both Kermits

are handling eight bit data so  that  no  information  is  lost.  If  an

eight-bit data path is not available (i.e. the remote Kermit needs to do

parity  checking  with  the eigth bit), then eight-bit quoting should be

used. Of course, BINARY files as well as Apple BASIC files will not have

much meaning on a different system.  If the user desires to edit a BASIC

file on a mainframe, for instance, he must convert it to a TEXT file be-

fore sending it over. After receiving the file back on  the  Apple,  the

user  may  convert  it  back to BASIC once again. The reason BASIC files

would be meaningless to a different machine is  that  the  Apple  stores

BASIC  keywords  as single character tokens to save space and processing

time. To convert a BASIC program to and from a TEXT  file,  consult  the

Apple DOS 3.3 Manual.

File information can be obtained by issuing the CATALOG command. For ex-

ample:

]CATALOG

DISK VOLUME 010

APPLE-DOS KERMIT                                                Page 157

*A 002 HELLO

B 078 KERMIT

A 002 READER

T 005 TESTFILE

]

When KERMIT-65 is receiving a file, the file it creates on diskette will

be  of the type indicated by the FILE-TYPE parameter.  The file will al-

ways be left in an unlocked state after it is closed by KERMIT-65.  When

sending  a file, KERMIT-65 will use the FILE-TYPE parameter to determine

how to detect an End-of-file condition. Thus, it is  important  to  have

this set properly in all cases.

Recommendations for archiving files

When  using a large system for archiving purposes, there is no reason to

convert Apple Basic programs into text files before sending  them  since

there is no need to edit them on the mainframe.

The  FILE-TYPE  parameter  must always be set correctly when sending and

receiving files.  Also,  when  sending  files  which  require  eight-bit

FILE-BYTE-SIZEs,   this  parameter  must  also  be  set  properly  since

KERMIT-65 does not change it automatically based on FILE-TYPE.

The procedure for archiving TEXT files is:

- Run Kermit on remote system

- SET FILE-BYTE-SIZE SEVEN-BIT ! On KERMIT-65

- SET FILE-TYPE-MODE TEXT ! On KERMIT-65

- Send files

The procedure for archiving APPLESOFT, INTEGER, and BINARY files is:

- Run Kermit on remote system

- Set File-byte-size to Eight-bit ! On Remote Kermit

- SET FILE-BYTE-SIZE EIGHT-BIT ! On KERMIT-65

- SET FILE-TYPE-MODE APPLESOFT !  (or  INTEGER,  or  BINARY)  On

KERMIT-65

- Send files

APPLE-DOS KERMIT                                                Page 158

13.2. Program Operation

HARDWARE CONSIDERATIONS

Prior  to  using  KERMIT-65  for transferring files, the modem interface

must be set to handle data in a certain manner. Firstly, the data format

should be 8 data bits and 1 stop bit. Secondly, the card should  be  set

to no parity. The baud rate (if adjustable) must be set to whatever rate

the  modem  can  handle.  For  the  Apple  Com  card  and the D.C. Hayes

Micromodem, these parameters are set correctly by default, so very  lit-

tle has to be done. For the Apple Super Serial Card, however, these have

to  be  set before using KERMIT-65. In all cases, use the same procedure

to connect to the mainframe host as is indicated in the section below on

Installing KERMIT.

Some mainframes cannot handle data in the format of 8 data  bits  and  1

stop bit because they may need parity checking (i.e. most IBM machines).

In this case, 7 data bits and 1 stop bit plus some parity setting (other

than  none) will usually work. When talking with such mainframes, binary

and  basic  files  on   the   Apple   cannot   be   transferred   unless

Eighth-bit-quoting is used.

CONVERSING WITH KERMIT-65

KERMIT-65's prompt is "KERMIT-65>".

To run KERMIT-65 and issue commands to it, type the following:

]BRUN KERMIT

STEVENS/CU - APPLE ][ KERMIT-65 - VER 2.1

Kermit-65>SEND TESTFILE

file is sent

Kermit-65>STATUS

performance statistics are printed

Kermit-65>Other commands

.

.

.

Kermit-65>EXIT

]

KERMIT-65 uses a TOPS-20 style command parser.

During  interactive  operation, you may use the ?-prompting help feature

("?") and recognition (ESC) features while typing commands.  A question

mark typed at any point in a command displays the options  available  at

APPLE-DOS KERMIT                                                Page 159

that  point;  typing  an  ESC character causes the current keyword to be

completed (or default value to be supplied).  If you have not typed suf-

ficient characters to uniquely specify the keyword or  filename  (or  if

there  is no default value) then a beep will be sounded and you may con-

tinue typing. Keywords may be abbreviated to any prefix that is unique.

There are several different Apple]['s which can  run  KERMIT-65.  Kermit

will  have  no  problems  running on an Apple ][ or Apple ][+ system. It

will run on the Apple //e as well, however, the 80-column  board  cannot

be used at this time.

Of the different communication devices available for the Apple ][, three

are supported:

- Apple Com Card

- D.C. Hayes Micromodem

- Apple Super Serial Card

It  is  possible  that  other cards may have operational characteristics

very similar or identical to one of the devices above. If  this  is  the

case,  it  may work using one of the currently available device drivers.

The user may want to try each of the above options to see if any of them

work.

KERMIT-65 must be told which device is being used so  that  it  may  run

with  the  correct device driver. It also must be told in which slot the

card resides. This may  be  done  with  the  'SET'  command  (documented

below).

13.3. Remote and Local Operation

KERMIT-65  is normally run in local mode. It may be run as a remote Ker-

mit as well although there is no advantage to  doing  things  that  way.

KERMIT-65  supports  User-mode commands for talking to a Server. It does

not currently support server mode.

13.4. KERMIT-65 Commands

THE SEND COMMAND

Syntax: SEND filespec

The SEND command causes a file to be sent from the Apple to  the  remote

system. The Filespec is the name of the file on the Apple diskette to be

sent.  The parser will not accept control characters and certain special

characters in a filename (i.e. a comma), so the user may have to  rename

the  file before it is sent. The user may also have problems in filename

compatibility with remote Kermits. If the remote Kermit  does  not  have

the facilities to beat the filename into a format that its system likes,

the  user  may  have  to rename the file before sending it.  The default

disk drive used for file transfers is the drive used to boot the  system

APPLE-DOS KERMIT                                                Page 160

or  the last drive accessed with a DOS command. This can be changed with

the 'SET DEFAULT-DISK' command (explained below). Either the slot or the

drive or both may be altered.

As a file is being sent, the screen displays either 'SENDING  PACKET...'

or  'WAITING  PACKET...'  followed  by  the absolute packet number since

start of transmission. If a packet must be transmitted several times and

it reaches the maximum retry count,  the  transfer  will  fail  and  the

'KERMIT-65>'  prompt  will  return.  If the remote Kermit sends an error

packet, the text of the packet will be displayed on the screen  and  the

prompt will return.

Currently,  a packet can be retransmitted manually by typing anything on

the keyboard. If a  'Q'  is  typed,  the  entire  transmission  will  be

aborted.

THE RECEIVE COMMAND

Syntax: RECEIVE [filespec]

The RECEIVE command tells KERMIT-65 to receive a file or file group from

the  other  system.  If only one file is being received, you may include

the optional filespec as the name to  store  the  incoming  file  under;

otherwise, the name is taken from the incoming file header.  If the name

in the header is not a legal filename in DOS 3.3, KERMIT-65 will attempt

to  change  it  into something legal. There are very few things that are

illegal in DOS 3.3. If FILE-WARNING is on and an  incoming  file  has  a

name  identical  to  a  file already existing on the diskette, KERMIT-65

will issue a warning to the user and attempt to modify the  filename  to

make it unique.

GET

Syntax: GET remote-filespec

The GET command requests a remote KERMIT server to send the file or file

group  specified  by  remote-filespec.   This command can be used with a

KERMIT server on the other end.

The remote filespec is any string that can be a legal file specification

for the remote system; it is not parsed or validated locally.

If the remote KERMIT is not capable of server functions, then  you  will

probably  get  an error message back from it like "Illegal packet type".

In this case, you must connect to the other Kermit, give a SEND command,

escape back, and give a RECEIVE command.

APPLE-DOS KERMIT                                                Page 161

THE CONNECT COMMAND

Syntax: CONNECT

Establish  a  terminal  connection  to  the  remote  system. Get back to

KERMIT-65 by typing the escape character followed by the  letter  C. The

escape  character  is  Control-@  by  default.  When you type the escape

character, several single-character commands are possible:

B   Send a BREAK Signal.

C   Close the connection and return to KERMIT-65.

S   Show status of the connection.

0   Send a null.

Connect-escape

Send the Connect-escape character itself.

?   List all the possible single-character arguments.

You can use the SET ESCAPE command to define a different escape  charac-

ter.

When  'CONNECTED',  KERMIT-65  will be passing characters entered on the

keyboard to the remote system, and passing characters  from  the  remote

system  to the Apple screen. If VT52-EMULATION is turned on, Kermit will

trap escape codes and simulate the appropriate functions of a VT52  ter-

minal.

On  an  Apple ][+ with an incomplete keyboard, special characters can be

obtained by prefixing regular characters with a right-arrow.  Also,  Up-

percase  is  shown  in inverse and lowercase characters are displayed as

normal uppercase characters.

Here are the rules for using the special 2/2+ input, to get  all  print-

able ASCII characters, and how they appear on the screen:

Special  meanings are applied in various contexts to certain characters.

The left and right arrow keys do special things, and sometimes  the  es-

cape key does as well.

For  letters, the keyboard is always in either default UPPERCASE mode or

default lowercase mode.  When in UPPERCASE, all letters typed  are  sent

out  as  uppercase.  In lowercase, all letters are sent as lowercase. To

reverse the case for  the  next  character  only,  hit  the  right-arrow

("prefix")  key. To switch the default case, hit the prefix-key twice in

a row.

For funny characters, the prefix key is also used  to  get  the  unusual

punctuation  characters  which  are not on the Apple keyboard. Here they

are:  (To represent the prefix character I am using the letter p).

To get                  Type    Appearence

Left Square Bracket     p(      [

Right Square Bracket    p)      ]

Left Curly Bracket      p<      {

Right Curly Bracket     p>      }

Underline               p-      _

APPLE-DOS KERMIT                                                Page 162

Backslash               p/      \

Tilde (wiggle)          p^      ~

Vertical Line           p.      |

The left-arrow key sends a rubout.

With  left-arrow and right arrow doing special things, its a little hard

to enter their characters (^H and ^U respectivly). There is therefore an

escape from prefix mode sequence.  If  you  type  prefix-ESC,  the  next

character is sent without any interpretation.

Currently,  it  is  impossible to turn this special input and display on

and off, so its a bit of a pain if you are on a Apple 2e.

THE HELP COMMAND

Syntax: HELP

Typing HELP alone prints a brief summary of the KERMIT-65 commands.

THE EXIT AND QUIT COMMANDS

Syntax: EXIT

Exit from KERMIT-65.  You can restart the program, provided you  haven't

run anything else, by typing 'CALL 2049'.

Syntax: QUIT

This is merely a synonym for EXIT.

THE SET COMMAND

Syntax: SET parameter [option] [value]

Establish  or  modify  various  parameters for file transfer or terminal

connection.  You can examine their values with the SHOW  command.    The

following parameters may be SET:

DEBUGGING       TERSE or VERBOSE packet information.

DEFAULT-DISK    Which Diskette drive is used for file transfer?

DEVICE-DRIVER   Which communication device is being used?

EIGHT-BIT-QUOTING

Should eight-bit-quoting be used?

ESCAPE          Character for terminal connection.

FILE-BYTE-SIZE  SEVEN or EIGHT significant bits in a byte.

FILE-TYPE       Of Apple DOS file being sent/received.

APPLE-DOS KERMIT                                                Page 163

FILE-WARNING    Warn users if incoming file exists?

IBM             For communicating with an IBM mainframe

KEYBOARD        ][+ or //e keyboard.

LOCAL-ECHO      Full or half duplex switch.

PARITY          Character parity to use

RECEIVE         Various parameters for receiving files

SEND            Various parameters for sending files

SLOT            Which slot # is communication device in?

VT52-EMULATION  Should Kermit emulate a VT52 when connected?

SET DEBUGGING

Syntax: SET DEBUGGING options

Record the packet traffic on your terminal.  Options are:

TERSE           Show packet info only (brief).

VERBOSE         Display  packet  field  descriptions  with  packet  info

(lengthy).

OFF             Don't  display  debugging  information  (this   is   the

default).

SET DEFAULT-DISK

Syntax: SET DEFAULT-DISK parameter value

This  command  will  tell  KERMIT-65 which disk drive should be used for

file transfers. The two parameters which may be set separately are  SLOT

and DRIVE. The value for SLOT ranges from 1 to 7. The value for DRIVE is

either 1 or 2.

SET DEVICE-DRIVER

Syntax: SET DEVICE-DRIVER parameter keyword

This  command  will  tell KERMIT-65 what type of communication device is

being used. Currently, three different  cards  are  supported,  however,

other  unsupported  cards  may  work  similar enough to one of the three

available that it may be possible to use them. KERMIT-65  must  also  be

told  where  the card is in the machine (see the SET SLOT command).  The

options for this set command are:

APPLE-COM-CARD  The old Apple communication card (300 baud).

APPLE-DOS KERMIT                                                Page 164

DC-HAYES        The D.C. Hayes Micromodem II (300 baud).

SUPER-SERIAL-CARD

The Apple Super Serial Card (300-19.2k baud).

SET ESCAPE

Syntax: SET ESCAPE hexidecimal-number

Specify  the  control  character you want to use to "escape" from remote

connections back to KERMIT-65.  The default is 0 (Control-@).  The  num-

ber  is  the  hex value of the ASCII control character, 1 to 37, for in-

stance 2 is Control-B.

SET FILE-BYTE-SIZE

Syntax: SET FILE-BYTE-SIZE parameter

Byte size for Apple  DOS  file  I/O.    The  choices  are  SEVEN-BIT  or

EIGHT-BIT.

SEVEN-BIT

When sending a file, shut the H.O. bit. When receiving, turn the

H.O.  bit  on.  This is done since text files are the only files

which should be sent as SEVEN-BIT files and text files only make

sense to the Apple if  they  are  encoded  in  'negative  ascii'

values.

EIGHT-BIT

Always  send  and  receive the bytes intact. All 8 bits are sig-

nificant.

SET FILE-TYPE

Syntax: SET FILE-TYPE parameter keyword

This will inform KERMIT-65 what type  of  DOS  file  is  being  sent  or

received.  It  is  important  that this is set correctly since KERMIT-65

must create a file of the appropriate type when receiving (and it has no

way of knowing what kind of file it is). When KERMIT-65 is  sending,  it

must  also  know  the type of file since that tells it how to detect the

actual end-of-file. The options for this parameter  are  APPLESOFT,  IN-

TEGER, TEXT and BINARY.

APPLESOFT

The file being sent/received is an Applesoft Basic program.

INTEGER The file being sent/received is an Integer Basic program.

TEXT    The file being sent/received is an ASCII Text file.

BINARY  The file being sent/received is a Binary image.

APPLE-DOS KERMIT                                                Page 165

SET FILE-WARNING

Syntax: SET FILE-WARNING ON or OFF

This  tells  KERMIT-65 whether to warn the user about incoming filenames

conflicting with existing files or not.

SET IBM

Syntax: SET IBM ON or OFF

SET IBM actually sets a number of parameters. When setting  it  on,  the

command  also turns on LOCAL-ECHO (half-duplex) and sets PARITY to MARK.

When setting IBM OFF, LOCAL-ECHO will revert back to OFF and PARITY will

be set to NONE.  It should be used when doing file transfers with an IBM

or similar mainframe.

SET KEYBOARD

Syntax: SET KEYBOARD 2P or 2E

SET KEYBOARD tells KERMIT-65 if the user has a full keyboard (2E) or not

(2P).  If the user is on an Apple ][+, this should be set to  2P  (which

is  the  default).  When set to that, certain character translations are

available by using the right-arrow key as a prefix character.

SET SLOT

Syntax: SET SLOT parameter

This option tells KERMIT-65 in which slot the  communication  device  is

located. The range for the parameter is 1-7.

THE SHOW COMMAND

Syntax: SHOW [option]

The SHOW command displays various information:

ALL             All parameter settings (this is quite long).

DEBUGGING       Debugging mode.

DEFAULT-DISK    Which Diskette drive is used for file transfer?

DEVICE-DRIVER   Which communication device is being used?

EIGHT-BIT-QUOTING

Should eight-bit-quoting be used?

ESCAPE          Character for terminal connection.

APPLE-DOS KERMIT                                                Page 166

FILE-BYTE-SIZE  SEVEN or EIGHT significant bits in a byte.

FILE-TYPE       Of Apple DOS file being sent/received.

FILE-WARNING    Warn users if incoming file exists?

IBM             For communicating with an IBM mainframe

KEYBOARD        ][+ or //e keyboard.

LOCAL-ECHO      Full or half duplex switch.

PARITY          Character parity to use

RECEIVE         Various parameters for receiving files

SEND            Various parameters for sending files

SLOT            Which slot # is communication device in?

VT52-EMULATION  Should Kermit emulate a VT52 when connected?

The above options are analogous to the equivalent SET commands.

THE STATUS COMMAND

Give  statistics  about the most recent file transfer. This includes in-

formation such as number of characters  sent/received,  number  of  data

characters sent/received, and last error encountered.

13.5. Customizing, Building, and Installing KERMIT-65

STANDARD INSTALLATION

The Procedure

The  procedure to bootstrap an assembled KERMIT object file to the Apple

is as follows:

1. On the Apple, Type in the APPLBT.BAS  program  supplied  (See

below).  It is recommended that the user save this program as

it may be needed to bootstrap newer  versions  of  KERMIT  or

APPHXL  in  the  future.  Also, type the APPLESOFT program in

with none of the REMs. It will execute quicker  and  take  up

less room.

2. Call and login to the mainframe on which the KERMIT-65 object

file resides. Do the following:

- ]IN#n ! Where n is between 1 and 7

- For Communication card, do the following:

APPLE-DOS KERMIT                                                Page 167

* Dial number for computer system.

* Seat phone receiver in modem cradle.

* ]<Cntrl/A><Cntrl/F> !Full duplex, 300 baud

- For the D.C. Hayes Micromodem, do the following:

* ]<Cntrl/A><Cntrl/F> ! Full duplex, 300 baud

* MICROMODEM II: BEGIN TERM <Cntrl/A><Cntrl/Q>

* MICROMODEM  II: DIALING: nnn-nnnn<cr> ! nnn-nnnn is

number of computer system

- For the Apple Super Serial Card, do the following:

* ]<Cntrl/A>

* APPLE SSC:0D ! 8 data bits, 1 stop bit

* ]<Cntrl/A>

* APPLE SSC:0P ! Force no parity

* ]<Cntrl/A>

* APPLE SSC:nB ! n = 6 for 300 baud, n = 8  for  1200

baud

* Dial number for computer system.

* Seat phone receiver in modem cradle.

* ]<Cntrl/A>

* APPLE  SSC:T ! Terminal mode, now talking to remote

host

3. In your directory  on  the  mainframe,  the  following  files

should be present:

- APPLBT.FOR

- APPHXL.HEX

- APPLEK.HEX

Compile  and execute APPLBT.FOR. This will be used along with

APPLBT.BAS on the Apple to load the APPHXL program. Once  AP-

PLBT  is executing on the mainframe, give control back to the

Apple and then run APPLBT.BAS on the Apple.  For  either  the

Communication  Card  or the D.C. Hayes Micromodem, the proce-

dure is:

- <Cntrl/A><Cntrl/X><cr> ! Give control to Apple's Brain

APPLE-DOS KERMIT                                                Page 168

- ]LOAD APPLBT.BAS<cr>

- ]LOMEM:9500<cr>

- ]RUN<cr>

4. Relocate and save APPHXL. Type the following:

- ]CALL -151<cr> ! Enter Apple's system monitor

- *9000<2000.2280M<cr> ! Move APPHXL from $2000 to $9000

- *<Cntrl/C><cr> ! Reenter APPLESOFT

- ]BSAVE APPHXL,A$9000,L$280<cr> ! Save APPHXL to disk

5. Now simply start executing APPHXL.

- ]CALL -151<cr> ! Enter monitor

- *9000G<cr> ! Start APPHXL

- SLOT  FOR MODEM CARD? (1 TO 7 )n ! 'n' is slot # of card

(no <cr>)

- ENTER FILENAME TO LOAD APPLEK.HEX<cr> ! Tell APPHXL what

to load

APPHXL will print the byte count and load  address  for  each

line  it  is receiving as it loads it into memory. At the end

of each line, it will print '[OK]' if the line  was  received

properly  or  '[CHECKSUM  ERROR]' if there was a problem with

it.

6. When APPHXL finishes type the following to the Apple:

]BSAVE KERMIT,A$801,L$4E00<cr> ! Save KERMIT to disk

7. The user may set up a turn-key system  by  having  the  hello

file  on  the disk load and run KERMIT. The user may also run

KERMIT-65, change the defaults  which  are  supplied  in  the

program  such  as  SLOT  and  DEVICE-DRIVER,  and then resave

KERMIT-65.  The next time it is run, the user will  not  have

to  set  these  values  again.  If the user does not set up a

turn-key system, he must start KERMIT-65 by typing:

]BRUN KERMIT<cr>        ! Execute KERMIT-65 on the Apple

The Apple will display the following:

STEVENS/CU - APPLE ][ KERMIT-65  VER. 2.1

KERMIT-65>

The user is now ready to transfer files.

NOTE: For those users with the Apple Super Serial  Card,  the  <cntrl/A>

APPLE-DOS KERMIT                                                Page 169

intercept  character should be shut off by typing <cntrl/A>Z to the card

before Kermit is run. The reason for this is that Kermit uses  <cntrl/A>

as  a  Start-of-Header  character and this character may never reach the

program if the card is taking it to be a command.

APPLBT.BAS - The APPLESOFT Bootstrap program

10  REM  - LOADER FOR HXLOAD

11  OAD = 0

100 N$ = "0123456789ABCDEF"

110 D$ =  CHR$ (4)

130  PRINT D$;"IN#2" : REM CHANGE '2' TO SLOT OF COMM. CARD IF NECESSARY

135  PRINT  CHR$ (1); CHR$ (6)

136  PRINT D$;"PR#2" : REM CHANGE '2' TO SLOT OF COMM. CARD IF NECESSARY

137  PR#2  : REM THIS LINE SHOULD BE HERE ONLY FOR THE APPLE COMM. CARD

140 C3 = 0

150  HOME

199  REM  - REQUEST NEXT LINE

200  REM - PUT A DOT ON THE SCREEN FOR EACH LINE RECEIVED

201 C3 = C3 + 1: POKE 1024 + C3, ASC (".")

202 L$ = "":Y2% = 1: PRINT

203  GET A$:L$ = L$ + A$:Y2% = Y2% + 1: IF Y2% < 81 THEN 203

205 C1 = 0:C2 = 0:I = 0

208  IF  LEFT$ (L$,1) >  = "0" AND  LEFT$ (L$,1) <  = "9" THEN 220

210 L$ =  RIGHT$ (L$, LEN (L$) - 1): GOTO 208

220 LL =  LEN (L$)

249  REM - FETCH THE DATA BYTE COUNT FOR THIS LINE

250  GOSUB 1000:C1 = C1 + B:CO = B

255  IF CO = 0 THEN 990

259  REM - CONSTRUCT THE LOAD ADDRESS FOR THIS LINE

260  GOSUB 1000:C1 = C1 + B:AD = B: GOSUB 1000:C1 = C1 + B

:AD = AD * 256 + B

265 REM - IF THE LATEST VERSION OF CROSS IS USED, THIS SHOULD NOT

BE NEEDED : REM AD = AD - 28672

266  IF AD < OAD THEN 990

267 OAD = AD

270  FOR X = 0 TO CO - 1

275  REM - GO GET A BYTE AND PUT IT IN THE NEXT MEMORY LOCATION

280  GOSUB 1000:C1 = C1 + B

290  POKE AD + X,B

300  NEXT X

310  GOSUB 1000:C2 = B: GOSUB 1000:C2 = C2 * 256 + B

320  IF C1<>C2 THEN POKE 1024+C3,ASC("E")

330  GOTO 201

990  FOR X = 1 TO 1000: NEXT X

995  PRINT D$;"IN#0": PRINT D$;"PR#0": HOME : END

999  REM  - GET BYTE

1000  GOSUB 1501:B = N1: GOSUB 1501:B = B * 16 + N1

1010  RETURN

1500  REM - GET NIBBLE

1501  IF  LEN (L$) = 0 THEN N1 = 0: RETURN

1510 H$ =  LEFT$ (L$,1)

1511  IF  LEN (L$) = 1 THEN L$ = "": GOTO 1525

1515 L$ =  RIGHT$ (L$, LEN (L$) - 1)

1520  REM - RETURN VALUE OF HEX NIBBLE

APPLE-DOS KERMIT                                                Page 170

1525  FOR X1 = 1 TO 16

1530  IF H$ =  MID$ (N$,X1,1) THEN 1610

1540  NEXT X1

1550 REM - DIGIT WAS NOT FOUND, RETURN ZERO

1560 N1 = 0: RETURN

1600  REM

1610 N1 = X1 - 1: RETURN

APPLBT.FOR - The Mainframe Bootstrap program

CHARACTER LINE*80,SENTNL*1

OPEN (UNIT=00,FILE='APPHXL.HEX',MODE='ASCII')

10 READ (UNIT=05,FMT=20) SENTNL

20 FORMAT (A1)

READ (UNIT=00,FMT=25,END=999) LINE

25 FORMAT(A80)

WRITE (UNIT=05,FMT=30) LINE

30 FORMAT(A80)

GO TO 10

999 READ (UNIT=05,FMT=20) SENTNL

STOP

END

ALTERNATIVE INSTALLATION PROCEDURES

HEXloading from Diskette

Once  the  user has a working version of KERMIT-65 on his system, he can

use this to load the .HEX files of future versions of KERMIT.  There  is

another  hexload program available called APPDXL. This program will load

a hex file from an Apple diskette into memory. To use this procedure, do

the following:

1. Start executing APPHXL.

- ]BLOAD APPHXL ! Get APPHXL into memory

- ]CALL -151<cr> ! Enter monitor

- *9000G<cr> ! Start APPHXL

- SLOT FOR MODEM CARD? (1 TO 7 )n ! 'n' is slot # of  card

(no <cr>)

- ENTER FILENAME TO LOAD APPDXL.HEX<cr> ! Tell APPHXL what

to load

APPHXL  will print what it is receiving on the screen as well

as loading it into memory.

2. Relocate and save APPDXL. Type the following:

APPLE-DOS KERMIT                                                Page 171

- ]CALL -151<cr> ! Enter Apple's system monitor

- *9000<2000.2500M<cr> ! Move APPDXL from $2000 to $9000

- *<Cntrl/C><cr> ! Reenter APPLESOFT

- ]BSAVE APPDXL,A$9000,L$500<cr> ! Save APPDXL to disk

3. Use  Kermit-65  to  transfer  the new version of itself over.

Make the Apple file a Text file. WARNING: This file will take

LOTS of space (about 180 sectors) so make sure  the  disk  is

reasonably  empty.    The transfer takes a LONG time also, so

please be patient.

4. Start executing APPDXL.

- ]BRUN APPDXL<cr> ! Start APPDXL

- ENTER FILENAME TO LOAD APPLEK.HEX<cr> ! Tell APPDXL what

to load

5. When APPDXL finishes type the following to the Apple:

]BSAVE KERMIT,A$801,L$4E00<cr> ! Save KERMIT to disk

The new version of Kermit is now on disk.

Using KERMIT-65 to transfer APPLEK.BIN

There is yet another way to Bootstrap a new version of  KERMIT  onto  an

Apple. If the user has an older version of KERMIT-65 and has access to a

machine  with  a  valid  copy  of  APPLEK.BIN,  they can simply transfer

APPLEK.BIN  using  their  version  of  KERMIT.  Be  sure  to   set   the

File-byte-size  to  Eight-bit,  and  the File-type-mode to Binary before

transfering the file since this is the actual object  code.  No  special

loading or conversion is needed. The file will be placed on the disk and

ready to run.

FILES SUPPLIED FOR KERMIT-65

The following files should be supplied on the distribution tape:

- APPLBT.BAS - Initial bootstrap program to load APPHXL

- APPLBT.FOR - Program on mainframe to talk to APPLBT.BAS

- APPHXL.M65 - Source of program to load KERMIT-65

- APPHXL.HEX - Assembled version of Hex load program

- APPDXL.M65  -  Source  of program to load KERMIT-65 from Apple

diskette

- APPDXL.HEX - Assembled version of Disk Hex load program

APPLE-DOS KERMIT                                                Page 172

- APPLEK.M65 - Source for the KERMIT-65 program

- APPLEK.HEX - Assembled version of KERMIT-65

- APPLEK.BIN  - Assembled version of KERMIT-65 (Eight-bit Binary

object code)

- CROSS.MAC - CROSS Microprocessor Assembler (Source)

- CROSS.EXE - CROSS Microprocessor Assembler (Object)

CUSTOMIZING AND BUILDING KERMIT-65

The source code to KERMIT-65 is in 6502 Assembler. It has been formatted

for CROSS which  is  a  micro-Cross  Assembler  program  which  runs  on

DECsystem-10s  and  DECSYSTEM-20s.  Customizations  would  be  made  the

easiest if CROSS were available.

KERMIT-65 currently supports the following communications devices:

- FTASER - The Apple Communication card

- FTHAYS - The D.C. Hayes Micromodem.

- FTSSC - The Apple Super Serial Card

All device drivers are included in the assembled version and may be used

by issuing a 'SET DEVICE-DRIVER' command to Kermit. If any of the device

drivers are not needed, it(they) may be  excluded  by  setting  the  ap-

propriate feature test to zero in the Feature test section of the source

code.  Excluding  one  or more device drivers can reduce the size of the

object code greatly. DO NOT disable all device drivers  since  KERMIT-65

will then have no way of talking over the communication device.

The  feature  test  FTCOM  must be set to the type of computer for which

KERMIT-65 is being assembled. The only machine  KERMIT-65  is  available

for currently is the Apple ][. This parameter must be set to FTAPPL.

After  setting  any  options  necessary  in  APPLEK.M65,  rename  it  to

KERMIT.M65, and do the following:

- .R CROSS<cr> ! Run CROSS Microprocessor Assembler

- *KERMIT.HEX/PTP:KIM=KERMIT.M65/M65 ! Generate .HEX file

This command will produce an ASCII HEX file which can be downline loaded

onto the Apple using APPHXL.  If  a  listing  is  desired,  one  can  be

produced  by  adding  ",KERMIT.LST"  after the "/PTP:KIM" in the command

line to CROSS.

KERMIT User Guide                                               Page 173

I. The ASCII Character Set

ASCII Code (ANSI X3.4-1968)

There  are  128 characters in the ASCII (American national Standard Code

for Information Interchange) "alphabet".  The characters are  listed  in

order of ASCII value; the columns are labeled as follows:

Bit             Even parity bit for ASCII character.

ASCII Dec       Decimal (base 10) representation.

ASCII Oct       Octal (base 8) representation.

ASCII Hex       Hexadecimal (base 16) representation.

EBCDIC Hex      EBCDIC   hexadecimal  equivalent  for  Kermit  translate

tables.

Char            Name or graphical representation of character.

Remark          Description of character.

The first group consists of nonprintable 'control' characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   000   000   00   00    NUL   ^@, Null, Idle

1   001   001   01   01    SOH   ^A, Start of heading

1   002   002   02   02    STX   ^B, Start of text

0   003   003   03   03    ETX   ^C, End of text

1   004   004   04   37    EOT   ^D, End of transmission

0   005   005   05   2D    ENQ   ^E, Enquiry

0   006   006   06   2E    ACK   ^F, Acknowledge

1   007   007   07   2F    BEL   ^G, Bell, beep, or fleep

1   008   010   08   16    BS    ^H, Backspace

0   009   011   09   05    HT    ^I, Horizontal tab

0   010   012   0A   25    LF    ^J, Line feed

1   011   013   0B   0B    VT    ^K, Vertical tab

0   012   014   0C   0C    FF    ^L, Form feed (top of page)

1   013   015   0D   0D    CR    ^M, Carriage return

1   014   016   0E   0E    SO    ^N, Shift out

0   015   017   0F   0F    SI    ^O, Shift in

1   016   020   10   10    DLE   ^P, Data link escape

0   017   021   11   11    DC1   ^Q, Device control 1, XON

0   018   022   12   12    DC2   ^R, Device control 2

1   019   023   13   13    DC3   ^S, Device control 3, XOFF

0   020   024   14   3C    DC4   ^T, Device control 4

1   021   025   15   3D    NAK   ^U, Negative acknowledge

1   022   026   16   32    SYN   ^V, Synchronous idle

0   023   027   17   26    ETB   ^W, End of transmission block

0   024   030   18   18    CAN   ^X, Cancel

1   025   031   19   19    EM    ^Y, End of medium

1   026   032   1A   3F    SUB   ^Z, Substitute

0   027   033   1B   27    ESC   ^[, Escape, prefix, altmode

1   028   034   1C   1C    FS    ^\, File separator

0   029   035   1D   1D    GS    ^], Group separator

0   030   036   1E   1E    RS    ^^, Record separator

1   031   037   1F   1F    US    ^_, Unit separator

The last four are usually associated with the control version  of  back-

KERMIT User Guide                                               Page 174

slash,  right  square  bracket, uparrow (or circumflex), and underscore,

respectively, but some terminals do not transmit these  control  charac-

ters.

The following characters are printable:

First, some punctuation characters.

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   032   040   20   40    SP    Space, blank

0   033   041   21   5A    !     Exclamation mark

0   034   042   22   7F    "     Doublequote

1   035   043   23   7B    #     Number sign, pound sign

0   036   044   24   5B    $     Dollar sign

1   037   045   25   6C    %     Percent sign

1   038   046   26   50    &     Ampersand

0   039   047   27   7D    '     Apostrophe, accent acute

0   040   050   28   4D    (     Left parenthesis

1   041   051   29   5D    )     Right parenthesis

1   042   052   2A   5C    *     Asterisk, star

0   043   053   2B   4E    +     Plus sign

1   044   054   2C   6B    ,     Comma

0   045   055   2D   60    -     Dash, hyphen, minus sign

0   046   056   2E   4B    .     Period, dot

1   047   057   2F   61    /     Slash

Numeric characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   048   060   30   F0    0     Zero

1   049   061   31   F1    1     One

1   050   062   32   F2    2     Two

0   051   063   33   F3    3     Three

1   052   064   34   F4    4     Four

0   053   065   35   F5    5     Five

0   054   066   36   F6    6     Six

1   055   067   37   F7    7     Seven

1   056   070   38   F8    8     Eight

0   057   071   39   F9    9     Nine

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   058   072   3A   7A    :     Colon

1   059   073   3B   5E    ;     Semicolon

0   060   074   3C   4C    <     Left angle bracket

1   061   075   3D   7E    =     Equal sign

1   062   076   3E   6E    >     Right angle bracket

0   063   077   3F   6F    ?     Question mark

1   064   100   40   7C    @     "At" sign

KERMIT User Guide                                               Page 175

Upper-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   065   101   41   C1    A

0   066   102   42   C2    B

1   067   103   43   C3    C

0   068   104   44   C4    D

1   069   105   45   C5    E

1   070   106   46   C6    F

0   071   107   47   C7    G

0   072   110   48   C8    H

1   073   111   49   C9    I

1   074   112   4A   D1    J

0   075   113   4B   D2    K

1   076   114   4C   D3    L

0   077   115   4D   D4    M

0   078   116   4E   D5    N

1   079   117   4F   D6    O

0   080   120   50   D7    P

1   081   121   51   D8    Q

1   082   122   52   D9    R

0   083   123   53   E2    S

1   084   124   54   E3    T

0   085   125   55   E4    U

0   086   126   56   E5    V

1   087   127   57   E6    W

1   088   130   58   E7    X

0   089   131   59   E8    Y

0   090   132   5A   E9    Z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   091   133   5B   AD    [     Left square bracket

0   092   134   5C   E0    \     Backslash

1   093   135   5D   BD    ]     Right square bracket

1   094   136   5E   5F    ^     Circumflex, up arrow

0   095   137   5F   6D    _     Underscore, left arrow

0   096   140   60   79    `     Accent grave

KERMIT User Guide                                               Page 176

Lower-case alphabetic characters (letters):

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

1   097   141   61   81    a

1   098   142   62   82    b

0   099   143   63   83    c

1   100   144   64   84    d

0   101   145   65   85    e

0   102   146   66   86    f

1   103   147   67   87    g

1   104   150   68   88    h

0   105   151   69   89    i

0   106   152   6A   91    j

1   107   153   6B   92    k

0   108   154   6C   93    l

1   109   155   6D   94    m

1   110   156   6E   95    n

0   111   157   6F   96    o

1   112   160   70   97    p

0   113   161   71   98    q

0   114   162   72   99    r

1   115   163   73   A2    s

0   116   164   74   A3    t

1   117   165   75   A4    u

1   118   166   76   A5    v

0   119   167   77   A6    w

0   120   170   78   A7    x

1   121   171   79   A8    y

1   122   172   7A   A9    z

More punctuation characters:

.....ASCII.... EBCDIC

Bit  Dec   Oct  Hex  Hex    Char  Remarks

0   123   173   7B   C0     {    Left brace (curly bracket)

1   124   174   7C   4F     |    Vertical bar

0   125   175   7D   D0     }    Right brace (curly bracket)

0   126   176   7E   7E     ~    Tilde

Finally, one more nonprintable character:

0    127   177  7F   07    DEL   Delete, rubout

KERMIT User Guide                                               Page 177

Index

DEFINE   65, 110

8080   141                            Define SET Macros   43

Delay   38

?-prompting   158                     DELETE   56

DEVICE-DRIVER   163

ANSI.SYS   105                        Dialup   15

APC   153                             Diskette   22

APPLE ][+   155                       DO Command   110

APPLESOFT   164                       DOS 3.3   155

ARPANET   65                          Downloading   142

ASCII   173                           Duplex   38

Autoanswer   17

Autodialer   14, 111                  EBCDIC   173

Eighth-Bit  Prefix     29, 30, 40, 55,

Batch Operation of Kermit-MS   95             56, 107, 139, 151

Baud   139, 150                       EMACS   113

Baud Rate   36, 103                   End Of File   50, 93, 104

Beeper   103                          End Of Line   41, 42

Bell   103                            End-Of-Line (EOL)   82

BINARY   164                          EOF   104

Binary Files   21, 29, 40, 55, 56, 107Error Recovery   20

BIOS   141                            Escape Character   11, 137, 139,  149,

Block Check   36, 139                         150

Bootstrap   142                       Escape Character for CONNECT   38, 62,

BREAK Simulation   61, 65                     105, 164

BYE   18, 19, 138, 149                Escape Sequence   10

Byte Size   49, 57, 63, 164           EXIT   35, 61, 162

Expunging Deleted Files   62

Cables   14, 16

Cancelling  a  File Transfer   30, 31,File Renaming   109

55, 56, 97, 98                File Warning   109

Capturing Files   44                  FILE-BYTE-SIZE   164

Checksum   6                          FILE-TYPE   164

Command Files   109                   File-Warning   137, 149, 165

Command Macro   110                   Filespec   159

Command Parsing   25                  FINISH   18, 19, 138, 149

CONNECT   9, 11, 111, 137, 149, 161   Flow Control   39, 105, 150

CONTINUE   35, 61

Control Characters   10, 173          Generation   56

Control-A   55                        Generic Kermit-80   141, 145

Control-C   35, 61                    Generic MS-DOS Kermit   122

Control-V   56                        GET   19, 137, 149, 160

Control-X   30, 31, 55, 56, 97, 98

Control-Z   30, 31, 55, 56, 97, 98    Handshake   39, 105

CP/M   50, 141                        Heath-19 Terminal  Emulation      105,

Crash   22                                    111, 131

Cross Assembler   144                 Help   51, 94, 158, 162

Hex File   145

Deadlock   22

Debugging   37, 62, 104, 150, 163     IBM   63, 81, 139, 150, 165

DEC Rainbow   152                     IBM PC   90

DECsystem-10   144                    Incomplete  File Disposition   31, 56,

DECSYSTEM-20   46, 144                        98

DEFAULT-DISK   163                    Incomplete File Transfer   39

KERMIT User Guide                                               Page 178

Initial Filespec   22, 29, 53, 54     Remote   11, 24

INTEGER   164                         Repeated  Character  Compression   29,

Interference   52                             30, 55, 56

Internal Modem   20                   Retry Limit   42

IOBYTE   145

ITS-Binary Format   63                SEND   11, 14, 19,  22,  29,  53,  81,

137, 149, 159

Kermit Commands   11                  Server   17, 32, 57

KERMIT Protocol   6                   SET   11, 82, 138, 150

Kermit server   17                    SHOW   12, 43, 66, 82, 151, 165

Key Redefinition   106                SLOT   165

KEYBOARD   165                        Smart Modem   20, 21, 111

Speed   65, 66

Line Sequence Numbers   55            Start Of Packet   41, 42

Local   11, 24, 136, 148              Superbrain   141

Local Echo   38

Local Echoing   106                   TAC Binary Mode   65

Local-Echo   138, 151                 TAKE   151

LOG   44, 138, 151                    TELENET   40, 43, 106

LOGOUT   138, 149                     TERSE   163

LRECL   82                            TEXT   164

Timeout     41, 42, 81, 109, 136, 142,

MAC80   145                                   148

Macro   110                           TIMER   139, 151

Message Interference   52             TOPS-20   46

META Key   113                        TRANSMIT   138

Mode Line   111                       TVT-Binary   65

Modem   20, 111

MS-DOS   90                           UNDELETE   56

NAK   21, 136, 148                    VERBOSE   163

NEC Advanced Personal Computer   153  Virtual Terminal   9, 11, 137, 149

Network   20                          VM/CMS   14, 81

Noise   6                             VT100   138

Normal Form for File Names   54, 63   VT52   138

Null Modem   14, 16

Warning   109, 138, 151

Packet   6, 8                         Wildcard   11, 13, 47, 92

Packet Length   41, 42                Word Size   49

Packet-Length   82

Padding   41, 42                      XON/XOFF   138, 173

Parity   20, 29, 30, 40, 55, 56,  106,

139, 151, 173                 Z80   141

Pause Between Packets   41, 42

PC-DOS   90

Prompt   9, 41

Protocol   6

QUIT   35, 61, 162

Quote   82

Rainbow 100   152

RECEIVE    11, 13, 14, 30, 55, 82, 97,

137, 149, 160

RECFM   82

Recognition   51, 94, 158

KERMIT User Guide                                                 Page i

Table of Contents

Ordering Information                                                   4

1. Introduction                                                        6

2. How to Use KERMIT                                                   8

2.1. The KERMIT Program                                                8

2.2. Talking to Two Computers at Once                                  9

2.3. Transferring a File                                              10

2.4. Basic KERMIT Commands                                            11

2.5. Real Examples                                                    12

2.5.1. PC to Host                                                12

2.5.2. Host to Host                                              14

2.5.3. Micro to Micro                                            16

2.6. Another Way -- The KERMIT Server                                 17

3. When Things Go Wrong                                               20

3.1. Communication Line Problems                                      20

3.2. The Transfer is Stuck                                            21

3.3. The Micro is Hung                                                22

3.4. The Remote Host Went Away                                        22

3.5. The Disk is Full                                                 22

3.6. Message Interference                                             23

3.7. Host Errors                                                      23

3.8. File is Garbage                                                  23

3.9. Junk after End of File                                           23

4. KERMIT Commands                                                    24

4.1. Remote and Local Operation                                       24

4.2. Command Interface                                                25

4.3. Notation                                                         26

4.4. Summary of KERMIT Commands                                       28

4.5. The SEND Command                                                 29

4.6. The RECEIVE Command                                              30

4.7. GET                                                              31

4.8. SERVER                                                           32

4.9. BYE                                                              32

4.10. FINISH                                                          33

4.11. REMOTE                                                          33

4.12. LOCAL                                                           34

4.13. CONNECT                                                         34

4.14. HELP                                                            35

4.15. TAKE                                                            35

4.16. EXIT, QUIT                                                      35

4.17. The SET Command                                                 35

4.18. DEFINE                                                          43

4.19. SHOW                                                            43

KERMIT User Guide                                                Page ii

4.20. STATISTICS                                                      43

4.21. LOG                                                             44

4.22. TRANSMIT                                                        44

5. KERMIT Implementations                                             45

6. DECSYSTEM-20 KERMIT                                                46

6.1. The DEC-20 File System                                           46

6.2. Program Operation                                                51

6.3. Remote and Local Operation                                       52

6.4. Conditioning Your Job for Kermit                                 53

6.5. KERMIT-20 Commands                                               53

6.6. Examples                                                         68

6.7. Installation                                                     71

7. VAX/VMS KERMIT                                                     72

8. IBM VM/CMS KERMIT                                                  81

9. UNIX KERMIT                                                        86

10. MS-DOS KERMIT                                                     90

10.1. The MS-DOS File System                                          91

10.1.1. File Specifications                                      91

10.1.2. File Formats                                             92

10.2. Program Operation                                               93

10.3. Kermit-MS Commands                                              95

10.3.1. Commands for File Transfer                               96

10.3.2. Commands for Connecting and Disconnecting                99

10.3.3. Commands for File Management                            100

10.3.4. The SERVER Command                                      102

10.3.5. The SET Command                                         103

10.3.6. The SHOW Command                                        109

10.3.7. Command Macros                                          110

10.4. Terminal Emulation                                             111

10.5. Installation of Kermit-MS                                      114

10.5.1. Try Again To Find A Kermit Disk                         114

10.5.2. Bootstrapping From the Communication Line               114

10.5.2.1. Use An Existing File Capture Facility            115

10.5.2.2. Type In Your Own Bootstrap                       115

10.6. Compatibility with Older Versions of MS-DOS Kermit             120

10.7. What's Missing                                                 120

10.8. Program Organization                                           121

10.9. Adding Support For New Systems                                 122

10.9.1. Generic MS-DOS Kermit                                   122

10.9.2. Adding System-Dependent Code                            123

10.10. Heath/Zenith-19 Control Codes                                 131

11. CP/M-80 KERMIT                                                   134

KERMIT User Guide                                               Page iii

11.1. Generic KERMIT-80                                              141

11.2. Installation                                                   141

11.2.1. Downloading Kermit-80                                   142

11.2.2. Building KERMIT.HEX                                     144

11.2.3. Generic Kermit-80                                       145

12. CP/M-86 KERMIT                                                   147

12.1. Kermit-86 Commands                                             149

12.2. Installation:                                                  152

12.3. DEC Rainbow 100 Support                                        152

12.4. NEC Advanced Personal Computer Support                         153

13. APPLE-DOS KERMIT                                                 155

13.1. The DOS 3.3 File System                                        155

13.2. Program Operation                                              158

13.3. Remote and Local Operation                                     159

13.4. KERMIT-65 Commands                                             159

13.5. Customizing, Building, and Installing KERMIT-65                166

I. The ASCII Character Set                                           173

Index                                                                177