Documentation for VT-200 Set up (VT200SET)

Written by Mark Northrup on 9/11/85

Miller Brewing Company

3939 West Highland Blvd.

Milwaukee, WI 53201

Documentation for VT-200 Set up (VT200SET)                              Page 2

1.  Introduction

This module was designed to down load pre-set commands to the VT-200 terminal.

The first position in the file (VT200SETU.PRM) must be either a semi-colon  or

the  first  digit  of  the  function  key  number  as  defined  in the "VT-200

Programmers Pocket Guide" page 39, and  page  83  of  the  "VT-200  Programmer

Reference Manual".  If the first character is not a semi-colon the first three

characters must have the form "nn/" where  nn=the  function  key  value,  i.e.

Function  key  6 has a value of 17.  After the "/" the command line that is to

be displayed when that key is pressed, this program translates the string into

the  hex  pairs required by the VT-200.  A tilde (~) will be translated into a

carriage return, all other symbols will be translated directly into their  hex

pair equivalent (including semi-colons that are not in position 1).

To  use  the  defined  keys,  press  the  shift  key  and  the  function   key

simultaneously  and  the  value  of  that key will be displayed, if a carriage

return (indicated by a "~" (tilde) in the file VT200SETU.PRM)  was  placed  in

the string it will execute.

As usual no warranty, expressed or implied, is made by the  submitter,  DECUS,

or  Digital  Equipment  Corporationas  to the accuracy of this document or the

functioning  of  the  described  software  and  related   material,   and   no

responsiblity  is  assumed  by  these  parties  in  connection therewith.  The

software, files and documentation is submitted "as is", for use at  the  users

risk.   And  "if any of your IM Force should be caught or killed the Secretary

will disavow any knowledge of your actions..."*.

2.  BEGIN

This section is the main loop of the program.  It  will  load  pointers  clean

buffers, get records, and convert them to hexadecimal strings for down loading

to the VT-200.  It will also test for buffer overflow, and file errors.

Should there be a file error other than end-of-file, this routine treats  them

as fatal, copies the I/O error code to register 5, and exits with an IOT.

REGISTER	DEFINITION

R0		ADDRESS OF FILE DESC. BLOCK

R1		NUMBER OF BYTES TO BE CLEARED

R2		ADDRESS OF BUFFER TO BE CLEARED

R3

R4

_______________

*  From an old "Mission Impossible" episode, it just seemed to go with the

rest.

Documentation for VT-200 Set up (VT200SET)                              Page 3

R5		Points to VARDAT that is used to initialize func. keys

3.  CPYEXT

Insert end of record marker in VARDAT buffer, that is required by the  VT-200,

send it to the users terminal, close the file and exit.

4.  SIZERR

When the key definitions contain more data than can be put into a VT-200, this

routine issues the following message "Too much data for key definition buffer,

passing what we can!  Last record to be analyzed is as follows:"  followed  by

the last record read from the file VT200SETU.PRM.  After notifying the user we

then try to recover by downloading as much as the VT-200 will permit.

5.  CLNBUF

We allow a record to be variable length, due to this  we  must  zero  out  the

record buffer before we get each new record.

REGISTER	DEFINITION

R0		ADDRESS OF FILE DESC. BLOCK

R1		NUMBER OF BYTES TO BE CLEARED

R2		ADDRESS OF BUFFER TO BE CLEARED

6.  CNVSTR

Verifies that the string is in the legal range (column 1  must  begin  with  a

";",  "1",  "2", or "3"), if it is not we branch to ERRREC, ignore the rest of

the record and issue a message.  If the string begins  with  a  semi-colon  we

will  not  get into this routine because BEGIN tests for that character before

we get to this routine (a semi-colon in column  1  is  a  comment).   We  then

convert  each  byte into its hexadecimal equivalent, unless it is a tilde (~).

A tilde will be translated into a carriage return (hex 0D).

REGISTER	DEFINITION

R0		Address of input buffer from file

Documentation for VT-200 Set up (VT200SET)                              Page 4

R1		High order nibble of byte from the file buffer

R2		Low order nibble of byte from the file buffer

R3

R4

R5		Address of output buffer, to define function keys

7.  ERRREC

This routine is entered when an invalid  key  assignment  has  been  made  its

function  is  to  display  this  error  message;  "Invalid function key number

ignoring this definition, continuing..."  and the  current  record.   It  then

recovers, and returns doing no further processing.

8.  HEXCON

A table to be used by calculated offset to obtain hexadecimal values  for  the

most or least significant portion of the ASCII byte under consideration.

9.  VARDAT

This is the main buffer area for the data going to  the  VT-200.   Any  way  I

calculated  the  buffer size it came out to (512.  + (3 bytes for function key

code number and a /) * 15 definable keys)) 557.  bytes, but when I tested this

program I found that 573.  bytes worked so we used it!

10.  SPILBF

Because the maximum number of  bytes  converted  to  hexadecimal  allowed  per

record  is  512 plus three leading control characters, we must provide a spill

area so we won't get a memory protect violation.

11.  VT200SETU.PRM

The rules for this file are that column 1 must begin with a ";", "1", "2",  or

"3".   Function  key  values  are  found  on page 39 of the "VT-200 Programmer

Pocket Guide", and page 83 of the "VT-200 Programmer Reference Manual".   Each

Documentation for VT-200 Set up (VT200SET)                              Page 5

Record can be a maximum of 256.  characters.  A "~" (tilde) will be translated

into a carriage return.  The example below is a sample  of  a  record  with  a

comment to be translated by VT200SET.

;	Sets Function key F11 to do a PIP for generic work

23/PIP~

INDEX

BEGIN . . . . . . . . . . . . . . . . . 2, 3

CLNBUF  . . . . . . . . . . . . . . . . 3

CPYEXT  . . . . . . . . . . . . . . . . 3

ERRREC  . . . . . . . . . . . . . . . . 3, 4

Function key  . . . . . . . . . . . . . 2, 4

HEXCON  . . . . . . . . . . . . . . . . 4

I/O error code  . . . . . . . . . . . . 2

Invalid function key number ignoring this definition, continuing... 4

IOT . . . . . . . . . . . . . . . . . . 2

Last record to be analyzed is as follows: 3

semi-colon  . . . . . . . . . . . . . . 2, 3

SIZERR  . . . . . . . . . . . . . . . . 3

SPILBF  . . . . . . . . . . . . . . . . 4

tilde . . . . . . . . . . . . . . . . . 2, 3, 5

Too much data for key definition buffer, passing what we can!  3

VARDAT  . . . . . . . . . . . . . . . . 3, 4

VT-200  . . . . . . . . . . . . . . . . 2, 3, 4

VT-200 Programmer Pocket Guide  . . . . 4

VT-200 Programmer Reference Manual  . . 2, 5

VT-200 Programmers Pocket Guide . . . . 2

VT200SET  . . . . . . . . . . . . . . . 5

VT200SETU.PRM . . . . . . . . . . . . . 2, 3, 4

CONTENTS

1.   Introduction  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   2

2.   BEGIN . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   2

3.   CPYEXT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

4.   SIZERR  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

5.   CLNBUF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

6.   CNVSTR  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   3

7.   ERRREC  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   4

8.   HEXCON  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   4

9.   VARDAT  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   4

10.  SPILBF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   4

11.  VT200SETU.PRM . . . . . . . . . . . . . . . . . . . . . . . . . . . .   4

INDEX  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . I-1