CHAPTER 1

USING THE GRAF ROUTINES ON YOUR SYSTEM

1.1  INTRODUCTION

The  programs contained in this account generate graphs

from x-y arrays of  data.   The  graphs  are  labelled  with

numbers  and optional labels, and the full-scale axis values

are selected automatically from the range  of  values  being

plotted.

There  are  lots  of  PLOTTING routines available which

draw lines  and  characters  on  various  plotting  devices.

However  I have not yet seen any GRAPHING routines which can

be used in scientific or engineering applications for graph-

ing  data in a simple manner.  I imagine lots of people have

written their own, just as we have.

The  graphing  routines  make use of a set of low level

plotting routines which follow the  Calcomp  standard.   The

interfaces  for  driving  a  number  of  different  plotting

devices are included, such as Tektronix 4010 compatible ter-

minals,  HP7221 plotters, Epson FX-80 printers.  More inter-

face  routines  can  be  easily  included  by  writing   the

appropriate low level routines and putting them in a new li-

brary.  Also included is a device independent library  which

generates a plot file which can later be plotted on any sup-

ported device.

There  were  two  main  considerations  as this package

evolved.  The GRAF subroutines must be easy to use, and must

not  take  up a lot of memory.  Most users feel that GRAF is

much easier to use than commercial graphics packages such as

DISSPLA,  since  only one or two subroutine calls generate a

nice graph.  And since we are running on 16-bit machines, we

USING THE GRAF ROUTINES ON YOUR SYSTEM              Page 1-2

don't  want  huge subroutines which consider every possibil-

ity.

This  package is known to run on PDP-11's under RSX-11M

V4.0 and later, and probably on earlier versions.   It  also

runs  on RSX-11M/Plus.  I have a version running on the VAX,

although as distributed it  will  take  some  modifications;

see the VAX chapter herein.

These  programs were developed at the National Research

Council of Canada, and are copyrighted by the Government  of

Canada.   They may be copied and used by anyone, but may not

be sold or used commercially without permission.

If  any  bugs  are  found, or improvements made, please

contact:

Dr. David Villeneuve

Division of Physics M23A

National Research Council

Ottawa, Ont.

Canada        K1A 0R6

613-993-9975

1.2  VERSION OF THIS DISTRIBUTION

This  is  the  Fall  1985 version.  It differs from the

Fall 1984 version on the Canadian SIG tape  in  that  it  no

longer  requires a privileged uic to generate it, the inclu-

sion of the Hershey  character  sets,  and  the  spline  fit

curves which jump over symbols.

1.3  GENERATING GRAF AT YOUR INSTALLATION

First,  copy the contents of the uic from the distribu-

tion medium into your favourite uic.  The command files  are

independent  of  the  uic.   A privileged account is not re-

quired unless it finds some utilities are not installed.

Then  say @MAKEGRAF and answer the questions.  You must

specify your Fortran compiler;  we use Fortran IV V2.6,  but

it  should  also work for F77.  It will ask for your Fortran

run-time  library,  which  is  probably  LB:[1,1]FOROTS   or

LB:[1,1]F4POTS;  if you have put it into LB:[1,1]SYSLIB then

just enter S.  It will ask if you have an FCSRES, which  all

tasks will be linked to if you answer yes.

USING THE GRAF ROUTINES ON YOUR SYSTEM              Page 1-3

It will then ask if you want the Hershey character set.

The basic device dependent plotting libraries  rely  on  the

hardware character generator for drawing text.  The plotting

tasks which plot graphs from device-independent  plot  files

generated  by  the PFLIB routines, however, are built with a

software character set which is a large table of pen strokes

for  each character.  There is a choice of two such software

character sets, the first being  a  single  set  of  English

stick characters which are fairly attractive, the second be-

ing a much larger set of simplex,  duplex,  complex,  Gothic

and script English, as well as Greek, cyrillic and mathemat-

ical characters, called Hershey, from the 1980 Sig tape.

I  suggest  the  Hershey  set  if you have lots of disk

space.  Each PLOTxx task takes about 250  blocks  with  Her-

shey.   If  you are tight for disk space, go for the smaller

set.

For  certain  applications,  you  may want to include a

software character set with a device-dependent library,  for

example TKLIB, if you want pretty interactive displays.  You

then have to include the software set at taskbuild time, ei-

ther  from  SWCHAR.OBS for the small set, or HERSHEY.OLB for

the Hershey set.  Look in MAKEGRAF.CMD,  at  the  BUILD  su-

broutine  near the bottom, for an example of how it is done,

or look at the *.TKB and *.ODL files generated.   Note  that

programs using Hershey must be overlaid.

Many  of  the  programs  are  written  in  an  extended

Fortran;  they are of file type FTE rather than  FTN.   They

require  the  use of a fortran preprocessor FTE which is in-

cluded in both task form and in source form.   You  probably

don't  want  to  know  anything  about  it since there are a

number of similar preprocessors about (but since you  asked,

the  reason  I wrote this one was that the others do not ac-

cept a legitimate Fortran program  as  input,  e.g.,  Ratfor

throws  up  at regular Fortran comments;  FTE passes Fortran

right through it, just deleting comments).   If  you  really

object to using FTE, you can use it to translate the program

into Fortran and modify that;  the translated text  has  ex-

tension TMP which you can change for FTN.  MAKEGRAF.CMD will

delete the TMP files so be quick.  If you do, you  may  want

to  put ".COMMENT" as the first line of the .FTE versions so

at least the comments pass through, but the translated  code

still looks kind of funny.

If  for  some reason the supplied FTE.TSK doesn't work,

or if your processor does not have  a  FPP,  the  source  is

given  and can be compiled and taskbuilt using FTEBUILD.CMD.

USING THE GRAF ROUTINES ON YOUR SYSTEM              Page 1-4

Instructions for FTE are in the source, FTEV5.FTN.

Finally,  I have also included some sample Fortran pro-

grams, SAMPLEn.FTN, where n=1-4.  Use these as test programs

once everything has been generated.  Also a sample DRAW com-

mand file, DRAWSAMPL.FIG, is included.  This  makes  use  of

the  DRAWSAMPn.DAT  files.   DRAWALL.FTE  will generate four

pages of all the characters available from the Hershey char-

acter  set.   This  should be task built with PFLIB and then

plotted with one of the PLOTxx  tasks,  assuming  they  were

build with the Hershey set.

1.4  FILES GENERATED BY MAKEGRAF.CMD

The  following  files are generated by MAKEGRAF.CMD and

should be kept.  Any other files may be archived or deleted.

GRAFLIB.OLB   Contains the graphing subroutines.

xxLIB.OLB     Device dependent plotting routines.

PLOTxx.TSK    Device dependent plotting from plotfiles.

DRAW.TSK      Drawgraph utility for generating figures.

SAMPLEn.FTN   Sample Fortran programs.

DRAWSAMPL.FIG Sample DRAW input file.

DRAWALL.OBJ   Compiled DRAWALL for tabulating Hershey.

1.5  USING GRAF AT YOUR INSTALLATION

The  writeup  for  the GRAF subroutines is given in the

next chapter.  This is the same writeup as used in our User-

guide   at   NRC,   and   so   it   contains   a  number  of

installation-dependent things.  In particular we use CCL and

so  a number of commands are handled by CCL.  A CCL template

file is included with the distribution which you may include

in  your  SYSCCL.CCL  file if you have one;  if not you will

have to run the PLOTxx tasks with RUN commands.

The command file MAKEGRAF.CMD will generate a number of

object libraries and tasks, all in the  same  uic  as  this.

Everything  is  uic  independent, so you can copy everything

into your favourite uic or you can wait till later and  copy

the  handfull  of  files that are actually used.  Below is a

list of the files generated and what they are called on  our

system (for comparison with the documentation below).

USING THE GRAF ROUTINES ON YOUR SYSTEM              Page 1-5

GRAFLIB.OLB.  Contains the object forms of the GRAF su-

broutines such as GRAF1,  GRAF2,  GRTITL,  etc.,  plus  some

string  manipulation routines which are used.  You will need

to reference this library every time you taskbuild a program

which calls these routines, or you may copy it into your own

general library.  Ours is called FTNLIB.

TKLIB.OLB.  Contains the object forms of the device de-

pendent plotting routines such as PLOT, SYMBOL,  etc.   This

library  must  be  referenced  after  GRAFLIB every time you

taskbuild a program which calls the GRAF routines  when  you

desire  the  graph  to be plotted immediately on a Tektronix

4010 compatible terminal which must be your  TI:   when  you

run the task.

HPLIB.OLB.   As  above,  except  plots  onto  an HP7221

plotter which must be connected with your TI:.

LPLIB.OLB.   As  above,  except  creates  a file LP.LST

which may be printed on an Epson FX-80  or  FX-100  printer.

This  provides fairly coarse graphs (72 points per inch) but

provides adequate hardcopy for most applications.  This  re-

quires  the use of virtual arrays, and so is not recommended

for direct plotting, but instead should be run from a  plot-

file (PFLIB below).

PFLIB.OLB.   This  library gives the plotting interface

for a device independent pseudo-device.  It generates a neu-

tral  plotfile (default PLOT.PLT) which may later be plotted

on any supported plotting device.

NPLIB.OLB.   This  supports  our Norpak RGP-3000 raster

graphics system.  Since you probably don't have this device,

I have not included it, so ignore references in the documen-

tation to the Norpak or to NP.  I also have not included any

VT55  routines  which are described below, because there are

few of these old terminals about.  We rarely  use  ours  for

graphics.

PLOTxx.TSK.  These tasks take a neutral plotfile as in-

put and plot the contents on the associated plotting  device

"xx",  where  "xx"  is HP, TK, or LP.  There is one of these

tasks for each xxLIB above except PFLIB.  If you don't  have

CCL,  when  you run these they will just prompt with a colon

-- you must enter the name of the  plotfile  to  be  plotted

(default  PLOT.PLT).  If you do have CCL you just say PLOTxx

plotfile.

USING THE GRAF ROUTINES ON YOUR SYSTEM              Page 1-6

When  using the graphics at a terminal (HP or TK) it is

necessary to say SET /NOWRAP=TI:  to  avoid  extra  carriage

control  characters  from  being  inserted  by  the terminal

driver.  Also the PLOTLP  task  creates  a  rasterized  file

LP.LST  which  should  be printed on an Epson FX-80 printer.

This should be handled in CCL to queue the file for printing

at the appropriate type of printer and marked for deletion.

The  documentation in the next chapter assumes that you

use our BUILD command which will  be  available  on  one  of

these symposia tapes someday.  To understand how the example

taskbuilding is done, you should understand how BUILD  works

for  us.   BUILD parses it command line into arguments which

are separated by commas;  arguments  with  subarguments  are

surrounded  by  parentheses.  The first argument is the name

of the task to be taskbuilt, and there  must  be  an  object

file  of  this name.  The second argument is the name of li-

braries to be  used;   the  dollar  sign  is  shorthand  for

SF:[5,3],  which  is where we keep all our common libraries.

BUILD always searches SF:[5,3]FTNLIB.OLB (your  GRAFLIB.OLB)

before  the  named libraries, so when you taskbuild you must

include your GRAFLIB explicitly, even though it isn't  named

in  the  examples below.  If you don't have any resident li-

braries (we have FORRES and FCSRES) you may taskbuild as

TKB myprog/CP/FP=

myprog,[YOUR]GRAFLIB/LB,[YOUR]xxLIB/LB,LB:[1,1]F4POTS/LB

CHAPTER 2

INSTALLING ON A VAX SYSTEM

For  the  VAX/VMS version, only the following files are

required.  Some minor changes to the  source  code  are  re-

quired  in  those files marked with an asterisk below.  Edit

the files, and look at the code whenever the word "VAX"  ap-

pears.

FTEV5.FTN (*)                 LPPLOT.FTE (*)

STRING.FTN                    PFSUBS.FTN

GRAF.FTE                      SWCHAR.FTN

GRAFADV.FTE                   PLOT.FTE

PARSCL.FTE                    DRAW.FTE

TRIG.FTN                      PLOTBUILD.COM

CONVERTNO.FTE                 FTC.COM

GETMCRVAX.FOR                 MAKEGRAF.COM

SYMBOSFUL.FTE

SYMBOS.FTE

TKPLOT.FTN

TKSUBS.FTN (*)

The  command  file is MAKEGRAF.COM, and doesn't ask any

questions.  Don't change the FTN  extensions  to  FOR  until

after  you've run the command file.  Then you may change the

names of the sample programs if you wish.

Note  that  the Hershey character set is not available,

since it is coded in Macro-11.  I started to convert to  VAX

Macro, then thought it would be stupid to have two versions.

I think it would be better to translate it into a  bunch  of

Fortran DATA statements, and code it all in Fortran which is

much more transportable.  It anyone  does  this  (I  won't),

please send me the code.

When  finished, you can copy these files to another di-

rectory.  I assign a logical symbol  which  refers  to  that

INSTALLING ON A VAX SYSTEM                          Page 2-2

directory,  which makes linking easier, and also lets others

use the libraries.  You may also want to include some  parts

of LOGINVAX.COM in your LOGIN.COM file.

PLOT%%.EXE

%%LIB.OLB

GRAFLIB.OLB

DRAW.EXE

TKRESET.TXT

The  PLOTLP task does not work, due to a problem in su-

broutine LPPLOT in module LPPLOT.FTE  where  the  rasterized

bitmap  is  written into the output file.  There seems to be

some peculiarity in VAX Fortran in assigning 8  bit  numbers

to  bytes,  giving an overflow error.  Someone with more VAX

experience will probably spot the problem immediately.

The  HP  7221  routines are not included because I have

not tried them on the VAX.  I suspect that  the  output  re-

quest  routine at the end of HPSUBS.FTN will have to be mod-

ified, since it uses QIO  so  that  an  RSX  system  without

typeahead  doesn't  lose characters.  Probably just READ and

WRITE statements would work alright.

Please  note  that  this  VAX  distribution is not well

tested.  The code works well under RSX, and I use  the  GRAF

subroutines  on  TEK  terminals  under VMS, but there may be

some problems which I didn't forsee.   Again,  comments  are

welcome.

CHAPTER 3

PLOTTING

This chapter describes the Fortran callable subroutines

which permit the graphing of functions in  a  fairly  simple

manner.  The first section describes the high level routines

which provide autoscaling labelled x-y graphs of  points  or

lines.   The  second section describes the low level "primi-

tive" routines which are used by the high level routines  to

provide device-independent plotting.

We  have  a number of different plotting devices:  Tek-

tronix emulating terminals (e.g.  Retrographics), VT55  ter-

minals,  Epson  FX-80  matrix  printers,  the  Norpak colour

raster display, and the HP7221B four  colour  plotter.   The

VT55  is not a true generalized plotting device, and as such

is treated differently from the others.

The plotting primitives are device independent.  At the

time the program is written, it is not necessary  to  decide

which  device  is to be used.  This decision is made at task

build time, when the device-dependent library  is  selected.

To build a program using these primitive routines is simple.

Just add the name of the appropriate library for the desired

output device to the BUILD command.  If you wish to have ac-

cess to more than one output device, you  may  specify  that

the  plot output be directed to a file, which may be plotted

later by the PLOTxx commands on various devices;   this  has

the  advantage that software character sets are used for the

plot.

PLOTTING                                            Page 3-2

Library    Device

$PFLIB     Device independent plot file.

$TKLIB     Tektronix type terminals; you must be

logged into one.

$HPLIB     HP7221B plotter; you must be logged

in through it.

$NPLIB     Norpak colour display; not necessary

to log in to it.

$LPLIB     Line printer output onto matrix printer.

For  example,  to  select the plot file output, use the

build command BUILD MYPROG,$PFLIB.  If you  require  another

library,  enclose  the  names  in parentheses, BUILD MYPROG,

($INENLIB,$PFLIB).

Usually   one   does   not   build   a   task   with  a

device-specific library for one of the hard copy device such

as  HP or LP.  For these, build with PFLIB instead, and then

plot on the appropriate hardcopy device via PLOTxx.  In fact

the  only  time  to  use a device-specific library at all is

when making interactive programs that ask questions, plot  a

graph,  and  ask  more  questions;  TKLIB is appropriate for

these.

If  you  select  a  plot file output (PFLIB), when your

program runs it will ask you to enter the name of  the  plot

file.   By just pressing return, you select the default name

PLOT.PLT.  If one exists already, a new version number  will

be  created.   You may enter any name you wish.  To plot the

contents of a plot file, enter the command PLOTxx, where  xx

is  the two-letter code for the selected device like the li-

brary names above (TK, HP, NP, LP).  If the plot  file  name

is not PLOT.PLT you must also put on the command the name of

the plot file (default extension .PLT).  It  will  ask  that

you hit return before each frame is displayed.  You can skip

some frames at the beginning by specifying the  first  frame

to  skip  after  the  filename on the command line;  you can

terminate plotting by entering control-z instead of  return.

If  you  enter a negative number for the first frame to plot

(e.g., PLOTHP myfile.PLT,-1), it will ask you to  enter  the

pen  numbers  to use for each pen colour;  this is occasion-

ally used on the plotter to override the  pen  colours,  for

example when using a single india ink pen.

Plot  files  produced  by PFLIB may be concatenated to-

gether using PIP to make a single larger file.  For example,

PIP PLOTALL.PLT=PLOT.PLT;* and then PLOTLP PLOTALL.

PLOTTING                                            Page 3-3

Note  that  these plotting routines all require the use

of logical unit 1 and local event flag 1, so do not  attempt

to open files on lun 1.

3.1  SAMPLE PLOTTING PROGRAM

Before  launching  into a description of all the avail-

able subroutines, look at the example Fortran program  below

to  see how simple it is to generate nice graphs.  Note that

while three subroutines were called, a single call to  GRAF1

would suffice to generate a reasonable curve.  All the other

routines described later are  optional  and  only  embellish

some features.  Don't let their number frighten you.

C SAMPLE PLOTTING PROGRAM

REAL X(100), Y(100)

C

C SET UP THE CURVE

C

DO 10 J=1,100

X(J) = J

10   Y(J) = SIN(X(J)/20.0) * EXP(-X(J)/35.0)

C

C GRAPH THE CURVE

C

CALL GRSYM (66)       !SQUARES WITH LINES

CALL GRAF1 (X,Y,100)  !PLOT THE GRAPH

C

C PLACE TITLE ON GRAPH

C

CALL GRTITL ('Section 4.1 Sample Plot', 100,

1 'X Axis label centred', 100, 'Y Axis', 100)

CALL EXIT

END

>FTN TESTPLOT

>BUILD TESTPLOT,$PFLIB

>RUN TESTPLOT

PLOTS -- Enter name of plot file [PLOT.PLT]:

>PLOTHP

PLOTS -- enter options (? for help): 0.33,20

>

PLOTTING                                            Page 3-4

3.2  GENERALIZED GRAPHING ROUTINES

These  are  a set of high level routines made up of the

low level routines, which allow the user to graph simple x-y

functions.   They  do  not require any plotter initiation or

termination -- this is all done for you.   At  the  simplest

extreme, a single call to GRAF1 produces a readable graph.

The  descriptions for the available routines are organ-

ized as follows.  First are the routines which actually gen-

erate  a  graph,  followed  by  those routines which can add

something to a graph, followed by  those  parameter  setting

routines  which  are  called before other routines to modify

their action, followed by misc routines.

3.2.1  ROUTINES WHICH GENERATE GRAPHS

3.2.1.1  GRAF1 (X, Y, NPTS)  -

This  plots  a  set  of  points  defined by the vectors

(X,Y), whose first NPTS elements are used.  The  scales  are

chosen  depending on the range of values in (X,Y).  The axes

are labelled and a grid is drawn.  Points are  connected  by

lines.   Calling this routine alone will generate a readable

graph, all other routines are optional.

3.2.1.2  GRAF2  (X,  Y,  NPTS,  MX,  MY,  ILOG, IOPT, TITLE,

XLABEL, YLABEL)  -

This  is similar to GRAF1 in that it plots a graph of y

versus x with the option of autoscaling.  It draws the  grid

and  labels it.  In addition, however, it may plot from 1 to

6 graphs on the same page in various locations, depending on

the  choice  of MX,MY.  GRxxTK, GRxxTK may be called before-

hand, GRLOC may be called afterwards, and the scales may  be

retrieved  afterwards  by GRGSCA and GRLSCA.  The log/linear

state is set as if a call to GRLOG were made  and  stays  in

effect when a later call to GRAF1 is made.  All other param-

eter setting routines for GRAF1 have no effect on GRAF2.

X  and  Y  are real arrays dimensioned to at least NPTS

elements.  If solid lines are to connect data points, one of

the  arrays  should  be  monotonic.   NPTS is the numbers of

points to be plotted.  If NPTS is negative,  the  number  of

PLOTTING                                            Page 3-5

points  is  taken  as the absolute value, and the points are

plotted on top of the existing graph using the scale  values

of the last call to GRAF2, without drawing a new grid.

MX and MY are integers in the range 1-3 which determine

where on the page the present graph is to go.   If  MX=1  it

fills  the  width  of  the  page;  if MX=2 it fills the left

half;  if MX=3 it fills the right half.  Similarly  for  MY.

Thus  MX=1,MY=2 draws a graph in the upper half of the page.

A new page is begun only when the top left graph is plotted,

so always begin at the top left.

ILOG  is  an integer which determines the linear or lo-

garithmic nature of the graph.  For ILOG=1 to 4,  the  graph

will  be  lin/lin, linx/logy, logx/liny, and log/log respec-

tively.

IOPT  is an integer which determines how the data curve

is to be drawn.  This parameter is coded as  a  decimal  in-

teger  as <ISSC>, where each letter represents a decimal di-

git, and <C> is the pen colour, <SS> is the plotting symbol,

and  <I>  is  the  increment  for plotting a symbol at every

<I>th point.  Use C=1-4 for the colours  black,  red,  green

and  blue  respectively.  <SS>=00 for no symbol, 01 for dots

of colour C, 65-72 for special symbols in black (see GRSYM).

<I> is 0 for plotting the symbol only at each point, greater

than 0 for plotting the symbol on top of the connected curve

at  every  <I>th oint.  A special case is IOPT=1 to 4, which

draws a connected curve of colour IOPT.

If  IOPT  is  negative, no data points are plotted, but

the grid is drawn and labelled, and the scale is chosen from

the  values  in  X,Y.  This way one can force the scale, for

example by using the first two values of X and  Y  to  store

the endpoints.

TITLE,  XLABEL  and  YLABEL are character strings to be

written on the top, bottom and left  side  respectively,  of

the  graph.   These  are  ASCIZ  strings which may be either

quoted strings or byte arrays with a zero byte at the end.

PLOTTING                                            Page 3-6

3.2.2  ROUTINES WHICH ADD TO AN EXISTING GRAPH

3.2.2.1  GRTITL  (TITLE,  LTITLE,  XLABEL,  LXLABEL, YLABEL,

LYLABEL)  -

This annotates the graph just drawn by GRAF1 with a ti-

tle above it, and labels on the x and y axes.  Each title is

a character string, followed by its length in bytes.  If the

titles are specified as quoted strings, the  length  may  be

set  to  a  large  number.  For example, CALL GRTITL ('graph

title', 100, 'x label', 100, 'y label', 100).  This must  be

called  after  the  GRAF1  call.   See the SYMBOL subroutine

under primitive routines to see what special characters such

as Greek and subscripts are available.

3.2.2.2  GRDATE  -

Places  the  date and time in the lower right corner of

the graph just drawn by GRAF1.  This must  be  called  after

the GRAF1 call.

3.2.2.3  SYMBOL  -

This can be called to write on the graph.  See the des-

cription in the next section under primitive routines.   Un-

less  it is followed by a call to GRTITL or GRDATE, you must

follow these primitive routines by CALL PLOTF to  flush  the

buffers.

3.2.2.4  GRAF1A  (XC,  YC,  NC, IC, ILINE, THICK, IPENC, XS,

YS, NS, IS, IPENS, DIA, ISYMB)  -

This  routine  may  be called after GRAF1 or GRAF2 have

generated a graph, with or without data points.   Using  the

last scale, it can plot a set of symbols and/or a curve such

that the curve jumps over the symbols.  This looks  nice  in

that  no  part of the line touches the symbol markers.  Also

special thick dashed lines are used, which are not available

from  the  basic  GRAF1 and GRAF2.  These embellishments are

included in  a  separate  routine  so  that  basic  graphing

routines can be kept as compact as possible.

XC,YC  are  the coordinates of a set of NC points which

define the line to be drawn.  The values in XC  must  be  in

PLOTTING                                            Page 3-7

ascending order.  IC is the interleave factor, so that every

IC'th point is plotted (usually IC=1).

ILINE  is the code for the line type to draw, between 1

and 10 (see GRDSHL for a description  of  the  line  types).

THICK  is the width of the line in inches (0.1 to 0.15 looks

good).  IPENC is the pen colour  to  use  when  drawing  the

line.

XS,YS  are  the  coordinates  of the NS points at which

symbols are to be drawn or avoided.  These points  need  not

bear  any relationship to those points in XC,YC, and can re-

present any number of points where the line is  not  desired

to  go.   However these points must be in ascending order in

x.

IS  is  the  interleave factor for the symbols, so that

every IS'th point is plotted (usually IS=1).

DIA is the diameter of the symbol in inches.  If DIA is

negative, no symbols are drawn, but the  line  still  avoids

those locations to the given ABS(DIA).

ISYMB is the code for the type of symbol to use at each

point.  See SYMBOL for a description of the special plotting

symbols  available in the range 65 to 101.  IPENS is the pen

colour to use when plotting the symbols.

3.2.3  PARAMETER SETTING ROUTINES

These  routines  optionally  come  before  the plotting

routines, and affect their action.  Most of them stay in ef-

fect  until  again  changed,  but some are reset immediately

after a graph is generated.

PLOTTING                                            Page 3-8

3.2.3.1  GRSYM (ISYM)  -

This defines the type of symbols to be used to plot the

points in all subsequent calls to GRAF1.  If not called,  it

defaults to ISYM=1.

ISYM > 0  the  colour  defined  by ISYM will be used to draw

lines between individual points.  The  default  is

1,  i.e.   a black pen.  See the notes in the next

section under  NEWPEN  for  a  discussion  of  pen

colours.  Generally ISYM will be in the range 1-4.

Note that if ISYM is positive, lines will be drawn

between  points,  and so one of the variables must

be monotonic to prevent retrace.

-65 < ISYM < 0  use pen colour abs(ISYM), but only put a dot

at each point.

ISYM < -64 use  special  symbols  at each point, as follows.

Square, delta, circle, diamond, del, plus,  times,

star,  correspond to -65 to -72 respectively.  See

the paragraph on SYMBOL below  for  the  available

symbols and codes.

ISYM > 64 use special symbols as above, but also connect the

points with a line.

3.2.3.2  GRLOG (LX, LY)  -

This sets the x and/or y axes as logarithmic in all fu-

ture calls to GRAF1, if LX or LY are set to 1.  If the value

is 0, the corresponding axis will be linear.  The default is

linear axes.

3.2.3.3  GRNOGR  -

This causes the next call to GRAF1 to be produced with-

out any grid at all.  Only the data points will be  plotted.

This  is  used  when  it is desired to overplot the previous

graph.

PLOTTING                                            Page 3-9

3.2.3.4  GRLSCA  -

This  causes  the  next  call to GRAF1 to use the scale

used on the previous graph.  This can be used in conjunction

with  GRNOGR to plot several curves on the same scale.  Oth-

erwise each new call to GRAF1 will result in a new scale be-

ing chosen depending on the range of the data.

3.2.3.5  GRNOHA, GRYEHA  -

These  control  the  drawing  of lines across the graph

axes.  GRNOHA says to draw small tick marks around the  edge

of  the graph, GRYEHA says draw lines right across the graph

at major tick marks.  The default is small ticks.

3.2.3.6  GRNOTK, GRYETK  -

These control the drawing of tick marks between the ma-

jor tick marks around the edge of the graph.  GRYETK says to

draw  10 minor ticks between the major ones.  GRNOTK says do

not draw minor ticks.

3.2.3.7  GRSSCA (X0, X1, DX, Y0, Y1, DY)  -

This causes the next call to GRAF1 to use the specified

scale values rather than choosing its own.  These values are

in  the  user's units.  For linear scales, X0 is the x value

which will be plotted on the left side of the graph,  X1  on

the  right  side.   DX is the spacing between grid lines and

labels, again in the user units.  For example, if one wished

to  plot the next graph from -5 to 15 marked every 2.5 in X,

and from 0 to 500 marked every 100 in Y:  CALL GRSSCA (-5.0,

15.0,  2.5,  0.0,  500.0,  100.0) For logarithmic scales, it

uses the log base ten of  the  range  of  values,  and  DX=1

always;   for  example, a log scale from 1E3 to 1E6 is given

as X0=3, X1=6, DX=1.

PLOTTING                                           Page 3-10

3.2.3.8  GRGSCA (X0, X1, DX, Y0, Y1, DY)  -

This  retrieves  the  scale values used by the previous

call to GRAF1.  See GRSSCA above for a  description  of  the

values.   This  is useful for plotting several curves on the

same graph.

3.2.3.9  GRGPAR (IPENS, HITES)  -

This  subroutine gets the parameters used by the graph-

ing routines, such as pen colours and  symbols  heights  for

various  parts  of the graphs.  The arguments must be dimen-

sioned as IPENS(10) and HITES(10).  The returned values cor-

respond  to  the  pen  number and character height in inches

used for the following components:

(1) Main title

(2) X and Y axis titles

(3) Numbers on the axes

(4) Plotting symbols, e.g. triangles and diamonds

(5) Date

(6) Major tick marks (HITE is length of ticks)

(7) Minor tick marks (HITE is length of ticks)

(8) Frame (HITE is width of line using 3 strokes)

(9) Curves (HITE is ignored; overridden by GRSYM and

GRAF2)

3.2.3.10  GRSPAR (IPEN, HITE)  -

This changes the parameters used to plot various compo-

nents of the graph.  See the description of GRGPAR above for

an  explanation of what the values correspond to.  Generally

one calls GRGPAR first to load the default values  into  the

arrays,  then changes the ones desired and then calls GRSPAR

to affect the changes.

PLOTTING                                           Page 3-11

3.2.4  MISCELLANEOUS ROUTINES

3.2.4.1  GRLOC (XP, YP, XUSER, YUSER)  -

This  converts  the user coordinates XUSER and YUSER to

plotter coordinates XP and YP in  inches,  using  the  scale

from  the  last  call to GRAF1 or GRAF2.  This is useful for

locating points on the graph for placing labels on points.

3.2.4.2  GRSPLI  (XOUT,  YOUT, NOUT, XOUT0, XOUT1, XIN, YIN,

NIN, WORK)  -

This  routine takes a set of points XIN,YIN, and gener-

ates a smooth curve which passes through all the points,  by

means  of  fitting  piecewise  cubic  splines to the points.

This routine is useful in putting a smooth curve  through  a

small  number of points which are relatively smooth, such as

calculated points along a theoretical curve.   It  does  not

look  very  good  with  a lot of points if there is a lot of

scatter, since it will put the curve through all the points,

unlike  least-squares  curve fitting.  The curve can then be

plotted by any of  the  usual  graphing  routines,  such  as

GRAF1.

It is capable of extrapolating outside the range of the

data, but since it uses cubic fits, they generally  curve  a

lot  more  than  you would wish.  Also the slope at the end-

points is a free variable, and so the ends of the curve  may

not be as you wish.  If this is the case, give an extra pair

of endpoints which go the way you want.

The input parameters XIN,YIN are the coordinates of the

points which define the curve, and NIN is the number of such

points.  NIN must be at least 4.

XOUT,YOUT are arrays dimensioned to at least NOUT which

will contain the coordinates of the smooth curve.  XOUT0 and

XOUT1  are the minimum and maximum value for x which will be

calculated.  XOUT will contain NOUT  equally  spaced  values

from XOUT0 to XOUT1, and YOUT will contain the corresponding

y values.  As mentioned above, there need not be  any  rela-

tionship  between  XOUT0,XOUT1 and the values in XIN, but it

is preferrable that they lie inside the range of XIN.

WORK  is a work array which must be provided for calcu-

lating coefficients, and it must be dimensioned to at  least

PLOTTING                                           Page 3-12

3*NIN.

The array XIN must increase from left to right.

3.2.4.3  GRDSHL (X, Y, IPEN, ITYPE, THICK)  -

This  routine  is  like  the primitive plotting routine

PLOT in that it draws lines, and in fact the first three ar-

guments  are  the  same as PLOT's.  However it is capable of

drawing various types of dashed and dotted  lines  of  arbi-

trary thickness.

X  and  Y are the coordinates in inches of the point to

move to.  If IPEN=3 the pen is raised before the  move,  but

if  IPEN=2  the  pen is lowered and a line is drawn from the

current point to X,Y.

ITYPE  is  an  integer  which determines the line type.

THICK is the thickness of the line in inches;   usually  0.1

to 0.15 looks good.

The dashes in the line will be continuous no matter how

small the increments in X and Y are between calls, until the

pen is raised at which time it starts again at the beginning

of the dash.

The available line types are as follows.

ITYPE   Line Type

0      No line

1      Solid line

2      Short dashes

3      Long and short dashes

4      Long dashes

5      1 long and 2 short dashes

6      Dots

7      1 long and 3 short dashes

8      2 long and 1 short dash

9      2 long and 2 short dashes

10      2 long and 3 short dashes

PLOTTING                                           Page 3-13

3.2.4.4  GRSCAL (X, Y, NPTS, X0, X1, DX, Y0, Y1, DY)  -

This  scans  through  the  array of points (X,Y), which

have NPTS elements, and chooses a nice set of scale  values.

This  routine is called automatically by GRAF1, but may find

other uses.

PLOTTING                                           Page 3-14

3.3  MORE SAMPLE PROGRAMS

Here  are more sample Fortran programs using the graph-

ing routines.

C THIS GENERATES A SAMPLE GRAPH USING GRAF.

C IT DEMONSTRATES HOW TO OVERPLOT ON THE SAME GRAPH.

C

REAL X(100), Y(100)

DO 10 J=1,100

X(J) = J-1

Y(J) = SIN(X(J)/5.0) * EXP(-X(J)/30.0)

10      CONTINUE

C

C DRAW THE CURVE USING THE DEFAULT BLACK LINE

C

CALL GRAF1 (X, Y, 100)

CALL GRTITL ('Sample 2', 100, 'X axis label', 100,

1       'Y axis label', 100)

C

C NOW OVERPLOT WITH SYMBOLS

C

CALL GRNOGR     !DON'T REDRAW GRID

CALL GRLSCA     !USE LAST SET OF SCALE VALUES (DON'T

AUTOSCALE NEW)

CALL GRSYM (-65)!USE SYMBOLS AT EACH DATA POINT

CALL GRAF1 (X, Y, 100)  !REPLOT POINTS

CALL GRDATE

CALL EXIT

END

C THIS DEMONSTRATES THE USE OF GRAF2, BY PLOTTING

C SEVERAL GRAPHS ON THE SAMPLE PLOT FRAME.

C

REAL X(100), Y(100)

DO 10 J=1,100

X(J) = J

Y(J) = SIN(X(J)/5.0) * EXP(-X(J)/30.0) + 1.0

10      CONTINUE

C

C DRAW THE CURVE, WITH AUTOSCALED AXES.

PLOTTING                                           Page 3-15

C

CALL GRYETK             !DRAW MINOR TICK MARKS

CALL GRAF2 (X, Y, 100, 2, 1, 1, 5661,

1       'Sample 4 - lin/lin', 'X axis', 'Y axis')

C

C DRAW ANOTHER CURVE BESIDE PREVIOUS ONE

C

CALL GRAF2 (X, Y, 100, 3, 1, 4, 9981,

1       'Sample 4 - log/log', 'X axis', 'Y axis')

CALL EXIT

END

3.4  PLOTTER PRIMITIVE ROUTINES

These are the routines which enable plotting to be done

independently of the actual plotting device used.  It is re-

commended  that  all programs use these routines rather than

the device specific  ones  if  possible.   The  hypothetical

plotting  device has a horizontal dimension of 12 inches and

a vertical dimension of 10 inches.  All plotting is done  in

these  units.  The actual size may be scaled to fit the real

plotter.

The  device  is  chosen at task build time by selecting

the appropriate library.  See the previous section.   Slight

differences in the actions of the routines are noted below.

Calls to these subroutines may be mixed with the higher

level routines described in the previous section.  For exam-

ple,  SYMBOL  may  be used to annotate graphs that have just

been drawn.  It is advisable to do  so  after  the  call  to

GRAF1, and before a call to either GRTITL or GRDATE.  If you

use these routines after all  such  calls  on  a  particular

graph,  you must call PLOTF to flush the buffer;  GRTITL and

GRDATE do this for you.

PLOTTING                                           Page 3-16

3.4.1  PLOTS

This  must be called before any other plotting routine.

It initializes both the software and the plotter device.  It

may  be  called repeatedly in the program if PLOTF is called

in between.  On terminals, it asks the user to hit  the  re-

turn  key and then clears the screen.  On the HP plotter, it

asks the user for a scaling factor and a plotting speed.  At

this  point  the paper is loaded.  The scale factor defaults

to 0.75, which reduces the 12 by 10 logical plotting surface

to  fit  onto  11 by 8.5 inch paper.  The plotting speed de-

faults to 36 cm/s, but may be slowed down for final  copies.

Generally  calls  to  to  PLOTS and PLOTF will bracket every

page plotted.

3.4.2  PLOTF

This  must  be called at the end of every plotting ses-

sion.   It  empties  the  internal  buffers  and  waits  for

transmission  to  finish.   It  may  also be called part way

through the program.

3.4.3  PLOT

PLOT (X, Y, IPEN)

This  is the basic plotting routine.  It causes the pen

(or beam) to move to location  (X,Y),  where  X  and  Y  are

measured  in  inches  from the current origin.  The value of

IPEN determines whether a line is drawn on the way.

IPEN = 3  =>  raise the pen before movement.

IPEN = 2  =>  lower the pen before movement.

IPEN = 1  =>  no change in pen status.

IPEN < 0  =>  same effect as abs(IPEN), but causes the

destination to become the new origin.

PLOTS sets the origin initially to the lower left corner.

PLOTTING                                           Page 3-17

3.4.4  SYMBOL

SYMBOL (X, Y, HEIGHT, TEXT, ANGLE, NCHARS)

This causes the NCHARS characters in TEXT to be written

starting at location (X,Y) in inches from the  current  ori-

gin.   ANGLE  is  the angle in degrees counterclockwise from

the positive X axis that the characters are  to  be  plotted

at.  HEIGHT is the character height in inches.  If TEXT is a

quoted string, NCHARS may be set to a large number  and  the

length is determined automatically.

If  the  program  is  taskbuilt with a plotting library

which outputs directly to a plotting device, SYMBOL will use

the  hardware  character generator.  On certain devices this

will look funny since characters will not  be  rotated  cor-

rectly.   However  software  character  sets  may be used to

overcome this.  Plots produced by PLOTxx  automatically  use

the software character sets.

If  NCHARS=-1  or  -2,  the decimal value of TEXT(1) is

used to select a special plotting symbol which  is  centered

at  X,Y.   If  NCHARS=-2  a  line is drawn from the previous

point to the current one, otherwise no line is drawn between

symbols.   For  TEXT(1),  one may use either an integer or a

quoted ascii character.  E.g., CALL SYMBOL (x, y, 0.15,  66,

0.0,  -1), where 66 could be replaced with 'B'.  The symbols

available are:

Int Code  Ascii Symbol     Int Code  Ascii Symbol

--------  ----- ------     --------  ----- ------------

65 or -65   A   square     97 or -97   a   solid square

66 or -66   B   delta      98 or -98   b   solid delta

67 or -67   C   circle     99 or -99   c   solid circle

68 or -68   D   diamond   100 or -100  d   solid diamond

69 or -69   E   del       101 or -101  e   solid del

70 or -70   F   plus

71 or -71   G   times

72 or -72   H   star

With  the hardware character set, only the lefthand co-

lumn is available;  solid symbols are drawn  as  their  open

counterparts.  The software character set includes the right

hand column of solid symbols, and draws them a  bit  better.

Again, PLOTxx uses the full set.

The  character string to be written may include certain

control characters which affect the size and position of the

characters.   These  control  characters consist of a dollar

PLOTTING                                           Page 3-18

sign ($) followed by one or two other characters,  as  shown

below.

$A  Shift up to above division line.

$B  Shift down to below divison line.

$C  Advance to the next line, single spacing.

$D  Shift down to first level subscript.

$E  Shift up to exponent level (smaller than superscript).

$G  Change to GREEK character set for next character only.

$H  Advance to next line, one and a Half spacing.

$I  Shift down to index level (smaller than subscript).

$J  Jump back to last level command position.

$L  Shift down to second level subscript.

$M  Change to Math character set.

$N  Shift to normal level.

$R  Restore position.

$S  Save position.

$T  Advance to next line, double spacing.

$U  Shift up to superscript level.

$V  Generate a vector/mean value bar between occurences,

($V...$V).

$W  Change to script character set.

$X  Return to entry character set.

$<SP>  Generates a half-space.

$n  Change to character set n, n=3,4,...,29.

$.  Generate a "Bullet".

$*  Generate a five pointed star "Bullet".

$/  Generate a division line between occurences, ($/...$/).

$\  End of text.

$-  Underline between occurences, ("$-...$-").

$=  Double underline between occurences, ("$=...$=").

$$  $

{   Shift up to exponent level, like $I, or back to

normal if after }.

}   Shift down to index level, like $I, or back to

normal if after {.

For  example,  "W/cm{2}  $Gx{0}".  If switched to char-

acter set 9, the program will switch back to the entry char-

acter set after any character is referenced, i.e.  to get an

integral sign from character set 9 and then  return  to  the

entry  character  set,  you  need only type:  $9I not $9I$X.

The same is true for $G.  To select  a  different  character

set,  use  $n,  where  n is the one or two digit code of the

character set.

Below  is  a list of the character sets available.  The

default is 3, but this may be changed on the PLOTxx  command

line  in  the  third parameter, usually to the duplex set 5,

PLOTTING                                           Page 3-19

e.g.  PLOTHP plot.plt,,5.  If this is  done,  then  $G  also

selects the complex Greek set.

3  Simplex Roman

4  Simplex Greek

5  Duplex Roman

6  Complex Roman

7  Complex Greek

8  Complex Italic

9  Math and special characters ($M)

10  Math and special characters

11  Gothic English

12  Simplex Script ($W)

13  Complex Script

14  Gothic Italian

15  Gothic German

16  Cyrillic

17  Triplex Roman

18  Triplex Italic

20  Miscellaneous characters.

3.4.5  SYMBOC

SYMBOC (X, Y, HEIGHT, TEXT, ANGLE, NCHARS, ICENTR)

This  is  identical to SYMBOL above, except for the new

last argument ICENTR, which determines how the text is to be

centered.  For ICENTR=-1, the text is left justified at X,Y;

for ICENTR=0 it is centered on  X,Y;   for  ICENTR=1  it  is

right  justified  at  X,Y.   The  bottom of the line of text

always touchs X,Y, regardless of angle.  Note that with  the

software  character  set,  using  multiple lines in a single

call, all the lines will be justified individually.

3.4.6  NUMBER

NUMBER (X, Y, HEIGHT, FNUM, ANGLE, NDEC)

This  causes the floating point number FNUM to be writ-

ten starting at location (X,Y) in inches  from  the  current

origin, with HEIGHT in inches, and at ANGLE degrees counter-

clockwise from the positive X axis.  NDEC is the  number  of

digits  after  the  decimal  point.   If NDEC=-1, it will be

written as an integer.  Otherwise it will be either in F  or

E format, depending on the value of the number.

PLOTTING                                           Page 3-20

3.4.7  NEWPEN

NEWPEN (IPEN)

This  is  used with output devices which are capable of

drawing in more than one colour.  If a monochrome device  is

used, it will ignore the choice of pen.

It  causes  subsequent  lines or characters to be drawn

using pen number IPEN.  IPEN may vary from 0 to 4 for the HP

plotter,  and from 0 to 255 for the Norpak.  However for the

Norpak, the lookup table is not set for colours higher  than

4;  this may be done by NPLLT, which has no effect on the HP

plotter.  Note that the pen is not actually changed until an

attempt  is  made to draw with it, to reduce unnecessary pen

motion.  The supplied colours are:

IPEN = 0  =>  put away current pen.

IPEN = 1  =>  black pen (white on Norpak)

IPEN = 2  =>  red pen

IPEN = 3  =>  green pen

IPEN = 4  =>  blue pen

Numbers  higher  than  4 give a blue pen on the HP, and give

whatever the lookup table has been set  to  on  the  Norpak.

This has no effect on monochrome devices.

3.4.8  PLOTNM (filenm, icode)

This is an optional routine which is ignored by all li-

brary versions except PFLIB.  It is used to give a  name  to

the output plot file through the ASCIZ string FILENM.  ICODE

is not used at this time, and should be coded with  a  zero.

If  this  routine  is called before the first call to PLOTS,

PLOTS will not prompt the user for  the  name  of  the  plot

file.   Note  that the filename extension .PLT should be in-

cluded in the filename given.  This is  useful  in  programs

which  know  the name of the plotfile already, such as names

derived from another known filename.

PLOTTING                                           Page 3-21

3.4.9  SOFTWARE CHARACTER SET

The PLOTxx commands which plot from plotfiles use large

tables of pen strokes to generate  the  characters  used  to

draw  text  via  the SYMBOL routine.  These tables require a

lot of memory;  to keep task size down for interactive tasks

which  use  the  device-specific  libraries,  such as TKLIB,

these libraries will instead use the hardware character  set

available on the output device itself.

However  if  better  quality interactive characters are

desired, an intermediate software character  set  is  avail-

able, which does not support Greek or fancy lettering.  This

is loaded by including the file SF:[5,3]SWCHAR.OBS as an ob-

ject file:

BUILD (progname,SF:[5,3]SWCHAR.OBS),$xxLIB

Since  the PLOTxx commands always use the software character

set, it is not necessary to specify SWCHAR if you build with

$PFLIB.

3.5  VT55 VIDEO TERMINAL

Because  the  VT55 terminals have limited capabilities,

they are not treated as generalized plotting devices and  so

cannot  use  the  general  routines  described above.  A su-

broutine PLOT55 is provided by DEC  to  handle  the  graphic

features  of the VT55.  Two high level routines are provided

in VT55LIB for graphing simple functions on  the  VT55.   To

build  a program using these routines, specify $VT55LIB as a

library.

3.5.1  GRAPH1

GRAPH1 (X, Y, NPTS)

This  plots a graph of a single function of Y versus X,

tabulated in the vectors X and Y.   The  scale  factors  are

chosen  automatically  and the axes are labelled.  The X va-

lues must be monotonic since multiple valued functions  can-

not  be displayed.  NPTS is the number of (X,Y) points to be

plotted.

PLOTTING                                           Page 3-22

3.5.2  GRAPH2

GRAPH2 (X1, Y1, NPTS1, X2, Y2, NPTS2)

This  plots a graph of two independent functions.  Oth-

erwise it is similar to GRAPH1.

3.5.3  TITLE1

TITLE1 (LINE, NCHARS)

This  writes  a  title at the top of a graph created by

GRAPH1 or GRAPH2.  The title is a character string in  LINE,

of length NCHARS.  If LINE is a quoted string, NCHARS may be

set to a large number.

3.6  DRAWGRAPH UTILITY

We  have a utility program called DRAWGRAPH or DRAW for

short which makes use of the graphing routines described  in

the  first  section  of  this  chapter.  Rather than calling

GRAF1 from inside a Fortran program which you have  written,

you  construct a file of special instructions for DRAW which

then draws the graph(s) into a plotfile.  This is  used  for

generating view-graphs for conferences and for publications.

It gives a bit more control over the more  detailed  aspects

of a plot, such as symbol size and pen selection.

The  input  file  should  have  extension  FIG, such as

VUGRAF.FIG.  Each line of the file should start with a verb,

some  of which are shown below.  The verb may be followed by

some optional numbers, separated by spaces or  commas,  e.g.

SYMBOL-HEIGHT  0.2.   The  verbs  must be given exactly, al-

though upper and lower case are ignored.  There are a number

of  parameter  setting verbs, then some plotting verbs (PLOT

and OVERPLOT) which put everything together into a graph.

To  specify  the  coordinates  of the data points to be

plotted, they should be in a  file  with  default  extension

DAT, each x-y pair on their own line with free format.  This

filename is then given with the PLOT  or  OVERPLOT  command.

The  data  points may also be included in the FIG file after

the PLOT command if no filename is given;  the data are then

terminated by an END verb.

PLOTTING                                           Page 3-23

It  is  preferable  to draw first the data points which

are plotted with symbols, then to plot the  curves  on  top,

because  the  program  will endeavour not to draw a curve on

top of a symbol already there.  It can remember the location

of a reasonable number of symbols.

To  run, just enter DRAW figname, and it will then make

the graph(s).  The plotfile may be plotted in the usual way,

e.g., PLOTHP myplot.  The command line may also contain more

arguments, each separated by spaces or  commas.   The  first

one,  the  name  of the input file which contains the direc-

tives, is required;  it will be given default extension .FIG

if  not already there.  The second argument is optional, and

may be the name of the plot file to create;  if  not  given,

it will be asked for.

DRAW inputfilename plotfilename P1 P2 ...  P9

The  rest of the command line is optional, and may con-

tain from one to nine more arguments.  These will be substi-

tuted  for  all  occurrences  of  the  string  %n%  in  each

directive line read, where n=1-9 is the number corresponding

to  which argument is desired.  %1% corresponds to the first

such argument, the third one on the command line.

3.6.1  Verbs Used before the Graph is Drawn

First,  some comments on setting scales.  If you do not

specify any scales, the program will choose a  nice  set  of

scales  from the range of the data.  However you may want to

override this mechanism, e.g.  if the x-axis is degrees from

0  to  180, you may want the axis labelled at 0, 30, 60, 90,

120, 150, and 180, rather than the default 0, 50, 100,  150,

200.   You  may use X-SCALE and Y-SCALE to do this (or equi-

valently X-FIRST, X-LAST, X-INC, etc.)

If  you want part of the axis not to be labelled or for

the first tick mark to start at some arbitrary location from

the  corner,  then  you  can  specify the values at the four

corners using X-SPAN  and  Y-SPAN  (or  equivalently  X-LEFT

etc.).   If  these are not given then the corners correspond

to the values set by X-SCALE and Y-SCALE.  You  can  mix  up

defaults  and  overrides  together;   the  program  tries to

determine what you want.

The  following are parameter setting commands, and must

be given before the PLOT command.  Their values stay in  ef-

fect until changed.

PLOTTING                                           Page 3-24

X-LEFT,  X-RIGHT,  Y-BOTTOM, Y-TOP set the range of the

graph in user's units.  For log scales, these values are the

log base 10 of the values, e.g.  1E-5 is set as -5.  Default

values are chosen from the range of the data.

X-FIRST,  X-LAST, Y-FIRST, Y-LAST, X-INC, Y-INC set the

values at which the major tick marks and numbers are written

on  the  axes.  For log scales, these are the log base 10 of

the value, and the increments must be set to 1.  Default va-

lues are chosen from the range of the data.

X-SCALE,  Y-SCALE  have three parameters, corresponding

to x-first, x-last, x-inc described in  the  previous  para-

graph.  This is an equivalent way of setting the scales, but

all 3 values must be given or will be set to zero.

X-SPAN,  Y-SPAN  have  two parameters, corresponding to

x-left, xright etc.  above.  Both must be given if this com-

mand is used.

X-LINEAR, X-LOG, Y-LINEAR, Y-LOG set the axes as either

linear or logarithmic.  Default is linear.

LEFT-EDGE,  RIGHT-EDGE,  BOTTOM-EDGE,  TOP-EDGE set the

location of the corners of the graph frame on  the  plotting

page.   These  are  in  inches on a 12 by 10 inch page.  You

must also leave room below and to the left for axis  labels.

Default is 2, 10, 2, 8.75 inches.

LINE-TYPE  specifies  the  type  of  line that is to be

drawn between data points.  It takes three parameters:   the

line  type  code  (1-9,  see  writeup  for GRTHKL), the line

thickness in inches, and the pen number  to  use.   The  de-

faults are 1 (solid line), 0.08, 1.

SYMBOL  says that symbols are to be drawn at every data

point, and selects the symbol to be used.  See  writeup  for

GRSYM  subroutine.   Parameter  is  in the range -65 to -72.

(SYMBOL-PEN sets colour).  The default is  to  draw  a  line

between points (see LINE-TYPE), but the default for symbol's

parameter is -66.

SPLINE  says  that  the  next PLOT or OVERPLOT is to be

fitted with a smooth curve which  is  generated  by  fitting

piecewise cubic splines to the data points.  This works best

with a small number of data points since  the  curve  passes

through  every  point.  The two parameters give the left and

right x-values between which to  draw  the  curve.   If  not

given,  the  curve  will be drawn between the first and last

PLOTTING                                           Page 3-25

points.  The x curve must go from left  to  right  monotoni-

cally.

X-FACTOR,  Y-FACTOR  give the scaling factors to trans-

form the data points.  These commands take two parameters, A

and  B,  and the corresponding coordinate is transformed ac-

cording to X' = AX + B.  This stays in effect until a  RESET

or  another X-FACTOR command.  The default transformation is

1, 0.

MAIN-TITLE, X-TITLE, Y-TITLE will print the rest of the

line at the corresponding position on the graph.

PUBLICATION  sets  a  number of parameters suitable for

graphs in publications.  The characters are such  that  they

will reduce to 2 mm in a 3.125 inch column.  The pen for the

main title is 2, so it may be used for notations when  doing

drafts;  for the final copy, remove pen 2 from the plotter.

VIEWGRAPH sets a number of parameters suitable for mak-

ing viewgraphs.

TICKS  /  NO-TICKS  controls the minor tick marks.  De-

fault is ticks.

GRID-LINES / NO-GRID-LINES controls the drawing of grid

lines right across the plot.  Default  is  no,  just  larger

ticks.

DATE  /  NO-DATE  controls printing date in lower right

corner.  Default is no date.

DECIMALS-X,  DECIMALS-Y  specifies the number of digits

after the decimal point to use when putting numbers  on  the

axes.   This  overrides  the automatic mechanism which occa-

sionally gets mixed up when scales are overridden.  A  posi-

tive  value gives that many decimal places, a negative value

specifies E format.

MAIN-TITLE-HEIGHT,    AXIS-TITLE-HEIGHT,   DATE-HEIGHT,

NOTE-HEIGHT, set the height in inches of the  characters  of

these labels.  Default 0.22, 0.22, 0.1, 0.22 inches.

NUMERAL-HEIGHT sets the height of the numbers which la-

bel the axis values.  Default is 0.16 inches.

SYMBOL-HEIGHT  sets  the  size of the plotting symbols.

Default is 0.22 inches.

PLOTTING                                           Page 3-26

MAJOR-TICK-LENGTH, MINOR-TICK-LENGTH sets the length of

the ticks around the graph.  Defaults are 0.25, 0.1 inches.

FRAME-THICKNESS  sets  the width of the frame, which is

drawn with three passes of the pen, each separated  by  this

distance.  Default is 0.012 inches.

MAIN-TITLE-PEN,  AXIS-TITLE-PEN, NUMERAL-PEN, DATE-PEN,

SYMBOL-PEN,   MAJOR-TICK-PEN,   MINOR-TICK-PEN,   FRAME-PEN,

NOTE-PEN, select the pen colours for the corresponding parts

of the graph.  Values are in range 1-4 for HP plotter.   De-

fault 1.

NOTE-ANGLE  is  the angle of the notes in degrees coun-

terclockwise from the x axis.  Default 0.

COMMENT  is  used  to  denote a comment line and is ig-

nored.

RESET  will  reset  the  scales  and attributes back to

their default values.  It does not affect  character  height

or pen choices.

3.6.2  Verbs to Draw the Graph

The  following commands generate the graph, so all par-

ameter setting commands should  have  been  given.   However

parameters  may be changed before OVERPLOTS to change scales

and plot characters, etc.

PLOT  filename will cause the named file to be read for

xy pairs, then the graph is plotted using the parameters set

up  to that point.  If filename is not given, the points are

read from the following lines up to an END directive.

OVERPLOT  filename is the same as PLOT, except that the

curve is plotted on top of the existing one.  No axes or la-

bels are drawn.

PLOTTING                                           Page 3-27

3.6.3  Verbs after the Graph is Drawn

These  commands should come after the PLOT command, and

may come before or after OVERPLOTs.

NOTE  x,y  text and NOTE-UU x,y text will plot the text

onto the graph.  X and y are the coordinates for  the  lower

left  point  of  the  text;  for NOTE the coordinates are in

inches, for NOTE-UU they are  in  the  current  user  units.

These  must  be used AFTER the PLOT command since scales are

not set until the first PLOT.

NOTE-CENTER,  NOTE-RIGHT  will  center or right-justify

the next note only, then NOTE will revert to left-justifying

the text.  See also NOTE-ANGLE.

3.6.4  Sample DRAWGRAPH Input File

Below  is  a sample of an input file to DRAWGRAPH which

is used to draw a figure for a publication.  The filename is

DRAWSAMPL.FIG,  and the plotfile is created by entering DRAW

DRAWSAMPL.  It first draws three sets of data  points  using

symbols,  then  draws  smooth  splined  curves through them.

Note that the curves jump over the symbols.   Finally  three

labels are written on the left to identify the curves.

Contents of file DRAWSAMPL.FIG:

PUBLICATION

Main-title 3/2 $Gx Fig 8

X-title Wavelength ($Glm)

Y-title 3/2 $Gx}0{ Intensity

Comment Change some parameters to suit this narrow graph

NO-ticks

no-y-axis

Numeral-height 0.2

Symbol-height 0.15

frame-thickness 0.01

left-edge  2

right-edge 7

top-edge 9

bottom-edge 1

Comment Set the scales

x-scale 7.05, 7.1, 0.05

PLOTTING                                           Page 3-28

x-span  7.005, 7.13

y-bottom 0

y-top 10.5

Comment Plot the data with symbos first

symbol -65

plot DRAWSAMP1.dat

symbol -66

overplot DRAWSAMP2.dat

symbol -68

overplot DRAWSAMP3.dat

Comment Plot smooth curves through the points

Line-type 1 0.05

spline

overplot DRAWSAMP1.dat

Line-type 2

spline

overplot DRAWSAMP2.dat

Line-type 3

spline

overplot DRAWSAMP3.dat

Comment Label the curves

note-uu 7.01 1.5 30{o}

note-uu 7.01 4.5 100{o}

note-uu 7.01 7.5 150{o}

The  program  will  read  from three data files, DRAWS-

AMPn.DAT.  Those should contain a number of lines  with  two

numbers  per  line  in  any format, corresponding to the x-y

coordinates of each point in any units.  These numbers could

also  have  been included after the PLOT or OVERPLOT verb in

the DRAWSAMPL.FIG file, with an  END  verb  after  the  last

line.

Contents of file DRAWSAMP1.DAT

7.0415E+00  6.3978E-01

7.0532E+00  3.6926E+00

7.0626E+00  2.3157E+00

7.0706E+00  2.4131E+00

7.0786E+00  3.3936E+00

7.0860E+00  2.8999E+00

7.0947E+00  5.8414E-01

7.1026E+00  1.8428E+00

7.1106E+00  2.3435E+00

7.1188E+00  5.5633E-01