Disk Save and Compress (DSC)  Tape Formats

1-April-81

I. INTRODUCTION

This note describes what I have learned of the format of DSC produced

magnetic tapes.  The information presented here is to be considered prelimi-

nary and subject to revision. The Stand-Alone DSC V2 was used to create test

tapes for this project.

Bill Burton

Texas Research Institute of Mental Sciences

1300 Moursund

Houston, Texas  77030

(713) 797-1976 ext 501

II. DSC TAPE STRUCTURE

Basically, DSC tapes are ANSI standard version 1 labeled magtapes.

Standard version 1 has been superceeded by version 3 (ANSI X3.27-1978).

Some of the label records are non-conforming or undefined in the version 3

standard.

The DSC tape volume consists of ANSI labeled volume and header

labels surrounding DSC formatted data records.

Record Number  Length  Contents

1	80.	VOL1 label record

volume identifier derived from the output file name

given in the DSC command line for the first output

file.

a. Truncated to 4 characters if necessary

b. Padded to 4 characters using the character X

c. Sequence number, 2 characters

01 for first reel of a set

02 for second, etc.

d. Other information is documented in ANSI Standard

Manual or RSX-11 I/O Operations Manual.

2	512.	Dummy bootstrap block which produces the message

"This volume does not contain a hardware bootable

system".

3	80.	HDR1 label record

a. File name specified in DSC command line

b. Volume identifier from VOL1 record

4	80.	HDR2 label record

a. An undefined record code (U) is non-standard

b. Logical record length 2064 bytes

c. Physical record length maximum 2064 bytes

d. The character M in byte 37, non-standard

e. Buffer offset 00 in bytes 51-52.

5	Tapemark

6	1040.	First DSC formatted record, contains

a. 8 word (16 bytes) DSC header with code=40

b. 512. bytes of DSC Bookkeeping information

(documented in section IV)

c. 512. byte Files-11 file header for the file

(1,1) [0,0]INDEXF.SYS;1

7. 	1040.	DSC Data record

should contain initialization data for disk

structure, 2 virtual blocks worth.

8.	2064.	DSC Data record

More initialization information for virtual

block 377,177777.

9. & up 528.	(or 1040. or 1552. or 2064.)

Contain DSC data records with DSC codes 1, 2, or 4

The details of these records are in section III.

DSC copies files to the output tape in file-number

order.  For each input file on the disk there is

1 type 2 record (of unknown function)

1 type 4 Files-11 Header record

as many type 1 data records as needed to

hold all virtual blocks allocated to the

input file.

At the end of data from a disk volume, there will be

Record number	Length	Contents

N	tapemark

N+1	80.	EOF1 record

Identical to HDR1 except bytes 55-60 now contain

the number of physical tape records between

HDR2 and EOF1.

N+2	80.	EOF2 record

Identical to HDR2 record

N+3	tapemark

N+4	tapemark, if this is the end of the tape volume OR

80.	HDR1 for the next output tape file

created using DSC /AP switch.

If the tape is filled, (ie reaches the End-of-Tape foil marker)

the EOF1 and EOF2 records will be replaced by EOV1 and EOV2 records.

The following reel in the set will start with a VOL1 label with

the output volume name incremented by 1.  The structure of all subsequent

reels is the same as described above for the first reel, except that data

records start immediately.  There will be no DSC initialization records

(6, 7, and 8. above) on subsequent reels.  It is possible for data from a

disk file to be split across two reels, and the data for a file can be

separated from its DSC header records or Files-11 header record when

changing reels.

III. DSC DATA RECORD FORMAT

Within the labeled file, DSC writes its own format records.

These records contain:

a. An 8. word header

b. 1, 2, 3, or 4-512. byte logical data blocks.

DSC header Format

word 1	Length of the data portion of the record, excluding ALL DSC

specific information.

word 2	DSC record-type code indicating the type of data in the data

portion of the record.

code = 1   Disk data record(s), images of input disk blocks

2   Prefix information.  I have not fully decoded the

data in these records.  May have some filename

and extent information.

4   Files-11 file header (records of code 4 only have

one 512. byte data block).

40   Special DSC initialization record.

a. 512. bytes of initialization information

see section IV.

b. 512. byte Files-11 header for INDEXF.SYS

words 3-4	For type 1 records, these words contain the Virtual Block Number

(VBN) of the of the first logical block in the DSC data record.

The low order part of the VBN is in word 3, the high-order

part is in the low byte of word 4.

I have not fully understood the use of these words for the

second data record for the INDEXF.SYS file.

word 5	The file number this record applies to

word 6	The file sequence number this record applies to.

words 7-8	Always seem to be zero.

IV. DSC INITIALIZATION RECORD

This record contains the usual 8-word DSC header with a code

of octal-40 in word 2.  The data portion of the record contains 512.

bytes of DSC control information plus the 512. byte Files-11 file header

for (1,1) [0,0]INDEXF.SYS;1

The DSC control information is formatted within the record as follows:

Bytes		Contents

1.- 12.	DSC Output name from the command line, ASCII padded with

spaces

13.- 24.	Input device name (e.g. DK1), ASCII, padded with spaces

25.- 36.	? null

37.- 48.	Input volume name, ASCII, padded with nulls

49.- 52.	? null

53.- 54.	? 1

55.- 58.	Number of blocks on the input device, binary

59.- 62.	Number of blocks in use on the device, binary

(not including bad blocks)

63.- 66.	Number of files in use on the device, binary

67.- 68.	?

69.- 70.	? null

71.- 72.	? Number of blocks allocated to index file

73.- 74.	?

75.- 76.	?

77.- 78.	?

79.- 80.	?

81.- 82.	?

83.- 84.	?

85.-510.	? null

511.-512.	checksum

Items marked with ? are items I have not identified.

V. DSC CODE 2 RECORDS

One of these records preceeds each files-11 file on the DSC tape

DSC header word 1 = 512

word 2 =   2

word 5 =   file number of the files-11 file

word 6 =   file sequence of the files-11 file.

DSC data block, 512. bytes

words 1-3  the string BACKUP

word    5  The number of bytes in the name string

words 6..  The file name string, in the form

DDU:[126,342]filename.ext;ver

the UIC 126,342 seems to be constant,regard-

less of what the source or directory uic was.

word 46.   Number of blocks allocated to the file

word 52.   (same as word 46.)

word 67.   Owning UIC group number

word 68.   Owning UIC member number.

There is some more information, but I have not determined what

it is, it was not necessary for restoring files.

VI. DSC FILES-11 HEADER RECORD

One of these records immediately follows the DSC code 2 record

and contains:

DSC header word 2 = 4  code for files-11 header record

word 5 = files-11 file number for this file

word 6 = files-11 file sequence number for this file.

The data portion of the record contains the 512. byte files-11

header block for the file.  It is an exact image of the header as it was

on the disk, including the map area.